3GPP TSG RAN WG1 #113	R1-2305137

Incheon, South Korea, May 22 – 26, 2023

Agenda item:		9.12.2

Source:	Nokia, Nokia Shanghai Bell

Title:	RAN1 impacts for power domain enhancements

Document for:		Discussion and Decision

# Introduction

This contribution relates to a work item agreed in RAN#94-e, namely “Further NR coverage enhancements” (the WI was revised in RAN#96 [1]). In this paper we consider power domain enhancements and the following objectives captured in the WID:

- Study and if necessary specify following power domain enhancements

  - Enhancements to realize increasing UE power high limit for CA and DC based on Rel-17 RAN4 work on "Increasing UE power high limit for CA and DC", in compliance with relevant regulations (RAN4, RAN1)

  - Enhancements to reduce MPR/PAR, including frequency domain spectrum shaping with and without spectrum extension for DFT-S-OFDM and tone reservation (RAN4, RAN1)

We discuss the scope of power domain enhancements and RAN1 specification impacts. We also provide CM/PAPR and receiver performance results of DMRS used for FDSS-SE.

# CA/DC objective discussion

## Rel-17 increased UE power high limit for CA and DC

Rel-17 Work Item on increasing UE power high limit for CA and DC [4], [5] introduced a new maximum output power (MOP) capability HigherPowerLimitCADC in particular for a UE supporting PC3 (23dBm) in one band (TDD or FDD) and PC2 (26dBm) in another band (TDD). With the new MOP capability the UE indicating PC2 as CA or DC configuration can use the full power from both PAs with maximum composite power of 27.8 dBm while Rel-16 UE would be limited to PC2 (26dBm) in CA or DC configuration. Hence, the new capability (per band combination) lifts the limitation due to PC2 CA or DC configuration and allows UE to transmit maximum power in both carriers, i.e., 27.8 dBm altogether.

Maximum uplink duty cycle is a closely related feature. In the existing specification, UE can indicate capability maxUplinkDutyCycle-interBandCA-PC2, which indicates the maximum percentage of uplink symbols during a certain evaluation period that UE can transmit to ensure SAR requirements while using a power class larger than the default UE power class for the band.

Based on the maximum uplink duty cycle, it is possible to operate using a specific power class as long as the percentage of UL symbols does not exceed maxUplinkDutyCycle-interBandCA-PC2. So if for example a UE is capable of 26 dBm (PC2) on one band and maxUplinkDutyCycle_PC2_FR1=50%, UE can operate using PC2 if the percentage of transmitted uplink symbols does not exceed 50% of the evaluation period. If the duty cycle is exceeded, UE falls back to the default power class of 23 dBm (PC3). Further, the term ΔPPowerClass,CA is set to 3 dB, which may lower the high and low limits (PCMAX_H and PCMAX_L) for the total configured max output power, depending also on the other parameters.

## Enhancements for information exchange

With the Rel-17 enhancements for increasing UE power high limit for CA and DC in place, it is worth considering whether gNB and UE can utilize the UE’s higher total maximum output power efficiently based on current specifications. Occasionally UE may be scheduled so frequently that the average percentage of UL symbols exceeds the duty cycle limit, causing UE to fallback to the default PC. Change on the power class impacts the way UE selects the configured maximum output power for each serving cell, and the impact may be different from a simple difference between the power classes. Currently there is no way for UE to inform gNB of such change. Correspondingly, the following was agreed in RAN1 #112, providing guidance for the future RAN1 discussions on the topic

Agreement

Further discussions in RAN1 concerning means to facilitate higher power transmissions in CA and DC, if applicable, can target increasing gNB awareness of UE’s Tx power, e.g., PHR reporting enhancement such as current power class, power class change, or application of P-MPR by UE (subject to RAN4’s input).

  - FFS: details.

During RAN4#106, the possible solutions for the information exchange between the UE and gNB were discussed (Issue 4 and 5 in Topic#2 in the WF [2]) although without consensus on them [3]. On the Issue 5, the following information reporting options are listed [2]:

    - Power class fallback ΔPPowerClass with aperiodic PHR.

    - Power class being used by the UE.

    - The sustainable duty cycle over a certain duration that would prevent triggering a power class fallback at the UE, as well as period of applicability of the ∆PPowerClass report.

    - Introduce a scheme for a UE to report uplink symbol evaluation period and starting timing.

    - Enhance the current power headroom reporting framework to enable P-MPR reporting (via MPE field) for FR1 carriers.

The possible solutions were discussed in RAN1#112bis-e, leading to the following observation:

Observation

RAN1 discussed advantages and disadvantages of solutions included in R1-2302270 (R4-2303701) on enhancements to realize increasing UE power high limit for CA and DC. Pros and cons of the inclusion in the PHR report of at least one of the following quantities have been analyzed for different reporting mechanisms, triggers, and reporting periodicities:

- ∆PPowerClass

- Power class

- P-MPR

- Start and length of evaluation period for power class fallback

- Estimated duration of power class fallback

- Estimated duration over which UE can sustain Pcmax before additional P-MPR is required

- Sustainable duty cycle to prevent a fallback

- Energy/power availability

Note: Discussion is still ongoing, and its full current content can be found in Section 2.1.2 of R1-2303924.

In the RAN1 discussions, the possible solutions were categorized into reactive and proactive enhancements, where, e.g., reporting of as ΔPPowerClass, current UE power class, or P-MPR were listed as reactive enhancements.

When considering the reporting of the currently used UE PC, either explicitly or via the ΔPPowerClass, information on the UE’s PC can be utilized by gNB for scheduling as well as for the use of UE configurations, as the power class impacts also other UE requirements in addition to the UE’s total configured maximum output power:

- UE PC can impact the MPR table that is applied: the UE PC has impact on the relationship between A-MPR value and RB allocation location and length for some bands and NS values (e.g., with NS_05 for band n1). This means that UE’s configured max transmit power can change based on PUSCH or PUCCH RBs allocation in a way that is unexpected to gNB.

- MSD requirements for a band combination can be significantly different for each PC and the difference can be over 10 dB. For example, the CA band combination n2-n77 has MSD due to IMD4 occurring on band n2 DL. The MSD is 8 dB for PC3 while it is 19.1 dB for PC2. Hence, the current PC can affect dramatically also PDSCH scheduling and whether CA is configured or not.

- Reasonable configurations of SRS transmit antenna port switching pattern and UL full power transmission mode depend on PC of UE, and UE fallback to lower PC can lead to inadequate performance.

Hence, we see value on indicating the UE’s current PC to the gNB.

However, the reporting of ∆PPowerClass does not always unambiguously indicate the used power class (further details can be found in [6]). For example, ∆PPowerClass value is changed when:

- IE powerBoostPi2BPSK is set to 1

- P-max of P dBm or lower is indicated (P is lower than advertised power class)

- UE is configured with SUL configurations

- the percentage of uplink symbols transmitted in a certain evaluation period is larger than the default value, or the configured maxUplinkDutyCycle-PC2-FR1, maxUplinkDutyCycle-PC2-FR1/2, or maxUplinkDutyCycle-PC1dot5-MPE-FR1 value.

- For TxD UE, during SRS antenna switching with t1ry(y>1) (details are omitted).

From these cases, UE power class requirements are changed only when the uplink duty cycle is exceeded or low P-max is configured. Hence, it is unclear how ΔPPowerClass reporting would indicate a change in the power class. These problems are avoided by reporting power class directly. Explicit power class reporting, instead of indirectly via ∆PPowerClass, can also be seen more futureproof. Hence, we prefer that the UE would report the PC itself directly.

Reporting of either UE PC or ∆PPowerClass in PHR is rather simple enhancements and straightforward to specify. One item to discuss is the conditions for the reporting. We see that gNB would benefit if UE reports a change in its PC when the change occurs instead of waiting for the next periodic event. Periodic reporting would cause also unnecessary PHR reporting and overhead. Event-based (i.e., PC change based) aperiodic reporting can mitigate the PHR reporting overhead while still improving gNB awareness. The reporting events can be limited with the PHR prohibiting timer.

Proposal 1: PHR can be configured to contain the current PC that is used by the UE per serving cell as well as the currently used CA PC.

- Aperiodic reporting triggered by PC change and limited by PHR prohibiting timer is supported.

Reporting of P-MPR on FR1 power headroom report represents also a simple, straightforward change. However, it is not clear to us how P-MPR alone, without any further assistance information e.g., on P-MPR duration, can be used to enhance network operation, as current PHR provides information about the available UE Tx power. UE may change the P-MPR due to reasons and at time instances that are unknown to the network and not necessarily impacted e.g., by UL scheduling. Additionally, the reported P-MPR value does not indicate e.g., how long the reported value would be valid. Hence, the possibilities to benefit from P-MPR value alone in the UL scheduling and link adaptation are not clear.

The potential value of the considered reactive reporting options can be enhanced with additional assistance information that helps gNB e.g., to proactively control, and possibly avoid, the PC changes by adjusting the UL scheduling. Correspondingly, several possibilities for such proactive enhancements were discussed in RAN1#112bis-e. However,  the mechanisms would require more thorough discussions and have somewhat larger specification impact than the simple reactive enhancements. Due to limited remaining time for the WI, we propose to deprioritize the proactive enhancements and to focus on reactive enhancements only.

Proposal 2: P-MPR and proactive reporting enhancements are deprioritized for Rel-18.

# MPR/PAR objective

## Work split between RAN1 and RAN4

The following was agreed in RAN1 #110bis-e.

Agreement

The following work split principles will be adopted in RAN1 for power domain enhancement throughout Rel-18 from RAN1 perspective and send LS to RAN4 in this meeting:

- RAN1 performs link level simulations of candidate solutions for power domain enhancements to study at least the SNR variation, PAPR/CM, and EVM, brought by each solution.

  - Transparent MPR/PAR reduction solutions can be considered as a benchmark for studying the performance of non-transparent solutions.

- RAN1 is not expected to perform RF simulations of candidate solutions for power domain enhancements

  - Results of RF simulations can be included in RAN1 contributions

- RAN1 will assess RAN1 specification impact of candidate MPR/PAR reduction solutions

  - A list of candidate solutions, including necessary parameters, from RAN1 perspective should be ready before the end of RAN1 #111, and should be included in an LS to RAN4.

- RAN1 understands that RAN4 is responsible for selecting the Rel-18 MPR/PAR reduction solution, if any.

The following was concluded in RAN1 #111

ormance comparison based on net gain results combining transmitter and receiver performance is performed by RAN4.

- No final decision would be taken by RAN1 on which MPR/PAR reduction solution, will be specified in Rel-18, if any, since this is RAN4’s responsibility.

    - It does not preclude RAN1 specification impact

The following was agreed in RAN4 #104bis-e.

Agreement:

- RAN4 follows below RAN1 agreements and focus on prepare for RF simulations

  - Establish evaluation parameters and side-conditions if any for both transparent and non-transparent schemes

    - The parameters and side-conditions will be updated if needed according to RAN1 input

    - Share the agreements with RAN1 that could affect RAN1 link level simulation

  - RAN4 can perform evaluations without RAN1 input for both transparent and non-transparent schemes

    - No discussion on simulation results of non-transparent scheme at least in RAN4#105

Agreement:

Actual conclusion of the MPR/PAR reduction methods should be based on net coverage gain results combining transmitter and receiver performance

Finally, the following was agreed in RAN4 #106bis-e.

Agreement:

- RAN4 agrees to analyse provided simulation results as attached to this WF via Excel and provide conclusions based on these at RAN4#107.

  - It is expected a decision on selecting the Rel-18 MPR/PAR reduction solution(s) according to the WID can be taken at RAN4#107

There has been considerable progress in evaluation assumptions in both RAN1 #111 and RAN4 #105. On top of that both RAN1 and RAN4 sent LSs summarizing the key parameters [7, 8]. Based on those, the assumptions behind LLS (RAN1) and RF simulations (RAN4) seem to be well aligned. RAN1 has sent LS [9] with link level results to RAN4 to support RAN4 decision of MPR/PAR reduction solutions for Rel-18 power domain enhancements. RAN4 has agreed to analyse simulation results provided by the companies and make a decision on selecting the Rel-18 MPR/PAR reduction solution(s) at RAN4#107.

Observation 1: Based on the progress in both RAN1 and RAN4, the simulation parameters are well aligned between RAN1 and RAN4

Observation 2: RAN4 does not need any further input from RAN1 for selecting the Rel-18 MPR/PAR reduction solution(s)

## RAN1 impact for supporting spectrum extension

The following was agreed in RAN4 #104bis-e and RAN1 #110bis-e.

Agreement: : (RAN4 #104bis-e) Ensure fair comparison between different methods by keeping the total allocated bandwidth, the spectral efficiency and resource in time domain the same for all compared cases as much as possible. In addition, it can be considered that efficiency not always the best judging criteria, e.g., there is a case that efficiency is of less concern than the link level benefit.

Agreement: (RAN4 #104bis-e) - For simulation purpose, tentatively, define extension/reservation factor (a) as Excess band size / Total allocation, where - Inband size: Occupied REs after DFT-block - Excess/reserved band size: The amount of spectrum extension. - Total allocation size (Inband size + Excess/reserved band size): Occupied REs after spectrum extension - The definition is tentative and needs final confirmation with RAN1 in the future meetings. Agreement (RAN1 #110bis-e) The following design aspects of frequency domain spectrum shaping with spectrum extension (FDSS-SE), are considered for studying MPR/PAR reduction enhancements in Rel-18: - Spectrum extension size is expressed in integer units of RBs. - Both DMRS and data symbols undergo spectrum shaping - FFS: - Which extensions factor(s) to consider, where extension factor (α) is given by spectrum extension size / Total allocation size. - Impact of shaping filter on FDSS-SE performance - How to extend DMRS sequence to spectrum extensions, based on either the existing ZC-sequence DMRS or low-PAPR DMRS for PUSCH (FG 16-6c) - How extension size is determined

Inband/DFT size:

Concerning the number of PRBs in the so-called inband, i.e., the number of PRBS obtained by subtracting the number of PRBs in the spectrum extension from the number of PRBs in the total allocation, we observe that its size would coincide with DFT size for the Transform Precoder. In this context, it is reasonable to enforce that current DFT size limitations defined for DFT-s-OFDM already for LTE and NR Rel-15 should be respected also for FDSS-SE. Based on that, DFT size should be expressed as multiple of 2a* 3b* 5c, where [a, b, c] are integers ≥ 0. This would mean that any inband size in FDSS-SE should correspond to a valid DFT size as per existing values, and FDSS-SE does not require the introduction of new DFT sizes as compared to legacy. Any other considered power domain enhancement based on spectrum extension should also abide to this principle.

Observation 3: Power domain enhancement does not require definition of additional DFT size options

Proposal 3: No new additional DFT size options to be introduced by RAN1 to support Rel-18 power domain enhancements.

Extension size:

Definition of extension factor has been agreed in RAN4 #104-bis-e for simulation purposes. This coincides with the definition agreed in RAN1#110bis-e. We propose to confirm this definition for the possible normative work.

Proposal 4: Confirm definition of extension factor (a) as excess band size / total allocation size.

In our RAN4 contribution [10] we analyze net gains and conclude that extension factor 0.25 maximizes net gain in most of the scenarios, hence we propose at least 0.25 to be agreed.

Proposal 5: Support at least extension factor (a) 0.25. Other extension factors are FFS.

FDRA indicator:

In terms of signaling, it is important to minimize the specification effort while preserving legacy operations as much as possible. For what concerns the FDRA indicator, two options seem possible:

- The indicator provides the start/end RBs for inband signal

- The indicator provides the start/end RBs for the total allocation (which would be identical to legacy)

Both options can preserve a certain number of aspects of legacy operations and require a minimum number of modifications:

- INBAND:

  - PROS: Signaling the inband allocation through FDRA indicator would yield an indication to a number of PRB which respect the restriction to valid DFT sizes only, and legacy TBS determination using MCS index would be maintained.

  - CONS: Conversely, the total number of PRBs to be used to generate the DMRS sequence for FDSS-SE according to existing agreements and to make use of the power control allocation would need to depend on the configured/indicated spectrum extension factor (implicitly or explicitly).

- TOTAL ALLOCATION:

  - PROS: Signaling the total allocation would be like legacy behavior. In this case, the FDRA would immediately provide the total number of PRBs to be used to generate the DMRS sequence for FDSS-SE according to existing agreements and to make use of the power control allocation would need to depend on the configured/indicated spectrum extension factor (implicitly or explicitly).

  - CONS: Conversely, TBS determination using MCS index would need to take into account the configured/indicated extension factor, and so would need to be the DFT size calculation for the transform precoder.

Overall, signaling inband allocation seems to be better option due to the direct relationship with the valid DFT size for the transform precoder and legacy TBS determination procedure. Signaling the total allocation can also work but is slightly less attractive in our view.

Proposal 6: First preference is to use existing FDRA indicator to signal inband allocation in the context of FDSS-SE. Second preference is to use the indicator to signal the total PRB allocation for FDSS-SE.

Fractional RB:

RAN1 has done the agreement above for studying MPR/PAR reduction enhancements in Rel-18, which states that “extension size is expressed in integer units of RBs”. This could cause ambiguous understanding in view of a possible normative work, since “extension size” could refer to the total size of the extension or the size of one side only. In this context, the RAN4 agreement above uses a slightly different wording, i.e., “excess/reserved band size”. However, in this case, the size is the size of the total excess band. Furthermore, it is worth observing that all scenarios for simulations included in the working assumptions made by RAN1 in Rel-18 include an even number of PRBs in the total extension. Further clarification on this aspect would be helpful.

Observation 4: Meaning of “extension size” in the agreement from RAN1 #110bis-e shown above should be clarified.

The following observation is also in order. Irrespective of the whether the total number of PRBs in the spectrum extension is an even or odd number, the total allocation size should be an integer number of PRBs. Consequently, a rounding issue may then occur for some configurations, depending on the considered inband size and extension factor, which may result in a total allocation size that may or may not be multiple of 12 REs, as illustrated in Table 1.

Table 1. Total allocation size (RBs) for possible values for extension factor (a)

Several approaches could be considered to tackle this issue:

- Consider orphan REs as guard band (unused band)

- Consider inband sizes resulting in fractional PRB as invalid allocations.

- Ceiling/flooring towards the next full RB (or even number of PRBs)

- Others.

From our perspective:

- Approach a has the benefit that extension would be available to all inband allocations. Drawback would be resources spent as guardbands.

- Approach b could be seen as least complex solution at least for the UE, however it has the drawback that the extension is not applicable for all inband allocations. This issue could be mitigated by agreeing several extension factors.

- Approach c also has the benefit that extension would be available to all inband allocations. Rounding could, however, complicate the FDSS filter design, since it would be equivalent to introducing a new extension factor.

In general, it can also be observed that this issue is particularly relevant for extension factor 0.125 because it does not have integer valued total allocations.

RAN1 should discuss pros and cons of the possible solutions to find the best approach to solve the rounding issue to yield only integer numbers of PRB allocations for the excess band, i.e., spectrum extension.

Proposal 7: Study solutions to yield only integer numbers of PRB allocations for the excess band, i.e., spectrum extension.

Transmit power control:

UE transmit power for PUSCH is determined by several factors, one of which being the total number of allocated RBs. The equation which provides such transmission power is based on the implicit assumption of equal average power density per RE, which is the typical PUSCH scenario.

However, and as discussed in previous meetings by few companies, the PSD of the REs used for FDSS-SE would be different from its plain DFT-s-OFDM counterpart, given the combined effect of the spectrum shaping filter coefficients and the spectrum extension factor (i.e., the amount of REs in the extension). Indeed, DFT-s-OFDM can be seen as an FDSS waveform where a rectangular FDSS filter is applied over the allocated BW. In other words, the power distribution over different REs is not uniform on average when FDSS with/without SE is applied, and regardless of whether the filter coefficients are normalized to ensure unit power of the filter or not.

Observation 5: The power distribution over REs is non-uniform after FDSS with/without SE.

As a result of the above situation, the relationship between throughput and transmission power, which underlies the current NR UL power control equation, may slightly change when moving from DFT-s-OFDM to FDSS-SE. This could have implications for the accuracy of the UL throughput estimation for a given transmission power.

As a matter of fact, this aspect may have a negligible impact on the actual performance of FDSS-SE, and has been neglected so far in the simulation campaigns carried out by RAN1 in Rel-18. However, and remarkably, link performance results obtained through simulations, and combined with transmitter performance, are sufficient to corroborate the appropriateness of the adoption of at least FDSS-SE as PAR/MPR reducing solution in Rel-18. This shows that optimization to the power control framework is not necessary to harness the benefits of the adoption of FDSS-SE, while it could indeed yield the optimal benefits when considered. Justifying corresponding spec impact could also be non-trivial and could require further RAN1 discussions.

Observation 6: Optimization to the power control framework may yield some benefits when considered, however it may not be necessary, or its spec impact unjustified, to harness the benefits of the adoption of FDSS-SE.

Given the above, and in case FDSS-SE is supported in Rel-18, it may be interesting to discuss whether and how to enhance the power control framework to account for the power density peculiarities of FDSS-SE and further improve its performance, if time allows it.

Several directions could be considered in this sense. Simple examples are:

- Based on NW operations: considering which receiver is being used and instruct the UE to adapt the transmission power accordingly, i.e., depending on whether receiver makes use of the data received over the spectrum extension or not.

- Based on UE operations: report details about the filter power distribution for a given FDSS-SE configuration, e.g., through PHR report or any other suitable existing UL message, so that NW can adjust the power control. Details related to events that may trigger such reports or their periodicity would have to be discussed, depending on the desired target.

Proposal 8: In case FDSS-SE is supported in Rel-18, RAN1 to discuss whether and how to enhance the power control framework to account for the power density peculiarities of FDSS-SE and further improve its performance, if time allows it.

Observation 7: In case FDSS- SE is supported in Rel-18 and RAN1 decide to further investigate enhancements to the power control framework, optimizations based on either NW’s or UE’s operations (or both) can be considered.

## DMRS sequence for FDSS with spectrum extension

It has been agreed in RAN1 #104bis-e that “Both DMRS and data symbols undergo spectrum shaping”.

Because spectrum shaping is applied similarly for both data and DMRS, it is important to study whether the peak to average power ratio (PAPR) and/or cubic metric (CM) is at least similar to that of data, and how the sequence impacts the channel estimation performance, so as to avoid DMRS limiting the coverage and/or the link performance. Furthermore, it is important to study whether the potential PAPR/CM differences between DMRS and data are limiting the actual OBO performance of the UE.

Current specification supports two sequence types, Type 1 and Type 2 for DMRS. Type 1 is a cyclically extended ZC sequence when the sequence length is larger than or equal to 36. Type 2 is pi/2 BPSK sequence (in time-domain) when the sequence length is larger than or equal to 30 and is used when pi/2 BPSK modulation is used for PUSCH. For shorter sequence lengths, Type 1 and Type 2 sequences are defined separately for each sequence lengths in Sections 5.2.2.2 and 5.2.3.2 of TS38.211.

In RAN1#112-bis-e, the following agreement was made:

In this section we study the PAPR, CM, OBO and channel estimation performance of different DMRS sequence options.

### DMRS performance for sequence lengths larger than or equal to 30

In RAN1#112, the following agreements were made to evaluate the potential DMRS options for sequence lengths larger than or equal to 30.

As per agreement, DMRSs transmitted by the UE will be shaped with the same filter as that used for PUSCH, while covering the whole allocation (i.e., also the extension band) such that the receiver may decide to also utilize the excess band signal for reception. We observe that if DMRS generation for PUSCH using Type 1 sequences is carried out as for data (i.e., symmetric extension according to 1 or using approach B as defined above) a considerable increase of PAPR and CM occurs. The reason behind such CM increase is that the current Rel-15 DMRS sequences involve cyclic extension already when the used Zadoff-Chu sequence is shorter than the number of allocated REs (hence, symmetric extension creates an additional extension on top, which has negative impact to PAPR/CM).

Figure 1. Block-diagram of DFT-s-OFDM transmitter with FDSS and spectral extension.

PAPR/CM could be reduced in several ways. For instance, one possibility could be to reuse for the spectrum extension the same logic as used for extending the Zadoff-Chu sequences cyclically within the allocated spectrum in Rel-15/16. In other words, the DMRS samples used in the spectrum extension would not be obtained by copying the content of some inband PRB(s), but rather by using the cyclic extension defined in Rel-15/16 on both sides of the sequence, i.e., the symmetric extension would be at RE level and not at PRB level in this case. Table 2 shows three examples of DMRS sample-to-RE mapping:

- No spectrum extensions, only inband sequence

- Symmetric extension of DMRS samples (per-PRB logic, listed Approach A.1.1 below)

- Symmetric extension of DMRS samples following Rel-15/16 (per-RE logic, listed Approach A.1.2 below).

Table 2. Different DMRS options

| - No spectrum extensions | - Symmetric extension of DMRS samples (per-PRB logic) |
| --- | --- |
| - Symmetric extension of DMRS samples following Rel-15/16 (per-RE logic) |  |

For the performance comparison, the following approaches are studied in this section:

A.1.1 Type 1 DMRS with FDSS and with symmetric spectrum extension using per-PRB logic

A.1.2 Type 1 DMRS with FDSS and with cyclic shift and symmetric extension, i.e., per-RE logic

A.1.3 Type 2 DMRS with FDSS and with symmetric spectrum extension

B.1.1 Type 1 DMRS with FDSS and without spectrum extension

B.1.2 Type 2 DMRS with FDSS and without spectrum extension

CM and PAPR performance

The performance of the five different approaches is illustrated in 2 to 5, which show PAPR and CM results for different parameters, using 3-tap filter [0.28,1,0.28]. It is observed that in this case type 1 DMRS without spectrum extension just using total allocation (approach B.1.1) both PAPR and CM are almost always worse than that of data, and if (approach A.1.1) is used the PAPR and CM is further degraded. However, if approach A.1.2 is used (per-RE logic), which is labelled as “modified” in the figures, a substantially lower PAPR and CM after the frequency domain filtering is displayed, being always on par with or better than data. This is expected since the Rel-15/16 logic does not depend on spectrum allocation size of filtering but rather on the properties of the DMRS sequence itself, which are fully respected by this approach. On the other hand, if type 2 sequences are used, further PAPR or CM reduction can be achieved, and whether extension is applied or not (approach A.1.3 or B.1.2) the PAPR/CM performance is always better than that of data. Figures 6 and 7 compare the performance also for 2-tap filter from which similar observations can be made.

Observation 8: For sequences longer than 24, for type 1 DMRS, generating legacy sequence using total allocation (i.e., without extension) results in worse PAPR/CM than that of data.

Observation 9: For sequences longer than 24, for type 1 DMRS, symmetric extension, using a per-PRB logic (i.e., extended similarly as data) results in considerable increase of PAPR and CM.

Observation 10: For sequences longer than 24, for type 1 DMRS, symmetric extension, using a per-RE logic results in considerable enhancement of PAPR and CM, being on par or better than that of data.

Observation 11: For sequences longer than 24, Type 2 DMRS either with or without symmetric spectrum extension results in considerable enhancement of PAPR and CM, being on par or better than that of data.

Receiver performance

Switching the focus to the receiver performance, the PAPR and CM reduction brought by type 2 sequences comes at the cost of a small performance degradation. Indeed, such sequences are not flat in frequency domain when long sequences are used, thus some channel estimation degradation compared to that of type 1 sequences may occur. This is evident when looking at Figures 8 and 9, where BLER performance of DMRS type 1 and DMRS type 2 is compared for FR1 Urban scenario (with other parameters following the assumptions used earlier in this work item), when different filters are used. Results show about 0.1-0.3dB receiver performance loss for type 2 compared to type 1, when Tx spectrum shaping filters are used as per FDSS-SE structure. However, the loss is even lower when compared with the performance degradation that can be observed for the QPSK baseline (Rel-17 PUSCH). Overall, we think such difference is marginal and should not be considered as detrimental for the net gain (as per RAN4’s evaluations), nor particularly relevant for practical purposes.

Observation 12: For sequences longer than 24, receiver performance loss for type 2 sequences compared to type 1 is about 0.1-0.3dB.

OBO performance

So far, we highlighted significant differences in PAPR/CM performance between different DMRS sequences. At the same time, to assess the full extent of the possible impact that such PAPR/CM difference can have on the transmitter performance a further study on OBO is needed. In this context, to ensure that the expected transmitter performance is observed in practice, OBO of FDSS-SE w/ or w/o DMRS should be identical or very similar. To this end, we studied the OBO of Type 1 DMRS sequence with total allocation (Approach B.1.1), which is what would be used with FDSS- SE according to the existing agreements, and which displays clear PAPR/CM loss compared to data. As can be seen from Figures 10, 11, 12 and 13, only marginal differences can be observed between DATA and DATA+DMRS cases, which can be considered negligible in the context of a simulation. This shows that although configuring Type 1 DMRS sequence with total allocation for FDSS-SE yields PAPR/CM increase for DMRS, this does not translate into OBO degradation. Therefore, the following can be observed:

- OBO for FDSS-SE is more than enough to compensate for link performance loss and enjoy benefits for both small and large PRB allocation size.

- No OBO degradation is observed if legacy DMRS sequences are mapped over the entire allocation of FDSS-SE.

  - This confirms that current agreements in RAN1 are sufficient at least from the point of view of supported DMRS sequences to enjoy the best performance of FDSS-SE.

Observation 13: Type1 DMRS with total allocation is not limiting the OBO performance even though there is PAPR/CM loss compared to data.

Based on the PAPR, CM, receiver and OBO results, we make the following proposal:

Proposal 9: For sequences longer than 24, For DMRS transmission when FDSS-SE is configured consider supporting type 1 and/or type 2 sequences without extension, i.e., using legacy sequences where the sequence length is determined by the total allocation.

Figure 3. PAPR of different DMRS approaches w.r.t. QPSK data for 3-tap filter [0.28 1 0.28] and 40 PRBs.

Figure 2. PAPR of different DMRS approaches w.r.t. QPSK data for 3-tap filter [0.28 1 0.28] and 8 PRBs.

Figure 4. CM of different DMRS approaches w.r.t. QPSK data for 3-tap filter [0.28 1 0.28] and 8 PRBs.

Figure 5. CM of different DMRS approaches w.r.t. QPSK data for 3-tap filter [0.28 1 0.28] and 40 PRBs.

Figure 7. PAPR of different DMRS approaches w.r.t. QPSK data for 2-tap filter [1 0.28] and 40 PRBs.

Figure 6. CM of different DMRS approaches w.r.t. QPSK data for 2-tap filter [1 0.28] and 40 PRBs.

Figure 7. PAPR of different DMRS approaches w.r.t. QPSK data for 2-tap filter [1 0.28] and 40 PRBs.

Figure 6. CM of different DMRS approaches w.r.t. QPSK data for 2-tap filter [1 0.28] and 40 PRBs.

Figure 8. SNR corresponding to 10% BLER for 8PRBs [FR1 100MHz CBW – Urban 4 GHz]

Figure 9. SNR corresponding to 10% BLER for 40PRBs [FR1 100MHz CBW – Urban 4 GHz]

Figure 10. OBO comparison between DATA and DATA+DMRS for 8 PRBs [FR1 20 MHz CBW – Rural 700 MHz]

Figure 11. OBO comparison between DATA and DATA+DMRS for 32 PRBs [FR1 20 MHz CBW – Rural 700 MHz]

Figure 12. OBO comparison between DATA and DATA+DMRS for 8 PRBs [FR1 100 MHz CBW – Urban 4 GHz]

Figure 13. OBO comparison between DATA and DATA+DMRS for 32 PRBs [FR1 100 MHz CBW – Urban 4 GHz]

### DMRS performance for sequence lengths below 30

For sequence lengths below 30 (before the extension), the following agreement has been made in RAN1#112 to study different DMRS sequence options:

For short sequences, we investigate the performance of the following approaches:

A.1.1 Type 1 DMRS with FDSS and symmetric spectrum extension

A.1.2 DFT transformed Type 1 DMRS with FDSS and symmetric spectrum extension

A.1.3 Type 2 DMRS with FDSS and symmetric spectrum extension

B.1.1 Type 1 DMRS with FDSS and without spectrum extension

B.1.2 Type 2 DMRS with FDSS and without spectrum extension

CM and PAPR performance

Figures 14 and 15 below illustrate PAPR and CM for the case of 6 PRBs total allocation. Type 1 sequences without extension here result in length 36 sequence, i.e., the sequences are defined in Section 5.2.2.1 of TS38.211, while type 1 sequences with extension are originally shorter, and thus taken from Section 5.2.2.2. Similarly for type 2 sequences, the sequences are taken either from Section 5.2.3.1. or 5.2.3.2 of TS38.211 for the case without extension and with extension, respectively.

It is observed that Type 1 sequences result in worse performance compared to data (either approach A.1.1 or B.1.1), while type 2 sequences achieve clearly better PAPR and CM either with or without spectrum extension (approaches A.1.3 or B.1.2). DFT transformed type 1 sequence (A.1.2) has similar performance to data in terms of PAPR/CM. Similar to the long sequence case, type 2 sequences achieve the lowest PAPR/CM for 2-tap filters as shown in Figures 16 and 17.

Observation 14: For DMRS sequences when the sequence length is shorter than 30 before the extension, Type 2 sequences give clearly lower PAPR and CM either with or without symmetric spectrum extension.

Receiver performance

In Figure 18 we further compare the BLER performance of DMRS types with different filters. We note that performance difference observed between type 1 and type 2 is smaller than with longer sequences. Similar performance difference applies also to DFT transformed type 1; receiver performance loss compared to type 1 is small, and surely marginal for practical purposes.

Observation 15: For sequences shorter than 30, receiver performance loss for type 2 sequences compared to type 1 is negligible (<0.04dB).

Figure 14. PAPR of different DMRS approaches w.r.t. QPSK data for 3-tap filter [0.28 1 0.28] and 6 PRBs.

Figure 15. CM of different DMRS approaches w.r.t. QPSK data for 3-tap filter [0.28 1 0.28] and 6 PRBs.

Figure 16. PAPR of different DMRS approaches w.r.t. QPSK data for 2-tap filter [1 0.28] and 6 PRBs.

Figure 17. CM of different DMRS approaches w.r.t. QPSK data for 2-tap filter [1 0.28] and 6 PRBs.

Figure 18. SNR corresponding to 10% BLER for 6PRBs [FR1 100MHz CBW – Urban]

# 4. 	Conclusion

In this paper we have discussed the scope of power domain enhancements, potential RAN1 specification impacts and provided CM/PAPR and receiver performance results of DMRS used for FDSS-SE. Based on the discussion and results, we make the following observations and proposals:

Observation 1: Based on the progress in both RAN1 and RAN4, the simulation parameters are well aligned between RAN1 and RAN4

Observation 2: RAN4 does not need any further input from RAN1 for selecting the Rel-18 MPR/PAR reduction solution(s)

Observation 3: Power domain enhancement does not require definition of additional DFT size options

Observation 4: Meaning of “extension size” in the agreement from RAN1 #110bis-e shown above should be clarified.

Observation 5: The power distribution over REs is non-uniform after FDSS with/without SE.

Observation 6: Optimization to the power control framework may yield some benefits when considered, however it may not be necessary, or its spec impact unjustified, to harness the benefits of the adoption of FDSS-SE.

Observation 7: In case FDSS- SE is supported in Rel-18 and RAN1 decide to further investigate enhancements to the power control framework, optimizations based on either NW’s or UE’s operations (or both) can be considered.

Observation 8: For sequences longer than 24, for type 1 DMRS, generating legacy sequence using total allocation (i.e., without extension) results in worse PAPR/CM than that of data.

Observation 9: For sequences longer than 24, for type 1 DMRS, symmetric extension, using a per-PRB logic (i.e., extended similarly as data) results in considerable increase of PAPR and CM.

Observation 10: For sequences longer than 24, for type 1 DMRS, symmetric extension, using a per-RE logic results in considerable enhancement of PAPR and CM, being on par or better than that of data.

Observation 11: For sequences longer than 24, Type 2 DMRS either with or without symmetric spectrum extension results in considerable enhancement of PAPR and CM, being on par or better than that of data.

Observation 12: For sequences longer than 24, receiver performance loss for type 2 sequences compared to type 1 is about 0.1-0.3dB.

Observation 13: Type1 DMRS with total allocation is not limiting the OBO performance even though there is PAPR/CM loss compared to data.

Observation 14: For DMRS sequences when the sequence length is shorter than 30 before the extension, Type 2 sequences give clearly lower PAPR and CM either with or without symmetric spectrum extension.

Observation 15: For sequences shorter than 30, receiver performance loss for type 2 sequences compared to type 1 is negligible (<0.04dB).

Proposal 1: PHR can be configured to contain the current PC that is used by the UE per serving cell as well as the currently used CA PC.

- Aperiodic reporting triggered by PC change and limited by PHR prohibiting timer is supported

Proposal 2: P-MPR and proactive reporting enhancements are deprioritized for Rel-18.

Proposal 3: No new additional DFT size options to be introduced by RAN1 to support Rel-18 power domain enhancements.

Proposal 4: Confirm RAN4 definition of extension factor (a) as excess band size / total allocation size.

Proposal 5: Support at least extension factor (a) 0.25. Other extension factors are FFS.

Proposal 6: First preference is to use existing FDRA indicator to signal inband allocation in the context of FDSS-SE. Second preference is to use the indicator to signal the total PRB allocation for FDSS-SE.

Proposal 7: Study solutions to yield only integer numbers (or even integer numbers) of PRB allocations for the excess band, i.e., spectrum extension.

Proposal 8: In case FDSS-SE is supported in Rel-18, RAN1 to discuss whether and how to enhance the power control framework to account for the power density peculiarities of FDSS-SE and further improve its performance, if time allows it.

Proposal 9: For sequences longer than 24, For DMRS transmission when FDSS-SE is configured consider supporting type 1 and/or type 2 sequences without extension, i.e., using legacy sequences where the sequence length is determined by the total allocation.

# References

- RP-221858, "New WI: Further NR coverage enhancements", China Telecom

- R4-2303560, “WF on coverage enhancement part 1”, Huawei, HiSilicon

- R4-2303701, “Reply LS on enhancements to realize increasing UE power high limit for CA and DC”

- RP-221302, “New WID: Increasing UE power high limit for CA and DC”, China Telecom, Qualcomm

- TR 38.830, “Study on NR coverage enhancements”, 3GPP

- R4-2307124, “UL duty cycle scheme improvement”, Nokia, Nokia Shanghai Bell

- R4-222084, “LS on RF simulation parameters to be used in RAN4 for power domain enhancements”

- R1-2212917, “LS to RAN4 for further information on RAN1 assumptions for LLS performance evaluation of MPR/PAR reduction solutions”

- R1-2302081, “LS to RAN4 for results of the LLS performance evaluation of MPR/PAR reduction solutions”

- R4-2308105, “Net gain analysis and next steps for MPR/PAR -objective”, Nokia, Nokia Shanghai Bell