3GPP TSG RAN WG1 #113								    		R1- 2305153

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:	9.4.1.2

Source:	InterDigital, Inc.

Title:	SL U physical layer design framework

Document for:	Discussion and Decision

# Introduction

In the last RAN1 meetings, different aspects of SL U physical layer design framework were discussed, the agreements are capture in [1]-[6] and listed in Appendix. In this contribution, we further discuss various aspects of PHY channel design framework for SL U operation. Specifically, we discuss the topics related to PSCCH/PSSCH, PSFCH and HARQ feedback, and S-SSB and synchronization.

# Discussion

PSCCH/PSSCH:

In the last RAN1 #112b meeting, we have the following agreements regarding two starting symbols for PSCCH/PSSCH transmission:

Agreement Regarding Tx UE behavior, at least when it initiates a COT: - For the 1st slot of a COT, the Tx UE chooses the earliest starting symbol for PSCCH/PSSCH transmission after clearing LBT. - Note: in the same slot, Tx UE can use the 2nd starting symbol only if LBT fails at the 1st starting symbol - FFS: whether/how to support that for the remaining slots of a COT, the Tx UE only chooses the 1st starting symbol for PSCCH/PSSCH transmission. - FFS applicable scenarios - e.g., at least for MCSt with no greater than 16us gap - e.g., at least for transmission with no greater than 16us gap from the previous transmission by any UE - FFS: Rx UE behavior FFS: COT sharing case

One remaining issue is whether to support the Tx UE choosing only the first starting symbol for PSCCH/PSSCH transmission for the remaining slots of a COT. In our view, it is desirable that the UE performs slot-based PSCCH/PSSCH transmission from the first starting symbol. It is because slot-based transmission, which requires the UE to transmit only one SCI in the slot, is more spectrum efficient than sub-slot-based transmission, which requires the UE to transmit multiple SCI in the slot. Therefore, if the UE acquires the COT from the first starting symbol, it should continue using slot-based transmission starting from the first starting symbol. Alternatively, if the UE acquires the COT from the second starting symbol, the UE can transmit sub-slot-based transmission for the first transmission. The UE then should switch to slot-based transmission, in which each transmission starts from the first starting symbol, for the remaining slots of the COT.

Proposal 1: Support the Tx UE to choose only the first starting symbol for PSCCH/PSSCH transmission for the remaining slots of a COT.

Regarding COT sharing, it is desirable for a responding UE to perform slot-based transmission and share the COT from the first starting symbol of a slot within the COT. It is because it can help increase the spectrum efficiency. In addition, it also can help reduce the SCI decoding of Rx UEs as the Rx UE can skip decoding SCI in the second starting symbol (e.g., switching from sub-slot-based SCI decoding to slot-based SCI decoding) if it detects another UE transmitting in a COT regardless of whether the transmission is from the COT initiator or responder.

Proposal 2: Support the responding UE to only perform slot-based transmission and shares the COT of the COT initiator UE from the first starting symbol.

We also have the following agreements in RAN1 #110b meeting regarding the number of AGC symbols for Tx/Rx UEs in the slots with two starting symbols:

Agreement For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission: - Regarding Tx UE behaviour: - If PSCCH/PSSCH transmission starts from 1st starting symbol, down-select one of the followings - Option 1: The PSCCH/PSSCH transmission has 2 symbols for AGC purpose - Option 2: The PSCCH/PSSCH transmission has only 1 symbol for AGC purpose - Option 3: The PSCCH/PSSCH transmission has 1 or 2 symbol(s) for AGC purpose depending on conditions, FFS details - If PSCCH/PSSCH transmission starts from 2nd starting symbol, the PSCCH/PSSCH transmission has only 1 symbol for AGC purpose - Regarding Rx UE behaviour, down-select one of the followings: - Option A: The Rx UE always monitors two AGC symbols in such slot - Option B: The Rx UE monitors two AGC symbols in such slot by default, but could drop monitoring the 2nd AGC symbol at least if it detects a PSCCH/PSSCH transmission starting from the 1st starting symbol. - FFS details - Option C: The Rx UE monitors two AGC symbols in such slot by default, but it is up to UE implementation whether to drop monitoring the 2nd AGC symbol. - Option D: It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot.

If the UE transmits PSCCH/PSSCH from the first starting symbol, we think that the second ACG symbol is necessary if there is another UE transmitting PSCCH/PSSCH at the second starting symbol. In this case the Rx UE converged ACG at the first starting symbol is different from the one at the second starting symbol. Therefore, the Rx UE needs to reconverge AGC again at the second starting symbol in the slot to receive PSCCH/PSSCH properly. However, if there is no other UE transmitting PSCCH/PSSCH at the second starting symbol in the slot, no ACG reconvergence is required. The Tx UE may be aware of whether there is a potential transmission in the middle of the slot. Therefore, it should be able to decide whether to have 1 or 2 symbol(s) for AGC. Such decision also needs to inform the Rx UE (e.g., in SCI) for PSCCH/PSSCH decoding purpose.

The Tx UE may be aware of whether there is a potential transmission in the middle slot (e.g., based on whether there is a transmission from another UE occurs at the second starting symbol). If no new transmission at the second starting symbol of slot is expected, the Tx UE only transmits one symbol for AGC purpose. Otherwise, if PSCCH/PSSCH transmission from another UE in the resource pool at the second starting symbols is expected, the UE need to transmit an additional symbol for AGC purpose.

Proposal 3: For Tx UE behavior for a slot with 2 candidate starting symbols, if PSCCH/PSSCH transmission starts from the first starting symbol:

- The PSCCH/PSSCH transmission has 1 symbol for AGC purpose if no transmission is expected at the second starting symbol in the resource pool. Otherwise, 2 symbols are used for AGC purpose.

It is expected that when a first UE acquires the channel and performs transmission in the COT, it will switch to slot-based transmission after the first transmission of the COT. If a PSCCH/PSSCH transmission at the second starting symbol is not allowed to share the COT, it is not expected to have another UE transmitting at the second starting symbol in the RB set. Otherwise, such new transmission can collide with the first UE’s transmission.  In this case, the Rx UE can drop monitoring the second ACG symbol.

Proposal 4: For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission, regarding Rx UE behavior, support Option B, in which the Rx UE monitors two AGC symbols in such slot by default but could drop monitoring the 2nd AGC symbol at least if it detects a PSCCH/PSSCH transmission starting from the 1st starting symbol.

In RAN1 #110b meeting, it was agreed to place PSCCH in the lowest subchannel of the corresponding PSSCH. One remaining issue is the time-domain location of PSCCH. R16/17 NR SL supports slot based PSCCH/PSSCH transmission, in which PSCCH is located at the first X symbols (e.g., X = 2 or 3 based on the resource pool (pre-)configuration). Such pre-defined PSCCH location within a PSSCH/PSCCH transmission can alleviate the UE SL SCI blind decoding processing. SL U supports two starting symbols, therefore, a pre-defined fixed PSCCH location is desired, and the resource pool configuration can e.g., (pre-)configure a set of symbols (e.g., the first 2 or 3 symbols after the starting symbol) in the time domain for PSCCH.

Proposal 5: PSCCH is mapped to the first 2 or 3 symbols after the starting symbol of a PSCCH/PSSCH.

Agreement Regarding frequency domain resource indication for interlace RB-based PSSCH transmission, support the followings: - Option A: Support that for one PSSCH transmission, the used interlace index(s) in different used RB sets are always the same - Option 1: Support explicitly indicating the used sub-channel index(s) and RB set index(s) - Frequency domain resource of PSSCH transmission is determined by an intersection of the resource blocks of the indicated sub-channel(s) and the union of the indicated set of RB sets and intra-cell guard bands between the indicated RB sets, if any - For a TB, the initial transmission and reservation of the resource(s) for retransmission(s) use the same number of sub-channel(s) and same number of RB set(s) - FFS: whether additionally support different number of RB set(s) in such case while keeping total number of sub-channels unchanged between initial transmission and retransmission(s) for a TB - Use X bits for indicating sub-channel index(s), and use Y bits for indicating contiguous RB set index(s) - R16 NR SL FRIV is reused as baseline - FFS details, e.g., signaling design, bit size, whether to consider bitmap design, whether/how the used interlace(s) can be non-contiguous, etc. - FFS others - E.g., considering one PSSCH transmission may occupy one or multiple RB sets, whether or not to re-define single-slot candidate resource, and update resource selection and/or signaling from MAC to PHY, etc.

In R16/17 SL, one single-slot candidate resource is defined as set of L consecutive subchannel in one slot, which can be used for PSCCH/PSSCH transmission. For multi-channel case, it was agreed that the set of interlace index(s) used for PSCCH/PSSCH in different RB sets are always the same. It also means that the set of subchannels used for PSCCH/PSSCH in different RB sets are the same. A single-slot candidate resource is defined for selecting resource to transmit PSCCH/PSSCH. Therefore, if PSCCH/PSSCH spans over multiple RB sets, a single-slot candidate resource should be redefined. Specifically, a single-slot candidate resource should be defined as a set of one or multiple contiguous sub-channels in one or multiple contiguous RB sets.

Proposal 6: Support a single-slot candidate resource for PSCCH/PSSCH transmission to be defined as a set of one or multiple consecutive subchannels in one or multiple consecutive RB sets.

One remaining issue for PSCCH/PSSCH spanning over multiple RB sets is whether to support PSCCH to be located in every RB set. In our view, it is desirable for SL U to support UEs with different bandwidths to use the same resource pool to communicate with each other. Therefore, Option 2, in which PSCCH is located in every RB set, should be supported. Transmitting PSCCH in every RB set is essential for sensing procedure. Otherwise, the UE performing sensing in an RB set different from the lowest RB set may not be able to properly decode the resource reservation information in its RB set.

Proposal 7: For multiple RB set transmission, support PSCCH to be located at every RB set of the corresponding PSSCH.

Regarding transmission of the UE within a COT, in our view, the UE can use the COT to transmit one or multiple TBs. Specifically, if the UE has one TB to transmit in the COT, the UE can use the COT to perform initial transmission and blind retransmissions of the TB. The blind retransmissions of the TB can be used to maintain the COT. The UE can also request HARQ feedback for such blind retransmission of the TB. If the UE has multiple TBs to transmit, the COT also can be used for transmission of multiple TBs, which may be motivated to reduce the LBT overhead since the UE only needs to perform one LBT to transmit multiple TBs.

Proposal 8: Support re-transmission and/or transmission of new TB in a COT.

In R16/17 SL, TRIV/FRIV is used to indicate/reserve future resources for transmission of the UE. Specifically, TRIV/FRIV is used to reserve at most three single-slot resources in the future for the UE. The current TRIV/FRIV is suitable for single-slot resource reservation only. It is expected that for each channel access time, the UE can obtain a COT consisting of multiple consecutive slots for transmissions of one or multiple TB. COT reservation is beneficial for interference avoidance since other UE can avoid selecting the reserved COT of the UE for LBT and transmission. Therefore, COT reservation should be supported in SL U. Moreover, TRIV/FRIV should be adapted to indicate/reserve a new COT.

Proposal 9: Support COT reservation of a COT consisting of multiple consecutive slots.

Proposal 10: TRIV/FRIV is used to indicate/reserve a new COT.

In R16/17 SL, network scheduling resource allocation (e.g., Mode 1) is supported, which allows the gNB to schedule both dynamic and configured grants to a UE. Dynamic grant allows the gNB to schedule sidelink resource flexibly to deal with a dynamic change of the sidelink resource usage. In SL U, it is expected that dynamic grant can be supported to help the gNB coordinating sidelink resource usage among different UEs in the system. New DCI information is needed to support the indication of LBT parameters since the network may need to request the UE to use specific LBT parameters including LBT type and channel access priority class based on the scheduling decision from the gNB.

Due to the uncertainty of the availability of the unlicensed channel, the UE may fail LBT multiple times before acquiring COT. Therefore, to increase the channel access opportunity to the UE, the network can schedule multiple channel access occasions in time and/or frequency domain (e.g., a time window and/or a set of frequency resources to perform LBT) to the UE in one DCI.

Contiguous transmission across multiple slots is necessary to maintain the COT initialized by a UE. Therefore, new DCI information is needed to support multiple-slot scheduling which may include the indication of the number of slots and a possibly a set of interlaces in each slot.

Proposal 11: Consider a scheduling DCI indicating at least the following information:

  - LBT type and the channel access priority class

  - A set of interlaces and time window for the UE to access the channel

  - Indication of number of allocated slots for one or more TBs scheduling

PSFCH and HARQ feedback:

In the RAN1 #112b meeting, to address small PSFCH capability and satisfy the OCB requirement for PSFCH transmission in unlicensed spectrum, RAN1 have the following agreements:

Agreement Regarding PSFCH transmission with 15 kHz and 30 kHz SCS: - RAN1 down-select one of followings in RAN1#113: - Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s) - K3 is (pre-)configured, FFS value range - On the K3 dedicated PRB(s), multiple CS pairs can be used as in legacy NR SL PSFCH transmission - When a PRB of common interlace and a dedicated PRB locate within the same 1 MHz bandwidth, UE only transmits on the dedicated PRB - FFS: whether any impact on meeting OCB requirement - Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace - Alt 3-2b: each PSFCH transmission occupies K4 dedicated PRB(s) and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set - K2=2 - K4 is (pre-)configured, FFS value range - FFS: how to meet PSD limitation - FFS: whether to introduce any restrictions on the locations of K4 dedicated PRB(s) and/or K2 common PRBs, e.g., whether/how they are on the same interlace - R16 NR SL PSSCH-PSFCH mapping is reused as baseline, FFS details - Note: companies are encouraged to give more details and analyze the specification impact - E.g., whether PSSCH transmissions on non-overlapped resources are mapped to non-orthogonal PSFCH resources, i.e., whether PSFCH collision may happen and whether/how to address it, etc. - E.g., whether introducing more than 6 CS pairs is needed - E.g., for group cast option 2, what’s the maximum group size that can be supported - E.g., how to support “more than 1 PSFCH occasion(s) per PSCCH/PSSCH” - FFS: regardless of which Alt above is selected, whether or not to support PRB-level cyclic shift hopping as in NR-U to reduce PAPR - FFS: whether IBE issue exists and whether/how to address it - E.g., whether to introduce guardband PRB/RE between common PRB and dedicated PRB

In our view out of all alternatives, Alt.1-1b, in which each PSFCH transmission occupies one common interlace and some dedicated PRB(s) are the most suitable solution.

- Proposal 12: Regarding PSFCH transmission under 15 kHz and 30 kHz SCS, support Alt. 1-1b, in which each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s).

- In RAN1 #111 meeting, we also have the following agreement for PSFCH transmission under 60kHz SCS:

Agreement Regarding PSFCH transmission under 60 kHz SCS, further study the following alternatives: - Alt 1: Each PSFCH transmission occupies K dedicated PRB(s) and some common PRBs - FFS details - Alt 2: Each PSFCH transmission occupies some dedicated PRBs - FFS details

Similar to PSFCH transmission under 15kHz and 30 kHz SCS, PSFCH transmission under 60kHz SCS can occupies K dedicated PRB(s) and some common PRBs. Therefore Alt. 1 should be supported.

- Proposal 13: Regarding PSFCH transmission under 60kHz SCS, support Alt. 1, in which each PSFCH transmission occupies K dedicated PRB(s) and some common PRBs.

Similar to K3, which is (pre-)configured in the resource pool, the value of K can be (pre-)configured. Regarding the (pre-)configured values of K and K3, in R16/17 SL, only one PRB is used to indicate HARQ feedback bit; therefore, at least K and K3 = 1 should be (pre-)configured for SL U. However, due to PSD restriction in SLU may be multiple PRB(s) can be used to increase the reliability of PSFCH transmission.

- Proposal 14: The value of both K and K3 are (pre-)configured in the associated resource pools. At least K=1 and K3 = 1 are supported.

- To address the PSFCH transmission drop due to LBT failure, RAN1 has the following agreements in the last RAN1 #112 meeting:

Agreement Regarding more than 1 PSFCH occasion per PSCCH/PSSCH transmission, support the followings: - One PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s) via (pre-)configuration - FFS value range of N - FFS detailed design of such N associated candidate PSFCH occasion(s) - E.g., they are in different slots of the same RB set, or in different RB sets of the same slot, or combination thereof, etc. - E.g., whether PSSCH transmission and its related PSFCH occasion(s) are in the same RB set(s) - FFS: whether to support that COT initiating UE can dynamically indicate which subset of the (pre-)configured PSFCH occasions within its COT are available for PSFCH transmissions. - FFS: whether other associated candidate PSFCH occasion(s) within its COT are used for PSSCH transmissions, and applicable scenarios. - FFS: whether AGC issue and PSFCH/PSSCH collision issue exist, and whether/how to address them - FFS other details - E.g., how to meet the HARQ RTT restriction - E.g., UE behavior on transmitting PSFCH - E.g., how to avoid the risk of losing the COT if the COT is interrupted by periodic PSFCH occasions

One remaining issues is the channel access for the (pre-)configured PSFCH occasion(s). In our view, PSFCH can qualify for Short Control Signaling based transmission (SCSt) in SL U and such transmission does not require the UE to perform Type-1 LBT. Instead, similar to S-SSB, LBT type 2A should be performed by the UE to access the (pre-)configured PSFCH occasion(s).

Proposal 15: Support PSFCH resources to be (pre-)configured in a resource pool for SCSt.

It was agreed that one PSCCH/PSSCH can have N associated candidate PSFCH occasion(s). In our view, the occasion(s) of the PSFCH can be in different slots. For example, in a resource pool of one RB set, to support multiple PSFCH occasion(s), such PSFCH occasion(s) should be in different slots. Otherwise, if all PSFCH occasion(s) are in the same slot, failing LBT to transmit in one PSFCH occasion(s) results in failing LBT in all PSFCH occasion(s). Regarding whether to allow PSFCH in different RB sets. In our view, it is beneficial to support PSFCH in different RB sets. It is because such configuration may help reduce the HARQ feedback latency.

Proposal 16: Support the associated PSFCH occasion(s) of a PSCCH/PSSCH can be in different RB sets and/or different slots.

One remaining issue is whether to support dynamic PSFCH indication. When Tx UE acquires the COT, it is beneficial for Tx UE to share its COT with the Rx UE to transmit PSFCH. The Tx UE can indicate which associated PSFCH occasion(s) to transmit the PSFCH. Such PSFCH occasion(s) can be in the middle, or the end of the COT based on the QoS of the TB and the COT information (e.g., COT duration) of the Tx UE. Such information is known at the Tx UE and the Tx UE can also indicate the PSFCH to the Rx UE. Therefore, it is beneficial to support dynamic indication of PSFCH occasion.

Proposal 17: Support the COT initiating UE to dynamically indicate which subset of the (pre-)configured PSFCH occasions within its COT are available for PSFCH transmissions.

In order to maintain the COT, the gap between any two transmissions should be within 16us. As the duration of PSFCH is higher than 16us, if there is no transmission in one PSFCH occasion(s), the COT cannot be maintained. Therefore, the Tx UE should transmit PSSCH in the remaining PSFCH occasion(s), in which the UE may not expect to receive HARQ feedback.

Proposal 18: Support the Tx UE to transmit PSSCH in the remaining subset of PSFCH occasion(s), which were not indicated to Rx UE(s) to transmit HARQ feedback.

Each PSCCH/PSSCH has N associated PSFCH occasion(s) and the Rx UE should attempt LBT and transmit on all occasion until it successfully transmits in at least one PSFCH occasion. However, in sidelink transmission, each TB has an associated packet delay budget (PDB) and if the TB is not delivered within the PDB, such TB can be considered as a failure TB. It is possible that a subset of associated PSFCH occasion(s) for one PSCCH/PSSCH is beyond the PDB. Therefore, transmitting HARQ feedback in these PSFCH occasion(s) is not necessary. The Rx UE should ignore the PSFCH occasion(s) beyond the PDB. Since the Tx UE is aware of the PDB of the TB, it knows the HARQ latency bound for one PSCCH/PSSCH. Such HARQ latency bound should be indicated to the Rx UE (e.g., in SCI) to support the Rx UE in determining when to stop LBT to transmit PSFCH.

Proposal 19: Support the Tx UE to indicate the HARQ latency bound for one PSCCH/PSSCH.

Proposal 20: Support the Rx UE to transmit PSFCH within the indicated HARQ latency bound.

It was agreed in RAN1 #109-e meeting to support R16 SL PSFCH format 0, which is used to report one HARQ ACK/NACK bit for one PSCCH/PSSCH transmission in SL U. If only R16 SL PSFCH format 0 is supported, the UE needs to acquire the channel to transmit HARQ ACK/NACK feedback for every PSCCH/PSSCH requiring HARQ feedback. Moreover, for HARQ enabled TB, the Tx UE needs to wait for HARQ feedback from the Rx UE before performing additional retransmission of the TB. Such a procedure may result in excessive transmission delay since the Tx and Rx UEs need to perform LBT before each transmission. However, in SL U, when the Tx UE acquires the COT, it is beneficial to perform contiguous transmissions of one or multiple TBs to maintain the COT and the Rx UE should report multiple HARQ feedback bits associated with multiple transmissions.

Proposal 21: Support reporting multiple HARQ ACK/NACK bits in one transmission in SL U.

For HARQ feedback, a Rx UE may transmit PSFCH in the same COT as the associated PSCCH/PSSCH or in a new COT (e.g., at the beginning of a new COT) initialized by the Rx UE. PSFCH can be (pre-)configured at the middle or the end of the Tx UE’s COT if PSFCH is transmitted in the same COT of the PSCCH/PSSCH. For PSFCH transmission at the middle of the COT, the Rx UE can transmit PSFCH at the guard symbol as well to maintain the COT; otherwise, the 16us transmission gap cannot be guaranteed to maintain the COT. For PSFCH transmission in the same COT, sequence-based transmission can be used to feedback one or multiple HARQ ACK/NACK bits. However, for PSFCH transmission in a different COT (e.g., Rx UE’s COT), the Rx UE can feedback HARQ ACK/NACK of all HARQ processes from the Tx UEs. Payload-based HARQ ACK/NACK feedback is more suitable for high number of number of HARQ ACK/NACK bits.

Proposal 22: Support new PSFCH format to convey multiple HARQ ACK/NACK bits in one transmission in SL U.

In Mode 1 resource allocation, the gNB may indicate a PUCCH resource after the last scheduled sidelink resource for the UE to forward the SL HARQ feedback from the Rx UE. If Tx UE reports ACK, it does not expect to receive further sidelink resources for the TB. Otherwise, if the UE reports NACK, it expects the gNB to schedule additional sidelink resources for the TB. However, for sidelink unlicensed spectrum, the Tx or Rx UEs may fail to access the channel (e.g., LBT failure). The gNB should be aware of different cases in sidelink to schedule sidelink resources properly. For example, if the Tx UE fails to access the channel, the gNB can schedule a different RB set, and, if the Rx fails to access the channel, the gNB can schedule additional PUCCH resources to wait for the Rx UE’s feedback. The impact of not knowing the channel access outcome at the gNB is more severe with muti-slot scheduling. If the Tx UE cannot access the channel due to LBT failure, the gNB can use multiple slots for a different Tx UE.

Proposal 23: Support reporting the LBT result to the gNB in the Mode 1 resource allocation.

In SL U it is expected that, to maintain the COT, the UE should perform contiguous PSCCH/PSSCH transmissions for one or multiple TBs. It is possible that multiple HARQ-enabled PSCCH/PSSCH transmissions in a COT are intended for the same or different Rx UEs. While PSFCH transmissions in the same COT with the corresponding PSCCH/PSSCH transmission is beneficial to reduce the LBT uncertainty, it may require multiplexing of PSFCH transmission at the same PSFCH occasion. Therefore, multiplexing of PSFCH transmissions at a PSFCH occasion in a COT should be further studied. For example, the Rx UEs may transmit PSFCHs for different PSCCH/PSSCHs at the end of the COT when the Tx UE finishes its PSCCH/PSSCH transmissions. In this case, PSFCHs can be time and/or frequency multiplexing. In another example, the Rx UEs may transmit PSFCHs in the middle of the COT for the PSCCH/PSSCHs transmitted early in the COT to ensure the HARQ feedback latency. Accordingly, PSFCHs can be frequency multiplexing to reduce PSCCH/PSSCH transmission interruption time.

Proposal 24: Study PSFCH transmissions for multiple HARQ corresponding to PSCCH/PSSCH transmissions in one COT.

S-SSB and synchronization

For S-SSB and synchronization, in the last RAN1 #112b meeting, the following agreements were made:

Agreement When the SL-BWP contains multiple RB sets, study the followings: - When UE attempts to transmit S-SSB in a S-SSB occasion (e.g., R16/17 S-SSB occasion, R18 additional candidate S-SSB occasion) - Alt 1: UE may transmit S-SSB repetition in more than one RB set - FFS details, e.g., location of such S-SSB repetition(s) (e.g., (pre-)configured and/or pre-defined), whether/how to address potential power reduction and/or potential fluctuation of PSBCH-RSRP - FFS: the relationship with UE’s COT - FFS: the scenario that UE may or may not transmit S-SSB repetition in more than one RB set - Note: whether UE can transmit S-SSBs over non-contiguous RB sets is subject to RAN4’s reply, details can be found in RAN1’s LS to RAN4 in R1-2304218

When a UE initiates a COT on multiple RB sets and the COT duration includes S-SSB slot(s), the COT maybe lost if not all the RB sets are used to transmit S-SSB. For example, in case where the COT overlaps with Rel-16/17 S-SSB occasions, those S-SSB slot(s) cannot be used for sidelink data transmission as those occasions are not part of resource pool. Instead, supporting S-SSB transmission on multiple RB sets can help keeping the COT after S-SSB slots and avoiding starting LBT type 1 to transmit in the subsequent slots.

Proposal 25: S-SSB transmission on multiple RB sets can be used to maintain a COT.

The UE can use the acquired RB sets for a COT to transmit S-SSB on multiple RB sets. One of the RB sets can always be used for S-SSB transmission to avoid blind detection of the S-SSB transmission on multiple RB sets.

Proposal 26: S-SSB is always transmitted on a default RB set.

Others

SL RS:

It is expected that PSCCH/PSSCH uses interlace-based transmission, which is different from R16/17 using subchannel-based transmission. Therefore, the SL RS (e.g., PTRS, DMRS, CSI RS) resource configuration should be further studied. In our view, we can take R16 SL RS design as baseline and adopt the design to the interlace structure. Specifically, we can reuse the existing PTRS, DMRS, and CSI RS configurations in R16 SL and apply the configurations in the interlace structure of PSCCH/PSSCH.

Proposal 27: Study SL RS (i.e., DMRS, PTRS, CSI-RS) structure taking R16 SL RS as baseline.

CSI reporting:

For unicast communication in R16/17 SL, CSI reporting is supported, in which the Rx UE may report CQI and RI to the Tx UE. CSI reporting can be used by Tx UE to determine its transmission schemes and parameters. It is thus beneficial to adapt CSI reporting for SL U considering R16/17 SL CSI reporting as a starting point. In our view, similar to R16/17 SL, MAC CE can be reused to report RI and CQI in CSI reporting. Moreover, the latency bound can be determined by the Tx UE and conveyed to the Rx UE via PC5 RRC and the Rx UE is expected to perform CSI reporting within the indicated latency bound. However, the latency bond can be relaxed compared to R16/17 SL due to the uncertainty of LBT.

Proposal 28: Study the support of CSI reporting with R16 CSI reporting as a starting point.

Power control:

OLPC is supported in R16/17 SL. In the OLPC formula, DL pathloss can be configured if sidelink communication uses a carrier shared with Uu to reduce the interference to gNB in the uplink direction. However, in SL U, it is not expected that the gNB will operate in the unlicensed carrier configured to sidelink. Therefore, further study is needed to evaluate whether DL pathloss is necessary for SL U OLPC.

In addition, In R16/17 SL, SL pathloss can be used for unicast transmission. In our view SL pathloss also can be used for OLPC for unicast transmission in SL U since it is beneficial to control the transmission power in the unicast link.

Proposal 29: Study whether SL and/or DL pathloss can be used for power control.

# Conclusion

In this contribution, we have discussed and propose the following:

Proposal 1: Support the Tx UE to choose only the first starting symbol for PSCCH/PSSCH transmission for the remaining slots of a COT.

Proposal 2: Support the responding UE to only perform slot-based transmission and shares the COT of the COT initiator UE from the first starting symbol.

Proposal 3: For Tx UE behavior for a slot with 2 candidate starting symbols, if PSCCH/PSSCH transmission starts from the first starting symbol:

- The PSCCH/PSSCH transmission has 1 symbol for AGC purpose if no transmission is expected at the second starting symbol in the resource pool. Otherwise, 2 symbols are used for AGC purpose.

Proposal 4: For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission, regarding Rx UE behavior, support Option B, in which the Rx UE monitors two AGC symbols in such slot by default but could drop monitoring the 2nd AGC symbol at least if it detects a PSCCH/PSSCH transmission starting from the 1st starting symbol.

Proposal 5: PSCCH is mapped to the first 2 or 3 symbols after the starting symbol of a PSCCH/PSSCH.

Proposal 6: Support a single-slot candidate resource for PSCCH/PSSCH transmission to be defined as a set of one or multiple consecutive subchannels in one or multiple consecutive RB sets.

Proposal 7: For multiple RB set transmission, support PSCCH to be located at every RB set of the corresponding PSSCH.

Proposal 8: Support re-transmission and/or transmission of new TB in a COT.

Proposal 9: Support COT reservation of a COT consisting of multiple consecutive slots.

Proposal 10: TRIV/FRIV is used to indicate/reserve a new COT.

Proposal 11: Consider a scheduling DCI indicating at least the following information:

  - LBT type and the channel access priority class

  - A set of interlaces and time window for the UE to access the channel

  - Indication of number of allocated slots for one or more TBs scheduling

- Proposal 12: Regarding PSFCH transmission under 15 kHz and 30 kHz SCS, support Alt. 1-1b, in which each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s).

- Proposal 13: Regarding PSFCH transmission under 60kHz SCS, support Alt. 1, in which each PSFCH transmission occupies K dedicated PRB(s) and some common PRBs.

- Proposal 14: The value of both K and K3 are (pre-)configured in the associated resource pools. At least K=1 and K3 = 1 are supported.

Proposal 15: Support PSFCH resources to be (pre-)configured in a resource pool for SCSt.

Proposal 16: Support the associated PSFCH occasion(s) of a PSCCH/PSSCH can be in different RB sets and/or different slots.

Proposal 17: Support the COT initiating UE to dynamically indicate which subset of the (pre-)configured PSFCH occasions within its COT are available for PSFCH transmissions.

Proposal 18: Support the Tx UE to transmit PSSCH in the remaining subset of PSFCH occasion(s), which were not indicated to Rx UE(s) to transmit HARQ feedback.

Proposal 19: Support the Tx UE to indicate the HARQ latency bound for one PSCCH/PSSCH.

Proposal 20: Support the Rx UE to transmit PSFCH within the indicated HARQ latency bound.

Proposal 21: Support reporting multiple HARQ ACK/NACK bits in one transmission in SL U.

Proposal 22: Support new PSFCH format to convey multiple HARQ ACK/NACK bits in one transmission in SL U.

Proposal 23: Support reporting the LBT result to the gNB in the Mode 1 resource allocation.

Proposal 24: Study PSFCH transmissions for multiple HARQ corresponding to PSCCH/PSSCH transmissions in one COT.

Proposal 25: S-SSB transmission on multiple RB sets can be used to maintain a COT.

Proposal 26: S-SSB is always transmitted on a default RB set.

Proposal 27: Study SL RS (i.e., DMRS, PTRS, CSI-RS) structure taking R16 SL RS as baseline.

Proposal 28: Study the support of CSI reporting with R16 CSI reporting as a starting point.

Proposal 29: Study whether SL and/or DL pathloss can be used for power control.

# References

- RAN1 Chairman’s note, RAN1#112b, April 2023.

- RAN1 Chairman’s note, RAN1#112, Feb. 2023.

- RAN1 Chairman’s note, RAN1#111e, Nov. 2022.

- RAN1 Chairman’s note, RAN1#110b, October 2022.

- RAN1 Chairman’s note, RAN1#110, August 2022.

- RAN1 Chairman’s note, RAN1#109e, May 2022.

# Appendix

RAN1 #109 agreements:

Agreement

SL BWP, SL resource pool in R16/R17 NR SL and RB set in R16 NR-U are reused for SL-U as baseline

- Only one SL BWP is (pre-)configured within a carrier

- The SL BWP is (pre-)configured to include one or multiple SL resource pools

- At least support that one SL resource pool can be (pre-)configured to include integer number of RB sets

  - FFS: whether/how to support one SL resource pool can include sub-set of PRBs of one RB set

  - FFS: the applicable resource pool

  - FFS: the impact on sub-channel size and number of sub-channels in a resource pool if sub-channel is supported

- PRBs within intra-cell guard band of two adjacent RB sets belong to a resource pool if the resource pool includes the two adjacent RB sets

  - FFS details, e.g., how such PRBs are used, the applicable resource pool, etc.

- FFS: whether R16/R17 NR SL S-SSB slots and/or new S-SSB slots (if supported) are excluded from resource pool

- FFS: which slots belong to resource pool, e.g., how to set the value of bitmap, whether to consider SL-U/NR-U operating in the same carrier and whether TDD configuration are considered, etc.

- FFS: the impact of PSCCH/PSSCH mapping to frequency resources on resource pool configuration, on sub-channel definition if sub-channel is supported, etc.

Agreement

For PSCCH and PSSCH in SL-U:

- Both R16/R17 NR SL contiguous RB-based and R16 NR-U interlace RB-based transmissions are considered as starting point

  - RAN1 strives to have unified design for both contiguous RB-based and interlace RB-based transmissions

  - FFS: whether/how to address IBE (In Band Emission) impact

Agreement

For PSCCH and PSSCH in SL-U:

- For interlace RB-based transmission (if supported), at least the following candidates can be discussed:

  - Frequency domain resource allocation granularity is one sub-channel for PSSCH transmission

    - FFS: Other resource allocation granularity, e.g., RB-level

  - 1 sub-channel equals K interlaces if sub-channel is supported

    - FFS details

  - Other candidates are not precluded

  - FFS: mapping of PSCCH to frequency resources

  - FFS: resource indication in time/frequency domain, e.g., how to handle using one RB set or multiple RB sets, etc.

Agreement

For slot structure in SL-U:

- At least R16/R17 NR SL slot-based PSCCH/PSSCH transmission is supported

  - FFS: whether/how to support additional starting symbol(s) within a slot for the PSCCH/PSSCH transmission

Agreement

For PSFCH and SL-HARQ in SL-U:

- At least R16 NR SL PSFCH format 0 is supported

  - FFS whether to introduce new PSFCH format

- FFS: how to meet OCB and PSD requirement for PSFCH transmission, e.g., using interlaced RB transmission, whether/how to avoid too small PSFCH capacity, etc.

- FFS: the locations of PSFCH resources, e.g., (pre-)configured, dynamically indicated, etc.

- FFS: whether/how to address PSFCH transmission dropping due to LBT failure, e.g., whether to have multiple PSFCH occasions for a PSSCH and the related PSSCH-PSFCH mapping relationship, impact on SL HARQ-ACK reporting to the gNB for Mode 1, etc.

- FFS: whether/how to address PSFCH and related PSSCH in different COTs

Agreement

For S-SSB and synchronization in SL-U:

- FFS the time domain locations of S-SSB resources, e.g., whether/how to introduce more candidate occasions compared with R16/R17 NR SL design, etc.

- Down-selection at least one of the following solutions to meet OCB and PSD requirement for S-SSB transmission

  - Option 1: Using interlaced RB transmission

  - Option 2: S-SSB multiplexing with other SL transmissions in the same slot

  - Option 3: Repetition of S-PSS/S-SSS/PSBCH in frequency domain

  - Option 4: S-PSS/S-SSS/PSBCH with wider bandwidth

- FFS: whether to support 4 symbols S-SSB

  - Note: 4 symbols S-SSB can be considered with options 1/2/3/4 above

- FFS whether the temporary exemption of OCB requirement is applicable for S-SSB transmission

- FFS whether any changes to R16/R17 NR SL synchronization procedure

RAN1 #110 agreements:

Agreement

For PSCCH and PSSCH in SL-U:

- Both R16/R17 NR SL contiguous RB-based and interlace RB-based transmissions similar to R16 NR-U are supported

Agreement

For PSCCH and PSSCH in SL-U:

- For interlace RB-based transmission

  - Frequency domain resource allocation granularity is one sub-channel for PSSCH transmission

  - 1 sub-channel equals K interlace

    - FFS: whether K is fixed as 1 or (pre-)configured

  - Discuss whether one or both of the following alternatives are supported

    - Alt 1: 1 sub-channel is confined within 1 RB set

    - Alt 2: 1 sub-channel spans 1 or multiple RB set(s) belonging to a resource pool

Agreement

To meet OCB and PSD requirement for PSFCH transmission, at least RB-based interlace is supported at least for 15 kHz and 30 kHz SCS, FFS details.

Agreement

If RAN1 decides that LBT is performed for S-SSB transmission, in addition to the S-SSB occasions in R16/R17 NR SL design, support additional candidate S-SSB occasions

- FFS the number and locations of additional candidate S-SSB occasions

- FFS when a UE transmits S-SSB on such additional candidate S-SSB occasions, and the related Rx UE’s behavior

Agreement

Regarding PSFCH transmission, at least the followings alternatives can be further studied

- Alt 1: each PSFCH transmission occupies a common interlace and zero or one or more dedicated PRB(s)

- Alt 2: each PSFCH transmission occupies an interlace, and may or may not further apply code domain enhancement (e.g., OCC, PRB-level cyclic shifts)

- Alt 3: each PSFCH transmission occupies some dedicated PRBs and some common PRBs

- FFS details of above alternatives

Agreement

If RAN1 decides that LBT is performed for PSFCH transmission, for the time and frequency domain locations of PSFCH resources, at least the followings alternatives can be further studied

- Alt 1: PSFCH resources are (pre-)configured

- Alt 2: PSFCH resources are dynamically indicated

- Combination of above alternatives are not precluded

- FFS details of above alternatives

Agreement

For S-SSB and synchronization in SL-U:

- No changes on R16 NR SL S-PSS/S-SSS sequence generation

- Continue studying the 4 options from the previous agreement and whether/how temporary exemption of OCB requirement is applicable for S-SSB transmission, e.g., how to meet the minimum of 2 MHz requirement under 15 kHz SCS

Agreement

For PSCCH and PSSCH resource indication in time/frequency domain:

- For time domain: R16 NR SL TRIV is reused as baseline

- For frequency domain:

  - further study sub-channel indexing and resource indication

- FFS: whether any enhancement needed on R16 NR SL TRIV/FRIV if new feature is introduced in SL-U, e.g., multi-slot consecutive transmission

RAN1 #110b agreements:

Agreement

For interlace RB-based PSCCH/PSSCH transmission in SL-U:

- Regarding 1 sub-channel equals K interlace(s)

  - At least K=1 and K=2 is supported for 15 kHz SCS

  - At least K=1 is supported for 30 kHz SCS

  - FFS: details related to multiple RB sets

Working assumption:

Support maximum 2 candidate starting symbols within a slot for a PSCCH/PSSCH transmission.

- RAN1 strives to have unified design for PSCCH/PSSCH transmission from 1st or 2nd starting symbol

- The candidate starting symbol(s) are intended for AGC purpose

  - FFS: other potential uses of the candidate starting symbol(s)

- FFS other details, e.g., applicable scenarios (including SCS), position of 2nd starting symbol, TBS determination, PSCCH blind decoding complexity, processing time constraints, etc.

- FFS whether 2 candidate starting symbols is also supported for slots with PSFCH

Agreement

To meet OCB and PSD requirement for S-SSB transmission, down-select between the followings for 15 kHz and 30 kHz SCS:

- Option 1: Using interlaced RB transmission for S-PSS/S-SSS/PSBCH

- Option 3: Repetition of S-PSS/S-SSS/PSBCH in frequency domain

- FFS: whether/how the above options apply to all or subset of channel type of S-PSS/S-SSS/PSBCH

- Note: RAN1 further study the relationship between above options and temporary OCB exemption, and the discussion on temporary OCB exemption can continue even if option 1 or option 3 is supported

FFS: how to handle 60 kHz SCS (if needed, not limited to option 1 or option 3)

Agreement

Regarding frequency domain resource indication for interlace RB-based PSSCH transmission:

- When more than one RB set is used for transmissions, down-select one of the followings

  - Option A: Support that the used interlace index(s) in different RB sets are always the same

  - Option B: Support that the used interlace index(s) in different RB sets can be different

- FFS details

Agreement

Regarding frequency domain resource indication for interlace RB-based PSSCH transmission:

- Down-select one of the followings

  - Option 1: Support explicitly indicating the used sub-channel index(s) and RB set index(s)

  - Option 2: Support explicitly indicating at least the used sub-channel index(s)

    - At least RB set index(s) is not explicitly indicated

- FFS details

Agreement

For PSCCH and PSSCH in SL-U:

- PSCCH is transmitted within 1 sub-channel

- At least support Option 1 below

  - Option 1: PSCCH locates in the lowest sub-channel of lowest RB set of corresponding PSSCH

    - Note: the lowest sub-channel may not be entirely contained in the lowest RB set

- FFS whether/how to handle the case where UEs supporting different bandwidths can use the same resource pool to communicate with each other, e.g., whether/how to additionally support Option 2 below

  - Option 2: PSCCH locates in every RB set of corresponding PSSCH

- Note: the above options do not imply any restriction on the mapping of sub-channels to PRBs.

- FFS other details

Agreement

Regarding usage of PRBs within intra-cell guard band of two adjacent RB sets:

- Such PRBs can be used for PSSCH transmission if and only if a UE can transmit on the respective LBT channels after performing channel access procedure in multi-channel case and the UE uses both of these two RB sets for PSSCH transmission

  - FFS details, e.g., handling of potential unequal sub-channel size, for interlaced RB based transmission, whether the PRB(s) in the intra-cell guard band have the same interlace index(s) as the PRBs for PSSCH transmission in these two RB sets

- Such PRBs are not used for PSCCH transmission

  - FFS: whether or not such PRBs are used for PSFCH/S-SSB transmission

Agreement

At least R16/R17 NR SL S-SSB slots are excluded from SL resource pool.

- Note: whether or not additional candidate S-SSB occasions are excluded from resource pool will be discussed after the details of additional candidate S-SSB occasions are clearer

Agreement

At least there is 1 PSFCH occasion per PSCCH/PSSCH transmission, FFS details

Agreement

To address PSFCH transmission dropping due to LBT failure, the followings are to be studied:

- Alt 1: Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission

- Alt 2: PSFCH resources are dynamically indicated

- Alt 3: Convey SL-HARQ feedback information in PSCCH/PSSCH, e.g., new SCI or new MAC-CE

- Alt 4: drop PSFCH transmission

- Alt 5: Support trigger based HARQ feedback reporting for non-numerical HARQ FB and one shot HARQ FB

- Combination of above alternatives are not precluded

- FFS details of above alternatives

Agreement

Regarding additional candidate S-SSB occasions:

- Their number and time domain locations are (pre-)configured or pre-defined

RAN1 #111 agreements:

Agreement

For slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- Regarding the location of 1st starting symbol, down-select one of the followings:

  - Option 1: it is fixed as symbol#0

  - Option 2: it is indicated by sl-StartSymbol as in R16 NR SL

- Regarding the location of 2nd starting symbol, down-select one of the followings:

  - Option A: it is a fixed location

    - FFS the location, e.g., symbol#4, #7, etc.

  - Option B: it is a (pre-)configured location per resource pool

    - FFS the details of candidate locations

- Note: assume symbol index in a slot starts from #0

Agreement

If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- TBS is determined based on a reference symbol length, down-select one of the followings:

  - Option 1: The reference symbol length is dynamically indicated by Tx UE

  - Option 2: The reference symbol length is determined based on 1st starting symbol

  - Option 3: The reference symbol length is determined based on 2nd starting symbol

  - Option 4: The reference symbol length is (pre-)configured

Agreement

Regarding PSFCH transmission under 15 kHz and 30 kHz SCS, RAN1 continues studying the following updated alternatives:

- Alt 1-1a: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

  - FFS: value of K3

- Alt 2-1a: each PSFCH transmission occupies 1 interlace, and further apply frequency-domain OCC

  - FFS: details of FD-OCC, e.g., OCC length, RB-level, RE-level, etc.

- Alt 2-2a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift

  - A UE transmits dedicated cyclic shift on K1 dedicated PRB(s) within this interlace, and transmits common cyclic shift on other PRBs of this interlace

  - FFS: value of K1

- Alt 2-3a: each PSFCH transmission occupies 1 interlace

- Alt 2-4a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift

  - A UE uses different cyclic shifts on different PRBs in the interlace

- Alt 3-1a: each PSFCH transmission occupies 1 dedicated PRB and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set

  - The above dedicated PRB and common PRBs are within 1 interlace

  - FFS: value of K2

- Alt 3-2a: each PSFCH transmission occupies 1 dedicated PRB and 2 common PRBs, where 2 common PRBs locate at the two edges of a RB set

- FFS: the impact of PSD limit, e.g., whether/how to handle the case when common PRB and dedicated PRB locate within the same 1 MHz bandwidth

- FFS: whether IBE issue exists and whether/how to address it

- Note: in the above descriptions

  - The dedicated PRB/cyclic shift conveys ACK/NACK information

- Note: as previously agreed: to meet OCB and PSD requirement for PSFCH transmission, at least RB-based interlace is supported at least for 15 kHz and 30 kHz SCS.

Agreement

Slots with PSFCH symbols only have 1 candidate starting symbol for PSCCH/PSSCH.

Agreement

For interlace RB-based PSCCH/PSSCH transmission in SL-U:

- Regarding mapping between sub-channel and interlace, RAN1 further study the followings:

  - Option 1: 1 sub-channel is defined and indexed within 1 RB set, and is periodically indexed across different RB sets within the resource pool

  - Option 2: 1 sub-channel is defined within 1 RB set, and is incrementally indexed firstly within an RB set, then across different RB sets within the resource pool

  - Option 3: 1 sub-channel is defined across all RB sets within the resource pool, i.e., 1 sub-channel includes K interlace(s) across all RB sets within the resource pool

  - Option 4: 1 sub-channel is defined within 1 RB set or 2 adjacent RB sets, and is incrementally indexed firstly within an RB set, then across different RB sets within the resource pool

  - Option 5: 1 sub-channel is defined within 1 RB set, and is incrementally indexed firstly across different RB sets within the resource pool, then across different interlaces in the RB set

  - FFS: whether/how to use intra-cell guardband PRBs

Agreement

For S-SSB transmission, down-select one or more of the following for 15 kHz and 30 kHz SCS:

- Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH

- Option 1-2: Using interlaced RB transmission for PSBCH only, and apply OCB exemption to S-PSS and S-SSS

- Option 3-1: Repeat S-PSS/S-SSS/PSBCH N times in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement

  - FFS details, e.g., the length of gap is (pre-)configured or pre-defined, value of N (e.g., N=2)

  - FFS gap of 0

- Option 3-2: Repeat only S-PSS/S-SSS K times in frequency domain, and PSBCH is rate matched. There is a gap between the repetition(s) to meet OCB requirement

  - FFS details, e.g., the length of gap is (pre-)configured or pre-defined, value of K

  - FFS gap of 0

  - FFS PSBCH resource

- Option 3-3: keep the legacy S-PSS/S-SSS/PSBCH while repeating PSBCH N times in frequency domain and rate-matching PSBCH to S-PSS/S-SSS symbols, and there is a gap between the PSBCH repetition(s) to meet OCB requirements

  - FFS details, e.g. the length of gap is (pre-)configured or pre-defined, value of N

- Option A: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH

- For Option 1-1 and 1-2 above

  - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set

- FFS: whether transient period issue exists and whether/how to address it

Agreement

For a slot with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- Regarding Tx UE behaviour:

  - If PSCCH/PSSCH transmission starts from 1st starting symbol, down-select one of the followings

    - Option 1: The PSCCH/PSSCH transmission has 2 symbols for AGC purpose

    - Option 2: The PSCCH/PSSCH transmission has only 1 symbol for AGC purpose

    - Option 3: The PSCCH/PSSCH transmission has 1 or 2 symbol(s) for AGC purpose depending on conditions, FFS details

  - If PSCCH/PSSCH transmission starts from 2nd starting symbol, the PSCCH/PSSCH transmission has only 1 symbol for AGC purpose

- Regarding Rx UE behaviour, down-select one of the followings:

  - Option A: The Rx UE always monitors two AGC symbols in such slot

  - Option B: The Rx UE monitors two AGC symbols in such slot by default, but could drop monitoring the 2nd AGC symbol at least if it detects a PSCCH/PSSCH transmission starting from the 1st starting symbol

    - FFS details

  - Option C: The Rx UE monitors two AGC symbols in such slot by default, but it is up to UE implementation whether to drop monitoring the 2nd AGC symbol

  - Option D: It is up to UE implementation to monitor 1 or 2 AGC symbol(s) in such slot

Agreement

To address PSFCH transmission dropping due to LBT failure, RAN1 down-select one of followings, or support the combination of followings:

- Alt 1: Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission

  - FFS other details, e.g., HARQ-ACK timeline

- Alt 2: PSFCH occasions are dynamically indicated

  - FFS: Whether/how to handle the case where some TB’s corresponding PSFCH cannot be transmitted within the same or different COT

  - FFS other details, e.g., dynamically indicate one or more PSFCH transmission(s), container of the indication, etc.

- FFS: Whether such PSFCH occasions are within the same or different COT of corresponding PSSCH

- FFS: Whether/how to address PSFCH collision if any

- FFS: Whether/how to handle the linearly decreased PSFCH capacity

Agreement

Regarding the number and location(s) of additional candidate S-SSB occasions, RAN1 further study the followings:

- Option 1: Reuse legacy NR SL design, and increase the available values in sl-NumSSB-WithinPeriod for each SCS

- Option 2: Each R16/R17 NR SL S-SSB slot has K corresponding additional candidate S-SSB occasion, and the gap between them is (pre-)configured

  - FFS details, e.g., value of K, details on gap length, etc.

- Option 3: The number and location(s) of additional candidate S-SSB occasions are separately (pre-)configured

- Option 4: Introduce M contiguous candidate S-SSB occasions in one S-SSB period

- Option 5: the number of candidate S-SSB occasions is (pre-)configured, and locations are determined based on the (pre-)configured number

Agreement

Regarding additional candidate S-SSB occasions:

- In the same S-SSB period, RAN1 further study the followings:

  - Alt 1: UE attempts to transmit on all or some of additional candidate S-SSB occasion(s) only when it fails to transmit on R16/R17 S-SSB occasion(s)

  - Alt 2: UE attempts to transmit on all additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s)

  - Alt 3: UE can attempt to transmit on all or some of additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s)

  - Alt 4: upon LBT failure on a (candidate) S-SSB occasion, a UE attempts to transmit on the subsequent additional candidate S-SSB occasion if within a period S-SSB transmission has not been transmitted in any prior occasions

- FFS details

Agreement

For contiguous RB-based PSCCH/PSSCH transmission in SL-U:

- Regarding mapping between sub-channel and PRBs, further study the following options:

  - Option 1 (sub-channel aligns with resource pool boundary): Same as in legacy NR SL, i.e., the mapping of sub-channel starts from the first PRB of the resource pool and mapped sequentially within the resource pool according to the sub-channel size

    - FFS: whether/how to use sub-channel(s) which include intra-cell guardband PRBs

    - FFS: whether/how to handle the case when the number of PRBs of the resource pool cannot be divided by sub-channel size

  - Option 2 (sub-channel aligns with RB set boundary): In each RB set, the mapping of sub-channel starts from the first PRB of the RB set and mapped sequentially within the RB set according to the sub-channel size

    - FFS: whether/how to use intra-cell guardband PRBs

    - FFS: whether/how to handle the case when the number of PRBs of one RB set cannot be divided by sub-channel size

  - Option 3 (sub-channel aligns with RB set boundary): In each RB set, the mapping of sub-channel starts from the first PRB of the RB set and mapped sequentially within the RB set and/or guardband PRB according to the sub-channel size

    - FFS: how to use intra-cell guardband PRBs

    - FFS: how to use the subchannel including PRBs in guardband

Agreement

Regarding PSFCH transmission under 60 kHz SCS, further study the following alternatives:

- Alt 1: Each PSFCH transmission occupies K dedicated PRB(s) and some common PRBs

  - FFS details

- Alt 2: Each PSFCH transmission occupies some dedicated PRBs

  - FFS details

Agreement

Regarding S-SSB, RAN1 further study the following:

- How to transmit S-SSB when a SL BWP contains multiple RB sets

RAN1 #112 agreements:

Agreement

For slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- The location of 1st starting symbol can be (pre)configured from {#0,#1,#2,#3,#4,#5,#6} per BWP

  - By default (if no (pre)configuration), the location of the 1st starting symbol is symbol#0

- The location of 2nd starting symbol is (pre-)configured from {#3,#4,#5,#6,#7} per BWP

  - It shall be configured such that within a slot, the number of symbols used for PSCCH/PSSCH transmission from 2nd starting symbol is not smaller than 6

  - It shall be configured such that within a slot, the 2nd starting symbol is later than the 1st starting symbol

- PSCCH/PSSCH transmission starting from 1st or 2nd starting symbol shall have the same ending symbol within a slot

- Note: assume symbol index in a slot starts from #0

Agreement

For interlace RB-based PSCCH/PSSCH transmission in SL-U:

- Regarding mapping between sub-channel and interlace, 1 sub-channel is defined and indexed within 1 RB set, and is periodically indexed across different RB sets within the resource pool

Agreement

Regarding PSFCH transmission with 15 kHz and 30 kHz SCS, RAN1 down-select one of followings, or support the combination of followings:

- Alt 1-1a: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

  - FFS: value of K3

- Alt 2-2a: each PSFCH transmission occupies 1 interlace, and further apply PRB-level cyclic shift

  - A UE transmits dedicated cyclic shift on K1 dedicated PRB(s) within this interlace, and transmits common cyclic shift on other PRBs of this interlace

  - FFS: value of K1

- Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace

- Alt 2-4a: each PSFCH transmission occupies 1 dedicated interlace and adopt PRB-level cyclic shift hopping as in NR-U

- Alt 3-2a: each PSFCH transmission occupies K4 dedicated PRB(s) and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set

  - FFS: value of K2, K4

- FFS: the impact of PSD limit, e.g., whether/how to handle the case when common PRB and dedicated PRB locate within the same 1 MHz bandwidth, e.g., drop common PRB or reduce power on common PRB in such case

- FFS: whether/how to reduce PAPR of PSFCH transmission

Agreement

To address PSFCH transmission dropping due to LBT failure:

- Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission

  - Down-select one or support both of the followings

    - Option 1: Such PSFCH occasion(s) are (pre-)configured

    - Option 2: Such PSFCH occasion(s) are (pre-)configured and dynamically indicated

    - FFS applicable scenarios, e.g., considering the applicability of COT sharing, MCSt, etc.

  - FFS other details

Agreement

For contiguous RB-based PSCCH/PSSCH transmission in SL-U:

- Regarding mapping between sub-channel and PRBs, down-select one of the followings during RAN1#112:

  - Option 1 (sub-channel aligns with resource pool boundary): Same as in legacy NR SL, i.e., the mapping of sub-channel starts from the first PRB of the resource pool and mapped sequentially within the resource pool according to the sub-channel size

    - FFS: how to deal with the remaining PRBs, e.g. for meeting OCB requirements

Agreement

For S-SSB transmission within 1 RB set, down-select to one or more of the following for 15 kHz and 30 kHz SCS:

- Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH

  - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set, e.g. whether 1 or 2 interlaces will be used for S-SSB

- Option 3-1: Transmit S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement

  - FFS details, e.g., the length of gap between repetitions is (pre-)configured or pre-defined, value of N (e.g., N=2), how to reduce PAPR, etc.

  - FFS gap of 0

- Option A: Apply OCB exemption to all of S-PSS/S-SSS/PSBCH

  - Continue studying how to meet the minimum 2 MHz requirements under 15 kHz SCS.

Working assumption

If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- At least for COT initiation, TBS is determined based on a reference number of symbols as follows:

  - Option 4: The reference number of symbols is determined by (pre-)configuration

  - FFS: value range

  - FFS: whether a different reference number of symbols is needed for transmission in a shared COT

Agreement

Down-select one or support both of the followings:

- Option 1: Additional candidate S-SSB occasions are excluded from resource pool

- Option 2: Additional candidate S-SSB occasions belong to resource pool

- Note: Companies are encouraged to consider aspects including: S-SSB resource overhead, Tx/Rx UE behavior (e.g., whether any blind detection in Option 2), applicable scenarios, etc.

Agreement

RAN1 further study the followings:

- Whether/how to maintain a COT when the COT contains multiple RB sets and includes S-SSB slot(s), e.g., whether to transmit S-SSB repetitions in more than one RB set, etc.

Agreement

RAN1 further study the followings:

- Whether any updates on power control are necessary considering PSD limit in unlicensed spectrum regulation.

RAN1 #112b agreements:

Agreement

Considering PSD limit in unlicensed spectrum regulation, RAN1 further study whether updates on power control is needed especially for PSFCH.

Agreement

Regarding PSFCH transmission with 15 kHz and 30 kHz SCS:

- RAN1 down-select one of followings in RAN1#113:

  - Alt 1-1b: each PSFCH transmission occupies 1 common interlace and K3 dedicated PRB(s)

    - K3 is (pre-)configured, FFS value range

    - On the K3 dedicated PRB(s), multiple CS pairs can be used as in legacy NR SL PSFCH transmission

    - When a PRB of common interlace and a dedicated PRB locate within the same 1 MHz bandwidth, UE only transmits on the dedicated PRB

      - FFS: whether any impact on meeting OCB requirement

  - Alt 2-3a: each PSFCH transmission occupies 1 dedicated interlace

  - Alt 3-2b: each PSFCH transmission occupies K4 dedicated PRB(s) and K2 common PRBs, where K2 common PRBs locate at the two edges of a RB set

    - K2=2

    - K4 is (pre-)configured, FFS value range

    - FFS: how to meet PSD limitation

    - FFS: whether to introduce any restrictions on the locations of K4 dedicated PRB(s) and/or K2 common PRBs, e.g., whether/how they are on the same interlace

- R16 NR SL PSSCH-PSFCH mapping is reused as baseline, FFS details

  - Note: companies are encouraged to give more details and analyze the specification impact

    - E.g., whether PSSCH transmissions on non-overlapped resources are mapped to non-orthogonal PSFCH resources, i.e., whether PSFCH collision may happen and whether/how to address it, etc.

    - E.g., whether introducing more than 6 CS pairs is needed

    - E.g., for group cast option 2, what’s the maximum group size that can be supported

    - E.g., how to support “more than 1 PSFCH occasion(s) per PSCCH/PSSCH”

- FFS: regardless of which Alt above is selected, whether or not to support PRB-level cyclic shift hopping as in NR-U to reduce PAPR

- FFS: whether IBE issue exists and whether/how to address it

  - E.g., whether to introduce guardband PRB/RE between common PRB and dedicated PRB

Working assumption

Additional candidate S-SSB occasions are excluded from resource pool

Agreement

A SL-BWP is (pre-)configured with either contiguous RB-based or interlace RB-based PSCCH/PSSCH transmission, i.e., not both.

Agreement

For interlace RB-based PSCCH/PSSCH transmission in SL-U, considering 1 sub-channel equals K interlace(s), support the followings:

- Option A:

  - TBS is determined based on a reference number of PRBs of one interlace within 1 RB set (denoted as N_ref), down-select one of the followings in RAN1#113:

    - Option A1: N_ref is a fixed value, e.g., 10, 11

    - Option A2: N_ref is (pre-)defined

      - e.g., N_ref is the average number of PRBs per interlace, which is determined by total number of PRBs of the RP divided by the number of interlaces.

    - Option A3: N_ref is (pre-)configured

    - Option A4: N_ref is dynamically indicated by Tx UE

    - Note: The number of PRBs within a sub-channel can be different among sub-channels in a single resource pool subject to (pre-)configuration.

    - FFS: for TBS determination, whether/how to handle the impact of additional available  PRB(s) in intra-cell guard band(s) for PSCCH/PSSCH transmission across multiple RB sets

Agreement

Regarding more than 1 PSFCH occasion per PSCCH/PSSCH transmission, support the followings:

- One PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s) via (pre-)configuration

  - FFS value range of N

  - FFS detailed design of such N associated candidate PSFCH occasion(s)

    - E.g., they are in different slots of the same RB set, or in different RB sets of the same slot, or combination thereof, etc.

    - E.g., whether PSSCH transmission and its related PSFCH occasion(s) are in the same RB set(s)

- FFS: whether to support that COT initiating UE can dynamically indicate which subset of the (pre-)configured PSFCH occasions within its COT are available for PSFCH transmissions.

  - FFS: whether other associated candidate PSFCH occasion(s) within its COT are used for PSSCH transmissions, and applicable scenarios.

  - FFS: whether AGC issue and PSFCH/PSSCH collision issue exist, and whether/how to address them

- FFS other details

  - E.g., how to meet the HARQ RTT restriction

  - E.g., UE behavior on transmitting PSFCH

  - E.g., how to avoid the risk of losing the COT if the COT is interrupted by periodic PSFCH occasions

Agreement

Regarding frequency domain resource indication for interlace RB-based PSSCH transmission, support the followings:

- Option A: Support that for one PSSCH transmission, the used interlace index(s) in different used RB sets are always the same

- Option 1: Support explicitly indicating the used sub-channel index(s) and RB set index(s)

  - Frequency domain resource of PSSCH transmission is determined by an intersection of the resource blocks of the indicated sub-channel(s) and the union of the indicated set of RB sets and intra-cell guard bands between the indicated RB sets, if any

  - For a TB, the initial transmission and reservation of the resource(s) for retransmission(s) use the same number of sub-channel(s) and same number of RB set(s)

    - FFS: whether additionally support different number of RB set(s) in such case while keeping total number of sub-channels unchanged between initial transmission and retransmission(s) for a TB

  - Use X bits for indicating sub-channel index(s), and use Y bits for indicating contiguous RB set index(s)

    - R16 NR SL FRIV is reused as baseline

    - FFS details, e.g., signaling design, bit size, whether to consider bitmap design, whether/how the used interlace(s) can be non-contiguous, etc.

- FFS others

  - E.g., considering one PSSCH transmission may occupy one or multiple RB sets, whether or not to re-define single-slot candidate resource, and update resource selection and/or signaling from MAC to PHY, etc.

Agreement

For contiguous RB-based PSCCH/PSSCH transmission in SL-U, regarding sub-channel(s) which include intra-cell guardband PRBs, down-select one or more of the followings in RAN1#113:

- Option 2: Such sub-channel(s) can be used for PSCCH/PSSCH transmission

  - Note: PRBs within intra-cell guard band are not used for PSCCH transmission as per previous agreement

- Option 3: Such sub-channel(s) cannot be used for PSCCH transmission, and can be used for PSSCH transmission

- FFS details, e.g., conditions to apply the above Option(s)

- FFS impacts on definition of candidate resource, and resource selection

Agreement

If a resource pool includes slots with 2 candidate starting symbols for a PSCCH/PSSCH transmission:

- TBS is determined based on a reference number of symbols (denoted as L_ref)

- Support the followings

  - Alt 1: Support Option 4 only

- Note: the options are as below

  - Option 4: The reference number of symbols is determined by (pre-)configuration

- FFS details, e.g., in TS 38.214 Clause 8.1.3.2, whether L_ref replaces Nsymbsh or sl-LengthSymbols or other usage of L_ref, whether NsymbPSFCH is determined in the same way as in legacy NR SL, etc.

Agreement

For interlace RB-based PSCCH/PSSCH transmission in SL-U, regarding details of mapping between sub-channel and interlace:

- In a resource pool with multiple RB sets, sub-channel with the same index is mapped to K interlace(s) with the same index(s) in different RB sets.

- In a resource pool, support the following

  - At least for the agreed case where one SL resource pool can be (pre-)configured to include integer number of RB sets

    - Option 2: sub-channel#0 is mapped to K interlace(s) starting from interlace#0

      - sub-channel#1 is mapped to K interlace(s) starting from interlace#K, and so on

      - At least support that the above K interlace(s) are contiguous

        - FFS: whether/how to support the above K interlace(s) are non-contiguous

  - FFS: if RAN1 agrees to support that one SL resource pool can be (pre-)configured to include sub-set of PRBs of one RB set, the mapping between sub-channel and interlace for this case will be further discussed

- Interlace is indexed as per NR-U

Agreement

When the SL-BWP contains multiple RB sets, study the followings:

- When UE attempts to transmit S-SSB in a S-SSB occasion (e.g., R16/17 S-SSB occasion, R18 additional candidate S-SSB occasion)

  - Alt 1: UE may transmit S-SSB repetition in more than one RB set

    - FFS details, e.g., location of such S-SSB repetition(s) (e.g., (pre-)configured and/or pre-defined), whether/how to address potential power reduction and/or potential fluctuation of PSBCH-RSRP

    - FFS: the relationship with UE’s COT

    - FFS: the scenario that UE may or may not transmit S-SSB repetition in more than one RB set

    - Note: whether UE can transmit S-SSBs over non-contiguous RB sets is subject to RAN4’s reply, details can be found in RAN1’s LS to RAN4 in R1-2304218

Agreement

For S-SSB transmission within 1 RB set, for 15 kHz and 30 kHz SCS, Alt6 is supported:

- Alt 6: Support both Option 3-1(revised) and Option B, and enable one of them by (pre-)configuration

Note: the Options are as below

- Option 1-1: Using interlaced RB transmission for all of S-PSS/S-SSS/PSBCH

  - FFS: whether/how to handle the case when each interlace has only 10 PRBs in a RB set, e.g. whether 1 or 2 interlaces will be used for S-SSB

- Option 3-1(revised): Transmit legacy S-PSS/S-SSS/PSBCH N times by repetition in frequency domain, and there is a gap between the repetition(s) to meet OCB requirement

  - FFS the length of gap between repetitions is (pre-)configured or pre-defined, value of N (e.g., N=2), whether/how to reduce PAPR.

  - FFS gap of 0

- Option A: Legacy S-SSB

  - Continue studying how to meet the minimum 2 MHz requirements under 15 kHz SCS for OCB exemption.

- Option B: Legacy S-SSB

  - RAN1 does not pursue further study on how to meet the minimum 2 MHz requirements under 15 kHz SCS for OCB exemption.

Note: Option A and B are applicable in region with no OCB requirement, or with OCB exemption.

Conclusion

Regarding additional candidate S-SSB occasions, in the same S-SSB period, UE can attempt to transmit on additional candidate S-SSB occasion(s) regardless of whether or not it transmitted on R16/R17 S-SSB occasion(s).

Agreement

For interlace RB-based PSCCH/PSSCH transmission in SL-U, support the following:

- Option 1: lowest sub-channel is the sub-channel with smallest sub-channel index

Agreement

Regarding Tx UE behavior, at least when it initiates a COT:

- For the 1st slot of a COT, the Tx UE chooses the earliest starting symbol for PSCCH/PSSCH transmission after clearing LBT.

  - Note: in the same slot, Tx UE can use the 2nd starting symbol only if LBT fails at the 1st starting symbol

- FFS: whether/how to support that for the remaining slots of a COT, the Tx UE only chooses the 1st starting symbol for PSCCH/PSSCH transmission.

  - FFS applicable scenarios

    - e.g., at least for MCSt with no greater than 16us gap

    - e.g., at least for transmission with no greater than 16us gap from the previous transmission by any UE

  - FFS: Rx UE behavior

FFS: COT sharing case