3GPP TSG RAN WG1 #113	                                    		                         R1-2305175

Incheon, Korea, May 22 – May 26, 2023

Agenda Item:	9.8.1

Source:	InterDigital, Inc.

Title:	Discussion on XR-specific capacity enhancements

Document for:	Discussion and Decision

# Introduction

The Rel-18 New WID on XR Enhancements for NR has the following objectives on capacity [1]:

Specify the enhancements related to capacity: - Multiple Configured Grant (CG) PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2); - Dynamic indication of unused CG PUSCH occasion(s) based on Uplink Control Information (UCI) by the UE (RAN1, RAN2); - Buffer Status Report (BSR) enhancements including at least new Buffer Status Table(s) (RAN2); - Delay reporting of buffered data in uplink (RAN2); - Discard operation of PDU Sets for DL and UL (RAN2, RAN3);

The agreements made during RAN1#112bis-e [2] are provided in the Appendix. In this contribution, we discuss the enhancements for multi-PUSCH CG and the dynamic indication of unused PUSCH occasions (UTO-UCI). We also provide evaluations results on XR capacity improvements when supporting the multi-PUSCH CG with UTO-UCI.

# Discussion

During the RAN1#112bis-e meeting [2], several open issues on enhancements for supporting multi-PUSCH CG were identified and discussed. In the following, we discuss the remaining aspects on:

- TDRA design for multi-PUSCH CG

- HARQ retransmissions

- Time duration/range of UTO-UCI

- Persistence and applicability of UTO-UCI

## 2.1 Multi-PUSCH CG

### 2.1.1 TDRA design

When supporting XR traffic in UL, the video encoder at the UE may generate different types of video frames (e.g. I-frame, P/B-frame) and encoding patterns at a particular frame rate (e.g. 60fps, 90fps). The generated video frames or PDU sets may consist of variable payload sizes corresponding to different TBs at each periodic occasion. During UL transmission, the PDUs in a PDU set may need to be delivered in multiple TBs within a PDU set delay bound (PSDB) [3]. The different TBs may be delivered in different PUSCH occasions within a CG period. For ensuring that the different TBs can be delivered within the delay bound, multiple slots in a CG period can be used, with slots consisting of one or multiple PUSCH occasions.

For delivering XR traffic within tight delay budget, the UL slots in a CG period are expected to be consecutive. That is, in a CG period possibly consisting of several TDD frames, the K UL slots (e.g. DDDSUDDDSU..) are expected to be consecutive, regardless of the presence of intermediary DL slots. The support for non-consecutive slots may be useful if there is wide separation or jitter during the arrival of the PDUs/PDU sets from higher layers. However, such separation is not necessary considering the existing DL slots between the UL slots in the existing TDD frame structure. Similarly, configuring multiple PUSCH occasions per slot is beneficial to support transmission of multiple TBs corresponding to different PDUs of a PDU set within tight PSDB. Even in this case, there is no benefit for configuring non-consecutive PUSCHs within a slot. As such, it is preferred to configure multiple consecutive PUSCH occasions per slot and multiple consecutive UL slots per CG period.

As per the agreement made during RAN1#112bis-e, the MCS and FDRA of the CG PUSCHs are the same between the different PUSCH occasions. In this case, when multiple PUSCH occasions are configured per slot, the symbol length of each PUSCH occasion (e.g. indicated by SLIV for the PUSCH occasions) and frequency domain parameters are expected to be the same. The following Figure 1 illustrates the multi-PUSCH CG configuration.

Figure 1: An illustration of multi-PUSCH CG configuration consisting of 2 consecutive PUSCH occasions per slot and 2 UL slots per CG period.

During RAN1#112bis-e, the following TDRA alternative designs for multi-PUSCH CG were agreed to be prioritized for further down scoping and/or modifications:

- Alternative A1: Type-A repetition

- Alternative B: NR-U framework

- Alternative C2: Rel-17 single DCI scheduling multiple PUSCHs

Alt-A1 may allow repurposing certain parameters such as rep-K or introducing new parameters to denote multiple PUSCHs. While Type-A repetition framework can be extended to support flexible configuration of different number of PUSCH occasions per slot and per CG period the spec impact is expected to be high, especially when supporting both repetition and non-repetition-based TBs with multi-PUSCH CG and Type 2 CG.

Alt-B, corresponding to extension from R16 NR-U CG [4], allows configuring parameters associated with multiple PUSCH occasions per slot and multiple slots in a CG period. However, for R16 NR-U CG such parameters can be configured only when cg-RetransmissionTimer is configured, which is associated with autonomous retransmission. Additionally, in NR-U the multiple PUSCH occasions in one or multiple slots are assumed to be consecutive. A related issue highlighted during the email discussion during RAN1#112bis-e [5], was on TDD configuration where a potential collision can occur between UL and DL slots when consecutive slots are configured for multi-PUSCH CG. To address this issue, the restriction on consecutive slots in NR-U should be relaxed to infer to consecutive UL slots, regardless of whether there may be any DL slots between the consecutive UL slots in TDD. In addition to relaxing the restriction on cg-RetransmissionTimer, extending the existing parameters associated with Alt-B are sufficient for supporting UL transmissions of XR data.

For supporting Type 1 multi-PUSCH CG with Alt-B based TDRA design, the parameters expected to be configured include 1) number of consecutive UL PUSCH occasions per slot and 2) number of consecutive UL slots per CG period. For Type 2 multi-PUSCH CG, the activation DCI can carry the MCS and FDRA as per legacy, and the same MCS and FDRA is applied for all configured PUSCH occasions. For both Type 1 and Type 2, the TDRA info corresponding to the K2 slot offset, PUSCH mapping type and SLIV, can be extended for supporting multi-PUSCH from the legacy Default A or PUSCH-TimeDomainAllocationList [6] tables. In this case, for Type 1 CG and Type 2 CG the TDRA info is configured in RRC signaling and indicated in activation DCI, respectively, as per legacy.

Alt-C2 allows configuring non-consecutive PUSCH occasions and non-consecutive slots. In this case, the UE determines the N PUSCH occasions for multi-PUSCH CG based on configuration in RRC and/or in activation DCI, indicating the TDRA row that contains the N entries corresponding to the N PUSCHs. Alt-C2 provides similar flexibility in terms of the parameters expected to be configured/indicated as that of Alt-B. The single DCI based scheduling design per Alt-C2 can also be applied for supporting retransmissions with multi-PUSCH CG. The extendedK2 parameter associated with pusch-TimeDomainAllocationListForMultiPUSCH allows K2 to be configurable per PUSCH. For supporting multi-PUSCH CG, the existing restriction to the TDRA field in DCI indicating a row with single SLIV [7] is to be relaxed so that multiple SLIVs per row can be supported. Although Alt-C2 allows flexible allocation of non-consecutive PUSCHs and slots with single DCI for Type 2 CG, whether/how such flexibility can be realized for Type 1 CG, and spec impact needed is unclear.

Based on the comparison of the pros and cons, aspects from both Alt-B and Alt-C2 can be considered for TDRA for multi-PUSCH CG. Given lower spec impact, Alt-B and the associated parameters are preferable to be considered as baseline at least for Type-1 multi-PUSCH CG. A similar case can be made in support for using Alt-C2 for Type-2 multi-PUSCH CG, given its advantages in terms of flexibility.

Proposal 1: TDRA design based on Alt-B (NR-U framework) can be considered at least for Type-1 multi-PUSCH CG enhancements

Proposal 2: TDRA based on Alt-C2 (Rel-17 single SCI scheduling multiple PUSCH) can be considered for Type-2 multi-PUSCH CG enhancements

Proposal 3: The following parameters are configured for multi-PUSCH CG configuration:

- Number of consecutive PUSCH occasions per slot

- Number of consecutive UL slots per CG period

### 2.1.2 HARQ Retransmissions

In the legacy Rel-16 NR CG design for retransmissions, the UE monitors for DCI upon transmitting a TB in a CG PUSCH occasion. If a DCI containing dynamic assignment is received before the expiry of the configuredGrantTimer, the UE assumes NACK for the TB and retransmits the indicated RV of the TB using the received assignment. If the UE does not receive DCI within the CG timer, the UE assumes ACK for the previous TB transmission and flushes the HARQ buffer.

Extending from the legacy retransmission procedure for multi-PUSCH CG enables the retransmissions of multiple TBs to be performed with dynamic assignments rather than the configured PUSCH occasions in CG. For example, if the DCI is received carrying the HARQ feedback for multiple TBs (with different HPIs) before the expiry of the CG timer, the UE performs retransmission of the TBs with NACK using the dynamic grants. Extending the legacy procedure also reduces the complexity at the UE for deciding on which of the PUSCH occasions are unused, since the UE does not need to account for whether any of the configured PUSCH occasions would be used for retransmissions.

For supporting retransmissions with multi-PUSCH CG, it is important for the HARQ feedback to be provided early given the strict delay budget associated with the XR traffic. Since each slot associated with a CG period can consist of one or multiple PUSCH occasions, the earliest opportunity to provide HARQ feedback can be in the first DL slot after a number of used PUSCH occasions in an UL slot in a CG period. For minimizing the amount of PDCCH monitoring, upon transmitting N TBs in N PUSCH occasions, the corresponding HARQ feedback and the dynamic assignments for retransmissions can be provided in a single DCI. Also, instead of starting and maintaining multiple CG timers for each TB, the UE can start a single CG timer after transmitting N TBs in N PUSCH occasions, during which the UE can monitor for the HARQ feedback.  For example, while CG timer is running, the UE can monitor for the DCI in a DL slot with an offset of L slots after transmitting in N PUSCH occasions. This would allow any retransmissions to be performed as early as in the next UL slot, even though the next UL slot may be within the same CG period.

Proposal 4:  A single CG timer is used for multi-PUSCH CG, that is started by UE after transmitting N TBs in N PUSCH occasions

Proposal 5: The HARQ feedback for multiple PUSCHs and dynamic grants for retransmissions in multi-PUSCH CG are provided in single DCI

Proposal 6: UE monitors for PDCCH carrying the HARQ feedback for multi-PUSCH CG in the DL slot that is L slots after transmitting N TBs in N PUSCH occasions

## 2.2 UTO-UCI for multi-PUSCH CG

The agreements made during RAN#112bis-e related to the UCI that provides info on unused CG PUSCH occasions or ‘UTO-UCI’ were associated with:

- contents (i.e. indication of consecutive or non-consecutive PUSCHs),

- location (i.e. every CG PUSCH that is transmitted), and

- format (i.e. bitmap where a bit corresponds to a TO within a time duration/range)

The following we discuss the remaining issues on UTO-UCI.

### 2.2.1 Time duration/range of UTO-UCI

One of the open issues from the previous meeting is on whether the UTO-UCI can indicate the unused CG PUSCH occasions corresponding to only one or multiple CG periods. To address this, it is important to recall the XR application and the associated traffic that is generated for UL transmission. The application may typically generate one or multiple PDU sets at different periodic occasions. When a data burst consisting of multiple PDU sets is generated, the UE can determine the usage of the PUSCH occasions for multiple CG periods based on the PDU sets in the UE buffer. Indicating early the amount of unused PUSCH occasions for multiple CG periods can be beneficial for the network by improving the possibility of reallocating any of the excess resources. As such, there is no reason why a restriction is needed for the UE to send the UTO-UCI that corresponds to only one CG period. The UTO-UCI can provide info on PUSCH usage for at least one CG period, with a possibility to indicate for multiple CG periods (e.g. in current and subsequent CG periods). For example, the indication of unused PUSCHs for multiple CG periods may be applicable when legacy CG (e.g. one PUSCH occasion per CG period) is configured. Hence, we propose the following:

Proposal 7: UTO-UCI indicates the unused CG PUSCHs corresponding to at least one CG period

One of the FFS from RAN1#112bis-e is on the time duration/range of the bitmap that indicates the unused PUSCH occasions. As per the agreement, each bit in the bitmap indicates whether a TO associated with a PUSCH occasion is unused. In this regard, the time duration or range of the bitfield in the bitmap depends on the number of configured PUSCH occasions. Whether the time duration/range corresponding to the bitmap spans one CG period or multiple CG periods can be configured by RRC.

Proposal 8: The time duration/range of the bitmap associated with UTO-UCI spans at least one CG period

### 2.2.2 Persistence and applicability of UTO-UCI

In RAN1#112bis-e, it was agreed that the UTO-UCI is included in every CG PUSCH that is transmitted. Whether the content of the UTO-UCI remains the same in each PUSCH occasion should be discussed. The UE can determine the quantity and which of the PUSCH occasions that are unused based on the amount of data in buffer and any delays due to late arrival of data from higher layers. Due to encoding of different types of video frames, the overall payload sizes of the data expected to be transmitted in each period can vary significantly. Also, the delays in the encoding can cause the arrival of some data to be delayed. For example, the UE may estimate the overall payload size of a PDU set based on the info on the number of PDUs in the PDU set in the header of the first PDU. The UE may initially estimate the number of PUSCH occasions that are unused/used based on the higher layer info and send the UTO-UCI in the first few PUSCH occasions. However, due to delays for receiving the subsequent PDUs of the PDU set, the UE may decide to update/override the contents of the UTO-UCI, based on the latest availability of the data in the buffer. In this case, in the UTO-UCI sent in later PUSCH occasions the UE may decide to report a higher number of unused PUSCH occasions compared to the UTO-UCI sent earlier. It is beneficial to allow overriding the UTO-UCI, at least the case where the PUSCH occasions reported earlier as ‘used’ to be updated as ‘unused’, since the UE can refine the PUSCH usage info based on the latest UE buffer status. To ensure that the UE does not update the UTO-UCI frequently and impact reliability of the UTO-UCI, the maximum number of times the UE may be allowed to override the UTO-UCI within a time duration can be configured via RRC. The evaluation results showing the capacity performance of multi-PUSCH CG in scenarios where the UTO-UCI is allowed to be overridden vs. the same UTO-UCI in all PUSCHs within in a CG period are given in Section 3.

Proposal 9: UTO-UCI can be updated/overridden at least for the case where the PUSCH occasions previously indicated as ‘used’ can be updated to ‘unused’

Proposal 10: The maximum number of times the UTO-UCI is allowed to be updated is configured via RRC

Another open issue from RAN1#112bis-e is on whether the UTO-UCI can be associated with multiple CG configurations. From the higher layer understanding XR traffic flows, the different flows (e.g. power/control and video) may have different periodicities and different payload sizes per period. As such, the UE may be configured with different CG configurations, each with a different set of parameters (e.g. start offset, periodicities, number of PUSCHs per period) which may be aligned with the different flows. In this case, it is not clear how the number of unused PUSCHs for one CG configuration can be inferred from the unused PUSCHs in another CG configuration. Since the UE can anyways send the UTO-UCI in the PUSCHs for each CG configuration individually, there is no motivation why a single UTO-UCI associated with multiple CG configurations is needed.

Proposal 11: UTO-UCI indicates the unused TOs associated with only one CG configuration

# Simulation Results

In this section, SLS results on capacity performance for the single stream traffic model in UL are provided. The SLS results are provided for Indoor Hotspot (InH) deployment scenario using baseline evaluation assumptions and parameters as listed in (Annex A) as per TR 38.385.

System capacity is defined as the maximum number of supported UEs out of which at least Y% of UEs are satisfied (e.g., PDB and PER requirements are met for UL traffic), where Y=90 (baseline) is used in the simulations. A UE is declared satisfied if more than 99% of packets are successfully transmitted within the PDB.

The system capacity achieved with CG (baseline) and enhanced CG schemes are compared with baseline DG. For the baseline DG the scheduling delay incurred due to the transmission of SR/BSR and PDCCH is assumed to have a mean value of 5ms, while no scheduling delay is assumed for the CG (baseline) and enhanced CG schemes. The capacity performance when using DG, CG and enhanced CG is evaluated for UL AR traffic with 10Mbps, and PDB of 10ms.

The baseline schemes used in the evaluations are:

- DG (with PF scheduling)

  - Single PDCCH schedules 1 PUSCH

  - Overhead: 2 PUCCH symbols per PUSCH

  - Mean scheduling delay: 5ms

The CG schemes evaluated in the simulations are:

- Baseline CG

  - Configured with 2 PUSCH occasions per CG period. The length of each PUSCH occasion is 2 symbols.

  - CG period is 10 ms (with 30 kHz SCS, this yields a 20 slot radio frame for the CG period)

- Enhanced multi-PUSCH CG with flexible re-allocation of PUSCH occasions.

  - Configured with at least 2 PUSCH occasions per slot. The length of each PUSCH occasion is 2 symbols.

  - Number of UL slots per CG period: 4

  - Periodicity: 10ms

  - Overhead: 1 PUCCH symbol per slot

  - For this scheme, we assume the UE can indicate to gNB the unused CG PUSCH occasions. The gNB can reconfigure/reallocate these PUSCH occasions and the associated resources to other UEs that need additional resources.

  - Number of CG reconfigurations done based on the UTO-UCI indication:

    - 1 reconfiguration (same UTO-UCI is sent in all UL slots)

    - 2 reconfigurations (two UTO-UCI versions are sent in slot 1 and slot 3, where the UTO-UCI sent in slot 3 updates/overrides the UTO-UCI sent in slot 1)

UL system capacity for Indoor hotspot (with parameters as defined in Table 1 in Annex) is evaluated for FR1 (4 GHz) using SU-MIMO with DG (baseline), CG, and enhanced CG with flexible re-allocation of PUSCH occasions.

Fig. 2 shows the UL results for system capacity for AR with PDB of 10ms in an InH deployment. It is observed that for the Uplink AR case, where aggregated video is sent in the UL, baseline CG as well as enhanced CG outperforms the baseline DG scheme. In the case of enhanced CG, corresponding to 2 reconfigurations (i.e. 2 UTO-UCI versions are sent), the UE sends the first UTO-UCI in the UL slot 1 which is used to perform initial CG reconfiguration,  and the second UTO-UCI in the UL slot 3 which is used perform second CG reconfiguration. Whereas in enhanced CG with 1 reconfiguration, the UE sends the UTO-UCI in the first UL (and remaining UL slots) and only a single CG reconfiguration is performed.

From the results in Fig. 2, the baseline DG can support 7 UEs per cell, baseline CG can support 9 UEs per cell, while enhanced CG (with 2 reconfigurations) outperforms both with a capacity of 10 UEs per cell. Baseline DG is severely impacted by scheduling delay of 5 ms, which heavily eats into the stringent 10ms PDB requirement, while reconfiguring CG just once per CG period of 10 ms is also insufficient to improve CG performance. Given the different factors corresponding to UL traffic such as variable PDU arrival time, variable payload sizes, tight delay budget and UL jitter, performing multiple CG reconfigurations based on multiple versions of UTO-UCI in a CG period results in better capacity compared to single CG reconfiguration for the enhanced CG. In this case, allowing UTO-UCI to be updated based on updated information on PUSCH usage allows to reallocate the CG resources more efficiently, and hence, improve capacity.

Figure 2: FR1 UL AR 10 Mbps (10ms PDB) for Indoor Hotspot scenario

Observation 1: Enhanced CG scheme based on updating/overriding of the UTO-UCI indication with flexible (re)allocation of CG PUSCH occasions outperforms the baseline CG scheme and the enhanced CG scheme where UTO-UCI updating is not supported.

# Conclusion

In this contribution, the following observation and proposals are made:

Observation 1: Enhanced CG scheme based on updating/overriding of the UTO-UCI indication with flexible (re)allocation of CG PUSCH occasions outperforms the baseline CG scheme and the enhanced CG scheme where UTO-UCI updating is not supported.

Proposal 1: TDRA design based on Alt-B (NR-U framework) can be considered at least for Type-1 multi-PUSCH CG enhancements

Proposal 2: TDRA based on Alt-C2 (Rel-17 single SCI scheduling multiple PUSCH) can be considered for Type-2 multi-PUSCH CG enhancements

Proposal 3: The following parameters are configured for multi-PUSCH CG configuration:

- Number of consecutive PUSCH occasions per slot

- Number of consecutive UL slots per CG period

Proposal 4: A single CG timer is used for multi-PUSCH CG, that is started by UE after transmitting N TBs in N PUSCH occasions

Proposal 5: The HARQ feedback for multiple PUSCHs and dynamic grants for retransmissions in multi-PUSCH CG are provided in single DCI

Proposal 6: UE monitors for PDCCH carrying the HARQ feedback for multi-PUSCH CG in the DL slot that is L slots after transmitting N TBs in N PUSCH occasions

Proposal 7: UTO-UCI indicates the unused CG PUSCHs corresponding to at least one CG period

Proposal 8: The time duration/range of the bitmap associated with UTO-UCI spans at least one CG period

Proposal 9: UTO-UCI can be updated/overridden at least for the case where the PUSCH occasions previously indicated as ‘used’ can be updated to ‘unused’

Proposal 10: The maximum number of times the UTO-UCI is allowed to be updated is configured via RRC

Proposal 11: UTO-UCI indicates the unused TOs associated with only one CG configuration

# Appendix A

The following show the conclusions and agreements made during RAN1#112bis-e:

Agreement For TDRA design for multi-CG PUSCH, prioritize Alt-A1, Alt-B, and Alt-C2 for further downscoping and/or modification from corresponding agreement in RAN1#112. - FFS: How to address TDD configuration issue Agreement For CG PUSCHs in a multi-PUSCHs CG configuration, MCS of the CG PUSCHs in the CG configuration are the same between different PUSCH occasions Agreement For CG PUSCHs in a multi-PUSCHs CG configuration, FDRA of the CG PUSCHs in the CG configuration are the same between different PUSCH occassions R1-2304045 Moderator Summary#2 - XR Specific Capacity Improvements Moderator (Ericsson) Agreement For dynamic indication of unused CG PUSCH transmission occasion(s) based on a UCI, the indicated “unused” CG PUSCH TO(s), if any, by the UCI in a CG PUSCH for a CG configuration - can be consecutive or non-consecutive CG PUSCH TO(s) in time domain [in one CG period] - FFS whether/how the unused TO(s) can be associated to multiple CG configuration. Note: FFSs and further details in corresponding agreement in RAN1#112 for the selected option are remained for further discussion Note: Above corresponds to Option 2 (w.r.t. agreement in RAN1#112) Agreement - Option 1: For a CG PUSCH configuration, the UTO-UCI is included in every CG PUSCH that is transmitted (that is Option 1 in corresponding agreement in RAN1#112) - FFS details - Note: The term “UTO-UCI” refers to the “UCI that provides information about unused CG PUSCH transmission occasions” for convenience. Agreement The UCI that provides information about unused CG PUSCH transmission occasions is defined as a “new UCI” (i.e. Alt. 1 of previous agreement). Agreement - With respect to PHY two-level priority, for a configured grant PUSCH configuration, the “UTO-UCI” has the same priority level as the configured grant PUSCH. - Note: The term “UTO-UCI” refers to the “UCI that provides information about unused CG PUSCH transmission occasions” for convenience. Agreement The existing CG-UCI encoding and multiplexing procedures are reused for encoding the “UTO-UCI” in a configured grant PUSCH in absence or presence of other UCIs being multiplexed in the PUSCH, by applying the following adjustments: - The “UTO-UCI” is used instead of CG-UCI in the corresponding procedures for encoding of CG-UCI and/or HARQ-ACK and/or CSI, whichever is present. - For determining the beta-offset, - Beta offset is configured for the “UTO-UCI” and applied when applicable. - If UTO-UCI and HARQ-ACK is not jointly encoded, the beta offset for the “UTO-UCI” is used in the procedures instead of CG-UCI beta offset, when applicable. - If UTO-UCI and HARQ-ACK is jointly encoded, HARQ-ACK beta offset is used in the procedures instead of CG-UCI beta offset - FFS on sequence generation order between UTO-UCI and HARQ-ACK - FFS on dropping rule between UTO-UCI and HARQ-ACK when joint encoding is not configured - Note: The term “UTO-UCI” refers to the “UCI that provides information about unused CG PUSCH transmission occasions” for convenience. Proposal 1-2-1 (updated3) From RAN1 perspective, for determination of HARQ process Ids associated to PUSCHs in multi-PUSCHs CG assuming one TB per PUSCH: - The HARQ process ID for the first configured/valid PUSCH in a period is determined based on the legacy CG procedure when cg-RetransmissionTimer is not configured, and applying the following formula, whichever is applicable - HARQ Process ID = [floor(X*(CURRENT_symbol – offset1) / periodicity) + offset2] modulo nrofHARQ-Processes - HARQ Process ID = [floor(X*(CURRENT_symbol – offset1) / periodicity) + offset2] modulo nrofHARQ-Processes + harq-ProcID-Offset2 - FFS whether in formulas above X is outside or inside floor operation, i.e. - HARQ Process ID = [X*floor( (CURRENT_symbol – offset1) / periodicity) + offset2] modulo nrofHARQ-Processes - HARQ Process ID = [X*floor((CURRENT_symbol – offset1) / periodicity) + offset2] modulo nrofHARQ-Processes + harq-ProcID-Offset2 - (Working Assumption) The HARQ process ID of the remaining configured/valid CG PUSCHs in the period is determined by incrementing the HARQ process ID of the preceding PUSCH in the period by Y with module operation with nrofHARQ-Processes or module operation with (nrofHARQ-Processes + harq-ProcID-Offset2), whichever applicable. - FFS whether X=1 or X= the number of configured PUSCHs in the CG period - FFS whether Y =1 or a value larger than 1, e.g. Y=2. - FFS: If Y>1, Y is determined based on RRC - FFS whether Offset 1= 0 or can be a non-zero value. - FFS: If offset1 is non-zero, how offset1 is determined (i.e., based on RRC) - FFS whether Offset 2= 0 or can be a non-zero value. - FFS: If offset2 is non-zero, how offset2 is determined (i.e., based on RRC or dynamically) - Note1: The equations will be updated accordingly when FFSs are clarified, e.g., if X=1, remove X; if Y=1, remove Y; if non-zero offset1 or Offset 2 is not supported, remove offset 1 or Offset 2. - Note2: A configured CG PUSCH is invalid if the CG PUSCH is dropped due to collision with DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or SSB. Agreement The UTO-UCI provides a bitmap where a bit corresponds to a TO within a time duration/range. The bit indicates whether the TO is “unused”. - FFS: Details including time duration/range Note: The term “UTO-UCI” refers to the “UCI that provides information about unused CG PUSCH transmission occasions” for convenience.

# Appendix B

The following show the parameters from TR 38.835 used in system level simulations for XR.

Table 1: Assumptions for System-level simulations

| Scenario | Indoor hotspot | Dense Urban |  |
| --- | --- | --- | --- |
| Layout | 120m x 50m ISD: 20m TRP numbers: 12 | 21 cell with wraparound ISD：200m |  |
| Carrier frequency | FR1:4GHz | FR1:4GHz |  |
| Bandwidth | FR1:100MHz | FR1:100MHz |  |
| Subcarrier spacing | FR1: 30 kHz | FR1:30kHz |  |
| BS height | 3m | 25m |  |
| UE height | hUT=1.5 m |  |  |
| BS noise figure | FR1: 5 dB | FR1: 5 dB |  |
| UE noise figure | FR1: 9 dB | FR1: 9 dB |  |
| BS receiver | MMSE-IRC MMSE-IRC |  |  |
| UE receiver |  |  |  |
| UE speed | 3 km/h |  |  |
| Channel estimation | Realistic |  |  |
| MCS | Up to 256QAM |  |  |
| BS antenna pattern | Ceiling-mount antenna radiation pattern, 5 dBi | 3-sector antenna radiation pattern, 8dBi |  |
| UE antenna pattern | FR1: Omni-directional, 0 dBi, | FR1: Omni-directional, 0 dBi, |  |
| TX power | gNB: FR1: 24dBm/20MHz; | gNB: FR1:44dBm/20MHz |  |
| gNB antenna configuration | gNB: - FR1:32Tx antenna port, (M,N,P,Mg,Ng; Mp,Np)=(4,4,2,1,1;4,4), (dH, dV) = (0.5, 0.5)λ The antenna tilt is 90 degrees. | gNB: - FR1:64 Tx antenna port, (M,N,P,Mg,Ng; Mp,Np)=(8,8,2,1,1;4,8), (dH, dV) = (0.5λ, 0.5λ) The antenna tilt is 12 degrees. |  |
| UE Tx power | Max Tx power: 23 dBm, (P0 = -90, alpha = 1.0) |  | Max Tx power: 23 dBm, (P0 = -74, alpha = 0.6) |
| UE antenna configuration | UE: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ |  |  |
| UE distribution | 100% of users are indoor 3km/h |  | 80% of users are indoor, 20%of users are outdoor |
| Number of UEs per cell | Up to 12 |  | Up to 8 |
| Transmission scheme | Reciprocity-based precoding |  |  |
| Scheduling Algorithm | DL: SU-MIMO with PF |  |  |
| TDD Frame structure | DDDSU (D:10D:2G:2U) |  |  |
| Target BLER | 10% first transmission BLER |  |  |
| HARQ/repetition | 3 HARQ retransmission |  |  |
| Channel estimation | Realistic Channel estimation |  |  |
| CSI acquisition | Realistic, CSI report periodicity 20ms, CSI processing delay is 4ms. CSI quantization |  |  |
| Overhead | 3 symbols per 14 symbol (2 symbol PDCCH+1 symbol DMRS) |  |  |
| UE receiver | MMSE-IRC |  |  |

Table 2: UL Traffic model for AR

|  | Aggregated video stream (for AR) |
| --- | --- |
| Packet Arrival distribution | Periodic (periodicity: 1/60fps) (no jitter) |
| Data rate | 10 Mbps |
| Packet Size distribution | Truncated Gaussian, same parameters as DL |
| Air Interface PDB | 10ms |
| Capacity KPI [X, PDB] | [99%,30ms] (baseline) |
| Jitter | Jitter: same model as for DL |

# Reference

[1]	RP-230786, New WID on XR Enhancements for NR, Nokia, Qualcomm

[2]	RAN1 chair notes, RAN1#112bis-e, April 2023

[3]	3GPP TR 38.835, Study on XR enhancements for NR (Release 18), Dec 2022

[4]	3GPP TS 38.331, Radio Resource Control (RRC) protocol specification (Release 17), v17.3.0, Jan 2023

[5]	R1-2304047, Moderator Summary - XR Specific Capacity Improvements, Moderator (Ericsson), April 2023

[6]	3GPP TS 38.214, Physical layer procedures for data (Release 17), v17.5.0, Mar 2023

[7]	3GPP TS 38.213, Physical layer procedures for control (Release 17), v17.5.0, Mar 2023