3GPP TSG RAN WG1 #113		                                      R1-2305176
Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:	9.7.1

Source:	InterDigital Inc.

Title:	Discussion on techniques in spatial and power domains

Document for:	Discussion and Decision

# Introduction

The Rel-18 New WID on Network energy savings for NR has the following objective [1]:

- Specify the following techniques in spatial and power domains - Specify necessary enhancements on CSI and beam management related procedures including measurement and report, and signaling to enable efficient adaptation of spatial elements (e.g. antenna ports, active transceiver chains) [RAN1, RAN2] - Specify necessary enhancements on CSI related procedures including measurement and report, and signaling to enable efficient adaptation of power offset values between PDSCH and CSI-RS [RAN1, RAN2] - Note: Above objectives are only for UE specific channels/signals - Note: Legacy UE CSI/CSI-RS capabilities applies when considering total number of CSI reports and requirements

This contribution proposes specification enhancements to support network energy saving techniques in spatial and power domains.

# Spatial-domain adaptation

For spatial-domain adaptation, RAN1 agreed to define two situations as “Type 1” and “Type 2” depending on whether all antenna elements, or only a subset of antenna elements that are associated to a logical antenna port are disabled/enabled. With Type 1, spatial adaptation does not modify the measured channel for a given antenna port if it is not disabled, while with Type 2, spatial adaptation may result in a different channel for a given antenna port. For example, Type 2 adaptation may result in wider beam when antenna elements are disabled, which in turn modifies properties of the channel.

CSI-RS resource configuration

RAN1 agreed to support configurability of NZP CSI-RS resource(s) for channel measurement within one resource setting corresponding to more than one spatial adaptation pattern. One or both of following approaches will be supported:

- A1-1-revised: a resource set with multiple resources is configured within a resource setting, where each resource is associated with only one spatial adaptation pattern

- A1-2-revised: For a resource configured in a resource set within a resource setting, the resource can be associated with more than one spatial adaptation patterns

  - One or more resources can be configured in the resource set for channel measurement.

RAN1 took a working assumption both approaches are supported. A1-1-revised is suited to Type 2 adaptation since the resulting multiple of antenna patterns require multiple NZP CSI-RS resources. A1-2-revised is suited to Type 1 adaptation since it supports CSI evaluation for different subset of antenna ports from the same resource. However, there is no need to capture this in specifications.

Proposal 1: Confirm the following working assumption: Al-1-revised and A1-2-revised are supported.

CSI-RS reporting configuration

RAN1 made following agreement in RAN1#112bis-e:

Agreement For CSI report configuration, if L>1 in a CSI report configuration, at least the following can be included for each sub-configuration for Type 1 SD adaptation - N1, N2 for single-panel and N1, N2, Ng for multi-panel - FFS: details on explicit indication or implicit derivation - Port subset indication when A1-2 is used (if A1-2 is supported) - FFS: details on explicit indication or implicit derivation - FFS: rank restriction - FFS: codebook subset restriction - FFS: supported codebook types for PMI, e.g., Type-I or Type-II - FFS: report quantity - FFS: reportFreqConfiguration - FFS: Group identity of NZP CSI-RS resource(s) in a resource set for channel measurement when A1-1 is used For CSI report configuration for type 2 SD adaptation, further study under which cases sub-configurations may or may not be needed including sub-configuration content.

For the port subset indication, explicit indication is preferred for flexibility. Each sub-configuration can be associated with a bitmap (or index to a bitmap, if some combinations are not possible) representing ports that are assumed to be enabled. The multiple port subsets can be configured under a sub-configuration IE within CSI-ReportConfig IE, or under NZP-CSI-RS-Resource IE along with sub-configuration index. The latter approach could be beneficial if sub-configurations are used for more than one CSI-ReportConfig that use the same NZP-CSI-RS resource.

Proposal 2: Support explicit configuration of multiple port subset indications within NZP CSI-RS resource configuration, where each port subset indication is associated to a sub-configuration.

The agreement states that N1, N2, Ng (if applicable) can be separately configured for each sub-configuration. Such parameters configured as part of codebookConfig IE. There are FFS points on whether rank restriction, codebook subset restriction, and supported codebook types for PMI. Separate configuration of N1, N2, (Ng) implies that codebook subset restriction is also separately configured since the latter depends on the former. For rank restriction, supported codebook types, report quantity and reportFreqConfiguration there is no need to support separate configuration. The separate configuration could be realized by configuring a CodebookConfig IE under a sub-configuration IE within CSI-ReportConfig IE, or by configuring multiple sets of parameters within CodebookConfig for the parameters that need to be separately configured. The latter approach is slightly preferred to limit increase of RRC overhead.

Proposal 3: Support configuration of multiple sets of (N1, N2) or (N1, N2, Ng) and codebook subset restriction parameters within a Codebook configuration, where each parameter set is associated to a sub-configuration.

To support beam management with Type 2 adaptation, it may be useful that the UE reports a CRI within a CSI resource set where one CSI resource set corresponds to a set of beams for a certain number of antenna elements turned on. This is feasible if a NZP CSI-RS resource set is associated to a sub-configuration.

Proposal 4: For Type 2 adaptation, support association of a NZP CSI-RS resource set to a sub-configuration.

Adaptation of spatial pattern

For aperiodic or semi-persistent CSI reporting on PUSCH, each trigger state can be configured with a subset of L sub-configurations for which CSI is to be reported.

Proposal 5: Support configuration of a subset of sub-configurations for each aperiodic trigger state or semi-persistent-on-PUSCH trigger state.

For periodic and semi-persistent CSI reporting on PUCCH, available payload on PUCCH is more limited and reporting for many spatial adaptation patterns in a single report may not be practical. Therefore, it would be useful to support dynamic indication of applicable sub-configuration(s) to allow the network to obtain these reports for a spatial adaptation state that is most relevant at a given time. It would be particularly beneficial to provide this information using UE-group common signaling to minimize overhead.

Proposal 6: A DCI can indicate sub-configuration(s) applicable to a periodic CSI or semi-persistent on PUCCH reporting configuration.

Proposal 7: The DCI indicating sub-configuration is received in a UE-group common search space.

PDCCH/PDSCH reception with Type 2 adaptation

When the network uses Type 2 spatial adaptation, different spatial adaptation patterns are expected to result in different beam widths, and therefore different channels from UE perspective. A change of spatial pattern may then have impact on channel estimation for PDCCH/PDSCH reception, similar to a change of beam. Such change of beam is normally handled by indicating a new TCI state for PDCCH and PDSCH reception where the new TCI state includes the CSI-RS corresponding to the new beam as QCL information. This implies that the required number of configured TCI states in a cell for which Type 2 spatial adaptation is utilized can increase significantly. In practice, at any given time the network would only need to use TCI states corresponding to a subset of all possible spatial patterns, corresponding to which energy saving mode it is operating in. Every time the network switches to a different energy saving mode and uses a different set of spatial patterns, it will need to activate a different subset of TCI states for each UE.

It may be beneficial to consider solutions that would reduce the above signaling overhead. For example, one could support groups of TCI states corresponding to different energy saving modes and activate a group of TCI states upon reception of signaling indicating a change of energy saving mode at cell level (such as a Cell DTX/DRX indication, if supported). Another possibility that would not require increase of the number of TCI states would be to support additional QCL sources within each TCI state, where each QCL source corresponds to a certain energy saving mode. In such case, no activation of TCI states is required when the UE is signaled or detects a change of energy saving mode or spatial adaptation.

Proposal 8: RAN1 to consider solutions reducing signaling overhead from changes of TCI states when Type 2 adaptation is used.

# Power-domain adaptation

In RAN1#112bis-e, RAN1 made following agreements on power-domain adaptation:

Agreement For power domain adaptation, for CSI(s) reporting, support configuration of more than one power offset values for PDSCH relative to CSI-RS - FFS: impact on CSI processing requirement - FFS: details on configuration/indication of the power offset values - FFS: whether/how to additionally consider the case where CSI-RS power is changed Agreement For power domain adaptation, support the following configuration(s) for CSI-RS resource configuration, - A1-2-power: one or more resources can be configured in a resource set within a resource setting and each resource can be associated with one or more power offset values - FFS: A1-1-power: a resource set with multiple resources is configured within a resource setting, where resources can have different power offset values - FFS: Details of how the different power offset values(s) are configured/indicated.

The above agreement to support A1-2-power can be realized by configuring a set of power offset values within a NZP-CSI-RS-Resource configuration instead of a single value as in current specification.

Proposal 9: Support configuration of multiple power offset values within NZP CSI-RS configuration.

One possibility for the indication is to configure which power offset(s) is/are applicable for a certain aperiodic trigger state or a certain periodic/semi-persistent on PUCCH reporting configuration. However, this approach can incur a lot of overhead, both in terms of number of required states and in terms of CSI reporting overhead with a larger number of power offset value(s) and/or when combined with spatial-domain adaptation. In practice, only one or two power offset values may be relevant at a given point in time especially for the case of periodic CSI reporting.

For this reason, it is recommended to adopt a mechanism that supports updating the power offset(s) applicable to a NZP CSI-RS resource that can be used in subsequent CSI reports, at least for CSI reporting on PUCCH. It is envisioned that a power offset update is relevant to multiple UEs since it is typically linked to on-going setting of PDSCH transmission power. Therefore, UE-group signaling seems more attractive to reduce overhead.

Proposal 10: Support signaling to dynamically update power offset(s) applicable to a NZP CSI-RS resource.

Dynamic update of power offset value

To dynamically update power offset value(s), one possible realization involves adding a group identity IE as part of the configuration of a NZP CSI-RS resource. The dynamic signaling then can include an adjustment of power offset and the group identity of applicable NZP CSI-RS resources. The maximum number of groups could be relatively small, e.g. 8 or 16. This enables dynamic signaling with low overhead.

Proposal 11: RRC configures a group identity for the purpose of power offset adjustment for each NZP CSI-RS resource.

Proposal 12: A DCI can indicate subset of configured values for power offset between PDSCH and CSI-RS applicable to a group of NZP CSI-RS resources for CSI reporting.

Several options can be considered for the dynamic signaling such as MAC CE, unicast DCI or UE-group common DCI. As network energy saving gains by power and/or spatial adaptation are feasible even when there is more than a few UEs connected to the cell, a group signaling approach seems attractive. This approach is feasible when a group identity is assigned to each NZP CSI-RS resource in UE-specific manner. The UE-group common DCI can support indications of  power offsets for one or more groups of resources.

Proposal 13: The DCI indicating subset of power offset value(s) is received in a UE-group common search space.

Other CSI report content

If a power offset adjustment is signaled by DCI, there is possibility of mismatch between UE and network in case the UE misses the DCI. The mismatch can be avoided if the CSI report includes an indication of the power offset adjustment assumed by the UE for the reported CSI. The additional overhead depends on the number of possible adjustment values, which should typically be quite small.

Proposal 14: CSI report contains indication of assumed power offset adjustment.

# Conclusion

In this contribution, the following proposals are made:

Proposal 1: Confirm the following working assumption: Al-1-revised and A1-2-revised are supported.

Proposal 2: Support explicit configuration of multiple port subset indications within NZP CSI-RS resource configuration, where each port subset indication is associated to a sub-configuration.

Proposal 3: Support configuration of multiple sets of (N1, N2) or (N1, N2, Ng) and codebook subset restriction parameters within a Codebook configuration, where each parameter set is associated to a sub-configuration.

Proposal 4: For Type 2 adaptation, support association of a NZP CSI-RS resource set to a sub-configuration.

Proposal 5: Support configuration of a subset of sub-configurations for each aperiodic trigger state or semi-persistent-on-PUSCH trigger state.

Proposal 6: A DCI can indicate sub-configuration(s) applicable to a periodic CSI or semi-persistent on PUCCH reporting configuration.

Proposal 7: The DCI indicating sub-configuration is received in a UE-group common search space.

Proposal 8: RAN1 to consider solutions reducing signaling overhead from changes of TCI states when Type 2 adaptation is used.

Proposal 9: Support configuration of multiple power offset values within NZP CSI-RS configuration.

Proposal 10: Support signaling to dynamically update power offset(s) applicable to a NZP CSI-RS resource.

Proposal 11: RRC configures a group identity for the purpose of power offset adjustment for each NZP CSI-RS resource.

Proposal 12: A DCI can indicate subset of configured values for power offset between PDSCH and CSI-RS applicable to a group of NZP CSI-RS resources for CSI reporting.

Proposal 13: The DCI indicating subset of power offset value(s) is received in a UE-group common search space.

Proposal 14: CSI report contains indication of assumed power offset adjustment.

# References

RP-223540, New WID: Network energy savings for NR, Huawei

TR 38.864, Study on network energy savings for NR, v18.0.0, Dec 2022

# Appendix: previous agreements

RAN1#112bis-e

R1-2303913	FL summary#1 for spatial and power domain techniques for R18 NES	Moderator (Huawei)

Agreement

Define necessary enhancements to support both types of spatial adaptation cases (as defined in RAN1#112) in Rel-18.

- Note: This does not imply explicit definition in specifications for adaptation types.

- Note: This does not imply explicit specification changes are made for both cases

Agreement

Support configurability of NZP CSI-RS resource(s) for channel measurement within one resource setting corresponding to more than one spatial adaptation patterns with at least one of the following

- A1-1-revised: a resource set with multiple resources is configured within a resource setting, where each resource is associated with only one spatial adaptation pattern

- A1-2-revised: For a resource configured in a resource set within a resource setting, the resource can be associated with more than one spatial adaptation patterns

  - One or more resources can be configured in the resource set for channel measurement.

R1-2303914	FL summary#2 for spatial and power domain techniques for R18 NES	Moderator (Huawei)

Agreement

At least support A2-2, i.e. one CSI report configuration contains multiple CSI report sub-configurations where each sub-configuration corresponds to one spatial adaptation pattern.

- FFS: impact on CSI processing requirement

Agreement

For a CSI report config with L sub-configuration(s), support a framework that enables a UE to report N CSI(s) in one reporting instance where the N CSI(s) are associated with N sub-configuration(s) from L (where 1≤N≤L) and each CSI corresponds to one sub-configuration.

- For discussion purpose, N=1 refers to single-CSI while N>1 refers to multi-CSI.

- For Semi-persistent/Aperiodic CSI reporting, support gNB trigger/indicate/activate report of N≤L CSIs where N>=1

- The maximum value of N and L are subject to UE capability

- Further study how to address/minimize additional UE complexity

The following bullet was not agreed due to objection from Apple and vivo

- For Periodic CSI reporting, at least the case of N=L is supported where N>=1

Conclusion

New CSI-RS resource (RE mapping) pattern is not introduced for R18 network energy savings purpose.

- Note: CSI-RS resource (RE mapping) pattern above refers to a row in TS 38.211 Table 7.4.1.5.3-1 determining CSI-RS locations within a slot.

Agreement

For power domain adaptation, for CSI(s) reporting, support configuration of more than one power offset values for PDSCH relative to CSI-RS

- FFS: impact on CSI processing requirement

- FFS: details on configuration/indication of the power offset values

- FFS: whether/how to additionally consider the case where CSI-RS power is changed

R1-2303915	FL summary#3 for spatial and power domain techniques for R18 NES	Moderator (Huawei)

Agreement

- For CSI feedback with CSI overhead/report payload reduction, further study whether/how to report a common value and/or a differential and/or joint coded value across same CSI quantity of different sub-configurations/adaptation patterns, at least for the following

  - CRI

  - RI

  - PMI

  - CQI

  - FFS: L1-RSRP

  - Other (new) report quantity, if any

- Further study whether/how it is feasible/possible for the UE to skip the evaluations of some sub-configurations/adaptation patterns to reduce the burden at the UE

R1-2303916	FL summary#4 for spatial and power domain techniques for R18 NES	Moderator (Huawei)

Agreement

For CSI report configuration, if L>1 in a CSI report configuration, at least the following can be included for each sub-configuration for Type 1 SD adaptation

- N1, N2 for single-panel and N1, N2, Ng for multi-panel

  - FFS: details on explicit indication or implicit derivation

- Port subset indication when A1-2 is used (if A1-2 is supported)

  - FFS: details on explicit indication or implicit derivation

- FFS: rank restriction

- FFS: codebook subset restriction

- FFS: supported codebook types for PMI, e.g., Type-I or Type-II

- FFS: report quantity

- FFS: reportFreqConfiguration

- FFS: Group identity of NZP CSI-RS resource(s) in a resource set for channel measurement when A1-1 is used

For CSI report configuration for type 2 SD adaptation, further study under which cases sub-configurations may or may not be needed including sub-configuration content

Agreement

For power domain adaptation, support the following configuration(s) for CSI-RS resource configuration,

- A1-2-power: one or more resources can be configured in a resource set within a resource setting and each resource can be associated with one or more power offset values

- FFS: A1-1-power: a resource set with multiple resources is configured within a resource setting, where resources can have different power offset values

- FFS: Details of how the different power offset values(s) are configured/indicated.

R1-2303917	FL summary#5 for spatial and power domain techniques for R18 NES	Moderator (Huawei)

Working Assumption

Al-1-revised and A1-2-revised are supported

- FFS: Which Type of SD adaptation A1-1-revised and A1-2-revised are applicable for

Agreement

For R18 NES, only legacy port configuration values (N1, N2) or (Ng, N1, N2) are supported.

- FFS: Whether/what restriction for A1-1-revised and A-1-2-revised w.r.t number of ports

Agreement

For Semi-persistent/Aperiodic CSI reporting with 1≤N≤L, study what enhancements to the current DCI and MAC-CE mechanisms are needed for gNB triggering/indication/activation of the N CSI(s) in a reporting instance, where the N CSI(s) are associated with N sub-configuration(s) from L in a report config.

Conclusion

From RAN1 perspective, there is no action needed for the LS R1-2302288 from SA5 this time.

RAN1#112

R1-2301964	FL summary#1 for spatial and power domain techniques for R18 NES	Moderator (Huawei)

Agreement

For the purpose of further discussions in RAN1 on NES spatial domain adaptations, consider the following cases

- Type 1: all antenna elements associated to a logical antenna port is disabled/enabled

- Type 2: part/subset of antenna elements associated to a logical antenna port is disabled/enabled

R1-2301965	FL summary#2 for spatial and power domain techniques for R18 NES	Moderator (Huawei)

Agreement

For spatial element adaptation, further study the following

  - A1-1) Each CSI-RS resource/resource set/resource setting can be associated with only one spatial adaptation pattern

    - FFS: Details on how the association is done

  - A1-2) Each CSI-RS resource/resource set/resource setting can be associated with one or more spatial adaptation patterns

    - FFS: Details on how the association is done

  - FFS: Details on the definition of “spatial adaptation patterns”

Agreement

For spatial element adaptation, further study the following

  - A2-1) Independent/separate CSI report configurations where each CSI report configuration corresponds to one spatial adaptation pattern

  - A2-2) One CSI report configuration contains multiple CSI report sub-configurations where each sub-configuration corresponds to one spatial adaptation pattern

    - FFS: Details of sub-configuration

R1-2301966	FL summary#3 for spatial and power domain techniques for R18 NES	Moderator (Huawei)

Agreement

For spatial domain adaptation, further study necessary enhancements for multiple CSI(s) where each CSI corresponds to a spatial adaptation pattern, e.g.

  - FFS: gNB indicates to UE which CSI(s) the UE shall report

  - FFS: the UE selects which CSI(s) are reported

  - FFS: multiple CSI(s) are reported in a joint CSI report

  - FFS: Overhead reduction for multiple CSI(s)

Note: UE complexity needs to be taken into account.

Agreement

For adaptation of power offset values between PDSCH and CSI-RS, further study the following

  - Where/how to configure multiple power offset values

    - Whether/how one or more power offset values are dynamically indicated to UE for CSI measurement/reporting, and PDSCH reception

    - Overhead reduction for CSI reports associated with multiple power offset values between PDSCH and CSI-RS

    - Whether other UE report content can be included

Agreement

For spatial and power domain adaptation, solution(s) based on adaptation within an active BWP is considered as baseline

Agreement

Discuss the signalling aspects for spatial/power domain adaptation for Rel-18 NES-capable UEs considering that

  - Whether there is a need for transition time per adaptation (for UE)

  - Whether/How to inform UE on spatial adaptation pattern update and/or PDSCH/CSI-RS transmission power change due to adaptation.