3GPP TSG RAN WG1 #113			R1-2305190

Incheon, Korea, May 22nd – May 26th, 2023

Agenda item:	9.12.1

Source:                Charter Communications Inc.

Title:                     PRACH coverage enhancements

Document for:	Discussion

# Introduction

In the RAN1#112b-e meeting [1], the following agreements, conclusions and working assumptions regarding further NR coverage enhancement on PRACH repetition were made.

Agreement

For multiple PRACH transmissions with same Tx beam, gNB can configure one or multiple values for the number of multiple PRACH transmissions.

- If multiple values are configured, PRACH resources differentiation between multiple PRACH transmissions with different number of multiple PRACH transmissions is supported.

- FFS: details

Working Assumption

For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, at least support that multiple PRACH are transmitted on separate ROs.

- Note: Separate RO means that the RO is separated with single PRACH transmission.

- FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning.

Working Assumption

For multiple PRACH transmissions with same Tx beam, to differentiate the multiple PRACH transmissions with single PRACH transmission, support that multiple PRACH are transmitted with separate preamble on shared ROs.

- Note: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission.

- FFS: whether Rel-17 framework of feature combination (FeatureCombination-r17) and additional RACH configuration (AdditionalRACH-Config-r17) can be reused for Rel-18 multiple PRACH transmissions to realize the corresponding PRACH resource partitioning.

Conclusion

For multiple PRACH transmissions within one RACH attempt, they are only transmitted over ROs associated with the same SSB/CSI-RS.

Note: This applies for multiple PRACH transmissions with same Tx beam, and also applies for multiple PRACH transmissions with different Tx beam (if supported).

Agreement

For multiple PRACH transmissions with same Tx beam in one RACH attempt, transmission power ramping is not applied within one RACH attempt.

Agreement

For multiple PRACH transmissions with same Tx beam, only one RAR window is supported for RAR monitoring for one RACH attempt.

  - FFS: the start position of the RAR window.

  - FFS: RA-RNTI.

Agreement

For multiple PRACH transmissions with same Tx beam, "RO group" is assumed for multiple PRACH transmissions with separate preamble on shared ROs and/or multiple PRACH transmissions on separate ROs, and one RO group consists of valid RO(s) for a specific number of multiple PRACH transmissions.

Note 1: All ROs in one RO group is associated with the same SSB(s).

Note 2: Shared or separate RO/preamble means that the RO/preamble is shared or separated with single PRACH transmission.

Note 3: whether/how to define “RO group” in specification will be discussed separately

Note 4: Valid RO(s) refers to what is defined in existing specification

FFS: whether and how to address collision between valid ROs for multiple PRACH transmissions and other existing ROs for legacy single PRACH transmission or other features, e.g., 2-step RACH.

FFS: the time span of RO group.

FFS: whether and how ROs can be shared between different RO groups for different number of multiple PRACH transmissions.

FFS: other details

Agreement

Support {2, 4, 8} for the number of multiple PRACH transmissions with same Tx beams.

# Discussion

- RO Group

RO Group Definition

An RO group consists of multiple valid ROs for an associated SSB. According to the agreement in RAN1#112 meeting [2], the number of valid ROs in a RO group is 2, 4, or 8. Further, these ROs are TDMed and can be selected consecutively or non-consecutively. Ax example of RO group with 4 ROs has been shown in Figure 1. A consecutive selection of the ROs reduces the latency and makes RO scheduling straightforward. On the other hand, a non-consecutive selection can overcome deep fading which improves the performance of non-coherent combining of the received signals at gNB. Moreover, a non-consecutive RO scheduling adds more degree of freedom at the cost of higher latency. For a non-consecutive RO selection, the time gap between the ROs could be same/different RO group sizes

Valid RO

Invalid RO

(a) An RO group with consecutively selected ROs

Selected RO

(b) An RO group with non-consecutively selected ROs

Figure 1: Illustration of an RO group for 4 PRACH repetition.

Observation 1: An RO group consists of multiple valid ROs which are selected consecutively or non-consecutively. The consecutive ROs have low latency and a straightforward selection. The non-consecutive ROs can achieve better performance and adds degree of freedom at the cost of higher latency.

Proposal 1: In an RO group all ROs are associated with an SSB. The following two options for an RO group is considered.

- An RO group consists of multiple valid ROs where the ROs are selected consecutively.

- An RO group consists of multiple valid ROs where the ROs are selected non-consecutively. The time gap between the ROs could be different for same/different RO group sizes.

RO Group Configuration

In the legacy RACH, there are only single ROs available for UEs to transmit their PRACH signal. In Rel-18, UEs have two options for PRACH transmission single ROs and newly defined RO groups. In particular, each RO group consists of different numbers of ROs. Based on the agreement in RAN1#112 meeting [2], there will be three different RO group sizes which are RO group with 2 ROs, RO group with 4 ROs, and RO group with 8 ROs. Each RO group is associated with an SSB at least one time. One of the primary challenges is RO groups configuration. That is, it is not clear how the locations of different RO groups associated with an SSB are determined. In this regard, three possible configurations are depicted in Figure 2. Figure 2(a) shows that the RO groups are separated in time while can share same preambles. In this configuration, each RO group has a different starting time position and UEs may experience different latency when they want to use different RO groups for PRACH transmission.

Proposal 2: Different RO groups associated with a same SSB are TDMed with shared preambles.

In the second configuration in Figure 2(b), all RO groups are fully overlapped and their starting time position is same. In RAN1 #110b-e [3], it is agreed that same preamble is used for multiple PRACH transmission in an RO group. However, using separate or shared preamble set for different RO groups associated with a same SSB has not yet been discussed. If a preamble set is shared among the overlapped RO groups shown in Figure 2(b), the preamble detection performance would be degraded. This is because the transmitted PRACH signals generated from same preamble within shared ROs will be added destructively at gNB. Thus, the PRACH signal due to small-size RO groups will be eliminated. To avoid this, overlapped RO groups with a shared preamble set can use OCCs or any other method such that the PRACH signals due to different RO groups are separable. Otherwise, overlapped RO groups with shared preamble set should not be supported.

Proposal 3: If a shared preamble set is used for overlapped RO groups for an associated SSB, at least using OCCs to separate different PRACH signals due to different RO groups is supported. Otherwise, a shared preamble set is not supported for overlapped RO groups with different sizes associated with a same SSB.

In contrast, if UEs are allowed to select a preamble from a separate set for different RO groups associated with an SSB, the overlapped RO groups can be used only in CBRA. This configuration is not feasible in CFRA where UE is allocated a specific preamble and cannot select different preambles for different RO groups.

Observation 2: If UEs are allowed to select preamble from a separate set for different RO groups associated with an SSB, the overlapped RO groups can be used only in CBRA.

Proposal 4: Separate preambles for overlapped RO groups with different sizes associated with a same SSB is supported only for CBRA.

The third option is FDMed RO groups shown by Figure 2(c). All the groups have the same starting position in time domain but different starting position in frequency domain with shared preambles. The main advantage of this configuration over the TDMed one is that different RO groups are available for UEs at the same time. Thus, UEs do not need to wait until transmission time for a specific RO group arrives which results in a low latency. Although, the implementation of the third configuration requires gNB to simultaneously generate three beams with different frequencies toward the same direction. To do this, at least three RF chains are required for every SSB.

- TDMed RO groups with shared preambles

- Overlapped RO groups.

- FDMed RO groups with shared preambles.

Figure 2: Three different configurations of RO groups with different sizes associated with one SSB.

Observation 3: The FDMed RO groups configuration associated with a same SSB has lower latency compared to the TDMed RO groups.

Observation 4: For the FDMed RO groups associated with a same SSB, at least three RF chains per SSB are needed at gNB in order to receive the different RO group signals.

Proposal 5: Different RO groups associated with a same SSB are FDMed with shared preambles is supported.

SSB-to-RO Group Mapping

The rules for SSB-to-RO mapping for single ROs can be extended to SSB-to-RO group mapping. Before discussing an SSB-to-RO group mapping for a large number of SSBs, mapping rules of two RO groups associated with two different SSBs are explained as follows.

- Separate RO groups with shared preambles

RO groups associated with two different SSBs can be TDMed or FDMed with shared preambles as shown by Figure 3 where it is assumed that the blue-color RO group is associated with SSB 0 and the green-color RO group is associated with SSB1. For separate RO groups in time domain two cases are possible as depicted in Figures 3(a) and (b). The mapping in figure 3(a) is a straightforward extension of the existing SSB-to-RO mapping. In Figure 3(b), the ROs within each RO group are selected non-consecutively. The first two ROs belonging to the blue-color RO group is associated with SSB0. The second two ROs belonging to the green-color RO group is associated with SSB1. The same mapping rule holds for the remaining ROs. This type of mapping is not common in the current SSB-to-RO mapping in which each single RO cannot be partitioned.

Another SSB-to-RO group mapping is that the two RO groups are FDMed while share the same preambles as shown by Figures 3(c) and (d). The mapping rule in Figure 3(c) is the generalization of the mapping rule for two single FDMed ROs. In Figure 3(d), the ROs belonging to the same RO group use intra-group frequency hopping. In particular, the first two ROs of the blue-color RO groups are FDMed with the first two ROs of the green-color RO group and so on.

In summary, the RO group mapping rule can be exactly same as the existing mapping rule as shown in Figures 3(a) and (c). On the other hand, the RO group mapping can be done by new rules as shown in Figures 3(b) and (d) which add more degree of freedom and flexibility.

Figure 3: SSB-to-RO group mapping for two RO groups associated with two different SSBs. (a) Two ROs are TDMed where the ROs within each RO group are selected consecutively. (b) Two ROs are TDMed where the ROs within each RO group are selected non-consecutively. (c) Two RO groups associated with different SSBs are FDMed where the ROs within each RO group are selected consecutively. (d) Two RO groups associated with different SSBs are FDMed where frequency hopping is applied for the ROs within each RO group.

- Shared RO groups with separate preambles

RO groups associated with different SSBs can fully or partially share their ROs with separate preambles. In order to elaborate on this type of mapping, the two RO groups mentioned above are considered. In Figure 4(a), the two RO groups fully share their ROs. That is all the RACH occasions are associated with both SSBs. This mapping rule is similar to the mapping of two SSBs to a single RO. In the second mapping scheme in Figure 4(b), only some of the ROs are shared between the two RO groups which adds more degree of freedom to SSB-to-RO group mapping.

- (b)

Figure 4: Two SSB-to-RO group mappings (a) with fully shared ROs (b) with partially shared ROs.

Observation 5: New mapping rules such as FDMed RO groups with intra-group frequency hopping and partially overlapped RO groups can add more degree of freedom and flexibility.

Proposal 6:

- When multiple RO groups associated with different SSBs are FDMed, intra-group frequency hopping is supported.

- Fully and partially overlapped RO groups associated with different SSBs are supported.

UEs determine the number of PRACH repetitions at least based on the measured SSB-RSRP [1]. In general, a high SSB-RSRP indicates that a single PRACH or a small number of PRACH repetitions is sufficient for a successful PRACH transmission. The number of repetitions after each PRACH failure is expected to increase from 2 to 4, and then to 8.  Therefore, unlike the legacy single ROs, different RO groups for a same associated SSB do not have the same chance to be used by UEs. Particularly, it is expected that an RO group with 2 ROs to be used more often than an RO group with 4 ROs and so on. Therefore, association of an RO group with 2 ROs with multiple SSBs increases the probability of collision. Inspired by this observation, RO groups with 2 ROs should be associated with a small number of SSBs. Whereas, RO groups with 4 and 8 ROs can be associated with a few SSBs.

Using the aforementioned SSB-to-RO group mapping rules and the characteristics of the RO groups with different sizes, a general SSB-to-RO group mapping for 8 SSBs is illustrated in Figure 5. Only one SSB is mapped to the RO groups with 2 ROs. The RO groups with 4 ROs are associated with 2 SSBs and the RO groups with 8 ROs are associated with 4 SSBs. Similar to the legacy mapping, first the SSBs are mapped in frequency domain and then in the time domain. Further, the different RO groups are separated in time domain with shared preambles. Those RO groups associated with more than one SSB either can separate the preambles or can apply a proper OCC method such that their PRACH signals is separated at gNB. Also, the intra-group frequency hopping illustrated in Figure 3(d) can be implemented to improve the performance.

Figure 5. an SSB-to-RO group mapping where the number of SSBs is 8.

Observation 6: Unlike the legacy single ROs, different RO groups for a same associated SSB do not have the same chance to be used by UEs. Particularly, it is expected that an RO group with 2 ROs to be used more often than an RO group with 4 ROs and so on.

Proposal 7: The following options are supported for SSB-to-RO group mapping.

- Different RO groups are associated with different number of SSBs.

- SSBs first are mapped to FDMed RO groups and then are mapped to TDMed RO groups.

  - Power Ramping

The main idea behind the multiple PRACH repetition is to improve the coverage without increasing UE’s maximum power level. In particular, after multiple PRACH re-transmission when UE’s power reaches to the maximum level, power ramping is not possible. To further enhance the coverage, multiple PRACH transmission is utilized. Thus, power ramping and multiple PRACH repetition are two important parameters that should be considered together in the PRACH coverage enhancement.

The following two options are considered for PRACH retransmission.

- Increasing number of PRACH repetitions before power ramping

The impact of multiple PRACH repetition on the coverage enhancement has been evaluated by different companies. The results demonstrate as the number of PRACH repetitions increases the coverage performance improves both in FR1 and FR2. Taking these results into account, one option for PRACH retransmission is to first increase the number of PRACH repetitions. If the number of PRACH repetition reaches to the maximum value, UE transmit power increases for the next PRACH transmission attempt. Figure 6 represents an example of the underlaying PRACH retransmission option. One of the drawbacks of this option is that by increasing the number of PRACH repetitions the coverage performance improves at the cost of using many ROs. As shown by Figure 6, after the third attempt, the number of PRACH repetitions becomes 8. This means after each failure, for the next attempt, UE should transmit 8 PRACH signals within 8 ROs. Due to the limited number of ROs, using the maximum number of PRACH repetitions after few failed attempts is inefficient.

Observation 7: Increasing the number of PRACH repetitions before power ramping is an inefficient way of using the available limited number of ROs.

Proposal 8: Increasing number of PRACH repetitions before power ramping for PRACH retransmission attempts is not supported.

Figure 6: Representation of PRACH retransmission attempts via increasing number of PRACH repetitions before power ramping.

- Power ramping before increasing the number of PRACH repetitions

In this option, after a failed PRACH transmission attempt, UE retransmits the PRACH signal with a higher power where the power level is calculated using powerRampingStep. The procedure continues until the power reaches to the maximum level. Next, if failure occurs, the number of PRACH repetitions increases accordingly. This procedure repeats until the number of retransmissions reaches to the maximum determined by preambleTransMax. In Figure 7, an example of power ramping before increasing the number of PRACH repetitions is depicted. It is observed that power ramping prior to increasing the number of PRACH repetition results in using smaller number of ROs.

Figure 7: Representation of PRACH retransmission attempts using power ramping before increasing the number of PRACH repetitions.

Proposal 9: For PRACH retransmission attempt first power ramping is applied. After the power reaches to the maximum level the number of PRACH repetitions increases.

# Summary

Our observations and proposals on PRACH coverage enhancements in this contribution are summarized as follows.

Observation 1: An RO group consists of multiple valid ROs which are selected consecutively or non-consecutively. The consecutive ROs have low latency and a straightforward selection. The non-consecutive ROs can achieve better performance and adds degree of freedom at the cost of higher latency.

Observation 2: If UEs are allowed to select preamble from a separate set for different RO groups associated with an SSB, the overlapped RO groups can be used only in CBRA.

Observation 3: The FDMed RO groups configuration associated with a same SSB has lower latency compared to the TDMed RO groups.

Observation 4: For the FDMed RO groups associated with a same SSB, at least three RF chains per SSB are needed at gNB in order to receive the different RO group signals.

Observation 5: New mapping rules such as FDMed RO groups with intra-group frequency hopping and partially overlapped RO groups can add more degree of freedom and flexibility.

Observation 6: Unlike the legacy single ROs, different RO groups for a same associated SSB do not have the same chance to be used by UEs. Particularly, it is expected that an RO group with 2 ROs to be used more often than an RO group with 4 ROs and so on.

Observation 7: Increasing the number of PRACH repetitions before power ramping is an inefficient way of using the available limited number of ROs.

Proposal 1: In an RO group all ROs are associated with an SSB. The following two options for an RO group is considered.

- An RO group consists of multiple valid ROs where the ROs are selected consecutively.

- An RO group consists of multiple valid ROs where the ROs are selected non-consecutively. The time gap between the ROs could be different for same/different RO group sizes.

Proposal 2: Different RO groups associated with a same SSB are TDMed with shared preambles.

Proposal 3: If a shared preamble set is used for overlapped RO groups for an associated SSB, at least using OCCs to separate different PRACH signals due to different RO groups is supported. Otherwise, a shared preamble set is not supported for overlapped RO groups with different sizes associated with a same SSB.

Proposal 4: Separate preambles for overlapped RO groups with different sizes associated with a same SSB is supported only for CBRA.

Proposal 5: Different RO groups associated with a same SSB is FDMed with shared preambles is supported.

Proposal 6:

- When multiple RO groups associated with different SSBs are FDMed, intra-group frequency hopping is supported.

- Fully and partially overlapped RO groups associated with different SSBs are supported.

Proposal 7: The following options are supported for SSB-to-RO group mapping.

- Different RO groups are associated with different number of SSBs.

- SSBs first are mapped to FDMed RO groups and then are mapped to TDMed RO groups.

Proposal 8: Increasing number of PRACH repetitions before power ramping for PRACH retransmission attempts is not supported.

Proposal 9: For PRACH retransmission attempt first power ramping is applied. After the power reaches to the maximum level the number of PRACH repetitions increases.

# References

- Chairman’s notes, RAN1#112b-e Meeting, April 2023

- Chairman’s notes, RAN1#112 Meeting, March 2023

- Chairman’s notes, RAN1#110b-e Meeting, October 2022