3GPP TSG RAN WG1 #113	R1- 2305197

Incheon, South Korea, May 22nd - May 26th, 2023

Agenda Item:	9.2.1

Source:	Fraunhofer IIS, Fraunhofer HHI

Title:	General aspects of AI/ML framework

Document for:	Discussion & Decision

# Introduction

Part of the objective of the SID in RP-213599 [1] on study of the 3GPP framework for AI/ML for air-interface with regards to potential specification impact consists of:

… - Assess potential specification impact, specifically for the agreed use cases in the final representative set and for a common framework: - PHY layer aspects, e.g., (RAN1) - Consider aspects related to, e.g., the potential specification of the AI Model lifecycle management, and dataset construction for training, validation and test for the selected use cases - Use case and collaboration level specific specification impact, such as new signalling, means for training and validation data assistance, assistance information, measurement, and feedback - Protocol aspects, e.g., (RAN2) - RAN2 only starts the work after there is sufficient progress on the use case study in RAN1 - Consider aspects related to, e.g., capability indication, configuration and control procedures (training/inference), and management of data and AI/ML model, per RAN1 input - Collaboration level specific specification impact per use case - Interoperability and testability aspects, e.g., (RAN4) - RAN4 only starts the work after there is sufficient progress on use case study in RAN1 and RAN2 - Requirements and testing frameworks to validate AI/ML based performance enhancements and ensuring that UE and gNB with AI/ML meet or exceed the existing minimum requirements if applicable - Consider the need and implications for AI/ML processing capabilities definition Note 1: specific AI/ML models are not expected to be specified and are left to implementation. User data privacy needs to be preserved. Note 2: The study on AI/ML for air interface is based on the current RAN architecture and new interfaces shall not be introduced.

In this contribution, we provide our views on Terminology and AI/ML model monitoring.

# Terminology

## Reinforcement Learning

Current definition (RAN1#109e)

Reinforcement Learning (RL): A process of training an AI/ML model from input (a.k.a. state) and a feedback signal (a.k.a.  reward) resulting from the model’s output (a.k.a. action) in an environment the model is interacting with.

The Reinforcement Learning (RL) research field is quite broad [4]:

- In “classical” RL and multi-armed bandits algorithms, an agent (AI/ML model) interacts with an environment and through training learns to improve (i.e., select outputs/actions that maximize the expected cumulative reward).

- In model-based RL, the agent learns a model of the environment using data collected during training and uses this model for online planning. In cases where a model of the environment is known, but its dimensionality hinders online planning in finite time, tree-based planners are utilized.

- In Imitation Learning, an agent learns to mimic the demonstrated behaviour (selected actions in the observed states) either utilizing offline, logged data or by querying the expert interactively.

- In Inverse RL, an agent is tasked to examine offline, logged data from several (expert and non-expert) explicit demonstrations or interactions during nominal operation to approximate the underlying reward function. This reward function is in turn utilized to train an agent to solve the underlying application.

- In Offline RL, the reward function is available, and the agent utilizes offline, logged data from the environment containing previous (expert and non-expert) interactions during nominal operation, aiming at learning a behaviour that achieves better performance from the best performing interactions in the dataset.

It is clear from the discussion that RL algorithms are not limited to only interacting with the environment. Hence, we would propose the following definition:

Proposal 1: Change the Reinforcement Learning (RL) Definition as follows:

Reinforcement Learning (RL): A process of training an AI/ML model (policy) to interact with an environment and take actions (model’s output) based on the environment’s current state (model’s input), with the goal of maximizing the expected cumulative reward (feedback signal). For the AI/ML model (policy) training, direct interaction with the environment, available logged data from the environment, or a combination of both can be used.

# AI/ML monitoring

The following agreements that are relevant to AI/ML monitoring were made in RAN1:

RAN1 #110-bis-e Agreement Study AI/ML model monitoring for at least the following purposes: model activation, deactivation, selection, switching, fallback, and update (including re-training). FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined) RAN1 #110-bis-e Agreement For model selection, activation, deactivation, switching, and fallback at least for UE sided models and two-sided models, study the following mechanisms: - Decision by the network - Network-initiated - UE-initiated, requested to the network - Decision by the UE - Event-triggered as configured by the network, UE’s decision is reported to network - UE-autonomous, UE’s decision is reported to the network - UE-autonomous, UE’s decision is not reported to the network FFS: for network sided models RAN1 #110-bis-e Agreement Study at least the following metrics/methods for AI/ML model monitoring in lifecycle management per use case: - Monitoring based on inference accuracy, including metrics related to intermediate KPIs - Monitoring based on system performance, including metrics related to system peformance KPIs - Other monitoring solutions, at least following 2 options. - Monitoring based on data distribution - Input-based: e.g., Monitoring the validity of the AI/ML input, e.g., out-of-distribution detection, drift detection of input data, or something simple like checking SNR, delay spread, etc. - Output-based: e.g., drift detection of output data - Monitoring based on applicable condition Note: Model monitoring metric calculation may be done at NW or UE RAN1 #110-bis-e Agreement Study the specification impact to support multiple AI models for the same functionality, at least including the following aspects: - Procedure and assistance signaling for the AI model switching and/or selection FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined) RAN1#110 Agreement In CSI compression using two-sided model use case, the following AI/ML model training collaborations will be further studied: - Type 1: Joint training of the two-sided model at a single side/entity, e.g., UE-sided or Network-sided. - Type 2: Joint training of the two-sided model at network side and UE side, respectively. - Type 3: Separate training at network side and UE side, where the UE-side CSI generation part and the network-side CSI reconstruction part are trained by UE side and network side, respectively. - Note: Joint training means the generation model and reconstruction model should be trained in the same loop for forward propagation and backward propagation. Joint training could be done both at single node or across multiple nodes (e.g., through gradient exchange between nodes). - Note: Separate training includes sequential training starting with UE side training, or sequential training starting with NW side training [, or parallel training] at UE and NW. RAN1#111 Agreement For UE-part/UE-side models, study the following mechanisms for LCM procedures: - For functionality-based LCM procedure: indication of activation/deactivation/switching/fallback based on individual AI/ML functionality - Note: UE may have one AI/ML model for the functionality, or UE may have multiple AI/ML models for the functionality. - FFS: Whether or how to indicate Functionality - For model-ID-based LCM procedure, indication of model selection/activation/deactivation/switching/fallback based on individual model IDs RAN1#111 Agreement Model Identification: - A process/method of identifying an AI/ML model for the common understanding between the NW and the UE - Note: The process/method of model identification may or may not be needed. - Note: Information regarding the AI/ML model may be shared during model identification. Functionality identification - A process/method of identifying an AI/ML functionality for the common understanding between the NW and the UE - Note: The process/method of functionality identification may or may not be needed. - Note: Information regarding the AI/ML functionality may be shared during functionality identification. - FFS: granularity of functionality, e.g., sub-use-case, scenarios, configurations, sites, etc. RAN1#111 Working Assumption Consider “proprietary model” and “open-format model” as two separate model format categories for RAN1 discussion. Proprietary-format models: - ML models of vendor-/device-specific proprietary format, from 3GPP perspective - NOTE: An example is a device-specific binary executable forma Open-format models: - ML models of specified format that are mutually recognizable across vendors and allow interoperability, from 3GPP perspective From RAN1 discussion viewpoint, RAN1 may assume that: - Proprietary-format models are not mutually recognizable across vendors, hide model design information from other vendors when shared. - Open-format models are mutually recognizable between vendors, do not hide model design information from other vendors when shared RAN1#112 Agreement For UE-side models and UE-part of two-sided models: - For AI/ML functionality identification - Reuse legacy 3GPP framework of Features as a starting point for discussion. - UE indicates supported functionalities/functionality for a given sub-use-case. - UE capability reporting is taken as starting point. - For AI/ML model identification - Models are identified by model ID at the Network. UE indicates supported AI/ML models. - In functionality-based LCM - Network indicates activation/deactivation/fallback/switching of AI/ML functionality via 3GPP signaling (e.g., RRC, MAC-CE, DCI). - Models may not be identified at the Network, and UE may perform model-level LCM. - Study whether and how much awareness/interaction NW should have about model-level LCM - In model-ID-based LCM, models are identified at the Network, and Network/UE may activate/deactivate/select/switch individual AI/ML models via model ID. FFS: Relationship between functionality identification and model identification FFS: Performance monitoring and RAN4 impact FFS: detailed understanding on model RAN1#112-bis-e Working Assumption Model selection; The process of selecting an AI/ML model for activation among multiple models for the same AI/ML enabled feature. Note: Model selection may or may not be carried out simultaneously with model activation. RAN1#112-bis-e Agreement - For AI/ML functionality identification and functionality-based LCM of UE-side models and/or UE-part of two-sided models: - Functionality refers to an AI/ML-enabled Feature/FG enabled by configuration(s), where configuration(s) is(are) supported based on conditions indicated by UE capability. - Correspondingly, functionality-based LCM operates based on, at least, one configuration of AI/ML-enabled Feature/FG or specific configurations of an AI/ML-enabled Feature/FG. - FFS: Signaling to support functionality-based LCM operations, e.g., to activate/deactivate/fallback/switch AI/ML functionalities - FFS: Whether/how to address additional conditions (e.g., scenarios, sites, and datasets) to aid UE-side transparent model operations (without model identification) at the Functionality level - FFS: Other aspects that may constitute Functionality - FFS: which aspects should be specified as conditions of a Feature/FG available for functionality will be discussed in each sub-use-case agenda. - For AI/ML model identification and model-ID-based LCM of UE-side models and/or UE-part of two-sided models: - model-ID-based LCM operates based on identified models, where a model may be associated with specific configurations/conditions associated with UE capability of an AI/ML-enabled Feature/FG and additional conditions (e.g., scenarios, sites, and datasets) as determined/identified between UE-side and NW-side. - FFS: Which aspects should be considered as additional conditions, and how to include them into model description information during model identification will be discussed in each sub-use-case agenda. - FFS: Relationship between functionality and model, e.g., whether a model may be identified referring to functionality(s). - FFS: relationship between functionality-based LCM and model-ID-based LCM

The working assumption is that the monitoring must have access both to the inputs and outputs of the model, as well as to any additional information, configuration or measurement from the NW and the UE. With this information, it is required to:

- influence short term actions on AI/ML model inference/operation such as switching AI/ML models or fallback to 3GPP legacy functionality.

- influence long-term actions related to AI/ML model maintenance (for example, a model needs to be deactivated/re-trained or needs to be monitored/evaluated more frequently/closely).

Observation 1: The AI/ML monitoring is not only limited to detecting AI/ML model performance degradation, but also suggests potential actions to mitigate the problem (e.g., model switching within the identified functionality).

Observation 2: The AI/ML monitoring influences both short-term (e.g., switch to a different AI/ML model or fallback to 3GPP legacy) and long-term actions (e.g., prolonged AI/ML model deactivation, AI/ML model re-training, or further tracking/evaluation of the AI/ML model).

## Evaluation of the applicability of inactive AI/ML models/functionalities

In the RAN1#112-bis-e meeting, the following Proposal was discussed:

Proposal 6-25e:

For the purpose of model activation/selection/switching, study necessity, feasibility and potential specification impact for methods to evaluate the applicability of inactive AI/ML models/functionalities, including the following examples:

- Evaluation by comparing the model's applicability condition to the current conditions

- Evaluation based on input data distribution

- Evaluation through model monitoring by using the inactive model(s) for monitoring purpose and measuring the inference accuracy/system performance

  - FFS: how to mitigate resulting system performance impact if any

Evaluation through model monitoring by using the inactive model(s) for monitoring purpose and measuring the inference accuracy/system performance, can have several shortcomings:

- Using inactive models in parallel and performing inference is computationally expensive for the UE.

- Determining if inactive model(s) are better than the active model, could require getting ground truth labels for monitoring purposes.

- If the UE is in a sub-region or a set of conditions where more than one models can be applied, there is the chance/danger that we are stuck with constant model switching.

- Using inactive models that perform the same task (so they can be activated under the same applicable conditions) but have different implementations (e.g., LSTM vs CNN, smaller/larger models, etc.) or belong to different functionalities (so might require different inputs) can be challenging.

Instead of using inactive model(s) for monitoring purpose, the collected monitoring data can be used to train an estimator that would be able to predict the performance of inactive model(s) in specific conditions or areas.

Observation 3: An estimator can be trained from data to predict the expected performance of inactive AI/ML model(s) without explicitly using them for monitoring purpose and measuring the inference accuracy/system performance.

Apart from the expected model performance, we must consider the associated cost of model activation and switching. For example, consider the model switching scenario of Figure 1, where the AI/ML models are one-sided models deployed at UE side. Model A is trained with data from Sub-area A (+ some data from the boundary with Sub-area B, so there is some buffer/overlap), model B is trained with data from Sub-area B (+ some data from the boundary with Sub-area A, so there is some buffer/overlap) and Model Z is a general model trained with data from several sites.

As an example, let’s assume that models A and Z implement the same functionality (so switching from A to Z could be transparent for the network). Let’s further assume that sub-area B is more “challenging” compared to sub-area A and thus requires a more complex AI/ML model. In this case, switching from model A to model B, could come with associated cost as follows:

- UE/NW coordination overhead:

  - If models A and B support the same functionality, model switching at the UE can be transparent to the network  low selection/activation/deactivation/switching cost.

  - If models A and Model B support different functionalities (e.g., different Set A/ Set B for BM or different inputs and side information for positioning)  signaling between the UE and the NW is required to coordinate on the functionality switching.

- UE measurement/reporting overhead: for example, switching to Model B might require measuring more beams for both Set B and Set A in a BM use case.

- Inference latency and energy consumption: Model B could have a significantly larger number of parameters compared to models A and Z.

- Overhead of model transfer: models A or B are not stored at the UE device. In this case, the models need to be downloaded from the NW or using the user plane, before activated.

Figure 1 A model-switching scenario. Dashed lines indicate the applicability of the respective models.

Proposal 2: A cost is associated with AI/ML functionality/model activation/deactivation/selection/switching. This cost can encapsulate, for example, the required overhead for measurement, signaling and coordination between UE and NW, as well as the complexity of the functionality/model to be activated.

In the end, a decision has to be made on whether activating (or switching to) a specific model is required, as well as when the activation/switching should occur. For example, for the UE in the location marked by the red square in Figure 1, the question is if switching to model B should occur after the model B boundary or not. Similarly, for the UE in the location marked by the grey triangle in Figure 1, the question is whether constant switching is acceptable as the UE moves (in case models A or B perform better in different parts of the trajectory) or a single model could be selected, possibly not achieving the best performance throughout the trajectory, but satisfying an overall performance constraint (e.g., a minimum positioning accuracy).

Eventually, depending on the required model performance and the maximum allowed cost, model activation/switching strategies like the one shown in Figure 2 can emerge. Here, models A and B are used when they have clear performance benefits and frequent functionality/model switching is not anticipated. In the middle, a large “buffer” zone where model Z (not performing as good as models A and B, but still fulfilling a performance constraint) is used, ensures acceptable performance without the necessity of frequent switching.

Figure 2 Final model-switching decisions for the scenario of Figure 1, where model Z is activated in the overlapping AB area. Solid lines indicate the model applied in each UE location, according to the performance/cost trade-off.

Proposal 3: To predict the expected performance of inactive AI/ML model(s) without explicitly using them for monitoring purpose and measuring the inference accuracy/system performance, train an estimator from data that can predict the expected benefit of activating the functionality/model, considering the expected performance/QoS the functionality/model will bring, as well as the expected cost due to selection/activation/deactivation/switching of the candidate functionality/model.

Observation 4: Considering the expected performance/cost trade-off, functionality/model selection/activation/deactivation/switching can be implemented with several options, e.g., as follows:

- The model with the best performance is always utilized, regardless of the associated cost.

- A performance requirement is provided. The performance estimator is queried and a list of candidate models that would be expected to fulfill the performance constraint is compiled. The final model to be activated is the one on this list with the smallest expected cost according to the cost estimator.

- A maximum acceptable cost is provided. The cost estimator is queried and a list of candidate models that would be expected to fulfill the cost constraint is compiled. The final model to be activated is the one on this list with the highest expected performance according to the performance estimator.

## Determine whether a deployed model performs well in an applicable condition before it can be activated

In the RAN1#112-bis-e meeting, the following Proposal was discussed:

Proposal 6-28d:

Study the necessity and feasibility of a procedure to validate a UE-side model or two-sided model after identification but prior to first active use, including at least:

- Validation based on using the model for validation purpose and measuring the inference accuracy/system performance

- Requirements for a model to be validated reliably (e.g., sufficient data coverage during validation)

- Note: This may or may not have specification impact.

To verify the correct operation of a UE-side or two-sided ML model after identification but prior to its first active use, activating the model for validation or using the model for monitoring purposes and evaluate its accuracy/performance in parallel with the active model is an option. The question here is how to ensure sufficient data coverage for validation, i.e., ensure that the input data cover events that do not frequently occur, since simply activating the model for a long period of time is both time-consuming and adds unnecessary complexity to the UE.

Observation 5: To verify the correct operation of a UE-side or two-sided ML model after identification but prior to its first active use, input data with sufficient coverage of both nominal operation and of events that do not frequently occur are required. Simply activating or using the model for monitoring for a long period of time is both time-consuming and adds unnecessary complexity to the UE.

If the training data contains information on these situations/events (e.g., in the form of specific distribution of RSRP measurements in Set B for beam management or specific patterns in the CSI/PRS for positioning), then the AI/ML inference entity (UE) – or several UEs in different locations, depending on the model – can be given these patterns as queries to look for. Once any UE detects the pattern, data can be collected for the model validation process. This dynamic data collection process also requires the availability of high-quality ground-truth labels for validating the model performance, which can be especially problematic for the positioning use-case, since PRUs might not be readily available in ad-hoc manner. A solution for this problem is obtaining high-quality ground truth labels obtained through landmark detection, as we propose in our companion contribution (R1-2305198).

Proposal 4: To ensure sufficient input data coverage during verification of a UE-side or two-sided ML model after identification but prior to its first active use, the UEs capable of performing this validation are given specific patterns of input data (and/or side information) as queries to look for. Once such patterns are detected, data collection for the model validation process is triggered.

## Fault management

An important functionality of AI/ML model monitoring is to detect performance degradation of the model, therefore we propose that AI/ML model monitoring has fault detection and management capabilities. The subtle detail here is that we can define the inference operation of a deployed model in 3GPP framework as “fault-free” when the model performance is adequate and as “faulty” when the model performance degrades. Even though different 3GPP AI/ML applications are envisioned (BM, Positioning, CSI compression), the sources (or root causes) of a fault (or AI/ML model performance degradation) are common (i.e., a temporary or permanent change in the radio environment that the AI/ML model was not trained for).

Proposal 5: The following concepts/terms shall be introduced:

- Fault: a specific problem caused by the performance degradation of the AI/ML model. For example, for a beam management model, the RSRP/SINR values of the chosen beams are declining.

- Fault indication: signs that could imply the existence of a fault. For example, a mismatch between the statistics of input data in the AI/ML model during Inference and the training data for the specific AI/ML model, could indicate a problem on the model’s performance.

- Fault type or root cause of a fault: the underlying reason a fault is observed. For example, we have a blockage or reflections in the radio environment and the AI/ML model’s performance degrades, as it is not trained for this.

Proposal 6: The AI/ML monitoring has extended AI/ML model fault management capabilities. It encapsulates a Fault Detection and a Fault Diagnosis function.

We define a “Fault Detection” function, which is responsible to determine if there are any indications for the presence of a fault (AI/ML model performance degradation) and a “Fault Diagnosis” function that attempts to determine the root cause of the problem, a “fault type,” and recommend – when possible – the best course of action to mitigate its effects on the system performance.

The main components, as well as the flow of execution in the monitoring and fault processing framework are shown in Figure 3. The functionality is as follows:

- The Fault Detection module observes and records specific aspects of the AI/ML model to detect signs of degrading performance (called fault indicators). Once such a sign is detected (e.g., the input data statistics in inference deviate further than a threshold value/limit compared to the statistics of the training data), an alarm is raised, and all the relevant data are forwarded to the Fault Diagnosis module.

- The Fault Diagnosis module analyzes the available data regarding the fault indication – along with relevant data from other analytics of the Fault Detection module. With the analysis, it determines the potential type of fault (if any). If a fault is indeed detected but the nature of the fault cannot be determined, the system switches to a safe, fallback function (for example performing a full beam sweep in the beam management use-case). The output of this module is a report with all information on the specific Fault type along with a recommendation on a course of action to mitigate the effects of the detected fault type.

Figure 3 High-level overview of the Monitoring and Fault Management Framework

Observation 6: The AI/ML monitoring requires access to specific information of the AI/ML model, like its intended functionality, expected input and output distributions, threshold values to detect divergence from nominal operation, etc.

Observation 7: For some aspects of functionality-based LCM, where models might not be identified in the Network, this information will potentially not be made available to external AI/ML monitoring.

To make informative decisions on the existence of a specific fault, both Fault Detection and Fault Diagnosis steps can process data and information from different sources, as well as request for additional measurements and reporting when required.

In all cases, the Fault Detection evaluates the input and output data of the model looking for fault indications. Moreover, it can leverage additional side information from the NW or further information and measurements from the UE.

In a similar way, the Fault Diagnosis evaluates the findings (Fault Indicators) of the Fault Detection module and can request new side information to determine the root cause of a Fault both from the NW and the UE.

In case the UE is the monitoring entity, there can be cases that simultaneous inference and fault detection and diagnosis operation is not possible. This can be either due to the computational complexity burden related to the UE capabilities, state of charge, etc. and/or due to potential overhead from monitoring-specific measurements/transmissions.

Proposal 7: Consider the requirements and applicability for simultaneous inference and monitoring at the UE.

Once the AI/ML monitoring session is completed, a monitoring report is made available. The Monitoring report could include:

- Model ID or Functionality ID,

- Model information (for models identified in the NW),

- Configuration of the Fault Detection and Diagnosis (for models identified in the NW),

- Probability of different Fault Types occurring,

- Time information associated with one or more Fault Indications,

- A score on the quality of the model under this monitoring session.

In addition, if the AI/ML monitoring recommends an action (e.g., model switching, model fine-tuning, additional measurements, long-term and more frequent AI/ML Model monitoring, etc.), the effect of this action on the QoS of the application (at least for real-time decisions/actions) should be included in the AI/ML Monitoring report retrospectively.

This way, even AI/ML models at the UE that are identified as logical models in the NW can benefit from NW information. As an example, consider the following:

- NW receives a fault detection report from the monitoring entity of a UE on a logical model.

- The report contains only the functionality ID and a fault indication without any information on model details and configuration.

- The NW, based on the report, retrieves similar fault detection reports from available historical data.

- The NW examines which recommended action(s) were provided in the past from AI/ML monitoring at the UE.

- The NW provides the monitoring entity of the UE with the same recommended action(s).

Proposal 8: The AI/ML monitoring at the UE, can provide information to the NW at least on AI/ML model functionality, detected fault indicators and associated recommended actions, even for models identified only as logical models in the NW.

## Model monitoring in two-sided AI/ML models

For two-sided models, that primarily utilized for CSI feedback enhancements, the AI/ML monitoring requirements become more stringent. As different possible training types (Type 1, 2 and 3) are envisioned for these types of models, the monitoring entity requirements in each case would vary. For example, in Type 1 training, where there is one training entity for joint training of both UE-side and gNB-side, based on the transmitted CSI from the UE to the gNB, if any fault or mismatch is detected by the gNB, a fault detection signal is sent to the training entity. Afterwards, the updated model in designed and both AI sides are informed. To successfully monitor the two-sided model and detect a valid fault, the information about the decoder or at least a version of the decoder is required anyway. In addition, the gNB may be informed about the encoder performed at the UE-side by model ID or functionality based data exchanges. Therefore, it is more realistic to assume that the fault detection is performed at the gNB-side, as indicated in Figure 2.

| Fault detection signal gNB UE Training entity CSI report Figure 2: Model monitoring for Type 1 training Model update | 1. CSI report 2.Fault detection signal gNB UE UE-side Training entity Figure 3: Model monitoring for Type 2, 3 training gNB-side Training entity 3.Update data exchange 4.Model update 1. CSI report 2.Fault detection signal gNB UE UE-side Training entity Figure 3: Model monitoring for Type 2, 3 training gNB-side Training entity 3.Update data exchange 4.Model update |
| --- | --- |

In Type 2, 3 training, there are two training entities corresponding to the UE-side and the gNB-side, respectively. As shown in Figure 3, if a fault in the AI model is detected at the gNB based on the CSI report signal, the gNB-training entity is informed for model update. Depending on the type of the training (Type 2 or Type 3) joint or separate model update is inferred at the gNB-side and UE-side vendors. Among the different types of the two-sided model training, Type 3 network-first model training has a potential challenge in CQI (channel quality index) evaluating and reporting. In this type of training, the UE is not aware of implemented decoder at the gNB-side. Therefore, some side-information should be exchanged between the UE and the gNB to calculate the CQI. This information may be included in the monitoring phase.

Proposal 9: In two-sided model for CSI feedback use case, the gNB monitors the performance of the AI model and detect any possible fault based on the transmitted CSI report from the UE.

# Conclusion

In this contribution, we have made the following observations:

Observation 1: The AI/ML monitoring is not only limited to detecting AI/ML model performance degradation, but also suggests potential actions to mitigate the problem (e.g., model switching within the identified functionality).

Observation 2: The AI/ML monitoring influences both short-term (e.g., switch to a different AI/ML model or fallback to 3GPP legacy) and long-term actions (e.g., prolonged AI/ML model deactivation, AI/ML model re-training, or further tracking/evaluation of the AI/ML model).

Observation 3: An estimator can be trained from data to predict the expected performance of inactive AI/ML model(s) without explicitly using them for monitoring purpose and measuring the inference accuracy/system performance.

Observation 4: Considering the expected performance/cost trade-off, functionality/model selection/activation/deactivation/switching can be implemented with several options, e.g., as follows:

- The model with the best performance is always utilized, regardless of the associated cost.

- A performance requirement is provided. The performance estimator is queried and a list of candidate models that would be expected to fulfill the performance constraint is compiled. The final model to be activated is the one on this list with the smallest expected cost according to the cost estimator.

- A maximum acceptable cost is provided. The cost estimator is queried and a list of candidate models that would be expected to fulfill the cost constraint is compiled. The final model to be activated is the one on this list with the highest expected performance according to the performance estimator.

Observation 5: To verify the correct operation of a UE-side or two-sided ML model after identification but prior to its first active use, input data with sufficient coverage of both nominal operation and of events that do not frequently occur are required. Simply activating or using the model for monitoring for a long period of time is both time-consuming and adds unnecessary complexity to the UE.

Observation 6: The AI/ML monitoring requires access to specific information of the AI/ML model, like its intended functionality, expected input and output distributions, threshold values to detect divergence from nominal operation, etc.

Observation 7: For some aspects of functionality-based LCM, where models might not be identified in the Network, this information will potentially not be made available to external AI/ML monitoring.

Based on the above observations, we make the following proposals:

Proposal 1: Change the Reinforcement Learning (RL) Definition as follows:

Reinforcement Learning (RL): A process of training an AI/ML model (policy) to interact with an environment and take actions (model’s output) based on the environment’s current state (model’s input), with the goal of maximizing the expected cumulative reward (feedback signal). For the AI/ML model (policy) training, direct interaction with the environment, available logged data from the environment, or a combination of both can be used.

Proposal 2: A cost is associated with AI/ML functionality/model activation/deactivation/selection/switching. This cost can encapsulate for example the required overhead for measurement, signaling and coordination between UE and NW, as well as the complexity of the functionality/model to be activated.

Proposal 3: To predict the expected performance of inactive AI/ML model(s) without explicitly using them for monitoring purpose and measuring the inference accuracy/system performance, train an estimator from data that can predict the expected benefit of activating the functionality/model, considering the expected performance/QoS the functionality/model will bring, as well as the cost due to selection/activation/deactivation/switching of the candidate functionality/model.

Proposal 4: To ensure sufficient input data coverage during verification of a UE-side or two-sided ML model after identification but prior to its first active use, the UEs capable of performing this validation are given specific patterns of input data (and/or side information) as queries to look for. Once such patterns are detected, data collection for the model validation process is triggered.

Proposal 5: The following concepts/terms shall be introduced:

- Fault: a specific problem caused by the performance degradation of the AI/ML model. For example, for a beam management model, the RSRP/SINR values of the chosen beams are declining.

- Fault indication: signs that could imply the existence of a fault. For example, a mismatch between the statistics of input data in the AI/ML model during Inference and the training data for the specific AI/ML model, could indicate a problem on the model’s performance.

- Fault type or root cause of a fault: the underlying reason a fault is observed. For example, we have a blockage or reflections in the radio environment and the AI/ML model’s performance degrades, as it is not trained for this.

Proposal 6: The AI/ML monitoring has extended AI/ML model fault management capabilities. It encapsulates a Fault Detection and a Fault Diagnosis function.

Proposal 7: Consider the requirements and applicability for simultaneous inference and monitoring at the UE.

Proposal 8: The AI/ML monitoring at the UE, can provide information to the NW at least on AI/ML model functionality, detected fault indicators and associated recommended actions, even for models identified only as logical models in the NW.

Proposal 9: In two-sided model for CSI feedback use case, model monitoring is performed at the gNB-side based on the transmitted CSI report from the UE.

# References

- 3GPP RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN#94e, Electronic Meeting, December 6th – 17th, 2021.

- 3GPP R1-2207123, “Evaluation on AI/ML for positioning accuracy enhancements” Fraunhofer IIS Fraunhofer HHI

- 3GPP R1-2212658, “Summary#1 of General Aspects of AI/ML Framework”

- Li, Y. (2017). Deep reinforcement learning: An overview. arXiv preprint arXiv:1701.07274.