3GPP TSG RAN WG1#113	           					     R1-2305206

Incheon, Korea, May 22nd – 26th, 2023

Agenda item:	9.2.4.1

Source: 	Lenovo

Title: 	Discussion on AI/ML Positioning Evaluations

Document for:	Discussion

# Introduction

The Rel-18 study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface NR positioning evolution was agreed upon during the RAN#94-e [1] meeting, where one of the objectives included the discussion on the performance and evaluation framework for evaluating AI/ML positioning use cases.

Furthermore, substantive progress made during the past meetings, regarding the simulation assumptions as well as general observations for the AI/ML positioning performance evaluations. The following agreements were made during the RAN1#112bis-e [2] meeting:

Agreement For evaluation of both the direct AI/ML positioning and AI/ML assisted positioning, company optionally adopt delay profile (DP) as a type of information for model input. - DP is a degenerated version of PDP, where the path power is not provided. Agreement For the evaluation of AI/ML based positioning, the study of model input due to different number of TRPs include the following approaches. Proponent of each approach provide analysis for model performance, signaling overhead (including training data collection and model inference), model complexity and computational complexity. - Approach 1: Model input size stays constant as NTRP=18. The number of TRPs (N’TRP) that provide measurements to model input varies. When N’TRP < NTRP, the remaining (NTRP N’TRP) TRPs do not provide measurements to model input, i.e., measurement value is set to 0. - Approach 1-A. The set of TRPs (N’TRP) that provide measurements is fixed. - Approach 1-B. The set of TRPs (N’TRP) that provide measurements can change dynamically. - Note: for Approach 1, one model is provided to cover the entire evaluation area. - Approach 2: The TRP dimension of model input is equal to the number of TRPs (N’TRP) that provide measurements as model input. When N’TRP < NTRP, the remaining (NTRP N’TRP) TRPs are ignored by the given model. For a given AI/ML model, the set of TRPs (N’TRP) that provide measurements is fixed. - For Approach 2: one model can be provided to cover the entire evaluation area, which is equivalent to deploying N’TRP TRPs in the evaluation area for positioning if ignoring the potential inference from the remaining (18 N’TRP) TRPs. - For Approach 2, if Nmodel (Nmodel >1) models are provided to cover the entire evaluation area, the total complexity (model complexity is the summation of the Nmodel models. Agreement In the evaluation of AI/ML based positioning, if N’TRP<18, the set of N’TRP TRPs that provide measurements to model input of an AI/ML model are reported using the TRP indices shown below. Agreement For AI/ML assisted positioning with TOA as model output, study the impact of labelling error to TOA accuracy and/or positioning accuracy. - The ground truth label error of TOA is calculated based on location error. The location error in each dimension of x-axis and y-axis can be modelled as a truncated Gaussian distribution with zero mean and standard deviation of L meters, with truncation of the distribution to the [-2*L, 2*L] range. - Value L is up to sources. - Other models of labelling error are not precluded - Other timing information, e.g., RSTD, as model output is not precluded. Agreement For AI/ML assisted positioning with LOS/NLOS indicator as model output, study the impact of labelling error to LOS/NLOS indicator accuracy and/or positioning accuracy. - The ground truth label error of LOS/NLOS indicator can be modelled as m% LOS label error and n% NLOS label error. - Value m and n are up to sources. Companies consider at least hard-value LOS/NLOS indicator as model output.

This contribution provides a brief discussion into the some of the open issues relating to the evaluation scenarios and KPIs for enabling a meaningful AI/ML positioning performance evaluation.

# AI/ML Model KPIs

## Positioning AI/ML Model Generalization

Many of the AI/ML based positioning systems are expected to be location or environment specific based on the data collected for training and thus the positioning performance may be limited to a certain scenario. In addition, generalization should also consider the movement of objects within a specific location or environment, which may be captured by the channel characteristics. The generalization capability of a positioning AI/ML model(s) should be evaluated by testing the performance of the simulated AI/ML model considering the following aspects:

- Different channel parameters of the indoor factory channel model, e.g., different clutter densities, different BS and UE heights

- Different UE speeds and rotations

- Different PRS configurations, e.g., comb patterns, repetitions, number of samples

- Different Tx/Rx beam configurations e.g., beamwidths, QCL assumptions.

The use of different InF scenarios for training and test data has already been agreed upon during the RAN1#110-bis-e meeting, which assists in evaluating the AI/ML generalizability performance.

In addition, in the case of a strongly generalized AI/ML model, it is important that the test set include samples from a UE that is not present during the training time. The size of such generalized data samples for the test data set should also be carefully considered across a variety of configurations/scenarios.

Proposal 1: The evaluation methodology should be designed under a common generalizability framework, including additional considerations such as positioning performance evaluations under different UE mobility options, Tx/Rx beam configurations and DL-PRS configurations.

### Positioning AI/ML Model Update

In the case of positioning, there are three choices in terms of AI/ML model deployment including the UE, gNB and LMF. These three nodes should also be well-coordinated in terms of the triggering, deployment and inference processing of the model.

Due to dynamics and characteristics of the radio channel environment as mentioned in the previous section, it is inevitable that a set of collected data will be at some point be outdated due to the 1) UE mobility/rotation 2) channel characteristics (NLOS, multipath, small- and large-scale fading effects, etc.). Thus, there is a need for a model monitoring and model update component which may be used to update the AI/ML model depending on the real-time dynamics or conditions of the evaluated environment at different points in time. This can also be fed back into the training model component to re-train the AI/ML model with the updated data. Model Robustness and Adaptability may be considered as part of the generalizability criteria. Table 1 presents the views on the definition of AI/ML model Robustness and Adaptability.

Table 1: Model Robustness and Adaptability

| Model Robustness | - The robustness of AI/ML models via model inference monitoring and triggering model update should be considered. - Since AI/ML models are largely dependent on the type of data used to train the models, the robustness of such derived models needs to validated/tested using different types of “good” and “bad” data samples due to the dependency on the channel models and associated assumptions used to generate the simulated data. - Further study robustness metrics |
| --- | --- |
| Adaptability | - Since radio channel and UE mobility are dynamic in nature, this may affect the overall performance of the trained models. - Model update procedure and associated accuracy of the model when it is deployed in an environment other than the environment the training data is extracted from should be evaluated. - This may depend on the chosen channel model and may affect aspects such as LOS/NLOS probability, multipath (reflectors and scatterers) and UE mobility assumptions at various locations. - Further study adaptability metrics |

## Positioning AI/ML Model Complexity

The complexity of the AI/ML positioning techniques can especially impact the UE positioning performance in terms of power consumption. AI/ML models which perform well in positioning scenarios, but have high complexity may not be realistic from an implementation perspective for UE-based approaches. However, at the same time UE-assisted positioning approaches can leverage the computational ability of the LMF and therefore complexity constraint may be relaxed to some extent.

Observation 1: AI/ML models for positioning require a careful balance between performance and complexity depending on the type of positioning mode (UE-assisted or UE-based).

Although FLOPs may be considered a meaningful metric of evaluating algorithmic complexity, the hardware and software platforms (e.g., AI/ML libraries used) used to derive the FLOP count should also be considered in the evaluation as AI/ML algorithms with same flop count may potentially lead to different runtimes on different platforms and systems.

The resources required to execute an AI/ML model should also be further considered in terms of execution time, memory, inputs and outputs as well hardware considerations. Therefore, each AI/ML model should be studied in an objective manner independent of the type of implementation, software platforms or hardware systems.

Further aspects of consideration for evaluating the AI/ML model complexity in the context of positioning include:

- Input size definition: Defined as the number of bits required to represent the input or features of an algorithm, e.g., training data set and largely depends on the type of data, which impacts the time complexity of an algorithm.

- Type of training including AI/ML in terms of online and offline training, typically offline training may require more time when compared to online training in an already deployed system

- Complexity type: AI/ML algorithmic complexity may be defined in terms of one the of the following types of complexity: worst-case, best-case, average-case and amortized complexity

A further aspect may be to characterize the AI/ML algorithm used for evaluation using the Big O notation, which is generally used to define the time/resources used to solve a computing problem including AI/ML algorithms. This is one of the methods to establish a fair comparison among companies for the different performance evaluations. During the RAN1#112 meeting [2], some discussions revolved around the definition of nominal computational complexity values, which aim to showcase non-optimized GPU/TPU implementations. Since this may also be applicable to other use cases including CSI and Beam management, the general definition can be established in AI 9.2.1 to serve as guidance on the complexity definition for all use cases.

Proposal 2: Await further discussions and progress on the definition “Nominal Computational Complexity” in AI 9.2.1 to establish a fair complexity comparison of performance evaluations among different companies.

# Conclusion

This discussion paper has noted the following observations with respect to the AI/ML positioning evaluations:

Observation 1: AI/ML models for positioning require a careful balance between performance and complexity depending on the type of positioning mode (UE-assisted or UE-based).

The discussion proposals are summarized as follows:

Proposal 1: The evaluation methodology should be designed under a common generalizability framework, including additional considerations such as positioning performance evaluations under different UE mobility options, Tx/Rx beam configurations and DL-PRS configurations.

Proposal 2: Await further discussions and progress on the definition “Nominal Computational Complexity” in AI 9.2.1 to establish a fair complexity comparison of performance evaluations among different companies

# References

- RP-213599, “New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, RAN#94-e, Dec. 2021.

- R1-2304168, “Session notes for 9.2 (Study on AI/ ML for NR air interface)”, Ad-hoc Chair (CMCC), RAN1#112bis-e, Apr. 2023.