3GPP TSG RAN WG1 #113			R1-2305223

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:	5

Source:	Apple

Title:	Discussion on RAN2 LS on RACH-less Handover

Document for:	Discussion/Decision

# Introduction

RAN2 sent an LS on RACH-less handover for NR NTN [1]. In the LS, three questions related to beam selection and power control for initial uplink transmission in NR NTN RACH-less handover are raised.

In this contribution, we discuss the RAN1 related topics for RACH-less handover.

# Discussion

In RAN2 LS on RACH-less handover [1], the recent RAN2 agreements on RACH-less handover are provided.

In legacy handover procedure, after receiving handover command via RRC connection reconfiguration from source cell, a UE sends random access preamble to target cell. After the UE receives random access response from target cell, which includes timing advance (TA) command, it sends handover complete via RRC connection reconfiguration complete to target cell.

In RACH-less handover, after receiving handover command from source cell, a UE waits for uplink grant from target cell. Once it receives uplink grant, it sends handover complete via RRC connection reconfiguration complete to target cell. Since there is no random access in the targe cell, the RACH-less handover solution offers significant reductions in the data connectivity interruption time at each handover, and hence reduced overall handover execution time.

In NR NTN RACH-less handover, a UE will either use a pre-allocated grant or use a dynamic grant for initial uplink transmission.

The first question in RAN2’s LS is as follows:

1. Regarding the pre-allocated grant for initial UL transmission, considering the similarity to Msg1 in RACH and the similarity to the initial UL transmission in CG-SDT, where PRACH/PUSCH resource is mapped to SSBs, whether the pre-allocated grant is provided with association to SSB(s)? If yes, whether a RSRP threshold is needed for SSB selection for initial UL transmission?

In NR, each SSB transmission with a transmit beam has an associated RACH occasion. After determining an SSB transmit beam, a UE selects the associated RACH occasion for its PRACH transmission. Depending on which RACH occasion of receiving PRACH from a UE, gNB is able to obtain the transmit beam to serve the UE.

In NR CG-SDT, one or more SSBs can be associated with each CG configuration. Based on the determined SSB, a UE selects the associated PUSCH resource (including a PUSCH occasion and a DMRS resource) per CG configuration for its PUSCH transmission. The SSB-to-PUSCH resource mapping within the CG configuration is implicitly defined.

We think the similar SSB-PRACH or SSB-PUSCH resource association mechanism could be applied to the pre-allocated grant for initial uplink transmission in NR NTN RACH-less handover. This allows target cell to learn the transmit beam to serve the UE, as well as facilitates target cell to receive the initial uplink transmission.

In NR CG-SDT, a UE triggers SDT only if the measured downlink RSRP is larger than or equal to a RSRP threshold. If the measured downlink RSRP is smaller than the RSRP threshold, UE will trigger the legacy RRC resume procedure. This mechanism ensures the reliability of SDT transmissions.

Similarly, to ensure the reliability of initial uplink transmission in RACH-less handover, UE needs to first compare its measured downlink RSRP with a RSRP threshold. UE is allowed to make the initial uplink transmission only if the measured downlink RSRP is larger than or equal to RSRP threshold. Otherwise, UE follows the legacy handover procedure. Hence, RSRP threshold is needed for SSB selection for initial uplink transmission.

Proposal 1: Regarding the pre-allocated grant for initial uplink transmission in NR NTN RACH-less handover, the association between the pre-allocated grant resources and SSB(s) is supported.

- RSRP threshold is needed for SSB selection for initial uplink transmission.

The second question in RAN2’s LS is as follows:

2. To monitor target cell PDCCH for dynamic grant for initial UL transmission, whether beam selection is needed (e.g., performed by NW with selected beam(s) indicated, or performed by UE)?

In NR NTN RACH-less handover where the initial uplink transmission is scheduled by a dynamic grant, UE needs to monitor target cell PDCCH. The receive beam of this PDCCH can be selected by UE based on its measurement of SSB. Specifically, the receive beam of PDCCH for dynamic grant for initial uplink transmission is derived from the SSB receive beam which has good RSRP measurement results. Through this way, network’s explcit beam indication is not needed.

Proposal 2: Beam selection is needed and the receive beam of PDCCH for dynamic grant for initial uplink transmission is selected by UE based on its measurement of SSB.

The last question in RAN2’s LS is as follows:

3. Regarding the power control for initial UL transmission, whether it follows the rules specified for PUSCH scheduled by Random Access grant or by configured grant or others?

The power control of PUSCH scheduled by random access grant (RAR) is a closed loop power control. The TPC command for PUSCH is indicated in RAR. The indicated value is an adjustment offset, which is based on the PRACH transmission power. In the initial uplink transmission, there is no baseline transmission power at UE side. Hence, the rule specified for PUSCH scheduled by RAR does not fit for initial transmission power control.

In type 1 configured grant, the power control parameters (e.g., “p0-PUSCH-Alpha”, “pathlossreferenceIndex”) are configured. If the initial uplink transmission is based on pre-allocated grant, then the configuraton of pre-allocated grant could have these power control parameters. If the initial uplink transmission is based on dynamic grant, then the dynamic grant could indicate these power control parameters. With that, the power control rule specified by configured grant could be applied for initial uplink transmission.

Proposal 3: The power control for initial uplink transmission follows the rules specified for configured grant.

The corresponding draft reply LS on RACH-less handover is in [2].

# Conclusion

In this contribution, we provided our views on beam selection and power control for initial uplink transmission in NR NTN RACH-less handover. Our proposals are as follows:

Proposal 1: Regarding the pre-allocated grant for initial uplink transmission in NR NTN RACH-less handover, the association between the pre-allocated grant resources and SSB(s) is supported.

- RSRP threshold is needed for SSB selection for initial uplink transmission.

Proposal 2: Beam selection is needed and the receive beam of PDCCH for dynamic grant for initial uplink transmission is selected by UE based on its measurement of SSB.

Proposal 3: The power control for initial uplink transmission follows the rules specified for configured grant.

# References

- R1-2304322, “LS to on RACH-less handover,” May 2023.

- R1-2305222, “Draft reply LS on RACH-less handover,” May 2023.