3GPP TSG RAN WG1 #113		R1-2305343

Incheon, May 22nd – 26th, 2023

Agenda item:	9.5.1.3

Source: 	Qualcomm Incorporated

Title: 	Resource Allocation for SL-PRS

Document for:		Discussion and Decision

# Introduction

A work item has been agreed for positioning in RAN #98-e with the following objective for sidelink positioning [1]:

- Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]: - Specify SL PRS for support of sidelink positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1]. - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum. - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified. - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2]. - Specify support of resource allocation for SL PRS: - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1]. - For resource allocation mechanism for SL PRS in Scheme 2: - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1]. - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1]. - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1]. - NOTE: For SL positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured. - Specify procedures for transmit power control for SL PRS transmissions at least based on open loop power control (OLPC) [RAN1]. - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2]. - Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios [RAN2, RAN3]: - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)). - Specify the protocol and procedures for SL positioning between UEs and LMF. - Specify signalling to NG-RAN for sidelink positioning and ranging service authorizations as needed. [RAN3, RAN2] - Specify corresponding new core requirements, as well as identifying and specify the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

In this contribution, we discuss the design of the dedicated and shared resource pools as well as how to perform Scheme 1 and Scheme 2 resource allocation in each.

Channels in the Dedicated Resource Pool

The dedicated resource pool does not contain legacy UEs and there is no backward compatibility constraint. Therefore, the design could be tailored to maximize resource utilization as we present in this section.

RAN1 agreed that at least SL-PRS is included in the dedicate resource pool with options the only PSCCH is included, and both PSCCH and PSSCH are included. RAN1 also agreed that SCI can be used for reserving or indicating SL-PRS resources:

Agreement For a dedicated resource pool for SL positioning, - With regards to which channels can be included in the resource pool in addition to SL-PRS, consider the following options: - Opt. 1: No other channel can be included beyond SL-PRS - Opt. 2: PSCCH which carries SCI associated with SL-PRS transmission(s) is included - Opt. 3: PSCCH which carries SCI associated with SL-PRS transmission(s) and PSSCH associated with SL-PRS transmission(s) are included - FFS: Details - FFS: definition of PSSCH associated with SL-PRS transmission(s) - Note: Companies are encouraged to provide their analysis and views on the above Agreement With regards to SL signaling of the reservation/indication of SL-PRS resource(s) for dedicated resource pool and shared resource pool (if supported) for positioning: - Option A.1: SCI can be used for reserving/indicating one or more SL-PRS resource(s) - Note: This does NOT mean that only SCI is being used. There can still be higher layer signaling for the purpose of indicating a part of SL-PRS configuration. - FFS: Whether SCI is single stage SCI or two stage SCI - FFS: SL-MAC-CE or other higher-layer signaling reservation/indication - Agreement For a dedicated resource pool for Positioning, - With regards to which channels can be included in the resource pool in addition to SL-PRS, option 1 (No other channel can be included beyond SL-PRS) is NOT pursued further. - Continue discussion between Option 2 and 3, and whether any other channel could also be included (e.g. PSFCH). Agreement For a dedicated resource pool for SL positioning, only a single stage SCI is used. PSCCH and associated SL-PRS are TDMed in the same slot. - FFS: whether SL-PRS can be transmitted in a slot without associated PSCCH

Given the agreement to use SCI for reserving or indicating SL-PRS resources and the lack of standalone control transmissions in sidelink, SL-PRS transmissions should be accompanied by PSCCH in the dedicated resources. If there are cases where only higher-layer reservations are used, then SL-PRS could be transmitted without PSCCH.

Both SCI-2 and SL-SCH require the presence of PSSCH DMRS. Hence, including either significantly reduces the number of symbols available for SL-PRS in a slot, negating the primary benefit of having a dedicated resource pool. Based on the above, we propose to only include PSCCH and SL-PRS in the dedicated resource pool (Option 2). Figure 1 illustrates the corresponding slot structure.

Proposal 1: Only SL-PRS and PSCCH which carries SCI associated with SL-PRS transmission(s) are included in the dedicated resource pool.

Figure 1 Slot structure in a dedicated resource pool.

# Channels in the Shared Resource Pool

RAN1 is discussing multiplexing of the different channels in a shared resource pool:

Agreement In a shared resource pool: - SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in the same slot - With regards to PSSCH and SL-PRS multiplexing, downselect one of the following alternatives in RAN1#113 meeting: - Alt. A.1: Only TDMing is supported - Alt. A.2: Only FDMing of PSSCH and SL-PRS is supported - FFS: Rate-matched around SL-PRS REs and/or PRB/sub-channel-level FDMing are supported potentially for different cases - Note: Rate-matched around SL-PRS REs is not applicable to comb-1 SL-PRS - Alt. A.3: Both Alt. A.1 and A.2 are supported in the specification - With regards to PSCCH and SL-PRS multiplexing, downselect one of the following alternatives in RAN1#113 meeting: - Alt. B.1: Only TDMing is supported - Alt. B.2: TDMing or PRB/sub-channel-based FDMing is supported - The PSSCH is used for (downselect one of the following alternatives in RAN1#113 meeting): - Alt. C.1: 2nd SCI only - Alt. C.2: 2nd SCI and SL-SCH - Alt. C.3: “2nd SCI only” or “2nd SCI and SL-SCH” - FFS: Handling of PT-RS and SL-PRS

To minimize impact on other transmissions in the shared resource pool, it is important to follow the sidelink communication principle of maintaining the same transmit power across all OFDM symbols. To avoid transients on the transmitter side, it is preferable to keep the transmission bandwidth the same across symbols. Allowing PSSCH to be mapped on all symbols, including those with SL-PRS by RE-level rate-matching avoids two issues, while maintaining backward compatibility.

Proposal 2: Regarding PSSCH and SL-PRS multiplexing in the shared resource pool, TDMing and FDMing of PSSCH and SL-PRS is supported with PSSCH rate-matched around SL PRS REs.

FDMing PSCCH and SL-PRS requires puncturing portions of some SL-PRS symbols. This will degrade positioning measurement performance and the exact impact has not been studied. Hence, we propose to not support this structure and to only allow TDMing between PSCCH and SL-PRS.

Proposal 3: Regarding PSCCH and SL-PRS multiplexing in the shared resource pool, only TDMing is supported.

SCI-2 is mapped starting on the first symbols with PSSCH DMRS in sidelink communications. In many DMRS patterns, there will be OFDM symbols prior to the first symbols PSSCH DMRS. If only SCI-2 is allowed on PSSCH without SL-SCH, these symbols will be left empty or partially, leading to transmission power and AGC inconsistencies. Another aspect is that SL-SCH carries MAC-CE and MAC headers, both of these can carry information that is not present in SCI-2 without introducing too many SCI-2 formats or very large ones. Due to these reasons, we propose to always include SL-SCH on PSSCH. This is similar to the case of IUC Scheme 1 on SCI 2-C where a MAC-CE is also included to preserve the slot structure and avoid the power issues.

Proposal 4: In the shared resource pool, PSSCH in a slot with SL-PRS is used for 2nd SCI and SL-SCH.

# Resource Allocation Procedure

## Resource Definition

In the shared resource pool, supported by disallowing multiplexing SL-PRS from different UEs on different comb offsets, the existing resource definition can be used: a resource is a set of contiguous sub-channels in a slot as defined in [6].

Proposal 5: In a shared resource pool, a resource used for transmission of SL-PRS is defined as a set of contiguous sub-channels in a slot, i.e. the existing definition for a sidelink resource is reused.

In the dedicated resource pool, the resource definition should be updated to incorporate the comb offset used, accommodating multiplexing of multiple UEs on different comb offsets.

Proposal 6: In a dedicated resource pool, a resource used for transmission of SL-PRS is defined as a set of contiguous sub-channels in a slot with a comb size, comb offset, and TDM partition.

When TDM is supported by the UE and enabled in the dedicated resource pool, the resource definition should account for the fact that the UE can use a portion of the available sidelink slots for its SL-PRS transmission. The TDM partitioning should be semi-static and not dynamic; otherwise, collisions and AGC issues would arise.

Proposal 7: Sub-slot TDM SL-PRS resource partitioning, when supported by the UE and enabled for the dedicated resource pool, is semi-static.

## Scheme 1 Resource Allocation

RAN1 agreed to no longer consider a UE receiving SL-PRS resources from the LMF, leaving gNB as the only source of signaling:

Agreement Regarding Scheme 1 SL-PRS resource allocation, do not further consider a transmitting UE to receive the SL-PRS resource allocation through higher layers from the LMF (i.e. Option 1 is not pursued further). Agreement For Scheme 1 SL-PRS resource allocation, a transmitting UE can receive a SL-PRS resource allocation signaling from gNB through a - Dynamic grant - FFS Reuse DCI format 3_0 for signalling SL-PRS resource allocation or Support a new DCI format (3_X) and consider DCI format 3_0 as a starting point - Configured grant type 1 - the SL-PRS transmission(s) follows the higher layer configuration - Configured grant type 2 - Support activating and releasing the configured grant using a new DCI format 3_X or 3_0 (to be down-selected between the two DCI formats) - The above mechanisms use NR Rel-16 mode-1 signaling as a starting point - FFS: whether same/different DCI format(s) are applied for shared pool and dedicated pool. - FFS: Further details

For the above to work, there needs to be signaling from the LMF to the gNB where the LMF provides the gNB with the requisite SL-PRS properties. The gNB would then provide the UE with a grant containing the resource allocation and respond to the LMF with allocation and configuration information. In this setting, the target UE of the SL-PRS transmission, would receive the SL-PRS configuration from the transmitter UE via SLPP messaging, then would search for and decode PSCCH to locate the resources used for SL-PRS. The last part is similar to how SL data reception operates in Mode 1 resource allocation.

We note that the existing DCI format and RRC configuration suffice to provide SL-PRS resource grants to the transmitter UE and a new DCI format is not needed. This existing signaling includes DCI 3-0 grants from as well as sidelink configured grants Type 1 and Type 2.

Proposal 8: LMF provides the gNB with requisite SL-PRS properties. gNB responds to the LMF with configuration and allocation information.

Proposal 9: DCI grants, configured sidelink grant Type 1, and configured sidelink grant Type 2 from Mode 1 are reused for Scheme 1 resource allocation.

Proposal 10: A receiver UE obtains SL-PRS configuration from the transmitter UE via SLPP messaging.

Proposal 11: A transmitter UE transmits SCI reserving resources for the accompanying SL-PRS similar to sidelink communications.

## Scheme 2 Resource Allocation

RAN1 agreed of consider a list of modifications to the resource selection procedure in the dedicated resource pool:

Agreement For Scheme 2, in a dedicated resource pool, using Rel-16 resource (re)-selection procedure as the starting point, consider at least the following potential modifications: - Modification 1: For the RS used to derive L1 SL-RSRP for resource exclusion: - Option 1: SL-PRS - Option 2: PSCCH DMRS - Option 3: PSSCH DMRS (if PSSCH is included in the dedicated resource pool) - Modification 2: For the resource selection window: - Option 1: for the derivation of the window, using the legacy approach as a starting point, substitute the Packet Delay Budget (PDB) with a new delay budget - Option 2: the selection window is provided by higher layers - Modification 3: For the SL-PRS priority: - Option 1: A single L1 SL-PRS priority is allowed in a resource pool - Option 2: Multiple L1 SL-PRS priority are allowed in a resource pool - Modification 4: For the definition of a candidate resource within the resource selection window: - Options TBD - Modification 5: For the reservation interval of SL-PRS: - Option 1: Provided by UE’s higher layers with values TBD. The set of values is (pre-)configured. - Modification 6: For the sensing window length (T0): - Option 1: Use the legacy (pre-)configuration with values (100 msec, 1100 msec) - Option 2: Equal to or larger than the largest reservation interval - Option 3: Provided by higher layers with values TBD - Modification 7: For the initial S-RSRP threshold & stepsize, target resource ratio X(%): - Options TBD - Modification 8: For the pre-emption of the reserved resources: - Options TBD - Note 1: Other potential modifications and/or other options within each modification are not precluded - Note 2: Multiple options for each potential modification may be supported

In terms of which RS to use for sensing RSRP measurements, Rel-16 NR Sidelink introduced the ability to (pre-)configure between PSCCH DMRS and PSSCH DMRS. PSCCH DMRS was included as a method to ensure forward compatibility with changes to PSSCH or PSSCH DMRS. Consequently, PSCCH DMRS can be used for sensing operations to select SL-PRS resources and no change to the sensing procedure is necessary.

Proposal 12: PSCCH DMRS is used for RSRP measurements for sensing when (re)selecting SL-PRS transmission resources in a dedicated resource pool.

In sidelink communications, higher layers provide a packet delay budget. Lower layers will use the packet delay budget to determine a resource selection window and then report available resources to MAC. Since SL-PRS are not a packet and do not have an associated packet delay budget, a replacement is needed to determine which resources lower layers use to report resource availability to higher layers. This can be done by higher layers directly indicating the resource selection window to lower layers. This modification would also allow higher layers to order transmissions, e.g. in an RTT session.

Proposal 13: Higher layers directly indicate the SL-PRS resource selection window to lower layers in both a dedicated resource pool and a shared resource pool.

While it would simpler procedurally to limit SL-PRS to a single priority value, this would make it difficult to meet potential QoS requirements for different applications. Hence, we propose to allow multiple priorities in the resource pool.

Proposal 14: Multiple L1 SL-PRS priority are allowed in a dedicated resource pool.

Sidelink communications have a (pre-)configured sensing window size with two values: 100ms and 1100ms to accommodate transmissions when periodic reservations are disabled or enabled, respectively. These values are sufficient for the dedicated resource pool, unless new reservations periodicities larger than 1000ms are introduced.

Proposal 15: For the sensing window length (T0) use the legacy (pre-)configuration with values (100 msec, 1100 msec).

In our view, there is no need to modify RSRP thresholds, step size, target resource ratio, pre-emption, or reevaluation.

Proposal 16: In a dedicated resource pool, the initial S-RSRP threshold & stepsize, target resource ratio X(%), pre-emption, and re-evaluation are reused from sidelink communications.

The Rel-17 inter-UE coordination framework cannot be reused for a dedicated resource pool due to lack of cross resource pool indication. This is not a simple issue of missing fields in signaling since the same resource pool could have different indices at different UEs, for example if the UEs have a different number of resource pools or the UEs are configured by different networks. For now, we propose to postpone inter-UE coordination discussions to focus on essential aspects of system functionality.

Proposal 17: Deprioritize inter-UE coordination in the dedicated resource pool until essential aspects of sidelink positioning are complete.

Triggering, Activation/Deactivation, and Configuration of SL-PRS

## Triggering

RAN1 agreed to support SL-PRS transmission triggering by the UE’s higher layers, this could also include receiving a higher layer message from another UE, and to receive a lower layer request that is passed to the UE’s higher layers:

Agreement In Scheme 2, with regards to the triggering of SL-PRS, - Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers - Working assumption: Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A. - Up to UE-B’s own higher layers to transmit SL-PRS in response to the lower layer request from UE-A - FFS: Lower layer signaling corresponds to SCI, MAC-CE, or SL-PRS

There is an FFS what the lower-layer request could with three options: SCI, MAC-CE, or SL-PRS. In our view, SL-PRS is a good candidate and does not incur overhead. In the case of RTT, the UE reports the reception of SL-PRS to its higher layers, which then could trigger the transmission of a response SL-PRS. SCI and MAC-CE do not provide any significant advantage over the already supported higher layer transmission and have overhead compared to SL-PRS. Therefore, we do not see the need to support them.

Proposal 18: Only SL-PRS reception is supported as a lower-layer request for SL-PRS transmission.

## Activation/Deactivation

Activation and deactivation starts and stops a periodic or semi-persistent process. In sidelink, there is no distinction between periodic and semi-persistent processes and they both are considered periodic processes. In Mode 1 resource allocation, periodic transmissions are achieved by configured grant type 1 and type 2, which are RRC and DCI activated, respectively. In Mode 2, there is no external activation or deactivation of periodic transmissions and the decision whether to use a periodic transmission or an aperiodic one is up to the UE’s higher layers. In Uu positioning, the LMF starts/stops periodic PRS transmissions ondemand at the gNB.

For Scheme 2 SL positioning operation, activation/deactivation by higher layers is a straightforward solution that follows existing sidelink design. Like triggering, this would also address issues related to security, scenario requirements, and inter-UE interaction.

Scheme 1 SL positioning operation can reuse the higher-layer activation/deactivation design from Scheme 2 to provide a unified framework, minimizing the differences between the two schemes. It could become part of the higher-layer triggering procedure as well.

Higher-layer activation and deactivation of SL-PRS periodic transmission is a suitable, unified solution for both Scheme 1 and Scheme 2 positioning operation. Hence, we propose to adopt it:

Proposal 19: Support SL-PRS transmission activation/deactivation at the physical layer by the UE’s own higher layers only.

## Configuration

In sidelink, there are three methods for providing configuration to a UE: RRC pre-configuration, RRC configuration from the network, and PC5-RRC configuration from another UE. The first two are interchangeable from a PHY point of view and referred to as (pre-)configuration in RAN1 agreements and specifications. PC5-RRC configuration is limited to unicast and hence introduced unnecessary limitations to positioning operations. The limitations of using PC5-RRC as the sole configuration method and transport as discussed in detail in our RAN2 contribution.

Observation 1: PC5-RRC configuration is limited to unicast and could only be applied for that cast type. Hence it is not suitable as the sole method for SL-PRS configuration.

Proposal 20: Higher layer signaling (SLPP) is used for configuration of SL-PRS.

The Rel-17 inter-UE coordination framework cannot be used for the dedicated resource pool due to lack of cross resource singaling ability. This issue goes beyond lack of fields in signaling since the same resource pool could have different indices at different UEs, for example if the UEs have a different number of resource pools or the UEs are configured by different networks.

Reservations and Signaling Details

RAN1 agreed to use at least SCI for indicating SL-PRS reservations or resources:

Agreement With regards to SL signaling of the reservation/indication of SL-PRS resource(s) for dedicated resource pool and shared resource pool (if supported) for positioning: - Option A.1: SCI can be used for reserving/indicating one or more SL-PRS resource(s) - Note: This does NOT mean that only SCI is being used. There can still be higher layer signaling for the purpose of indicating a part of SL-PRS configuration. - FFS: Whether SCI is single stage SCI or two stage SCI - FFS: SL-MAC-CE or other higher-layer signaling reservation/indication Agreement With regards to the SCI signaling in a shared resource pool, in addition to SL PRS transmission, the UE transmits - Opt. 1: SCI1-A & a 2nd stage SCI format are used for SL-PRS indication - FFS: Details including a new or existing 2nd stage SCI

In addition to indicating SL-PRS resources in SCI-1, we propose to also include them in higher-layer messages (SLPP) that can be sent on a sidelink communication resource pool prior to the SL-PRS transmission when possible. This allows the receiver to anticipate when SL-PRS would arrive. In addition to SL-PRS time and frequency resources, higher layer message would include positioning method, periodicity, comb size and offset, scrambling seed parameter, cast type, QoS, and identification information. This also reduces the size of required L1 messages.

Proposal 21: Higher layers message (SLPP) contain positioning method, SL-PRS periodicity, comb size, comb offset, scrambling seed parameter, cast type, QoS, and identification information. In addition, they can include SL-PRS time and frequency resources.

In a dedicated resource pool and as discussed earlier, an SCI-1 would accompany SL-PRS in the same transmission. This would be a new SCI-1 format that contains SL-PRS time and frequency allocation, periodicity, and identifiers associating the SL-PRS with the positioning session and SL-PRS configuration index. Using an index into a list of configurations provided by SLPP/LPP instead of explicitly including all the configuration fields maintains a limit SCI format size.

Proposal 22: In a dedicated resource pool, the new SCI includes SL-PRS time and frequency allocation, periodicity, positioning session identifiers, and an index to an SL-PRS configuration provided by SLPP/LPP.

In the shared resource pool, and to maintain backward compatibility, part of this information would be included in a new SCI-2 format.

Proposal 23: In a shared resource pool, legacy SCI-1A format is used to indicate transmission time-frequency resources and periodicity.

Proposal 24: In a shared resource pool, a new SCI-2 format is introduced and includes at least identifiers associating the SL-PRS with the positioning session (e.g. SLPP UE ID) and SL-PRS configuration index.

# Congestion Control

Congestion control is an essential item required for proper functionality of a sidelink system. Without it, the system cannot adapt to changes in load. In a shared resource pool, the existing congestion control mechanism can be reused with minimal changes, e.g. including SL-PRS slots in the RSSI measurements. Similarly, in a dedicated resource pool, the existing mechanism can be reused, potentially with minor updates to measurements.

Agreement For Scheme 2 SL-PRS resource allocation, specify congestion control mechanisms using the existing congestion control mechanisms as a starting point. - Study at least the following aspects on potential changes over the existing congestion control mechanisms: - CBR and CR definition for SL-PRS - Which parameters of a SL-PRS configuration could be impacted by the congestion control mechanism, the mapping between congestion measurements, SL-PRS priority and SL-PRS parameters - CR and CBR measurement time window - Congestion control processing time - Number of CBR ranges - Whether any proposed changes could be applicable to shared resource pools in addition to the dedicated resource pool

Proposal 25: Reuse sidelink communications CBR and CR based congestion control in the dedicated and shared resource pools.

# Conclusions

Observation 1: PC5-RRC configuration is limited to unicast and could only be applied for that cast type. Hence it is not suitable as the sole method for SL-PRS configuration.

Proposal 1: Only SL-PRS and PSCCH which carries SCI associated with SL-PRS transmission(s) are included in the dedicated resource pool.

Proposal 2: Regarding PSSCH and SL-PRS multiplexing in the shared resource pool, TDMing and FDMing of PSSCH and SL-PRS is supported with PSSCH rate-matched around SL PRS REs.

Proposal 3: Regarding PSCCH and SL-PRS multiplexing in the shared resource pool, only TDMing is supported.

Proposal 4: In the shared resource pool, PSSCH in a slot with SL-PRS is used for 2nd SCI and SL-SCH.

Proposal 5: In a shared resource pool, a resource used for transmission of SL-PRS is defined as a set of contiguous sub-channels in a slot, i.e. the existing definition for a sidelink resource is reused.

Proposal 6: In a dedicated resource pool, a resource used for transmission of SL-PRS is defined as a set of contiguous sub-channels in a slot with a comb size, comb offset, and TDM partition.

Proposal 7: Sub-slot TDM SL-PRS resource partitioning, when supported by the UE and enabled for the dedicated resource pool, is semi-static.

Proposal 8: LMF provides the gNB with requisite SL-PRS properties. gNB responds to the LMF with configuration and allocation information.

Proposal 9: DCI grants, configured sidelink grant Type 1, and configured sidelink grant Type 2 from Mode 1 are reused for Scheme 1 resource allocation.

Proposal 10: A receiver UE obtains SL-PRS configuration from the transmitter UE via SLPP messaging.

Proposal 11: A transmitter UE transmits SCI reserving resources for the accompanying SL-PRS similar to sidelink communications.

Proposal 12: PSCCH DMRS is used for RSRP measurements for sensing when (re)selecting SL-PRS transmission resources in a dedicated resource pool.

Proposal 13: Higher layers directly indicate the SL-PRS resource selection window to lower layers in both a dedicated resource pool and a shared resource pool.

Proposal 14: Multiple L1 SL-PRS priority are allowed in a dedicated resource pool.

Proposal 15: For the sensing window length (T0) use the legacy (pre-)configuration with values (100 msec, 1100 msec).

Proposal 16: In a dedicated resource pool, the initial S-RSRP threshold & stepsize, target resource ratio X(%), pre-emption, and re-evaluation are reused from sidelink communications.

Proposal 17: Deprioritize inter-UE coordination in the dedicated resource pool until essential aspects of sidelink positioning are complete.

Proposal 18: Only SL-PRS reception is supported as a lower-layer request for SL-PRS transmission.

Proposal 19: Support SL-PRS transmission activation/deactivation at the physical layer by the UE’s own higher layers only.

Proposal 20: Higher layer signaling (SLPP) is used for configuration of SL-PRS.

Proposal 21: Higher layers message (SLPP) contain positioning method, SL-PRS periodicity, comb size, comb offset, scrambling seed parameter, cast type, QoS, and identification information. In addition, they can include SL-PRS time and frequency resources.

Proposal 22: In a dedicated resource pool, the new SCI includes SL-PRS time and frequency allocation, periodicity, positioning session identifiers, and an index to an SL-PRS configuration provided by SLPP/LPP.

Proposal 23: In a shared resource pool, legacy SCI-1A format is used to indicate transmission time-frequency resources and periodicity.

Proposal 24: In a shared resource pool, a new SCI-2 format is introduced and includes at least identifiers associating the SL-PRS with the positioning session (e.g. SLPP UE ID) and SL-PRS configuration index.

Proposal 25: Reuse sidelink communications CBR and CR based congestion control in the dedicated and shared resource pools.

- References

- RP-223549, “New WID on Expanded and Improved NR Positioning,” Intel Corporation, CATT, Ericsson. RAN #98-e.

- Chair’s notes, RAN1 #109-e, Electronic meeting.

- Chair’s notes, RAN1 #110, Toulouse.

- Chair’s notes, RAN1 #111, Toulouse.

- R1- 2301417, “Reference Signal Design for SL Positioning,” Qualcomm Inc. RAN1 #112, Athens.

- 3GPP TS 38.214, “Physical layer procedures for data (Release 17),” 3GPP.