3GPP TSG RAN WG1 #113		R1-2305404

Incheon, Korea, May 22nd – May 26th, 2023

Source:	OPPO

Title:	DMRS enhancement for Rel-18 MIMO

Agenda Item:	9.1.3.1

Document for:	Discussion and Decision

Introduction

In RAN1#112bis meeting, Rel-18 DMRS enhancement was discussed with the following agreements/conclusions:

Agreement

For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, for S-DCI based M-TRP,

- Support all rows of DMRS port combinations and Number of DMRS CDM group(s) without data for Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH for S-TRP, in addition to row 30 for 1CW in RAN1#112 agreement.

  - If MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) is introduced to certain row(s) for S-TRP, the MU-restriction is applied to the same row(s) for S-DCI based M-TRP.

Agreement

For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH, for S-DCI based M-TRP case, support all the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data.

- All rows for Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH for S-TRP.

  - If MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) is introduced to certain row(s) for S-TRP, the MU-restriction is applied to the same row(s) for S-DCI based M-TRP.

- For one CW, add new row 68 in Table 7.3.1.2.2-2A-X.

  - For row 68, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group).

Table 7.3.1.2.2-2A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  |
| --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| … | … | … | … |
| 68 | 2 | 0,2,3 | 1 |

Agreement

For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH, for S-DCI based M-TRP case, support all the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data.

- All rows for Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH for S-TRP.

  - If MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) is introduced to certain row(s) for S-TRP, the MU-restriction is applied to the same row(s) for S-DCI based M-TRP.

- For one CW, add new row 60 in Table 7.3.1.2.2-3A-X.

  - For row 60, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group).

Table 7.3.1.2.2-3A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| … | … | … |
| 60 | 2 | 0,2,3 |

Agreement

For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH, for S-DCI based M-TRP case, support all the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data.

- All rows for Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH for S-TRP.

  - If MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) is introduced to certain row(s) for S-TRP, the MU-restriction is also applied to the same row(s) for S-DCI based M-TRP.

- For one CW, add new row 128 in Table 7.3.1.2.2-4A-X.

  - For row 128, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group).

Table 7.3.1.2.2-4A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  |
| --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| … | … | … | … |
| 128 | 2 | 0,2,3 | 1 |

Agreement

For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case,

  - For 2 CWs,

      - Alt.1: Confirm the working assumption in RAN1#112 with modification (in red).

        - Alt.3-1: Support at least row 0-3 for 2 CWs in Table 4-0.

Table 4-0: DMRS ports for 2CWs.

| Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 2 | 0,1,2,3,8 |
| 1 | 2 | 0,1,2,3,8,10 |
| 2 | 2 | 0,1,2,3,8,9,10 |
| 3 | 2 | 0,1,2,3,8,9,10,11 |
| [4] | [2] | [0,1,2,3,10] |
| [5] | [2] | [0,1,8,2,3,10] |
| [6] | [2] | [0,1,8,2,3,10,11] |
| [7] | [2] | [0,1,8,9,2,3,10,11] |
| [8] | [2] | [0,2,3,8,9] |
| [9] | [2] | [0,1,2,3,8,9] |

FFS: Additional rows (rows 4~9) if there is technical justification.

Agreement

For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case,

  - For 1 CW,

    - Do not support row 21-22

    - FFS: Whether to support row 23

| One Codeword: Codeword 0 enabled, ]Codeword 1 disabled |  |  |
| --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 21 | [2] | [8-10] |
| 22 | [2] | [8-11] |

Agreement

For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, support all rows of DMRS port combinations and Number of DMRS CDM group(s) without data in Table 7.3.1.2.2-2-X.

- FFS: For row 9-11, 24-30, 55-60, and 81-83 (if agreed) in one CW, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) or UE capability.

- FFS: The total number of rows for eType1 DMRS ports with maxLength =2 for PDSCH at least for S-TRP case does not exceed 64.

Table 7.3.1.2.2-2-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | [0 | 2 | 0-4 | 2] |
| 1 | 1 | 1 | 1 | [1 | 2 | 0,1,2,3,4,6 | 2] |
| 2 | 1 | 0,1 | 1 | [2 | 2 | 0,1,2,3,4,5,6 | 2] |
| 3 | 2 | 0 | 1 | [3 | 2 | 0,1,2,3,4,5,6,7 | 2] |
| 4 | 2 | 1 | 1 | 4 | 2 | 0,1,2,3,8 | 1 |
| 5 | 2 | 2 | 1 | 5 | 2 | 0,1,2,3,8,10 | 1 |
| 6 | 2 | 3 | 1 | 6 | 2 | 0,1,2,3,8,9,10 | 1 |
| 7 | 2 | 0,1 | 1 | 7 | 2 | 0,1,2,3,8,9,10,11 | 1 |
| [8 | 2 | 2,3 | 1] | [8 | 1 | 0,1,4,5,8 | 2] |
| [9 | 2 | 0-2 | 1] | [9 | 1 | 0,1,4,5,8,12 | 2] |
| [10 | 2 | 0-3 | 1] | [10 | 1 | 0,1,4,5,8,9,12 | 2] |
| [11 | 2 | 0,2 | 1] | [11 | 1 | 0,1,4,5,8,9,12,13 | 2] |
| 12 | 2 | 0 | 2 | [12 | 2 | 0,1,4,5,8 | 2] |
| 13 | 2 | 1 | 2 | [13 | 2 | 0,1,4,5,8,12 | 2] |
| 14 | 2 | 2 | 2 | [14 | 2 | 0,1,4,5,8,9,12 | 2] |
| 15 | 2 | 3 | 2 | [15 | 2 | 0,1,4,5,8,9,12,13 | 2] |
| 16 | 2 | 4 | 2 | [16 | 2 | 2,3,6,7,10 | 2] |
| 17 | 2 | 5 | 2 | [17 | 2 | 2,3,6,7,10,14 | 2] |
| 18 | 2 | 6 | 2 | [18 | 2 | 2,3,6,7,10,11,14 | 2] |
| 19 | 2 | 7 | 2 | [19 | 2 | 2,3,6,7,10,11,14,15 | 2] |
| 20 | 2 | 0,1 | 2 | [20 | 2 | 0,1, 2,3,10 | 1] |
| 21 | 2 | 2,3 | 2 | [21 | 2 | 0,1,8,2,3,10 | 1] |
| 22 | 2 | 4,5 | 2 | [22 | 2 | 0,1,8, 2,3,10,11 | 1] |
| 23 | 2 | 6,7 | 2 | [23 | 2 | 0,1,8,9,2,3,10,11 | 1] |
| [24 | 2 | 0,4 | 2] | [24 | 1 | 0,1,4,5,12 | 2] |
| [25 | 2 | 2,6 | 2] | [25 | 1 | 0,1,8,4,5,12 | 2] |
| [26 | 2 | 0,1,4 | 2] | [26 | 1 | 0,1,8,4,5,12,13 | 2] |
| [27 | 2 | 2,3,6 | 2] | [27 | 1 | 0,1,8,9,4,5,12,13 | 2] |
| [28 | 2 | 0,1,4,5 | 2] | [28 | 2 | 0,1,4,5,12 | 2] |
| [29 | 2 | 2,3,6,7 | 2] | [29 | 2 | 0,1,8,4,5,12 | 2] |
| [30 | 2 | 0,2,4,6 | 2] | [30 | 2 | 0,1,8,4,5,12,13 | 2] |
| 31 | 1 | 8 | 1 | [31 | 2 | 0,1,8,9,4,5,12,13 | 2] |
| 32 | 1 | 9 | 1 | [32 | 2 | 2,3,6,7,14 | 2] |
| 33 | 1 | 8,9 | 1 | [33 | 2 | 2,3,10,6,7,14 | 2] |
| 34 | 2 | 8 | 1 | [34 | 2 | 2,3,10,6,7,14,15 | 2] |
| 35 | 2 | 9 | 1 | [35 | 2 | 2,3,10,11,6,7,14,15 | 2] |
| 36 | 2 | 10 | 1 | [36 | 2 | 0,2,3,8,9 | 1] |
| 37 | 2 | 11 | 1 | [37 | 2 | 0,1,2,3,8,9 | 1] |
| 38 | 2 | 8,9 | 1 |  |  |  |  |
| 39 | 2 | 10,11 | 1 |  |  |  |  |
| [40 | 2 | 8-10 | 1] |  |  |  |  |
| [41 | 2 | 8-11 | 1] |  |  |  |  |
| [42 | 2 | 8,10 | 1] |  |  |  |  |
| 43 | 2 | 8 | 2 |  |  |  |  |
| 44 | 2 | 9 | 2 |  |  |  |  |
| 45 | 2 | 10 | 2 |  |  |  |  |
| 46 | 2 | 11 | 2 |  |  |  |  |
| 47 | 2 | 12 | 2 |  |  |  |  |
| 48 | 2 | 13 | 2 |  |  |  |  |
| 49 | 2 | 14 | 2 |  |  |  |  |
| 50 | 2 | 15 | 2 |  |  |  |  |
| 51 | 2 | 8,9 | 2 |  |  |  |  |
| 52 | 2 | 10,11 | 2 |  |  |  |  |
| 53 | 2 | 12,13 | 2 |  |  |  |  |
| 54 | 2 | 14,15 | 2 |  |  |  |  |
| [55 | 2 | 8,12 | 2] |  |  |  |  |
| [56 | 2 | 10,14 | 2] |  |  |  |  |
| [57 | 2 | 8,9,12 | 2] |  |  |  |  |
| [58 | 2 | 10,11,14 | 2] |  |  |  |  |
| [59 | 2 | 8,9,12,13 | 2] |  |  |  |  |
| [60 | 2 | 10,11,14,15 | 2] |  |  |  |  |
| 61 | 2 | 8,10,12,14 | 2 |  |  |  |  |
| 62 | 1 | 0,1,8 | 1 |  |  |  |  |
| 63 | 1 | 0,1,8,9 | 1 |  |  |  |  |
| 64 | 2 | 0,1,8 | 1 |  |  |  |  |
| 65 | 2 | 0,1,8,9 | 1 |  |  |  |  |
| 66 | 2 | 2,3,10 | 1 |  |  |  |  |
| 67 | 2 | 2,3,10,11 | 1 |  |  |  |  |
| [69 | 1 | 0,1,8 | 2] |  |  |  |  |
| [70 | 1 | 0,1,8,9 | 2] |  |  |  |  |
| [71 | 1 | 4,5,12 | 2] |  |  |  |  |
| [72 | 1 | 4,5,12,13 | 2] |  |  |  |  |
| [73 | 2 | 0,1,8 | 2] |  |  |  |  |
| [74 | 2 | 0,1,8,9 | 2] |  |  |  |  |
| [75 | 2 | 4,5,12 | 2] |  |  |  |  |
| [76 | 2 | 4,5,12,13 | 2] |  |  |  |  |
| [77 | 2 | 2,3,10 | 2] |  |  |  |  |
| [78 | 2 | 2,3,10,11 | 2] |  |  |  |  |
| [79 | 2 | 6,7,14 | 2] |  |  |  |  |
| [80 | 2 | 6,7,14,15 | 2] |  |  |  |  |
| [81 | 2 | 5,8,9 | 2] |  |  |  |  |
| [82 | 2 | 7,10,11 | 2] |  |  |  |  |
| [83 | 2 | 7,12,13 | 2] |  |  |  |  |

Agreement

For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, support all rows of DMRS port combinations and Number of DMRS CDM group(s) without data in Table 7.3.1.2.2-3-X.

- FFS: For rows 9, 10, 20-23, 33, 34, 44-46, 60-62 (if agreed) in one CW, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) or UE capability.

Table 7.3.1.2.2-3-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=1

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  | Two codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 1 | 0 | 0 | 3 | 0-4 |
| 1 | 1 | 1 | 1 | 3 | 0-5 |
| 2 | 1 | 0,1 | [2 | 3 | 12-16] |
| 3 | 2 | 0 | [3 | 3 | 12-17] |
| 4 | 2 | 1 | 4 | 2 | 0,1,2,3,12 |
| 5 | 2 | 2 | 5 | 2 | 0,1,2,3,12,14 |
| 6 | 2 | 3 | 6 | 2 | 0-3,12-14 |
| 7 | 2 | 0,1 | 7 | 2 | 0-3,12-15 |
| 8 | 2 | 2,3 | [8 | 3 | 0,1,2,3,12] |
| [9 | 2 | 0-2] | [9 | 3 | 0,1,2,3,12,14] |
| [10 | 2 | 0-3] | [10 | 3 | 0-3,12-14] |
| 11 | 3 | 0 | [11 | 3 | 0-3,12-15] |
| 12 | 3 | 1 | [12 | 2 | 0,2,3,12,13] |
| 13 | 3 | 2 | [13 | 2 | 0,1,2,3,14] |
| 14 | 3 | 3 | [14 | 2 | 0,1,12,2,3,14] |
| 15 | 3 | 4 | [15 | 2 | 0,1,12,2,3,14,15] |
| 16 | 3 | 5 | [16 | 2 | 0,1,12,13,2,3,14,15] |
| 17 | 3 | 0,1 | [17 | 3 | 0,1,2,3,14] |
| 18 | 3 | 2,3 | [18 | 3 | 0,1,12,2,3,14] |
| 19 | 3 | 4,5 | [19 | 3 | 0,1,12,2,3,14,15] |
| [20 | 3 | 0-2] | [20 | 3 | 0,1,12,13,2,3,14,15] |
| [21 | 3 | 3-5] |  |  |  |
| [22 | 3 | 0-3] |  |  |  |
| [23 | 2 | 0,2] |  |  |  |
| 24 | 1 | 12 |  |  |  |
| 25 | 1 | 13 |  |  |  |
| 26 | 1 | 12,13 |  |  |  |
| 27 | 2 | 12 |  |  |  |
| 28 | 2 | 13 |  |  |  |
| 29 | 2 | 14 |  |  |  |
| 30 | 2 | 15 |  |  |  |
| 31 | 2 | 12,13 |  |  |  |
| 32 | 2 | 14,15 |  |  |  |
| [33 | 2 | 12-14] |  |  |  |
| [34 | 2 | 12-15] |  |  |  |
| 35 | 3 | 12 |  |  |  |
| 36 | 3 | 13 |  |  |  |
| 37 | 3 | 14 |  |  |  |
| 38 | 3 | 15 |  |  |  |
| 39 | 3 | 16 |  |  |  |
| 40 | 3 | 17 |  |  |  |
| 41 | 3 | 12,13 |  |  |  |
| 42 | 3 | 14,15 |  |  |  |
| 43 | 3 | 16,17 |  |  |  |
| [44 | 3 | 12-14] |  |  |  |
| [45 | 3 | 15-17] |  |  |  |
| [46 | 3 | 12-15] |  |  |  |
| [47 | 2 | 12,14] |  |  |  |
| 48 | 1 | 0,1,12 |  |  |  |
| 49 | 1 | 0,1,12,13 |  |  |  |
| 50 | 2 | 0,1,12 |  |  |  |
| 51 | 2 | 0,1,12,13 |  |  |  |
| 52 | 2 | 2,3,14 |  |  |  |
| 53 | 2 | 2,3,14,15 |  |  |  |
| 54 | 3 | 0,1,12 |  |  |  |
| 55 | 3 | 0,1,12,13 |  |  |  |
| 56 | 3 | 2,3,14 |  |  |  |
| 57 | 3 | 2,3,14,15 |  |  |  |
| 58 | 3 | 4,5,16 |  |  |  |
| 59 | 3 | 4,5,16,17 |  |  |  |
| [60 | 3 | 13,15,17] |  |  |  |
| [61 | 3 | 13,15] |  |  |  |
| [62 | 2 | 13,15] |  |  |  |

Agreement

For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, support all rows of DMRS port combinations and Number of DMRS CDM group(s) without data in Table 7.3.1.2.2-4-X.

- FFS: For rows 9, 10, 20-23, 42-47, 67, 68, 78-80, 100-105, and 153-158 (if agreed) in one CW, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) or UE capability.

Table 7.3.1.2.2-4-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=2

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | [0 | 3 | 0-4 | 1] |
| 1 | 1 | 1 | 1 | [1 | 3 | 0-5 | 1] |
| 2 | 1 | 0,1 | 1 | [2 | 2 | 0,1,2,3,6 | 2] |
| 3 | 2 | 0 | 1 | [3 | 2 | 0,1,2,3,6,8 | 2] |
| 4 | 2 | 1 | 1 | [4 | 2 | 0,1,2,3,6,7,8 | 2] |
| 5 | 2 | 2 | 1 | [5 | 2 | 0,1,2,3,6,7,8,9 | 2] |
| 6 | 2 | 3 | 1 | 6 | 2 | 0,1,2,3,12 | 1 |
| 7 | 2 | 0,1 | 1 | 7 | 2 | 0-3,12,14 | 1 |
| 8 | 2 | 2,3 | 1 | 8 | 2 | 0-3,12-14 | 1 |
| [9 | 2 | 0-2 | 1] | 9 | 2 | 0-3,12-15 | 1 |
| [10 | 2 | 0-3 | 1] | [10 | 3 | 0,1,2,3,12 | 1] |
| 11 | 3 | 0 | 1 | [11 | 3 | 0-3,12,14 | 1] |
| 12 | 3 | 1 | 1 | [12 | 3 | 0-3,12-14 | 1] |
| 13 | 3 | 2 | 1 | [13 | 3 | 0-3,12-15 | 1] |
| 14 | 3 | 3 | 1 | [14 | 1 | 0,1,6,7,12 | 2] |
| 15 | 3 | 4 | 1 | [15 | 1 | 0,1,6,7,12,18 | 2] |
| 16 | 3 | 5 | 1 | [16 | 1 | 0,1,6,7,12,13,18 | 2] |
| 17 | 3 | 0,1 | 1 | [17 | 1 | 0,1,6,7,12,13,18,19 | 2] |
| 18 | 3 | 2,3 | 1 | [18 | 2 | 0,1,6,7,12 | 2] |
| 19 | 3 | 4,5 | 1 | [19 | 2 | 0,1,6,7,12,18 | 2] |
| [20 | 3 | 0-2 | 1] | [20 | 2 | 0,1,6,7,12,13,18 | 2] |
| [21 | 3 | 3-5 | 1] | [21 | 2 | 0,1,6,7,12,13,18,19 | 2] |
| [22 | 3 | 0-3 | 1] | [22 | 2 | 2,3,8,9,14 | 2] |
| [23 | 2 | 0,2 | 1] | [23 | 2 | 2,3,8,9,14,20 | 2] |
| 24 | 3 | 0 | 2 | [24 | 2 | 2,3,8,9,14,15,20 | 2] |
| 25 | 3 | 1 | 2 | [25 | 2 | 2,3,8,9,14,15,20,21 | 2] |
| 26 | 3 | 2 | 2 | [26 | 3 | 0,1,6,7,12 | 2] |
| 27 | 3 | 3 | 2 | [27 | 3 | 0,1,6,7,12,18 | 2] |
| 28 | 3 | 4 | 2 | [28 | 3 | 0,1,6,7,12,13,18 | 2] |
| 29 | 3 | 5 | 2 | [29 | 3 | 0,1,6,7,12,13,18,19 | 2] |
| 30 | 3 | 6 | 2 | [30 | 3 | 2,3,8,9,14 | 2] |
| 31 | 3 | 7 | 2 | [31 | 3 | 2,3,8,9,14,20 | 2] |
| 32 | 3 | 8 | 2 | [32 | 3 | 2,3,8,9,14,15,20 | 2] |
| 33 | 3 | 9 | 2 | [33 | 3 | 2,3,8,9,14,15,20,21 | 2] |
| 34 | 3 | 10 | 2 | [34 | 3 | 4,5,10,11,16 | 2] |
| 35 | 3 | 11 | 2 | [35 | 3 | 4,5,10,11,16,22 | 2] |
| 36 | 3 | 0,1 | 2 | [36 | 3 | 4,5,10,11,16,17,22 | 2] |
| 37 | 3 | 2,3 | 2 | [37 | 3 | 4,5,10,11,16,17,22,23 | 2] |
| 38 | 3 | 4,5 | 2 | [38 | 2 | 0,1,2,3,14 | 1] |
| 39 | 3 | 6,7 | 2 | [39 | 2 | 0,1,12,2,3,14 | 1] |
| 40 | 3 | 8,9 | 2 | [40 | 2 | 0,1,12,2,3,14,15 | 1] |
| 41 | 3 | 10,11 | 2 | [41 | 2 | 0,1,12,13,2,3,14,15 | 1] |
| [42 | 3 | 0,1,6 | 2] | [42 | 3 | 0,1,2,3,14 | 1] |
| [43 | 3 | 2,3,8 | 2] | [43 | 3 | 0,1,12,2,3,14 | 1] |
| [44 | 3 | 4,5,10 | 2] | [44 | 3 | 0,1,12,2,3,14,15 | 1] |
| [45 | 3 | 0,1,6,7 | 2] | [45 | 3 | 0,1,12,13,2,3,14,15 | 1] |
| [46 | 3 | 2,3,8,9 | 2] | [46 | 1 | 0,1,6,7,18 | 2] |
| [47 | 3 | 4,5,10,11 | 2] | [47 | 1 | 0,1,12,6,7,18 | 2] |
| 48 | 1 | 0 | 2 | [48 | 1 | 0,1,12,6,7,18,19 | 2] |
| 49 | 1 | 1 | 2 | [49 | 1 | 0,1,12,13,6,7,18,19 | 2] |
| 50 | 1 | 6 | 2 | [50 | 2 | 0,1,6,7,18 | 2] |
| 51 | 1 | 7 | 2 | [51 | 2 | 0,1,12,6,7,18 | 2] |
| 52 | 1 | 0,1 | 2 | [52 | 2 | 0,1,12,6,7,18,19 | 2] |
| 53 | 1 | 6,7 | 2 | [53 | 2 | 0,1,12,13,6,7,18,19 | 2] |
| 54 | 2 | 0,1 | 2 | [54 | 2 | 2,3,8,9,20 | 2] |
| 55 | 2 | 2,3 | 2 | [55 | 2 | 2,3,14,8,9,20 | 2] |
| 56 | 2 | 6,7 | 2 | [56 | 2 | 2,3,14,8,9,20,21 | 2] |
| 57 | 2 | 8,9 | 2 | [57 | 2 | 2,3,14,15,8,9,20,21 | 2] |
| 58 | 1 | 12 | 1 | [58 | 3 | 0,1,6,7,18 | 2] |
| 59 | 1 | 13 | 1 | [59 | 3 | 0,1,12,6,7,18 | 2] |
| 60 | 1 | 12,13 | 1 | [60 | 3 | 0,1,12,6,7,18,19 | 2] |
| 61 | 2 | 12 | 1 | [61 | 3 | 0,1,12,13,6,7,18,19 | 2] |
| 62 | 2 | 13 | 1 | [62 | 3 | 2,3,8,9,20 | 2] |
| 63 | 2 | 14 | 1 | [63 | 3 | 2,3,14,8,9,20 | 2] |
| 64 | 2 | 15 | 1 | [64 | 3 | 2,3,14,8,9,20,21 | 2] |
| 65 | 2 | 12,13 | 1 | [65 | 3 | 2,3,14,15,8,9,20,21 | 2] |
| 66 | 2 | 14,15 | 1 | [66 | 3 | 4,5,10,11,22 | 2] |
| [67 | 2 | 12-14 | 1] | [67 | 3 | 4,5,16,10,11,22 | 2] |
| [68 | 2 | 12-15 | 1] | [68 | 3 | 4,5,16,10,11,22,23 | 2] |
| 69 | 3 | 12 | 1 | [69 | 3 | 4,5,16,17,10,11,22,23 | 2] |
| 70 | 3 | 13 | 1 |  |  |  |  |
| 71 | 3 | 14 | 1 |  |  |  |  |
| 72 | 3 | 15 | 1 |  |  |  |  |
| 73 | 3 | 16 | 1 |  |  |  |  |
| 74 | 3 | 17 | 1 |  |  |  |  |
| 75 | 3 | 12,13 | 1 |  |  |  |  |
| 76 | 3 | 14,15 | 1 |  |  |  |  |
| 77 | 3 | 16,17 | 1 |  |  |  |  |
| [78 | 3 | 12-14 | 1] |  |  |  |  |
| [79 | 3 | 15-17 | 1] |  |  |  |  |
| [80 | 3 | 12-15 | 1] |  |  |  |  |
| [81 | 2 | 12,14 | 1] |  |  |  |  |
| 82 | 3 | 12 | 2 |  |  |  |  |
| 83 | 3 | 13 | 2 |  |  |  |  |
| 84 | 3 | 14 | 2 |  |  |  |  |
| 85 | 3 | 15 | 2 |  |  |  |  |
| 86 | 3 | 16 | 2 |  |  |  |  |
| 87 | 3 | 17 | 2 |  |  |  |  |
| 88 | 3 | 18 | 2 |  |  |  |  |
| 89 | 3 | 19 | 2 |  |  |  |  |
| 90 | 3 | 20 | 2 |  |  |  |  |
| 91 | 3 | 21 | 2 |  |  |  |  |
| 92 | 3 | 22 | 2 |  |  |  |  |
| 93 | 3 | 23 | 2 |  |  |  |  |
| 94 | 3 | 12,13 | 2 |  |  |  |  |
| 95 | 3 | 14,15 | 2 |  |  |  |  |
| 96 | 3 | 16,17 | 2 |  |  |  |  |
| 97 | 3 | 18,19 | 2 |  |  |  |  |
| 98 | 3 | 20,21 | 2 |  |  |  |  |
| 99 | 3 | 22,23 | 2 |  |  |  |  |
| [100 | 3 | 12,13,18 | 2] |  |  |  |  |
| [101 | 3 | 14,15,20 | 2] |  |  |  |  |
| [102 | 3 | 16,17,22 | 2] |  |  |  |  |
| [103 | 3 | 12,13,18,19 | 2] |  |  |  |  |
| [104 | 3 | 14,15,20,21 | 2] |  |  |  |  |
| [105 | 3 | 16,17,22,23 | 2] |  |  |  |  |
| 106 | 1 | 12 | 2 |  |  |  |  |
| 107 | 1 | 13 | 2 |  |  |  |  |
| 108 | 1 | 18 | 2 |  |  |  |  |
| 109 | 1 | 19 | 2 |  |  |  |  |
| 110 | 1 | 12,13 | 2 |  |  |  |  |
| 111 | 1 | 18,19 | 2 |  |  |  |  |
| 112 | 2 | 12,13 | 2 |  |  |  |  |
| 113 | 2 | 14,15 | 2 |  |  |  |  |
| 114 | 2 | 18,19 | 2 |  |  |  |  |
| 115 | 2 | 20,21 | 2 |  |  |  |  |
| 116 | 1 | 0,1,12 | 1 |  |  |  |  |
| 117 | 1 | 0,1,12,13 | 1 |  |  |  |  |
| 118 | 2 | 0,1,12 | 1 |  |  |  |  |
| 119 | 2 | 0,1,12,13 | 1 |  |  |  |  |
| 120 | 2 | 2,3,14 | 1 |  |  |  |  |
| 121 | 2 | 2,3,14,15 | 1 |  |  |  |  |
| 122 | 3 | 0,1,12 | 1 |  |  |  |  |
| 123 | 3 | 0,1,12,13 | 1 |  |  |  |  |
| 124 | 3 | 2,3,14 | 1 |  |  |  |  |
| 125 | 3 | 2,3,14,15 | 1 |  |  |  |  |
| 126 | 3 | 4,5,16 | 1 |  |  |  |  |
| 127 | 3 | 4,5,16,17 | 1 |  |  |  |  |
| [129 | 1 | 0,1,12 | 2] |  |  |  |  |
| [130 | 1 | 0,1,12,13 | 2] |  |  |  |  |
| [131 | 1 | 6,7,18 | 2] |  |  |  |  |
| [132 | 1 | 6,7,18,19 | 2] |  |  |  |  |
| [133 | 2 | 0,1,12 | 2] |  |  |  |  |
| [134 | 2 | 0,1,12,13 | 2] |  |  |  |  |
| [135 | 2 | 6,7,18 | 2] |  |  |  |  |
| [136 | 2 | 6,7,18,19 | 2] |  |  |  |  |
| [137 | 2 | 2,3,14 | 2] |  |  |  |  |
| [138 | 2 | 2,3,14,15 | 2] |  |  |  |  |
| [139 | 2 | 8,9,20 | 2] |  |  |  |  |
| [140 | 2 | 8,9,20,21 | 2] |  |  |  |  |
| [141 | 3 | 0,1,12 | 2] |  |  |  |  |
| [142 | 3 | 0,1,12,13 | 2] |  |  |  |  |
| [143 | 3 | 6,7,18 | 2] |  |  |  |  |
| [144 | 3 | 6,7,18,19 | 2] |  |  |  |  |
| [145 | 3 | 2,3,14 | 2] |  |  |  |  |
| [146 | 3 | 2,3,14,15 | 2] |  |  |  |  |
| [147 | 3 | 8,9,20 | 2] |  |  |  |  |
| [148 | 3 | 8,9,20,21 | 2] |  |  |  |  |
| [149 | 3 | 4,5,16 | 2] |  |  |  |  |
| [150 | 3 | 4,5,16,17 | 2] |  |  |  |  |
| [151 | 3 | 10,11,22 | 2] |  |  |  |  |
| [152 | 3 | 10,11,22,23 | 2] |  |  |  |  |
| [153 | 3 | 7,12,13 | 2] |  |  |  |  |
| [154 | 3 | 9,14,15 | 2] |  |  |  |  |
| [155 | 3 | 11,16,17 | 2] |  |  |  |  |
| [156 | 3 | 9,18,19 | 2] |  |  |  |  |
| [157 | 3 | 18,19,20 | 2] |  |  |  |  |
| [158 | 3 | 21,22,23 | 2] |  |  |  |  |

Conclusion

No consensus to support MAC CE based switching between Rel.15 DMRS ports and Rel.18 DMRS ports for PDSCH

Agreement

For > 4 layers PUSCH with Rel.18 eType 1/eType 2 DMRS ports, support at least the same DMRS port combination(s) as that for rank = 5,6,7,8 for PDSCH with Rel.18 eType 1/eType 2 DMRS ports at least for full or non-coherent UL codebook based PUSCH and non-codebook based PUSCH.

Agreement

Confirm the following Working Assumption in RAN1#112 at least for NCB based PUSCH:

- To support PUSCH with rank = 5-8, support the following for enhancement of DMRS port allocation tables.

  - Option 1: Separate DMRS ports tables for rank 5,6,7,8 for each of eType1/eType2 and maxLength=1/2 (similar to the current UL DMRS ports table).

    - FFS: whether/how to reuse the reserved field in antenna ports field for other purposes can be discussed in AI9.1.4.2 [or AI9.1.3.1].

- Note: The above Working Assumption for CB based PUSCH may be confirmed later.

Working Assumption

- Adopt Table 7.3.1.1.2-12B/13B/14B/15B/16B/17B/20B/21B/22B/23B to support signalling >4 ranks PUSCH with Rel-15 DMRS ports at least for full or non-coherent UL codebook based PUSCH and non-codebook based PUSCH.

- FFS: Whether/how some of bits in the antenna ports field can be reused for other purpose for >4 ranks PUSCH.

Table 7.3.1.1.2-12B: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-4 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-13B: Antenna port(s), transform precoder is disabled, dmrs-Type= 1, maxLength=2, rank = 6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,6 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-14B: Antenna port(s), transform precoder is disabled, dmrs-Type= 1, maxLength=2, rank = 7

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,5,6 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-15B: Antenna port(s), transform precoder is disabled, dmrs-Type= 1, maxLength=2, rank = 8

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,4,5,6,7 | 2 |
| 1-15 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-16B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=1, rank=5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 3 | 0-4 |
| 1-15 | Reserved | Reserved |

Table 7.3.1.1.2-17B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=1, rank=6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 3 | 0-5 |
| 1-15 | Reserved | Reserved |

Table 7.3.1.1.2-20B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=5

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 3 | 0-4 | 1 |
| 1 | 2 | 0,1,2,3,6 | 2 |
| 12-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-21B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=6

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 3 | 0-5 | 1 |
| 1 | 2 | 0,1,2,3,6,8 | 2 |
| 2-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-22B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=7

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,6,7,8 | 2 |
| 1-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-23B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=8

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0,1,2,3,6,7,8,9 | 2 |
| 1-31 | Reserved | Reserved | Reserved |

Agreement

For RAN1#112 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PUSCH.

- Support row 7 for rank2, row1 for rank3, row 1 for rank4.

Table 7.3.1.1.2-9-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 7 | 2 | 9,11 |

Table 7.3.1.1.2-10-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 1 | 2 | 8-10 |

Table 7.3.1.1.2-11-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 1 | 2 | 8-11 |

Agreement

For Rel.18 eType1/eType2 DMRS ports with maxLength=1/2 for PDSCH/PUSCH, if Rel.18 eType1/eType2 DMRS ports is configured by RRC, the DCI size of antenna ports field in DCI format 1_1/1_2/0_1/0_2 is increased by at least 1-bit from Rel.17.

- Note: it does not preclude future possibility to support more than 1-bit, if RAN1 agree the necessity.

Conclusion

For MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports,

- For PUSCH, there is no restriction.

Agreement

For 8Tx PUSCH, specify the factor related to PUSCH to PTRS power ratio per layer per RE () based on the following principles.

- Principle 1: When the ptrs-Power configures 01, the factor () is 10log10(L), where L is the total number of PUSCH layers.

- Principle 2: When the ptrs-Power configures 00, the factor () is determined as the following

  - Principle 2.1: For fully coherent TPMIs, the factor () is 10log10(L), where L is the total number of PUSCH layers.

  - Principle 2.2: For non-coherent TPMIs, the factor () is 10log10(Qp), where Qp is the number of PTRS ports scheduled to the UE.

  - Principle 2.3: For non-codebook PUSCH, the factor () is 10log10(Qp), where Qp is the number of PTRS ports scheduled to the UE.

  - FFS: The factor () for partial coherent TPMIs

Agreement

For partial/non-coherent PUSCH with rank=5-8 transmission (i.e. non of the CWs is disabled) with one PTRS port, PTRS-DMRS association for PUSCH is the following.

- The size of PTRS-DMRS association field is 2-bit in DCI format 0_1/0_2.

  - The CW with the higher MCS is selected in case of two CWs.

    - Note: in case of PUSCH retransmission, the initial MCS is used for CW selection.

  - If the MCS is the same for two CWs, the PTRS port is associated with the first CW.

Table 7.3.1.1.2-25: PTRS-DMRS association for UL PTRS port 0

| Value | DMRS port |
| --- | --- |
| 0 | 1st scheduled DMRS port with the CW |
| 1 | 2nd scheduled DMRS port with the CW |
| 2 | 3rd scheduled DMRS port with the CW |
| 3 | 4th scheduled DMRS port with the CW |

Conclusion

For “The CW with the higher MCS” in RAN1#112 agreement of PTRS-DMRS association field for full-coherent PUSCH with rank=5~8 PUSCH with one port PTRS, following is clarified.

- Note: in case of PUSCH retransmission, the initial MCS is used for CW selection.

Agreement

For two PTRS ports for partial/non-coherent PUSCH, PTRS-DMRS association for PUSCH with up to 8 layers is down selected from the following.

- Alt.1: The size of PTRS-DMRS association field is 4-bit in DCI format 0_1/0_2.

Table 1: PTRS-DMRS association for UL PTRS ports 0 and 1

| Value of MSB | DMRS port | Value of LSB | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 | 1 | 2nd DMRS port which shares PTRS port 1 |
| 2 | 3rd DMRS port which shares PTRS port 0 | 2 | 3rd DMRS port which shares PTRS port 1 |
| 3 | 4th DMRS port which shares PTRS port 0 | 3 | 4th DMRS port which shares PTRS port 1 |

- Alt.2: The size of PTRS-DMRS association field is 2-bit in DCI format 0_1/0_2.

  - The CW with the higher MCS is selected in case of two CWs.

  - If the MCS is the same for two CWs, the PTRS port is associated with the first CW.

Table 2: PTRS-DMRS association for UL PTRS ports 0 and 1

| Value of MSB | DMRS port | Value of LSB | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 | 1 | 2nd DMRS port which shares PTRS port 1 |

- Alt.3: The size of PTRS-DMRS association field is 2-bit in DCI format 0_1/0_2.

  - For PUSCH with rank 5-8, 2-bit of antenna ports field is reused in addition to 2-bit PTRS-DMRS association in DCI format 0_1/0_2, and total 4-bit is used for PTRS-DMRS association.

Table 1: PTRS-DMRS association for UL PTRS ports 0 and 1

| Value of MSB | DMRS port | Value of LSB | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 | 1 | 2nd DMRS port which shares PTRS port 1 |
| 2 | 3rd DMRS port which shares PTRS port 0 | 2 | 3rd DMRS port which shares PTRS port 1 |
| 3 | 4th DMRS port which shares PTRS port 0 | 3 | 4th DMRS port which shares PTRS port 1 |

- Alt.4: The size of PTRS-DMRS association field is 2-bit in DCI format 0_1/0_2.

Table 2: PTRS-DMRS association for UL PTRS ports 0 and 1

| Value of MSB | DMRS port | Value of LSB | DMRS port |
| --- | --- | --- | --- |
| 0 | 1st DMRS port which shares PTRS port 0 | 0 | 1st DMRS port which shares PTRS port 1 |
| 1 | 2nd DMRS port which shares PTRS port 0 | 1 | 2nd DMRS port which shares PTRS port 1 |

In this contribution, we further discuss the details for DMRS enhancement in Rel-18.

Discussion

  - Enhancement for more orthogonal DMRS ports

eType1 PDSCH DMRS with maxLength=1

For Rel-18 eType1 DMRS with maxLength=1 for PDSCH, whether rows 4~9 are supported for 2 CWs needs further study. For some UE implementation (e.g. per CW channel estimation), mapping the layers within one CW to one CDM group can simplify the implementation and reduce the detection latency. However, the legacy port combinations without this restriction are already supported and mandated to UE. Considering the port indication is dynamical, it is impossible to implement two channel estimators at UE with dynamic switching. Then to support the legacy combinations, the legacy channel estimator should anyway be applied and the benefit of one-CW-one-CDM-group mapping cannot be obtained. In this case, the benefit of Row 4-9 cannot be justified.

Proposal 1: For Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, Row 4-9 for two CWs is not supported.

eType1 PDSCH DMRS with maxLength=2

One essential issue for DMRS port indication is whether MU-restriction or UE capability is needed for some rows with multiple CDM groups. In our understanding, the MU-restriction is much more necessary in Rel-18 than in Rel-15 since more ports are introduced in one CDM group and UE needs to estimate interference from more DMRS ports than in Rel-15. For DMRS ports in more than one CDM groups, or associated with more than one TD-OCC codes in one CDM group, we have the following observations for difference cases:

- Case 1: If the DMRS ports are legacy ports with MU-restriction in Rel-15, the MU-restriction should be maintained in Rel-18.

- Case 2: If the DMRS ports are legacy ports without MU-restriction in Rel-15, additional MU-restriction similar to Case 1 can be specified in Rel-18.

- Case 3: If the DMRS ports are new DMRS ports in Rel-18, MU-restriction is also preferred considering UE complexity. Otherwise, at least UE capability for these port combinations should be introduced.

Based on the above observation, we have the following considerations for Rel-18 eType1 DMRS with maxLength=2 for PDSCH:

- The signaling overhead should be considered for both S-TRP and m-TRP case. Based on our analysis, 64 Rows are sufficient for both cases.

- For single CW case, legacy port combinations, e.g. Row 9-11, 24-30, can be supported with MU-MIMO restriction.

- For single CW case, Row 55 and 56 can be removed to avoid additional signaling overhead. Row 55-56 is equivalent to Row 24-25, and cannot provide any additional use case.

- For two CWs case, Row 8-11, Row 24-27 and Row 36-37 are not needed, which have similar overhead and performance as Row 4-7.

- For two CWs, the use case of Row 12-19 and Row 28-35 is also unclear to us, considering a UE with two CWs is not likely to be scheduled with MU-MIMO.

Proposal 2: For Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH

- The total number of rows does not exceed 64 for S-TRP and m-TRP case.

- For single CW, Row 9-11, 24-30 can be supported with MU-MIMO restriction.

- For single CW, Row 55 and 56 can be removed.

- For two CWs, Row 8-19, Row 24-37 can be removed.

eType2 PDSCH DMRS with maxLength=1

Similar to Rel-18 eType1 DMRS with maxLength=1 for PDSCH, we have the following considerations for Rel-18 eType2 DMRS with maxLength=1:

- The signaling overhead should be considered for both S-TRP and m-TRP case. Based on our analysis, 64 Rows are sufficient for both cases.

- For single CW case, legacy port combinations, e.g. 9-10, 20-23, can be supported with MU-MIMO restriction.

- For single CW case, Row 33-34 and Row 44-46 can be removed, which is for SU-MIMO scenarios only.

- For two CWs, the use case of Row 2-3 and Row 8-12is also unclear to us, considering a UE with two CWs is not likely to be scheduled with MU-MIMO.

Proposal 3: For Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH

- The total number of rows does not exceed 64 for S-TRP and m-TRP case.

- For single CW, Row 9-10, 20-23 can be supported with MU-MIMO restriction.

- For single CW, Row 33-34 and Row 44-46 can be removed.

- For two CWs, Row 2-3 and Row 8-12 is can be removed.

eType2 PDSCH DMRS with maxLength=2

Similar to Rel-18 eType1 DMRS with maxLength=2 for PDSCH, we have the following considerations for Rel-18 eType2 DMRS with maxLength=2:

- The signaling overhead should be considered for both S-TRP and m-TRP case. Based on our analysis, 128 Rows are sufficient for both cases.

- For single CW, Row 67-68, 78-80, 100-105, and 153-158 can be removed to avoid additional overhead. They are equivalent to previous rows, and cannot provide any additional use case.

- For two CWs, Row 14-17, Row 46-49 are not needed, which have similar overhead and performance as Row 6-9.

- For two CWs, considering a UE with two CWs is not likely to be scheduled with MU-MIMO, the use case of Row 10-13, Row 18-37, Row 42-45 and Row 50-69 is also unclear to us.

Proposal 4: For Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH

- The total number of rows does not exceed 128 for S-TRP and m-TRP case.

- For single CW, Row 9-10, 20-23 and 42-47 can be supported with MU-MIMO restriction.

- For single CW, Row 67-68, 78-80, 100-105, and 153-158 can be removed.

- For two CWs, Row 10-37 and Row 42-69 can be removed.

  - Uplink DMRS to support 4 and more layers per UE

For Rel.15 DMRS ports with >4 layers transmission, it was agreed that the same DMRS port combination(s) as that for rank = 5,6,7,8 for PDSCH is reused at least for full or non-coherent UL codebook. For partial coherent codebook, some companies proposed DMRS design to support layers to DMRS port mapping that layers associated to the same antenna port group are multiplexed into the same DMRS CDM group. However, DMRS port indication is another issue independent from codebook design. The above layer to DMRS port mapping can be achieved via appropriate codebook design instead of DMRS table design. Based on current DMRS port combination(s) for rank = 5,6,7,8 for PDSCH, two CDM groups with DMRS port combination(s) of 3+2, 3+3, 4+3, 4+4 are supported. As long as corresponding codebook design is supported (e.g. 3+2, 3+3, 4+3, 4+4 layers mapped to the first and second antenna port group), layers associated to the same antenna port group can be multiplexed into the same DMRS CDM group.

Proposal 5: For >4 layers PUSCH with Rel-15 DMRS, the same DMRS port combination(s) as that for rank = 5,6,7,8 for PDSCH is reused for partial-coherent UL codebook.

- Layers to DMRS port mapping that layers associated to the same antenna port group are multiplexed into the same DMRS CDM group can be achieved via appropriate layer splitting.

In Rel-15, up to two PTRS ports were supported for uplink 4 ports transmission. For 8 ports transmission in Rel-18, whether more PTRS ports should be introduced needs further study. The following aspects should be considered regarding this issue:

- PTRS ports would occupy different subcarriers in many OFDM symbol. That is, the overhead of 4 PTRS ports would double that of 2 PTRS ports.

- Two non-coherent antenna groups can share the same PTRS ports. In Rel-15, even 4Tx non-coherent UE only supports at most 2 PTRS ports.

- The required number of PTRS ports may depended on the antenna architecture. Considering the advanced UE with 8 Tx ports is not likely to introduce more oscillators, more PTRS ports needs to be justified.

- Similar to Rel-15 downlink PTRS, only single symbol DMRS is supported when UL PTRS is configured. In this case, based on Rel-15 DMRS, only maximal 6 layers transmission can be supported with PTRS.

Considering the above aspects, it is proposed that more than two PTRS ports are not supported in Rel-18.

Proposal 6: More than two PTRS ports are not supported for >4 layers transmission.

Since more antenna groups and more DMRS ports are introduced in Rel-18, the PTRS power boosting should be extended to support Ng=2/4 and up to 8 layers transmission.

- In Rel-15, only two antenna groups are supported for partial-coherent codebook. In Rel-18, at least Ng=2,4 need to be supported. The PTRS power boosting would be different with different number of antenna groups, to ensure constant power across symbols in each antenna. For example, with RRC configuration “00” for power boosting, for Ng=2 and Rank=4, the power boosting would be 3/6 dB for Qp=1/2 PTRS ports, while for for Ng=4 and Rank=4, the power boosting would be 0/3 dB for Qp=1/2 PTRS ports, since the power cannot be borrowed among antenna groups and the power is independent among groups.

- If UE supports up to 8 layers transmission, the PTRS power boosting should also be extended for case of >4 layers transmission. For Rel-15 DMRS, 5-6 layers should be considered, while for Rel-18 DMRS, 5-8 layers should be considered.

For partial-coherent TPMI when UL-PTRS-power=00, the situation that power cannot be borrowed among antenna groups should be considered. In this case, power boosting should only consider the layers/PTRS ports transmitted within the same antenna group, and the number of transmitted PTRS ports which needs rate matching. Then the power boosting factor for a PTRS port can be expressed as

= Log2(L)+log2(Qp)= Log2(L* Qp)

where L is the number of layers transmitted in the same antenna group as the DMRS port associated with the PTRS port, and Qp is the total number of actually transmitted PTRS ports, which can be 1 or 2.

Proposal 7: For partial coherent TPMI, the factor for a PTRS port can be equal to Log2(L*Qp), where L is the number of layers transmitted in the same antenna group as the DMRS port associated with the PTRS port, Qp is the total number of transmitted PTRS ports.

In the past meeting, the PTRS-DMRS association for partial/non-coherent PUSCH with up to 8 layers and two PTRS was discussed and 4 alternatives were listed for down selection.

- Alt.1 provide full flexibility with additional two bits overhead.

- Alt.2 is difficult to work. It is very possible that the DMRS ports associated with different PTRS ports are mapped to different CWs. In this case, selection of CW with higher MCS doesn’t work at all.

- Alt.3 is an optimization for overhead reduction based on Alt.1. Whether it is available depended on the output of DMRS indication design.

- For Alt.4, only part of DMRS ports can be indicated, and the overhead is the same as legacy. There may be some performance loss.

Based on the above analysis, Alt.1 is preferred, which is directly extended from current association table.

Proposal 8: For two PTRS ports for partial/non-coherent PUSCH with up to 8 layers, support Alt 1 for PTRS-DMRS association indication table.

Conclusion

In this contribution, we discuss the detailed design and signaling for Rel-18 DMRS enhancement. In summary, we have the following proposals for study of Rel-18 DMRS:

Proposal 1: For Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, Row 4-9 for two CWs is not supported.

Proposal 2: For Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH

- The total number of rows does not exceed 64 for S-TRP and m-TRP case.

- For single CW, Row 9-11, 24-30 can be supported with MU-MIMO restriction.

- For single CW, Row 55 and 56 can be removed.

- For two CWs, Row 8-19, Row 24-37 can be removed.

Proposal 3: For Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH

- The total number of rows does not exceed 64 for S-TRP and m-TRP case.

- For single CW, Row 9-10, 20-23 can be supported with MU-MIMO restriction.

- For single CW, Row 33-34 and Row 44-46 can be removed.

- For two CWs, Row 2-3 and Row 8-12 is can be removed.

Proposal 4: For Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH

- The total number of rows does not exceed 128 for S-TRP and m-TRP case.

- For single CW, Row 9-10, 20-23, 42-47 can be supported with MU-MIMO restriction.

- For single CW, Row 67-68, 78-80, 100-105, and 153-158 can be removed.

- For two CWs, Row 10-37, Row 42-69 can be removed.

Proposal 5: For >4 layers PUSCH with Rel-15 DMRS, the same DMRS port combination(s) as that for rank = 5,6,7,8 for PDSCH is reused for partial-coherent UL codebook.

- Layers to DMRS port mapping that layers associated to the same antenna port group are multiplexed into the same DMRS CDM group can be achieved via appropriate layer splitting.

Proposal 6: More than two PTRS ports are not supported for >4 layers transmission.

Proposal 7: For partial coherent TPMI, the factor for a PTRS port can be equal to Log2(L*Qp), where L is the number of layers transmitted in the same antenna group as the DMRS port associated with the PTRS port, Qp is the total number of transmitted PTRS ports.

Proposal 8: For two PTRS ports for partial/non-coherent PUSCH with up to 8 layers, support Alt 1 for PTRS-DMRS association indication table.

References

- RP-213598, New WID: MIMO Evolution for Downlink and Uplink, Samsung (Moderator), 3GPP TSG RAN Meeting #94e, Dec. 6th – 17th, 2021.