3GPP TSG RAN WG1 #113		R1-2305411

Incheon, Korea, May 22nd – 26th, 2023

Source:	OPPO

Title:	Discussion on LS on unified TCI-state and fast SCell activation

Agenda Item:	5

Document for:	Discussion and Decision

- Introduction

RAN2 sent LS [1] to ask the RAN1 to provide response to one question on qcl-info configuration for SCell activation. In this contribution, we will discuss the related issue and make suggestion on RAN1 answer to RAN2.

- Discussion

In LS [1], RAN2 asks RAN1 to provide response to the following questions related with SCell activation:

Question: Is the reference signal used for fast SCell activation allowed to be configured with the qcl-Info that is indicated by TCI-state from the dl-OrJointTCI-stateList, by the current RAN1 specification?

The UE can be configured with aperiodic CSI-RS for tracking for fast SCell activation. In particular, the RRC parameter SCellActivationRS-Config is used to configure the reference signal for fast activation:

SCellActivationRS-Config-r17 ::= SEQUENCE { scellActivationRS-Id-r17 SCellActivationRS-ConfigId-r17, resourceSet-r17 NZP-CSI-RS-ResourceSetId, gapBetweenBursts-r17 INTEGER (2..31) OPTIONAL, -- Need R qcl-Info-r17 TCI-StateId, ... }

The qcl-Info configure one TCI state that provides QCL source and QCL type for each NZP CSI-RS resource configured in resourceSet. The unified TCI state was introduced in Rel-17. The joint TCI state or DL TCI state can be used to provided QCL source and QCL type for one NZP CSI-RS resource. Therefore, to the question asked by RAN2, the TCI state from dl-OrJointTCI-stateList can be configured to the qcl-info for fast SCell activation.

Proposal 1: The response to RAN2’s question is:

- The reference signal used for fast SCell activation can be configured with the qcl-info that is indicated by a TCI state from the dl-OrJointTCI-stateList.

- Conclusions

In this contribution, we present our views on question asked by RAN2 and the following proposal is made:

Proposal 1: The response to RAN2’s question is:

- The reference signal used for fast SCell activation can be configured with the qcl-info that is indicated by a TCI state from the dl-OrJointTCI-stateList.

- Reference

RP-2304549 LS on unified TCI-state and fast SCell activation