3GPP TSG RAN WG1 #113		R1-2305429

Incheon, Korea, May 22nd – May 26th, 2023

Source:	OPPO

Title:                     Discussion on resource allocation for SL positioning reference signal

Agenda Item:       9.5.1.3

Document for:	Discussion and Decision

- Introduction

The study on expanded and improved NR positioning had been completed in RAN1#111, and the item was converted to work item in RAN#98 with following objectives on SL positioning[1]:

- Specify solutions for support of SL positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]:

  - Specify SL PRS for support of SL positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1].

    - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum.

    - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified.

  - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2].

  - Specify support of resource allocation for SL PRS:

    - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1].

      - For resource allocation mechanism for SL PRS in Scheme 2:

        - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1].

        - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1].

    - Support resource allocation for shared resource pool with Rel-16/17/18 SL communication and dedicated resource pool for SL PRS [RAN1].

      - NOTE: For SL positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 SL communication, backward compatibility with legacy Rel-16/17 UEs should be ensured.

  - Specify procedures for transmit power control for SL PRS transmissions at least based on open loop power control (OLPC) [RAN1].

  - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2].

  - Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios [RAN2, RAN3]:

    - Specify the protocol and procedures for SL positioning between UEs (Protocol for SL positioning procedures (SLPP)).

    - Specify the protocol and procedures for SL positioning between UEs and LMF.

  - Specify signalling to NG-RAN for SL positioning and ranging service authorizations as needed. [RAN3, RAN2]

  - Specify corresponding new core requirements, as well as identifying and specify the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

In the last meeting resource allocation for SL positioning reference signal was discussed with considerable number of agreements, in this paper we further discuss potential issues on this topic and give our views.

- Discussion

  - Dedicated Resource Pool

Following agreements on dedicated resource pool for SL PRS were achieved in the last RAN1 meeting[2]:

Agreement For a dedicated resource pool for positioning: - No additional slots are needed to be supported Agreement For SL-PRS transmission, either dedicated resource pool(s) or shared resource pool(s) or both can be (pre-)configured in the only SL BWP of a carrier. - A UE can be (pre-)configured with one or more dedicated SL resource pools. - A UE can be (pre-)configured with one or more shared SL resource pools. Agreement For a dedicated resource pool for SL positioning, only a single stage SCI is used. PSCCH and associated SL-PRS are TDMed in the same slot. - FFS: whether SL-PRS can be transmitted in a slot without associated PSCCH

In SL communication, the number of OFDM symbols that can be used for SL operation within a slot is not smaller than 7 to accommodate a PSCCH/PSSCH and corresponding DMRS. For dedicated SL PRS resource pool, this restriction can be relaxed as SL PRS may not need so much OFDM symbols. Furthermore, to support IUC scheme 2 PSFCH resources should be configured within the dedicated SL PRS resource pool.

Proposal 1: For a dedicated SL PRS resource pool, slots with less than 7 OFDM symbols for SL operation and slots configured with PSFCH resources can be included.

In current SL communication, PSCCH may occupy up to 25 PRBs in up to 3 OFDM symbols (75 PRBs in total), this structure may degrade the accuracy of SL positioning as the bandwidth of SL PRS in the OFDM symbols for PSCCH are reduced. From this sense it is desirable to use less OFDM symbols for PSCCH in SL positioning. Furthermore, the minimum number of PRBs for the PSCCH should also be bounded (which is 20 PRBs in existing SL) to guarantee the reliability of PSCCH.

Proposal 2: In a dedicated resource pool PSCCH carrying the SCI for reserving/indicating one or more SL PRS resource(s) uses at most one OFDM symbol.

The symbol for PSCCH should be adjacent to the associated SL PRS symbols, otherwise additional AGC symbol needs to be inserted in between. As agreed in the last meeting PSCCH and associated SL-PRS are TDMed in the same slot, therefore, one symbol can be used for PSCCH with same bandwidth as the associated SL PRS, as shown in Figure 1. The benefit to use one symbol for PSCCH is that the bandwidth of PSCCH is maximized and the DMRS of PSCCH can be used for TOA estimation.

- The multiplexing between PSCCH and associated SL PRS

Proposal 3: In a dedicated resource pool the OFDM symbol used for PSCCH carrying the SCI for reserving/indicating one or more SL PRS resource(s) is adjacent to the OFDM symbol(s) for associated SL PRS, and occupies same bandwidth as the associated SL PRS.

In the last meeting fields to be included in SCI for SL-PRS were discussed [3]. One open issue is whether Cast type is needed. In SL communication this filed is in effect used to indicated HARQ-feedback types[4], however, SL PRS has no CRS and therefore no HARQ-feedback for it, hence this field should not be included. In addition, as discussed in subclause 2.5 SL PRS can be used as the lower signaling to request UE-B to transmit SL-PRS, therefore SL PRS request field is also unnecessary. Whether Session ID is visible to physical layer or not is unclear, whether this field should be included or not should be decided by upper layer.

Proposal 4: In the dedicated resource pool, SCI for SL-PRS does not include “Cast type” field and “SL-PRS request” field, whether “Session ID” is included or not is up to higher layer.

With regard to SL PRS time domain resource allocation within a dedicated resource pool SL PRS resource-based allocation should be used, according to the definition of SL PRS resource agreed in the last meeting, a SL PRS resource may occupy a sub-slot or a slot.

Proposal 5: For SL PRS time domain resource allocation in a dedicated resource pool SL PRS resource-based allocation is used.

Besides to support scheme 2, it is unnecessary to transmit PSCCH in the same slot. Firstly, there is no need to indicate the resource(s) and transmission manner of a SL PRS (including SL PRS sequence, comb size, number of symbols/repetitions, etc.) in the same slot, as the latency requirement of SL positioning is not as urgent as SL communication, these parameters could be indicated ahead of SL PRS slot, in particular for the periodically or (semi-)persistently transmitted SL PRS. Transmitting PSCCH in every slot would increase the overhead dramatically.

For instance, to support UE-based or UE-assisted absolute positioning with TDOA method, some RSUs could be deployed as anchor nodes and play a role similar as TRPs in Uu positioning, as shown in Figure 2, the anchor UEs are supposed to transmit SL PRS periodically. In this scenario, as in Uu positioning, the SL PRS resources can be indicated with higher layer signaling. For dedicated resource pool, the higher layer signaling can be transmitted in another resource pool associated with the dedicated resource pool, the associated resource pool may be also used for measurement report. For shared resource pool, the higher layer signaling can be transmitted within the same resource pool as SL PRS.

Proposal 6: In a dedicated/shared Scheme 1 resource pool, SL PRS transmission without an associated PSCCH within a slot from a single UE perspective should be supported, SL-MAC-CE or other higher-layer signaling is used for SL PRS resource reservation/indication.

- Absolute positioning based on RSU type anchor nodes in V2X scenario

  - Shared Resource Pool

Following agreements on shared resource pool for SL PRS were achieved in the last RAN1 meeting [2]:

Agreement TDM-based multiplexing in a slot of SL PRS from different UEs is NOT supported for a shared resource pool. Agreement With regards to the SCI signaling in a shared resource pool, in addition to SL PRS transmission, the UE transmits - Opt. 1: SCI1-A & a 2nd stage SCI format are used for SL-PRS indication - FFS: Details including a new or existing 2nd stage SCI Agreement In a shared resource pool: - SL-PRS, associated PSCCH and PSSCH scheduled by the PSCCH are included in the same slot - With regards to PSSCH and SL-PRS multiplexing, downselect one of the following alternatives in RAN1#113 meeting: - Alt. A.1: Only TDMing is supported - Alt. A.2: Only FDMing of PSSCH and SL-PRS is supported - FFS: Rate-matched around SL-PRS REs and/or PRB/sub-channel-level FDMing are supported potentially for different cases - Note: Rate-matched around SL-PRS REs is not applicable to comb-1 SL-PRS - Alt. A.3: Both Alt. A.1 and A.2 are supported in the specification - With regards to PSCCH and SL-PRS multiplexing, downselect one of the following alternatives in RAN1#113 meeting: - Alt. B.1: Only TDMing is supported - Alt. B.2: TDMing or PRB/sub-channel-based FDMing is supported - The PSSCH is used for (downselect one of the following alternatives in RAN1#113 meeting): - Alt. C.1: 2nd SCI only - Alt. C.2: 2nd SCI and SL-SCH - Alt. C.3: “2nd SCI only” or “2nd SCI and SL-SCH” - FFS: Handling of PT-RS and SL-PRS

In the agreement achieved in the last meeting there are 3 alternatives, i.e., Alt. C.1~C.3, on the contents contained in PSSCH for down-selection in this meeting. As location service and communication service for a UE are triggered independently, a UE may or may not have SL-SCH (including the SL-SCH for positioning related data, such as measurement report) when it needs to transmit SL PRS, it does not make sense to mandate SL PRS to be transmitted with SL-SCH all the time, this means Alt. C.2 should not be considered and the benefit of Alt. C.3 is conditional. However, to support transmitting both 2nd SCI and SL-SCH in one PSSCH would introduce many specifications impacts to both physical layer and MAC layer, including:

- New TBS determination scheme;

- Complicated multiplexing scheme to accommodate PSSCH corresponding to data in addition to 2nd SCI;

- New resource reselection triggering condition;

- New resource pool selection procedure;

- New resource size determination mechanism;

- etc.

Proposal 7: In shared resource pool PSSCH associated with SL PRS in a slot is used to convey 2nd SCI only, i.e., to support Alt. C.1.

With regards to PSSCH and SL-PRS multiplexing, we have not seen the need to support FDMing of PSSCH and SL-PRS:

- SL PRS transmission is indicated in PSSCH in shared resource pool, the mapping of the PSSCH has to be independent of the associated SL PRS, therefore RE-level multiplexing between PSSCH and SL PRS is not feasible;

- RE-level multiplexing cannot be applied to comb-1 SL PRS, if supported the multiplexing scheme would undesirably link with comb size.

- Depending on the PRBs/sub-channels occupied by PSSCH, the number of remaining PRSs/sub-channels may be limited, transmitting a narrow bandwidth SL PRS there would not contribute to the positioning of the UE. Rather PSSCH should always occupy all the allocated PRBs/sub-channels in a OFDM symbol to improve the reliability of 2nd SCI.

- Comparing to transmitting SL PRS only in a OFDM symbol, multiplexing SL PRS with PSSCH would lose the power boosting gain for SL PRS.

Proposal 8: In shared resource pool, PSSCH and associated SL-PRS are only TDMed in a slot, i.e., Alt. A.1 is supported.

FDMing of SL PRS and PSCCH would not contribute to the positioning of the UE either since the bandwidth of the SL PRS is reduced. In addition, legacy UE may measure SL RSRP on PSCCH DMRS and assuming PSD of REs in all OFDM symbols within a slot are same as PSCCH DMRS. If SL PRS with comb size larger than 1 is transmitted in the OFDM symbol with PSCCH, the PSD of PSCCH and also PSCCH DMRS have to be boosted to ensure same transmission power with other OFDM symbols, which would impact SL PRS measurement of legacy UE and further sensing-based resource selection and power control at end. Therefore, in OFDM symbols with PSCCH UE should transmit PSSCH to occupy all the REs within the allocated PRBs/sub-channels. Note that, as legacy UE may also measure SL RSRP on PSSCH DMRS if (pre-)configured, the restriction above should also be applied to the OFDM symbols with PSSCH DMRS.

Proposal 9: In shared resource pool, PSCCH and associated SL-PRS are only TDMed in a slot, i.e., Alt. B.1 is supported.

Proposal 10: In shared resource pool, in OFDM symbols with PSCCH or PSSCH DMRS, UE should transmit PSSCH to occupy all REs within allocated PRBs.

As agreed in the last meeting SCI1-A & a 2nd stage SCI format are used for SL-PRS indication, whether a new or existing 2nd stage SCI is for FFS. 2nd SCI format and coding rate are indicated in SCI 1-A, and in shared resource pool SCI 1-A cannot be changed at all for backward compatibility. But as of now there is only one more 2nd SCI format can be indicated in SCI 1-A, as shown in Table 8.3.1.1-1 of [4] also reproduced below. Therefore, the only remaining state for 2nd SCI format indication should be used prudently.

Table 8.3.1.1-1: 2nd-stage SCI formats

| Value of 2nd-stage SCI format field | 2nd-stage SCI format |
| --- | --- |
| 00 | SCI format 2-A |
| 01 | SCI format 2-B |
| 10 | SCI format 2-C |
| 11 | Reserved |

Looking into the 3 exiting 2nd stage SCI formats, SCI 2-A and 2nd SCI 2-B have no reserved bits, but SCI format 2-C has at least 6 padding bits if it is used for Requesting IUC information, as SCI 2-C with same size is also used for providing IUC information when more bits are needed. Hence it is possible to use SCI 2-C or a new 2nd SCI format with same size as SCI 2-C for SL PRS indication if the number of bits needed are less than the size of current SCI 2-C. In this case 2nd SCI format can be indicated as SCI format 2-C in the SCI 1-A for SL PRS indication, and the 2nd SCI for SL PRS indication can be distinctive from that for IUC Requesting by setting one padding bit to 1.

Proposal 11: With regards to the SCI signaling in a shared resource pool, “2nd-stage SCI format” field in SCI 1-A is set to “10”, and the size of the 2nd stage SCI for SL-PRS indication is same as current SCI 2-C.

  - Scheme 1

In the last meeting following agreement was achieved on Scheme 1[2]:

Agreement For Scheme 1 SL-PRS resource allocation, a transmitting UE can receive a SL-PRS resource allocation signaling from gNB through a - Dynamic grant - FFS Reuse DCI format 3_0 for signalling SL-PRS resource allocation or Support a new DCI format (3_X) and consider DCI format 3_0 as a starting point - Configured grant type 1 - the SL-PRS transmission(s) follows the higher layer configuration - Configured grant type 2 - Support activating and releasing the configured grant using a new DCI format 3_X or 3_0 (to be down-selected between the two DCI formats) - The above mechanisms use NR Rel-16 mode-1 signaling as a starting point - FFS: whether same/different DCI format(s) are applied for shared pool and dedicated pool. - FFS: Further details

The resource granularity of resource allocation Mode 1 in SL communication is one slot in time domain and one sub-channel in frequency domain, while according to the definition of SL PRS resource agreed in the last meeting for dedicated resource pool, a SL PRS resource may occupy a sub-slot or a slot, in frequency domain it may corresponds to a bomb offset under a certain comb size. In the sense a new DCI format should be introduced for dedicated resource pool. As the total number of different DCI sizes configured to monitor should not be larger than 4 for a cell, the size of the new DCI format should be aligned to existing DCI format 3-0. However, to differentiate it from DCI format 3-0 a new RNTI should be used to scramble the new DCI format.

Proposal 12: For Scheme 1 SL-PRS resource allocation in dedicated resource pool, support a new DCI format (3_X) with same size as DCI format 3_0, and a RNTI different from SL-RNTI or SL-CS-RNTI is used for the scrambling of DCI format 3_X.

As discussed above in shared resource pool SL-SCH and SL PRS should not be contained within one PSSCH, thus DCI format 3_X can be used for shared resource pool as well. In current SL communication PUCCH resource is allocated together with the SL grant to report SL HARQ-ACK information, however, if the SL grant is used for SL PRS transmission the corresponding PUCCH resource is not necessary.

Proposal 13: For Scheme 1 SL-PRS resource allocation in shared resource pool, DCI format 3_X as dedicated resource pool is used.

Proposal 14: No PUCCH resource is allocated together with the SL grant for SL PRS transmission in shared resource pool.

  - Scheme 2

2.2.1 Sensing-based resource selection

In the last meeting following agreements were achieved on sensing-based resource selection:

Agreement Confirm the working assumption: Sensing-based and random selection can be allowed in the same resource pool. - Note: It is possible to (pre-)configure a resource pool to exclusively use sensing-based resource allocation. Agreement For the scheme 2 sensing-based resource allocation: - Alt. 2: Rel-16 resource (re)-selection procedure with periodic and without periodic reservations is the starting point for the design of SL-PRS in the dedicated resource pool. - Note: This means that Rel-17 partial sensing is not considered a starting point for the design Agreement For Scheme 2, in a dedicated resource pool, using Rel-16 resource (re)-selection procedure as the starting point, consider at least the following potential modifications: - Modification 1: For the RS used to derive L1 SL-RSRP for resource exclusion: - Option 1: SL-PRS - Option 2: PSCCH DMRS - Option 3: PSSCH DMRS (if PSSCH is included in the dedicated resource pool) - Modification 2: For the resource selection window: - Option 1: for the derivation of the window, using the legacy approach as a starting point, substitute the Packet Delay Budget (PDB) with a new delay budget - Option 2: the selection window is provided by higher layers - Modification 3: For the SL-PRS priority: - Option 1: A single L1 SL-PRS priority is allowed in a resource pool - Option 2: Multiple L1 SL-PRS priority are allowed in a resource pool - Modification 4: For the definition of a candidate resource within the resource selection window: - Options TBD - Modification 5: For the reservation interval of SL-PRS: - Option 1: Provided by UE’s higher layers with values TBD. The set of values is (pre-)configured. - Modification 6: For the sensing window length (T0): - Option 1: Use the legacy (pre-)configuration with values (100 msec, 1100 msec) - Option 2: Equal to or larger than the largest reservation interval - Option 3: Provided by higher layers with values TBD - Modification 7: For the initial S-RSRP threshold & stepsize, target resource ratio X(%): - Options TBD - Modification 8: For the pre-emption of the reserved resources: - Options TBD - Note 1: Other potential modifications and/or other options within each modification are not precluded - Note 2: Multiple options for each potential modification may be supported

In SL communication the RS used for SL RSRP measurement in sensing operation is (pre-)configured as either PSCCH DMRS or PSSCH DMRS because the bandwidth of PSCCH could be as small as 10 PRBs. Depending on the structure of PSCCH in dedicated resource pool, the RS for SL RSRP measurement in dedicated resource pool could be PSCCH DMRS and/or SL PRS. E.g., if the bandwidth and number of symbols for PSCCH are limited in a dedicated resource pool, SL PRS should be considered for SL RSRP measurement. We believe PSSCH should not be included in dedicated resource pool.

Proposal 15: Depending on the structure of PSCCH in dedicated resource pool, PSCCH DMRS and/or SL PRS is used for SL RSRP measurement, and PSSCH is not included in dedicated resource pool.

The priority of SL PRS, similar as the priority of SL data, should be decided by upper layer, because it depends on the QoS of positioning service.  Therefore, the number of priority levels for SL PRS should be decided by upper layer.

Proposal 16: Whether single or multiple priority are allowed in dedicated resource pool is up to higher layer.

In case of shared resource pool, the priority of SL PRS and the priority of SL data should be comparable.

Proposal 17: In shared resource pool, the priority of SL PRS provided by higher layer and the priority of SL data should be comparable.

In SL communication a candidate resource within the resource selection window is defined as {slot, LsubCH}. In dedicated resource pool SL PRS resource is defined, which include part or all of OFDM symbols within a slot in time domain, and in frequency domain it could include a certain comb offset under a given comb size. As discussed in our companion contribution [5], the bandwidth of SL PRS should be the same as the bandwidth of the resource pool, and only one comb size should be allowed in the set of OFDM symbols of a SL PRS resource. Hence a candidate resource within the resource selection window can be defined as SL PRS resource in a dedicated resource pool.

Proposal 18: A candidate resource within the resource selection window is defined as SL PRS resource in a dedicated resource pool.

As TDM-based multiplexing of SL PRS from different UEs in a slot is supported, in order to reuse current resource allocation Mode 2 in SL communication for scheme 2 resource selection, and also simply UE implementation and SCI design, it is necessary to restrict that candidate resources within the resource selection window for a UE correspond to same set of OFDM symbols in a slot, i.e., the candidate resources only have distinction in frequency domain (different comb offsets). Taking Figure 3 as an example, candidate resources within the resource selection window should be constrained within one of the SL PRS time resources.

- TDMed SL PRS resources within a slot

Proposal 19: When TDM-based multiplexing of SL PRS from different UEs in a slot is supported, candidate resources within the resource selection window for a UE correspond to same set of OFDM symbols in a slot.

A UE reserving resource for another UE was discussed in the last meeting. As this is not an essential feature for SL positioning, it is not supported in resource selection mode 2 (the agreed baseline for Scheme 2) for SL communication either, and if supported it would introduce to many specification impacts. Thus, it should not be supported in Rel-18.

Proposal 20: Do not support that a UE reserves a SL-PRS resource for the transmission of another UE.

2.2.2 Congestion control

In the last meeting following agreements were achieved on congestion control:

Agreement For Scheme 2 SL-PRS resource allocation, specify congestion control mechanisms using the existing congestion control mechanisms as a starting point. - Study at least the following aspects on potential changes over the existing congestion control mechanisms: - CBR and CR definition for SL-PRS - Which parameters of a SL-PRS configuration could be impacted by the congestion control mechanism, the mapping between congestion measurements, SL-PRS priority and SL-PRS parameters. - CR and CBR measurement time window - Congestion control processing time - Number of CBR ranges - Whether any proposed changes could be applicable to shared resource pools in addition to the dedicated resource pool

To support congestion control in SL positioning for both shared resource pool and dedicated resource pool, the following as congestion control mechanism for SL communication can be reused:

- Congestion control processing time is same as SL communication;

- Up to 16 CBR ranges as SL communication are defined;

- CBR measurement time window is [100ms or 100 slots];

- CR measurement time window is [1000ms or 1000 slots];

Proposal 21: The following as congestion control mechanism for SL communication are reused for congestion control in SL positioning for both shared resource pool and dedicated resource pool:

- Congestion control processing time is same as SL communication;

- Up to 16 CBR ranges as SL communication are defined;

- CBR measurement time window is [100ms or 100 slots];

- CR measurement time window is [1000ms or 1000 slots];

In shared resource pool a UE transmitting SL PRS also uses sub-channel as resource granularity, hence the definition of CBR and CR as SL communication can be reused.

Proposal 22: The definition of CR and CBR in shared resource pool are same as those for SL communication.

In dedicated resource pool SL PRS resource is defined, and one slot may be split into different SL PRS resources. Thus, CR can be defined based on the number of SL PRS resources occupied by a UE. As different SL PRS resources may be used by different UEs, assuming SL PRS always occupy same bandwidth as the resource pool, SL RSSI measured on different OFDM symbols in a slot may be different. In this case CBR can be defined as the portion of OFDM symbols with SL RSSI exceeds the (pre-)configured threshold.

Proposal 23: In dedicated resource pool CR is defined based on the number of SL PRS resources occupied by a UE, and CBR is defined as the portion of OFDM symbols with SL RSSI exceeding the (pre-)configured threshold.

In SL communication parameters that can be impacted by congestion control including CR limit, max/min MCS, max/min sub-channel number, max transmission number and max transmission power [6]. Following the principle at least parameters below for SL PRS can be adjusted according to measured CBR level:

- CR limit;

- Maximum transmission power of SL PRS;

- Max re-transmission number or min periodicity of SL PRS;

Proposal 24: At least following parameters for SL PRS can be adjusted according to measured CBR level:

- CR limit;

- Max transmission power of SL PRS;

- Max transmission number or minimum periodicity of SL PRS;

2.2.3 Inter-UE Coordination

For resource selection Mode 2 in Rel-17, two inter-UE coordination (IUC) schemes were introduced to mitigate the performance degradation caused by hidden node problem and half duplex problem. In IUC scheme 1, if receiving an explicit request from UE-B or triggered by a condition, UE-A can provide preferred or non-preferred resource set to UE-B with MAC CE and/or SCI-2C. The resource set is taken into account by UE-B during mode 2 resource selection.

UE autonomous resource selection is also supported for SL PRS transmission. In shared resource pool a UE transmitting SL PRS uses same resource structure as SL communication, and follows same resource selection mechanism, IUC scheme 1 as SL communication can be almost reused. Only adaption needed foreseen is the condition for UE-A to determine preferred/non-preferred resource set. As if preferred/non-preferred resource set is used for SL PRS transmission, UE-A is not a destination UE of a TB for whose transmission the preferred/non-preferred resource set is being determined (as captured in 8.1.4A of [7]), rather a destination UE of a SL PRS.

Proposal 25: IUC scheme 1 is supported for SL PRS resource selection in shared resource pool.

For the dedicated SL PRS resource pool, as there may be no PSSCH transmission inside, how to transmit the IUC information and explicit request (both are carried by PSSCH) is a problem.

Proposal 26: FFS whether IUC scheme 1 is supported for SL PRS resource selection in dedicated resource pool.

IUC scheme 2 introduced in Rel-17 is used to indicate potential resource conflict on resources reserved by UE-B in the future, and the conflict information is carried by PSFCH. In study item, it was agreed that SCI can be used for reserving one or more SL PRS resources, which means resource reservation is also considered for SL PRS transmission. It is desirable to also support IUC scheme 2 for SL PRS so as to reduce resource collision between one SL PRS and another SL PRS or PSCCH/PSSCH. For shared resource pool, the PSFCH resource configured for IUC Scheme 2 can be used to indicate resource conflict between SL PRS and another SL transmission, IUC scheme 2 as SL communication can be mostly reused with trivial adaption. For dedicated resource pool, PSFCH resources should also be configured if IUC scheme 2 is supported. Note that in Rel-17 SL communication, the PRBs in the OFDM symbols for PSFCH can be (pre-)configured for either HARQ-ACK feedback or conflict information indication, hence the slots with PSFCH resources can be included in both dedicated resource pool for SL PRS and another resource pool to share the resources in the PSFCH symbols.

Proposal 27: IUC scheme 2 should be applied to SL PRS resource selection in shared resource pool and dedicated resource pool for SL positioning.

In the beginning of Rel-17 IUC Scheme 2 was also considered to indicate the resource collision on already used resource(s), however, as HARQ feedback is supported in SL communication, this feature is not supported at the end. This feature may be beneficial for SL positioning as HARQ feedback is not applicable for PRS. If PSFCH resources are included in the resource pool for SL PRS, the collision on the already used SL PRS resource(s) can also be supported to trigger additional SL PRS transmission.

Proposal 28: Conflict indication for the already used SL PRS resource(s) should be supported for SL positioning.

  - Triggering for SL PRS transmission

In the last meeting following agreement was achieved on triggering for SL PRS transmission [2]:

Agreement In Scheme 2, with regards to the triggering of SL-PRS, - Support SL-PRS transmission triggering at the physical layer by the UE’s own higher layers - Working assumption: Support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A. - Up to UE-B’s own higher layers to transmit SL-PRS in response to the lower layer request from UE-A - FFS: Lower layer signaling corresponds to SCI, MAC-CE, or SL-PRS

As discussed below in current SL communication destination ID is included in SCI to support unicast or groupcast, if the mechanism is reused for unicast or groupcast of SL PRS the receiver can know whether it is the intended receiver of the SL PRS, in this case the SCI that contains the destination ID can be used to trigger the receiver to transmit SL PRS in RTT-based positioning.

Proposal 29: In Scheme 1 or Scheme 2, support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A, the lower layer signaling is SCI.

  - Unicast, Groupcast, Broadcast

In SL communication 4 cast types are supported, i.e., Unicast, groupcast when HARQ information includes both ACK or NACK, groupcast when HARQ information includes only NACK, and broadcast. The cast type is indicated by “Cast type indicator” field in SCI-2A/B/C, in addition, a “Destination ID” field is also included to indicate the target receiver(s) in different cast types, where the destination ID is provided by higher layer. In SL positioning, “Cast type indicator” and “Destination ID” fields as SL communication can be included SCI to support the 3 cast types. Note that the SCI used to convey the 2 fields should be 1st stage SCI in SL positioning to avoid the multiplexing between SL PRS and PSSCH within one slot.

Proposal 30: “Cast type indicator” and “Destination ID” fields as SL communication should be included in SCI to support unicast, groupcast and broadcast, where the destination ID is provided by higher layer.

- Conclusion

In this contribution, we discussed the potential issues on resource allocation for SL PRS, we have following and proposals:

Proposal 1: For a dedicated SL PRS resource pool, slots with less than 7 OFDM symbols for SL operation and slots configured with PSFCH resources can be included.

Proposal 2: In a dedicated resource pool PSCCH carrying the SCI for reserving/indicating one or more SL PRS resource(s) uses at most one OFDM symbol.

Proposal 3: In a dedicated resource pool the OFDM symbol used for PSCCH carrying the SCI for reserving/indicating one or more SL PRS resource(s) is adjacent to the OFDM symbol(s) for associated SL PRS, and occupies same bandwidth as the associated SL PRS.

Proposal 4: In the dedicated resource pool, SCI for SL-PRS does not include “Cast type” field and “SL-PRS request” field, whether “Session ID” is included or not is up to higher layer.

Proposal 5: For SL PRS time domain resource allocation in a dedicated resource pool SL PRS resource-based allocation is used.

Proposal 6: In a dedicated/shared Scheme 1 resource pool, SL PRS transmission without an associated PSCCH within a slot from a single UE perspective should be supported, SL-MAC-CE or other higher-layer signaling is used for SL PRS resource reservation/indication.

Proposal 7: In shared resource pool PSSCH associated with SL PRS in a slot is used to convey 2nd SCI only, i.e., to support Alt. C.1.

Proposal 8: In shared resource pool, PSSCH and associated SL-PRS are only TDMed in a slot, i.e., Alt. A.1 is supported.

Proposal 9: In shared resource pool, PSCCH and associated SL-PRS are only TDMed in a slot, i.e., Alt. B.1 is supported.

Proposal 10: In shared resource pool, in OFDM symbols with PSCCH or PSSCH DMRS, UE should transmit PSSCH to occupy all REs within allocated PRBs.

Proposal 11: With regards to the SCI signaling in a shared resource pool, “2nd-stage SCI format” field in SCI 1-A is set to “10”, and the size of the 2nd stage SCI for SL-PRS indication is same as current SCI 2-C.

Proposal 12: For Scheme 1 SL-PRS resource allocation in dedicated resource pool, support a new DCI format (3_X) with same size as DCI format 3_0, and a RNTI different from SL-RNTI or SL-CS-RNTI is used for the scrambling of DCI format 3_X.

Proposal 13: For Scheme 1 SL-PRS resource allocation in shared resource pool, DCI format 3_X as dedicated resource pool is used.

Proposal 14: No PUCCH resource is allocated together with the SL grant for SL PRS transmission in shared resource pool.

Proposal 15: Depending on the structure of PSCCH in dedicated resource pool, PSCCH DMRS and/or SL PRS is used for SL RSRP measurement, and PSSCH is not included in dedicated resource pool.

Proposal 16: Whether single or multiple priority are allowed in dedicated resource pool is up to higher layer.

Proposal 17: In shared resource pool, the priority of SL PRS provided by higher layer and the priority of SL data should be comparable.

Proposal 18: A candidate resource within the resource selection window is defined as SL PRS resource in a dedicated resource pool.

Proposal 19: When TDM-based multiplexing of SL PRS from different UEs in a slot is supported, candidate resources within the resource selection window for a UE correspond to same set of OFDM symbols in a slot.

Proposal 20: Do not support that a UE reserves a SL-PRS resource for the transmission of another UE.

Proposal 21: The following as congestion control mechanism for SL communication are reused for congestion control in SL positioning for both shared resource pool and dedicated resource pool:

- Congestion control processing time is same as SL communication;

- Up to 16 CBR ranges as SL communication are defined;

- CBR measurement time window is [100ms or 100 slots];

- CR measurement time window is [1000ms or 1000 slots];

Proposal 22: The definition of CR and CBR in shared resource pool are same as those for SL communication.

Proposal 23: In dedicated resource pool CR is defined based on the number of SL PRS resources occupied by a UE, and CBR is defined as the portion of OFDM symbols with SL RSSI exceeding the (pre-)configured threshold.

Proposal 24: At least following parameters for SL PRS can be adjusted according to measured CBR level:

- CR limit;

- Max transmission power of SL PRS;

- Max transmission number or minimum periodicity of SL PRS;

Proposal 25: IUC scheme 1 is supported for SL PRS resource selection in shared resource pool.

Proposal 26: FFS whether IUC scheme 1 is supported for SL PRS resource selection in dedicated resource pool.

Proposal 27: IUC scheme 2 should be applied to SL PRS resource selection in shared resource pool and dedicated resource pool for SL positioning.

Proposal 28: Conflict indication for the already used SL PRS resource(s) should be supported for SL positioning.

Proposal 29: In Scheme 1 or Scheme 2, support UE-A to request UE-B to transmit SL-PRS via lower layer signaling sent by UE-A, the lower layer signaling is SCI.

Proposal 30: “Cast type indicator” and “Destination ID” fields as SL communication should be included in SCI to support unicast, groupcast and broadcast, where the destination ID is provided by higher layer.

- References

- RP-230328, Revised WID on Expanded and Improved NR Positioning;

- RAN1 Chairman’s Notes, RAN1#112bis-e, April, 2023;

- R1-2304233, Moderator Summary #4 on resource allocation for SL PRS, RAN1#112bis-e

- 3GPP TS 38.212 V17.5.0 (2023-03), 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Multiplexing and channel coding (Release 17)

- R1-2305427, Discussion on SL positioning reference signal, OPPO

- 3GPP TS 38.331 V17.4.0, 3rd Generation Partnership; Technical Specification Group Radio Access; Radio Resource Control (RRC) protocol specification (Release 17)

- 3GPP TS 38.214 V17.5.0 (2023-03), Technical Specification;3rd Generation Partnership Project; Technical Specification Group Radio Access Network; NR; Physical layer procedures for data (Release 17)