3GPP TSG RAN WG1 #113		R1-2305541

Incheon, Korea, May 22nd – May 26th, 2023

Agenda item:	9.13

Source:	Samsung

Title:	Enhancements to operate NR on dedicated spectrum less than 5 MHz

Document for:	Discussion and Decision

# Introduction

The following objectives shall be included for dedicated FDD spectrum in FR1:

- Identify and specify necessary changes to NR physical layer with minimum specification impact to operate in spectrum allocations from approximately 3 MHz up to below 5 MHz [RAN1]:

  - Restrict to subcarrier spacing of 15kHz and the use of normal cyclic prefix.

  - For SSB:

    - Reuse PSS/SSS specification without puncturing.

    - PBCH based on current design

  - Identify and specify necessary minimum changes to PDCCH, CSI-RS/TRS, PUCCH, and PRACH for functional support based on existing design, without optimization.

In RANP#95-e, a new WID on NR support for the dedicated spectrum less than 5MHz bandwidth for FR1 was approved [1]:

Conclusion

For transmission bandwidths of <5MHz for 3MHz channel bandwidth, for CSI-RS other than for RRM measurements, no enhancements are needed.

FFS: CSI-RS for RRM

Agreement

- For transmission BWs for 3MHz and 5MHz channel BW, send an LS to RAN plenary for operators input for the following and RAN plenary guidance,

  - For 5MHz channel BW, whether to allow/support transmission BW(s) for physical channels of approximate 3 MHz up to below 5 MHz. What is the recommended transmission BW(s) to consider?

  - For 3MHz channel BW, whether to allow/support transmission BW(s) for physical channels of approximate 3 MHz. What is the recommended transmission BW(s) to consider?

- No intention to change the WID scope and TU

Working Assumption

For transmission bandwidth[s] of <5MHz, for PBCH, in the case[s] that available PRBs for PBCH transmission is less than 20PRB,

- PBCH based on RB-level puncturing (i.e., PBCH encoding is based on 20PRB. The encoded bits and DMRS are mapped to 20PRBs based on legacy SSB structure, and those PRBs that fall outside of available PRBs for PBCH transmission are punctured)

- Note: No other optimization is needed

In RAN1#112 [2], the following conclusion, agreement and working assumption were made, regarding the design details for the dedicated spectrum with less than 5MHz bandwidth.

This contribution discusses the remaining issues for supporting NR to operate on dedicated spectrum with less than 5MHz bandwidth, including the impact to legacy bands and legacy UEs, enhancement to SS/PBCH block, enhancement to CORESET#0, and other design considerations.

# Impact to Legacy Bands and Legacy UEs

RAN Plenary has discussed question 1 on legacy bands and UE operation, and concluded the following:

- In some bands where the <5MHz feature is planned to be deployed there may be legacy NR UEs, whereas in others there are no legacy NR UEs.

- In order to limit the impact to any legacy UEs in the same frequency range, it would be helpful if the sync raster can be differentiated for the less-than-5MHz channels.

- It is assumed that UE support of the <5MHz feature is band-specific and optional.

RAN Plenary has discussed question 2 on the feature list to be considered, and concluded that the less-than-5MHz WI in Rel-18 should consider single-carrier operation, excluding RedCap. In addition, UE speeds up to 500km/h should be targeted for Band n100 without impact to RAN1.

In the LS from RANP to RAN#4 (cc RAN1) [3], the targeted development scenario of the dedicated spectrum with less than 5MHz channel bandwidth was clarified.

It is noted that legacy UEs are possibly deployed in the same frequency range as the new Rel-18 UEs supporting the dedicated spectrum with less than 5 MHz channel bandwidth, and RANP also suggested different sync raster entries to distinguish the legacy bands and new bands with less than 5 MHz channel bandwidth, which is feasible at least for PCells. Then, at least from RAN1 perspective, there is no need for further enhancement on the indication to bar legacy UEs to access the new bands with less than 5 MHz channel bandwidth, e.g., by indication in PBCH payload or by indication by PSS/SSS sequence enhancement, and whether to consider enhancement to support such functionality by SIB1 could be left for RAN2.

Proposal 1: From RAN1 perspective, there is no need to consider further enhancement to access barring.

- Whether such functionality is indicated by SIB1 can be up to RAN2.

# SS/PBCH Block Enhancement

## Truncation of SS/PBCH Block Bandwidth

In legacy NR system, a SS/PBCH block occupies 240 subcarriers in the frequency domain, wherein PSS and SSS occupy 127 subcarriers, and PBCH occupies 240 subcarriers, as shown in Figure 1. For 15 kHz subcarrier spacing, the bandwidth of the legacy SS/PBCH block is 3.6 MHz, which is larger than the available bandwidth of the dedicated spectrum with minimum channel bandwidth as 3MHz. Therefore, only part of PBCH could be transmitted in the dedicated spectrum, and some method of truncation is needed. Considering reusing PSS and SSS as described in the WID, the bandwidth of SS/PBCH block after truncation shall be no smaller than 12 RBs.

RAN Plenary has discussed the possible transmission bandwidth options for 3 MHz and 5 MHz channel bandwidths for the spectrum allocations on the bands of interest in this work item, and concluded the following:

- For the 3MHz channel bandwidth in band n100 (max channel utilization 15 PRBs as already agreed in RAN1/RAN4):

  - PBCH transmission bandwidth is 12 PRBs

  - CORESET#0 transmission bandwidth is to be decided by RAN1

- RAN1 is requested to consider whether the above also applies for other bands with 3MHz channel bandwidth, or whether the PBCH transmission bandwidth is 15 PRBs for such bands.

- For the 5MHz channel bandwidth:

  - PBCH transmission bandwidth is 20 PRBs

  - CORESET#0 transmission bandwidth is to be decided by RAN1

- Other details (including sync raster details) are to be progressed in the WGs.

Also, the LS from RANP to RAN1 [4] clarifies the bandwidth of SS/PBCH block after truncation at least for band n100:

As indicated in the LS, for 3MHz channel bandwidth in band n100, the bandwidth of SS/PBCH block is truncated to 12 RBs, and since the PSS and SSS also occupy 12 RBs, the SS/PBCH block structure after truncation shall be fixed as the center 12 RBs of the legacy SS/PBCH block structure, which is, the lowest 4 RBs and highest 4 RBs of the legacy SS/PBCH block structure are truncated. An illustration of the SS/PBCH block structure after truncation is shown in Figure 1.

Figure 1 Illustration of truncation of legacy SS/PBCH block bandwidth into 12 RBs.

For other bands with 3MHz channel bandwidth, RAN1 needs to further consider to down-select from using the same truncated SS/PBCH block structure with 12 RBs or another truncated SS/PBCH block structure with 15 RBs, and we support reusing the same truncated SS/PBCH block structure with 12 RBs due to the following reasons.

First, the SS/PBCH block bandwidth directly impacts the synchronization raster design, as indicated in the RAN4 LS [5]. For example, in RAN4 design principle, the upper bound of achievable synchronization raster interval is limited based on the value of the SS/PBCH block bandwidth, as shown in Figure 2. More precisely, synchronization raster interval ≤ channel bandwidth - SS/PBCH block bandwidth + channel raster interval. Hence, if the channel bandwidth is same as the SS/PBCH block bandwidth (e.g. both as 15 RBs for the case we focus on), synchronization raster interval has to be equal to the channel raster interval, which is 100 kHz in the context of 3 MHz channel bandwidth, and is much smaller than the legacy bands with 5 MHz minimum channel bandwidth. In Rel-15 NR, to avoid the channel bandwidth same as the SS/PBCH block bandwidth, the original design of 24 RB as SS/PBCH block bandwidth has been reduced to 20 RBs, and we expect the same principle shall be applied for 3 MHz channel bandwidth.

Figure 2 Illustration of relationship between SS/PBCH block bandwidth and synchronization raster interval.

Table 1 summarizes the estimated number of synchronization raster entries (the estimation can be subject to minor deviation depending on the selection of the starting synchronization raster entry) for band n8/26/28, wherein the synchronization raster entry intervals are rounded down to the closest integer as a multiple of 100 kHz and assuming a cluster of 3 entries with 100 kHz interval as in the legacy synchronization raster design. It can be observed that setting the SS/PBCH block bandwidth same as the channel bandwidth (e.g. both as 15 RBs) will lead to more than 4 times the complexity comparing to the band with legacy synchronization raster, which is not acceptable for initial cell search; while using SS/PBCH block structure with 12 RB bandwidth could only increase the complexity by 56.98%.

Table 1 Summary of synchronization raster design for different combination of SSB BW and CH BW.

| SSB BW (RB) | 20 | 15 | 12 |
| --- | --- | --- | --- |
| channel BW (RB) | 25 | 15 | 15 |
| channel raster interval (kHz) | 100 | 100 | 100 |
| maximum sync raster interval (kHz) | 1000 | 100 | 600 |
| # sync raster entries for n8 | 78 | 321 | 123 |
| # sync raster entries for n26 | 78 | 321 | 123 |
| # sync raster entries for n28 | 102 | 423 | 159 |
| total # sync raster entries for n8/n26/n28 | 258 | 1065 | 405 |
| increased complexity |  | 312.79% | 56.98% |

Observation 1: Truncating SS/PBCH block bandwidth to 15 RBs will lead to unacceptable complexity in initial cell search.

Moreover, another reason to reuse the same 12 RB based SS/PBCH block structure is that no obvious performance degradation observed comparing to 15 RB based SS/PBCH block structure. It is true that assuming the same EPRE before and after truncation, smaller bandwidth of SS/PBCH block leads to worse detection performance of PBCH. However, since the SS/PBCH block may not be able to FDM with other signal or channel in the channel with 3 MHz channel bandwidth (e.g., the remaining several PRBs may not be available for meaningful bandwidth of other signal or channel), power boosting of the SS/PBCH block can be assumed such that the overall transmission power over the channel remains the same as the one before truncation.

Figure 3 Detection performance of PBCH according to different bandwidth.

Figure 3 compares the PBCH detection performance according to different PBCH bandwidths. It can be observed that the PBCH detection performance degrades when the PBCH bandwidth is truncated to a smaller value, assuming the same EPRE before and after truncation. However, with power boosting (i.e., assuming same total transmission power over the channel), the degradation of performance can almost be compensated, and very minor performance degradation is observed comparing PBCH bandwidth as 15 RBs and 12 RBs (e.g., around 0.1 dB).

Observation 2: With power boosting on EPRE of the SS/PBCH block and maintaining the whole transmission power, the truncation of PBCH bandwidth from 15 RBs to 12 RBs has very minor impact to PBCH detection performance.

Finally, supporting two different SS/PBCH block structures after truncation would lead to more specification work, from both RAN1 and RAN4 perspectives. For example, RAN1 needs to consider two different CORESET#0 configurations, one based on 12 RB based SS/PBCH block structure, and another based on 15 RB based SS/PBCH block structure, and it’s not obvious at this moment whether a common configuration table can be applied for the two cases. For another example, RAN4 needs to come up with two different synchronization raster design to accommodate the different SS/PBCH block bandwidths, wherein such dependency is already implied by the LS from RAN4 [5] and analysed in the previous section of this contribution. Since this would be the last RAN1 meeting for this WI, we believe introducing additional work load is not preferable at this timing.

Based on above discussions, we have the following proposal.

Proposal 2: For 3 MHz channel bandwidth in all applicable bands, SS/PBCH block bandwidth is truncated to 12 RBs, and a fixed SS/PBCH block structure with 12 RBs bandwidth is supported (e.g., the center 12 RBs of the legacy SS/PBCH block structure).

## EPRE of SS/PBCH Block

In the legacy NR, a UE assumes that SSS, PBCH DM-RS, and PBCH data have same EPRE, and the ratio of PSS EPRE to SSS EPRE in a SS/PBCH block is either 0 dB or 3 dB, which is based on the legacy SS/PBCH block structure wherein 240 - 127 = 113 empty subcarriers are FDMed with 127 PSS subcarriers in the first OFDM symbol within the SS/PBCH block. If the SS/PBCH block structure with truncated bandwidth only has 12 RBs, 3 dB power boosting for PSS subcarriers is not applicable, since the number of empty subcarriers becomes 144-127 = 17. For this case, only 0 dB shall be assumed by the UE between PSS EPRE and SSS EPRE.

Proposal 3: For SS/PBCH block structure with 12 RB bandwidth after truncation, PSS EPRE and SSS EPRE in the SS/PBCH block are the same.

# CORESET#0 Enhancement

## CORESET#0 Configuration

First of all, since SS/PBCH bock only has a bandwidth of 12 RBs, we didn’t see a need to support a transmission bandwidth between 3 MHz and 5MHz at least for initial BWP configured by MIB, and the initial BWP configured by MIB shall also support 12 RBs as its bandwidth (i.e., CORESET#0 bandwidth) to be consistent with the SS/PBCH block after truncation.

In RAN1#111, two high-level options were agreed for CORESET#0 configuration for 3 MHz channel bandwidth: Option 1: Reusing existing configuration table; Option 2: Using a new CORESET#0 configuration table. In this section, we further discuss the details of the two options. In the legacy NR system, a UE determines PRB location of CORESET #0 based on an offset to the first PRB of the corresponding SS/PBCH block, wherein the offset is indicated in MIB as described in Table 13-1. For bands with 3 MHz minimum channel bandwidth, either Option 1 or Option 2 can work, with some modification to current specification.

For Option 1, one sub-option (i.e., Option 1a) to consider is to truncate the CORESET#0 bandwidth based on the indicated configuration, such that the CORESET#0 bandwidth after the truncation can be aligned with the SS/PBCH block (i.e., 12 RBs). It is also noted that the truncation can happen after the CCE-to-REG mapping such that there is no need to enhance the CCE-to-REG mapping rule. Another sub-option (i.e., Option 1b) for Option 1 to consider is using the reserved row in the legacy table to support CORESET#0 bandwidth as 12 RBs. In this way, the legacy CCE determination method can automatically support the newly added CORESET#0 bandwidth as 12 RBs, and no other enhancement is needed. Comparing to the sub-option of truncation, the only difference is lack of flexibility on configuring either 2 or 3 OFDM symbols for CORESET#0, since there is only one row available as reserved.

For Option 2, when designing the new table, the newly considered bandwidth for CORESET#0 shall be 12 RBs, since in the legacy CORESET#0 configuration, NRBCORESET×NsymbCORESET should be an integer multiple of 12 to avoid extra specification impact to legacy CCE-to-REG mapping for CORESET#0. In this way, legacy CCE determination method can automatically support the newly added CORESET#0 bandwidth as 12 RBs, and no other enhancement is needed. For another aspect, since the SS/PBCH block bandwidth is also 12 RBs, it is natural to support the frequency offset in the table as 0 RB, wherein the offset is interpreted as the one between the lowest RB of CORESET#0 for 3MHz channel bandwidth and the lowest RB of the truncated SS/PBCH block. An illustration of the CORESET#0 configuration and CCE determination following the legacy method is shown in Figure 4.

Figure 4 Illustration of the CORESET#0 design with 12 RB bandwidth and 0 RB offset.

Moreover, in the legacy NR system, the selection of the CORESET#0 table is based on the minimum channel bandwidth associated with the band, and such principle shall not be changed to support new bands with 3 MHz minimum channel bandwidth. Hence, for Option 1, the legacy table will be used for bands with 3 MHz, or 5 MHz, or 10 MHz minimum channel bandwidth, and for Option 2, the newly introduced table shall be used for bands with 3 MHz minimum channel bandwidth.

Based on above discussion, we provided two example CORESET#0 configuration tables – Table 2 and Table3, corresponding to Option 1b and Option 2, respectively.

Table 2 Example CORESET#0 configuration table for frequency bands with minimum channel bandwidth 3MHz, 5 MHz or 10 MHz

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs NRBCORESET | Number of Symbols NsymbCORESET | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 2 |
| 2 | 1 | 24 | 2 | 4 |
| 3 | 1 | 24 | 3 | 0 |
| 4 | 1 | 24 | 3 | 2 |
| 5 | 1 | 24 | 3 | 4 |
| 6 | 1 | 48 | 1 | 12 |
| 7 | 1 | 48 | 1 | 16 |
| 8 | 1 | 48 | 2 | 12 |
| 9 | 1 | 48 | 2 | 16 |
| 10 | 1 | 48 | 3 | 12 |
| 11 | 1 | 48 | 3 | 16 |
| 12 | 1 | 96 | 1 | 38 |
| 13 | 1 | 96 | 2 | 38 |
| 14 | 1 | 96 | 3 | 38 |
| 15 | 1 | 12 | 2 | 0 |

Table 3 Example CORESET#0 configuration table for frequency bands with minimum channel bandwidth 3MHz

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs NRBCORESET | Number of Symbols NsymbCORESET | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 2 |
| 2 | 1 | 24 | 2 | 4 |
| 3 | 1 | 24 | 3 | 0 |
| 4 | 1 | 24 | 3 | 2 |
| 5 | 1 | 24 | 3 | 4 |
| 6 | 1 | 12 | 2 | 0 |
| 7 | 1 | 12 | 3 | 0 |
| 8 | Reserved |  |  |  |
| 9 | Reserved |  |  |  |
| 10 | Reserved |  |  |  |
| 11 | Reserved |  |  |  |
| 12 | Reserved |  |  |  |
| 13 | Reserved |  |  |  |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |

Proposal 4: For CORESET#0 configuration table:

- Reuse the legacy principle of using the minimum channel bandwidth to determine which CORESET#0 configuration table to use;

- For 3 MHz channel bandwidth, support a single CORESET#0 bandwidth as 12 RBs;

- No need to support CORESET#0 bandwidth between 13 and 24 RBs;

- For Option 1, two sub-options can work:

  - Option 1a: truncate the CORESET#0 bandwidth to 12 RBs after the CCE-to-REG mapping;

  - Option 1b: use the reserved row in the legacy table to support CORESET#0 configuration with 12 RB bandwidth and 0 RB offset (Table 2 as an example);

- For Option 2, the new table shall include configurations satisfying the following (Table 3 as an example):

  - Reuse all legacy configurations with 24 RB CORESET#0 bandwidth;

  - Add new configurations with 12 RB CORESET#0 bandwidth and 0 RB offset;

  - UE determines the CORESET#0 bandwidth according to the table directly and no puncturing is needed.

## Type0-PDCCH Detection

In RAN1#111, agreement was made considering various potential enhancements to Type0-PDCCH for 3 MHz channel bandwidth, targeting for potential detection performance loss of Type0-PDCCH due to reduced number of RBs for CORESET#0.

Since RAN4 has already agreed the maximum number of RBs for 3 MHz channel bandwidth is 15 RBs, based on such restriction, the maximum number of AL that can be supported for Type0-PDCCH is limited to 4. This restriction can result in the potential performance degradation of PDCCH detection performance, however, the enhancements considered in the options may have tremendous specification impact. Actually, since the focused spectrum are only for low frequency bands, and all the transmission power could be allocated within the 3 MHz channel bandwidth, which implies the EPRE can be higher than 5 MHz system assuming the same total transmission power. The link performance of PDCCH is expected to be recovered by such power bosting, since the payload for Type0-PDCCH is limited and the channel coding gain is almost the same as the power boosting gain. Therefore, there may not be any DL coverage issue for Type0-PDCCH with maximal utilization of the RBs in the channel bandwidth. To this end, we didn’t see the need for supporting a change to the interleaver, or an enhanced CCE-to-REG mapping, or a new AL level, or a new PDCCH rate matching.

Proposal 5: For Type0-PDCCH, no enhancement with specification impact is needed, and conclude that the maximum number of AL for Type0-PDCCH is 4.

# Other Considerations

## CSI-RS for RRM

In the legacy NR system, a BWP can be configured with smaller than 5 MHz bandwidth already, after initial access, which is the also one used for CSI-RS based RRM. In this sense, the bands with 3 MHz channel bandwidth can follow the same method as legacy NR system to perform CSI-RS based RRM, and no enhancement is needed.

Proposal 6: RAN1 shall conclude that no enhancement to CSI-RS for RRM is needed.

## PUCCH Frequency Hopping

By not supporting transmission bandwidth between 3 MHz and 5 MHz, there is no need to consider PUCCH FH disabling because there is no misalignment between the gNB and the UE about the number of RBs.

Proposal 7: RAN1 shall conclude that no need to support PUCCH FH disabling.

# Conclusion

Proposals and observations from this contribution are summarized as follow:

Proposal 1: From RAN1 perspective, there is no need to consider further enhancement to access barring.

- Whether such functionality is indicated by SIB1 can be up to RAN2.

Observation 1: Truncating SS/PBCH block bandwidth to 15 RBs will lead to unacceptable complexity in initial cell search.

Observation 2: With power boosting on EPRE of the SS/PBCH block and maintaining the whole transmission power, the truncation of PBCH bandwidth from 15 RBs to 12 RBs has very minor impact to PBCH detection performance.

Proposal 2: For 3 MHz channel bandwidth in all applicable bands, SS/PBCH block bandwidth is truncated to 12 RBs, and a fixed SS/PBCH block structure with 12 RBs bandwidth is supported (e.g., the center 12 RBs of the legacy SS/PBCH block structure).

Proposal 3: For SS/PBCH block structure with 12 RB bandwidth after truncation, PSS EPRE and SSS EPRE in the SS/PBCH block are the same.

Proposal 4: For CORESET#0 configuration table:

- Reuse the legacy principle of using the minimum channel bandwidth to determine which CORESET#0 configuration table to use;

- For 3 MHz channel bandwidth, support a single CORESET#0 bandwidth as 12 RBs;

- No need to support CORESET#0 bandwidth between 13 and 24 RBs;

- For Option 1, two sub-options can work:

  - Option 1a: truncate the CORESET#0 bandwidth to 12 RBs after the CCE-to-REG mapping;

  - Option 1b: use the reserved row in the legacy table to support CORESET#0 configuration with 12 RB bandwidth and 0 RB offset (Table 2 as an example);

- For Option 2, the new table shall include configurations satisfying the following (Table 3 as an example):

  - Reuse all legacy configurations with 24 RB CORESET#0 bandwidth;

  - Add new configurations with 12 RB CORESET#0 bandwidth and 0 RB offset;

  - UE determines the CORESET#0 bandwidth according to the table directly and no puncturing is needed.

Proposal 5: For Type0-PDCCH, no enhancement with specification impact is needed, and conclude that the maximum number of AL for Type0-PDCCH is 4.

Proposal 6: RAN1 shall conclude that no enhancement to CSI-RS for RRM is needed.

Proposal 7: RAN1 shall conclude that no need to support PUCCH FH disabling.

# Reference

[1] RP-222645, “NR support for dedicated spectrum less than 5MHz for FR1”, Nokia, Nokia Shanghai Bell.

[2] Chairman notes, 3GPP RAN1#112, Athens, Greece, February, 2023.

[3] R1-2302276, Response to LS from RAN4 on spectrum less than 5 MHz, RAN.

[4] R1-2302275, Response to LS from RAN1 on transmission bandwidths for NR on dedicated spectrum less than 5 MHz, RAN.

[5] R1-2302269, Reply LS on NR support for dedicated spectrum less than 5 MHz for FR1, RAN4.