3GPP TSG RAN WG1 #113			R1-2305544

Incheon, South Korea, May 22nd – 26th, 2023

Agenda Item:		9.16.8

Source:			Samsung

Title:			Discussion on UE features for eRedCap

Document for:		Discussion and decision

1 Introduction

In this contribution, we discuss UE features for eRedcap.

2 Discussion

2.1 UE feature on BB bandwidth reduction

In previous RAN1 meetings [1-4], agreements related to BB bandwidth reduction for eRedcap are provided as below.

Agreement (RAN1#110bis) - For UE BB bandwidth reduction, a UE is not expected to receive an UL grant in a DCI with a PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable. - For UE BB bandwidth reduction, a UE is not expected to be configured with a CG grant with a PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable. Agreement (RAN1#111) - For UE BB complexity reduction, a UE is not expected to receive an UL grant in a RAR or in a DCI scrambled with TC-RNTI with a Msg3 PUSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable. - For UE BB complexity reduction, a UE is able to receive a DL assignment in a DCI with a unicast PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. - The number of PRB scheduled in DCI is not larger than the maximum number of PRB agreed in previous agreement from 110b-e Agreement (RAN1#112) - For UE BB bandwidth reduction, for PUSCH, select the following option for the maximum number of PRBs that the UE can transmit per slot or per hop, if applicable: - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS - For UE BB bandwidth reduction, for PDSCH (for both unicast and broadcast), select the following option for the maximum number of PRBs that the UE can process per slot: - Option 3: 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS - For UE BB complexity reduction, a UE is not expected to perform 2-step RACH with a MsgA PUSCH resource spanning a bandwidth of more than ~5 MHz per slot or per hop, if applicable. - For UE BB bandwidth reduction, for RAR (PDSCH) to Rel-18 RedCap UEs, the scheduling of RAR PDSCH is allowed to be larger than the maximum number of unicast PRBs that the UE can process per slot. - When the scheduling of RAR PDSCH is within the maximum number of unicast PRBs that the UE can process per slot, the legacy time between RAR reception and Msg3 transmission (not smaller than NT,1 + NT,2 + 0.5 ms) is applied. - When the scheduling of RAR PDSCH is larger than the maximum number of unicast PRBs that the UE can process per slot, - The UE receives the RAR and correspondingly transmits Msg3 if the TDRA for Msg3 in UL grant in RAR indicates that the time between RAR reception and Msg3 transmission is NOT smaller than NT,1 + NT,2 + 0.5 + X ms. - FFS: value(s) of X - Otherwise, the UE behavior is up to the UE implementation. - Note: it does not mean early indication is needed Agreement (RAN1#112bis) - For UE BB complexity reduction, a UE is able to receive a Msg4 PDSCH resource allocation spanning a bandwidth of more than ~5 MHz per slot. - The UE is not required to process a Msg4 PDSCH with a larger number of PRBs than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

Based on above agreements, it is subject to eRedcap to support BB bandwidth reduction with BW3 for PUSCH and PR3 for PDSCH.

Proposal #1: Introduce a capability for eRedcap to support BB bandwidth reduction with following restrictions:

  - For PUSCH, including DG PUSCH, CG PUSCH, Msg3 PUSCH and MsgA PUSCH, resource allocation is not expected to span a bandwidth of more than ~5 MHz per slot or per hop, and the maximum number of PRBs that the UE can transmit per slot or per hop is 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS

  - For unicast PDSCH, the maximum number of unicast PRBs that the UE can process per slot is 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS

  - For Msg4 PDSCH, UE is not required to process Msg4 PDSCH when the scheduling of Msg4 PDSCH is larger than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

  - For RAR PDSCH, when the scheduling of RAR PDSCH is larger than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS,

    - The UE receives the RAR and correspondingly transmits Msg3 if the TDRA for Msg3 in UL grant in RAR indicates that the time between RAR reception and Msg3 transmission is NOT smaller than NT,1 + NT,2 + 0.5 + X ms.

      - X is down selected between 0.5/0.25 and 1/0.5 ms for 15/30 kHz SCS

    - Otherwise, the UE behavior is up to the UE implementation.

2.2 UE feature on peak data rate reduction

In previous RAN1 meetings [1-4], agreements related to peak data rate for eRedcap are provided as below.

Agreement (RAN1#110bis) - UE peak data rate reduction is supported at least as an add-on to UE BB bandwidth reduction, - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ X. - FFS: the value of X - If UE peak data rate reduction is supported as a standalone feature, - The constraint vLayers·Qm·f ≥ 4 is relaxed to vLayers·Qm·f ≥ Y. - FFS: the value of Y - Note: Whether this option is supported will be decided in RAN plenary. Agreement (RAN1#112) For the relaxed constraint X in the earlier RAN1 agreement, down-select between X = 3 and X = 3.2.

Moreover, the following was endorsed in RAN #99 on UE peak data rate.

Rel-18 eRedCap UE capable of 20MHz + PR1 and Rel-18 eRedCap UE capable of BW3/PR3 + PR1 are designed/targeted to same peak data rate, i.e., 10Mbps Note 1: Peak data rate of "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is same including unicast and broadcast respectively. Note 2: PRB processing capability of "Rel-18 eRedCap: UE capable of 20MHz + PR1" is not limited to "25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS" and it corresponds to PRB size corresponding to 20 MHz. Note 3: The only difference between "Rel-18 eRedCap: UE capable of 20MHz + PR1" and "Rel-18 eRedCap: UE capable of BW3/PR3 + PR1" is Note 2 and vLayers·Qm·f in order to have the same peak rate. Note 4: The initial access procedure of Rel-18 eRedCap UE capable of 20MHz + PR1 is realized by following: - Same as Rel-18 eRedCap UE capable of BW3/PR3 + PR1

Based on above agreements, it is subject to eRedcap to support peak data rate as add-on to BB bandwidth reduction and a standalone feature.

Proposal #2: Introduce a capability for eRedcap to support peak data rate as add-on to BB bandwidth reduction.

Proposal#3: Introduce a capability for eRedcap to support peak data rate as a standalone feature.

2.3 UE feature on additional separate early indication

In RAN Plenary#98 meeting, additional separate early indication is included in the revised WID [6]. In previous RAN2 meeting, following agreements on additional separate early indication in Msg3 were made as below [7-8]：

Agreement (RAN2#121) - Introduce Msg3/MsgA PUSCH based early indication for Rel-18 eRedCap. FFS how to implement this in the spec (e.g., new LCIDs or not). Agreement (RAN2#121bis) - Working assumption: Use two new LCID values to support Msg3 early identification for eRedCap UE (can be revised and discussed together with other R18 WIs, if R18 WIs may occupy relatively many LCIDs).

Based on above agreements, it is subject to eRedcap to support additional separate early indication in Msg3.

Proposal #4: Introduce a capability for eRedcap to support additional separate early indication in Msg3.

In the last RAN1 meeting, additional separate early indication in Msg1 is supported by most companies [5]. In order to avoid scheduling restriction for RAR PDSCH and Msg3 for Rel-17 eRedcap UE, as discussed in our contribution [9], we propose additional separate early indication in Msg1 should be supported.

Proposal #5: Introduce a capability for eRedcap to support additional separate early indication in Msg1.

3 Conclusion

This paper discussed UE features for eRedcap. Based on the discussions, we have the following proposals:

Proposal #1: Introduce a capability for eRedcap to support BB bandwidth reduction with following restrictions:

  - For PUSCH, including DG PUSCH, CG PUSCH, Msg3 PUSCH and MsgA PUSCH, resource allocation is not expected to span a bandwidth of more than ~5 MHz per slot or per hop, and the maximum number of PRBs that the UE can transmit per slot or per hop is 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS

  - For unicast PDSCH, the maximum number of unicast PRBs that the UE can process per slot is 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS

  - For Msg4 PDSCH, UE is not required to process Msg4 PDSCH when the scheduling of Msg4 PDSCH is larger than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS.

  - For RAR PDSCH, when the scheduling of RAR PDSCH is larger than 25 PRBs for 15 kHz SCS and 12 PRBs for 30 kHz SCS,

    - The UE receives the RAR and correspondingly transmits Msg3 if the TDRA for Msg3 in UL grant in RAR indicates that the time between RAR reception and Msg3 transmission is NOT smaller than NT,1 + NT,2 + 0.5 + X ms.

      - X is down selected between 0.5/0.25 and 1/0.5 ms for 15/30 kHz SCS

    - Otherwise, the UE behavior is up to the UE implementation.

Proposal #2: Introduce a capability for eRedcap to support peak data rate as add-on to BB bandwidth reduction.

Proposal#3: Introduce a capability for eRedcap to support peak data rate as a standalone feature.

Proposal #4: Introduce a capability for eRedcap to support additional separate early indication in Msg3.

Proposal #5: Introduce a capability for eRedcap to support additional separate early indication in Msg1.

Reference

Chairman’s notes for RAN1#110bis Meeting

Chairman’s notes for RAN1#111 Meeting

Chairman’s notes for RAN1#112 Meeting

Chairman’s notes for RAN1#112bis Meeting

R1-2303936, FL summary #4 on Rel-18 RedCap UE complexity reduction, Ericsson

R1-223544, Revised WID on Enhanced support of reduced capability NR devices, Ericsson

Chairman’s notes for RAN2#121 Meeting

Chairman’s notes for RAN2#121bis Meeting

R1-2305525, Further UE complexity reduction for eRedCap, Samsung