3GPP TSG-RAN WG1 #113	R1-2305578

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item:	9.16.9

Source:	Ericsson

Title:	UE features for MCE

Document for:	Discussion

# 1	Introduction

In this contribution, we provide our views on the UE features for multi-carrier enhancements.

# 2	Views on the FGs

- FG 49-1

  - Component 2: OK to confirm the text in square brackets

  - Component 3: OK to confirm the text in square brackets with the understanding that both SCS and carrier type is same for scheduling and co-scheduled cells

  - Component 4: Suggest deleting the FFS, i.e. component 4 value applies to all cases. In our view the maximum number of sets and maximum number of co-scheduled cells in a set are enough –additional limits on maximum total number of cells across sets is not needed.

  - Component 5: UE reports maximum number of sets of cells across all PUCCH groups (i.e. no need to differentiate between PUCCH groups.

  - Component 6: OK to confirm the text in square brackets. Additional limit on maximum total number of cells across sets on a scheduling cell is not needed.

  - Component 7: Type 2 HARQ CB is mandatory from Rel-15, prevalent in CA scenarios. Hence, it should be included in the basic FGs.

  - Component 9: Both Type-2 and Type-1A should be supported in the basic FG definition.

  - Regarding number of unicast DCI(s), for same SCS between the scheduling and co-scheduled cells, the existing limits (1D+1U for FDD scheduling cell and 1D+2U or 2D+1U for TDD scheduling cell) can be applicable per slot of scheduling cell.

  - Regarding whether this FG is separated for the case when scheduling cell is not included in a set of cells and/or when scheduling cell is not the reference cell for the set, we prefer not separating into new FGs for either case.

  - Regarding whether to introduce new FG for monitoring of DCI 0_3/1_3 and legacy DCI scheduling for cell(s) in a set, new FG is not needed since legacy CCS capability can be reused, i.e. if UE indicates FG 6-10 and also indicates support for DCI 0_3/1_3, then UE supports monitoring of both DCI 0_3/1_3 and legacy DCI for scheduling cell(s) in a set.

- FG 49-1b

  - For components 4,5,6,7,9 and support of new FG for monitoring of DCI 0_3/1_3 and legacy DCI scheduling, we have same comments as for FG 49-1 above.

  - Regarding number of unicast DCI(s), for different SCS between the scheduling and co-scheduled cells, having smaller limits in basic FG is OK although this leads to significantly reduced throughput, particularly for low SCS-scheduling-high SCS. Hence, we think another FG for indication of increased number of unicast DCIs (like FG 18-5c/5d) should also be introduced like the following.

    - Introduce an FG 49-1b-1 for indication of increased number of unicast DCIs for DL

  - Processing up to X unicast DCI scheduling for DL per set of cells

    - X is based on pair of (scheduling CC SCS, SCS of the set of cells):

    - Candidate value(s) of X

      - X={2} for (15kHz,30kHz), (30kHz,60kHz), (60kHz,120 kHz)

      - X={2,4} for (15 kHz,120 kHz), (15 kHz,60 kHz), (30kHz,120kHz)

    - X applies per slot of scheduling CC

- FG 49-2

  - For components 2,3,4,5,6, and a) for the case separate FG when scheduling cell is not included in a set of cells or is a reference cell and b) support of new FG for monitoring of DCI 0_3/1_3 and legacy DCI scheduling and number of unicast DCIs, we have same comments as for FG 49-1 above.

  - Component 9: Both Type-2 and Type-1A should be supported in the basic FG.

- 49-2b

  - For components 4,5,6 and support of new FG for monitoring of DCI 0_3/1_3 and legacy DCI scheduling, we have same comments as for FG 49-1 above.

  - For number of unicast DCIs like 49-1b, increased number of unicast DCIs should be supported via another FG.

    - Introduce an FG 49-2b-1 for indication of increased number of unicast DCIs for UL

  - Processing up to X unicast DCI scheduling for UL per set of cells

    - X is based on pair of (scheduling CC SCS, SCS of the set of cells):

    - Candidate value(s) of X

      - X={2} for (15kHz,30kHz), (30kHz,60kHz), (60kHz,120 kHz)

      - X={2,4} for (15 kHz,120 kHz), (15 kHz,60 kHz), (30kHz,120kHz)

    - X applies per slot of scheduling CC

- FG 49-3

  - On the P(S)Cell, baseline should support monitoring of both legacy DCI formats and DCI format 0_3/1_3. Additionally, it should be noted that ‘legacy DCI formats’ and DCI format 0_3/1_3 are for scheduling of the same scheduled cell from the same scheduling cell.

  - Discussion on FG 49-3 should be limited to case of monitoring on an SCell.

  - Monitoring of both legacy DCI formats and DCI format 0_3/1_3 on P(S)Cell if P(S)Cell is scheduling cell for a set of cells is included in basic FGs.

- FG 49-4a

  - 49-4a is not needed. Nominal RBG size of Configuration 3 for FDRA type 0 is included in basic FGs.

- FG 49-4b

  - 49-4 is not needed. FDRA Type 1 granularity of 2, 4, 8, or 16 consecutive RBs based RIV for DCI format 1_3/0_3 is included in basic FGs.

- FG 49-5

  - Type 2 HARQ CB is mandatory from Rel-15, prevalent in CA scenarios. Hence, it should be included in the basic FGs.

  - 49-5 is not needed. Type 2 HARQ CB support for DCI format 1_3 is included in basic FGs.

# References

R1-2304282, “Initial RAN1 UE features list for Rel-18 NR after RAN1#112bis-e”, Moderators (AT&T, NTT DOCOMO, INC.), RAN1#112-bis-e, April 2023.