3GPP TSG RAN WG1 #113			R1-2305616

Incheon, Korea, May 22nd – May 26th, 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on PRACH coverage enhancements

Agenda Item:	9.12.1

Document for: 	Discussion and Decision

- Introduction

At RAN#96-e meeting, revised WID on further NR coverage enhancements for NR was approved with the objective as follows [1]:

- Specify following PRACH coverage enhancements (RAN1, RAN2) - Multiple PRACH transmissions with same beams for 4-step RACH procedure - Study, and if justified, specify PRACH transmissions with different beams for 4-step RACH procedure - Note 1: The enhancements of PRACH are targeting for FR2, and can also apply to FR1 when applicable. - Note 2: The enhancements of PRACH are targeting short PRACH formats, and can also apply to other formats when applicable. - Study and if necessary specify following power domain enhancements - Enhancements to realize increasing UE power high limit for CA and DC based on Rel-17 RAN4 work on “Increasing UE power high limit for CA and DC”, in compliance with relevant regulations (RAN4, RAN1) - Enhancements to reduce MPR/PAR, including frequency domain spectrum shaping with and without spectrum extension for DFT-S-OFDM and tone reservation (RAN4, RAN1) - Specify enhancements to support dynamic switching between DFT-S-OFDM and CP-OFDM (RAN1)

In this contribution, remaining issues for PRACH coverage enhancement are discussed.

- Discussions

2.1 Multiple PRACH transmissions with the same Tx beam

Issue 1: Application use case

According to the WID objectives, multiple PRACH transmissions with the same Tx beam will be specified for 4-step RACH procedure in Rel-18. Considering there are multiple RACH types, e.g., contention free RACH (CFRA) and contention based RACH (CBRA), application use cases for multiple PRACH transmissions needs to be clarified first.

From specification perspective, multiple PRACH transmissions for CFRA may have smaller impact than multiple PRACH transmissions for CBRA, and also smaller impact on legacy UEs.

From coverage extension perspective, the realistic coverage would generally be determined based on initial access signal/channels. Thus, PRACH coverage extension during initial access (e.g., CBRA) would be very important. For RACH during CONNECTED MODE (e.g., CFRA and CBRA), PRACH coverage extension may also be needed. For example, multiple PRACH transmissions for CFRA can improve PRACH detection rate in SNR limited scenarios, e.g. for handover and beam failure recovery cases.

Therefore, multiple PRACH transmissions should be supported for both CBRA and CFRA, considering both RACH during initial access and RACH in CONNECTED mode in Rel-18.

Proposal 1: Support multiple PRACH transmissions for RA during initial access (e.g. CBRA) and RA during CONNECTED MODE (e.g. CFRA and CBRA).

Issue 2: RO group

Periodic feature for RO group

At RAN1#112bis-e meeting, periodic feature of RO group configuration/determination was discussed, where the following proposal was discussed and very near to be agreed. The intention of introducing such periodic feature is to make the RO group determination/configuration simpler, i.e. only RO group in one time period needs to be configured/determined, with the same pattern repeated in each period. It is similar to the introduction of SSB-to-RO association pattern period in legacy specification..

Proposal A set of RO group(s) for a configured number of multiple PRACH transmissions is [determined/configured] within a time period X, starting from frame 0. The [determined/configured] set of RO groups repeats every period X. - FFS: the determination of time period X. - FFS: whether the same time period X is applied to all the configured number(s) of multiple PRACH transmissions.

The main controversial point of this proposal is about the determination of time period X. Three options were discussed in last meeting:

- Opt. 1: K PRACH configuration period(s)

- Opt. 2. K SSB-to-RO association period(s)

- Opt. 3. K SSB-to-RO association pattern period(s)

For opt 1, as one PRACH configuration period may not accommodate ROs for a burst of SSBs, and RO-to-SSB mapping may be different in different PRACH configuration periods, opt 1 may complicate RRC configuration policy. For opt 2, considering invalid ROs are not mapped to any SSB index, the length/duration of SSB-to-RO association periods may be different. Opt 2 may also complicate RRC configuration policy. For opt 3, the duration of per SSB-to-RO association pattern period is fixed, and ROs for an integer of SSB bursts is ensured. Therefore, opt 3 is the simplest solution.

Fig 1: Example of PRACH configuration period, association period, and association pattern period

Another discussion point is whether same time period X would be applied to all the configured number(s) of PRACH transmissions. In our view, either same or separate time period X for different configured number of PRACH transmissions can work. From simplicity perspective, a common time period X applied for all configured number of multiple PRACH transmissions is slightly preferred.

Proposal 2:

- A set of RO group(s) for a configured number of multiple PRACH transmissions is determined/configured within a time period X, starting from frame 0. The determined/configured set of RO groups repeats every period X.

- The time period X is K SSB-to-RO association pattern periods.

- The same time period X can be applied for all configured number of PRACH transmissions.

RO group configuration/determination

At RAN1#112bis-e meeting, two options to determine/configure RO group were discussed, as in the following proposal.

Proposal Consider one or both of the following options to realize RO group(s) determination/configuration. - Option 1: RO group(s) are implicitly determined based on network configuration. - RO group is determined at least by the following parameters {time and frequency start position (or start RO), the number of ROs within the RO group} - FFS: whether the parameters can be derived based on some rules without explicit configuration. - FFS: whether the starting RB of ROs within an RO group can be different at different time instances, if supported, the details. - Option 2: RO group(s) are directly configured by network, e.g., via SIB1. - FFS: details. - Note: details of characteristics of the RO groups [, e.g., whether partial RO overlap is supported, max duration of the RO group (if any), starting RO and so on] are up to RAN1, while details on how to realize the direct configuration of RO groups by network [, e.g., signalling,] are up to RAN2.

For option 1, RO groups are determined based on some parameters and certain rules. Option 1 requires more RAN1 specification effort than option 2, e.g. identification of needed parameters, and definition for rules to determine RO group(s) based on some parameters.

For option 2, each RO in a RO group is directly signaled by network. Less RAN1 work is expected than option 1. Moreover, option 2 can provide better flexibility since gNB can indicate any allowed combination of RO positions for ROs within one RO group, e.g. smart RO group configurations to make RO resource utilization ratio as high as possible. However, option 2 may cause overhead issue, especially if the signaling of RO group(s) is via SIB1.

Based on above analysis, both option 1 and option 2 have pros and cons. From our perspective, we slightly prefer option 1 considering the signaling overhead concern of option 2. But option 2 is also fine to make RAN1 design simpler. In RAN1#112bis meeting, companies had different preferences for this issue, and a proposal to support both was proposed as a middle way.  In other words, both options are supported and it is up to gNB to apply either one. For example, if RO group(s) are explicitly indicated by gNB, RO group(s) would be determined by the direct configuration. Otherwise, RO group(s) would be determined by some parameters and certain rule. In our understanding, whether it is a feasible solution depends on whether separate UE capabilities would be required for the two options. If separate UE capabilities would not be required, it is a good solution to move forward. However, if separate UE capabilities would be required, gNB needs to consider both UEs only capable of option 1 and UEs only capable of option 2 when configuring PRACH resources. In this case, the PRACH resource configuration needs to be very careful, which is not very preferable from network operator perspective. From our perspective, we don’t think separate UE capabilities are necessary for the two methods.

Proposal 3: For determination/configuration of RO group(s), support both option 1 (implicitly determined based on network configuration) and option 2 (directly configured by network), and no separate UE capabilities are required for the two methods.

As analyzed above, more details need to be clarified for option 1 if option 1 is supported.

For the case of same frequency location for ROs within an RO group, the listed parameters {time and frequency start position (or start RO), the number of ROs within the RO group} in the agreement are sufficient. Regarding the time and frequency start position (or start RO), the time starting position can be defined/configured by time domain index among ROs associated with the same SSB/CSI-RS within a certain time period, and the frequency starting position can be defined/configured as frequency domain index among FDMed ROs associated with the same SSB/CSI-RS in the same time instance.

Different frequency positions for ROs within one RO group can bring frequency hopping gain, which is beneficial for PRACH coverage improvement. It can be realized by allocating different frequency index for different time occasions in one RO group, when multiple FDMed ROs associated with the same SSB/CSI-RS in the same time instance. For example, a frequency index offset among different ROs within one RO group can be defined/indicated to achieve such PRACH hopping like scheme.

- Example 1: If frequency index #m is for the k-th RO in RO group, the frequency index for the (k+1)-th RO in the RO group would be #(m+off_set) mod N, where N is the number of FDMed ROs for the SSB in the one time occasion, and off_offset is the frequency offset.

- Example 2: For odd-indexed RO (e.g. 1st, 3rd, 5th, 7th RO) in one RO group, frequency index is the same as the first RO (e.g. frequency index #m). For even-indexed RO (e.g. 2nd, 4th, 6th, 8th RO) in one RO group, frequency index is (m+ offset_index) mod N, where N is the number of FDMed ROs for the SSB in the one time occasion, and off_offset is the frequency offset.

- Example 3: For the first K/2 TDMed ROs in one RO group, frequency index is the same as the first RO (e.g. frequency index #m). For the last K/2 ROs in RO group, frequency index is (m+ offset_index) mod N, where N is the number of FDMed ROs for the SSB in the one time occasion, and off_offset is the frequency offset.

Fig 2: different frequency locations for ROs within one RO group

In example 1, K PRACHs are transmitted on min(K, N) frequency hops with each PRACH transmission shifted in frequency domain relative to the previous PRACH transmission. In example 2 and 3, K PRACHs are transmitted on 2 hops. The example 2 is similar to PUSCH/PUCCH frequency hopping scheme without DMRS bundling, while example 3 is similar to PUSCH/PUCCH frequency hopping case with two DMRS bundle groups.

Proposal 4: Support different frequency locations for ROs within one RO group, when there are multiple valid FDMed ROs in one time occasion for one SSB/CSI-RS.

- It can be achieved by indicating/defining a frequency domain index offset among different ROs within one RO group.

Overlapping of RO groups

Whether to allow overlapping of RO groups was discussed in last meeting.

For RO groups for the same number of PRACH transmissions, collision among different RO groups may cause some issue. For example, if different UEs transmits the same number of multi-PRACH on collided RO groups as in Fig 2, when gNB detects 4 PRACH transmissions with preamble index for 2 PRACH transmissions in the 4 ROs, gNB is not able to differentiate the 4 PRACH transmissions, and gNB doesn’t know when to send RAR. Sometimes partial RO group overlapping can work without any issue, e.g. smart RO group configuration by gNB as proposed by some companies in last meeting, while it increases gNB detection complexity. Therefore, RO overlapping between multiple RO groups for the same number of PRACH transmissions is not preferred.

Fig 2: collision for RO groups for the same number of PRACH transmissions

Proposal 5: One RO should not be included in multiple RO groups associated with the same SSB/CSI-RS for the same number of PRACH transmissions.

In order to support differentiation between different number of multiple PRACH transmissions, separate PRACH resources for different number of PRACH transmissions are needed. Following two alternatives can be considered:

- Alt 1: Separate preamble resources on shared RO resources for different number of multiple PRACH transmissions.

- Alt 2: Separate RO resources for different number of multiple PRACH transmissions.

For Alt 1, RO groups for different number of PRACH transmissions can be overlapped. For Alt 2, ROs in RO groups for different number of PRACH transmissions should be not overlapped.

Alt 2 needs more RO resource than Alt 1, while Alt 2 may be easier to implement joint detection for each number of repetitions. For Alt 1, the number of PRACH transmissions is obtained based on decoded preamble, gNB is not certain about the exact RO group location before decoding the preamble on a RO included in multiple RO groups. If gNB can only perform joint detection for multi-PRACH (i.e. decoding one of multiple PRACH transmissions is not feasible), in a RO included in RO groups for different number of PRACH transmissions, gNB needs to try to detect multi-PRACH in multiple RO groups with different preamble resource. For Alt 2, for each RO, the RO group including the RO is certain. gNB can perform joint detection on ROs within one RO group.

Proposal 6: To differentiate different number of PRACH transmissions, support separate RO resource for different number of PRACH transmissions.

Issue 3: PRACH transmission power

According to the legacy PRACH transmission power calculation equation for single PRACH transmission, if without maximum UE transmission power limitation, the target PRACH reception power at gNB side is aimed to be (preambleReceivedTargetPower+ DELTA_PREAMBLE), after the pathloss is compensated.

Directly reusing the legacy TPC equation for each PRACH transmission of multiple PRACH transmissions will result in higher power than required, assuming 10*log10(number of repetitions) dB gain. Such an excessive power may cause larger interference on other UEs. To avoid the issue, adjustment on target preamble reception power for multiple PRACH transmissions can be applied based on, e.g. a power reduction offset dependent on the number of PRACH transmissions or the number of ROs in the RO group, which is similar to PRACH transmission power determination for LTE MTC PRACH repetitions.

Proposal 7: For multiple PRACH transmissions, the preamble received target power is adjusted based on the number of PRACH transmissions.

- PREAMBLE_RECEIVED_TARGET_POWER =  preambleReceivedTargetPower – Multi-PRACH-offset-RepK + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER–1)× PREAMBLE_POWER_RAMPING_STEP,

  - where the value of Multi-PRACH-offset-RepK is dependent on actual transmitted number of PRACH transmissions or dependent on the number of ROs in the RO group.

Issue 4: Number of multiple PRACH transmissions

At RAN1#112 meeting, it was agreed gNB can configure one or multiple values from {2, 4, 8} for the number of PRACH transmissions. When multiple candidate values for number of multiple PRACH transmissions are configured, how to determine the value needs to be discussed.

For PDCCH ordered RA, it is also possible to indicate the number of transmissions in PDCCH order. Following PDCCH order indication is simpler and more efficient than UE autonomous determination based on certain condition. A possible method is to indicate the value via ‘reserved bits’ in the DCI directly. Another method is that the PDCCH order indicates multiple mask indexes when triggering RA with multiple PRACH transmissions, with each mask index for corresponding PRACH transmission. For example, similar to the TDRA table design for single DCI scheduling multiple PDSCHs/PUSCHs, a mask index table with each row including one or multiple number of mask indexes can be considered, and the PDCCH order indicates a row index of the table, as the following Table 1. For this case, the number of PRACH transmissions is implicitly determined by the number of mask indexes.

Table 1: Example of PDCCH ordering indicating multiple mask indexes

| Row index | PRACH mask index #0 | PRACH mask index #1 | ……. | PRACH mask index #K |
| --- | --- | --- | --- | --- |
| 0 | 2 | - |  | - |
| 1 | 1 | 3 |  | - |
| 2 | 2 | 4 | …. | 8 |

When UE-specific signaling is not available for RA, e.g., RA initiated for initial access, it should be allowed that UE determines the number of PRACH transmissions (including whether single or multiple PRACH transmissions) based on certain condition(s) by itself. It was agreed in RAN1#111 meeting that at least RSRP of the selected SSB/CSI-RS is used to determine the number of PRACH transmissions for the first RACH attempt.

The main controversial point is whether to rely on other factors than SSB-RSRP to determine the number of PRACH transmissions for the first RACH attempt. In last meeting, some companies proposed to limit that multiple PRACH transmissions are triggered only when maximum transmission power is reached. However, as discussed in Issue 3, when the PRACH transmission power is determined with considering the number of PRACH transmissions, it is not necessary to impose maximum transmission power reached as the condition to trigger multiple PRACH transmissions. To make the rule as simple as possible, only relying on SSB-RSRP to determine the number of PRACH transmissions is enough.

For retransmission RACH attempt, SSB-RSRP is also useful to reflect the current channel condition and required SNR gap. Unified rule for determining number of PRACH transmissions as the first RACH attempt is simple.

The number of SS-RSRP thresholds is dependent on the configured number of candidate values for the number of multiple PRACH transmissions. For example, if candidate values for multiple PRACH transmissions configured by gNB are {2, 4, 8}, UE can determine the number of PRACH transmissions (from 1, 2, 4, 8) by comparing the configured SS-RSRP thresholds and RSRP of the selected CSI-RS/SSB.

Proposal 8: When multiple values are configured for the number of multiple PRACH transmissions,

- For PDCCH ordered RA, the PDCCH order DCI indicates the number of PRACH transmissions, which can be explicitly indicated by ‘reserved bits’, or implicitly indicated by the number of indicated mask indexes.

- For RA without PDCCH order, UE determines the number of PRACH transmissions only based on SSB-RSRP threshold for any RACH attempt.

Issue 5: RAR reception

At RAN1#112bis meeting, it was agreed that single RAR window for multiple PRACH transmissions starts after the last RO in the RO group.

Agreement The starting point of RAR window is after the last symbol of the last valid RO in the RO group corresponding to the multiple PRACH transmissions. Note: Valid RO(s) refers to what is defined in existing specification, i.e., Section 8.1 in TS 38.213. Note: The last valid RO is irrespective of whether the PRACH transmission on the last valid RO in the RO group is dropped or not

The remaining issue for RAR reception is how to determine RA-RNTI for RAR monitoring. In our understanding, it may depend on whether to allow RO group overlapping for same number of PRACH transmissions, as discussed in Issue 2.

If RO group overlapping for the same number of PRACH transmissions is not allowed, or at least the first/last RO is not allowed to be included in multiple RO groups for the same number of PRACH transmissions, the simplest method is to calculate RA-RNTI based on the first/last RO in the RO group.

If RO group overlapping is allowed for the same number of PRACH transmissions, and the first/last RO is allowed to be included in multiple RO groups for the same number of PRACH transmissions, RA-RNTI calculation needs to be based on all ROs in one RO group.

As discussed in Issue 2, we prefer to avoid RO group overlapping for the same number of PRACH transmissions. Therefore, RA-RNTI calculation based on the first/last RO is preferred.

Proposal 9: For monitoring RAR for multiple PRACH transmissions within one RACH attempt,

- RA-RNTI is calculated based on the first or last PRACH transmission.

2.2 Multiple PRACH transmissions with different beams

At RAN1#112 meeting, a conclusion was made that multiple PRACH transmissions are only transmitted on ROs associated with the same SSB/CSI-RS. In our understanding, there are two possibilities for this case:

- Alt 1: UE uses different narrow Tx beams corresponding to the same SSB/CSI-RS.

- Alt 2: UE uses different Tx beams corresponding to different SSBs/CSI-RSs.

The Alt 1 scheme is helpful to identify the best Msg 3 Tx beam, which is beneficial to improve Msg 3 coverage. In our understanding, most designs for multiple PRACH transmissions with same Tx beam can be reused for multiple PRACH transmissions with different Tx beams. The additional specification effort would not be large.

If identification of best Msg 3 Tx beam is pursued, gNB needs to perform decoding for each PRACH transmission of different Tx beams, while multiple PRACH transmissions with same Tx beam doesn’t require separate decoding for each PRACH transmission. Therefore, gNB needs to differentiate multiple PRACH transmissions with same Tx beam and with different Tx beams. Separate PRACH resource for multiple PRACH transmissions with different Tx beams from multiple PRACH transmissions with same Tx beam is needed. The same method used for separation of PRACH resource for single-PRACH and for multi-PRACH with same Tx beam can be reused. The specification impact is small.

Proposal 10: Support multiple PRACH transmissions with different Tx beams over ROs associated with the same SSB/CSI-RS.

- PRACH resources differentiation between multiple PRACH transmissions with different Tx beams and multiple PRACH transmissions with same Tx beam is supported.

- Conclusion

In this contribution, we discussed possible solutions for PRACH coverage enhancements. We have following proposals:

Proposal 1: Support multiple PRACH transmissions for RA during initial access (e.g. CBRA) and RA during CONNECTED MODE (e.g. CFRA and CBRA).

Proposal 2:

- A set of RO group(s) for a configured number of multiple PRACH transmissions is determined/configured within a time period X, starting from frame 0. The determined/configured set of RO groups repeats every period X.

- The time period X is K SSB-to-RO association pattern periods.

- The same time period X can be applied for all configured number of PRACH transmissions.

Proposal 3: For determination/configuration of RO group(s), support both option 1 (implicitly determined based on network configuration) and option 2 (directly configured by network), and no separate UE capabilities are required for the two methods.

Proposal 4: Support different frequency locations for ROs within one RO group, when there are multiple valid FDMed ROs in one time occasion for one SSB/CSI-RS.

- It can be achieved by indicating/defining a frequency domain index offset among different ROs within one RO group.

Proposal 5: One RO should not be included in multiple RO groups associated with the same SSB/CSI-RS for the same number of PRACH transmissions.

Proposal 6: To differentiate different number of PRACH transmissions, support separate RO resource for different number of PRACH transmissions.

Proposal 7: For multiple PRACH transmissions, the preamble received target power is adjusted based on the number of PRACH transmissions.

- PREAMBLE_RECEIVED_TARGET_POWER =  preambleReceivedTargetPower – Multi-PRACH-offset-RepK + DELTA_PREAMBLE + (PREAMBLE_POWER_RAMPING_COUNTER–1)× PREAMBLE_POWER_RAMPING_STEP,

  - where the value of Multi-PRACH-offset-RepK is dependent on actual transmitted number of PRACH transmissions or dependent on the number of ROs in the RO group.

Proposal 8: When multiple values are configured for the number of multiple PRACH transmissions,

- For PDCCH ordered RA, the PDCCH order DCI indicates the number of PRACH transmissions, which can be explicitly indicated by ‘reserved bits’, or implicitly indicated by the number of indicated mask indexes.

- For RA without PDCCH order, UE determines the number of PRACH transmissions only based on SSB-RSRP threshold for any RACH attempt.

Proposal 9: For monitoring RAR for multiple PRACH transmissions within one RACH attempt,

- RA-RNTI is calculated based on the first or last PRACH transmission.

Proposal 10: Support multiple PRACH transmissions with different Tx beams over ROs associated with the same SSB/CSI-RS.

- PRACH resources differentiation between multiple PRACH transmissions with different Tx beams and multiple PRACH transmissions with same Tx beam is supported.

References

- RP-221858, Revised WID on Further NR coverage enhancements, June 2022, China Telecom

- 3GPP, RAN1#110bis-e meeting, Chairman’s notes

- 3GPP, RAN1#111 meeting, Chairman’s notes

- 3GPP, RAN1#112 meeting, Chairman’s notes

- 3GPP, RAN1#112bis meeting, Chairman’s notes