3GPP TSG RAN WG1 #113			           R1-2305677

Incheon, Korea, May 22nd – May 26th, 2023

Agenda Item: 9.7.2

Source: MediaTek Inc.

Title: On NW Energy Saving Enhancements for Cell DTX/DRX Mechanism

Document for: Discussion and Decision

# Introduction

In RAN1#112-bis-e meeting, the following agreements are achieved [2][3]:

Agreement From RAN1 point of view, Rel-18 UE supporting cell DTX does not expect to receive and/or process the following signals/channels from the gNB, during non-active periods of cell DTX. The list of signals/channels may be updated based on RAN2/RAN4 input and other signals/channels are not precluded from further discussions. - Periodic/Semi-persistent CSI-RS configured in CSI report configuration in CSI-ReportConfig with reportQuantity including RI (for CSI reporting) - FFS: - PDCCH in USS - UE behaviour for retransmission - if some specific RNTI scrambled PDCCH in USS will be excluded from cell DTX operation - PDCCH in Type-3 CSS - UE behaviour for retransmission - if some specific RNTI scrambled PDCCH in Type-3 CSS will be excluded from cell DTX operation - PRS - CSI-RS configured by measObjectNR (for RRM) - CSI-RS associated with RadioLinkMonitoringConfig and BeamFailureDectection (for RLM and BFD) - Periodic CSI-RS configured with trs-Info ‘true’ (for tracking) - Periodic/Semi-persistent CSI-RS (for BM) - FFS on how to differentiate (if needed) with other CSI-RS used for CSI reports for BM - FFS: Whether the same or different UE behaviour is applicable with or without C-DRX - FFS: Whether the list of impacted signals/channels can be configurable - FFS: Whether there will be exception case(s) for UE receiving and/or processing listed signals/channels during non-active periods of DTX - FFS: RAN1 to consider impact on system if the channels/signals are not transmitted during non-active period Agreement Study L1 signalling for enhancing cell DTX/DRX including activation/deactivation for a single configuration which will have the following characteristics: - PDCCH based signaling - FFS: Whether enhancing legacy DCI or introducing new DCI - FFS: DCI content - FFS: Whether L1 signaling is UE specific DCI or group common DCI - FFS: Timer or validity duration based activation/deactivation of cell DTX/DRX - FFS: whether to specify a reference time for activation/deactivation of cell DTX/DRX - FFS: If multiple Cell DTX/DRX patterns are to be supported - FFS on detailed UE behavior upon reception of L1 signaling at least including application delay - FFS how to guarantee reliability of the L1 signaling - FFS whether the L1 signal can be monitored in non-active periods. Agreement From RAN1 point of view, Rel-18 UE supporting cell DRX is not expected to transmit the following signals/channels to the gNB during non-active periods of cell DRX. The list of signals/channels may be updated based on RAN2/RAN4 input and other signals/channels are not precluded from further discussions. - Periodic/Semi-persistent CSI report - Periodic/Semi-persistent SRS - FFS: SRS for positioning - FFS: - HARQ feedback for SPS PDSCH - FFS whether there will be exception case(s) for UE transmitting listed signals/channels during non-active periods of DRX - FFS Whether the listed signals/channels can be configurable by gNB - FFS: Whether the same or different UE behavior is applicable with or without C-DRX - FFS: RAN1 to consider impact on system if the channels/signals are not transmitted during non-active period Further study the following in RAN1: - Handling of HARQ-ACK codebook generation when configured with cell DTX/DRX - Handling of PUCCH deferral operation during non-active periods of cell DRX - Handling of overlapping channels where a least a channel overlaps with non-active periods of cell DTX/DRX - Handling of signals/channels that can be received/transmitted repeatedly during non-active periods of cell DTX/DRX - Handling of PUCCH switching during non-active period to an active cell - Other enhancements are not precluded. Agreement For PDDCH monitoring, further work on Rel-18 NES in RAN1 is to follow the RAN2 agreement below: 10. The understanding for the gNB scheduling behaviour for new transmissions during Cell DTX non-active period is that the gNB does not schedule UE-specific dynamic grants/assignments, even if the UE is in C-DRX Active Time. UE doesn’t monitor PDCCH for dynamic grants/assignments for new transmissions during Cell DTX non-active period, even if the UE is in C-DRX Active time. FFS how to deal with any exceptions (e.g. SR if agreed and RACH). Working Assumption Support of L1 signaling at least for activation/deactivation of a cell DTX and/or DRX configuration is feasible (e.g., in terms of enabling/disenabling the feature) from RAN1 perspective. - This does not imply that L1 activation/deactivation is supported in Rel-18\ - Note: Reliability, overhead, and benefits are FFS

In this contribution, we will particularly focus on the following two aspects for the mechanism of cell DTX/DRX:

- Section 2: UE Behaviors in Non-Active Periods of Cell DTX/DRX

- Section 3: Design Considerations of L1 Indication for Cell DTX/DRX

# UE Behaviors in Non-Active Periods of Cell DTX/DRX

With the implementation of cell DTX to improve network energy efficiency, there are periods when the gNB system is in a non-active state. During these non-active periods, RAN2 confirms that the gNB does not schedule any UE-specific dynamic grants or assignments. As a result, PDCCH processing related to dynamic grants can be reduced, contributing to further energy savings for Release 18 UEs that support NES.

To provide maximum flexibility and maintain consistency with RAN2 decisions, it is proposed that during the non-active periods of cell DTX, the behavior of UEs with respect to PDCCH processing in DRX Active time should be determined and led by RAN2. This will allow for efficient and well-coordinated cross-WG work for cell DTX/DRX.

Observation 1: During cell DTX non-active period, RAN2 agrees that the gNB does not schedule UE-specific dynamic grants/assignments. Accordingly, PDCCH processing related to dynamic grants can be saved for Rel-18 UEs supporting NES, whereas the detailed design can be up to RAN2.

Proposal 1: During non-active periods of cell DTX, UE behavior on PDCCH processing in DRX Active time is up to RAN2.

While the cell DTX mechanism is designed to save energy during non-active periods, it is essential for the UE to continue maintaining link and beam management. Failing to do so could negatively impact network performance and user experience, potentially undermining the benefits of the cell DTX mechanism for network energy saving.

To ensure that network performance and user experience are not compromised during cell DTX non-active periods, the following aspects should remain unaffected:

- CSI-RS configured by measObjectNR (for Radio Resource Management)

- CSI-RS associated with RadioLinkMonitoringConfig and BeamFailureDetection (for Radio Link Monitoring and Beam Failure Detection)

- Periodic CSI-RS configured with trs-Info set to 'true' (for tracking purposes)

- Periodic/Semi-persistent CSI-RS (for Beam Management)

By preserving these critical functions during cell DTX non-active periods, the 5G NR system can achieve a balance between energy savings and maintaining network performance and user satisfaction.

Observation 2: During cell DTX non-active period, UE still needs to maintain link and beam management; otherwise, network energy saving based on cell DTX mechanism can compromise network performance and user experience.

Proposal 2:  During non-active periods of cell DTX, the following are not impacted

  - CSI-RS configured by measObjectNR (for RRM)

  - CSI-RS associated with RadioLinkMonitoringConfig and BeamFailureDectection (for RLM and BFD)

  - Periodic CSI-RS configured with trs-Info ‘true’ (for tracking)

  - Periodic/Semi-persistent CSI-RS (for BM)

# Design Considerations of L1 Indication for Cell DTX/DRX

Cell DTX/DRX mechanisms can greatly contribute to network energy savings, particularly in scenarios with light network loads, such as when the network supports numerous Voice over IP (VoIP) users [3][4]. Therefore, it is essential to design L1 indications for cell DTX/DRX that minimize signaling overhead to ensure efficient energy savings in these use cases.

Observation 3: Cell DTX/DRX can provide significant network energy saving for scenarios with light network load, including the case there are large number of VoIP users in the network. L1 indication for cell DTX/DRX should aim for minimum signaling overhead for such case.

In situations where the gNB refrains from scheduling UE-specific dynamic grants or assignments during cell DTX non-active periods, there is a risk of UEs missing the cell DTX indication and consequently failing to receive scheduled data. To address this concern, L1 indications for cell DTX/DRX should strive to achieve maximum coverage, minimizing the possibility of such issues arising.

Observation 4: Since gNB does not schedule UE-specific dynamic grants/assignments during cell DTX non-active period, there is risk a UE missing cell DTX indication will also miss its data scheduling. L1 indication for cell DTX/DRX should aim for maximum coverage to prevent the issue.

By implementing cell-wise indications, paging indications can effectively minimize signaling overhead while maintaining guaranteed coverage performance. This approach ensures an optimized balance between energy savings and signal coverage.

Observation 5: Paging indication can minimize signaling overhead by cell-wise indication with guaranteed coverage performance.

In light of the observations above, RAN1 should consider extending paging indication DCI formats to include the activation and deactivation of at least one cell DTX/DRX configuration. This adaptation will help to further enhance energy savings in 5G NR systems while ensuring reliable service and performance for users. Incorporating these changes into the DCI format will facilitate the effective implementation of cell DTX/DRX mechanisms in a wide range of use cases, including those with large numbers of VoIP users.

Proposal 3: RAN1 considers extending paging indication DCI formats for indication of activation/deactivation for at least a single configuration of cell DTX/DRX.

# Summary

The cell DTX/DRX mechanism is an essential technique in achieving network energy saving for 5G NR systems. This approach involves controlling DTX and DRX periods, allowing radio equipment to enter low-power states when there is no data to transmit or receive. However, it is crucial to strike a balance between energy savings, network performance, and user experience to ensure an efficient and sustainable communication infrastructure. In this contribution the following observations and proposals are provided:

Observation 1: During cell DTX non-active period, RAN2 agrees that the gNB does not schedule UE-specific dynamic grants/assignments. Accordingly, PDCCH processing related to dynamic grants can be saved for Rel-18 UEs supporting NES, whereas the detailed design can be up to RAN2.

Proposal 1: During non-active periods of cell DTX, UE behavior on PDCCH processing in DRX Active time is up to RAN2.

Observation 2: During cell DTX non-active period, UE still needs to maintain link and beam management; otherwise, network energy saving based on cell DTX mechanism can compromise network performance and user experience.Observation 2: During cell DTX non-active period, UE still needs to maintain link and beam management; otherwise, network energy saving based on cell DTX mechanism can compromise network performance and user experience.

Proposal 2:  During non-active periods of cell DTX, the following are not impacted

  - CSI-RS configured by measObjectNR (for RRM)

  - CSI-RS associated with RadioLinkMonitoringConfig and BeamFailureDectection (for RLM and BFD)

  - Periodic CSI-RS configured with trs-Info ‘true’ (for tracking)

  - Periodic/Semi-persistent CSI-RS (for BM)

Observation 3: Cell DTX/DRX can provide significant network energy saving for scenarios with light network load, including the case there are large number of VoIP users in the network. L1 indication for cell DTX/DRX should aim for minimum signaling overhead for such case.

Observation 4: Since gNB does not schedule UE-specific dynamic grants/assignments during cell DTX non-active period, there is risk a UE missing cell DTX indication will also miss its data scheduling. L1 indication for cell DTX/DRX should aim for maximum coverage to prevent the issue.

Observation 5: Paging indication can minimize signaling overhead by cell-wise indication with guaranteed coverage performance.

Proposal 3: RAN1 considers extending paging indication DCI formats for indication of activation/deactivation for at least a single configuration of cell DTX/DRX.

# Reference

“Draft Report of 3GPP TSG RAN WG1 #112-bis-e v0.1.0”, online available @ https://www.3gpp.org/ftp/tsg_ran/WG1_RL1/TSGR1_112b-e/Report

R1-2304239, “Discussion summary #6 of issues for enhancements on cell DTX/DRX mechanism”, Intel, RAN1#112-bis-e

R1-2212259, “NW Energy Savings Performance Evaluation”, MediaTek, RAN1#111

3GPP TR 38.864, “Study on network energy savings for NR (Release 18)”, Dec., 2022; online available: https://portal.3gpp.org/desktopmodules/Specifications/SpecificationDetails.aspx?specificationId=3987

RP-230566, “Revised WID: Network energy savings for NR”, RAN#99