3GPP TSG RAN WG1 #113	R1-2305683

Incheon, Korea, May 22nd – May 26th, 2023

Agenda item:		9.1.2

Source:		Mavenir

Title:	Discussion on CSI enhancement

Document for:		Discussion and Decision

# 1	Introduction

Agreement

For the Rel-18 TRS-based TDCP reporting, regarding the quantization of wideband normalized amplitude value,

- At least the following size-Q quantization alphabet is supported:  where

  - TBD: supported value(s) of N (e.g.  or a larger value), Q, s (e.g. ½, ¼, 1/8, …), whether a center threshold is also supported (and if so, higher-layer configured)

- FFS: Whether different schemes can be supported for different use cases

Agreement

For the Rel-18 TRS-based TDCP reporting, for TDCP measurement and calculation, confirm the following working assumption as an agreement with the following change

- KTRS ≥1 TRS resource set(s) can be configured in the CSI reporting setting when ReportQuantity is ‘tdcp’

  - Note: the TRS resource set(s) configured for TDCP report do not impact or impose any new requirements on the UE behavior when processing TRS used as QCL type A/D source for reception of PDxCH.

- No further spec enhancement on TRS is supported

- [All the TRS resources in the configured resource set(s) share the same RE locations]

- FFS: Whether to add further restrictions on the TRS resource set(s) on, e.g. QCL relationship, power control, [RE location], slot offset between TRS resource set(s), relation with resource set used for legacy usage

Agreement

For the Rel-18 TRS-based TDCP reporting,

- Support the following D (delay) values: 4 symbols, 1 slot, 2 slots, 3 slots, 4 slots, 5 slots

- Working assumption: Support the following D (delay) values in a separate UE Feature Group: 6 slots, 10 slots

FFS: The value of Dbasic

FFS: Applicability of each D value candidate for different SCS values and/or other parameters (e.g. Y, quantization)

Agreement

For the Rel-18 TRS-based TDCP reporting, regarding the quantization of wideband normalized amplitude value, down-select (by RAN1#113) from the following candidates:

- Alt1: N=2Q-1 where Q=5, s={1/5, ¼, 1/3}

- Alt2: N=2Q where Q=3, s={¼, 1/3, ½, 2/3, ¾}

- Alt3: N=2Q where Q=4, s={¼, ½, 2/3, ¾}

- Alt4: N={2Q –1, …, 2Q+1 –1} (i.e., 7-15) where Q=3, s={1/5, ¼, 1/3, 2/5, ½, 3/5, 2/3, ¾, 4/5}

- Alt4A: N={2Q , 2Q+0.5,…, 2Q+1-0.5} (i.e., 8, 8.5,…,15.5) where Q=3, s={1/5, ¼, 1/3, 2/5, ½, 3/5, 2/3, ¾, 4/5}

Once an alternative is selected, reducing the number of candidate values for s is not precluded.

Companies can simulate each alternative with and without a configurable center threshold

The following agreements about TRS-based TDCP report was made during RAN1 #112bis-e [1].

Agreement

For the Rel-18 TRS-based TDCP reporting, regarding the value of parameter Y, in addition to Y=1, support Y=2, 3, 4

- FFS: Whether Y=7 is also supported

Agreement

For the Rel-18 TRS-based TDCP reporting, regarding phase quantization, down-select (by RAN1#113) from the following candidates:

- Alt1. 1-bit (early vs. late) phase indicator

- Alt2. 3-bit (8-PSK) uniform quantization

- Alt3. 4-bit (16-PSK) uniform quantization (full reuse of Rel-16 eType-II W2 phase quantization)

- Alt4. Adaptive/gNB-configurable phase quantizer e.g. , where

  - : legacy (Rel.16) based

    - Linear: legacy -PSK

    - Exponential: legacy Rel.16 amplitude,  or

  - a slope value from  depending on the amplitude ) of the 1st correlation (smallest delay), e.g. the slope decreases towards 0 as  increases towards 1

- Alt5. A given correlation phase value  is quantized to  based on the following alphabet (where  denotes delay):

- Alt6. A given correlation phase value  is quantized to  based on the following alphabet (where  denotes delay and p(.) denotes amplitude quantization values used for Rel-16 e-TypeII codebook and ):

  - Mode 1: ,

  - Mode 2:

  - The quantization mode is selected by UE and reported to gNB.

- Alt7. A given correlation phase value  is quantized to  based on the following alphabet: , with , . TBD value(s) of

The evaluation should consider the impact of delay tracking operation at the UE where the phase difference between two slots can be close to zero.

Note: This proposal doesn’t preclude the UE supporting only smaller delay values (e.g. 4-symbol only) for the phase report (which is already optional)

In this contribution, we provide some views and simulation results about TRS-based TDCP reporting.

# TRS-based TDCP reporting

## Quantization of amplitude

There are first 3 candidates by RAN#112bis-e the quantized values of wideband normalized amplitude as below.

- Alt1: N=2Q-1 where Q=5, s={1/5, ¼, 1/3}

- Alt2: N=2Q where Q=3, s={¼, 1/3, ½, 2/3, ¾}

- Alt3: N=2Q where Q=4, s={¼, ½, 2/3, ¾}

Where Q is number of bits for the quantization of amplitude which can define the range of quantized values, and s can tune granularity.

To evaluate the 3 candidates, we calculated and plot all values in the following figures.

For each value of parameter s, Alt 1 contains the value of 0, and covers most of values from 0 to 1. Alt 2&3 can only uniformly cover part values, which is limited to suitable specific use case.

Figure-1. Quantization of normalized amplitude.

Table-1. The maximum and minimum values of quantization for each s in these 3 alternatives.

| Alt 1 | Max. | Min. (expect 0) |
| --- | --- | --- |
| s = 1/5 | 0.9318 | 0.0830 |
| s = 1/4 | 0.9954 | 0.1591 |
| s = 1/3 | 0.9992 | 0.2063 |

| Alt 2 | Max. | Min. |
| --- | --- | --- |
| s = 1/4 | 0.7500 | 0.1591 |
| s = 1/3 | 0.8425 | 0.2063 |
| s = 1/2 | 0.9375 | 0.2929 |
| s = 2/3 | 0.9752 | 0.3700 |
| s = 3/4 | 0.9844 | 0.4054 |

| Alt 3 | Max. | Min. |
| --- | --- | --- |
| s = 1/4 | 0.9375 | 0.1591 |
| s = 1/2 | 0.9752 | 0.2063 |
| s = 2/3 | 0.9994 | 0.3700 |
| s = 3/4 | 0.9998 | 0.4054 |

Observation 1. Alt 1 can cover the values of [0 1] in each value of s parameter, in special of values closed to 1.

Observation 2. For Alt 2 & 3, s={1/4, 1/3, 1/2} cover most of values, but s={2/3, ¾} would lose the amplitude smaller than 0.3.

For use cases of R16 Type I & Type II codebook switching and SRS periodicity, threshold should be larger than 0.9. Therefore, it is better to chose fine resolution in [0.9, 1].

For other use cases, other values of amplitude (i.e. around 0.7, 0.5) might be reserved in advance.

Alt 1 guarantees the precision in [0.9 1] and offers more values out of this area, even it is needed 1 more bit than Alt 2&3.

Proposal 1. Quantization of amplitude: support Alt 1, N=2Q-1 where Q=5, s={1/3}.

## Quantization of phase

Due to TRS-based frequency and time synchronization every slot, reporting phase would be not uniform and close to 0.

- Alt 5. A given correlation phase value  is quantized to  based on the following alphabet (where  denotes delay):

For Alt 5, we have quantized phase as below.

Figure-2. Quantization of phase.

Alt 5 is nonlinear quantization which is more suitable TDCP-reporting phase with CFO compensation.

Proposal 2. Quantization of phase:  support Alt 5, where Q=3.

## TDCP parameters

According to RAN1#112 [1], normalized time correlation has been agreed for TDCP reporting. Corresponding with various delays, the normalized time correlation values in specific UE velocities are following in Table-1.

Table-2. The values of normalized time correlation corresponding with various delays in specific UE velocities.

Other relevant parameters are complied with Section 5.1.

| UE velocity | Max. Doppler Shift | Normalized time correlation Tac |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Delay = 4 symbols | Delay = 1 slot | Delay = 5 slots | Delay = 20 slots |
| 3 km/h | 9.72 Hz | 1.0000 | 0.9992 | 0.9942 | 0.9229 |
| 30 km/h | 97.22 Hz | 0.9976 | 0.9711 | 0.5189 | 0.2198 |
| 120 km/h | 388.89 Hz | 0.9657 | 0.6873 | 0.1715 | -0.0144 |

Observation 3.  For given UE velocity, different delays result in different time correlations.

There are several results of previous simulations in [3]. Some latest results have been plotted in the below.

Updated results of estimated Doppler compared with different velocities are seen as below.

Figure-2. SNR 0dB, UE velocity vs estimated Doppler spread by TD correlation over 100ns delay spread TDL-A.

Figure-3. SNR 30dB, UE velocity vs estimated Doppler spread by TD correlation over 100ns delay spread TDL-A.

Above figures has been illustrated that different lags between TRS pairs would take distinct performance with changing UE velocity, particularly in SNR0dB.

Observation 4. 20-slot delay has shown worse accuracy. Delay <= 5 slots can ensure the estimation for time variation of channel. 5-slot delay is better for smaller UE velocity (<=30km/h), whereas 1-slot delay is suitable in scenario of higher velocity.

For basic feature Y=1 with delay≤ Dbasic symbols, simulation results are shown that the compromised choice of Dbasic is 5 slots, then the estimated accuracy can be assured in various velocities.  For example, lower than roughly 30km/h, the choice in this scenario is 5-slot delay.

Proposal 3. For basic feature Y=1 with delay≤ Dbasic symbols, Dbasic is 5 slots.

After TDCP-reporting time correlation, SRS prediction based TDCP reporting can be considered. For anti-aging, SRS prediction from channel correlation when UE velocity is high.  For this use case, more delays can offer more information.

Proposal 4. For Y>1, D value is 10 slots, Y = 7 can be supported in special use case.

# Conclusion

In this contribution, we discussed TRS-based TDCP reporting. The following proposals have been made:

Proposal 1. Quantization of amplitude: support Alt 1, N=2Q-1 where Q=5, s={1/3}.

Proposal 2. Quantization of phase:  support Alt 5, where Q=3.

Proposal 3. For basic feature Y=1 with delay≤ Dbasic symbols, Dbasic is 5 slots.

Proposal 4. For Y>1, D value is 10 slots, Y = 7 can be supported in special use case.

The following observations have been made:

Observation 1. Alt 1 can cover the values from 0 to 1 in each value of s parameter, in special of values closed to 1.

Observation 2. For Alt 2 & 3, s={1/4, 1/3, 1/2} cover most of values, but s={2/3, ¾} would lose the amplitude smaller than 0.3. Observation 1.  For given UE velocity, different delays result in different time correlations.

Observation 3.  For given UE velocity, different delays result in different time correlations.

Observation 4. 20-slot delay has shown worse accuracy. Delay <= 5 slots can ensure the estimation for time variation of channel. 5-slot delay is better for smaller UE velocity (<=30km/h), whereas 1-slot delay is suitable in scenario of higher velocity.

# Reference

| [1] | “Chairman’s Notes 3GPP RAN1 #112,” 27th February – 3rd March 2023. |
| --- | --- |
| [2] | 3GPP, TR38.214 Physical layer procedures for data. |
| [3] | R1-2300900, “Discussion on CSI enhancement”. |
| [4] | 3GPP, TR38.901 Study on channel model for frequencies from 0.5 to 100 GHz. |

# Appendix

## Link level assumptions

### TRS measurement

There is link level assumption for TRS measurement as Table-2. Based on our understanding, we have chosen parts of parameter for channel model. For delay spread 10, 100, 1000ns are corrsponding to very short, nominal, very long delay spread according to [4]. For UE velocity, we assum 3km/h, 30km/h, 120km/h are low, medium, high speed.

Table-3. Link level assumption for TRS based doppler accuracy

| Parameters | Values |
| --- | --- |
| Carrier frequency and subcarrier spacing | 3.5 GHz with 30 kHz SCS |
| System bandwidth | 20 MHz |
| TRS bandwidth | 20 MHz |
| Channel model | TDL-A |
| Delay spread | 10, 100, 1000ns |
| UE velocity | 3 km/h, 30 km/h, 120 km/h |
| Antenna at UE | 1 |
| Antenna at gNB | 1 |
| Evaluation metrics for measurement accuracies | RMS error, Standard deviation (STD) |