3GPP TSG RAN WG1 #113	R1- 2305688

Incheon, Korea, May 22nd – May 26th, 2023

Agenda item:		9.2.2.1

Source:		Mavenir

Title:	Evaluation on AI/ML for CSI feedback enhancement

Document for:		Discussion and Decision

# 1  Introduction

According to discussion in 3GPP RAN1#112bis-e meeting, some progress has been made on evaluation methodology for CSI feedback enhancement. The following agreements and conclusions were extracted from the Chair’s note [1]:

Working Assumption

For the per layer CSI payload size X/Y/Z in the templates of CSI compression, as a clarification, the X/Y/Z ranges in the working assumption achieved in RAN1#112 meeting is applicable to Max rank = 1/2. For Max rank (v) = 3/4, the per layer basis X/Y/Z ranges are re-determined as:

- X is <=160/vbits

- Y is 200/vbits-280/vbits

- Z is >=460/vbits

Agreement

To evaluate the performance of the intermediate KPI based monitoring mechanism for CSI compression, for Step2 of the model monitoring methodology, the per sample KPIDiff is considered for

- Case 1: NW side monitoring of intermediate KPI, where the monitoring accuracy is evaluated for a given ground-truth CSI format (e.g., quantized ground-truth CSI with 8 bits scalar, R16 eType II-like method, etc.) or SRS measurements, where

  - KPIActual is calculated with the output CSI at the NW side and the given ground-truth CSI format or SRS measurements.

  - KPIGenie is calculated with output CSI (as for KPIActual) and the ground-truth CSI of Float32.

  - Note: if Float32 is used for KPIActual, the monitoring accuracy is 100% if KPIActual and KPIGenie are based on the same CSI sample.

- Case 2: UE side monitoring of intermediate KPI with a proxy model, where the monitoring accuracy is evaluated for the output of the proxy model at UE:

  - Case 2-1: the proxy model is a proxy CSI reconstruction part, and KPIActual is calculated based on the inference output of the proxy CSI reconstruction part at UE and the ground-truth CSI.

    - Note: if the proxy CSI reconstruction model is the same as the actual CSI reconstruction model at the NW, the monitoring accuracy is 100%

  - Case 2-2: the proxy model directly outputs intermediate KPI (KPIActual)

  - KPIGenie is calculated with the output CSI at the NW side and the same ground-truth CSI.

  - FFS how to train the proxy model and the resulting monitoring performance, to be reported by companies.

  - FFS whether/how to evaluate the generalization performance of the proxy model.

- Case 3: others are not precluded

In this contribution, we provide our views on further details for evaluation methodology and share some initial evaluation results.

# 2  CSI compression

## 2.1 AI/ML Model Description

An autoencoder architecture is applied to compress and reconstruct CSI matrix. There are encoder and quantization modules at the UE side, de-quantization and decoder modules at the BS side. The encoder takes the original eigenvectors V as the input and compresses it into a codeword. Then, the codeword is sent to quantization module and fed back to the BS as a bit stream. Dequantization module recovers the codeword from the received bit stream. After that, the BS reconstructs original eigenvectors from codeword.

Figure 1 The structure of AI/ML based CSI compression

Specifically, the input of the encoder includes eigenvectors for N subbands, which are formulated as V=V1…VN, where Vn denotes the eigenvector for the n-th subband. The codewords can be formulated as VEn=fencoder(V), where fencoder(∙) represents the function of the encoder. The quantizer at the UE side maps the floating-point vector to a quantized bit stream for CSI feedback. The quantization module can be formulated as VQ=fquantizer(VEn), where fquantizer(∙) represents the function of the quantizer. In our simulation, scalar quantization is used. The de-quantizer recovers the compressed CSI from the feedback CSI bit stream and sends it as the input to the decoder. The de-quantized CSI can be formulated as VDe=fdequantizer(VQ) where fdequantizer(∙)represents the function of the de-quantizer. The decoder recovers the eigenvectors. The recovered eigenvectors can be formulated as V=fdecoder(VDe), where fdecoder(∙) represents the function of the decoder.

## 2.2 Evaluation results for generalization performance

The main focuses is to study the generalization impact of different scenario(CDL_A/CDL_B).  The simulation assumption show in Table2.

Case 1:  the training data set is generated for CDL_A channel model with 30k samples, and testing with the same  CDL_A channel model. The SGCS and GCS performance is shown in Fig. 2.

Case 2:  the training data set is generated for CDL_B channel model with 30k samples, and testing with the same  CDL_B channel model. The SGCS and GCS performance is shown in Fig. 2.

Case 3:  the training data set is generated for mix  CDL_A and CDL_B channel model with 30k samples, and testing with the  CDL_A channel model. The SGCS and GCS performance is shown in Fig. 2.

Case 4:  the training data set is generated for mix  CDL_A and CDL_B channel model with 30k samples, and testing with the same  CDL_B channel model. The SGCS and GCS performance is shown in Fig. 2.

Case 5:  the training data set is generated for CDL_A channel model with 30k samples, and testing with the same  CDL_B channel model. The SGCS and GCS performance is shown in Fig. 2.

Case 6:  the training data set is generated for CDL_B channel model with 30k samples, and testing with the same  CDL_A channel model. The SGCS and GCS performance is shown in Fig. 2.

Figure 2 Evaluation results for CSI compression with model generalization

Observation 1: For different scenarios, the SGCS degradation is about 1%~2% when training set and testing set are mismatching.

Observation 2: Training on mixing dataset with CDL_A and CDL_B can improve the generalization performance (about 1.4%/0.6%) of AI/ML model for CDL_A and CDL_B.

Proposal1: Mixed dataset should be considered as a effective way to increase AI model generation.

Proposal2: AI model performance should be evaluated on datasets with different scenario and configuration.

## 2.3 Evaluation results for scalability performance

The input dimension of AI model is corresponding to the input precoder matrices, i.e., the subband number and port number for each single layer. Different frequency granularity or different ports number can cause different input dimension of AI model. The AI models for different input dimensions need to be trained independently, which may lead to difficulty in generalizing AI models.

Figure3 shows Diagram of scalability over different antenna port numbers by truncation/padding. we choose 32ports/16ports as the input of AI model. To achieve the scalability in this evaluation, we use truncation/ padding to keep the same input size of AI model. The evaluation results are shown in Table 1.

Figure 3 Diagram of scalability over different antenna port numbers by truncation/padding

| Train | Test | SGCS |
| --- | --- | --- |
| 32ports/25k | 32ports/5k | 0.896 |
| 16ports/25k | 16ports/5k | 0.927 |
| 32ports+16ports/50k | 32ports/5k | 0.875(-2.4%) |
| 32ports+16ports/50k | 32ports/5k | 0.912(-1.6%) |

Observation 3: For scalability of AI/ML-based CSI compression over 32/16 antenna port numbers, the SGCS margin between the port-specific models and the unified scalable model supporting 32/16 antenna port numbers is less than 2.4%.

Proposal3: Padding/truncation is used to achieve the scalability.

# 3 Conclusions

In this contribution, we discuss the evaluations on AI/ML for CSI feedback enhancement and provide preliminary simulation results. We have the following observations and proposals.

Observation 1: For different scenarios, the SGCS degradation is about 1%~2% when training set and testing set are mismatching.

Observation 2: Training on mixing dataset with CDL_A and CDL_B can improve the generalization performance (about 1.4%/0.6%) of AI/ML model for CDL_A and CDL_B.

Observation 3: For scalability of AI/ML-based CSI compression over 32/16 antenna port numbers, the SGCS margin between the port-specific models and the unified scalable model supporting 32/16 antenna port numbers is less than 2.4%

Poposal1: Mixed dataset should be considered as a effective way to increase AI model generation.

Proposal2: AI model performance should be evaluated on datasets with different scenario and configuration.

Table 1. Evaluation results for CSI compression with model generalization/scalability, [Max rank value], [Scenario/configuration]

|  |  | Source 1 |  | … |
| --- | --- | --- | --- | --- |
| CSI generation part | AL/ML model backbone | Transformer |  |  |
|  | Pre-processing | / |  |  |
|  | Post-processing | / |  |  |
|  | Number of parameters/M | 8 |  |  |
| CSI reconstruction part | AL/ML model backbone | Transformer |  |  |
|  | [Pre-processing] | / |  |  |
|  | [Post-processing] | / |  |  |
|  | Number of parameters/M | 8 |  |  |
| Common description | Input type | Raw channel matrix |  |  |
|  | Output type | Raw channel matrix |  |  |
|  | Quantization /dequantization method | Scalar quantization |  |  |
|  | Generalization/Scalability method description if applicable, e.g., truncation, adaptation layer, etc. | / |  |  |
|  | Input/output scalability dimension if applicable, e.g., N>=1 NW part model(s) to M>=1 UE part model(s) | / |  |  |
| Dataset description | Ground-truth CSI quantization method | Float32 |  |  |
| [Other assumptions/settings agreed to be reported] |  |  |  |  |
| Generalization Case 1 | Train (setting#A, size/k) | CDL_A#30k |  |  |
|  | Test (setting#A, size/k) | CDL_A#12k |  |  |
| GCS, layer 1 | CSI feedback payload X(64bit) | 0.9759 |  |  |
|  | CSI feedback payload Y(128bit) | 0.9802 |  |  |
|  | CSI feedback payload Z(256bit) | 0.9865 |  |  |
| SGCS, layer 1 | CSI feedback payload X(64bit) | 0.9524 |  |  |
|  | CSI feedback payload Y(128bit) | 0.9608 |  |  |
|  | CSI feedback payload Z(256bit) | 0.9732 |  |  |
| Generalization Case 2 | Train (setting#B, size/k) | CDL_B#30k |  |  |
|  | Test (setting#B, size/k) | CDL_B#12k |  |  |
| GCS, layer 1 | CSI feedback payload X(64bit) | 0.9698 |  |  |
|  | CSI feedback payload Y(128bit) | 0.9784 |  |  |
|  | CSI feedback payload Z(256bit) | 0.9843 |  |  |
| SGCS, layer 1 | CSI feedback payload X(64bit) | 0.9405 |  |  |
|  | CSI feedback payload Y(128bit) | 0.9573 |  |  |
|  | CSI feedback payload Z(256bit) | 0.9678 |  |  |
| Generalization Case 3 | Train (setting#A+#B, size/k) | CDL_A&CDL_B#30k |  |  |
|  | Test (setting#A, size/k) | CDL_A#12k |  |  |
| GCS, layer 1 | CSI feedback payload X(64bit) | 0.9683 |  |  |
|  | CSI feedback payload Y(128bit) | 0.9731 |  |  |
|  | CSI feedback payload Z(256bit) | 0.9802 |  |  |
| SGCS, layer 1 | CSI feedback payload X(64bit) | 0.9376 |  |  |
|  | CSI feedback payload Y(128bit) | 0.9469 |  |  |
|  | CSI feedback payload Z(256bit) | 0.9610 |  |  |
| Generalization Case 4 | Train (setting#A+#B, size/k) |  | CDL_A&CDL_B#30k |  |
|  | Test (setting #B, size/k) |  | CDL_B#12k |  |
| GCS, layer 1 | CSI feedback payload X(64bit) |  | 0.9614 |  |
|  | CSI feedback payload Y(128bit) |  | 0.9716 |  |
|  | CSI feedback payload Z(256bit) |  | 0.9812 |  |
| SGCS, layer 1 | CSI feedback payload X(64bit) |  | 0.9243 |  |
|  | CSI feedback payload Y(128bit) |  | 0.9440 |  |
|  | CSI feedback payload Z(256bit) |  | 0.9628 |  |
| Generalization Case 5 | Train (setting#A, size/k) |  | CDL_A#30k |  |
|  | Test (setting #B, size/k) |  | CDL_B#12k |  |
| GCS, layer 1 | CSI feedback payload X(64bit) |  | 0.9569 |  |
|  | CSI feedback payload Y(128bit) |  | 0.9602 |  |
|  | CSI feedback payload Z(256bit) |  | 0.9783 |  |
| SGCS, layer 1 | CSI feedback payload X(64bit) |  | 0.9157 |  |
|  | CSI feedback payload Y(128bit) |  | 0.9220 |  |
|  | CSI feedback payload Z(256bit) |  | 0.9571 |  |
| Generalization Case 6 | Train (setting#B, size/k) |  | CDL_B#30k |  |
|  | Test (setting #A, size/k) |  | CDL_A#12k |  |
| GCS, layer 1 | CSI feedback payload X(64bit) |  | 0.9432 |  |
|  | CSI feedback payload Y(128bit) |  | 0.9567 |  |
|  | CSI feedback payload Z(256bit) |  | 0.9734 |  |
| SGCS, layer 1 | CSI feedback payload X(64bit) |  | 0.8896 |  |
|  | CSI feedback payload Y(128bit) |  | 0.9153 |  |
|  | CSI feedback payload Z(256bit) |  | 0.9475 |  |

# 4. References

- Chairman’s notes, RAN1 #112b-e.

# Appendix

Table 1 Simulation Assumption

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD (TDD is not precluded), OFDM |
| Carrier frequency | 2GHz as baseline, optional for 4GHz |
| Bandwidth | 10MHz or 20MHz |
| Subcarrier spacing | 15kHz |
| Nt | 32 |
| Nr | 4 |
| Channel model | CDL-A, CDL-B |
| Delay spread | 300ns |