3GPP TSG RAN WG1 #113	R1- 2305722

Incheon (KR), 22nd – 26th May, 2023

Agenda item:		9.16.7

Source:	Nokia, Nokia Shanghai Bell

Title:	Initial views on UE features for NR Sidelink Evolution WI

Document for:		Discussion and Decision

# Introduction

RAN1 started discussion Rel-18 UE features for selected topics in RAN1#112bis-e [1]. In RAN1#113 an extended set of topics will be included, where rapporteur’s input on UE features for each WI will be presented for discussion as well. Hence, in this contribution we provide our initial views on UE features for NR Sidelink Evolution WI [2].

# Discussion

Based on the agreements reached so far under the NR Sidelink Evolution WI, a skeleton of UE FGs structure is proposed in Table 1 below. There is no direct dependence between sidelink unlicensed and NR-U FGs defined in earlier releases, and hence any pre-requisites related to those features need to be evaluated case by case and not assumed by default. FR2 related objectives are for study only in Rel-18, and hence no UE FGs are expected due to those. For carrier aggregation the work is yet to start pending conclusion on coexistence work, as per RAN#99 decision, and hence no UE FGs should be defined for those at this point of time, but those can be added later once the feature itself becomes more clear.

Table 1: Set of new Rel-18 UE capabilities for NR Sidelink Evolution

| FG | FG name | Components | Note |
| --- | --- | --- | --- |
| 47-1 | Receiving NR sidelink in unlicensed spectrum |  | Potentially one can reuse existing Rel-16 and Rel-17 FGs for this purpose |
| 47-2 | Transmitting NR sidelink mode 1 on unlicensed spectrum scheduled by NR Uu | - Type 1 and Type 2 (2A/2B/2C) channel access procedures - COT sharing - FFS others | FFS if existing Rel-16 and Rel-17 FGs can be used as pre-requisites or if corresponding functionalities need to be copied to new FG |
| 47-3 | Transmitting NR sidelink mode 2 on unlicensed spectrum | - Type 1 and Type 2 (2A/2B/2C) channel access procedures - COT sharing - FFS others | FFS if existing Rel-16 and Rel-17 FGs can be used as pre-requisites or if corresponding functionalities need to be copied to new FG |
| 47-4 | Synchronization sources for NR sidelink on unlicensed spectrum |  | Potentially one can reuse existing Rel-16 and Rel-17 FGs for this purpose |
| 47-5 | Dynamic resource pool sharing for coexistence between NR sidelink and LTE sidelink transmissions | - Determination of candidate resource set for NR SL considering the LTE SL reserved resources by other LTE SL UE - Determine candidate resource set for NR SL considering the LTE SL resources selected to be used for LTE SL module’s own LTE SL transmission - FFS others | Decisions still pending |

Proposal: Consider table 1 as starting point for discussions on NR Sidelink Evolution WI.

# Conclusion

In this contribution we presented our initial views on UE features for NR Sidelink Evolution, and we have made the following observations and proposals:

Proposal: Consider table 1 as starting point for discussions on NR Sidelink Evolution WI.

# References

[1]  R1-2304282, “Initial RAN1 UE features list for Rel-18 NR after RAN1#112bis-e,” Moderators (AT&T, NTT DOCOMO, INC.), Apr. 2023

[2]  RP-230077, “WID revision: NR sidelink evolution,” OPPO, Mar. 2023