3GPP TSG RAN WG1 #113	R1-2305725

Incheon (KR), 22nd – 26th May, 2023

Agenda item:		9.16.11

Source:	Nokia, Nokia Shanghai Bell

Title:	Initial views on UE features for IoT NTN enhancements

Document for:		Discussion and Decision

# Introduction

RAN1 started discussion Rel-18 UE features for selected topics in RAN1#112bis-e [1]. In RAN1#113 an extended set of topics will be included, where rapporteur’s input on UE features for each WI will be presented for discussion as well. Hence, in this contribution we provide our initial views on UE features for IoT NTN enhancements WI [2].

# Discussion

The following new FGs can be identified for IoT-NTN

- NB-IoT NTN and eMTC NTN UEs

  - Disabling HARQ feedback

  - GNSS assistance information reporting as GNSS position fix time duration

  - GNSS validity duration reporting MAC-CE (in connected mode)

  - eNB-triggered GNSS measurements with GNSS measurement gaps

Baseline FG for HARQ-ACK disabling for NB-IoT and eMTC NTN UEs should be

| HARQ-ACK disabling for NB-IoT NTN | - RRC-based enabling and disabling - Direct DCI based enabling and disabling when only DCI-based enabling/disabling is configured - Direct DCI based enabling/disabling overriding of RRC based enabling/disabling in case both are configured |
| --- | --- |
| HARQ-ACK disabling for eMTC NTN | - For CE Mode A and B: RRC-based enabling and disabling - For CE Mode B: Direct DCI based enabling and disabling when only DCI-based enabling/disabling is configured - For CE Mode B: Direct DCI based enabling/disabling overriding of RRC based enabling/disabling in case both are configured |

Baseline FG for GNSS assistance for NB-IoT and eMTC NTN UE should be

| GNSS assistance for NB-IoT NTN | - GNSS assistance information reporting as GNSS position fix time duration - GNSS validity duration reporting MAC-CE (in connected mode) - eNB-triggered GNSS measurements with GNSS measurement gaps |
| --- | --- |
| GNSS assistance for eMTC NTN | - GNSS assistance information reporting as GNSS position fix time duration - GNSS validity duration reporting MAC-CE (in connected mode) - eNB-triggered GNSS measurements with GNSS measurement gaps |

# Conclusion

In this contribution the following structure is proposed for the IoT NTN work item’s UE capabilities:

| HARQ-ACK disabling for NB-IoT NTN | - RRC-based enabling and disabling - Direct DCI based enabling and disabling when only DCI-based enabling/disabling is configured - Direct DCI based enabling/disabling overriding of RRC based enabling/disabling in case both are configured |
| --- | --- |
| HARQ-ACK disabling for eMTC NTN | - For CE Mode A and B: RRC-based enabling and disabling - For CE Mode B: Direct DCI based enabling and disabling when only DCI-based enabling/disabling is configured - For CE Mode B: Direct DCI based enabling/disabling overriding of RRC based enabling/disabling in case both are configured |
| GNSS assistance for NB-IoT NTN | - GNSS assistance information reporting as GNSS position fix time duration - GNSS validity duration reporting MAC-CE (in connected mode) - eNB-triggered GNSS measurements with GNSS measurement gaps |
| GNSS assistance for eMTC NTN | - GNSS assistance information reporting as GNSS position fix time duration - GNSS validity duration reporting MAC-CE (in connected mode) - eNB-triggered GNSS measurements with GNSS measurement gaps |

# References

- R1-2304282, “Initial RAN1 UE features list for Rel-18 NR after RAN1#112bis-e,” Moderators (AT&T, NTT DOCOMO, INC.), Apr. 2023

- RP-223519, “Revised WID on IoT NTN enhancements”, MediaTek Inc.