3GPP TSG RAN WG1 #113	R1-2305861

Incheon, Korea, 22nd – 26th May, 2023

Title:	Reply LS on Rel-18 Tx switching across 3/4 bands

Response to:	R4-2306623/R1-2304313

Release:		Release 18

Work Item:	NR_MC_enh-Core

Source:	Nokia, Nokia Shanghai Bell

To:	RAN4

CC:	RAN2

Contact person:	[TBD]

Attachments:	none

# Introduction

RAN1 would like to thank RAN4 for their LS on Rel-18 Tx Switching across 3 or 4 bands.

Considering the questions RAN4 raises in the LS, RAN1 would like to provide the following comments:

RAN4 would like to ask RAN1 one question:

In sub-clause 6.1.6 of TS 38.214, it is specified that:

If an uplink switching is triggered for an uplink transmission starting at T0, after T0-Toffset, the UE is not expected to cancel the uplink switching, or to trigger any other new uplink switching occurring before T0 for any other uplink transmission that is scheduled after T0-Toffset, where Toffset is the UE processing procedure time defined for the uplink transmission triggering the switch given in clause 5.3, clause 5.4, clause 6.2.1, clause 6.4 and in clause 9 of [6, TS 38.213].

On the definition and understanding of T0:

- When gNB provides sufficient time between the end of the UL transmission on the switch-from carrier and the start of the UL transmission on the switch-to carrier, in RAN4 understanding, the T0 is the starting time for an uplink transmission on the switched-to carrier from network scheduling perspective and the starting time of actual uplink transmission from UE perspective.

RAN1 comment: This is correct. In this case, the UE has no need to cut or drop any intended transmissions and thus the transmission start from the secheduling perspective and the actual start of the transmission occur at the same time instant T0.

- When gNB does not provide sufficient time between the end of the UL transmission on the switch-from carrier and the start of the UL transmission on the switch-to carrier, and the switching period is located at the switch-to carrier according to the RRC signalling uplinkTxSwitchingPeriodLocation, UE may omit uplink transmission on certain symbol(s) on the switch-to carrier. In this case, is T0 the starting time of uplink transmission from network scheduling perspective, or the starting time of actual uplink transmission from UE perspective?

RAN1 comment: In this case the time instant T0 is the scheduler’s intended transmission starting point, while the actual transmission may start only after the time instant T0 due to the UE omitting the beginning of the scheduler’s intended transmission.

# 2 Actions

To 3GPP RAN4

ACTION: RAN1 respectfully asks RAN4 to take the provided feedback into account in their further work.

# 3	Dates of next TSG RAN WG1 meetings

TSG-RAN WG1 Meeting #114		21 – 25 August, 2023			Toulouse, FR

TSG-RAN WG1 Meeting #114-bis	9 – 13 October, 2023			Xiamen, CN

TSG-RAN WG1 Meeting #115		13 – 17 November, 2023		Chicago, US