3GPP TSG RAN WG1 #113	R1-2305862

Incheon, Korea, 22nd – 26th May, 2023

Title:	Reply LS on report of switching periods in Rel-18 UL Tx switching

Response to:	R2-2304567/R1-2304333

Release:		Release 18

Work Item:	NR_MC_enh-Core

Source:	Nokia, Nokia Shanghai Bell

To:	RAN2

CC:	RAN4

Contact person:	[TBD]

Attachments:	none

# Introduction

RAN1 would like to thank RAN2 for their LS on report of switching periods in Rel-18 UL Tx switching.

On Question 1 (the 1st question addressed to RAN1):

Agreement: On the length of switching period: - For UL switching period with Tx switching across 3 or 4 bands, RAN4 agreed to reuse the same set of values as in Rel-16/17, i.e., {35 us, 140 us, 210 us} for UL CA and SUL. - The length of switching period is applied per band pair for each band combination. - For each band pair, the switching period can be the same or different for 1Tx-2Tx switching and 2Tx-2Tx switching based on UE reporting, which is similar as in Rel-17. - Note: For UE reporting different periods for 1Tx-2Tx switching and 2Tx-2Tx switching for a band pair, similar to Rel-17, it is RAN4 understanding that the 2Tx-2Tx switching period is applied when 2Tx-2Tx switching mode is configured.

RAN2 could not achieve conclusion, but has agreed an intention below in RAN2#121bis-e:

In support of RAN4 agreement, RAN2 intend to introduce support for two per-band-pair UE capabilities, a length of a switching period, for 1Tx-2Tx switching (like Rel-16) and that for 2Tx-2Tx switching (like Rel-17).

Question 1. (To RAN1 and RAN4)

RAN2 respectfully asks RAN1 and RAN4 to take above agreement on RAN2 intention into account and asks for feedback if there is any issue.

RAN1 response to question 1: RAN1 sees no issue with the above agreement or the agreed intention, and they are in-line with the work done by RAN1

On Question 4 (the 2nd question addressed to RAN1):

RAN2 respectfully asks RAN4 which of the options below matches RAN4 understanding on the selection of applied switching periods when both switching periods of 2Tx-2Tx switching and 1Tx-2Tx switching can be reported for the same band pair.

Option 1: Based on implicit rules, e.g. 2Tx-2Tx switching period is only applicable when performing UL switching between two bands (e.g. 2P+0P<=>0P+2P) and 1Tx-2Tx period is applied for the other switching cases (e.g. UL Tx switching that involves 3 or 4 bands, such as band A + band B<=>band C, band A+ band B <=>band C + band D). FFS on the switching case of 2P+0P<=>1P+1P.

Option 2: Based on explicit RRC configuration, i.e., gNB configures which period is applied. FFS on the granularity of the configuration.

Question 4. (To RAN1)

RAN2 respectfully asks RAN1 to take above discussion on RAN2 and question to RAN4 into account and asks for feedback if there is any issue.

RAN1 response to question 4: The current draft CR for 38.214 is not changing the Rel-17 logic when two bands are involved in the switch, even if there are more bands in the UL Tx Switching configuration. For switching cases with more than 2 bands involved in the switch, the draft CR is defining the rules on which switching period to apply for a given switching case, i.e. option 1 is in-line with the current RAN1 assumption.

# 2 Actions

To 3GPP RAN2

ACTION: RAN1 respectfully asks RAN2 to take the provided feedback into account in their further work.

# 3	Dates of next TSG RAN WG1 meetings

TSG-RAN WG1 Meeting #114		21 – 25 August, 2023			Toulouse, FR

TSG-RAN WG1 Meeting #114-bis	9 – 13 October, 2023			Xiamen, CN

TSG-RAN WG1 Meeting #115		13 – 17 November, 2023		Chicago, US