3GPP TSG RAN WG1 #113			                                 R1-2306130

Incheon, Korea, May 22nd – May 26th, 2023

Source:	Moderator (NTT DOCOMO)

Title:	FL summary on DMRS#3

Agenda item:	9.1.3.1

Document for:	Discussion and Decision

- Introduction

In RAN#94-e meeting, a new Rel-18 WID on MIMO [1] was agreed. From 7 objectives, there are two objectives for DMRS enhancements, as shown below.

- Study, and if justified, specify larger number of orthogonal DMRS ports for downlink and uplink MU-MIMO (without increasing the DM-RS overhead), only for CP-OFDM, - Striving for a common design between DL and UL DMRS - Up to 24 orthogonal DM-RS ports, where for each applicable DMRS type, the maximum number of orthogonal ports is doubled for both single- and double-symbol DMRS […] - Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

This document contains summary of the company’s tdocs and FL proposals.

In this meeting, discussion for potential RRC/MAC CE impact is prioritized.

- Sect. 2.1.2 (1CW)/2.1.4(1CW)/2.2: DCI size of antenna ports field (potential RRC impact).

- Sect. 2.4: MU-restriction b/w R15 and R18 (potential RRC impact, if Alt.2 in P2.4A is agreed).

- Sect. 2.6: eType1 DMRS for Msg.A PUSCH(potential RRC impact, if agreed)..

- Sect. 3.4: Max number of PTRS ports (potential RRC impact, if 4port PTRS is agreed).

- Sect. 3.7: RRC parameter of timeDensity of PTRS (potential RRC impact, if Alt.2 is agreed).

- Objective #3 (increasing DMRS ports)

  - Antenna ports table for PDSCH

RRC based switching of antenna ports table

As dynamic switching between separate Rel-18 DMRS ports with FD-OCC4 and legacy DMRS ports with FD-OCC2 is not supported, there is no need to indicate legacy DMRS port and R18 DMRS port in the same table by DCI. Therefore, one simple way is to specify new DMRS indication tables individually for Rel-18 DMRS. RRC signaling can be used for switching between separate DMRS indication tables for Rel-18 DMRS ports with FD-OCC4 and legacy DMRS ports with FD-OCC2.

FL Proposal 2.1

- For the antenna ports indication of PDSCH/PUSCH, specify new DMRS port tables for Rel.18 eType1/eType2 DMRS ports with maxLength = 1/2, respectively.

  - RRC signaling is used for semi-static switching between separate DMRS port tables for Rel.18 DMRS ports with FD-OCC4 and legacy DMRS ports with FD-OCC2.

Support/fine: Docomo, OPPO, HW/Hi, Google, Samsung, ZTE, Lenovo, New H3C, InterDigital, Futurewei, CMCC

Ericsson, vivo, Nokia/NSB, Fraunhofer IIS/HHI, CATT, LGE, MTK, Sharp, Spreadtrum

No:

| Company | Comment |
| --- | --- |
| Docomo | Support. |
| OPPO | Support. |
| Huawei, HiSilicon | Generally fine with the proposal. In terms of the Note, if I remember correctly, this is already discussed and most companies seem to support separate table? FL: For the note, we discussed it in RAN1#112 online, but Nokia/NSB (if I remember correctly) commented it can be left to editor. |
| Google | OK. |
| Samsung | We are generally fine. Regarding RRC signaling in 1st bullet, we would like to understand whether the signaling is based on the RRC parameter supporting eType1/eType2 DMRS, or a separate parameter rather than the RRC parameter supporting eType1/eType2 DMRS. FL: For the RRC parameters, one simple approach is to add new 1-bit parameter (enabler) to enable Rel.18 DMRS. Multiple companies proposed it in their tdocs in RAN1#113 and rapporteur proposal in R1-2305494.zip is the same as this approach. For the detail of RRC parameter, let’s discuss separately from the above proposal. Regarding HW’s comment on separate table for SDCI MTRP and STRP, we also have similar understanding (i.e., separate table) because table index is different in the previous agreements (e.g,, Table 7.3.1.2.2-1-X for STRP, Table 7.3.1.2.2-1A-X for SDCI MTRP). But “up to spec editor” is also okay with us. |
| ZTE | Support. |
| Lenovo | Support |
| New H3C | Fine with this proposal in general. But motivation of note is n’t clear to us and from technical perspective, the group need decide whether the DMRS port tables for sDCI mTRP and the DMRS port tables for sTRP are common or separate FL: But, the common or separate table has no difference for UE behavior. It is just how to specify the agreement in the spec. |
| FL | I removed the note. Now I think we don’t need to discuss it explicitly; it is not relevant to the sub-bullet of the proposal. |
| InterDigital | Support |
| Futurewei | Fine with the latest proposal. |
| CMCC | Support |
| Ericsson | Fine with the proposal. |
| vivo | Support |
| Nokia/NSB | Fine with the proposal. |
| Fraunhofer IIS/HHI | Support |
| CATT | Support. |
| LGE | Support |
| MediaTek | Support |
| Sharp | Support |
| Spreadtrum | Support the latest proposal. |

(done) eType1, maxLength1

No remaining issue. The following is agreed DMRS combinations.

Table 7.3.1.2.2-1-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=1

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Notes | Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 1 | 0 | Cat. 1 | 0 | 2 | 0,1,2,3,8 |
| 1 | 1 | 1 |  | 1 | 2 | 0,1,2,3,8,10 |
| 2 | 1 | 0,1 |  | 2 | 2 | 0,1,2,3,8,9,10 |
| 3 | 2 | 0 |  | 3 | 2 | 0,1,2,3,8,9,10,11 |
| 4 | 2 | 1 |  | [4] | [2] | [0,1,2,3,10] |
| 5 | 2 | 2 |  | [5] | [2] | [0,1,8,2,3,10] |
| 6 | 2 | 3 |  | [6] | [2] | [0,1,8,2,3,10,11] |
| 7 | 2 | 0,1 |  | [7] | [2] | [0,1,8,9,2,3,10,11] |
| 8 | 2 | 2,3 |  | [8] | [2] | [0,2,3,8,9] |
| 9 | 2 | 0-2 |  | [9] | [2] | [0,1,2,3,8,9] |
| 10 | 2 | 0-3 |  |  |  |  |
| 11 | 2 | 0,2 |  |  |  |  |
| 12 | 1 | 8 | Cat.2 |  |  |  |
| 13 | 1 | 9 |  |  |  |  |
| 14 | 1 | 8,9 |  |  |  |  |
| 15 | 2 | 8 |  |  |  |  |
| 16 | 2 | 9 |  |  |  |  |
| 17 | 2 | 10 |  |  |  |  |
| 18 | 2 | 11 |  |  |  |  |
| 19 | 2 | 8,9 |  |  |  |  |
| 20 | 2 | 10,11 |  |  |  |  |
| 21 | [2] | [8-10] |  |  |  |  |
| 22 | [2] | [8-11] |  |  |  |  |
| 23 | [2] | [8, 10], [9, 11] |  |  |  |  |
| 24 | 1 | 0,1,8 | Cat.3 |  |  |  |
| 25 | 1 | 0,1,8,9 |  |  |  |  |
| 26 | 2 | 0,1,8 |  |  |  |  |
| 27 | 2 | 0,1,8,9 |  |  |  |  |
| 28 | 2 | 2,3,10 |  |  |  |  |
| 29 | 2 | 2,3,10,11 |  |  |  |  |

eType1, maxLength2

In the online on Monday, the following was agreed.

FL Proposal 2.1.2 (eType1, maxLength=2) - For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, support/remove the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data in RAN1#112bis-e agreement. - For 1CW, - 1) Support row 8-11 for 1 CW. - For row 9-11 in one CW, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group). - 2) For row 24-30, 55-60, 69-80, down select from the following: - Alt.2-1: Support row 24-30 and row 55-60 without MU restriction. Remove row 69-80. - Alt.2-2: Support row 69-80 without MU restriction. Support row 24-30 with MU restriction. Remove row 55-60. - Alt.2-3: Support row 24-30 with MU restriction. Remove row 55-60 and 69-80. - Alt 2-4: Support row 69-80 without MU restriction. Remove row 24-30, 55-60. - Alt 2-5: Remove row 24-30, 55-60, 69-80 due to no consensus to support them. - Atl 2-6: Support rows 24-30 and rows 59-60 without MU restriction. Remove rows 69-80 and rows 55-58. - 3) For row 81-82: - Alt.3-1: - If RAN1 agree row 26-27 without MU restriction, - Support row 81-82 without MU restriction. - Else, - Remove row 81-82. - Alt.3-2: Remove 81-82. - 4) [Remove row 83.] - For 2CW, - 5 [Remove row 0-3.] - 6) Support row 20-23 if row 4-7 are supported for eType1 maxLength=1. Else, remove row 20-23. - 7) Down select from the following: - Alt.7-1: Support row 8-19 and remove row 24-35. - Alt.7-2: Support row 24-35 and remove row 8-19. - Alt.7-3: Remove row 8-19 and 24-35. - 8) Support row 36-37 if row 8-9 are supported for eType1 maxLength=1. Else, remove row 36-37.

The following is the currently agreed in RAN1#112bis-e and RAN1#113. The remaining rows are highlighted.

Table 7.3.1.2.2-2-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=2

| One Codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | [0 | 2 | 0-4 | 2] |
| 1 | 1 | 1 | 1 | [1 | 2 | 0,1,2,3,4,6 | 2] |
| 2 | 1 | 0,1 | 1 | [2 | 2 | 0,1,2,3,4,5,6 | 2] |
| 3 | 2 | 0 | 1 | [3 | 2 | 0,1,2,3,4,5,6,7 | 2] |
| 4 | 2 | 1 | 1 | 4 | 2 | 0,1,2,3,8 | 1 |
| 5 | 2 | 2 | 1 | 5 | 2 | 0,1,2,3,8,10 | 1 |
| 6 | 2 | 3 | 1 | 6 | 2 | 0,1,2,3,8,9,10 | 1 |
| 7 | 2 | 0,1 | 1 | 7 | 2 | 0,1,2,3,8,9,10,11 | 1 |
| 8 | 2 | 2,3 | 1 | [8 | 1 | 0,1,4,5,8 | 2] |
| 9 | 2 | 0-2 | 1 | [9 | 1 | 0,1,4,5,8,12 | 2] |
| 10 | 2 | 0-3 | 1 | [10 | 1 | 0,1,4,5,8,9,12 | 2] |
| 11 | 2 | 0,2 | 1 | [11 | 1 | 0,1,4,5,8,9,12,13 | 2] |
| 12 | 2 | 0 | 2 | [12 | 2 | 0,1,4,5,8 | 2] |
| 13 | 2 | 1 | 2 | [13 | 2 | 0,1,4,5,8,12 | 2] |
| 14 | 2 | 2 | 2 | [14 | 2 | 0,1,4,5,8,9,12 | 2] |
| 15 | 2 | 3 | 2 | [15 | 2 | 0,1,4,5,8,9,12,13 | 2] |
| 16 | 2 | 4 | 2 | [16 | 2 | 2,3,6,7,10 | 2] |
| 17 | 2 | 5 | 2 | [17 | 2 | 2,3,6,7,10,14 | 2] |
| 18 | 2 | 6 | 2 | [18 | 2 | 2,3,6,7,10,11,14 | 2] |
| 19 | 2 | 7 | 2 | [19 | 2 | 2,3,6,7,10,11,14,15 | 2] |
| 20 | 2 | 0,1 | 2 | [20 | 2 | 0,1, 2,3,10 | 1] |
| 21 | 2 | 2,3 | 2 | [21 | 2 | 0,1,8,2,3,10 | 1] |
| 22 | 2 | 4,5 | 2 | [22 | 2 | 0,1,8, 2,3,10,11 | 1] |
| 23 | 2 | 6,7 | 2 | [23 | 2 | 0,1,8,9,2,3,10,11 | 1] |
| [24 | 2 | 0,4 | 2] | [24 | 1 | 0,1,4,5,12 | 2] |
| [25 | 2 | 2,6 | 2] | [25 | 1 | 0,1,8,4,5,12 | 2] |
| [26 | 2 | 0,1,4 | 2] | [26 | 1 | 0,1,8,4,5,12,13 | 2] |
| [27 | 2 | 2,3,6 | 2] | [27 | 1 | 0,1,8,9,4,5,12,13 | 2] |
| [28 | 2 | 0,1,4,5 | 2] | [28 | 2 | 0,1,4,5,12 | 2] |
| [29 | 2 | 2,3,6,7 | 2] | [29 | 2 | 0,1,8,4,5,12 | 2] |
| [30 | 2 | 0,2,4,6 | 2] | [30 | 2 | 0,1,8,4,5,12,13 | 2] |
| 31 | 1 | 8 | 1 | [31 | 2 | 0,1,8,9,4,5,12,13 | 2] |
| 32 | 1 | 9 | 1 | [32 | 2 | 2,3,6,7,14 | 2] |
| 33 | 1 | 8,9 | 1 | [33 | 2 | 2,3,10,6,7,14 | 2] |
| 34 | 2 | 8 | 1 | [34 | 2 | 2,3,10,6,7,14,15 | 2] |
| 35 | 2 | 9 | 1 | [35 | 2 | 2,3,10,11,6,7,14,15 | 2] |
| 36 | 2 | 10 | 1 | [36 | 2 | 0,2,3,8,9 | 1] |
| 37 | 2 | 11 | 1 | [37 | 2 | 0,1,2,3,8,9 | 1] |
| 38 | 2 | 8,9 | 1 |  |  |  |  |
| 39 | 2 | 10,11 | 1 |  |  |  |  |
| [40 | 2 | 8-10 | 1] |  |  |  |  |
| [41 | 2 | 8-11 | 1] |  |  |  |  |
| [42 | 2 | 8,10 | 1] |  |  |  |  |
| 43 | 2 | 8 | 2 |  |  |  |  |
| 44 | 2 | 9 | 2 |  |  |  |  |
| 45 | 2 | 10 | 2 |  |  |  |  |
| 46 | 2 | 11 | 2 |  |  |  |  |
| 47 | 2 | 12 | 2 |  |  |  |  |
| 48 | 2 | 13 | 2 |  |  |  |  |
| 49 | 2 | 14 | 2 |  |  |  |  |
| 50 | 2 | 15 | 2 |  |  |  |  |
| 51 | 2 | 8,9 | 2 |  |  |  |  |
| 52 | 2 | 10,11 | 2 |  |  |  |  |
| 53 | 2 | 12,13 | 2 |  |  |  |  |
| 54 | 2 | 14,15 | 2 |  |  |  |  |
| [55 | 2 | 8,12 | 2] |  |  |  |  |
| [56 | 2 | 10,14 | 2] |  |  |  |  |
| [57 | 2 | 8,9,12 | 2] |  |  |  |  |
| [58 | 2 | 10,11,14 | 2] |  |  |  |  |
| [59 | 2 | 8,9,12,13 | 2] |  |  |  |  |
| [60 | 2 | 10,11,14,15 | 2] |  |  |  |  |
| 61 | 2 | 8,10,12,14 | 2 |  |  |  |  |
| 62 | 1 | 0,1,8 | 1 |  |  |  |  |
| 63 | 1 | 0,1,8,9 | 1 |  |  |  |  |
| 64 | 2 | 0,1,8 | 1 |  |  |  |  |
| 65 | 2 | 0,1,8,9 | 1 |  |  |  |  |
| 66 | 2 | 2,3,10 | 1 |  |  |  |  |
| 67 | 2 | 2,3,10,11 | 1 |  |  |  |  |
| [69 | 1 | 0,1,8 | 2] |  |  |  |  |
| [70 | 1 | 0,1,8,9 | 2] |  |  |  |  |
| [71 | 1 | 4,5,12 | 2] |  |  |  |  |
| [72 | 1 | 4,5,12,13 | 2] |  |  |  |  |
| [73 | 2 | 0,1,8 | 2] |  |  |  |  |
| [74 | 2 | 0,1,8,9 | 2] |  |  |  |  |
| [75 | 2 | 4,5,12 | 2] |  |  |  |  |
| [76 | 2 | 4,5,12,13 | 2] |  |  |  |  |
| [77 | 2 | 2,3,10 | 2] |  |  |  |  |
| [78 | 2 | 2,3,10,11 | 2] |  |  |  |  |
| [79 | 2 | 6,7,14 | 2] |  |  |  |  |
| [80 | 2 | 6,7,14,15 | 2] |  |  |  |  |
| [81 | 2 | 5,8,9 | 2] |  |  |  |  |
| [82 | 2 | 7,10,11 | 2] |  |  |  |  |
| [83 | 2 | 7,12,13 | 2] |  |  |  |  |

- ROUND-1

For 1CW

- 1) Row 8-11: These rows are supported for eType1, maxLength=1. Straightforward way is to support them. Same as maxLength=1, row 9-11 should have MU restriction. Note that row 9-11 is necessary for sDCI mTRP (2+1 layer, 2+2 layer).

- 2) Row 24-30, 55-60, 69-80 (Num of front load symbol=2): These rows are useful to increase the max number of total DMRS ports for MU. To meet the WID, at least either of row 24-30/55-60 or row 69-80 would be supported without MU restriction.

- 3) Row 81-82: The benefit is these rows enables MU of 3+3+2 layers. Row 81 {5,8,9} can be multiplexed with row 26 {0,1,4} and row 53 {12,13}. Row 82 {7,10,11} can be multiplexed with row 27 {2,3,6} and row 54 {14,15}. However, whether these rows have use-case depends on whether the row 26-27 are supported without MU restriction.

- 4) Row 83: The use-case is to be multiplexed with 2+2+3+3+4 layers, and it can be multiplexed with row 27 {2,3,6}, row 28 {0,1,4,5}, row 33 {8,9}, and row 54 {14,15}. Is this the benefit?

For 2CWs

- 5) Row 0-3: Is there any use-case compared to the agreed 4-7?

- 6) Row 20-23: Should be supported if row 4-7 are supported for eType1 maxLength=1. Else, should be removed.

- 7) Row 8-19 and row 24-35: The benefit is these rows use only one CDM group for 5-8 DMRS ports (i.e. minimize DMRS overhead, enable efficient multiplexing with another UE in another CDM group). The difference between row 8-19 and row 24-35 is that row 24-35 enables one CW maps to DMRS ports with one TD-OCC index.

- 8) Row 36-37: Should be supported if row 8-9 are supported for eType1 maxLength=1.

Concern:

3) Lenovo, Nokia/NSB, QC: Remove the rows.  this is covered in Alt.3-2.

4) HW/Hi, ZTE (if majority want to keep)

5) HW/Hi, ZTE, Samsung, Lenovo, New H3C  Add [ ].

Please provide your views.

| Company | Comment |
| --- | --- |
| Docomo | Support. For 2), either is ok. If 1) is not acceptable for some UE vendors, we are fine with Alt.2-2. For 7), either is ok. Considering Alt.7-2 would be easier for UE vendors, we think Alt.7-2 is better. |
| OPPO | For 2), we prefer the following Alt.2-3: - Alt.2-3: Support row 24-30 with MU restriction. Remove row 55-60 and 69-80. For 7), we prefer the following Alt.7-3, since they Row 8-11, Row 24-27 have similar overhead and performance as Row 4-7, and a UE with two CWs is not likely to be scheduled with MU-MIMO. - Alt.7-3: Remove row 8-19 and 24-35. |
| Huawei, HiSilicon | Regarding 1), support. Regarding 2), support row 24-30 and 59-60 without restriction. For row 55-56: The benefit of row 24 and 25 is that only TD-OCC dispreading is adopted during channel estimation, which means other co-scheduled DMRS ports with different FD-OCC are not expected to be within the same CDM group. Following this principle, entry 55 and 56 are useless. For row 57-58: The functionality can be perfectly replaced by row 81-82. Regarding 3), support. Thanks FL for the technical analysis. Regarding 4), not support. Row 83 aims to support {3+3+3+3+3} and its superset. Regarding 5), not support. We prefer to keep the legacy rows. Regarding 6), support to remove 20-23. Regarding 7), support row 8-11. Row 12-19 can be further decided based on whether to support MU-MIMO for 2CWs. Regarding 8), support. |
| Google | We are ok to support row 8-11 |
| Samsung | For 1), we support the proposal. For 5), we are fine to keep it. For 7), we have same view with OPPO. For 6) and 8), we support to remove row 20-23 and row 36-37. |
| ZTE | For 1), support rows 8-11. For 2), support rows 24-30 and rows 55-60. Remove rows 69-80. (Alt.2-1) - For row 30, introduce MU-MIMO restriction as legacy. - If “Antenna ports” field can be extended more that 6 bits with >64 rows, we can be fine to support rows 69-80 additionally for improving the scheduling flexibility. For 3), support rows 81-82. - Support both rows 26-27 and rows 81-82 without MU restriction. For 4), fine to keep it if majority prefer. For 5), the legacy rows 0-3 should be kept. For 6), remove rows 20-23. For 7), support rows 8-19 and remove rows 24-35. (Alt.7-1) For 8), remove rows 36-37. |
| Lenovo | For 1), support For 2), support Alt.2-1. For 3), prefer not to include them if total number of rows exceed 64. For 5), prefer to include row 0-3 with including legacy design. For 7), prefer to remove both row 8-19 and row 24-35. For 6) and 8), support |
| New H3C | For 1) we support For 2), we support Alt.2-1 For3） support For 5) don’t support For 6) and 8) remove row 20-23 |
| QC | We thank FL for the great effort. We understand this is a very complicated issue to handle. We have the following comments to the proposal. For single CW: For row 9-11, we are fine them with MU restriction, given M-TRP needs these rows. For row 24-30, 55-60, 69-80, we object Alt 2-1, because it seems technically wrong to us. For example, row 24 = {0,4} does not work in MU from both NW and UE perspective. From NW point of view, there is no {1,5} or {1,8,9}, {5,12,13} to pair with {0,4}. It is not clear to us {0,4} can pair with which rows in the table in a proper way. The only way to do paring seems {0,4}+{1}+{8,9}+{5}+{12,13}. But why not do pairing as {0,1}+{8,9}+{4}+{5}+{12,13}, without introducing row {0,4} in the table. On UE side, as we explained in last meeting already, only 1CW capable UE cannot handle MU with row 24-30, 55-60, as it requires UE to estimate 8 DMRS ports. We can consider another example {0,1,4}. It is impossible to do 2 rank 3 MU with {0,1,4}, because there is no {5, 12, 13}. However, if we replace {0,1,4} by {0,1,8}, it is very easy to pair with {4,5,12} to do 2 rank 3 MU. In summary, row 24-30, 55-60 are targeting optimizing SU performance in high delay spread channel. They are not designed for MU pairing. On the other hand, Alt 2-2 can use row 69-80 for MU, and can use row 24-30 for SU performance optimization. Given Alt 2-2 is much better than Alt 2-1, there is no point to select Alt 2-1. Anyway, if we are listing all alternatives for down selection, it is fair to list all alternatives proposed by companies. Our proposal is Alt 2-3, which should be added in the list. And a default alternative 2-4, which is for the case down selection fails, can also be added. - For row 24-30, 55-60, 69-80, down select from the following: - Alt.2-1: Support row 24-30 and row 55-60 without MU restriction. Remove row 69-80. - Alt.2-2: Support row 69-80 without MU restriction. Support row 24-30 with MU restriction. Remove row 55-60. - Alt.2-3: Support row 24-30 with MU restriction. Remove row 55-60 and 69-80. - Alt 2-4: Support row 69-80 without MU restriction. Remove row 24-30, 55-60. - Alt 2-5: remove row 24-30, 55-60, 69-80 due to no consensus to support them. For row 81-83, we think it is unnecessary optimization for corner scheduling cases. We support remove them. For two CW: We are fine to remove row 0-3. We suggest resolving the other FFS rows following the decision made for eType 1, maxlength 1. |
| CMCC | Row 8-11: Support Row 24-30, 55-60, 69-80 (Num of front load symbol=2): Support row 24-30 is enough. Row 55-60 and row 69-80 have same function with row 24-30, there is no need to support all of them. Rows for 2CWs: We prefer to remove row 8-37. |
| Ericsson | - Support. - Support, prefer Alt2-2. - Support. - Fine with proposal. - Slightly prefer to keep the legacy rows as they are the same for Rel-15 DMRS - Fine with the proposal - Fine with the proposal, either 8-19 or 24-35 shall be kept. - The rows 36 and 37 shall be kept to achieve better performance for 2 CWs. |
| Xiaomi | About 2): First, we believe that supporting row 69-80 is beneficial. The channel estimate performance is better if the indicated DMRS ports for a UE is in the same CDM group. In addition, we support to introduce MU restriction to row 24-30 and row 55-60. |
| Nokia/NSB | - Support - Prefer alt 2-2 - No need. - Fine with the proposal. - OK - Support - No need. We don’t see the gain. - Support |
| Apple | - Fine - Prefer alt 2-3, and definitely cannot support alt 2-1 - Fine - Support - Fine - Fine - Fine, prefer Alt 7-3 - Fine |
| Fraunhofer IIS/HHI | 1) Support 5) OK to keep rows 0-3 6) and 8) Support 7) Similar view as OPPO. Likelihood for MU-MIMO with 2CW for the target UE is less. |
| CATT | For 2), we prefer Alt.2-3 proposed by OPPO. For 3), we propose to remove row 81-82 no matter whether other rows are w/o MU restriction. |
| LGE | For 1), Support For 2), Support Alt.2-1. For 3), Don’t support. For 4), Fine For 5), Don’t support. For 6-8), I think we should discuss this after “maxLength=1” are decided. |
| MediaTek | 1) Support 2) Support Alt. 2-5 3) Support Alt. 3-2 4, 5, 6) Support |
| Sharp | For Proposal 2.1.2, 1) Support. 2) Support Alt 2-1. 7) Support Alt 7-2. |

- ROUND-2

Please comment to the following remaining issues.

FL Proposal 2.1.2

- For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, support Alt.2-X for case 2) in RAN1#113 agreement.

  - For 1CW,

    - 2) For row 24-30, 55-60, 69-80, down select from the following:

      - Alt.2-1: Support row 24-30 and row 55-60 without MU restriction. Remove row 69-80.

      - Alt.2-2: Support row 69-80 without MU restriction. Support row 24-30 with MU restriction. Remove row 55-60.

      - Alt.2-2A: Support row 73-80 without MU restriction. Support up to 4 rows from row 24-30 with MU restriction. Remove row 55-60.

        - FFS: which 4 rows from row 24-30 is selected.

      - Alt.2-3: Support row 24-30 with MU restriction. Remove row 55-60 and 69-80.

      - Alt 2-4: Support row 69-80 without MU restriction. Remove row 24-30, 55-60.

      - Alt 2-5: Remove row 24-30, 55-60, 69-80 due to no consensus to support them.

      - Atl 2-6: Support rows 24-30 and rows 59-60 without MU restriction. Remove rows 69-80 and rows 55-58.

Summary of companies’ views for case2) in offline

|  | Support/fine | Concern | For sTRP | For sDCI mTRP |
| --- | --- | --- | --- | --- |
| Alt.2-1 | ZTE, SS, LG, Sharp | QC: object Alt 2-1 | 64 rows | 65 rows |
| Alt.2-2 | Ericsson, Xiaomi, vivo, QC, Apple, Intel, Docomo | OPPO: one more bit is needed for Alt2-2 compared to Alt.2-3, 2-4 and 2-5. | 70 rows | 71 rows |
| Alt.2-2A |  |  | 63 rows | 64 rows |
| Alt.2-3 | OPPO, vivo, Apple, QC(don’t prefer, but can accept) |  | 58 rows | 59 rows |
| Alt.2-4 | Intel, QC(1st preference), Nokia/NSB, Apple, OPPO, Docomo |  | 63 rows | 64 rows |
| Alt.2-5 | Apple, Nokia/NSB, QC, OPPO |  | 51 rows | 52 rows |
| Alt.2-6 |  | QC: object Alt 2-6 | 60 rows | 61 rows |

    - 3) For row 81-82:

      - Alt.3-1:

        - If RAN1 agree row 26-27 without MU restriction,

          - Support row 81-82 without MU restriction.

        - Else,

          - Remove row 81-82.

      - Alt.3-2: Remove 81-82.

    - 4) [Remove row 83.]

  - For 2CW,

    - 5 [Remove row 0-3.]

    - 6) remove row 20-23.

    - 7) Down select from the following:

      - Alt.7-1: Support row 8-19 and remove row 24-35.

      - Alt.7-2: Support row 24-35 and remove row 8-19.

      - Alt.7-3: Remove row 8-19 and 24-35.

FL: Regarding to case 2), as QC asked in online, please also provide your views whether the set of rows 24-30, 55-60, 69-80, 81-82, 83 for 1CW is necessary for MU-MIMO compared to the agreed rows.

Please provide your views.

| Company | Comment |
| --- | --- |
| OPPO | As mentioned by QC above, we don’t think these rows are necessary for MU-MIMO. We can be fine with 24-30 only with MU restriction for backward compatibility. |
| Ericsson | 1CW, 2): prefer Alt 2-2. 2CW, 7): prefer Alt 7-1 For QC’s question, we need add combinations for rank3 and rank4 using new Rel-18 DMRS ports to increase the MU-MIMO capacity. Rows 69-80 shall be supported. And for legacy combination rows 24-30 shall also be supported to make sure same performance as legacy can be achieved. |
| FL | If time allows, I will bring the proposal of case2 for next online. Please update your name in the above table. Currently, total 51 rows are supported. Alt.2-1 adds +13 rows, Alt.2-2 adds +19 rows, Alt.2-3 adds 7 rows, Alt.2-4 adds +12 rows, Alt.2-5 adds 0 rows, Alt.2-6 adds +9 rows. So, Alt.2-2 exceeds 64 rows, and if Alt.2-2 is supported, whether to increase the DCI size or keep the DCI size and introduce RRC parameter to select the rows would be discussed later. I realize Alt.2-1 also exceed 64. In Alt.2-1, the total number of rows for sTRP is 64, but for sDCI mTRP, it becomes 65. To avoid to exceed the rows than 64, another approach is to select row73-80 and 4 from row 24-30 (total: 63 for sTRP and 64 for sDCI mTRP). I added Alt.2-2A as modified version. If companies has strong opinion not to exceed 64 rows, Alt.2-2A can be compromised proposal. |
| ZTE | Do not support Alt 2-2A. If it is needed to limit the total number of rows under 64 (i.e., ≤ 63 rows in total since one row is reserved for (0, 2, 3) in SDCI MTRP), we think it should be support rows 24-29 and 55-59 and remove according to the following reasons: - Rows 24-30 are the legacy, where only row 30 with MU restriction as specified in the spec. Given that Rel-18 DMRS mainly aims for MU-MIMO scenario as stated in WID, row 30 can be removed. - Rows 55-60 are mirrored from rows 24-30 with different category, hence rows 55-59 need to be supported for MU-MIMO. - Consider DMRS indication field is limited to 6-bit as some companies proposed, it is fine to us to remove rows 69-80 as we mentioned before. - Besides, rows 81-83 is automatically precluded due to the limitation of “6-bit” DMRS indication field. Although we are not fans to limit DMRS indication field size, we can only compromise to this by adopting the following Alt 2-1A for taking progress. Alt.2-1A: Support row 24-29 and row 55-59 without MU restriction. Remove row 30, 60, 69-80. |

- ROUND-3

Please comment to the following remaining issues.

FL Proposal 2.1.2A

- For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, for case 2) in RAN1#113 agreement,

  - For 1CW,

    - 2) For row 24-30, 55-60, 69-80, down select from the following:

      - Alt.2-1: Support row 24-30 and row 55-60 without MU restriction. Remove row 69-80.

      - Alt.2-2: Support row 69-80 without MU restriction. Support row 24-30 with MU restriction. Remove row 55-60.

      - Alt.2-2A: Support row 73-80 without MU restriction. Support up to 4 rows from row 24-30 with MU restriction. Remove row 55-60.

        - FFS: which 4 rows from row 24-30 is selected.

        - FFS: whether the number of antenna ports field is 6bits.

      - Alt.2-3: Support row 24-30 with MU restriction. Remove row 55-60 and 69-80.

      - Alt 2-4: Support row 69-80 without MU restriction. Remove row 24-30, 55-60.

      - Alt 2-5: Remove row 24-30, 55-60, 69-80 due to no consensus to support them.

      - Atl 2-6: Support rows 24-30 and rows 59-60 without MU restriction. Remove rows 69-80 and rows 55-58.

Another alternative (agree row 73-80 at least, and discuss remaining rows later):

FL Proposal 2.1.2B

- For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, for case 2) in RAN1#113 agreement,

  - For 1CW,

    - 2) For row 24-30, 55-60, 69-80, support at least support row 73-80 without MU restriction. In addition to that, following options can be considered:

      - Alt.2-2A: Support up to 4 rows from row 24-30 with MU restriction. Remove row 55-60.

        - FFS: which 4 rows from row 24-30 is selected.

        - FFS: whether the number of antenna ports field is 6bits.

      - Alt.2-2B2: Support more than 4 rows from row 24-30 with MU restriction.

        - FFS: which 4 rows from row 24-30 is selected.

        - FFS: whether the number of antenna ports field is 6bits.

        - FFS: row 69-72, 55-60.

From FL perspective, both set of row 24-30/55-60 and row 69-80 intends to increase the max number of MU (e.g. 4+4+4+4 layers). In that sense, either option can achieve the same purpose. If some UE vendors cannot implement DMRS ports across two CDM groups without MU restriction, the only possible option is to use row 69-80 without MU restriction. BTW, as Nokia/NSB commented on Tuesday offline, the row 69-72 is not needed, because it does not increase the max number of MU. Hence, row 73-80 should be supported without MU restriction.

Please provide your views.

| Company | Comment |
| --- | --- |
| MediaTek | We do not agree with this down selection in proposal 2.1.2A. We propose to put back Alt2-3,2-4 and 2-5 and remove Alt2-1 |
| OPPO | For Proposal 2.1.2A, it is not fair to delete Alt 2-3, 2-4 and 2-5, and only keep Alt 2-1 and 2-2 which needs larger overhead. We are fine with the compromised Alt 2-2A, or keep all the alternatives and down selection based on majority view. |
| QC | For FL Proposal 2.1.2A, we still object Alt 2-1 and Alt 2-6. Similar to MTK and OPPO comment, if Alt 2-1 is kept on the table for future down selection, we request put back Alt2-3,2-4 and 2-5, just for fairness. The reason we object Alt 2-1 are 1) 69-80 offer the same or even better MU scheduling flexibility. 2) UE implementation complexity increases drastically to support 24-30, 55-60 without MU restriction. To answer ZTE’s question why Rel-15 does not have MU restriction to 24-30: The reason is because Rel-18 doubled # DMRS per CDM group which make UE’s MU detection and channel estimation much more difficult. For FL Proposal 2.1.2B, we support it. It can achieve the full MU scheduling flexibility that network needs. Furthermore, it can introduce some rows for SU MIMO performance optimization, without increase UE implementation complexity. This is a good way forward benefit both NW and UE side. If there is no consensus, which is very unfortunate, but we are also fine to accept “Alt 2-5: Remove row 24-30, 55-60, 69-80 due to no consensus to support them”. |
| Lenovo | We think Alt2-5 may be considered if the views are so diverse from different companies. |
| Apple | Proposal 2.1.2A: We cannot support Alt 2-1 and also share similar view as MTK, QC, Oppo that if Alt 2-1 is kept on the table, we should put back Alt2-3,2-4 and 2-5, to be fair. Proposal 2.1.2B: We can support it |
| Nokia/NSB | Fine with both proposals. We think 11 rows (#24-25, 55-56, 69-72, 81-83) are not necessary. 26-29+57-60 are the alternative to 73-80. #30 is good to have. Since we have 81 rows including yellow highlight. So, to go less than 63 (1 for mTRP), at least 14 rows should be removed. We think (#26,27,57,58, 4 UE with 3 layers) and (#28,29,59,60 for 4 UE with 4 layers) are a set. If we need one more, we prefer (#28,29,59,60). |
| LGE | We support both proposal, and prefer to increase antenna port field by 1 bit. |
| Ericsson | We support Alt2-2. The legacy combinations 24-30 need to be supported. Maintaining the legacy combinations and behaviour are more important; the impact of adding 1 more bit in the DCI is much trivial. On row 69-72 we prefer to keep, besides removing them would anyway not resolve the 1-bit issue. |
| Sharp | We are currently fine with Alt 2-1, Alt 2-2, and Alt 2-2A, while our first preference is Alt 2-1. For Alt 2-1, if some UE vendors cannot implement DMRS ports across two CDM groups without MU restriction, we can introduce MU-MIMO restriction for row 30. For Alt 2-2, for MU of {4+4+4+4}, we think difference between Alt 2-1 and Alt 2-2 is the following: - Alt 2-1: DMRS ports for each UE are associated with 2 FD-OCC indexes and 2 TD-OCC indexes - Alt 2-2: DMRS ports for each UE are associated with 4 FD-OCC indexes. Therefore, technical difference is marginal. However, we think row 24-29 with MU-MIMO restriction are not needed, because these are reasonable for MU-MIMO with each other. Therefore, Alt 2-2A is also reasonable. |
| Xiaomi | FL Proposal 2.1.2: For case 2), our first choice is Alt.2-2 because whether the number of antenna ports field is 6 bits is FFS. If increasing DCI by 1 bit is that necessary, Alt.2-4 is our second choice. Considering alt.2-1 is put back, alt.2-4 also can be a possible alternative. FL Proposal 2.1.2B: Support |

eType2, maxLength1

In the online on Wednesday, the following agreement was made.

Agreement - For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, support/remove the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data in RAN1#112bis-e agreement. - For 1CW, - 1) [Support row 9-10 and row 20-23.] - 2) For row 33-34, 44-46, down select from the following: - Alt.2-1: Support row 33-34 and row 44-46. - Alt.2-2: Remove row 33-34 and row 44-46. - 3) For row 60-62, down select from the following: - Alt.3-1: Support row 60-62. - Alt.3-2: Remove row 60-62. - For 2CW, - 4) For row 2-3. - Alt.4-1: Support row 2-3. - Alt.4-2: Remove row 2-3. - 5) [Support row 8-11.] - 6) Remove row 12. - 7) Support row 13-20 if row 8-9 are supported for eType1 maxLength=1. Else, remove row 13-20.

The following is the currently agreed in RAN1#112bis-e and RAN1#113. The remaining rows are highlighted.

Table 7.3.1.2.2-3-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=1

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  | Two codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |
| --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| 0 | 1 | 0 | 0 | 3 | 0-4 |
| 1 | 1 | 1 | 1 | 3 | 0-5 |
| 2 | 1 | 0,1 | [2 | 3 | 12-16] |
| 3 | 2 | 0 | [3 | 3 | 12-17] |
| 4 | 2 | 1 | 4 | 2 | 0,1,2,3,12 |
| 5 | 2 | 2 | 5 | 2 | 0,1,2,3,12,14 |
| 6 | 2 | 3 | 6 | 2 | 0-3,12-14 |
| 7 | 2 | 0,1 | 7 | 2 | 0-3,12-15 |
| 8 | 2 | 2,3 | [8 | 3 | 0,1,2,3,12] |
| [9 | 2 | 0-2] | [9 | 3 | 0,1,2,3,12,14] |
| [10 | 2 | 0-3] | [10 | 3 | 0-3,12-14] |
| 11 | 3 | 0 | [11 | 3 | 0-3,12-15] |
| 12 | 3 | 1 | [12 | 2 | 0,2,3,12,13] |
| 13 | 3 | 2 | [13 | 2 | 0,1,2,3,14] |
| 14 | 3 | 3 | [14 | 2 | 0,1,12,2,3,14] |
| 15 | 3 | 4 | [15 | 2 | 0,1,12,2,3,14,15] |
| 16 | 3 | 5 | [16 | 2 | 0,1,12,13,2,3,14,15] |
| 17 | 3 | 0,1 | [17 | 3 | 0,1,2,3,14] |
| 18 | 3 | 2,3 | [18 | 3 | 0,1,12,2,3,14] |
| 19 | 3 | 4,5 | [19 | 3 | 0,1,12,2,3,14,15] |
| [20 | 3 | 0-2] | [20 | 3 | 0,1,12,13,2,3,14,15] |
| [21 | 3 | 3-5] |  |  |  |
| [22 | 3 | 0-3] |  |  |  |
| [23 | 2 | 0,2] |  |  |  |
| 24 | 1 | 12 |  |  |  |
| 25 | 1 | 13 |  |  |  |
| 26 | 1 | 12,13 |  |  |  |
| 27 | 2 | 12 |  |  |  |
| 28 | 2 | 13 |  |  |  |
| 29 | 2 | 14 |  |  |  |
| 30 | 2 | 15 |  |  |  |
| 31 | 2 | 12,13 |  |  |  |
| 32 | 2 | 14,15 |  |  |  |
| [33 | 2 | 12-14] |  |  |  |
| [34 | 2 | 12-15] |  |  |  |
| 35 | 3 | 12 |  |  |  |
| 36 | 3 | 13 |  |  |  |
| 37 | 3 | 14 |  |  |  |
| 38 | 3 | 15 |  |  |  |
| 39 | 3 | 16 |  |  |  |
| 40 | 3 | 17 |  |  |  |
| 41 | 3 | 12,13 |  |  |  |
| 42 | 3 | 14,15 |  |  |  |
| 43 | 3 | 16,17 |  |  |  |
| [44 | 3 | 12-14] |  |  |  |
| [45 | 3 | 15-17] |  |  |  |
| [46 | 3 | 12-15] |  |  |  |
| [47 | 2 | 12,14] |  |  |  |
| 48 | 1 | 0,1,12 |  |  |  |
| 49 | 1 | 0,1,12,13 |  |  |  |
| 50 | 2 | 0,1,12 |  |  |  |
| 51 | 2 | 0,1,12,13 |  |  |  |
| 52 | 2 | 2,3,14 |  |  |  |
| 53 | 2 | 2,3,14,15 |  |  |  |
| 54 | 3 | 0,1,12 |  |  |  |
| 55 | 3 | 0,1,12,13 |  |  |  |
| 56 | 3 | 2,3,14 |  |  |  |
| 57 | 3 | 2,3,14,15 |  |  |  |
| 58 | 3 | 4,5,16 |  |  |  |
| 59 | 3 | 4,5,16,17 |  |  |  |
| [60 | 3 | 13,15,17] |  |  |  |
| [61 | 3 | 13,15] |  |  |  |
| [62 | 2 | 13,15] |  |  |  |

- ROUND-1

For 1CW

- 1) Row 9-10, 20-23: These rows are the same DMRS ports as R15. If you have concern to support them, please make a comment. Note that row 9-10 is necessary for sDCI mTRP (2+1 layer, 2+2 layer).

- 2) Row 33-34, 44-46: Whether these rows are beneficial depends on whether row 9-10, 20-23 has MU restriction.

- 3) Row 60-62: Whether these rows are beneficial depends on whether the row(s) to be paired has MU restriction.

For 2CWs

- 4) Row 2-3: Is there any use-case compared to the agreed rows?

- 5) Row 8-11: The benefit is CDM group#2 is not used for DMRS or data, which can be used for another UE’s DMRS. In this case, different UE uses different CDM groups, and there should be no issue for MU.

- 6) Row 12: Is there any use-case compared to the agreed rows?

- 7) Row 13-20: Should be supported if row 4-7 are supported for eType1 maxLength=1. Else, should be removed.

FL Proposal 2.1.3

- For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, support/remove the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data in RAN1#112bis-e agreement.

  - For 1CW,

    - 1) Support row 9-10 and row 20-23.

    - 2) For row 33-34, 44-46, down select from the following:

      - Alt.2-1: Support row 33-34 and row 44-46.

      - Alt.2-2: Remove row 33-34 and row 44-46.

    - 3) For row 60-62, down select from the following:

      - Alt.3-1: Support row 60-62.

      - Alt.3-2: Remove row 60-62.

  - For 2CW,

    - 4) For row 2-3.

      - Alt.4-1: Support row 2-3.

      - Alt.4-2: Remove row 2-3.

    - 5) [Support row 8-11.]

    - 6) Remove row 12.

    - 7) Support row 13-20 if row 8-9 are supported for eType1 maxLength=1. Else, remove row 13-20.

Please provide your views.

| Company | Comment |
| --- | --- |
| Docomo | Support. 2) Prefer Alt.2-1. 3) Support Alt.3-1. 4) Either is ok. |
| OPPO | For 1), we think MU restriction is needed to support row 9-10 and row 20-23. The MU-restriction is much more necessary in Rel-18 than in Rel-15 since more ports are introduced in one CDM group and UE needs to estimate interference from more DMRS ports than in Rel-15. For 2), we prefer Alt2-2. For 3), we prefer Alt3-2. For 4), we prefer Alt4-2. For 5), we think row 8-11 can be removed, since a UE with two CWs is not likely to be scheduled with MU-MIMO. For 6), we are fine. For 7), we are fine. |
| Huawei, HiSilicon | Regarding 1), support. Regarding 2), support Alt.2-2. Regarding 3), support Alt.3-1. Regarding 4), can be further decided based on whether to support MU-MIMO for 2CWs. Regarding 5), can be further decided based on whether to support MU-MIMO for 2CWs. Regarding 6), support. Regarding 7), support to remove row 13-20. |
| Google | OK with the proposal |
| Samsung | For 1), we support the proposal. For 2), we prefer to have Alt2-2. For 4), we prefer to have Alt4-2. For 5), we prefer to remove row 8-11. For 6), we support the proposal. For 7), we support to remove row 13-20. |
| ZTE | For 1), support rows 9-10 and rows 20-23. For 2), support rows 33-34 and rows 44-46. (Alt.2-1) For 3), remove rows 60-62. (Alt.3-2) For 4), support rows 2-3. (Alt.4-1). For 5), support rows 8-11. For 6), remove row 12. For 7), remove rows 13-20. |
| Lenovo | For 1), support For 2), prefer Alt.2-1 For 3), prefer Alt.3-2 For 4), prefer Alt.4-1 For 5), prefer not to include 8-11 For 6) and 7), support. |
| New H3C | For 1), support the proposal. For 2), prefer to have Alt2-2. For 3), support Alt.3-1 For 4), prefer to have Alt4-2. For 5)-7), support the proposal |
| QC | For single CW: For row 9-10, given they are needed for M-TRP, we are fine to support them with MU restriction, based on the same reason OPPO mentioned. For row 20-23, we don’t support them. In Rel-15, to support rank 3 or 4, there is no other choice but put 3 DMRS ports in two CDM group. In Rel-18, with doubled # DMRS ports, we have better MU design choice, which is put 3 DMRS ports in one CDM group, like those already agreed row 48-59. With rows 48-58, there is no need to support row 20-23. Furthermore, it is hard to do MU pairing with row 20-23, for example {0,1,2} cannot pair with {12,13}, {3, 14,15} because {3, 14,15} does not exist. Due to similar reason, we don’t support rows 33-34,44-46. For row 60-62, they are for corner scheduling cases. We don’t support them. For two CWs: Given 2CWs does not support MU, row 2-3 has no use cases. They should be removed. Row 12 can be removed. For other FFS rows, they can be resolved following the decision made for eType 1, maxLength 1. |
| Xiaomi | - Support - Support alt.2-2 - Support alt.3-2 4) Support row 2-3 6) Support to remove row 12. |
| Nokia/NSB | - Support - Support Alt 2-2 - Support Alt 3-2 - Support Alt 4-2 - Fine - Support - Fine. |
| Apple | - We cannot support row 9-10 without MU-restriction and for rows 20-23, share similar view as QC - Support Alt 2-2 - Support Alt 3-2 - Support Alt 4-2 - Similar view as Oppo - Fine - Fine |
| Fraunhofer IIS/HHI | 1) Support 3) Support Alt. 3-2 4) Support Alt. 4-2 5) Prefer to remove rows 8-11 |
| CATT | Support. For 2) Alt.2-2 is preferred. For 3) Alt.3-2 is preferred. |
| LGE | For 1), Support the proposal. For 2), Support Alt.2-1 For 3), Support Alt.3-2 For 4), Support Alt.4-2 For 5), Support the proposal. For 6), Support the proposal. For 7), Support the proposal. |
| Sharp | For Proposal 2.1.3, 2) Support Alt 2-1. 3) Support Alt 3-2. 4) OK with either. |

- ROUND-2

Please comment to the following remaining issues.

  - For 1CW,

    - 1) [Support row 9-10 and row 20-23.]

    - 2) For row 33-34, 44-46, down select from the following:

      - Alt.2-1: Support row 33-34 and row 44-46.

      - Alt.2-2: Remove row 33-34 and row 44-46.

    - 3) For row 60-62, down select from the following:

      - Alt.3-1: Support row 60-62.

      - Alt.3-2: Remove row 60-62.

  - For 2CW,

    - 4) For row 2-3.

      - Alt.4-1: Support row 2-3.

      - Alt.4-2: Remove row 2-3.

    - 5) [Support row 8-11.]

FL: Regarding to case 2), as QC asked in online, please also provide your views whether the set of rows 9-10, 20-23, 33-34, 44-46, 60-62 for 1CW is necessary for MU-MIMO compared to the agreed rows.

BTW, QC’s last question is if there will be no consequence for case2 or case3), whether these rows are not supported. My understanding is the same (these rows are not supported), and if that is the common understanding, we can make the WA as agreement.

FL Proposal 2.1.3B

Confirm the following WA in RAN1#113: Offline consensus

For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, support/remove the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data in RAN1#112bis-e agreement.

- For 1CW,

  - 1) [Support row 9-10 and row 20-23.]

  - 2) For row 33-34, 44-46, down select from the following:

    - Alt.2-1: Support row 33-34 and row 44-46.

    - Alt.2-2: Remove row 33-34 and row 44-46.

  - 3) For row 60-62, down select from the following:

    - Alt.3-1: Support row 60-62.

    - Alt.3-2: Remove row 60-62.

- For 2CW,

  - 4) For row 2-3.

    - Alt.4-1: Support row 2-3.

    - Alt.4-2: Remove row 2-3.

  - 5) [Support row 8-11.]

  - 6) Remove row 12.

  - 7) Support row 13-20 if row 8-9 are supported for eType1 maxLength=1. Else, remove row 13-20.

Please provide your views.

| Company | Comment |
| --- | --- |
| OPPO | As mentioned by QC above, we don’t think these rows are necessary for MU-MIMO. We can be fine with row 9-10 and row 20-23 with MU restriction for backward compatibility. |
| QC |  |

eType2, maxLength2

In the online on Wednesday, the following was agreed.

FL Proposal 2.1.4 (DMRS port table for eType2, maxLength2) - For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, support/remove the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data in RAN1#112bis-e agreement. - For 1CW, - 1) [Support row 9-10, 20-23]. - 2) For row 42-47, 100-105, 129-152, down select or modify from the following: - Alt.2-1: Support row 42-47 and row 100-105 without MU restriction. Remove row 129-152. - Alt.2-2: Support row 129-152 without MU restriction. Support row 42-47 with MU restriction. Remove row 100-105. - Alt.2-3: Support row 42-47 with MU restriction. Remove row 100-105 and 129-152. - Alt 2-4: Support row 129-152 without MU restriction. Remove row 42-47, 100-105. - Alt 2-5: remove row 42-47, 100-105, 129-152 due to no consensus to support them. - Alt 2-6: Support row 42-47 and 103-105 without MU restriction, remove 100-102 and 129-152 - 3) For row 67-68, 78-80: - Alt.3-1: - If RAN1 agree row 9-10, 20-23 without MU restriction - Support row 67-68, 78-80 without MU restriction. - Else, - Remove row 67-68, 78-80. - Alt.3-2: Remove 67-68,78-80. - 4) For row 153-158, down select from the following. - Alt.4-1: Support row 153-158 without MU restriction. - Alt.4-2: Remove row 153-158. - For 2CW, - 5) [Remove row 0-5.] - 6) [Support row 10-13.] - 7) Down select from the following: - Alt.7-1: Support row 14-37 and remove row 46-69. - Alt.7-2: Support row 46-69 and remove row 14-37. - Alt.7-3: Remove row 14-37 and row 46-69. - 8) remove row 38-45.

The following is the currently agreed in RAN1#112bis-e and RAN1#113. The remaining rows are highlighted.

Table 7.3.1.2.2-4-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=2

| One codeword: Codeword 0 enabled, Codeword 1 disabled |  |  |  | Two Codewords: Codeword 0 enabled, Codeword 1 enabled |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols | Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| 0 | 1 | 0 | 1 | [0 | 3 | 0-4 | 1] |
| 1 | 1 | 1 | 1 | [1 | 3 | 0-5 | 1] |
| 2 | 1 | 0,1 | 1 | [2 | 2 | 0,1,2,3,6 | 2] |
| 3 | 2 | 0 | 1 | [3 | 2 | 0,1,2,3,6,8 | 2] |
| 4 | 2 | 1 | 1 | [4 | 2 | 0,1,2,3,6,7,8 | 2] |
| 5 | 2 | 2 | 1 | [5 | 2 | 0,1,2,3,6,7,8,9 | 2] |
| 6 | 2 | 3 | 1 | 6 | 2 | 0,1,2,3,12 | 1 |
| 7 | 2 | 0,1 | 1 | 7 | 2 | 0-3,12,14 | 1 |
| 8 | 2 | 2,3 | 1 | 8 | 2 | 0-3,12-14 | 1 |
| [9 | 2 | 0-2 | 1] | 9 | 2 | 0-3,12-15 | 1 |
| [10 | 2 | 0-3 | 1] | [10 | 3 | 0,1,2,3,12 | 1] |
| 11 | 3 | 0 | 1 | [11 | 3 | 0-3,12,14 | 1] |
| 12 | 3 | 1 | 1 | [12 | 3 | 0-3,12-14 | 1] |
| 13 | 3 | 2 | 1 | [13 | 3 | 0-3,12-15 | 1] |
| 14 | 3 | 3 | 1 | [14 | 1 | 0,1,6,7,12 | 2] |
| 15 | 3 | 4 | 1 | [15 | 1 | 0,1,6,7,12,18 | 2] |
| 16 | 3 | 5 | 1 | [16 | 1 | 0,1,6,7,12,13,18 | 2] |
| 17 | 3 | 0,1 | 1 | [17 | 1 | 0,1,6,7,12,13,18,19 | 2] |
| 18 | 3 | 2,3 | 1 | [18 | 2 | 0,1,6,7,12 | 2] |
| 19 | 3 | 4,5 | 1 | [19 | 2 | 0,1,6,7,12,18 | 2] |
| [20 | 3 | 0-2 | 1] | [20 | 2 | 0,1,6,7,12,13,18 | 2] |
| [21 | 3 | 3-5 | 1] | [21 | 2 | 0,1,6,7,12,13,18,19 | 2] |
| [22 | 3 | 0-3 | 1] | [22 | 2 | 2,3,8,9,14 | 2] |
| [23 | 2 | 0,2 | 1] | [23 | 2 | 2,3,8,9,14,20 | 2] |
| 24 | 3 | 0 | 2 | [24 | 2 | 2,3,8,9,14,15,20 | 2] |
| 25 | 3 | 1 | 2 | [25 | 2 | 2,3,8,9,14,15,20,21 | 2] |
| 26 | 3 | 2 | 2 | [26 | 3 | 0,1,6,7,12 | 2] |
| 27 | 3 | 3 | 2 | [27 | 3 | 0,1,6,7,12,18 | 2] |
| 28 | 3 | 4 | 2 | [28 | 3 | 0,1,6,7,12,13,18 | 2] |
| 29 | 3 | 5 | 2 | [29 | 3 | 0,1,6,7,12,13,18,19 | 2] |
| 30 | 3 | 6 | 2 | [30 | 3 | 2,3,8,9,14 | 2] |
| 31 | 3 | 7 | 2 | [31 | 3 | 2,3,8,9,14,20 | 2] |
| 32 | 3 | 8 | 2 | [32 | 3 | 2,3,8,9,14,15,20 | 2] |
| 33 | 3 | 9 | 2 | [33 | 3 | 2,3,8,9,14,15,20,21 | 2] |
| 34 | 3 | 10 | 2 | [34 | 3 | 4,5,10,11,16 | 2] |
| 35 | 3 | 11 | 2 | [35 | 3 | 4,5,10,11,16,22 | 2] |
| 36 | 3 | 0,1 | 2 | [36 | 3 | 4,5,10,11,16,17,22 | 2] |
| 37 | 3 | 2,3 | 2 | [37 | 3 | 4,5,10,11,16,17,22,23 | 2] |
| 38 | 3 | 4,5 | 2 | [38 | 2 | 0,1,2,3,14 | 1] |
| 39 | 3 | 6,7 | 2 | [39 | 2 | 0,1,12,2,3,14 | 1] |
| 40 | 3 | 8,9 | 2 | [40 | 2 | 0,1,12,2,3,14,15 | 1] |
| 41 | 3 | 10,11 | 2 | [41 | 2 | 0,1,12,13,2,3,14,15 | 1] |
| [42 | 3 | 0,1,6 | 2] | [42 | 3 | 0,1,2,3,14 | 1] |
| [43 | 3 | 2,3,8 | 2] | [43 | 3 | 0,1,12,2,3,14 | 1] |
| [44 | 3 | 4,5,10 | 2] | [44 | 3 | 0,1,12,2,3,14,15 | 1] |
| [45 | 3 | 0,1,6,7 | 2] | [45 | 3 | 0,1,12,13,2,3,14,15 | 1] |
| [46 | 3 | 2,3,8,9 | 2] | [46 | 1 | 0,1,6,7,18 | 2] |
| [47 | 3 | 4,5,10,11 | 2] | [47 | 1 | 0,1,12,6,7,18 | 2] |
| 48 | 1 | 0 | 2 | [48 | 1 | 0,1,12,6,7,18,19 | 2] |
| 49 | 1 | 1 | 2 | [49 | 1 | 0,1,12,13,6,7,18,19 | 2] |
| 50 | 1 | 6 | 2 | [50 | 2 | 0,1,6,7,18 | 2] |
| 51 | 1 | 7 | 2 | [51 | 2 | 0,1,12,6,7,18 | 2] |
| 52 | 1 | 0,1 | 2 | [52 | 2 | 0,1,12,6,7,18,19 | 2] |
| 53 | 1 | 6,7 | 2 | [53 | 2 | 0,1,12,13,6,7,18,19 | 2] |
| 54 | 2 | 0,1 | 2 | [54 | 2 | 2,3,8,9,20 | 2] |
| 55 | 2 | 2,3 | 2 | [55 | 2 | 2,3,14,8,9,20 | 2] |
| 56 | 2 | 6,7 | 2 | [56 | 2 | 2,3,14,8,9,20,21 | 2] |
| 57 | 2 | 8,9 | 2 | [57 | 2 | 2,3,14,15,8,9,20,21 | 2] |
| 58 | 1 | 12 | 1 | [58 | 3 | 0,1,6,7,18 | 2] |
| 59 | 1 | 13 | 1 | [59 | 3 | 0,1,12,6,7,18 | 2] |
| 60 | 1 | 12,13 | 1 | [60 | 3 | 0,1,12,6,7,18,19 | 2] |
| 61 | 2 | 12 | 1 | [61 | 3 | 0,1,12,13,6,7,18,19 | 2] |
| 62 | 2 | 13 | 1 | [62 | 3 | 2,3,8,9,20 | 2] |
| 63 | 2 | 14 | 1 | [63 | 3 | 2,3,14,8,9,20 | 2] |
| 64 | 2 | 15 | 1 | [64 | 3 | 2,3,14,8,9,20,21 | 2] |
| 65 | 2 | 12,13 | 1 | [65 | 3 | 2,3,14,15,8,9,20,21 | 2] |
| 66 | 2 | 14,15 | 1 | [66 | 3 | 4,5,10,11,22 | 2] |
| [67 | 2 | 12-14 | 1] | [67 | 3 | 4,5,16,10,11,22 | 2] |
| [68 | 2 | 12-15 | 1] | [68 | 3 | 4,5,16,10,11,22,23 | 2] |
| 69 | 3 | 12 | 1 | [69 | 3 | 4,5,16,17,10,11,22,23 | 2] |
| 70 | 3 | 13 | 1 |  |  |  |  |
| 71 | 3 | 14 | 1 |  |  |  |  |
| 72 | 3 | 15 | 1 |  |  |  |  |
| 73 | 3 | 16 | 1 |  |  |  |  |
| 74 | 3 | 17 | 1 |  |  |  |  |
| 75 | 3 | 12,13 | 1 |  |  |  |  |
| 76 | 3 | 14,15 | 1 |  |  |  |  |
| 77 | 3 | 16,17 | 1 |  |  |  |  |
| [78 | 3 | 12-14 | 1] |  |  |  |  |
| [79 | 3 | 15-17 | 1] |  |  |  |  |
| [80 | 3 | 12-15 | 1] |  |  |  |  |
| [81 | 2 | 12,14 | 1] |  |  |  |  |
| 82 | 3 | 12 | 2 |  |  |  |  |
| 83 | 3 | 13 | 2 |  |  |  |  |
| 84 | 3 | 14 | 2 |  |  |  |  |
| 85 | 3 | 15 | 2 |  |  |  |  |
| 86 | 3 | 16 | 2 |  |  |  |  |
| 87 | 3 | 17 | 2 |  |  |  |  |
| 88 | 3 | 18 | 2 |  |  |  |  |
| 89 | 3 | 19 | 2 |  |  |  |  |
| 90 | 3 | 20 | 2 |  |  |  |  |
| 91 | 3 | 21 | 2 |  |  |  |  |
| 92 | 3 | 22 | 2 |  |  |  |  |
| 93 | 3 | 23 | 2 |  |  |  |  |
| 94 | 3 | 12,13 | 2 |  |  |  |  |
| 95 | 3 | 14,15 | 2 |  |  |  |  |
| 96 | 3 | 16,17 | 2 |  |  |  |  |
| 97 | 3 | 18,19 | 2 |  |  |  |  |
| 98 | 3 | 20,21 | 2 |  |  |  |  |
| 99 | 3 | 22,23 | 2 |  |  |  |  |
| [100 | 3 | 12,13,18 | 2] |  |  |  |  |
| [101 | 3 | 14,15,20 | 2] |  |  |  |  |
| [102 | 3 | 16,17,22 | 2] |  |  |  |  |
| [103 | 3 | 12,13,18,19 | 2] |  |  |  |  |
| [104 | 3 | 14,15,20,21 | 2] |  |  |  |  |
| [105 | 3 | 16,17,22,23 | 2] |  |  |  |  |
| 106 | 1 | 12 | 2 |  |  |  |  |
| 107 | 1 | 13 | 2 |  |  |  |  |
| 108 | 1 | 18 | 2 |  |  |  |  |
| 109 | 1 | 19 | 2 |  |  |  |  |
| 110 | 1 | 12,13 | 2 |  |  |  |  |
| 111 | 1 | 18,19 | 2 |  |  |  |  |
| 112 | 2 | 12,13 | 2 |  |  |  |  |
| 113 | 2 | 14,15 | 2 |  |  |  |  |
| 114 | 2 | 18,19 | 2 |  |  |  |  |
| 115 | 2 | 20,21 | 2 |  |  |  |  |
| 116 | 1 | 0,1,12 | 1 |  |  |  |  |
| 117 | 1 | 0,1,12,13 | 1 |  |  |  |  |
| 118 | 2 | 0,1,12 | 1 |  |  |  |  |
| 119 | 2 | 0,1,12,13 | 1 |  |  |  |  |
| 120 | 2 | 2,3,14 | 1 |  |  |  |  |
| 121 | 2 | 2,3,14,15 | 1 |  |  |  |  |
| 122 | 3 | 0,1,12 | 1 |  |  |  |  |
| 123 | 3 | 0,1,12,13 | 1 |  |  |  |  |
| 124 | 3 | 2,3,14 | 1 |  |  |  |  |
| 125 | 3 | 2,3,14,15 | 1 |  |  |  |  |
| 126 | 3 | 4,5,16 | 1 |  |  |  |  |
| 127 | 3 | 4,5,16,17 | 1 |  |  |  |  |
| [129 | 1 | 0,1,12 | 2] |  |  |  |  |
| [130 | 1 | 0,1,12,13 | 2] |  |  |  |  |
| [131 | 1 | 6,7,18 | 2] |  |  |  |  |
| [132 | 1 | 6,7,18,19 | 2] |  |  |  |  |
| [133 | 2 | 0,1,12 | 2] |  |  |  |  |
| [134 | 2 | 0,1,12,13 | 2] |  |  |  |  |
| [135 | 2 | 6,7,18 | 2] |  |  |  |  |
| [136 | 2 | 6,7,18,19 | 2] |  |  |  |  |
| [137 | 2 | 2,3,14 | 2] |  |  |  |  |
| [138 | 2 | 2,3,14,15 | 2] |  |  |  |  |
| [139 | 2 | 8,9,20 | 2] |  |  |  |  |
| [140 | 2 | 8,9,20,21 | 2] |  |  |  |  |
| [141 | 3 | 0,1,12 | 2] |  |  |  |  |
| [142 | 3 | 0,1,12,13 | 2] |  |  |  |  |
| [143 | 3 | 6,7,18 | 2] |  |  |  |  |
| [144 | 3 | 6,7,18,19 | 2] |  |  |  |  |
| [145 | 3 | 2,3,14 | 2] |  |  |  |  |
| [146 | 3 | 2,3,14,15 | 2] |  |  |  |  |
| [147 | 3 | 8,9,20 | 2] |  |  |  |  |
| [148 | 3 | 8,9,20,21 | 2] |  |  |  |  |
| [149 | 3 | 4,5,16 | 2] |  |  |  |  |
| [150 | 3 | 4,5,16,17 | 2] |  |  |  |  |
| [151 | 3 | 10,11,22 | 2] |  |  |  |  |
| [152 | 3 | 10,11,22,23 | 2] |  |  |  |  |
| [153 | 3 | 7,12,13 | 2] |  |  |  |  |
| [154 | 3 | 9,14,15 | 2] |  |  |  |  |
| [155 | 3 | 11,16,17 | 2] |  |  |  |  |
| [156 | 3 | 9,18,19 | 2] |  |  |  |  |
| [157 | 3 | 18,19,20 | 2] |  |  |  |  |
| [158 | 3 | 21,22,23 | 2] |  |  |  |  |

For 1CW

- 1) Row 9-10, 20-23: These rows are the same DMRS ports as R15. If you have concern to support them, please make a comment. Note that row 9-10 is necessary for sDCI mTRP (2+1 layer, 2+2 layer).

- 2) Row 42-47, 100-105, 129-152 (Num of front load symbol=2): These rows are useful to increase the max number of total DMRS ports for MU. To meet the WID, at least either of row 42-47/100-105 or row 129-152 would be supported without MU restriction.

- 3) Row 67-68, 78-80: Whether these rows are beneficial depends on whether row 9-10, 20-23 has MU restriction.

- 4) Row 153-155: Whether these rows are beneficial depends on whether the row(s) to be paired has MU restriction.

For 2CWs

- 5) Row 0-5: Is there any use-case compared to the agreed rows?

- 6) Row 10-13: The benefit is CDM group#2 is not used for DMRS or data, which can be used for another UE’s DMRS. In this case, different UE uses different CDM groups, and there should be no issue for MU.

- 7) Row 14-37 and 46-69: The benefit is these rows use only one CDM group for 5-8 DMRS ports (i.e. minimize DMRS overhead, enable efficient multiplexing with another UE in another CDM group). The difference between row 14-37 and row 46-69 is that row 46-69 enables one CW maps to DMRS ports with one TD-OCC index.

- 8) Row 38-45: Should be supported if row 4-7 are supported for eType1 maxLength=1. Else, should be removed.

FL Proposal 2.1.4

- For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, support/remove the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data in RAN1#112bis-e agreement.

  - For 1CW,

    - 1) Support row 9-10, 20-23.

    - 2) For row 42-47, 100-105, 129-152, down select from the following:

      - Alt.2-1: Support row 42-47 and row 100-105 without MU restriction. Remove row 129-152.

      - Alt.2-2: Support row 129-152 without MU restriction. Support row 42-47 with MU restriction. Remove row 100-105.

      - Alt.2-3: Support row 42-47 with MU restriction. Remove row 100-105 and 129-152.

      - Alt 2-4: Support row 129-152 without MU restriction. Remove row 42-47, 100-105.

      - Alt 2-5: remove row 42-47, 100-105, 129-152 due to no consensus to support them.

    - 3) For row 67-68, 78-80:

      - Alt.3-1:

        - If RAN1 agree row 9-10, 20-23 without MU restriction

          - Support row 67-68, 78-80 without MU restriction.

        - Else,

          - Remove row 67-68, 78-80.

      - Alt.3-2: Remove 67-68,78-80.

    - 4) For row 153-155, down select from the following.

      - Alt.4-1: Support row 153-155 without MU restriction.

      - Alt.4-2: Remove row 153-155.

  - For 2CW,

    - 5) [Remove row 0-5.]

    - 6) [Support row 10-13.]

    - 7) Down select from the following:

      - Alt.7-1: Support row 14-37 and remove row 46-69.

      - Alt.7-2: Support row 46-69 and remove row 14-37.

      - Alt.7-3: Remove row 14-37 and row 46-69.

    - 8) Support row 38-45 if row 4-7 are supported for eType1 maxLength=1. Else, remove row 38-45.

Please provide your views.

| Company | Comment |
| --- | --- |
| Docomo | Support. 2) We prefer Alt.2-1, but we can live with Alt.2-2. 7) Either is ok. Considering Alt.7-2 would be easier for UE vendors, we think Alt.7-2 is better. |
| OPPO | For 1), similar to the case with single symbol, MU restriction is needed. For 2), we prefer the following Alt.2-3 - Alt.2-3: Support row 42-47 with MU restriction. Remove row 100-105 and 129-152. For 3), support to remove row 67-68, 78-80. For 4), support Alt.4-2. For 5), support. For 6) prefer to remove Row 10-13, considering a UE with two CWs is not likely to be scheduled with MU-MIMO. For 7), we prefer Alt.7-3 below, since Row 14-17 and Row 46-49 have similar overhead and performance as Row 6-9, and a UE with two CWs is not likely to be scheduled with MU-MIMO, via Row 18-37 and Row 50-69. - Alt.7-3: Remove row 14-37 and row 46-69. For 8), support. |
| Huawei, HiSilicon | Regarding 1), support. Regarding 2), support row 42-47 and 103-105 without restriction. For row 100-102: The functionality can be perfectly replaced by row 153-155. Regarding 3), support to remove 67-68 and 78-80. Regarding 4), Support Alt.4-1. Regarding 5), not support. We prefer to keep the legacy rows. Regarding 6), can be further decided based on whether to support MU-MIMO for 2CWs. Regarding 7), support row 14-17. Row 18-37 can be further decided based on whether to support MU-MIMO for 2CWs. Regarding 8), support to remove 38-45. |
| Google | OK with the proposal |
| ZTE | For 1), support rows 9-10 and rows 20-23. For 2), support rows 42-47 and rows 100-105 without MU restriction. Remove row 129-152. (Alt.2-1) - If “Antenna ports” field can be extended more that 7 bits with >128 rows, we can be fine to support rows 129-152 additionally for improving the scheduling flexibility. For 3), support row 67-68, 78-80. - For row 68, introduce MU-MIMO restriction as legacy. For 4), support rows 153-155. (Alt.4-1). For 5), the legacy rows 0-5 should be kept. For 6), support rows 10-13. For 7), support rows 14-37 and remove rows 46-69. (Alt.7-1) For 8), remove rows 38-45. |
| Lenovo | For 1), support For 2), prefer Alt.2-1 For 3), support For 4), prefer Alt.4-2 For 5), support to include row 0-5 to align with legacy design For 6), prefer not to include 10-13 For 7), prefer to remove both row 14-37 and row 46-69. For 8), support |
| New H3C | OK in general |
| QC | For Single CW: For row 9-10, given they are needed for M-TRP, we are fine to support them with MU restriction, based on the same reason OPPO mentioned. For row 20-23, they don’t follow MU pairing principle which try to fill a CDM group or a TD-OCC first, before moving to other CDM group or TD-OCC. We don’t support them. Furthermore, it is not convenient to do MU paring with them. For example, how to do 2 rank 3 MU pairing with row 42 {0, 1, 6}, given that there is no {7,18,19}. Furthermore, with already agreed row 116-127, why we still need row 20-23, given 112-127 have same or even better MU paring functionality? Similar comment for 67-68,78-80, with already agreed row 116-127, why we still need row 67-68,78-80, given 112-127 have same or even better MU paring functionality? Similarly, for 42-47, 100-105, they don’t support them. We support row 129-150 as they follow the legacy MU pairing principle. Furthermore, it is more convenient to do MU pairing, e.g., 2 rank 3 MU can be easily achieved with {0,1,12}+{6,7,18}. For the proposal, since we are listed all alternatively. We think we could list all alternatives for fairness and completeness. - ) For row 42-47, 100-105, 129-152, down select from the following: - Alt.2-1: Support row 42-47 and row 100-105 without MU restriction. Remove row 129-152. - Alt.2-2: Support row 129-152 without MU restriction. Support row 42-47 with MU restriction. Remove row 100-105. - Alt.2-3: Support row 42-47 with MU restriction. Remove row 100-105 and 129-152. - Alt 2-4: Support row 129-152 without MU restriction. Remove row 42-47, 100-105. - Alt 2-5: remove row 42-47, 100-105, 129-152 due to no consensus to support them. For row 153-158, they should be removed as they are targeting for corner cases. For two CWs: For row 0-5, they can be removed. There are no clear use cases for them. We can resolve other FFS rows following the decision made for eType 1 maxLength 1. |
| Xiaomi | 1) Support 2) Support alt.2-2 |
| Nokia/NSB | - Support - Support Alt 2-2 - Not support. Remove - No need - Fine - Fine - No need both. NR doesn’t support 2CW with MU-MIMO, so no gain. - Fine |
| Apple | - Do not support 9-10 unless MU-restriction is added and share similar view as QC for row 20-23 - On rows 42-47, share similar views as QC - On rows 67-68, 78-80, share similar views as QC - Support Alt 4-2 - Support - Fine - Support Alt 7-3 - Fine |
| Fraunhofer IIS/HHI | 1) Support 2) Support Alt. 2-2 or 2-3 6) and 7) – Similar view as OPPO. MU-MIMO unlikely with 2CW. Remove rows 10-13 and prefer Alt. 7-3. |
| CATT | For 2), support Alt.2-3 proposed by OPPO. For 3), we propose to remove row 67-68, 78-80 no matter whether other rows are w/o MU restriction. For 4), Alt.4-2 is preferred. |
| LGE | For 1), support For 2), prefer Alt.2-1 For 3), support For 4), prefer Alt.4-2 For 5-8), I think we should discuss this after “maxLength=1” are decided. |
| Sharp | For Proposal 2.1.4, 2) Support Alt 2-1. 3) Support Alt 3-1. 4) Support Alt 4-2. 7) Support Alt 7-2. |

- ROUND-2

Please comment to the following remaining issues.

FL Proposal 2.1.4

- For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, support/remove the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data in RAN1#112bis-e agreement.

  - For 1CW,

    - 1) [Support row 9-10, 20-23].

    - 2) For row 42-47, 100-105, 129-152, down select from the following:

      - Alt.2-1: Support row 42-47 and row 100-105 without MU restriction. Remove row 129-152.

      - Alt.2-2: Support row 129-152 without MU restriction. Support row 42-47 with MU restriction. Remove row 100-105.

      - Alt.2-3: Support row 42-47 with MU restriction. Remove row 100-105 and 129-152.

      - Alt 2-4: Support row 129-152 without MU restriction. Remove row 42-47, 100-105.

      - Alt 2-5: remove row 42-47, 100-105, 129-152 due to no consensus to support them.

    - 3) For row 67-68, 78-80:

      - Alt.3-1:

        - If RAN1 agree row 9-10, 20-23 without MU restriction

          - Support row 67-68, 78-80 without MU restriction.

        - Else,

          - Remove row 67-68, 78-80.

      - Alt.3-2: Remove 67-68,78-80.

    - 4) For row 153-155, down select from the following.

      - Alt.4-1: Support row 153-155 without MU restriction.

      - Alt.4-2: Remove row 153-155.

  - For 2CW,

    - 5) [Remove row 0-5.]

    - 6) [Support row 10-13.]

    - 7) Down select from the following:

      - Alt.7-1: Support row 14-37 and remove row 46-69.

      - Alt.7-2: Support row 46-69 and remove row 14-37.

      - Alt.7-3: Remove row 14-37 and row 46-69.

    - 8) Support row 38-45 if row 4-7 are supported for eType1 maxLength=1. Else, remove row 38-45.

FL: I updated only the red part. Please check the above proposal is acceptable.

Please provide your views.

| Company | Comment |
| --- | --- |
| OPPO | For 1) for 1 CW, we still think MU restriction is needed for row 9-10, 20-23. For 6) for 2 CWs, we prefer to remove Row 10-13, considering a UE with two CWs is not likely to be scheduled with MU-MIMO. |

  - DCI size of antenna ports field for PDSCH/PUSCH

FL: Potential RRC impact, if agreed (To be discussed depending on the outcome of sect. 2.1.2/2.1.4). No need to input your views in this section for now.

In RAN1#113bis-e, it was agreed that the existing antenna ports field is increased with at least 1-bit. The max number of rows for R18 DMRS ports are summarized in below tables. Based on the previous agreements and proposals in section 2.3.1 – 2.3.4, additional 1-bit is enough except Type1, maxLength2 and Type2, maxLength2 for PDSCH. Although some of rows are still with [], it is possible that the number of rows exceeds the limitation.

Table 2.2A: The size of antenna ports field in DCI format 1_1/1_2 for PDSCH.

| DMRS type | R15 DMRS ports | R18 DMRS ports (+1bit) | The max. number of rows for R18. |
| --- | --- | --- | --- |
| Type1, maxLength1 | 4-bit | 5-bit | 32 |
| Type1, maxLength2 | 5-bit | 6-bit | 64 |
| Type2, maxLength1 | 5-bit | 6-bit | 64 |
| Type2, maxLength2 | 6-bit | 7-bit | 128 |

Table 2.2B: The size of antenna ports field in DCI format 0_1/0_2 for PUSCH.

| DMRS type | R15 DMRS ports | R18 DMRS ports (+1bit) | The max. number of rows for R18. |
| --- | --- | --- | --- |
| Type1, maxLength1 | 3-bit | 4-bit | 16 |
| Type1, maxLength2 | 4-bit | 5-bit | 32 |
| Type2, maxLength1 | 4-bit | 5-bit | 32 |
| Type2, maxLength2 | 5-bit | 6-bit | 64 |

If the max number of rows exceeds the above tables, how to handle it can be discussed (e.g. increase antenna ports field with 2-bit, or introduce RRC signaling to select some of the rows).

  - Antenna ports field for PUSCH (rank 1-4)

(void)eType1, maxLength1 (rank 1-4)

No remaining issue.

eType1, maxLength2 (rank 1-4)

For eType1, maxLength=1, DMRS ports table for PUSCH is agreed in RAN1#112bis-e. It supports all possible rows of PDSCH including the rows with [ ] and removed rows. Hence, I propose to support all possible rows for PDSCH. The rows with [ ] for PDSCH are highlighted in yellow, and removed rows for PDSCH are highlighted in blue. My proposal is to agree all rows in the proposal (included the highlighted rows). However, if you have concern on some rows, please make a comment.

FL Proposal 2.3.2

- For the antenna ports indication in DCI format 0_1/0_2 for Rel.18 eType1 DMRS ports with maxLength = 2 for PUSCH, following Table 7.3.1.1.2-12-X, Table 7.3.1.1.2-13-X, Table 7.3.1.1.2-14-X, and Table 7.3.1.1.2-15-X are supported.

  - Note: Row(s) agreed for PUSCH does not imply it is also agreed for PDSCH.

Table 7.3.1.1.2-12-X: Antenna port(s), transform precoder is disabled, dmrs-Type=eType1, maxLength=2, rank = 1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |
| 2 | 2 | 0 | 1 |
| 3 | 2 | 1 | 1 |
| 4 | 2 | 2 | 1 |
| 5 | 2 | 3 | 1 |
| 6 | 2 | 0 | 2 |
| 7 | 2 | 1 | 2 |
| 8 | 2 | 2 | 2 |
| 9 | 2 | 3 | 2 |
| 10 | 2 | 4 | 2 |
| 11 | 2 | 5 | 2 |
| 12 | 2 | 6 | 2 |
| 13 | 2 | 7 | 2 |
| 14 | 1 | 8 | 1 |
| 15 | 1 | 9 | 1 |
| 16 | 2 | 8 | 1 |
| 17 | 2 | 9 | 1 |
| 18 | 2 | 10 | 1 |
| 19 | 2 | 11 | 1 |
| 20 | 2 | 8 | 2 |
| 21 | 2 | 9 | 2 |
| 22 | 2 | 10 | 2 |
| 23 | 2 | 11 | 2 |
| 24 | 2 | 12 | 2 |
| 25 | 2 | 13 | 2 |
| 26 | 2 | 14 | 2 |
| 27 | 2 | 15 | 2 |
| 28-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-13-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=2, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0,1 | 1 |
| 1 | 2 | 0,1 | 1 |
| 2 | 2 | 2,3 | 1 |
| 3 | 2 | 0,2 | 1 |
| 4 | 2 | 0,1 | 2 |
| 5 | 2 | 2,3 | 2 |
| 6 | 2 | 4,5 | 2 |
| 7 | 2 | 6,7 | 2 |
| 8 | 2 | 0,4 | 2 |
| 9 | 2 | 2,6 | 2 |
| 10 | 1 | 8,9 | 1 |
| 11 | 2 | 8,9 | 1 |
| 12 | 2 | 10,11 | 1 |
| 13 | 2 | 8,10 | 1 |
| 14 | 2 | 8,9 | 2 |
| 15 | 2 | 10,11 | 2 |
| 16 | 2 | 12,13 | 2 |
| 17 | 2 | 14,15 | 2 |
| 18 | 2 | 8,12 | 2 |
| 19 | 2 | 10,14 | 2 |
| 20-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-14-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=2, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-2 | 1 |
| 1 | 2 | 0,1,4 | 2 |
| 2 | 2 | 2,3,6 | 2 |
| 3 | 2 | 8-10 | 1 |
| 4 | 2 | 8,9,12 | 2 |
| 5 | 2 | 10,11,14 | 2 |
| 6 | 1 | 0,1,8 | 1 |
| 7 | 2 | 0,1,8 | 1 |
| 8 | 2 | 2,3,10 | 1 |
| 9 | 1 | 0,1,8 | 2 |
| 10 | 1 | 4,5,12 | 1 |
| 11 | 2 | 0,1,8 | 2 |
| 12 | 2 | 4,5,12 | 2 |
| 13 | 2 | 2,3,10 | 2 |
| 14 | 2 | 6,7,14 | 2 |
| 15 | 2 | 5,8,9 | 2 |
| 16 | 2 | 7,10,11 | 2 |
| 17 | 2 | 7,12,13 | 2 |
| 18-31 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-15-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=2, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-3 | 1 |
| 1 | 2 | 0,1,4,5 | 2 |
| 2 | 2 | 2,3,6,7 | 2 |
| 3 | 2 | 0,2,4,6 | 2 |
| 4 | 2 | 8-11 | 1 |
| 5 | 2 | 8,9,12,13 | 2 |
| 6 | 2 | 10,11,14,15 | 2 |
| 7 | 2 | 8,10,12,14 | 2 |
| 8 | 1 | 0,1,8,9 | 1 |
| 9 | 2 | 0,1,8,9 | 1 |
| 10 | 2 | 2,3,10,11 | 1 |
| 11 | 1 | 0,1,8,9 | 2 |
| 12 | 1 | 4,5,12,13 | 2 |
| 13 | 2 | 0,1,8,9 | 2 |
| 14 | 2 | 4,5,12,13 | 2 |
| 15 | 2 | 2,3,10,11 | 2 |
| 16 | 2 | 6,7,14,15 | 2 |
| 17-31 | Reserved | Reserved | Reserved |

Please provide your views.

| Company | Comment |
| --- | --- |
| Docomo | Support. |
| Huawei, HiSilicon | Can be postponed until the DMRS port combinations for DL is decided. |
| Google | OK |
| Samsung |  |
| ZTE | Support. |
| Lenovo | Agree with Huawei’s comment that discussion can be postponed. |
| ZTE | Support. |
| QC | For PUSCH, we are fine to support rows with DMRS ports across two CDM groups or two TD-OCC codes, to boost UL SU performance. There is no MU restriction needed for those rows, because gNB knows a UE is co-scheduled with MU or not. But, given we had agreement that DL and UL support the same DMRS ports combinations, we want to add a note to clarify that, a row agreed for PUSCH does not imply it is also agreed for PDSCH. Basically, we need make a few exceptions on top of existing agreement that DL and UL support the same DMRS ports combinations. We hope this point can be clarified and captured as a note in the proposal, before we can agree on it. |
| CMCC | Support |
| Ericsson | Support. PUSCH doesn’t have restrictions and therefore more combinations shall be supported to achieve flexible scheduling in the uplink. |
| Xiaomi | OK |
| vivo | Postponed the discussion until the outcome of DL tables. |
| Nokia/NSB | As long as the number of DCI bit doesn’t increase, generally fine. |
| Apple | Postpone until outcome of DL |
| CATT | We propose to discuss this issue after DMRS ports for PDSCH and STxMP are agreed. |
| LGE | Support |
| Spreadtrum | Postponed the discussion. |
| Ericsson1 | There’re combinations for rank1 and rank2 that we’d like to add. The thinking is these rows would enable more flexible PUSCH scheduling for 1 and 2 ranks for different scenarios, f.e. rows with “Number of DMRS CDM group(s) without data” = 1 give less overhead. Rank 1 and 2 are the most typical rank for PUSCH and therefore we would like to maximumly enable different combinations utilizing the Rel-18 DMRS design if the DCI size is not impacted. Below are the rows we would like to add. Note rows with blue color would enable PUSCH DMRS co-scheduling using only one CDM group; rows with black color use ports across CDM group to achieve more robust performance in the uplink.s Table 1: Antenna port(s), transform precoder is disabled, dmrs-Type=eType1, maxLength=2, rank = 1 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 28; 1; 8; 2. 29; 1; 9; 2. 30; 1; 12; 2. 31; 1; 13; 2. Table 2: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 2 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 20; 2; 0,2; 2. 21; 2; 1,3; 1. 22; 2; 1,3; 2. 23; 2; 4,6; 2. 24; 2; 5,7; 2. 25; 2; 8,10; 2. 26; 2; 9,11; 1. 27; 2; 9,11; 2. 28; 2; 12,14; 2. 29; 2; 13,15; 2. 30; 1; 0,1; 2. 31; 1; 12,13; 2. |

eType2, maxLength1 (rank 1-4)

The rows with [ ] for PDSCH are highlighted in yellow, and removed rows for PDSCH are highlighted in blue. My proposal is to agree all rows in the proposal (included the highlighted rows). However, if you have concern on some rows, please make a comment.

FL Proposal 2.3.3

- For the antenna ports indication in DCI format 0_1/0_2 for Rel.18 eType2 DMRS ports with maxLength = 1 for PUSCH, following Table 7.3.1.1.2-16-X, Table 7.3.1.1.2-17-X, Table 7.3.1.1.2-18-X, and Table 7.3.1.1.2-19-X are supported.

  - Note: Row(s) agreed for PUSCH does not imply it is also agreed for PDSCH.

Table 7.3.1.1.2-16-X: Antenna port(s), transform precoder is disabled, dmrs-Type=eType2, maxLength=1, rank = 1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0 |
| 1 | 1 | 1 |
| 2 | 2 | 0 |
| 3 | 2 | 1 |
| 4 | 2 | 2 |
| 5 | 2 | 3 |
| 6 | 3 | 0 |
| 7 | 3 | 1 |
| 8 | 3 | 2 |
| 9 | 3 | 3 |
| 10 | 3 | 4 |
| 11 | 3 | 5 |
| 12 | 1 | 12 |
| 13 | 1 | 13 |
| 14 | 2 | 12 |
| 15 | 2 | 13 |
| 16 | 2 | 14 |
| 17 | 2 | 15 |
| 18 | 3 | 12 |
| 19 | 3 | 13 |
| 20 | 3 | 14 |
| 21 | 3 | 15 |
| 22 | 3 | 16 |
| 23 | 3 | 17 |
| 24-31 | Reserved | Reserved |

Table 7.3.1.1.2-17-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=1, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 1 | 0,1 |
| 1 | 2 | 0,1 |
| 2 | 2 | 2,3 |
| 3 | 3 | 0,1 |
| 4 | 3 | 2,3 |
| 5 | 3 | 4,5 |
| 6 | 2 | 0,2 |
| 7 | 1 | 12,13 |
| 8 | 2 | 12,13 |
| 9 | 2 | 14,15 |
| 10 | 3 | 12,13 |
| 11 | 3 | 14,15 |
| 12 | 3 | 16,17 |
| 13 | 2 | 12,14 |
| 14 | 3 | 13,15 |
| 15 | 2 | 13,15 |
| 16-31 | Reserved | Reserved |

Table 7.3.1.1.2-18-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=1, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-2 |
| 1 | 3 | 0-2 |
| 2 | 3 | 3-5 |
| 3 | 2 | 12-14 |
| 4 | 3 | 12-14 |
| 5 | 3 | 15-17 |
| 6 | 1 | 0,1,12 |
| 7 | 2 | 0,1,12 |
| 8 | 2 | 2,3,14 |
| 9 | 3 | 0,1,12 |
| 10 | 3 | 2,3,14 |
| 11 | 3 | 4,5,16 |
| 12 | 3 | 13,15,17 |
| 13-31 | Reserved | Reserved |

Table 7.3.1.1.2-19-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=1, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) |
| --- | --- | --- |
| 0 | 2 | 0-3 |
| 1 | 3 | 0-3 |
| 2 | [2 | 12-15] |
| 3 | [3 | 12-15] |
| 4 | 1 | 0,1,12,13 |
| 5 | 2 | 0,1,12,13 |
| 6 | 2 | 2,3,14,15 |
| 7 | 3 | 0,1,12,13 |
| 8 | 3 | 2,3,14,15 |
| 9 | 3 | 4,5,16,17 |
| 10-31 | Reserved | Reserved |

Please provide your views.

| Company | Comment |
| --- | --- |
| Docomo | Support. |
| Huawei, HiSilicon | Can be postponed until the DMRS port combinations for DL is decided. |
| Google | OK |
| ZTE | Support. |
| Lenovo | Agree with Huawei’s comment that discussion can be postponed. |
| QC | We don’t need {13,15,17} for rank 3, which has 3 ports across 3 CDM groups. This goes a bit too far in terms of optimization. For PUSCH, we are fine to support rows with DMRS ports across two CDM groups or two TD-OCC codes, to boost UL SU performance. There is no MU restriction needed for those rows, because gNB knows a UE is co-scheduled with MU or not. But, given we had agreement that DL and UL support the same DMRS ports combinations, we want to add a note to clarify that, a row agreed for PUSCH does not imply it is also agreed for PDSCH. Basically, we need make a few exceptions on top of existing agreement that DL and UL support the same DMRS ports combinations. We hope this point can be clarified and captured as a note in the proposal, before we can agree on it. |
| CMCC | Postpone the discussion. |
| Xiaomi | Fine |
| vivo | Postponed the discussion until the outcome of DL tables. |
| Nokia/NSB | Similar view as etype1. |
| Apple | Postpone until outcome of DL |
| CATT | We propose to discuss this issue after DMRS ports for PDSCH and STxMP are agreed. |
| Sharp | We prefer to postpone. |
| Spreadtrum | Postponed the discussion. |

eType2, maxLength2 (rank 1-4)

The rows with [ ] for PDSCH are highlighted in yellow, and removed rows for PDSCH are highlighted in blue. My proposal is to agree all rows in the proposal (included the highlighted rows). However, if you have concern on some rows, please make a comment.

FL Proposal 2.3.4

- For the antenna ports indication in DCI format 0_1/0_2 for Rel.18 eType2 DMRS ports with maxLength = 2 for PUSCH, following Table 7.3.1.1.2-20-X, Table 7.3.1.1.2-21-X, Table 7.3.1.1.2-22-X, and Table 7.3.1.1.2-23-X are supported.

  - Note: Row(s) agreed for PUSCH does not imply it is also agreed for PDSCH.

Table 7.3.1.1.2-20-X: Antenna port(s), transform precoder is disabled, dmrs-Type=eType2, maxLength=2, rank = 1

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0 | 1 |
| 1 | 1 | 1 | 1 |
| 2 | 2 | 0 | 1 |
| 3 | 2 | 1 | 1 |
| 4 | 2 | 2 | 1 |
| 5 | 2 | 3 | 1 |
| 6 | 3 | 0 | 1 |
| 7 | 3 | 1 | 1 |
| 8 | 3 | 2 | 1 |
| 9 | 3 | 3 | 1 |
| 10 | 3 | 4 | 1 |
| 11 | 3 | 5 | 1 |
| 12 | 3 | 0 | 2 |
| 13 | 3 | 1 | 2 |
| 14 | 3 | 2 | 2 |
| 15 | 3 | 3 | 2 |
| 16 | 3 | 4 | 2 |
| 17 | 3 | 5 | 2 |
| 18 | 3 | 6 | 2 |
| 19 | 3 | 7 | 2 |
| 20 | 3 | 8 | 2 |
| 21 | 3 | 9 | 2 |
| 22 | 3 | 10 | 2 |
| 23 | 3 | 11 | 2 |
| 24 | 1 | 0 | 2 |
| 25 | 1 | 1 | 2 |
| 26 | 1 | 6 | 2 |
| 27 | 1 | 7 | 2 |
| 28 | 1 | 12 | 1 |
| 29 | 1 | 13 | 1 |
| 30 | 2 | 12 | 1 |
| 31 | 2 | 13 | 1 |
| 32 | 2 | 14 | 1 |
| 33 | 2 | 15 | 1 |
| 34 | 3 | 12 | 1 |
| 35 | 3 | 13 | 1 |
| 36 | 3 | 14 | 1 |
| 37 | 3 | 15 | 1 |
| 38 | 3 | 16 | 1 |
| 39 | 3 | 17 | 1 |
| 40 | 3 | 12 | 2 |
| 41 | 3 | 13 | 2 |
| 42 | 3 | 14 | 2 |
| 43 | 3 | 15 | 2 |
| 44 | 3 | 16 | 2 |
| 45 | 3 | 17 | 2 |
| 46 | 3 | 18 | 2 |
| 47 | 3 | 19 | 2 |
| 48 | 3 | 20 | 2 |
| 49 | 3 | 21 | 2 |
| 50 | 3 | 22 | 2 |
| 51 | 3 | 24 | 2 |
| 52 | 1 | 12 | 2 |
| 53 | 1 | 13 | 2 |
| 54 | 1 | 18 | 2 |
| 55 | 1 | 19 | 2 |
| 56-63 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-21-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=2, rank = 2

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 1 | 0,1 | 1 |
| 1 | 2 | 0,1 | 1 |
| 2 | 2 | 2,3 | 1 |
| 3 | 3 | 0,1 | 1 |
| 4 | 3 | 2,3 | 1 |
| 5 | 3 | 4,5 | 1 |
| 6 | 2 | 0,2 | 1 |
| 7 | 3 | 0,1 | 2 |
| 8 | 3 | 2,3 | 2 |
| 9 | 3 | 4,5 | 2 |
| 10 | 3 | 6,7 | 2 |
| 11 | 3 | 8,9 | 2 |
| 12 | 3 | 10,11 | 2 |
| 13 | 1 | 0,1 | 2 |
| 14 | 1 | 6,7 | 2 |
| 15 | 2 | 0,1 | 2 |
| 16 | 2 | 2,3 | 2 |
| 17 | 2 | 6,7 | 2 |
| 18 | 2 | 8,9 | 2 |
| 19 | 1 | 12,13 | 1 |
| 20 | 2 | 12,13 | 1 |
| 21 | 2 | 14,15 | 1 |
| 22 | 3 | 12,13 | 1 |
| 23 | 3 | 14,15 | 1 |
| 24 | 3 | 16,17 | 1 |
| 25 | 2 | 12,14 | 1 |
| 26 | 3 | 12,13 | 2 |
| 27 | 3 | 14,15 | 2 |
| 28 | 3 | 16,17 | 2 |
| 29 | 3 | 18,19 | 2 |
| 30 | 3 | 20,21 | 2 |
| 31 | 3 | 22,23 | 2 |
| 32 | 1 | 12,13 | 2 |
| 33 | 1 | 18,19 | 2 |
| 34 | 2 | 12,13 | 2 |
| 35 | 2 | 14,15 | 2 |
| 36 | 2 | 18,19 | 2 |
| 37 | 2 | 20,21 | 2 |
| 38-63 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-22-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=2, rank = 3

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-2 | 1 |
| 1 | 3 | 0-2 | 1 |
| 2 | 3 | 3-5 | 1 |
| 3 | 3 | 0,1,6 | 2 |
| 4 | 3 | 2,3,8 | 2 |
| 5 | 3 | 4,5,10 | 2 |
| 6 | 2 | 12-14 | 1 |
| 7 | 3 | 12-14 | 1 |
| 8 | 3 | 15-17 | 1 |
| 9 | 3 | 12,13,18 | 2 |
| 10 | 3 | 14,15,20 | 2 |
| 11 | 3 | 16,17,22 | 2 |
| 12 | 1 | 0,1,12 | 1 |
| 13 | 2 | 0,1,12 | 1 |
| 14 | 2 | 2,3,14 | 1 |
| 15 | 3 | 0,1,12 | 1 |
| 16 | 3 | 2,3,14 | 1 |
| 17 | 3 | 4,5,16 | 1 |
| 18 | 1 | 0,1,12 | 2 |
| 19 | 1 | 6,7,18 | 2 |
| 20 | 2 | 0,1,12 | 2 |
| 21 | 2 | 6,7,18 | 2 |
| 22 | 2 | 2,3,14 | 2 |
| 23 | 2 | 8,9,20 | 2 |
| 24 | 3 | 0,1,12 | 2 |
| 25 | 3 | 6,7,18 | 2 |
| 26 | 3 | 2,3,14 | 2 |
| 27 | 3 | 8,9,20 | 2 |
| 28 | 3 | 4,5,16 | 2 |
| 29 | 3 | 10,11,22 | 2 |
| 30 | 3 | 7,12,13 | 2 |
| 31 | 3 | 9,14,15 | 2 |
| 32 | 3 | 11,16,17 | 2 |
| 33 | 3 | 9,18,19 | 2 |
| 34 | 3 | 18,19,20 | 2 |
| 35 | 3 | 21,22,23 | 2 |
| 36-63 | Reserved | Reserved | Reserved |

Table 7.3.1.1.2-23-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType2, maxLength=2, rank = 4

| Value | Number of DMRS CDM group(s) without data | DMRS port(s) | Number of front-load symbols |
| --- | --- | --- | --- |
| 0 | 2 | 0-3 | 1 |
| 1 | 3 | 0-3 | 1 |
| 2 | 3 | 0,1,6,7 | 2 |
| 3 | 3 | 2,3,8,9 | 2 |
| 4 | 3 | 4,5,10,11 | 2 |
| 5 | 2 | 12-15 | 1 |
| 6 | 3 | 12-15 | 1 |
| 7 | 3 | 12,13,18,19 | 2 |
| 8 | 3 | 14,15,20,21 | 2 |
| 9 | 3 | 16,17,22,23 | 2 |
| 10 | 1 | 0,1,12,13 | 1 |
| 11 | 2 | 0,1,12,13 | 1 |
| 12 | 2 | 2,3,14,15 | 1 |
| 13 | 3 | 0,1,12,13 | 1 |
| 14 | 3 | 2,3,14,15 | 1 |
| 15 | 3 | 4,5,16,17 | 1 |
| 16 | 1 | 0,1,12,13 | 2 |
| 17 | 1 | 6,7,18,19 | 2 |
| 18 | 2 | 0,1,12,13 | 2 |
| 19 | 2 | 6,7,18,19 | 2 |
| 20 | 2 | 2,3,14,15 | 2 |
| 21 | 2 | 8,9,20,21 | 2 |
| 22 | 3 | 0,1,12,13 | 2 |
| 23 | 3 | 6,7,18,19 | 2 |
| 24 | 3 | 2,3,14,15 | 2 |
| 25 | 3 | 8,9,20,21 | 2 |
| 26 | 3 | 4,5,16,17 | 2 |
| 27 | 3 | 10,11,22,23 | 2 |
| 28-63 | Reserved | Reserved | Reserved |

Please provide your views.

| Company | Comment |
| --- | --- |
| Docomo | Support. |
| Huawei, HiSilicon | Can be postponed until the DMRS port combinations for DL is decided. |
| Google | OK |
| ZTE | Support. |
| Lenovo | Agree with Huawei’s comment that discussion can be postponed. |
| QC | For PUSCH, we are fine to support rows with DMRS ports across two CDM groups or two TD-OCC codes, to boost UL SU performance. There is no MU restriction needed for those rows, because gNB knows a UE is co-scheduled with MU or not. But, given we had agreement that DL and UL support the same DMRS ports combinations, we want to add a note to clarify that, a row agreed for PUSCH does not imply it is also agreed for PDSCH. Basically, we need make a few exceptions on top of existing agreement that DL and UL support the same DMRS ports combinations. We hope this point can be clarified and captured as a note in the proposal, before we can agree on it. |
| CMCC | Postpone the discussion. |
| Xiaomi | Fine |
| vivo | Postponed the discussion until the outcome of DL tables. |
| Nokia/NSB | OK to postpone. |
| Apple | Postpone until outcome of DL |
| CATT | We propose to discuss this issue after DMRS ports for PDSCH and STxMP are agreed. |
| Sharp | We prefer to postpone. |
| Spreadtrum | Postponed the discussion. |

(Void)

  - MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports

In RAN1#112bis-e, the following FL proposal was discussed.

FL Proposal 2.5B - For MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports, - 1) For PUSCH, there is no restriction. - 2) For PDSCH, there is no additional restriction between Rel.18 UE1 indicated with Rel-18 Legacy ports (eType1: ports 1000-1007, eType2: ports 1000-1011) and Rel.15/18 UE2 indicated with Rel.15 DMRS ports in a CDM group from Rel.17 spec. - Note: MU-MIMO restriction in Rel.17 is applied. - 3) For PDSCH, between Rel.18 UE1 indicated with Rel-18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.15 UE2 indicated with Rel.15 DMRS ports in a CDM group, - UE does not expect such MU-MIMO within a CDM group. - 4) For PDSCH, between Rel.18 UE1 indicated with Rel-18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.18 UE2 indicated with Rel.15 DMRS ports in a CDM group, down select from the following. - Alt.1: UE does not expect such MU-MIMO within a CDM group. - Alt.2: Rel.18 UE2 configured with Rel.15 DMRS ports can be signaled, to indicate that there may be another Rel.18 UE1 with Rel.18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) in the same CDM group, so that the Rel.18 UE2 can assume FD-OCC length 4 for channel estimation of Rel.15 DMRS ports. - Dedicated UE capability is introduced. - The signaling is at least by RRC (FFS: whether to support DCI based signaling). - Alt.3: Introduce restriction that the UE indicated with Rel.15 DMRS ports is not expected to be co-scheduled with a UE indicated with Rel.18 DMRS ports if the orthogonality of length-2 FD-OCC between the co-scheduled DMRS ports cannot be satisfied.

After email discussion, the wording of “UE does not expect such MU-MIMO within a CDM group” was controversial, and we only made the following conclusion for 1).

Conclusion • For MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports, - 1) For PUSCH, there is no restriction.

Since we need to prioritize potential RRC impact, I’d like to check whether Alt.2 for 4) is supported or not. Other part of the proposal has no RRC impact.

FL Proposal 2.4A

- For MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports,

  - 4) For PDSCH, between Rel.18 UE1 indicated with Rel-18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.18 UE2 indicated with Rel.15 DMRS ports in a CDM group, support the following.

    - Alt.2: Rel.18 UE2 configured with Rel.15 DMRS ports can be signaled, to indicate that there may be another Rel.18 UE1 with Rel.18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) in the same CDM group, so that the Rel.18 UE2 can assume FD-OCC length 4 for channel estimation of Rel.15 DMRS ports.

      - Dedicated UE capability is introduced.

      - The signaling is at least by RRC (FFS: whether to support DCI based signaling).

FL: Potential RRC impact, if agreed.

Support/fine: Docomo, Google, ZTE, NewH3C, CMCC, Ericsson, Apple, LGE, Sharp

No: OPPO, HW/Hi, Samsung, Lenovo, FW, QC, Xiaomi, vivo, Nokia/NSB, Fraunhofer, CATT, MTK, Spreadtrum

FL Proposal 2.4A2 (Alternative proposal)

- For MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports,

  - 4) For PDSCH, between Rel.18 UE1 indicated with Rel-18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.18 UE2 indicated with Rel.15 DMRS ports in a CDM group, there is no consensus to support the following.

    - Alt.2: Rel.18 UE2 configured with Rel.15 DMRS ports can be signaled, to indicate that there may be another Rel.18 UE1 with Rel.18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) in the same CDM group, so that the Rel.18 UE2 can assume FD-OCC length 4 for channel estimation of Rel.15 DMRS ports.

      - Dedicated UE capability is introduced.

      - The signaling is at least by RRC (FFS: whether to support DCI based signaling).

In RAN1#112bis-e, most of companies are ok with 2), however, QC mentioned that Rel.18 MU-restriction should be applied to Rel.18 UE (Additional MU restriction to rows of Cat.1 is under discussion). Hence, the following modified proposal is suggested.

FL Proposal 2.4B (for conclusion)

- For MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports,

  - 2) For PDSCH, there is no additional restriction between Rel.18 UE1 indicated with Rel-18 Legacy ports (eType1: ports 1000-1007, eType2: ports 1000-1011) and Rel.15-18 UE2 indicated with Rel.15 DMRS ports in a CDM group from Rel.17 spec.

    - Note1: MU-MIMO restriction in Rel.15-17 is applied to Rel.15-17 UE.

    - Note2: MU-MIMO restriction in Rel.18 is applied to Rel.18 UE.

FL: Seems stable.

| Company | Comment |
| --- | --- |
| Docomo | Proposal 2.4A: Support. If R18 RRC is configured, 1-bit is increased for antenna ports field, and R18 DMRS cannot be configured to some UE (e.g. cell edge UE). Hence, Alt.2 has benefit that gNB can multiplex Ues with R18 DMRS ports, while the DCI size of some Ues is maintained with Rel.15. Proposal 2.4B: OK. |
| OPPO | For Proposal 2.4A: we prefer Alt.1. For Proposal 2.4B: Fine. |
| Huawei, HiSilicon | Proposal 2.4A: Given that RRC-based semi-static indication may incur unnecessary channel estimation performance loss, we prefer Alt.3. Proposal 2.4B: Fine. |
| Google | Support both proposals |
| Samsung | For Proposal 2.4A: we prefer not to have any restriction, since even MU-MIMO with non-orthogonal OCC combinations could be possible in current specification (by using DMRS sequence). For Proposal 2.5B: we can live with it. |
| ZTE | Proposal 2.4A: Support. Proposal 2.4B: Fine with conclusion. |
| Lenovo | For Proposal 2.4A: we prefer Alt.1. For Proposal 2.4B: We are fine with it. |
| New H3C | OK for both roposal in general |
| Futurewei | Proposal 2.4A: we prefer Alt.3. Proposal 2.4B: Fine. |
| QC | For Proposal 2.4A: Alt 1 is the simplest solution. We are also open to discuss Alt 2. But the wording of Alt 2 seems not clear to us. Is the signaling a bit to indicate whether there is co-scheduled MU or not? If the signaling is in the direction of signaling the scheduling decision of other UE, we suggest to check with RAN4 WI progress on NW assisted signaling for advance MU demod (RP-230314), which is actually discussing the same signaling. For Alt 3, we see channel estimation might work, but the interference estimation seems does not work with this approach. For Proposal 2.4B: We are fine with the proposal. |
| CMCC | Proposal 2.4A: Support. Proposal 2.4B: Fine with conclusion. |
| Ericsson | 2.1A: Support. 2.1B: Support. |
| Xiaomi | FL Proposal 2.4A: It is not clear to us that the Rel.18 UE2 can assume FD-OCC length 4 for channel estimation of Rel.15 DMRS ports. Does it mean the R15 DMRS for R18 UE2 changes to R18 DMRS. Then it is not the multiplexing between R18 DMRS and R15 DMRS anymore, right? Therefore, we are not a fan of al.2. |
| vivo | 2.1A: Don’t support. The current proposal is only for the case based on orthogonal DMRS ports. We would like to mention again that MU-MIMO can be co-scheduled by different DMRS scrambling IDs or even different analog beams with non-orthogonal DMRS ports in the current network. No such restriction is specified in current spec that the DMRS ports used in MU-MIMO must be orthogonal. Therefore, no additional restriction is needed. 2.1B: Support. |
| Nokia/NSB | 2.1A: we prefer Alt 3. 2.1B: fine. |
| Apple | 2.4A: Prefer Alt 1, but can live with Alt 2 2.4B: Fine |
| Fraunhofer IIS/HHI | 2.4A: Prefer Alt 1 2.4B: Fine |
| CATT | Proposal 2.4A: Not support. gNB can always schedule these two Ues in one CDM group with different DMRS scrambling IDs or different TCI states. Therefore, the additional signaling is not needed. Proposal 2.4B: Support. |
| LGE | Proposal 2.4A: Support. Proposal 2.4B: Fine with conclusion. |
| MediaTek | 2.4A: Support Alt. 1 2.1B: Fine |
| Sharp | Proposal 2.4A: Support. Proposal 2.4B: Fine. |
| Spreadtrum | 2.4A: We prefer fixed rule without RRC/DCI signaling. Alt 1 is simpler. 2.4B: Support. |

- ROUND-2

For P2.4A, it seems there is no consensus to support P2.4A. The following is suggested.

FL Proposal 2.4A2 (Alternative proposal)

- For MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports,

  - 4) For PDSCH, between Rel.18 UE1 indicated with Rel-18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.18 UE2 indicated with Rel.15 DMRS ports in a CDM group, there is no consensus to support the following.

    - Alt.2: Rel.18 UE2 configured with Rel.15 DMRS ports can be signaled, to indicate that there may be another Rel.18 UE1 with Rel.18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) in the same CDM group, so that the Rel.18 UE2 can assume FD-OCC length 4 for channel estimation of Rel.15 DMRS ports.

      - Dedicated UE capability is introduced.

      - The signaling is at least by RRC (FFS: whether to support DCI based signaling).

For P2.4B, it seems all companies are ok with it.

FL Proposal 2.4B (for conclusion)

- For MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports,

  - 2) For PDSCH, there is no additional restriction between Rel.18 UE1 indicated with Rel-18 Legacy ports (eType1: ports 1000-1007, eType2: ports 1000-1011) and Rel.15-18 UE2 indicated with Rel.15 DMRS ports in a CDM group from Rel.17 spec.

    - Note1: MU-MIMO restriction in Rel.15-17 is applied to Rel.15-17 UE.

    - Note2: MU-MIMO restriction in Rel.18 is applied to Rel.18 UE.

If you cannot agree the FL proposal 2.4.A2 or 2.4B, please indicate below.

| Company | Comment |
| --- | --- |
| Ericsson | For 2.4A, if we agree on this conclusion, does it mean “Alt.1: UE does not expect such MU-MIMO within a CDM group.” is automatically applied? FL: Not exactly. The above proposal does not intend to update spec. to explicitly say Alt.1. Whether to update the spec. for Alt.1 or not can be discussed later, because it has no RRC impact. Fine with 2.4B. |

- ROUND-3

On Wednesday online, the following conclusions were made.

Conclusion FL Proposal 2.4A2 (MU b/w R15 DMRS ports and R18 DMRS ports) - For MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports, - 4) For PDSCH, between Rel.18 UE1 indicated with Rel-18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.18 UE2 indicated with Rel.15 DMRS ports in a CDM group, there is no consensus to support the following. - Alt.2: Rel.18 UE2 configured with Rel.15 DMRS ports can be signaled, to indicate that there may be another Rel.18 UE1 with Rel.18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) in the same CDM group, so that the Rel.18 UE2 can assume FD-OCC length 4 for channel estimation of Rel.15 DMRS ports. - Dedicated UE capability is introduced. - The signaling is at least by RRC (FFS: whether to support DCI based signaling). Conclusion MU b/w R15 DMRS ports and R18 DMRS ports - For MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports, - 2) For PDSCH, there is no additional restriction between Rel.18 UE1 indicated with Rel-18 Legacy ports (eType1: ports 1000-1007, eType2: ports 1000-1011) and Rel.15-18 UE2 indicated with Rel.15 DMRS ports in a CDM group from Rel.17 spec. - Note1: MU-MIMO restriction in Rel.15-17 is applied to Rel.15-17 UE and Rel-18 UE configured with Rel-15 DMRS port(s) - Note2: MU-MIMO restriction in Rel.18 is applied to Rel.18 UE configured with Rel-18 DMRS port(s)

Since we have not enough time, and the companies intention are the same (i.e. not support the both cases), I’d like to suggest the following proposal and conclude all remaining issue in sect.2.4.

FL Proposal 2.4C for conclusion

- There is no consensus to support the following cases for MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports:

  - 3) For PDSCH, between Rel.18 UE1 indicated with Rel-18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.15 UE2 indicated with Rel.15 DMRS ports in a CDM group.

  - 4) For PDSCH, between Rel.18 UE1 indicated with Rel-18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.18 UE2 indicated with Rel.15 DMRS ports in a CDM group.

FL Proposal 2.4D for conclusion

- The following MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports is not supported:

  - 3) For PDSCH, between Rel.18 UE1 configured with Rel-18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.15 UE2 configured with Rel.15 DMRS ports in a CDM group.

  - 4) For PDSCH, between Rel.18 UE1 configured with Rel-18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.18 UE2 configured with Rel.15 DMRS ports in a CDM group.

  - Note: The above has no spec. impact.

FL Proposal 2.4E: Offline consensus

- The following MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports are not supported:

  - 3) For PDSCH, between Rel.18 UE1 indicated with Rel-18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.15-17 UE2 indicated with Rel.15 DMRS ports in a CDM group.

    - UE does not expect such MU-MIMO within a CDM group

  - FFS: 4) For PDSCH, between Rel.18 UE1 indicated with Rel-18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.18 UE2 indicated with Rel.15 DMRS ports in a CDM group.

    - UE does not expect such MU-MIMO within a CDM group

Please provide your views.

| Company | Comment |
| --- | --- |
| MediaTek | Support in principle, however, we want to clarify that no consensus in this case mean not supported. We can’t have ambiguous behaviour where gNB may or may not use the same CDM group for case 3 and 4. |
| OPPO | We have similar view with MediaTek. If no consensus means not support, we are fine. |
| QC | Same comment as MTK. We prefer to capture this in a way similar to agreements made in last meeting such as “UE is not expect …” |
| Lenovo | We are fine with clear statement with “Not support” for clarification. |
| Fraunhofer IIS/HHI | Fine with not supporting the above MU-MIMO cases. |
| Apple | Similar view as MTK and QC |
| LGE | Support in principle. |
| FL | I added P2.4D/2.4E. However, these proposals were discussed in the last meeting. Vivo, ZTE, and some companies has concern to P2.4E, because gNB can MU with different scrambling ID, TCI state, etc. Such king of non-orthogonal MU should be not restricted. So, I tried to add “if DMRS scrambling IDs are the same between UEs” to P2.4E. However, multiple companies were not ok with it. My suggestion is to take P2.4D as compromise of both side, and close this discussion. If it is not agreeable, we can discuss it in the next meeting, and if the situation is not changed in the next meeting, I will propose P2.4C. If it is also not agreed, we cannot agree anything (it will be equivalent as P2.4C). |
| Ericsson | For 4) we are fine to have some clarification for Rel-18 UE, for 3) we don’t think this is needed for Rel-15 UE. |
| Xiaomi | Fine with P2.4D. |
| vivo | P2.4D is a good compromise, we are fine with it. |

  - MU-MIMO scheduling restriction within a CDM group

In section 5.1.6 in TS38.214, MU-MIMO scheduling restriction is specified as following.

For DM-RS configuration type 1, - if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 9, 10, 11 or 30} in Table 7.3.1.2.2-1 and Table 7.3.1.2.2-2 of Clause 7.3.1.2 of [5, TS 38.212], or - if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 9, 10, 11 or 12} in Table 7.3.1.2.2-1A and {2, 9, 10, 11, 30 or 31} in Table 7.3.1.2.2-2A of Clause 7.3.1.2 of [5, TS 38.212], or - if a UE is scheduled with two codewords, the UE may assume that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE. For DM-RS configuration type 2, - if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 10 or 23} in Table 7.3.1.2.2-3 and Table 7.3.1.2.2-4 of Clause 7.3.1.2 of [5, TS38.212], or - if a UE is scheduled with one codeword and assigned with the antenna port mapping with indices of {2, 10, 23 or 24} in Table 7.3.1.2.2-3A and {2, 10, 23 or 58} in Table 7.3.1.2.2-4A of Clause 7.3.1.2 of [5, TS 38.212], or - if a UE is scheduled with two codewords, the UE may assume that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE.

In RAN1#112bis-e, the following proposal was discussed. However, the most of companies were not ok with it, especially it is more severe MU-restriction from Rel.15 (e.g. for Type2).

FL Proposal 2.6A - Adopt the following MU scheduling restriction for Rel.18 DMRS ports for PDSCH: - If the DMRS ports of a UE are in more than one CDM groups, the UE does not expect DMRS ports from a co-scheduled UE in a same CDM group as the UE. - The above applies to both single symbol and dual symbol DMRS. - Furthermore, for dual symbol DMRS, if the DMRS ports of a UE are associated with more than one TD-OCC codes in one CDM group, the UE does not expect DMRS ports from a co-scheduled UE in a same CDM group as the UE.

Since this is not urgent issue, one option is to postpone the discussion until when DMRS ports combinations are agreed. Another option is to proceed the discussion at least some part. One way forward is to discuss MU-restriction at least for the 1st sub-bullet for eType1 only, because the restriction in the 1st bullet in FL proposal 2.6A is the same MU-restriction in Type1. However, the 2nd sub-bullet seems new restriction from Rel.15 (e.g. row 24-27 for Type1, maxLength=2 does not have MU-restriction in Rel.15-17). Please provide your views whether 1) and 2) are acceptable.

FL Proposal 2.5A (For eType1)

- Adopt the following MU scheduling restriction for Rel.18 eType1 DMRS ports for PDSCH,

  - 1) If the DMRS ports of a UE are in more than one CDM groups, the UE does not expect DMRS ports from a co-scheduled UE in a same CDM group as the UE.

    - The above applies to both single symbol and dual symbol DMRS.

  - 2) Furthermore, for dual symbol DMRS, if the DMRS ports of a UE are associated with more than one TD-OCC codes in one CDM group, the UE does not expect DMRS ports from a co-scheduled UE in a same CDM group as the UE.

  - FFS: MU-restriction for eType2.

Two CWs

Another thing is MU restriction for 2CWs. In Rel.15, if UE is scheduled with two CWs, “orthogonal DMRS ports” are not used to another UE. Whether to reuse this principle can be also discussed. Note that text of “orthogonal DMRS ports” looks to include even FDM in different CDM groups, however, in Rel.15-17, there was no rows for DMRS ports with 2CWs to be FDMed due to the limited number of DMRS ports. Based on the previous discussion, it seems no companies has concern to support MU in different CDM group. Note that MU in different CDM group is allowed even between Rel.15 UE and Rel.18 UE.

Agreement (RAN1#110) Support MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports. - For MU-MIMO by different CDM groups, no MU-MIMO scheduling restriction of PUSCH/PDSCH (i.e. MU-MIMO between Rel.15 UE and Rel.18 UE is allowed). - For MU-MIMO within a CDM group, study whether and how to support MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports for PDSCH. - Note: the study includes MU-MIMO between Rel.15 UE and Rel.18 UE, and between Rel.18 Ues. - Note: PUSCH above is CP-OFDM waveform.

Following options can be considered. From FL perspective, Alt.1 should be a baseline for the discussion. Alt.2 may not be helpful, but it means no MU-restriction in the CDM group for double symbol DMRS in one CDM group. The reason is that multiple companies mention issue of MU for >1 CDM groups in their tdocs. Alt.3 seems too ambitious, but, let’s hear feedbacks.

FL Proposal 2.5B

- If UE is scheduled PDSCH with 2CWs for Rel.18 eType1/eType2 DMRS ports with maxLength=1/2,

  - For MU-MIMO within the same CDM group(s),

    - Alt.1: UE does not expect to be multiplexed with other DMRS ports in the same CDM group.

    - Alt.2: UE does not expect to be multiplexed with other DMRS ports in the same CDM group, if UE is indicated with DMRS ports on more than one CDM groups.

      - For UE with DMRS ports in one CDM group, UE can be multiplexed with other DMRS ports in the different CDM group(s).

    - Alt.3: UE can be multiplexed with other DMRS ports in the different CDM group(s).

  - For MU-MIMO by the different CDM group(s),

    - Alt.A1: UE can be multiplexed with other DMRS ports in the different CDM group(s).

    - Alt.A2: UE does not expect to be multiplexed with other DMRS ports in the same or different CDM group.

Please provide your views.

| Company | Comment |
| --- | --- |
| Docomo | Proposal 2.5A: 1) is ok. 2) is additional restriction from R15, and we prefer to no have it. Proposal 2.5B: Support. We are ok with Alt.1 which is same as Rel.15. |
| OPPO | We think for a UE scheduled with 2CWs, MU-MIMO multiplexing is a corner case. Hence, we propose to reuse the restriction in Rel-15 without mentioning MU-MIMO, e.g. - If UE is scheduled PDSCH with 2CWs for Rel.18 eType1/eType2 DMRS ports with maxLength=1/2, UE does not expect to be multiplexed with other DMRS ports in the same CDM group. |
| Huawei, HiSilicon | Proposal 2.5A: Support to postpone the discussion until DMRS ports combinations are decided. Proposal 2.5B: Open to discuss. By the way, the current formulation of this proposal is a little bit confusing for us (e.g., why ‘multiplexing with other DMRS ports in the different CDM group(s)’ in the sub-sub-sub-bullet is also included in the first sub-bullet, which literally says ‘MU-MIMO within the same CDM group(s)’?). |
| Google | We failed to see the need for both proposals. |
| Samsung | For Proposal 2.5A: we prefer to have legacy MU restriction only. For Proposal 2.5B: we are open to discuss, but we have similar view with OPPO, i.e., the case would be rare when UE is scheduled with 2CW and multiplexed with other UE by MU-MIMO. |
| ZTE | Proposal 2.5A: Do NOT support. As per our very detailed elaborations in the last meeting, we do believe it is sufficient to reuse Rel-15 rule with respective to MU-MIMO restriction. Proposal 2.5B: Support with Alt 1. |
| Lenovo | Proposal 2.5A: We prefer to have legacy MU restriction only. Proposal 2.5B: We are open for discussion but prefer not to introduce additional MU-MIMO scheduling restriction for Rel.18 eType1/eType2 DMRS. |
| New H3C | Open to discuss both proposal |
| Futurewei | We are open to discuss both proposals. |
| QC | Proposal 2.5A: We understand 2) is additional restriction from Rel-15. But 2) is needed due to Rel-18 doubled # DMRS ports in a CDM group, which makes UE incapable to handle DMRS ports across two TD-OCC. From technical perspective, restriction 1) and 2) are due to exactly the same reason. If we agree on 1), we don’t see any technical reason not agreeing on 2). Similarly for type 2 DMRS, the same restriction should be applied. We don’t see why the technical issue on UE side disappear with type 2 DMRS. Proposal 2.5B: We have a concern on the proposal to support MU-MIMO by different CDM groups for 2 CWs PDSCH. Given 2CWs PDSCH decoding requires high SNR, I think we should be careful to introduce MU-MIMO for 2 CW, unless careful study is done to show MU interference does not impact 2 CW PDSCH decoding performance. The impact of MU on 2 CWs PDSCH might be on two aspects. - Although the DMRS of MU in other CDM group is orthogonalized to target UE, but the interference from MU on data cannot be avoided. Given 2 CWs PDSCH data needs relatively high SNR, interference from MU on data might push SINR on data lower than required SNR for 2 CWs PDSCH decoding. - Due to UE’s capability, it is unlikely a UE can estimate more than 8 DMRS ports. Then for MU DMRS ports in other CDM group, the interference covariance matrix estimation can only be Ryy based. Given Ryy estimation cannot explore channel estimation/combining gain, it’s performance will be worse than Rhh based interference covariance estimation. With the above, we don’t agree with the proposal. We prefer follow Rel-15 spec, which is not allowing MU for 2 CW PDSCH, regardless on same and different CDM groups. |
| CMCC | Proposal 2.5A: Support legacy restriction only. Do not support “2)”. Proposal 2.5B: Prefer Alt.1 which is same as Rel.15. |
| Xiaomi | FL Proposal 2.5A: OK FL Proposal 2.5B: We are not sure whether 2CWs will be supported in MU-MIMO yet. |
| Vivo | Both proposals are not needed now. We think such MU-MIMO restriction can be specified one by one after the DMRS ports combinations are agreed, like what in the current TS38.214, since we have agreed many rows can’t be used for MU-MIMO in the previous agreements. |
| Nokia/NSB | Proposal 2.5A: similar view with Samsung and other companies. We are OK with 1), but not support for 2) TD-OCC restriction. Proposal 2.5B: fine with Alt 1. Supporting MU-MIMO for 2CW is out of scope in Rel-18. |
| Appl | Proposal 2.5A: Support Proposal 2.5B: Support Alt 1 that is same as legacy behavior |
| Fraunhofer IIS/HHI | Proposal 2.5A: Support Proposal 2.5B: Prefer not to support MU-MIMO with 2 CWs. Support Alt. 1 for first sub-bullet. |
| CATT | Proposal 2.5A: Not support. The number of orthogonal ports in one CDM group is doubled in Rel.18 and the restriction is not needed. Proposal 2.5B: Open to discuss. Legacy rule (Alt.1) is slightly preferred. |
| LGE | For Proposal 2.5A: We also prefer to have legacy MU restriction only. For Proposal 2.5B: Support Alt 1. |
| MediaTek | 2.5A: Support 2.5B: support Alt. 1 |

- ROUND-2

For P2.5A, please check companies’ comments, and keep discussion.

For P2.5B, it seems all companies are ok with Alt.1. For 2CWs, we also need to discuss whether FDM (Alt.A1) is allowed. For supporter of Alt.A1, please check QC/OPPO/SS’s comment. As compromise, please check the following proposal.

FL Proposal 2.5B(updated)

- If UE is scheduled PDSCH with 2CWs for Rel.18 eType1/eType2 DMRS ports with maxLength=1/2,

  - 1) For MU-MIMO within the same CDM group(s),

    - Alt.1: UE does not expect to be multiplexed with other DMRS ports in the same CDM group.

    - Alt.2: UE does not expect to be multiplexed with other DMRS ports in the same CDM group, if UE is indicated with DMRS ports on more than one CDM groups.

      - For UE with DMRS ports in one CDM group, UE can be multiplexed with other DMRS ports in the different CDM group(s).

    - Alt.3: UE can be multiplexed with other DMRS ports in the different CDM group(s).

  - 2) For MU-MIMO by the different CDM group(s), support Alt.A2 as default UE capability of 2CW supporting Rel.18 DMRS ports. Support Alt.A1 as optional capability.

    - Alt.A1: UE can be multiplexed with other DMRS ports in the different CDM group(s).

    - Alt.A2: UE does not expect to be multiplexed with other DMRS ports in the same or different CDM group.

FL Proposal 2.5C (alternative)

- If UE is scheduled PDSCH with 2CWs for Rel.18 eType1/eType2 DMRS ports with maxLength=1/2,

  - the UE may assume that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE.

Please provide your views.

| Company | Comment |
| --- | --- |
| OPPO | Not support. We propose to reuse the restriction in Rel-15 for Rel-18 DMRS, which is: - If a UE is scheduled with two codewords, the UE may assume that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE. We don’t know why MU-MIMO for a UE with two CWs, which was not supported in Rel-15, can be supported in Rel-18 (with CJT transmission and Rel-18 DMRS). |
| Lenovo | We prefer not to support this proposal since we think MU-MIMO for 2CW is out of scope in Rel-18. |
| FL | I added P2.5C as alternative. It is the same as legacy. |
| Ericsson | Support 2.5B. |
| OPPO | Support 2.5C. The wording in current spec. seems better but we are generally fine. |
| QC | Support 2.5C which follows Rel-15 legacy. |
| FL | I used the spec. wording to P2.5C. |

- ROUND-3

On Wednesday online, the following proposal was discussed.

FL Proposal 2.5C (alternative)

- If UE is scheduled PDSCH with 2CWs for Rel.18 eType1/eType2 DMRS ports with maxLength=1/2,

  - the UE may assume that all the remaining orthogonal antenna ports are not associated with transmission of PDSCH to another UE.

FL Proposal 2.5D (altanative2)

- If UE is scheduled PDSCH with 2CWs for Rel.18 eType1/eType2 DMRS ports with maxLength=1/2,

  - Alt.1: UE does not expect to be multiplexed with other DMRS ports in the same CDM group.

  - FFS: multiplexing with other DMRS ports in the different CDM group

FL Proposal 2.5E (altanative3)

- If UE is scheduled PDSCH with 2CWs for Rel.18 eType1/eType2 DMRS ports with maxLength=1/2,

  - Alt.1: UE does not expect to be multiplexed with other DMRS ports in the same CDM group, if the other DMRS ports are 2 CWs.

  - FFS: multiplexing with other DMRS ports in the different CDM group

  - FFS: multiplexing with other DMRS ports with 1 CW in the same CDM group

Since we didn’t discuss it before, please add your name in the table.

|  | Support/fine | Concern |
| --- | --- | --- |
| P2.5C | Lenovo, OPPO, QC, Docomo, MTK, Fraunhofer IIS/HHI, Apple, Sharp | ZTE |
| P2.5D | Ericsson, Docomo |  |

Please provide your views, if any.

| Company | Comment |
| --- | --- |
| MediaTek | We support P2.5C |
| OPPO | Support P2.5C. The Rel-18 DMRS enhancement is motivated by CJT scenarios with more multiplexing UEs. We don’t think MU-MIMO with two CWs can be supported in this case. |
| QC | We support Proposal 2.5C. |
| Lenovo | We support P2.5C |
| Fraunhofer IIS/HHI | Support P2.5C |
| Apple | Support P2.5C |
| LGE | Support P2.5C |
| Ericsson | Support 2.5D. There’s no need for such restriction as in 2.5C. 2 CW varies from rank5 to rank 8, with 2 CW for one UE in one CDM group, it should be possible to schedule another UE using a second CDM group. Network can take care of the paring of UEs in MU scenario. |
| Sharp | We prefer Proposal 2.5C. We think 2CW is for SU-MIMO scenario. |
| Xiaomi | Support P2.5C |
| FL | I added P2.5E per ZTE’s request. It becomes very small step, but better than nothing. |
| vivo | Support P2.5C |

  - (void)

  - Other proposals

Following proposals are also proposed.

| Proposals | Companies |
| --- | --- |
| - Study on OCC disabling scheme for new DMRS type (Rel.17 feature in above 52.6GHz). | Samsung |
| - Precluding simultaneous configuration of PDSCH processing capability 2, More than 1 additional DMRS symbol, 8Rx, or single DCI based NCJT, with Rel.18 DMRS ports. | Huawei/HiSilicon |
| - Additional scheduling restriction of orphan RE issue for eType1 | Vivo, CATT, Lenovo, Google, Samsung |
| - Scheduling restriction to avoid odd number of PRBs for eType1 DMRS is extended to eType2 DMRS | Qualcomm, Apple |
| - Scheduling restrictions of PDSCH among MU-MIMO Ues | QC |
| - PTRS power boosting for PDSCH with Rel-18 DMRS ports | Lenovo |

Please provide your views on the above proposals, or other aspects which are not included in the summary, if any.

| Company | Comment |
| --- | --- |
| Docomo | For 2), we have concern to preclude joint configuration with existing Rel.15-17 features. Especially, 1 additional DMRS symbol is widely used in the current NW, and if we preclude it, we lose the use-case scenario of Rel.18 DMRS. |
| Huawei, HiSilicon | Regarding 2), @Docomo, actually what we want to preclude is “more than 1 additional DMRS symbol”. For the combination of Rel.18 DMRS and each of the aforementioned feature, the detailed reason is given in our contribution. We sincerely recommend companies to take this into consideration. Regarding 3), open to discuss. |
| Docomo2 | Re Huawei for 2), sorry for our confusion. We see the intention. |
| Lenovo | With the introduced large DMRS ports, we think the following table from TS 38.214 shall be extended to up to eight layer PDSCH transmission. Table 4.1-2: PT-RS EPRE to PDSCH EPRE per layer per RE () epre-Ratio; The number of PDSCH layers with DM-RS associated to the PT-RS port. 1; 2; 3; 4; 5; 6. 0; 0; 3; 4.77; 6; 7; 7.78. 1; 0; 0; 0; 0; 0; 0. 2; reserved. 3; reserved. |

- Specifying objective #5 (>4 layers PUSCH DMRS)

  - Separate DMRS port table for PUSCH for CB

In RAN1#112bis-e, following agreement was made. In this meeting, multiple companies propose to confirm the WA for CB based PUSCH. However, joint TRI and TPMI indication is under discussion in AI9.1.4.2, hence it is reasonable to wait the outcome of the discussion.

Agreement Confirm the following Working Assumption in RAN1#112 at least for NCB based PUSCH: - To support PUSCH with rank = 5-8, support the following for enhancement of DMRS port allocation tables. - Option 1: Separate DMRS ports tables for rank 5,6,7,8 for each of eType1/eType2 and maxLength=1/2 (similar to the current UL DMRS ports table). - FFS: whether/how to reuse the reserved field in antenna ports field for other purposes can be discussed in AI9.1.4.2 [or AI9.1.3.1]. - Note: The above Working Assumption for CB based PUSCH may be confirmed later.

  - DMRS ports combo for partial coherent PUSCH

This issue has been discussed in multiple meetings. In RAN1#112bis-e, some companies mentioned this issue is related to the outcome of CW to antenna port group mapping in AI 9.1.4.2 and we should wait to discuss in this agenda. However, considering only 2 meetings are left, we should make a decision. Please check the benefit and detail of Alt.1 from Nokia/NSB.

Nokia/NSB:

For partial coherent codebook, it is beneficial to map the coherent antenna port group into the same CDM group. This is already supported for multi-TRP DM-RS port mapping. For supporting non-coherent joint transmission from two TRPs, the DMRS ports from the different TRPs are mapped to different CDM group. Thus, we propose to apply the same principle for UL partial coherent codebook. - For Ng=2, UE has two 4-TX antenna groups, and layers associated with 4 TX from an antenna group can be mapped to a single CDM group. - For DRMS type 1, layers associated with antenna group 0 are mapped to CDM group 0, and layers associated with antenna group 1 are mapped to CDM group 1. - For DMRS type 2, - if 2 CDM groups are supported, same rule as DMRS type 1 is supported - if 3 CDM groups are supported, (rank=5 or 6, single symbol), strive to minimize mixed mapping. - For Ng=4, UE has four 2-TX antenna groups, and layers associated with 2 TX from an antenna group can be mapped to a single CDM group. - For DRMS type 1, layers associated with antenna group 0,2 are mapped to CDM group 0, and layers associated with antenna group 1,3 are mapped to CDM group 1. - For DMRS type 2, - if 2 CDM groups are supported, same rule as DMRS type 1 is supported - if 3 CDM groups are supported, (rank=5 or 6, single symbol), - when layers are generated from 3 antenna groups, map layers to 3 CDM groups in order. - When layers are generated from 4 antenna groups, map layers into antenna ports in order. With this rule, for rank=5,6,7,8, signaling of DMRS port mapping can be skipped or just 1 bit is required.

However, if the situation is not changed, my suggestion will to take Alt.2, which means the same table is used for full/partial/non-coherent UL codebook. Note that it does not preclude future possibility to add other rows (which can be used for partial coherent codebook).

FL Proposal 3.2

- For > 4 layers PUSCH with Rel.15 Type1/Type2 DMRS ports and Rel.18 eType 1/eType 2 DMRS ports, for partial coherent UL codebook, support Alt.2:

  - Alt.1: DMRS ports combination(s) that the same antenna group into the same DMRS CDM group.

  - Alt.2: DMRS ports combination(s) for full/non-coherent UL codebook is reused.

    - The same DMRS port tables are used for full/partial/non-coherent UL codebook.

Alt.1: IDC, Nokia/NSB

Alt.2: Docomo, OPPO, HW/Hi, Google (WA), SS, ZTE, Lenovo, NewH3C, QC, CMCC, Xiaomi, vivo, Apple, LGE, Spreadtrum

| Company | Comment |
| --- | --- |
| Docomo | Proposal 3.2: Support Alt.2. We are fine to add a few rows for partial coherent TPMIs, if needed. |
| OPPO | Support Alt.2. Layers to DMRS port mapping that layers associated to the same antenna port group are multiplexed into the same DMRS CDM group can be achieved via appropriate layer splitting in 9.1.4.2. |
| Huawei, HiSilicon | Support Alt.2. |
| Google | We think we can take Alt2 as the working assumption. |
| Samsung | Support Alt2. |
| ZTE | Support Alt 2. |
| Lenovo | Support Alt 2. |
| New H3C | Fine with Alt.2 |
| InterDigital | Support Alt.1 and do not support Alt2. If we go with Alt2., there are additional complexities related to mapping of the layers to antenna group that will arise. To keep it simple and straightforward, Alt1 should be used. The decision on this should be left to 9.1.4.2. |
| QC | Support Alt 2 which is a unified design for full/partial/non-coherent codebooks. |
| CMCCf | Support Alt2. In Rel-15, same DMRS port combinations and tables are used for full or partial-coherent or non-coherent codebooks. For example, for dmrs-Type=1, maxLength=2, and rank=3 UL transmission, the DMRS port combination {0,1,2} can be indicated to UE, where DMRS ports 0 and 1 are within one CDM group, and DMRS port 2 is in another CDM group. The second and third layers associated to the same antenna port group are multiplexed into different DMRS CDM group, and none of restriction is added to DMRS port combination(s) for partial coherent UL codebook. From our perspective, the restriction of DMRS port combination(s) for partial coherent UL codebook is also not needed for 8 TX UL transmission as well, and a unified design is used for full, partial, and non-coherent UL codebook. |
| Xiaomi | Support Alt.2. |
| vivo | Support Alt 2. |
| Nokia/NSB | Support Alt 1. This is natural way between non-coherent ports. |
| Apple | Alt 2 |
| LGE | Support Alt 2. |
| Spreadtrum | Support Alt2. |

- ROUND-2

There is no change for P3.2. Based on offline discussion with IDC, IDC thinks different antenna coherent group should be mapped to one CDM group (e.g. If 2+3 rank combinations are supported for rank5, 2 DMRS ports should be mapped to the 1st CDM group and 3 DMRS ports should be mapped to the 2nd CDM group). Most of companies are ok with Alt.2. For proponents of Alt.2, please provide your views on this part.

FL Proposal 3.2 (for working assumption): Offline consensus

- For > 4 layers PUSCH with Rel.15 Type1/Type2 DMRS ports and Rel.18 eType 1/eType 2 DMRS ports, for partial coherent UL codebook, support Alt.2:

  - Alt.1: DMRS ports combination(s) that the same antenna group into the same DMRS CDM group.

  - Alt.2: DMRS ports combination(s) for full/non-coherent UL codebook is reused.

    - The same DMRS port tables are used for full/partial/non-coherent UL codebook.

Alt.1: IDC, Nokia/NSB

Alt.2: Docomo, OPPO, HW/Hi, Google (WA), SS, ZTE, Lenovo, NewH3C, QC, CMCC, Xiaomi, vivo, Apple, LGE, Spreadtrum

Please provide your views.

| Company | Comment |
| --- | --- |
| OPPO | Support. |
| Lenovo | Support the updated proposal |
| FL | I changed the proposal as working assumption. I discussed with AI9.1.4.2 FL, and his concern is that the above agreement may impact on precoder discussion in AI9.1.4.2. One possibility is to make it as working assumption, and the other way is to make agreement but adding the note “Note: the above agreement does not impact to the discussion on AI9.1.4.2”. My recommendation is to make it as working assumption for now. |

  - Antenna port(s) table for PUSCH (rank 5-8) for R18 DMRS

In RAN1#112bis-e, the following agreement was made. Additional rows can be discussed for PUSCH.

Agreement For Rel.18 eType1/eType2 DMRS ports with maxLength=1/2 for PDSCH/PUSCH, if Rel.18 eType1/eType2 DMRS ports is configured by RRC, the DCI size of antenna ports field in DCI format 1_1/1_2/0_1/0_2 is increased by at least 1-bit from Rel.17. - Note: it does not preclude future possibility to support more than 1-bit, if RAN1 agree the necessity.

eType1, maxLength1 (rank 5-8)

Per the above agreement in RAN1#112bis-e, rows agreed for PDSCH are automatically agreed for PUSCH. Other than such combinations, if additional combinations are needed for PUSCH, please input in the following table.

| Company | Comment |
| --- | --- |
| Ericsson | We propose to add the yellow highlighted rows for PUSCH. Note that the combinations used more new ports would enable co-scheduling of this UE with legacy PUSCH ports. Table 7.3.1.1.2-xx: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=1, rank = 5 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 2; 0,1,2,3,8. 1; 2; 3,8,9,10,11 or [8,9,3,10,11]. 2; 2; 2,3,9,10,11. Table 7.3.1.1.2-xx: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=1, rank = 6 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 2; 0,1,2,3,8,9. 1; 2; 2,3,8,9,10,11. 2; 2; 1,3,8,9,10,11. Table 7.3.1.1.2-xx: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=1, rank = 7 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 2; 0,1,2,3,8,9,10. 1; 2; 1,2,3,8,9,10,11 or [1,8,9,2,3,10,11]. Table 7.3.1.1.2-xx: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=1, rank = 8 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 2; 0,1,2,3,8,9,10,11 or [0,1,8,9,2,3,10,11]. |

eType1, maxLength2 (rank 5-8)

Per the above agreement in RAN1#112bis-e, rows agreed for PDSCH are automatically agreed for PUSCH. Other than such combinations, if additional combinations are needed for PUSCH, please input in the following table.

| Company | Comment |
| --- | --- |
| Ericsson | We propose to add the yellow highlighted rows for PUSCH. Note that the combinations used more new ports would enable co-scheduling of this UE with legacy PUSCH ports. Table 7.3.1.1.2-xx: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 5 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 2; 0,1,2,3,8; 1. 1; 2; 3,8,9,10,11 or [8,9,3,10,11]; 1. 2; 2; 2,3,9,10,11; 1. 3; 1; 0,1,4,5,8; 2. 4; 2; 0,1,4,5,8; 2. 5; 2; 2,3,6,7,10; 2. Table 7.3.1.1.2-xx: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 6 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 2; 0,1,2,3,8,9; 1. 1; 2; 2,3,8,9,10,11; 1. 2; 2; 1,3,8,9,10,11; 1. 3; 1; 0,1,4,5,8,9; 2. 4; 2; 0,1,4,5,8,9; 2. 5; 2; 2,3,6,7,10,11; 2. Table 7.3.1.1.2-xx: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 7 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 2; 0,1,2,3,8,9,10; 1. 1; 2; 1,2,3,8,9,10,11 or [1,8,9,2,3,10,11]; 1. 2; 2; 0,1,4,5,8,9,12; 2. 3; 1; 0,1,4,5,8,9,12; 2. 4; 2; 2,3,6,7,10,11; 2. 5; 2; 9,10,11,12,13,14,15; 2. Table 7.3.1.1.2-xx: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 8 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 2; 0,1,2,3,8,9,10,11 or [0,1,8,9,2,3,10,11]; 1. 1; 2; 8,9,10,11,12,13,14,15; 2. 2; 2; 0,1,4,5,8,9,12,13; 2. 3; 1; 0,1,4,5,8,9,12,13; 2. 4; 2; 2,3,6,7,10,11,14,15; 2. |

eType2, maxLength1 (rank 5-8)

Per the above agreement in RAN1#112bis-e, rows agreed for PDSCH are automatically agreed for PUSCH. Other than such combinations, if additional combinations are needed for PUSCH, please input in the following table.

| Company | Comment |
| --- | --- |

eType2, maxLength2 (rank 5-8)

Per the above agreement in RAN1#112bis-e, rows agreed for PDSCH are automatically agreed for PUSCH. Other than such combinations, if additional combinations are needed for PUSCH, please input in the following table.

| Company | Comment |
| --- | --- |

  - Max number of PTRS ports

We discussed the following proposal since RAN1#110bis-e, however, there was no consensus.

FL proposal#3.2A: - For 8Tx PUSCH, support up to 4 ports PTRS for CP-OFDM. Support/fine: InterDigital, CATT, Lenovo, Apple, Qualcomm, Xiaomi, LGE Docomo, MediaTek, CMCC, Ericsson No: Samsung, OPPO, Nokia/NSB, vivo, Spreadtrum, If the above proposal is not acceptable, following will be the consequence. It means 2-port PTRS in the current spec.is reused. FL proposal#3.2B: (for conclusion) - For 8Tx PUSCH, no consensus to support up to 4 ports PTRS for CP-OFDM. Support/fine: Google, ZTE, Spreadtrum, OPPO, vivo, Samsung, Nokia/NSB, Docomo,…

In RAN1#112bis-e, the following comments are provided. From FL perspective, the note added by QC to Proposal 3.2B is not acceptable. Such kind of discussion can be done in AI9.1.4.2.

| Company | Comment (RAN1#112bis-e) |
| --- | --- |
| Docomo | Support FL Proposal 3.2A, because Ng (antenna coherent groups) was agreed with 1, 2, 4. If different antenna groups do not share the same PA, different phase noise would be observed for different antenna groups. Hence, each DMRS port(s) should be associated with one PTRS port, and the total number of PTRS ports should be up to 4. But, considering the situation, and 8 PTRS ports will be not used for 8Tx non-coherent codebook, we can accept FL proposal#3.2B. |
| Google | Support 3.2B. Ng does not mean number of panels. Currently only 2 panels are supported for STxMP. |
| InterDigital | Support Proposal 3.2A. To properly support Ng=4, that may represent antenna units pointed to four different directions, 4 PTRS ports should be supported. |
| OPPO | Support Proposal 3.2B. We don’t think we need N PTRS ports for N antenna groups. |
| Nokia/NSB | Support the proposal. 4 PTRS require high overhead but no clear gain. (we can come back if we support simultaneous TX to 4 TRPs in FR2) |
| Lenovo | Support FL Proposal 3.2A. |
| QC | We support FL Proposal 3.2A, based on the reason as DOCOMO mentioned. And we don’t think it is feasible for antenna groups to share a same PA/Oscillator. We don’t agree with FL Proposal 3.2B, which effectively excluding Ng=4 for 8 Tx in Rel-18. If we have to take the conclusion due to controversial views, we request to add a note to the conclusion. Updated FL proposal#3.2B: (for conclusion) - For 8Tx PUSCH, no consensus to support up to 4 ports PTRS for CP-OFDM. Note: This conclusion effectively excludes the support of Ng=4 for 8 Tx PUSCH with the use of PTRS. |
| MediaTek | Proposal 3.2A: Support |
| ZTE | Support FL’s proposal#3.2B. Note that 8Tx UL aims for PUSCH transmission in FR1, we fail to see the motivation of supporting 4 PTRS ports in Rel-18. Subsequently, we do not agree the Note suggested by QC. |
| Ericsson | Support proposal #3.2A. |
| Sharp | Support FL proposal 3.2B. |
| Xiaomi | Support FL Proposal 3.2A. |
| Apple | Proposal 3.2A: Support. As discussed earlier, if we support up to 8 Tx UL partial-coherent operation over 4 port groups (panels) with up to 8 layers, we need 4 port PTRS |
| New H3C | Support FL proposal 3.2B. |
| China Telecom | Support FL proposal 3.2B. |
| Samsung | Support FL proposal#3.2B. Supporting 4 PTRS ports may have bad effect on UL throughput which is the key motivation to have up to 8 layers. And also, up to 8-layer would be appropriate on FR1 which PTRS is optionally used. Hence, we think increasing the maximum number of PTRS ports is not needed. |
| Spreadtrum | Support proposal#3.2B. The note suggested by QC is not always true. |
| CMCC | Support FL proposal#3.2A. If four antenna groups are assumed for UE’s antenna layouts, then maximal PT-RS ports to be configured may be 4. |
| LGE | Support FL Proposal 3.2A, For 8 Tx antenna ports, there can be no coherence at all among 8 antenna ports or 4 pairs of coherent antenna ports can be considered depending on antenna implementation. In this case, 2 PT-RS ports may not be sufficient for phase noise estimation from more than two phase noise sources. |
| Vivo | Support proposal#3.2B. |

Considering the situation, and only 2 meetings are left, my suggestion is to take the following (with small update from “up to 4” to “more than 2”).

FL proposal#3.4: (for conclusion)

- For 8Tx PUSCH, no consensus to support more than 2 ports PTRS for CP-OFDM.

  - [Note: This conclusion effectively excludes the support of Ng=4 for 8 Tx PUSCH with the use of PTRS in FR2.]

| Company | Comment |
| --- | --- |
| Docomo | OK |
| OPPO | Support. |
| Huawei, HiSilicon | Fine. |
| Google | OK |
| Samsung | Support. |
| ZTE | Support. |
| Lenovo | Ok. |
| New H3C | OK in general |
| QC | It is unfortunate that no consensus can be achieved for this issue. But then isn’t the following note just a statement of a fact due to this conclusion? We are not sure why this note cannot be added to the conclusion. In our view, this note should be added to make sure the consequence of this conclusion is clear to the whole industry. “Note: This conclusion effectively excludes the support of Ng=4 for 8 Tx PUSCH with the use of PTRS.” |
| Xiaomi | Ok |
| vivo | Fine |
| Nokia/NSB | Fine. No need for QC’s note. |
| Apple | Share similar view as QC and support the note. Otherwise, we don’t see how 2 ports PTRS can support Ng=4 for 8 Tx PUSCH |
| LGE | OK |
| Sharp | Support. |
| FL | Although I think it is not proper to add the note, I added the note with [ ]. I think it is applicable to FR2 only. |
| Spreadtrum | Support. |

- ROUND-2

Based on offline discussion with QC, QC can accept the P3.4 without the note. So, the proposal should be stable.

FL proposal#3.4: (for conclusion)

- For 8Tx PUSCH, no consensus to support more than 2 ports PTRS for CP-OFDM.

  - [Note: This conclusion effectively excludes the support of Ng=4 for 8 Tx PUSCH with the use of PTRS in FR2.]

If you have concern, please provide your views:

| Company | Comment |
| --- | --- |
| OPPO | Support without the note. |
| Lenovo | Support the updated proposal. |

  - PTRS-DMRS association

Two port PTRS

For two port PTRS for partial/non-coherent PUSCH, the following agreement was made in RAN1#112bis-e.

Agreement For two PTRS ports for partial/non-coherent PUSCH, PTRS-DMRS association for PUSCH with up to 8 layers is down selected from the following. - Alt.1: The size of PTRS-DMRS association field is 4-bit in DCI format 0_1/0_2. Table 1: PTRS-DMRS association for UL PTRS ports 0 and 1 Value of MSB; DMRS port; Value of LSB; DMRS port. 0; 1st DMRS port which shares PTRS port 0; 0; 1st DMRS port which shares PTRS port 1. 1; 2nd DMRS port which shares PTRS port 0; 1; 2nd DMRS port which shares PTRS port 1. 2; 3rd DMRS port which shares PTRS port 0; 2; 3rd DMRS port which shares PTRS port 1. 3; 4th DMRS port which shares PTRS port 0; 3; 4th DMRS port which shares PTRS port 1. - Alt.2: The size of PTRS-DMRS association field is 2-bit in DCI format 0_1/0_2. - The CW with the higher MCS is selected in case of two CWs. - If the MCS is the same for two CWs, the PTRS port is associated with the first CW. Table 2: PTRS-DMRS association for UL PTRS ports 0 and 1 Value of MSB; DMRS port; Value of LSB; DMRS port. 0; 1st DMRS port which shares PTRS port 0; 0; 1st DMRS port which shares PTRS port 1. 1; 2nd DMRS port which shares PTRS port 0; 1; 2nd DMRS port which shares PTRS port 1. - Alt.3: The size of PTRS-DMRS association field is 2-bit in DCI format 0_1/0_2. - For PUSCH with rank 5-8, 2-bit of antenna ports field is reused in addition to 2-bit PTRS-DMRS association in DCI format 0_1/0_2, and total 4-bit is used for PTRS-DMRS association. Table 1: PTRS-DMRS association for UL PTRS ports 0 and 1 Value of MSB; DMRS port; Value of LSB; DMRS port. 0; 1st DMRS port which shares PTRS port 0; 0; 1st DMRS port which shares PTRS port 1. 1; 2nd DMRS port which shares PTRS port 0; 1; 2nd DMRS port which shares PTRS port 1. 2; 3rd DMRS port which shares PTRS port 0; 2; 3rd DMRS port which shares PTRS port 1. 3; 4th DMRS port which shares PTRS port 0; 3; 4th DMRS port which shares PTRS port 1. - Alt.4: The size of PTRS-DMRS association field is 2-bit in DCI format 0_1/0_2. Table 2: PTRS-DMRS association for UL PTRS ports 0 and 1 Value of MSB; DMRS port; Value of LSB; DMRS port. 0; 1st DMRS port which shares PTRS port 0; 0; 1st DMRS port which shares PTRS port 1. 1; 2nd DMRS port which shares PTRS port 0; 1; 2nd DMRS port which shares PTRS port 1.

Summary of companies’ views

|  | Support/fine | Concern |
| --- | --- | --- |
| Alt.1 | Ericsson, ZTE, Nokia/NSB, Apple, IDC(1st pref) , Spreadtrum, Xiaomi, Lenovo, CMCC, OPPO, Sharp, QC, MTK, Spreadtrum |  |
| Alt.2 | vivo, HW/Hi, IDC(2nd pref), Samsung | Nokia/NSB (It limits the PTRS transmission for a single codeword), QC |
| Alt.3 | Nokia/NSB, CATT, Docomo, LGE, Google (2nd) | QC |
| Alt.4 | Nokia/NSB, Google | QC |

Please provide your views.

| Company | Comment |
| --- | --- |
| Docomo | Support Alt.3. Alt3 has the best flexibility (same as Alt.1), and it does not increase DCI overhead. We believe Alt.3 is the best solution and we don’t see drawback of Alt.3. Our understanding of Alt.3 is the following: - Alt.3: DCI field of PTRS-DMRS association field is fixed to 2-bit. If rank 1-4 is indicated, 2-bit PTRS-DMRS association field is used by the existing spec. If rank 5-8 is indicated, both 2-bit PTRS-DMRS association field and 2-bit of reserved field in antenna ports field are jointly used to indicate PTRS-DMRS association. Based on the previous agreements, we believe there is enough reserved field in antenna ports field if rank 5-8 is indicated, and we think Alt.3 is feasible. |
| OPPO | Prefer Alt.1. Based on current DMRS design, it is very difficult that there would be always two reserved bits for rank 5-8 for all DMRS configuration. Alt.1 is a safer way. |
| Huawei, HiSilicon | In order to harvest similar overhead reduction benefit to full-coherent-PUSCH case, the DCI overhead of PTRS-DMRS association should remains 2 bit. In terms of the alternatives, we prefer to postpone the discussion after TPMI is decided in 9.1.4.2. |
| Google | We are also ok with Alt3 as a second choice. But current formulation (The Table) for Alt3 may not be accurate enough. |
| Samsung | We support Alt2 which can have same principle with FC/PC/NC & 1-port PTRS which are already agreed. We are also fine with postponing after finalizing PC/NC codebook design in AI 9.1.4.2. |
| ZTE | Support Alt 1. Notably, considering the very limited time budget in terms of one additional meeting left and also the progress of 8Tx PC/NC codebook design in AI 9.1.4.2 so far, we think it is NOT feasible to postpone this discussion herein. In other words, it is indeed need to make decision now. Our analyses of each are listed as follows: - For Alt 1, it is totally in line with the legacy rule to provide the sufficient flexibility of PTRS-DMRS association. - For Alt 2, it is mandatory to limit that two PTRS ports can only be used for one out of two CWs, which will lead to phase tracking issue of the CW without PTRS. To prove this, simulation result is provided in our tdoc (R1-2304395), it proves that the probability of the best two layers of the first two SINR associated with 2 CWs is even larger than 80%. Hence there is no any reason to restrict the two PTRS ports associated with the DMRS with one CW of higher MCS for partial and non coherent CB PUSCH. - For Alt 3, it is not friendly to forward compatible for antenna ports field. Besides, DCI field combination (e.g., field 1 + some reserved bits in field 2) is not mainstream approach in NR so far. Last, given that more than 10 bits were introduced additionally for MTRP TDM PUSCH repetition in Rel-17, we do fail to see the logic that DCI overhead saving for 2 bits is deemed necessary. - For Alt 4, it is unaware to gNB side to decide the down-selection of DMRS port number of each PTRS, which is an uncompleted solution. |
| Lenovo | Support Alt.1 |
| New H3C | Open to discuss |
| QC | Support Alt 1, which is the most straightforward solution. Alt 2 does not support the scenario where one PTRS port for CW1 and another PTRS port for CW2. With Alt 2, both PTRS ports are used for a same CW, which seems a problem/restriction. Alt 3 unnecessarily complicates the way to interpret DCI. Alt 4 restricts PTRS only can be associated to 4 out of the 8 DMRS ports. |
| CMCC | Support Alt.1. The actual number of UL PT-RS port(s) is determined based on TPMI and/or number of layers as: PUSCH antenna port 1000, 1002, 1004 and 1006 in indicated TPMI share PT-RS port 0, and PUSCH antenna port 1001, 1003, 1005 and 1007 in indicated TPMI share PT-RS port 1. |
| Xiaomi | Support Alt.1 |
| vivo | Support Alt 2. It can be postponed after the outcome of codebook design in AI 9.1.4.2. |
| Nokia/NSB | Support Alt 1 as baseline. Alt 3 can be discussed later. |
| Apple | Alt 1 |
| LGE | We prefer Alt 3. We think 4 bits are needed for flexibility on 8 port DMRS-PTRS association, Alt 1 is an extension from 2 bits to 4 bits and Alt 3 is added a 2-bit dci field for rank 5-8. And those 2 bits can be reused in the reserved field in the UL antenna port field for 2CW. |
| MediaTek | Support Alt. 1 |
| Sharp | Support Alt 1. |
| Spreadtrum | Support Alt1. |

- ROUND-2

Based on the round1 discussion, is Alt.1 acceptable for all?

If you have concern, please provide your views:

| Company | Comment |
| --- | --- |
| OPPO | Support Alt.1. |
| Lenovo | Support Alt.1. |

  - PTRS power boosting

The only issue remained from last meeting is partial coherent TPMIs for row 00. For partial coherent case, the assumption of port coherency scheme needs to be considered to determine the value of . In AI 9.1.4.2, it has been agreed that two or four coherent groups are assumed for 8 Tx partial coherent UE, and the antenna ports within one group are assumed to be full coherent.

Agreement (RAN1#112) For partially coherent uplink precoding by an 8TX UE codebook, - When Ng=2 - Precoding design is based on Rel-15 UL 4TX codebook, - Full-coherent precoders are used - FFS whether partial-coherent precoders are needed - When Ng=4, down-select from, - Alt1: - Precoding design is based on Rel-15 UL 2TX codebook, - Full-coherent precoders are used - Alt2: - Precoding design is based on Rel-15 UL 4TX codebook, - Partial-coherent precoders are used Agreement (RAN1#112bis-e) For partially coherent uplink precoding by an 8TX UE codebook, Ng=4, Alt1 is supported where - Precoding design is based on Rel-15 UL 2TX codebook, - Full-coherent precoders are used - Further study codebook size reduction

Following views are provided in their tdocs in RAN1#113.

IDC (Spreadtrum, CATT, Lenovo, QC provides similar proposal): For this case, the power cannot be borrowed between layers in different group. In this case, the UE may only support 10log⁡(Lx) dB PTRS power boost, where Lx is the number of coherently precoded PUSCH layers. In addition, UE may also boost 10log⁡(Qpx) dB power from the same PUSCH layer, where Qpx is the number of PTRS ports associated to each DMRS group. Therefore, PTRS power boost ratio can be 10 log⁡(Lx)+10 log⁡(Qpx) . However, this ratio cannot exceed the power in the full coherent case, i.e.,  10logL dB. In view of this, the PTRS power boost ratio for partial coherent codebook based may be determined by min10 log⁡(Lx) +10 log⁡(Qpx) , 10logL.

Nokia/NSB: Each group is using fully coherent TPMI only, which means it is possible to combine all power of the layers  from the same antenna group. So, it can boost power according to the number of the layers associated with the PTRS ports (Lp). In addition, when the number of PTRS ports are two, 3dB can be boosted by barrowing muted REs for the other PTRS ports.

So, it is proposed that, the factor (αPTRSPUSCH) is 10log10QpLp

OPPO: For partial-coherent TPMI when UL-PTRS-power=00, the situation that power cannot be borrowed among antenna groups should be considered. In this case, power boosting should only consider the layers/PTRS ports transmitted within the same antenna group, and the number of transmitted PTRS ports which needs rate matching. Then the power boosting factor for a PTRS port can be expressed as

= 10Lg(L)+10lg(Qp)= 10Lg(L* Qp)

where L is the number of layers transmitted in the same antenna group as the DMRS port associated with the PTRS port, and Qp is the total number of actually transmitted PTRS ports, which can be 1 or 2.

Google: For the coherent and non-coherent TPMIs, the agreement above follows the above 3 principles well, and the power boosting upper bound is 9dB. For partial-coherent TPMIs, the same principle should be maintained.

- Principle 1: The transmission power for each RF chain is consistent

- Principle 2: The total transmission power across all the RF chains is consistent

- Principle 3: The EPRE ratio between PT-RS and PUSCH should not exceed 9dB

FL proposal#3.6:

- For 8Tx PUSCH, when the ptrs-Power configures 00, the factor () for partial coherent TPMIs is down selected from the following:

- Alt.1: min10 log⁡(Lx) +10 log⁡(Qp) , 10logL, where Lx is the number of PUSCH layers which are precoded coherently with the PUSCH layer where PTRS port x is associated with, Qp is the number of PTRS ports scheduled to the UE, and L is the total number of PUSCH layers.

- Alt.2: 10 log⁡(Lx) +10 log⁡(Qp), where Lx is the number of PUSCH layers which are precoded coherently with the PUSCH layer where PTRS port x is associated with, and Qp is the number of PTRS ports scheduled to the UE.

- Alt.3: 10lg(Lx) +10lg(Qp), where Lx is the number of layers transmitted in the same antenna group as the DMRS port associated with the PTRS port x, and Qp is the number of PTRS ports scheduled to the UE.

FL: In RAN1#112bis-e, whether to add the following text was discussed in case that the max value of the factor may be 12 dB. However, it seems none of the above option does not exceed 9dB, and we don’t need to add the following text.

- Send LS to RAN4 to inform that RAN1 made the above agreement and ask if any impact to UL MIMO requirements in RAN4 (such as PAPR, MPR, EVM, intermod, etc.) when the power of Tx port transmitting PTRS on PTRS RE is 12 dB boosted over the power of the same Tx port on PUSCH REs for 8 layer PUSCH.

| Company | Comment |
| --- | --- |
| Docomo | Support, and support Alt.1. No need to send LS to RAN4. |
| OPPO | There is some copy-paste error in our contribution, so we update the Alt.3 accordingly. And we support the updated Alt.3. |
| Huawei, HiSilicon | Support Alt.2. |
| Google | Support Alt1 |
| ZTE | Support Alt2. For Alt 1, it will lead to power fluctuation between PTRS vs PUSCH per OFDM symbol in time domain, especially for layer combinations {2+3} and {3+4} in 2CWs case. |
| New H3C | Slightly prefer Alt.2 |
| Lenovo | Support Alt.1 |
| QC | May I miss something. But it seems to me the three options are essentially the same (despite there are some wording differences to describe Lx). The only actually difference is option 1 has capped the power boost at 10logL, where option 2/3 don’t have the cap. In our view, the cap is needed. For example, assume Qp=2, if L=5 layers are split into 2+3 layers on two antenna groups. The second antenna group can do 10*log10(3)+10*log10(2)=7.8dB, which exceed 10*log10(5)=7dB. Apparently, the PTRS power after power boost cannot exceed 5 layer PUSCH power, therefore, the cap 10*log10(L) should be applied. |
| CMCC | Support Alt.2. |
| vivo | Prefer Alt.2 |
| Nokia/NSB | Fine with Alt 2. We don’t understand why it is limited by the number of layers. As long as we agreed to support upto 12dB boosting for full-coherent case, it can be a limit, and for partial coherent case, 4 layers x 2 ports are the maximum case, so no further limit is needed. |
| Apple | Fine with Alt. 2 |
| Spreadtrum | We think Alt2 can be baseline, and further discuss whether to support exceeding the power in the full coherent case. |

- ROUND-2

Question to OPPO: Do you intend lg (X) means log10 (X)? If so, what is the difference from Alt.2?

Please continue discussion whether the power limitation of 10logLis necessary.

FL proposal#3.6B:

- For 8Tx PUSCH, when the ptrs-Power configures 00, the factor () for partial coherent TPMIs is down selected from the following:

- Alt.1: min10 log⁡(Lx) +10 log⁡(Qp) , 10logL, where Lx is the number of PUSCH layers which are precoded coherently with the PUSCH layer where PTRS port x is associated with, Qp is the number of PTRS ports scheduled to the UE, and L is the total number of PUSCH layers.

- Alt.2: 10 log⁡(Lx) +10 log⁡(Qp), where Lx is the number of PUSCH layers which are precoded coherently with the PUSCH layer where PTRS port x is associated with, and Qp is the number of PTRS ports scheduled to the UE.

| Company | Comment |
| --- | --- |
| OPPO | Yes to the question. Alt.3 is actually the same as Alt.2, with different wording. We think the limitation is not needed, which will limit the PTRS transmit power when the layer number mapped to different antenna groups is different. So we support Alt.2. |
| Lenovo | Support Alt2/Alt3. We think 10 log⁡(Lx) +10 log⁡(Qp) is always no larger than 10logL, where Lx is assumed to be the minimum number of PUSCH layers which are precoded coherently with the PUSCH layers where different PTRS ports are associated with, since according to current specification, the power boosting for different PTRS ports is same. |
| FL | I removed Alt.3. To proponent of Alt.1, please check Lenovo’s comment above. On the other hand, QC shows an example why the power limitation is necessary for rank5. Please check it above in the 1st round. |
| Ericsson | We support either Alt.3 or Alt.2. We don’t think the cap is needed. |

- ROUND-3

On Wednesday online, the following was agreed.

FL proposal#3.6B: - For 8Tx PUSCH, when the ptrs-Power configures 00, the factor () for partial coherent TPMIs is down selected from the following: - Alt.1: min10 log⁡(Lx) +10 log⁡(Qp) , 10logL, where Lx is the number of PUSCH layers which are precoded coherently with the PUSCH layer where PTRS port x is associated with, Qp is the number of PTRS ports scheduled to the UE, and L is the total number of PUSCH layers. - Alt.2: 10 log⁡(Lx) +10 log⁡(Qp), where Lx is the number of PUSCH layers which are precoded coherently with the PUSCH layer where PTRS port x is associated with, and Qp is the number of PTRS ports scheduled to the UE.

In the 1st round, QC shows an example why the limitation is necessary.

| QC | May I miss something. But it seems to me the three options are essentially the same (despite there are some wording differences to describe Lx). The only actually difference is option 1 has capped the power boost at 10logL, where option 2/3 don’t have the cap. In our view, the cap is needed. For example, assume Qp=2, if L=5 layers are split into 2+3 layers on two antenna groups. The second antenna group can do 10*log10(3)+10*log10(2)=7.8dB, which exceed 10*log10(5)=7dB. Apparently, the PTRS power after power boost cannot exceed 5 layer PUSCH power, therefore, the cap 10*log10(L) should be applied. |
| --- | --- |

Please provide your views, if any.

| Company | Comment |
| --- | --- |
| QC | In legacy release, and in Rel-18 for full/non-coherent 8 Tx, in all those cases, the power on PTRS RE after power boost never exceed power of PUSCH RE. Without the cap, PTRS RE power can exceed PUSCH RE (as shown in above example. Another example is rank 3 split into 1+2 layers, with Qp=2), is this uneven Tx power spectrum density what we want, given uneven Tx power spectrum density will increase inter-mod, inband/outband leakage, etc? |
| Nokia/NSB | Though EPRE is proportional to the number of layers, real PTRS TX power is the function of the number of the coherent ports. So, we don’t need to limit the max power by the number of layers. For example, 5 layers with 3+2, the total TX power of all 3 layers and 2 layers are the same of sum of 4 ports for Ng=2. Regardless of the number of layers, the total power is always the same. We support Alt 2. |
| Ericsson | Thanks QC for the explanation. The UE is designed for 8 layer PUSCH transmission, the PTRS power boost would anyway smaller than 9dB. It is not clear to us how much this small power difference would impact the f.e. outband leakage. We support Alt. 2. |

  - (void)

  - Other proposals

Following proposals are also proposed.

| Proposals | Companies |
| --- | --- |
| - UE indicates the number of PTRS ports associated to Ng as part of its capability | IDC |

Please provide your views on the above proposals, or other aspects which are not included in the summary, if any.

| Company | Comment |
| --- | --- |

- Conclusion

The following FL proposals are made.

FL Proposal 2.4E: Offline consensus

- The following MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports is not supported:

  - 3) For PDSCH, between Rel.18 UE1 indicated with Rel-18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.15-17 UE2 indicated with Rel.15 DMRS ports in a CDM group.

    - UE does not expect such MU-MIMO within a CDM group

  - FFS: 4) For PDSCH, between Rel.18 UE1 indicated with Rel-18 New ports (eType1: ports 1008-1015, eType2: ports 1012-1023) and Rel.18 UE2 indicated with Rel.15 DMRS ports in a CDM group.

    - UE does not expect such MU-MIMO within a CDM group

FL Proposal 2.1.2C(alternative) for working assumption

- For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, for case 2) in RAN1#113 agreement,

  - For 1CW,

    - 2) For row 24-30, 55-60, 69-80, support at least support row 73-80 without MU restriction.

      - FFS: for others

FL Proposal 2.1.2B

- For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, for case 2) in RAN1#113 agreement,

  - For 1CW,

    - 2) For row 24-30, 55-60, 69-80, support at least support row 73-80 without MU restriction. In addition to that, following options can be considered:

      - Alt.2-2A: Support up to 4 rows from row 24-30 with MU restriction. Remove row 55-60.

        - FFS: which 4 rows from row 24-30 is selected.

        - FFS: whether the number of antenna ports field is 6bits.

      - Alt.2-2B2: Support more than 4 rows from row 24-30 with MU restriction.

        - FFS: which 4 rows from row 24-30 is selected.

        - FFS: whether the number of antenna ports field is 6bits.

        - FFS: row 69-72, 55-60.

References

| [1] | R1-2304350 | On increasing the number of orthogonal DM-RS ports for MU-MIMO | FUTUREWEI |
| --- | --- | --- | --- |
| [2] | R1-2304377 | Discussions on increased number of orthogonal DMRS ports | New H3C Technologies Co., Ltd. |
| [3] | R1-2304395 | DMRS enhancement for UL/DL MU-MIMO and 8 Tx UL SU-MIMO | ZTE, China Telecom |
| [4] | R1-2304424 | Further Details on DMRS Enhancements | InterDigital, Inc. |
| [5] | R1-2304437 | On increased number of orthogonal DMRS ports for MU-MIMO and 8 Tx UL SU-MIMO | Ericsson |
| [6] | R1-2304466 | Further discussion on DMRS enhancements | vivo |
| [7] | R1-2304545 | Discussion on increased number of orthogonal DMRS ports | Spreadtrum Communications |
| [8] | R1-2304639 | Discussion on DMRS enhancements in Rel-18 | Huawei, HiSilicon |
| [9] | R1-2304708 | Remaining issues of DMRS enhancements in Rel-18 | CATT |
| [10] | R1-2304818 | DM-RS Enhancements for Rel-18 NR | Intel Corporation |
| [11] | R1-2304837 | On DMRS Enhancement | Google |
| [12] | R1-2304876 | Discussion on DMRS enhancement | xiaomi |
| [13] | R1-2304953 | Discussion of increased number of orthogonal DMRS ports | Lenovo |
| [14] | R1-2304996 | Discussion on increased number of orthogonal DMRS ports | NEC |
| [15] | R1-2305059 | On increased number of orthogonal DMRS ports | Fraunhofer IIS, Fraunhofer HHI |
| [16] | R1-2305080 | Discussion on increased number of orthogonal DMRS ports | CMCC |
| [17] | R1-2305229 | Views on supporting increased number of orthogonal DMRS ports | Apple |
| [18] | R1-2305291 | Increased number of orthogonal DMRS ports | LG Electronics |
| [19] | R1-2305321 | Design for increased number of orthogonal DMRS ports | Qualcomm Incorporated |
| [20] | R1-2305404 | DMRS enhancement for Rel-18 MIMO | OPPO |
| [21] | R1-2305500 | Views on DMRS enhancements | Samsung |
| [22] | R1-2305586 | Discussion on DMRS enhancements | NTT DOCOMO, INC. |
| [23] | R1-2305670 | Increased number of orthogonal DMRS ports | MediaTek Inc. |
| [24] | R1-2305734 | Increased number of orthogonal DMRS ports | Sharp |
| [25] | R1-2305751 | Rel-18 UL and DL DMRS Enhancements | Nokia, Nokia Shanghai Bell |

Appendix

RAN1#109e agreements:

EVM Agreement - LLS is used for objective #3 (increasing DMRS ports for MU-MIMO) in Rel.18 MIMO, while SLS can be used optionally. Agreement - No EVM discussion is needed for objective #5 (>4 layers PUSCH DMRS) in AI 9.1.3.1 (DMRS) in Rel.18. Agreement - LLS for increasing DMRS ports in AI 9.1.3.1 in Rel.18: - Evaluated channel: PDSCH as baseline (Companies can additionally submit evaluation results of PUSCH). - Evaluation metric: - BLER for fixed MCS and rank as baseline - User throughput for adaptive MCS and rank as optional - MSE or NMSE of DMRS as optional - Evaluation baseline (i.e. compared with): - For evaluation of enhanced single-symbol DMRS, baseline refers to Rel.15 single-symbol DMRS or Rel.15 double-symbol DMRS. - For evaluation of enhanced double-symbol DMRS, baseline refers to Rel.15 double-symbol DMRS. Agreement - Following evaluation assumptions are used for LLS for increasing DMRS ports in AI 9.1.3.1 in Rel.18. Parameter; Value. Duplex, Waveform; TDD, OFDM Note: FDD, OFDM is not precluded. Carrier Frequency; 4 GHz. Subcarrier spacing; 30kHz. Channel Model; CDL-B or CDL-C in TR 38.901 with 30ns or 300ns delay spread as baseline for MU-MIMO and SU-MIMO Note: Other delay spread is not precluded. Note: Simulation using TDL-A with 30ns or 300ns for MU-MIMO is not precluded.. Delay spread; Baseline: 30ns, 300ns Optional: 1000ns. UE velocity; Baseline: 3km/h, 30km/h Optional: 60km/h, 120km/h. Allocation bandwidth; 20MHz Note: Other bandwidth smaller than 20MHz is not precluded. MIMO scheme; Baseline: MU-MIMO Optional: SU-MIMO. BS antenna configuration; Companies can select and need to report which option(s) are used between - 32 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded.. UE antenna configuration; Companies can select and need to report which option(s) are used between 4RX: (M, N, P, Mg, Ng, Mp, Np) = (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 2RX: (M, N, P, Mg, Ng, Mp, Np) = (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configuration is not precluded.. MIMO Rank; 1, 2, or 4 per UE (rank fixed or rank adaptation). UE number for MU-MIMO; 1, 2, 4, 8, or 12. Precoding and precoding granularity; For PDSCH: Companies can select and need to report which option(s) are used between - [ZF or SVD] based sub-band precoding (with 4PRB precoding granularity) on ideal channel knowledge - CSI codebook based sub-band precoding (with 4PRB precoding granularity) on ideal CSI feedback. For PUSCH: Companies can select and need to report which option(s) are used between - [ZF or SVD] based wide-band precoding on ideal channel knowledge - Codebook based wide-band precoding on ideal CSI feedback.. Feedback delay for precoding; 5ms. DMRS type; Type 1E and/or Type 2E, which are enhanced DMRS that are based on the legacy RE mappings of DMRS Type 1/2, where the enhanced DMRS support larger DMRS ports. Note: The terminology of Type 1E and/or Type 2E is for discussion purpose.. DMRS configurations; Baseline: - Single symbol DMRS without additional DMRS symbols and 1 additional DMRS symbol - Double symbol DMRS without additional DMRS symbols. Note: evaluation of other additional DMRS symbol(s) are not precluded.. DMRS mapping type; Mapping type A (slot based) for PDSCH. Mapping type A (slot based) for PUSCH.. Link adaptation; - Fixed modulation, coding and rank for BLER evaluation as baseline. - Adaptation of both MCS and rank for throughput evaluation as optional.. HARQ; Baseline: Off Optional: On (HARQ with max. 4 re-transmissions) for throughput evaluation. Channel estimation; Realistic channel estimation with ideal info of frequency sync, SNR, doppler and delay spread. Receiver type; MMSE as baseline. EVM; No radio impairments. Agreement - For LLS assumptions for increasing DMRS ports in AI 9.1.3.1 in Rel.18: - Precoding assumption of PUSCH, “[ZF or SVD]” in RAN1#109e agreement is updated by - Alt.2-2: SVD Agreement For LLS assumptions for increasing DMRS ports in AI 9.1.3.1 in Rel.18: - Precoding assumption of PDSCH, “[ZF or SVD]” in RAN1#109e agreement is updated by SVD. Agreement - For MU-MIMO LLS of PDSCH, for evaluation of SVD/CSI-codebook based sub-band precoding, companies shall report the pre-coding assumption of interference of co-scheduled UEs from the following: - Alt.1: calculated by pre-coder of channel of each co-scheduled UE. - For precoding assumption of PDSCH, precoder of target UE and precoder of co-scheduled UE are generated independently. - Companies can report a set of azimuth and zenith angle offset used for evaluation (For example, azimuth angle offsets from [30 o, 60 o, 90 o] and zenith angle offset from [3o, 6o] can be considered). - Alt.2: calculated by random pre-coder (i.e. precoder selected randomly from a predefined set of precoders) which is different from the pre-coder of target UE. - For precoding assumption of PDSCH, only the channel of one target UE, i.e. Hd, needs to be modelled. Precoder is generated based on Hd to obtain the precoder for this UE only. The interference from co-scheduled UEs can be modelled as, , wherein Wi can be randomly selected from a predefined set of precoders - Companies shall report how to generate the predefined set of precoders for simulation. - Alt.3: the same pre-coder as scheduled UE. - PDSCH interference and interfering DMRS ports are emulated using the same pre-coder as for the scheduled UE. - Power offset of the co-scheduled UE is one value from {0dB, -3dB, -6dB} as fixed evaluation parameter. Other values are not precluded. - For precoding assumption of PDSCH, only the channel of one target UE, i.e. Hd, needs to be modelled. Precoder for the target UE (denoted as Wd) is generated based on Hd only. Denote the precoding matrix/vector of the ith co-scheduled UEs as Wi, and Wi=Wd (Wi for all th co-scheduled UEs are same). Then the interference from co-scheduled UEs can be modelled as .​ For the above Alt.1-3, only PDSCH performance of the target UE is evaluated, while interference of both PDSCH and DMRS of co-scheduled UE(s) is simulated. Agreement - For SLS assumption for increasing DMRS ports in AI 9.1.3.1 in Rel.18, - Scenario: Dense Urban (Macro only) at 4GHz is a baseline. Other scenarios (e.g. Umi, Uma) are not precluded. - Following evaluation assumptions are used for SLS. Parameter; Value. Scenario; Dense Urban (macro only). Carrier frequency; 4GHz. Duplex, Waveform; TDD, OFDM Note: FDD, OFDM is not precluded. Multiple access; OFDMA. Frequency Range; FR1 only.. Inter-BS distance; 200 m. Channel model; According to the TR 38.901. Antenna setup and port layouts at gNB; Companies need to report which option(s) are used between - 32 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 16 ports: (M, N, P, Mg, Ng, Mp, Np) = (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ Other configurations are not precluded.. Antenna setup and port layouts at UE; 4RX: (M, N, P, Mg, Ng, Mp, Np) = (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 2RX: (M, N, P, Mg, Ng, Mp, Np) = (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ for (rank 1,2) Other configurations are not precluded.. BS Tx power; 41 dBm for 10MHz, 44dBm for 20MHz, 47dBm for 40MHz. BS antenna height; 25 m. BS noise figure; 5 dB. UE noise figure; 9 dB. UE antenna height & gain; Follow TR36.873. Modulation; Up to 256 QAM. Coding on PDSCH; LDPC Max code-block size=8448bit. Numerology; Slot/non-slot; 14 OFDM symbols per slot. SCS; 30 kHz. Simulation bandwidth; 20 MHz. Number of RBs; 52 for 30 kHz SCS. Frame structure; Slot Format 0 (all downlink) for all slots. MIMO scheme; SU/MU-MIMO with rank adaptation is a baseline For low RU, SU-MIMO or SU/MU-MIMO with rank adaptation are assumed For medium/high RU, SU/MU-MIMO with rank adaptation is assumed. MIMO layers; For all evaluation, companies to provide the assumption on the maximum MU layers (e.g. 8 or 12). CSI feedback; Feedback assumption at least for baseline scheme CSI feedback periodicity (full CSI feedback): 5 ms, Scheduling delay (from CSI feedback to time to apply in scheduling): 4 ms. Overhead; Companies shall provide the downlink overhead assumption. Traffic model; Baseline: FTP1 with 50% Resource Utilization Optional: Full buffer. UE distribution; [80%] indoor (3km/h), [20%] outdoor (30km/h). UE receiver; MMSE-IRC as the baseline receiver. Feedback assumption; Realistic. Channel estimation; Realistic. For increasing orthogonal DMRS ports Agreement - Specify to increase the max. number of DMRS ports for PDSCH/PUSCH larger than Rel.15 for CP-OFDM without increasing the DMRS overhead. - Strive to have common design of DMRS enhancement for PDSCH and PUSCH for a given DMRS Type. Agreement - The maximum number of enhanced DMRS ports in Rel.18 is doubled from Rel.15 DMRS ports: - For DMRS type 1, the max. number of enhanced DMRS ports in Rel.18 for PDSCH/PUSCH is - Single symbol DMRS: 8 DMRS ports. - Double symbol DMRS: 16 DMRS ports. - For DMRS type 2, the max. number of enhanced DMRS ports in Rel.18 for PDSCH/PUSCH is - Single symbol DMRS: 12 DMRS ports. - Double symbol DMRS: 24 DMRS ports. Agreement - To increase the number of DMRS ports for PDSCH/PUSCH, evaluate and, if needed, specify one or more from the following options: - Opt.1 (enhance FD-OCC): Introduce larger FD-OCC length than Rel.15 (e.g. 4 or 6). - Study aspect includes potential performance degradation in large delay spread, potential scheduling restriction, backward compatibility. - Opt.2 (enhance TD-OCC): Utilize TD-OCC over non-contiguous DMRS symbols (e.g. TD-OCC across front/additional DMRS symbols) - Study aspect includes potential performance degradation in high UE velocity, potential scheduling restriction (e.g. how to apply freq. hopping), potential DMRS configuration restriction (e.g. restriction of the number of additional DMRS), backward compatibility. - Opt.3 (Sparser frequency allocation): increase the number of CDM groups (e.g. larger number of comb/FDM). - Study aspect includes potential performance degradation in large delay spread, backward compatibility. - Opt.4 (using TDMed DMRS symbol): reusing additional DMRS symbols to increase orthogonal DMRS ports - Study aspect includes potential performance degradation in high UE velocity, potential DMRS configuration restriction (e.g. restriction of the number of additional DMRS), backward compatibility. - Opt.5 TD-OCC over non-contiguous DMRS symbols combined with FD-OCC or FDM: reusing additional DMRS symbol(s) to improve channel estimation performance. - Study aspect includes potential performance degradation in high UE velocity, potential scheduling restriction (e.g. how to apply freq. hopping), potential DMRS configuration restriction (e.g. restriction of the number of additional DMRS), backward compatibility. - The same option can be applied to both single symbol DMRS and double symbol DMRS. Agreement - To increase the max. number of DMRS ports for PDSCH/PUSCH compared to Rel.15 DMRS for CP-OFDM without increasing the DMRS overhead, - Study whether/how to enable MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports, as well as whether/how to enable MU-MIMO among Rel.18 DMRS ports, in the same or different CDM group. Agreement - To increase the max. number of orthogonal DMRS ports for PDSCH/PUSCH larger than Rel.15 - Study whether/how to support DCI-based dynamic antenna ports indication of Rel.18 DMRS ports and/or Rel.15 DMRS ports. - Study whether/how to reuse the antenna port indication table in 38.212 as much as possible for both PDSCH and PUSCH - Study the potential need for MU scheduling restrictions in the design of the enhanced antenna port indication table in 38.212 for DL PDSCH. For 8 Tx UL SU-MIMO Agreement - Study the following potential DMRS enhancement for potential support of more than 4 layers SU-MIMO PUSCH. - Extend DMRS port allocation table for rank 5~8 - Note: DL DMRS table can be a reference - Enhancement for DMRS to PTRS mapping - Study whether to utilize Rel.18 DMRS ports for more than 4 layers SU-MIMO PUSCH. - Note: the above study does not imply more than 4 layers SU-MIMO PUSCH is supported. - Note: other study for potential DMRS enhancement for potential support of more than 4 layers SU-MIMO PUSCH is not precluded.

RAN1#110 agreements:

For increasing orthogonal DMRS ports Working Assumption - To increase the number of DMRS ports for PDSCH/PUSCH, support at least Opt.1 (introduce larger FD-OCC length than Rel.15 (e.g. 4 or 6)). - FFS: FD-OCC length for Rel.18 DMRS type 1 and type 2. - FFS: Whether it is needed to handle potential performance issues of Opt 1. For example, study if there is performance loss in case of large delay spread scenario. If needed, how (e.g. additionally support other options). Agreement - For enhanced FD-OCC length for DMRS of PDSCH/PUSCH, support the following FD-OCC length: - For Rel.18 DMRS type 1, down select from the following in RAN1#110bis-e: - Opt.1-1: Length 6 FD-OCC is applied to 6 REs of DMRS within a PRB within an CDM group - Opt.1-2: Length 4 FD-OCC is applied to 4 REs of DMRS within a PRB or across consecutive PRBs within an CDM group - For Rel.18 DMRS type 2: - Length 4 FD-OCC is applied to 4 REs of DMRS within a PRB within an CDM group - FFS: Support of length 6 FD-OCC Agreement - Support MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports. - For MU-MIMO by different CDM groups, no MU-MIMO scheduling restriction of PUSCH/PDSCH (i.e. MU-MIMO between Rel.15 UE and Rel.18 UE is allowed). - For MU-MIMO within a CDM group, study whether and how to support MU-MIMO between Rel.15 DMRS ports and Rel.18 DMRS ports for PDSCH. - Note: the study includes MU-MIMO between Rel.15 UE and Rel.18 UE, and between Rel.18 UEs. - Note: PUSCH above is CP-OFDM waveform. Agreement For increased DMRS ports for enhanced FD-OCC, study whether/how to support DCI based switching between DMRS port(s) associated with length 2 FD-OCC and DMRS port(s) associated with length M FD-OCC (where M > 2). For 8 Tx UL SU-MIMO Agreement - For support of more than 4 layers SU-MIMO PUSCH, study the following potential enhancements for PTRS-DMRS association. - Whether to support more than 2-port UL PTRS. - Whether to increase the DCI size of PTRS-DMRS association field in DCI format 0_1/0_2. Agreement For > 4 layers PUSCH, support rank = 5,6,7,8 for both DMRS type 1/2, and for both single-symbol/double-symbol DMRS.

RAN1#110bis-e agreements:

For increasing orthogonal DMRS ports Conclusion - For discussion purpose, definition of Rel.15 DMRS ports and Rel-18 DMRS ports are: - Rel.15 Type 1/Type 2 DMRS ports: DMRS ports with FD-OCC length =2. - Rel.18 eType 1/eType 2 DMRS ports: DMRS ports with FD-OCC length >2. - Following figure as an example shows difference between Rel.15 Type 1 DMRS ports and Rel.18 eType 1 DMRS ports. Agreement Confirm the working assumption in RAN1#110 with the following update: - To increase the number of DMRS ports for PDSCH/PUSCH, support at least Opt.1 (introduce larger FD-OCC length than Rel.15 (e.g. 4 or 6)). - FFS: FD-OCC length for Rel.18 DMRS type 1 and type 2. - FFS: Whether it is needed to handle potential performance issues of Opt 1. For example, study if there is performance loss in case of large delay spread scenario. If needed, how (e.g. additionally support other options). Agreement For enhanced FD-OCC length for DMRS of PDSCH/PUSCH for Rel.18 eType 1 DMRS, support - Opt.1-2: Length 4 FD-OCC is applied to 4 REs of DMRS within a PRB or across consecutive PRBs within an CDM group Agreement For FD-OCC length 4 for DMRS of PDSCH/PUSCH for Rel.18 eType 1/eType 2 DMRS, support one from the following FD-OCCs (to be selected in RAN1#111): - Opt.1-1: Walsh matrix (Hadamard code): FD-OCC index; wf(0); wf(1); wf(2); wf(3). 0; +1; +1; +1; +1. 1; +1; -1; +1; -1. 2; +1; +1; -1; -1. 3; +1; -1; -1; +1. - Opt.1-2: Cyclic shift with {0, π, π/2, 3π/2}: FD-OCC index; wf(0); wf(1); wf(2); wf(3). 0; +1; +1; +1; +1. 1; +1; -1; +1; -1. 2; +1; +j; -1; -j. 3; +1; -j; -1; +j. Agreement For Rel.18 eType 1/eType 2 DMRS ports of PDSCH/PUSCH with FD-OCC length 4, association between DMRS port indexes, CDM group index, FD-OCC index, and TD-OCC index (across consecutive DMRS symbols, if any) are determined by the following Table 1 and Table 2. - The p in Table 1 and Table 2 corresponds to DMRS port index for PUSCH. - DMRS port index for PDSCH is determined by p +1000 in Table 1 and Table 2. Table 1. Rel.18 eType 1 DMRS ports for PUSCH p; CDM group index; FD-OCC index; TD-OCC index. 0; 0; 0; 0. 1; 0; 1; 0. 2; 1; 0; 0. 3; 1; 1; 0. 4; 0; 0; 1. 5; 0; 1; 1. 6; 1; 0; 1. 7; 1; 1; 1. 8; 0; 2; 0. 9; 0; 3; 0. 10; 1; 2; 0. 11; 1; 3; 0. 12; 0; 2; 1. 13; 0; 3; 1. 14; 1; 2; 1. 15; 1; 3; 1. Table 2. Rel.18 eType 2 DMRS ports for PUSCH p; CDM group index; FD-OCC index; TD-OCC index. 0; 0; 0; 0. 1; 0; 1; 0. 2; 1; 0; 0. 3; 1; 1; 0. 4; 2; 0; 0. 5; 2; 1; 0. 6; 0; 0; 1. 7; 0; 1; 1. 8; 1; 0; 1. 9; 1; 1; 1. 10; 2; 0; 1. 11; 2; 1; 1. 12; 0; 2; 0. 13; 0; 3; 0. 14; 1; 2; 0. 15; 1; 3; 0. 16; 2; 2; 0. 17; 2; 3; 0. 18; 0; 2; 1. 19; 0; 3; 1. 20; 1; 2; 1. 21; 1; 3; 1. 22; 2; 2; 1. 23; 2; 3; 1. Agreement For FD-OCC length 4 in Rel.18 eType 1 DMRS for PDSCH, support the following: - Introduce UE capability to report whether UE can be scheduled PDSCH without the scheduling restriction for FD-OCC length 4 in Rel.18 eType 1 DMRS. - If this capability is not supported by the UE, UE expects that gNB shall apply the scheduling restriction for PDSCH for FD-OCC length 4 in Rel.18 eType 1 DMRS. - The scheduling restriction above means satisfying all of the following at least for other than M-TRP PDSCH transmission with FDM 2a or FDM 2b scheme. - The number of consecutively scheduled PRBs for PDSCH is even. - The number of PRBs offset of scheduled PDSCH from point A (common resource block 0) is even. - FFS: Restriction on scheduling of different UEs in case of MU-MIMO. - FFS: Scheduling restriction for M-TRP PDSCH transmission with FDM 2a or FDM 2b scheme. - Note1: Up to UE how to implement DMRS channel estimation. - Note2: No further RAN1 specification enhancement is introduced to handle the orphan REs (e.g. if the total number of REs of DMRS in a CDM group is not multiples of 4, how to handle the remainder of REs) for UE that is scheduled PDSCH without the scheduling restriction. - Note 3: Other scheduling restrictions, if identified in future meetings, are not precluded. Conclusion For FD-OCC length 4 in Rel.18 eType 1 DMRS for PUSCH, - No spec. enhancement is needed to handle orphan RE issue (e.g. if the total number of REs of DMRS in a CDM group is not multiples of 4, how to handle the remainder of REs), because gNB (receiver) can decide whether the scheduling restriction is needed or not. For 8 Tx UL SU-MIMO Agreement For more than 4 layers SU-MIMO PUSCH, support - Both Rel.15 Type 1/Type 2 DMRS ports and Rel.18 eType 1/eType 2 DMRS ports. - For UE supporting Rel.18 eType 1/eType 2 DMRS ports, UE can be indicated with either of Rel.15 Type 1/Type 2 DMRS ports or Rel.18 eType 1/eType 2 DMRS ports. - RRC based indication is supported as the baseline. FFS whether DCI based indication is further needed. - For UE not supporting Rel.18 eType 1/eType 2 DMRS ports, UE can be indicated with Rel.15 Type 1/Type 2 DMRS ports only.

RAN1#111 agreements:

For increasing orthogonal DMRS ports Agreement For FD-OCC length 4 for PDSCH/PUSCH, select the following: - Opt.1-1 (Walsh matrix) for PDSCH - Opt.1-2 (Cyclic shift) for PUSCH Agreement - For the antenna ports indication in Rel.18 eType1/eType2 DMRS ports with maxLength = 1/2 for PDSCH, all of the following port combinations can be indicated: - Cat. 1) Legacy port indexes (eType 1: p=0~7, eType 2: p=0~11) - Cat. 2) New port indexes (eType 1: p=8~15, eType 2: p=12~23) - Cat. 3) Legacy port indexes and New port indexes at least within a CDM group at least for maxLength=1 (eType 1: up to 4 ports from {0, 1, 8, 9} and/or up to 4 ports from {2, 3, 10, 11}, eType 2: up to 4 ports from {0, 1, 12, 13} and/or up to 4 ports from {2, 3, 14, 15} and/or up to 4 ports from {4, 5, 16, 17}) at least for S-TRP case, - For up to 4 ranks, only one CDM group is used per UE. For larger than 4 ranks, more than one CDM groups can be used per UE. - FFS: Whether to increase the size of antenna ports field in DCI format 1_1/1_2, or introduce new DCI field for antenna ports indication, or not. - FFS: Whether the new antenna port(s) table is specified or not. - FFS: MU restrictions for certain entries. e.g., DMRS ports = {0,2}, or {8,10}, etc. - FFS: Cat.3 for M-TRP case. - Note: DMRS port index for PDSCH is determined by p +1000 Agreement For length 2 TD-OCC (across consecutive DMRS symbols, if any) for DMRS of PDSCH/PUSCH for Rel.18 eType 1/2 DMRS, support Opt.1: - Opt.1: TD-OCC index; Wt(0); Wt(1). 0; +1; +1. 1; +1; -1. Agreement - For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, support the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data. - FFS: Antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH for M-TRP case. Table 7.3.1.2.2-1-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=1 One Codeword: Codeword 0 enabled, Codeword 1 disabled; Two Codewords: Codeword 0 enabled, Codeword 1 enabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s); Notes; Value; Number of DMRS CDM group(s) without data; DMRS port(s); Notes. 0; [1]; [0]; Cat. 1; [0]; [2]; [0,1,2,3,8]; [Rank 5-8 with one DMRS symbol]. 1; [1]; [1]; [1]; [2]; [0,1,2,3,8,10]. 2; [1]; [0,1]; [2]; [2]; [0,1,2,3,8,9,10]. 3; 2; 0; [3]; [2]; [0,1,2,3,8,9,10,11]. 4; 2; 1. 5; 2; 2. 6; 2; 3. 7; 2; 0,1. 8; 2; 2,3. 9; [2]; [0-2]. 10; [2]; [0-3]. 11; [2]; [0,2]. 12; [1]; [8]; Cat.2. 13; [1]; [9]. 14; [1]; [8,9]. 15; 2; 8. 16; 2; 9. 17; 2; 10. 18; 2; 11. 19; 2; 8,9. 20; 2; 10,11. 21; [2]; [8-10]. 22; [2]; [8-11]. 23; [2]; [8, 10], [9, 11]. 24; [1]; [0,1,8]; Cat.3. 25; [1]; [0,1,8,9]. 26; 2; 0,1,8. 27; 2; 0,1,8,9. 28; 2; 2,3,10. 29; 2; 2,3,10,11. For 8 Tx UL SU-MIMO Agreement - For > 4 layers PUSCH, support new antenna ports tables for rank = 5,6,7,8 for both single-symbol/double-symbol DMRS. - For Type 1/Type 2 Rel.15 DMRS ports, new antenna ports tables are the following: - The same DMRS port combination(s) as that for rank = 5,6,7,8 for PDSCH is reused at least for full or non-coherent UL codebook. - For Rel.18 eType1/eType2 DMRS ports, - New antenna ports tables with new DMRS port combinations are used for rank = 5,6,7,8 (FFS: details). - Note: Whether the DMRS port combination allows to use single symbol DMRS for rank = 5,6,7,8 should be checked. - FFS: For partial coherent UL codebook, support layers to DMRS port mapping that layers associated to the same antenna port group are multiplexed into the same DMRS CDM group. - FFS: One or more than one DMRS port combination(s) for each rank and TPMI - Note: New DMRS port combinations above does not preclude the new antenna ports tables including the current DMRS port combination(s) for PDSCH for rank = 5,6,7,8 in Rel.15-17. - FFS: Whether the antenna ports combinations for rank = 5,6,7,8 can be indicated by the reserved entries of existing antenna ports tables for rank =1,2,3,4, if the rank is indicated together with DMRS antenna ports. Agreement - For full-coherent PUSCH with rank 5-8, UE shall expect only one port PTRS to be configured. - Down select from the following in RAN1#112: - Alt.1: the size of PTRS-DMRS association field is 2bit in DCI format 0_1/0_2. - FFS: Association with the CW with the higher MCS. Table 7.3.1.1.2-25B: PTRS-DMRS association for UL PTRS port 0 Value; DMRS port. 0; 1st scheduled DMRS port with the CW with the higher MCS. 1; 2nd scheduled DMRS port the CW with the higher MCS. 2; 3rd scheduled DMRS port the CW with the higher MCS. 3; 4th scheduled DMRS port the CW with the higher MCS. - Alt.2: The size of PTRS-DMRS association field is 3bit in DCI format 0_1/0_2, and the following PTRS-DMRS association for UL PTRS port 0 is specified in TS38.212. Table 7.3.1.1.2-25B: PTRS-DMRS association for UL PTRS port 0 Value; DMRS port. 0; 1st scheduled DMRS port. 1; 2nd scheduled DMRS port. 2; 3rd scheduled DMRS port. 3; 4th scheduled DMRS port. 4; 5th scheduled DMRS port. 5; 6th scheduled DMRS port. 6; 7th scheduled DMRS port. 7; 8th scheduled DMRS port.

RAN1#112 agreements:

For increasing orthogonal DMRS ports Agreement - For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, support at least support the following rows: - For 1 CW, - 1) Row 0-2, 12-14, 24-25 (rows with Number of DMRS CDM group(s) without data = 1) Agreement For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, at least support the following rows: - For 1 CW, - 2) Row 9-11 - For the above rows, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group). Working Assumption For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, for 2 CWs, - Alt.3-1: Support at least row 0-3 for 2 CWs in Table 4-0. Table 4-0: DMRS ports for 2CWs. Two Codewords: Codeword 0 enabled, Codeword 1 enabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 2; 0,1,2,3,8. 1; 2; 0,1,2,3,8,10. 2; 2; 0,1,2,3,8,9,10. 3; 2; 0,1,2,3,8,9,10,11. Agreement For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH for S-DCI based M-TRP, support at least the following row(s): - For one CW, support at least row 30 in the following table. - For the above row, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group). - FFS: other rows are not precluded Table 7.3.1.2.2-1A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=1 One Codeword: Codeword 0 enabled, Codeword 1 disabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s). …; …; …. 30; 2; 0,2,3. Agreement For Rel.18 eType1/eType2 DMRS ports for PDSCH/PUSCH, support Alt.1 for PTRS RE mapping. - Alt 1: Different RE offsets set for different Rel.18 DMRS port indexes as shown in Table 4 Table 4 Different RE offsets set for different Rel.18 DMRS port indexes DM-RS antenna port p (p for PUSCH, p+1000 for PDSCH). DM-RS Configuration type 1; DM-RS Configuration type 2. resourceElementOffset; resourceElementOffset. offset00; offset01; offset10; offset11; offset00; offset01; offset10; offset11. 0; 0; 2; 6; 8; 0; 1; 6; 7. 1; 2; 4; 8; 10; 1; 6; 7; 0. 2; 1; 3; 7; 9; 2; 3; 8; 9. 3; 3; 5; 9; 11; 3; 8; 9; 2. 4; -; -; -; -; 4; 5; 10; 11. 5; -; -; -; -; 5; 10; 11; 4. 8; 4; 6; 10; 0; -; -; -; -. 9; 6; 8; 0; 2; -; -; -; -. 10; 5; 7; 11; 1; -; -; -; -. 11; 7; 9; 1; 3; -; -; -; -. 12; -; -; -; -; 6; 7; 0; 1. 13; -; -; -; -; 7; 0; 1; 6. 14; -; -; -; -; 8; 9; 2; 3. 15; -; -; -; -; 9; 2; 3; 8. 16; -; -; -; -; 10; 11; 4; 5. 17; -; -; -; -; 11; 4; 5; 10. For 8 Tx UL SU-MIMO Working assumption - To support PUSCH with rank = 5-8, support the following for enhancement of DMRS port allocation tables. - Option 1: Separate DMRS ports tables for rank 5,6,7,8 for each of eType1/eType2 and maxLength=1/2 (similar to the current UL DMRS ports table). - FFS: whether/how to reuse the reserved field in antenna ports field for other purposes can be discussed in AI9.1.4.2 [or AI9.1.3.1]. Agreement - For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PUSCH, following Table 7.3.1.1.2-8-X, Table 7.3.1.1.2-9-X, Table 7.3.1.1.2-10-X, and Table 7.3.1.1.2-11-X are supported. - FFS: Whether to increase the size of antenna ports field in DCI format 0_1/0_2 or not. - Note: Antenna ports tables for Rel.18 eType2 DMRS ports with maxLength = 1/2 and eType1 DMRS ports with maxLength = 2 for PUSCH are to be discussed separately. Table 7.3.1.1.2-8-X: Antenna port(s), transform precoder is disabled, dmrs-Type=eType1, maxLength=1, rank = 1 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 1; 0. 1; 1; 1. 2; 2; 0. 3; 2; 1. 4; 2; 2. 5; 2; 3. 6; 1; 8. 7; 1; 9. 8; 2; 8. 9; 2; 9. 10; 2; 10. 11; 2; 11. 12-15; Reserved; Reserved. Table 7.3.1.1.2-9-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 2 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 1; 0,1. 1; 2; 0,1. 2; 2; 2,3. 3; 2; 0,2. 4; 1; 8,9. 5; 2; 8,9. 6; 2; 10,11. [7]; [2]; [8,10]. 8-15; Reserved; Reserved. Table 7.3.1.1.2-10-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 3 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 2; 0-2. [1]; [2]; [8-10]. 2; 1; 0,1,8. 3; 2; 0,1,8. 4; 2; 2,3,10. 5-15; Reserved; Reserved. Table 7.3.1.1.2-11-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 4 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 2; 0-3. [1]; [2]; [8-11]. 2; 1; 0,1,8,9. 3; 2; 0,1,8,9. 4; 2; 2,3,10,11. 5-15; Reserved; Reserved. Agreement - For full-coherent PUSCH with rank 5-8 with one port PTRS, support Alt.1 in the RAN1#111 agreement with the following update - Alt.1: the size of PTRS-DMRS association field is 2bit in DCI format 0_1/0_2. - FFS: Association with The CW with the higher MCS is selected in case of two CWs. - If the MCS is the same for two CWs, the PTRS port is associated with the first CW. Table 7.3.1.1.2-25B: PTRS-DMRS association for UL PTRS port 0 Value; DMRS port. 0; 1st scheduled DMRS port with the CW with the higher MCS. 1; 2nd scheduled DMRS port the CW with the higher MCS. 2; 3rd scheduled DMRS port the CW with the higher MCS. 3; 4th scheduled DMRS port the CW with the higher MCS.

RAN1#112bis-e agreements:

For increasing orthogonal DMRS ports Agreement (eType1, maxLength1) For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, for S-DCI based M-TRP, - Support all rows of DMRS port combinations and Number of DMRS CDM group(s) without data for Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH for S-TRP, in addition to row 30 for 1CW in RAN1#112 agreement. - If MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) is introduced to certain row(s) for S-TRP, the MU-restriction is applied to the same row(s) for S-DCI based M-TRP. Agreement (eType1, maxLength2) For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH, for S-DCI based M-TRP case, support all the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data. - All rows for Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH for S-TRP. - If MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) is introduced to certain row(s) for S-TRP, the MU-restriction is applied to the same row(s) for S-DCI based M-TRP. - For one CW, add new row 68 in Table 7.3.1.2.2-2A-X. - For row 68, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group). Table 7.3.1.2.2-2A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=2 One Codeword: Codeword 0 enabled, Codeword 1 disabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. …; …; …; …. 68; 2; 0,2,3; 1. Agreement (eType2, maxLength1) For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH, for S-DCI based M-TRP case, support all the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data. - All rows for Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH for S-TRP. - If MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) is introduced to certain row(s) for S-TRP, the MU-restriction is applied to the same row(s) for S-DCI based M-TRP. - For one CW, add new row 60 in Table 7.3.1.2.2-3A-X. - For row 60, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group). Table 7.3.1.2.2-3A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=1 One Codeword: Codeword 0 enabled, Codeword 1 disabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s). …; …; …. 60; 2; 0,2,3. Agreement (eType2, maxLength2) For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH, for S-DCI based M-TRP case, support all the following rows of DMRS port combinations and Number of DMRS CDM group(s) without data. - All rows for Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH for S-TRP. - If MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) is introduced to certain row(s) for S-TRP, the MU-restriction is also applied to the same row(s) for S-DCI based M-TRP. - For one CW, add new row 128 in Table 7.3.1.2.2-4A-X. - For row 128, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group). Table 7.3.1.2.2-4A-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=2 One Codeword: Codeword 0 enabled, Codeword 1 disabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. …; …; …; …. 128; 2; 0,2,3; 1. Agreement For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, - For 2 CWs, - Alt.1: Confirm the working assumption in RAN1#112 with modification (in red). - Alt.3-1: Support at least row 0-3 for 2 CWs in Table 4-0. Table 4-0: DMRS ports for 2CWs. Two Codewords: Codeword 0 enabled, Codeword 1 enabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 2; 0,1,2,3,8. 1; 2; 0,1,2,3,8,10. 2; 2; 0,1,2,3,8,9,10. 3; 2; 0,1,2,3,8,9,10,11. [4]; [2]; [0,1,2,3,10]. [5]; [2]; [0,1,8,2,3,10]. [6]; [2]; [0,1,8,2,3,10,11]. [7]; [2]; [0,1,8,9,2,3,10,11]. [8]; [2]; [0,2,3,8,9]. [9]; [2]; [0,1,2,3,8,9]. FFS: Additional rows (rows 4~9) if there is technical justification. Agreement For RAN1#111 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, - For 1 CW, - Do not support row 21-22 - FFS: Whether to support row 23 One Codeword: Codeword 0 enabled, Codeword 1 disabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s). 21; [2]; [8-10]. 22; [2]; [8-11]. Agreement For the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, support all rows of DMRS port combinations and Number of DMRS CDM group(s) without data in Table 7.3.1.2.2-2-X. - FFS: For row 9-11, 24-30, 55-60, and 81-83 (if agreed) in one CW, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) or UE capability. - FFS: The total number of rows for eType1 DMRS ports with maxLength =2 for PDSCH at least for S-TRP case does not exceed 64. Table 7.3.1.2.2-2-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType1, maxLength=2 One Codeword: Codeword 0 enabled, Codeword 1 disabled; Two Codewords: Codeword 0 enabled, Codeword 1 enabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols; Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 1; 0; 1; [0; 2; 0-4; 2]. 1; 1; 1; 1; [1; 2; 0,1,2,3,4,6; 2]. 2; 1; 0,1; 1; [2; 2; 0,1,2,3,4,5,6; 2]. 3; 2; 0; 1; [3; 2; 0,1,2,3,4,5,6,7; 2]. 4; 2; 1; 1; 4; 2; 0,1,2,3,8; 1. 5; 2; 2; 1; 5; 2; 0,1,2,3,8,10; 1. 6; 2; 3; 1; 6; 2; 0,1,2,3,8,9,10; 1. 7; 2; 0,1; 1; 7; 2; 0,1,2,3,8,9,10,11; 1. [8; 2; 2,3; 1]; [8; 1; 0,1,4,5,8; 2]. [9; 2; 0-2; 1]; [9; 1; 0,1,4,5,8,12; 2]. [10; 2; 0-3; 1]; [10; 1; 0,1,4,5,8,9,12; 2]. [11; 2; 0,2; 1]; [11; 1; 0,1,4,5,8,9,12,13; 2]. 12; 2; 0; 2; [12; 2; 0,1,4,5,8; 2]. 13; 2; 1; 2; [13; 2; 0,1,4,5,8,12; 2]. 14; 2; 2; 2; [14; 2; 0,1,4,5,8,9,12; 2]. 15; 2; 3; 2; [15; 2; 0,1,4,5,8,9,12,13; 2]. 16; 2; 4; 2; [16; 2; 2,3,6,7,10; 2]. 17; 2; 5; 2; [17; 2; 2,3,6,7,10,14; 2]. 18; 2; 6; 2; [18; 2; 2,3,6,7,10,11,14; 2]. 19; 2; 7; 2; [19; 2; 2,3,6,7,10,11,14,15; 2]. 20; 2; 0,1; 2; [20; 2; 0,1, 2,3,10; 1]. 21; 2; 2,3; 2; [21; 2; 0,1,8,2,3,10; 1]. 22; 2; 4,5; 2; [22; 2; 0,1,8, 2,3,10,11; 1]. 23; 2; 6,7; 2; [23; 2; 0,1,8,9,2,3,10,11; 1]. [24; 2; 0,4; 2]; [24; 1; 0,1,4,5,12; 2]. [25; 2; 2,6; 2]; [25; 1; 0,1,8,4,5,12; 2]. [26; 2; 0,1,4; 2]; [26; 1; 0,1,8,4,5,12,13; 2]. [27; 2; 2,3,6; 2]; [27; 1; 0,1,8,9,4,5,12,13; 2]. [28; 2; 0,1,4,5; 2]; [28; 2; 0,1,4,5,12; 2]. [29; 2; 2,3,6,7; 2]; [29; 2; 0,1,8,4,5,12; 2]. [30; 2; 0,2,4,6; 2]; [30; 2; 0,1,8,4,5,12,13; 2]. 31; 1; 8; 1; [31; 2; 0,1,8,9,4,5,12,13; 2]. 32; 1; 9; 1; [32; 2; 2,3,6,7,14; 2]. 33; 1; 8,9; 1; [33; 2; 2,3,10,6,7,14; 2]. 34; 2; 8; 1; [34; 2; 2,3,10,6,7,14,15; 2]. 35; 2; 9; 1; [35; 2; 2,3,10,11,6,7,14,15; 2]. 36; 2; 10; 1; [36; 2; 0,2,3,8,9; 1]. 37; 2; 11; 1; [37; 2; 0,1,2,3,8,9; 1]. 38; 2; 8,9; 1. 39; 2; 10,11; 1. [40; 2; 8-10; 1]. [41; 2; 8-11; 1]. [42; 2; 8,10; 1]. 43; 2; 8; 2. 44; 2; 9; 2. 45; 2; 10; 2. 46; 2; 11; 2. 47; 2; 12; 2. 48; 2; 13; 2. 49; 2; 14; 2. 50; 2; 15; 2. 51; 2; 8,9; 2. 52; 2; 10,11; 2. 53; 2; 12,13; 2. 54; 2; 14,15; 2. [55; 2; 8,12; 2]. [56; 2; 10,14; 2]. [57; 2; 8,9,12; 2]. [58; 2; 10,11,14; 2]. [59; 2; 8,9,12,13; 2]. [60; 2; 10,11,14,15; 2]. 61; 2; 8,10,12,14; 2. 62; 1; 0,1,8; 1. 63; 1; 0,1,8,9; 1. 64; 2; 0,1,8; 1. 65; 2; 0,1,8,9; 1. 66; 2; 2,3,10; 1. 67; 2; 2,3,10,11; 1. [69; 1; 0,1,8; 2]. [70; 1; 0,1,8,9; 2]. [71; 1; 4,5,12; 2]. [72; 1; 4,5,12,13; 2]. [73; 2; 0,1,8; 2]. [74; 2; 0,1,8,9; 2]. [75; 2; 4,5,12; 2]. [76; 2; 4,5,12,13; 2]. [77; 2; 2,3,10; 2]. [78; 2; 2,3,10,11; 2]. [79; 2; 6,7,14; 2]. [80; 2; 6,7,14,15; 2]. [81; 2; 5,8,9; 2]. [82; 2; 7,10,11; 2]. [83; 2; 7,12,13; 2]. Agreement For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 1 for PDSCH, at least for S-TRP case, support all rows of DMRS port combinations and Number of DMRS CDM group(s) without data in Table 7.3.1.2.2-3-X. - FFS: For rows 9, 10, 20-23, 33,34, 44-46, 60-62 (if agreed) in one CW, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) or UE capability. Table 7.3.1.2.2-3-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=1 One codeword: Codeword 0 enabled, Codeword 1 disabled; Two codewords: Codeword 0 enabled, Codeword 1 enabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s); Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 1; 0; 0; 3; 0-4. 1; 1; 1; 1; 3; 0-5. 2; 1; 0,1; [2; 3; 12-16]. 3; 2; 0; [3; 3; 12-17]. 4; 2; 1; 4; 2; 0,1,2,3,12. 5; 2; 2; 5; 2; 0,1,2,3,12,14. 6; 2; 3; 6; 2; 0-3,12-14. 7; 2; 0,1; 7; 2; 0-3,12-15. 8; 2; 2,3; [8; 3; 0,1,2,3,12]. [9; 2; 0-2]; [9; 3; 0,1,2,3,12,14]. [10; 2; 0-3]; [10; 3; 0-3,12-14]. 11; 3; 0; [11; 3; 0-3,12-15]. 12; 3; 1; [12; 2; 0,2,3,12,13]. 13; 3; 2; [13; 2; 0,1,2,3,14]. 14; 3; 3; [14; 2; 0,1,12,2,3,14]. 15; 3; 4; [15; 2; 0,1,12,2,3,14,15]. 16; 3; 5; [16; 2; 0,1,12,13,2,3,14,15]. 17; 3; 0,1; [17; 3; 0,1,2,3,14]. 18; 3; 2,3; [18; 3; 0,1,12,2,3,14]. 19; 3; 4,5; [19; 3; 0,1,12,2,3,14,15]. [20; 3; 0-2]; [20; 3; 0,1,12,13,2,3,14,15]. [21; 3; 3-5]. [22; 3; 0-3]. [23; 2; 0,2]. 24; 1; 12. 25; 1; 13. 26; 1; 12,13. 27; 2; 12. 28; 2; 13. 29; 2; 14. 30; 2; 15. 31; 2; 12,13. 32; 2; 14,15. [33; 2; 12-14]. [34; 2; 12-15]. 35; 3; 12. 36; 3; 13. 37; 3; 14. 38; 3; 15. 39; 3; 16. 40; 3; 17. 41; 3; 12,13. 42; 3; 14,15. 43; 3; 16,17. [44; 3; 12-14]. [45; 3; 15-17]. [46; 3; 12-15]. [47; 2; 12,14]. 48; 1; 0,1,12. 49; 1; 0,1,12,13. 50; 2; 0,1,12. 51; 2; 0,1,12,13. 52; 2; 2,3,14. 53; 2; 2,3,14,15. 54; 3; 0,1,12. 55; 3; 0,1,12,13. 56; 3; 2,3,14. 57; 3; 2,3,14,15. 58; 3; 4,5,16. 59; 3; 4,5,16,17. [60; 3; 13,15,17]. [61; 3; 13,15]. [62; 2; 13,15]. Agreement For the antenna ports indication in Rel.18 eType2 DMRS ports with maxLength = 2 for PDSCH, at least for S-TRP case, support all rows of DMRS port combinations and Number of DMRS CDM group(s) without data in Table 7.3.1.2.2-4-X. - FFS: For rows 9, 10, 20-23, 42-47, 67, 68, 78-80, 100-105, and 153-158 (if agreed) in one CW, introduce MU-MIMO restriction (i.e. UE does not expect to be multiplexed with other DMRS ports in the same CDM group) or UE capability. Table 7.3.1.2.2-4-X: Antenna port(s) (1000 + DMRS port), dmrs-Type=eType2, maxLength=2 One codeword: Codeword 0 enabled, Codeword 1 disabled; Two Codewords: Codeword 0 enabled, Codeword 1 enabled. Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols; Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 1; 0; 1; [0; 3; 0-4; 1]. 1; 1; 1; 1; [1; 3; 0-5; 1]. 2; 1; 0,1; 1; [2; 2; 0,1,2,3,6; 2]. 3; 2; 0; 1; [3; 2; 0,1,2,3,6,8; 2]. 4; 2; 1; 1; [4; 2; 0,1,2,3,6,7,8; 2]. 5; 2; 2; 1; [5; 2; 0,1,2,3,6,7,8,9; 2]. 6; 2; 3; 1; 6; 2; 0,1,2,3,12; 1. 7; 2; 0,1; 1; 7; 2; 0-3,12,14; 1. 8; 2; 2,3; 1; 8; 2; 0-3,12-14; 1. [9; 2; 0-2; 1]; 9; 2; 0-3,12-15; 1. [10; 2; 0-3; 1]; [10; 3; 0,1,2,3,12; 1]. 11; 3; 0; 1; [11; 3; 0-3,12,14; 1]. 12; 3; 1; 1; [12; 3; 0-3,12-14; 1]. 13; 3; 2; 1; [13; 3; 0-3,12-15; 1]. 14; 3; 3; 1; [14; 1; 0,1,6,7,12; 2]. 15; 3; 4; 1; [15; 1; 0,1,6,7,12,18; 2]. 16; 3; 5; 1; [16; 1; 0,1,6,7,12,13,18; 2]. 17; 3; 0,1; 1; [17; 1; 0,1,6,7,12,13,18,19; 2]. 18; 3; 2,3; 1; [18; 2; 0,1,6,7,12; 2]. 19; 3; 4,5; 1; [19; 2; 0,1,6,7,12,18; 2]. [20; 3; 0-2; 1]; [20; 2; 0,1,6,7,12,13,18; 2]. [21; 3; 3-5; 1]; [21; 2; 0,1,6,7,12,13,18,19; 2]. [22; 3; 0-3; 1]; [22; 2; 2,3,8,9,14; 2]. [23; 2; 0,2; 1]; [23; 2; 2,3,8,9,14,20; 2]. 24; 3; 0; 2; [24; 2; 2,3,8,9,14,15,20; 2]. 25; 3; 1; 2; [25; 2; 2,3,8,9,14,15,20,21; 2]. 26; 3; 2; 2; [26; 3; 0,1,6,7,12; 2]. 27; 3; 3; 2; [27; 3; 0,1,6,7,12,18; 2]. 28; 3; 4; 2; [28; 3; 0,1,6,7,12,13,18; 2]. 29; 3; 5; 2; [29; 3; 0,1,6,7,12,13,18,19; 2]. 30; 3; 6; 2; [30; 3; 2,3,8,9,14; 2]. 31; 3; 7; 2; [31; 3; 2,3,8,9,14,20; 2]. 32; 3; 8; 2; [32; 3; 2,3,8,9,14,15,20; 2]. 33; 3; 9; 2; [33; 3; 2,3,8,9,14,15,20,21; 2]. 34; 3; 10; 2; [34; 3; 4,5,10,11,16; 2]. 35; 3; 11; 2; [35; 3; 4,5,10,11,16,22; 2]. 36; 3; 0,1; 2; [36; 3; 4,5,10,11,16,17,22; 2]. 37; 3; 2,3; 2; [37; 3; 4,5,10,11,16,17,22,23; 2]. 38; 3; 4,5; 2; [38; 2; 0,1,2,3,14; 1]. 39; 3; 6,7; 2; [39; 2; 0,1,12,2,3,14; 1]. 40; 3; 8,9; 2; [40; 2; 0,1,12,2,3,14,15; 1]. 41; 3; 10,11; 2; [41; 2; 0,1,12,13,2,3,14,15; 1]. [42; 3; 0,1,6; 2]; [42; 3; 0,1,2,3,14; 1]. [43; 3; 2,3,8; 2]; [43; 3; 0,1,12,2,3,14; 1]. [44; 3; 4,5,10; 2]; [44; 3; 0,1,12,2,3,14,15; 1]. [45; 3; 0,1,6,7; 2]; [45; 3; 0,1,12,13,2,3,14,15; 1]. [46; 3; 2,3,8,9; 2]; [46; 1; 0,1,6,7,18; 2]. [47; 3; 4,5,10,11; 2]; [47; 1; 0,1,12,6,7,18; 2]. 48; 1; 0; 2; [48; 1; 0,1,12,6,7,18,19; 2]. 49; 1; 1; 2; [49; 1; 0,1,12,13,6,7,18,19; 2]. 50; 1; 6; 2; [50; 2; 0,1,6,7,18; 2]. 51; 1; 7; 2; [51; 2; 0,1,12,6,7,18; 2]. 52; 1; 0,1; 2; [52; 2; 0,1,12,6,7,18,19; 2]. 53; 1; 6,7; 2; [53; 2; 0,1,12,13,6,7,18,19; 2]. 54; 2; 0,1; 2; [54; 2; 2,3,8,9,20; 2]. 55; 2; 2,3; 2; [55; 2; 2,3,14,8,9,20; 2]. 56; 2; 6,7; 2; [56; 2; 2,3,14,8,9,20,21; 2]. 57; 2; 8,9; 2; [57; 2; 2,3,14,15,8,9,20,21; 2]. 58; 1; 12; 1; [58; 3; 0,1,6,7,18; 2]. 59; 1; 13; 1; [59; 3; 0,1,12,6,7,18; 2]. 60; 1; 12,13; 1; [60; 3; 0,1,12,6,7,18,19; 2]. 61; 2; 12; 1; [61; 3; 0,1,12,13,6,7,18,19; 2]. 62; 2; 13; 1; [62; 3; 2,3,8,9,20; 2]. 63; 2; 14; 1; [63; 3; 2,3,14,8,9,20; 2]. 64; 2; 15; 1; [64; 3; 2,3,14,8,9,20,21; 2]. 65; 2; 12,13; 1; [65; 3; 2,3,14,15,8,9,20,21; 2]. 66; 2; 14,15; 1; [66; 3; 4,5,10,11,22; 2]. [67; 2; 12-14; 1]; [67; 3; 4,5,16,10,11,22; 2]. [68; 2; 12-15; 1]; [68; 3; 4,5,16,10,11,22,23; 2]. 69; 3; 12; 1; [69; 3; 4,5,16,17,10,11,22,23; 2]. 70; 3; 13; 1. 71; 3; 14; 1. 72; 3; 15; 1. 73; 3; 16; 1. 74; 3; 17; 1. 75; 3; 12,13; 1. 76; 3; 14,15; 1. 77; 3; 16,17; 1. [78; 3; 12-14; 1]. [79; 3; 15-17; 1]. [80; 3; 12-15; 1]. [81; 2; 12,14; 1]. 82; 3; 12; 2. 83; 3; 13; 2. 84; 3; 14; 2. 85; 3; 15; 2. 86; 3; 16; 2. 87; 3; 17; 2. 88; 3; 18; 2. 89; 3; 19; 2. 90; 3; 20; 2. 91; 3; 21; 2. 92; 3; 22; 2. 93; 3; 23; 2. 94; 3; 12,13; 2. 95; 3; 14,15; 2. 96; 3; 16,17; 2. 97; 3; 18,19; 2. 98; 3; 20,21; 2. 99; 3; 22,23; 2. [100; 3; 12,13,18; 2]. [101; 3; 14,15,20; 2]. [102; 3; 16,17,22; 2]. [103; 3; 12,13,18,19; 2]. [104; 3; 14,15,20,21; 2]. [105; 3; 16,17,22,23; 2]. 106; 1; 12; 2. 107; 1; 13; 2. 108; 1; 18; 2. 109; 1; 19; 2. 110; 1; 12,13; 2. 111; 1; 18,19; 2. 112; 2; 12,13; 2. 113; 2; 14,15; 2. 114; 2; 18,19; 2. 115; 2; 20,21; 2. 116; 1; 0,1,12; 1. 117; 1; 0,1,12,13; 1. 118; 2; 0,1,12; 1. 119; 2; 0,1,12,13; 1. 120; 2; 2,3,14; 1. 121; 2; 2,3,14,15; 1. 122; 3; 0,1,12; 1. 123; 3; 0,1,12,13; 1. 124; 3; 2,3,14; 1. 125; 3; 2,3,14,15; 1. 126; 3; 4,5,16; 1. 127; 3; 4,5,16,17; 1. [129; 1; 0,1,12; 2]. [130; 1; 0,1,12,13; 2]. [131; 1; 6,7,18; 2]. [132; 1; 6,7,18,19; 2]. [133; 2; 0,1,12; 2]. [134; 2; 0,1,12,13; 2]. [135; 2; 6,7,18; 2]. [136; 2; 6,7,18,19; 2]. [137; 2; 2,3,14; 2]. [138; 2; 2,3,14,15; 2]. [139; 2; 8,9,20; 2]. [140; 2; 8,9,20,21; 2]. [141; 3; 0,1,12; 2]. [142; 3; 0,1,12,13; 2]. [143; 3; 6,7,18; 2]. [144; 3; 6,7,18,19; 2]. [145; 3; 2,3,14; 2]. [146; 3; 2,3,14,15; 2]. [147; 3; 8,9,20; 2]. [148; 3; 8,9,20,21; 2]. [149; 3; 4,5,16; 2]. [150; 3; 4,5,16,17; 2]. [151; 3; 10,11,22; 2]. [152; 3; 10,11,22,23; 2]. [153; 3; 7,12,13; 2]. [154; 3; 9,14,15; 2]. [155; 3; 11,16,17; 2]. [156; 3; 9,18,19; 2]. [157; 3; 18,19,20; 2]. [158; 3; 21,22,23; 2]. Conclusion No consensus to support MAC CE based switching between Rel.15 DMRS ports and Rel.18 DMRS ports for PDSCH. Agreement For Rel.18 eType1/eType2 DMRS ports with maxLength=1/2 for PDSCH/PUSCH, if Rel.18 eType1/eType2 DMRS ports is configured by RRC, the DCI size of antenna ports field in DCI format 1_1/1_2/0_1/0_2 is increased by at least 1-bit from Rel.17. - Note: it does not preclude future possibility to support more than 1-bit, if RAN1 agree the necessity. Agreement For RAN1#112 agreement of the antenna ports indication in Rel.18 eType1 DMRS ports with maxLength = 1 for PUSCH. - Support row 7 for rank2, row1 for rank3, row 1 for rank4. Table 7.3.1.1.2-9-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 2 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 7; 2; 9,11. Table 7.3.1.1.2-10-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 3 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 1; 2; 8-10. Table 7.3.1.1.2-11-X: Antenna port(s), transform precoder is disabled, dmrs-Type= eType1, maxLength=1, rank = 4 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 1; 2; 8-11. Conclusion For MU-MIMO within a CDM group between Rel.15 DMRS ports and Rel.18 DMRS ports, - For PUSCH, there is no restriction. For 8 Tx UL SU-MIMO Agreement Confirm the following Working Assumption in RAN1#112 at least for NCB based PUSCH: - To support PUSCH with rank = 5-8, support the following for enhancement of DMRS port allocation tables. - Option 1: Separate DMRS ports tables for rank 5,6,7,8 for each of eType1/eType2 and maxLength=1/2 (similar to the current UL DMRS ports table). - FFS: whether/how to reuse the reserved field in antenna ports field for other purposes can be discussed in AI9.1.4.2 [or AI9.1.3.1]. - Note: The above Working Assumption for CB based PUSCH may be confirmed later. Agreement For 8Tx PUSCH, specify the factor related to PUSCH to PTRS power ratio per layer per RE () based on the following principles. - Principle 1: When the ptrs-Power configures 01, the factor () is 10log10(L), where L is the total number of PUSCH layers. - Principle 2: When the ptrs-Power configures 00, the factor () is determined as the following - Principle 2.1: For fully coherent TPMIs, the factor () is 10log10(L), where L is the total number of PUSCH layers. - Principle 2.2: For non-coherent TPMIs, the factor () is 10log10(Qp), where Qp is the number of PTRS ports scheduled to the UE. - Principle 2.3: For non-codebook PUSCH, the factor () is 10log10(Qp), where Qp is the number of PTRS ports scheduled to the UE. - FFS: The factor () for partial coherent TPMIs Working Assumption - Adopt Table 7.3.1.1.2-12B/13B/14B/15B/16B/17B/20B/21B/22B/23B to support signalling >4 ranks PUSCH with Rel-15 DMRS ports at least for full or non-coherent UL codebook based PUSCH and non-codebook based PUSCH. - FFS: Whether/how some of bits in the antenna ports field can be reused for other purpose for >4 ranks PUSCH. Table 7.3.1.1.2-12B: Antenna port(s), transform precoder is disabled, dmrs-Type=1, maxLength=2, rank = 5 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 2; 0-4; 2. 1-15; Reserved; Reserved; Reserved. Table 7.3.1.1.2-13B: Antenna port(s), transform precoder is disabled, dmrs-Type= 1, maxLength=2, rank = 6 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 2; 0,1,2,3,4,6; 2. 1-15; Reserved; Reserved; Reserved. Table 7.3.1.1.2-14B: Antenna port(s), transform precoder is disabled, dmrs-Type= 1, maxLength=2, rank = 7 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 2; 0,1,2,3,4,5,6; 2. 1-15; Reserved; Reserved; Reserved. Table 7.3.1.1.2-15B: Antenna port(s), transform precoder is disabled, dmrs-Type= 1, maxLength=2, rank = 8 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 2; 0,1,2,3,4,5,6,7; 2. 1-15; Reserved; Reserved; Reserved. Table 7.3.1.1.2-16B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=1, rank=5 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 3; 0-4. 1-15; Reserved; Reserved. Table 7.3.1.1.2-17B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=1, rank=6 Value; Number of DMRS CDM group(s) without data; DMRS port(s). 0; 3; 0-5. 1-15; Reserved; Reserved. Table 7.3.1.1.2-20B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=5 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 3; 0-4; 1. 1; 2; 0,1,2,3,6; 2. 12-31; Reserved; Reserved; Reserved. Table 7.3.1.1.2-21B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=6 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 3; 0-5; 1. 1; 2; 0,1,2,3,6,8; 2. 2-31; Reserved; Reserved; Reserved. Table 7.3.1.1.2-22B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=7 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 2; 0,1,2,3,6,7,8; 2. 1-31; Reserved; Reserved; Reserved. Table 7.3.1.1.2-23B: Antenna port(s), transform precoder is disabled, dmrs-Type= 2, maxLength=2, rank=8 Value; Number of DMRS CDM group(s) without data; DMRS port(s); Number of front-load symbols. 0; 2; 0,1,2,3,6,7,8,9; 2. 1-31; Reserved; Reserved; Reserved. Agreement For > 4 layers PUSCH with Rel.18 eType 1/eType 2 DMRS ports, support at least the same DMRS port combination(s) as that for rank = 5,6,7,8 for PDSCH with Rel.18 eType 1/eType 2 DMRS ports at least for full or non-coherent UL codebook based PUSCH and non-codebook based PUSCH. Agreement For two PTRS ports for partial/non-coherent PUSCH, PTRS-DMRS association for PUSCH with up to 8 layers is down selected from the following. - Alt.1: The size of PTRS-DMRS association field is 4-bit in DCI format 0_1/0_2. Table 1: PTRS-DMRS association for UL PTRS ports 0 and 1 Value of MSB; DMRS port; Value of LSB; DMRS port. 0; 1st DMRS port which shares PTRS port 0; 0; 1st DMRS port which shares PTRS port 1. 1; 2nd DMRS port which shares PTRS port 0; 1; 2nd DMRS port which shares PTRS port 1. 2; 3rd DMRS port which shares PTRS port 0; 2; 3rd DMRS port which shares PTRS port 1. 3; 4th DMRS port which shares PTRS port 0; 3; 4th DMRS port which shares PTRS port 1. - Alt.2: The size of PTRS-DMRS association field is 2-bit in DCI format 0_1/0_2. - The CW with the higher MCS is selected in case of two CWs. - If the MCS is the same for two CWs, the PTRS port is associated with the first CW. Table 2: PTRS-DMRS association for UL PTRS ports 0 and 1 Value of MSB; DMRS port; Value of LSB; DMRS port. 0; 1st DMRS port which shares PTRS port 0; 0; 1st DMRS port which shares PTRS port 1. 1; 2nd DMRS port which shares PTRS port 0; 1; 2nd DMRS port which shares PTRS port 1. - Alt.3: The size of PTRS-DMRS association field is 2-bit in DCI format 0_1/0_2. - For PUSCH with rank 5-8, 2-bit of antenna ports field is reused in addition to 2-bit PTRS-DMRS association in DCI format 0_1/0_2, and total 4-bit is used for PTRS-DMRS association. Table 1: PTRS-DMRS association for UL PTRS ports 0 and 1 Value of MSB; DMRS port; Value of LSB; DMRS port. 0; 1st DMRS port which shares PTRS port 0; 0; 1st DMRS port which shares PTRS port 1. 1; 2nd DMRS port which shares PTRS port 0; 1; 2nd DMRS port which shares PTRS port 1. 2; 3rd DMRS port which shares PTRS port 0; 2; 3rd DMRS port which shares PTRS port 1. 3; 4th DMRS port which shares PTRS port 0; 3; 4th DMRS port which shares PTRS port 1. - Alt.4: The size of PTRS-DMRS association field is 2-bit in DCI format 0_1/0_2. Table 2: PTRS-DMRS association for UL PTRS ports 0 and 1 Value of MSB; DMRS port; Value of LSB; DMRS port. 0; 1st DMRS port which shares PTRS port 0; 0; 1st DMRS port which shares PTRS port 1. 1; 2nd DMRS port which shares PTRS port 0; 1; 2nd DMRS port which shares PTRS port 1. Agreement For partial/non-coherent PUSCH with rank=5-8 transmission (i.e. non of the CWs is disabled) with one PTRS port, PTRS-DMRS association for PUSCH is the following. - The size of PTRS-DMRS association field is 2-bit in DCI format 0_1/0_2. - The CW with the higher MCS is selected in case of two CWs. - Note: in case of PUSCH retransmission, the initial MCS is used for CW selection. - If the MCS is the same for two CWs, the PTRS port is associated with the first CW. Table 7.3.1.1.2-25: PTRS-DMRS association for UL PTRS port 0 Value; DMRS port. 0; 1st scheduled DMRS port with the CW. 1; 2nd scheduled DMRS port with the CW. 2; 3rd scheduled DMRS port with the CW. 3; 4th scheduled DMRS port with the CW. Conclusion For “The CW with the higher MCS” in RAN1#112 agreement of PTRS-DMRS association field for full-coherent PUSCH with rank=5~8 PUSCH with one port PTRS, following is clarified. - Note: in case of PUSCH retransmission, the initial MCS is used for CW selection.