3GPP TSG RAN WG1 #114bis	R1-2308858

Xiamen, China, 9th – 13th October, 2023

Agenda item:		8.16.13

Source:	Nokia, Nokia Shanghai Bell

Title:	UE features for NR IoT NTN enhancements

WI code:	IoT_NTN_enh-Core

Document for:		Discussion and Decision

# Introduction

UE features for Rel-18 NR IoT NTN enhancementswere first discussed in RAN1#112bis-e, and discussion was continued in RAN1#113 and RAN1#114.  The rapporteur has compiled the UE feature groups and features for each WI and the outcome of previous discussion for these feature groups including Rel-18 IoT NTN enhancements is captured in [1]. In this contribution, we provide our inputs to the UE features for Rel-18 NR IoT NTN enhancements.

# Discussion

In RAN1#114 agreement was made that separate FGs (FG 44-1a/1b and FG 44-1e/44-1f) would be included for semi-static and dynamic HARQ feedback disabling; however further consideration was allowed for whether a separate FG for joint support of semi-static and dynamic HARQ feedback disabling would be supported.  The motivation for a joint FG is not clear and could create further confusion/redundancy when considering indication for both joint and independent support of FGs.  For these reasons, a separate FG for joint support is not needed.

Observation 1: 	A separate FG is not needed for joint support of semi-static and dyanmic HARQ feedback disabling.

For FG 2-3a, 2-3b, 2-4a and 2-4b, we propose confirming the addition of text, “during the initial access stage and in connected mode,” in component 1. to further clarify UE behavior for effective report of GNSS position fix time update to eNB for cases e.g. when UE moves.

Proposal 1: 	Confirm addition, of text indicating component 1 GNSS position fix is reported at least during initial access and while UE is in RRC connected mode in FG 2-3a and FG 2-4a.

For FG 2-4a and 2-4b, the following agreement was reached in RAN1#111 regarding GNSS position fix for IoT NTN in LTE:

Agreement

For GNSS measurement in RRC connected, if eNB aperiodically triggers connected UE to make GNSS measurement, UE can re-acquire GNSS position fix with a gap

- FFS details of gap configuration

The UE may re-acquire GNSS autonomously (when configured by the network) if UE does not receive eNB trigger to make GNSS measurement

- FFS based on configured timing

Based on this agreement it is clear that

- Aperiodic GNSS measurement is mandatory for a Rel18 IoT NTN UE. Although whether and when UE should do aperiodic GNSS measurement is based on eNB triggering, but if only eNB triggers an aperiodic GNSS measurement, UE should perform it.

- Autonomous GNSS measurement is optional for a Rel18 IoT NTN UE as UE “may” do autonomous GNSS measurement.

- Aautonomous GNSS position fix may be re-acquired if the UE does not receive a trigger from the eNB. For this reason it seems clear that the ability to receive a network trigger to re-acquire GNSS position fix is a pre-requisite for autonomous GNSS re-acquisition.

Proposal 2: For FG 2-4a and FG 2-4b include Rel-18 FG 2-3a and Rel-18 FG 2-3b respectively as pre-requisites.

The proposed revisions for discussion in RAN1#114bis can be found in Table 1.

Proposal 3: 	Adopt the proposed revisions captured in Table 1 regarding UE features for Rel-18 IoT NTN enhancements.

Table 1: Proposed revisions to UE feature groups support for Rel-18 IoT NTN

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs) | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. IoT_NTN_enh | 2-1a | Semi-static HARQ feedback disabling for eMTC CE mode B | 1. UE gets RRC configuration for disabling HARQ feedback per UE per process | Rel. 17 2-1 | Yes | N/A | Release 18 eMTC UE with CE mode B cannot disable HARQ feedback | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 1 in CE mode B Note: this applies to single-TB case [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-1b | Dynamic HARQ feedback disabling for eMTC CE mode B | 1. UE receives DCI indication to directly indicate / override RRC configuration for disabling HARQ feedback | Rel. 17 2-1 | Yes | N/A | Release 18 eMTC UE with CE mode B cannot disable HARQ feedback | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 3 in CE mode B Note: this applies to single-TB case [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-1d | Semi-static HARQ feedback disabling for eMTC CE mode A | 1. UE gets RRC configuration for disabling HARQ feedback per UE per process | Rel. 17 2-1 | Yes | N/A | Release 18 eMTC UE with CE mode A cannot disable HARQ feedback | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 1 in C mode A Note: this applies to single-TB case [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-2 | Semi-static HARQ feedback disabling for SPS PDSCH for eMTC CE Mode A | UE reports ACK/NACK for the first SPS PDSCH after activation if enabled, and follow per-process HARQ feedback enabled/disabled configuration otherwise | Rel. 17 2-1 | Yes | N/A | Release 18 eMTC UE Mode A cannot disable HARQ feedback for SPS PDSCH | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 1 in C mode A [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-1e | Semi-static HARQ feedback disabling for NB-IoT | 1. UE gets RRC configuration for disabling HARQ feedback per UE per process | Rel. 17 2-1b | Yes | N/A | Release 18 NB-IoT UE cannot disable HARQ feedback | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 1 Note: this applies to single-TB case [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-1f | Dynamic HARQ feedback disabling for NB-IoT | 1. UE receives DCI indication to directly indicate / override RRC configuration for disabling HARQ feedback | Rel. 17 2-1b | Yes | N/A | Release 18 NB-IoT UE cannot disable HARQ feedback | [Per UE/Per band] | No | No | Note: HARQ disabling with Option 3 Note: this applies to single-TB case [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-3a | GNSS position fix in RRC Connected state for eMTC—triggered | 1. UE reports GNSS position fix time duration for measurement at least during the initial access stage and in connected mode 2. UE receives eNB GNSS measurement trigger 4. UE re-acquires GNSS position fix within a configured gap 5. UE reports the remaining GNSS validity duration with MAC CE in connected mode | Rel. 17 2-1 | Yes | N/A | Release 18 eMTC UE cannot get triggered GNSS position fix in RRC Connected state | [Per UE/Per band] | No | No | [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] Note: This applies to non-DRX | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-4a | GNSS position fix in RRC Connected state for eMTC—autonomous | 1. UE re-acquires GNSS autonomously (when configured by the network) if it does not receive eNB GNSS measurement trigger 2. UE reports GNSS position fix time duration for measurement at least during the initial access stage and in connected mode 3. UE reports the remaining GNSS validity duration with MAC CE in connected mode | Rel. 18 2-3a Rel. 17 2-1 | Yes | N/A | Release 18 eMTC UE cannot get autonomous GNSS position fix in RRC Connected state | [Per UE/Per band] | No | No | Note: This applies to non-DRX [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-3b | GNSS position fix in RRC Connected state for NB-IoT—triggered | 1. UE reports GNSS position fix time duration for measurement at least during the initial access stage and in connected mode 2. UE receives eNB GNSS measurement trigger 4. UE re-acquires GNSS position fix within a configured gap 5. UE reports the remaining GNSS validity duration with MAC CE in connected mode | Rel. 17 2-1b | Yes | N/A | Release 18 NB-IoT UE cannot get triggered GNSS position fix in RRC Connected state | [Per UE/Per band] | No | No | Note: This applies to non-DRX [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |
| 2. IoT_NTN_enh | 2-4b | GNSS position fix in RRC Connected state for NB-IoT—autonomous | 1. UE re-acquires GNSS autonomously (when configured by the network) if it does not receive eNB GNSS measurement trigger 2. UE reports GNSS position fix time duration for measurement at least during the initial access stage and in connected mode 3. UE reports the remaining GNSS validity duration with MAC CE in connected mode | Rel. 18 2-3b, Rel. 17 2-1b |  |  | Release 18 NB-IoT UE cannot get autonomous GNSS position fix in RRC Connected state | [Per UE/Per band] | No | No | Note: This applies to non-DRX [Note: RAN1 kindly asks RAN2 to design signalling such that GSO/NGSO differentiation is possible] | Optional with capability signalling |

# Summary

The following proposals are for discussion on UE features for Rel-18 IoT NTN enhancements:

Observation 1: 	A separate FG is not needed for joint support of semi-static and dyanmic HARQ feedback disabling.

Proposal 1: 	Confirm addition, of text indicating component 1 GNSS position fix is reported at least during initial access and while UE is in RRC connected mode in FG 2-3a and FG 2-4a.

Proposal 2: 	For FG 2-4a and FG 2-4b include Rel-18 FG 2-3a and Rel-18 FG 2-3b respectively as pre-requisites.

Proposal 3: 	Adopt the proposed revisions captured in Table 1 regarding UE features for Rel-18 IoT NTN enhancements.

References

- R1-2308518, ”Updated RAN1 UE features list for Rel-18 LTE after RAN1#114,” Moderators (AT&T, NTT DOCOMO, INC.)