3GPP TSG RAN WG1 #114bis									       R1-2309101

Xiamen, China, October 9th – October 13th, 2023

Source:	vivo

Title:	Discussion on Rel-18 XR UE features

Agenda Item:	8.16.6

Document for:	Discussion and Decision

- Introduction

In RAN1#114 meeting, UE features for Rel-18 XR were discussed. Following initial input on UE features for XR was introduced, while the FFS parts in yellow need further discussion [1]. In this contribution, we provide our view on UE features for XR.

| 50. NR_XR_Enh | 50-1 | Multi-PUSCHs for Configured Grant | 1. Determination of time-domain resource allocation for CG-PUSCHs associated to a multi-PUSCHs CG FFS whether to separate this FG for type-1 and type-2 CG FFS whether to separate this FG for multiple CG configurations FFS whether to separate this FG for shared spectrum FFS whether to report maximum supported number of configured CG-PUSCH TOs in one CG period |  | Yes | N/A | UE is not able to support Multi-PUSCHs per one period in Configured grant in licensed band |  |  |  |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50. NR_XR_Enh | 50-2 | UCI indication of unused CG-PUSCH transmission occasions | 1. Multiplexing of the Unused transmission occasions UCI (UTO-UCI) on a CG-PUSCH FFS whether to merge this FG into FG 50-1 FFS whether to separate this FG for UTO-UCI multiplexing with HARQ-ACK |  | Yes | N/A | UE is not able to indicate the unused resources in CG |  |  |  |  |  | Optional with capability signaling |

- Discussion

  - Multiple CG PUSCH occasions per CG period

| 50. NR_XR_Enh | 50-1 | Multi-PUSCHs for Configured Grant | 1. Determination of time-domain resource allocation for CG-PUSCHs associated to a multi-PUSCHs CG FFS whether to separate this FG for type-1 and type-2 CG FFS whether to separate this FG for multiple CG configurations FFS whether to separate this FG for shared spectrum FFS whether to report maximum supported number of configured CG-PUSCH TOs in one CG period |  | Yes | N/A | UE is not able to support Multi-PUSCHs per one period in Configured grant in licensed band |  |  |  |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - Regarding the 1st FFS for FG 50-1, we think FG 50-1 can be applied to either Type 1 CG or Type 2 CG. If different maximum numbers of CG PUSCH occasions per CG period for Type 1 and Type 2 CG need to be considered, it is also acceptable that maximum numbers of CG PUSCH occasions per CG period for Type 1 CG and Type 2 CG are separately reported.

Proposal 1: For FG 50-1, support one of the following

- Alt-1: FG 50-1 can be applied to both Type 1 CG and Type 2 CG.

- Alt-2: FG 50-1 can be separated for Type 1 CG or Type 2 CG, where maximum number of CG PUSCH occasions per CG period for Type 1 CG or Type 2 CG is separately reported.

  - Regarding the 2nd FFS for FG 50-1, it was discussed in RAN1#114 meeting, and the following options were proposed to be down-selected.

Agreement Select one of the following options: - Option 1: Introduce a new capability to indicated maximum number of multi-PUSCH CG configurations (at least 2) per BWP of a serving cell and across all serving cells - FG 50-1 as pre-requisite. - FG 11-9 NOT as pre-requisite - Option 2: Introduce a new capability to indicated maximum number of multi-PUSCH CG configurations (at least 2) per BWP of a serving cell and across all serving cells. The maximum number should not exceed the corresponding maximum number of CG configurations indicated by FG 11-9. - FG 50-1 as pre-requisite. - FG 11-9 as pre-requisite - Option 3: Maximum number of multi-PUSCH CG configuration per BWP of a serving cell is one.

  - From our perspective, we support Option 2. If CG resources are configured to serve XR UL traffic, potentially with large and variable packet size, one or multiple legacy CG configurations, and/or one or multiple multi-PUSCH CG configurations can be configured as the CG resources, up to network configuration. In this regard, it is natural and simple that all CG resources supported by the UE are indicated by FG 11-9, including the maximum number of CG configurations supported by the UE, some or all of which may be further enhanced to support configuring multiple CG PUSCH occasions per CG period, i.e. as multi-PUSCH CG configurations. In other words, a new capability can be introduced to indicate the maximum number of multi-PUSCH CG configurations, from the maximum number of CG configurations indicated by FG 11-9. Therefore, the maximum number of multi-PUSCH CG configurations should not exceed the maximum number of CG configurations indicated by FG 11-9, i.e., Option 2 is selected.

Proposal 2: A separate UE capability is introduced to indicate maximum number of multi-PUSCH CG configurations (at least 2) per BWP of a serving cell and across all serving cells, with FG 50-1 and FG 11-9 as pre-requisite.

  - Regarding the 3rd FFS for FG 50-1, we think there is no need to introduce FG 50-1 for shared spectrum. Note that for shared spectrum, FG 10-28 was introduced in Rel-16 NR-U, where a UE can report to support the configuration of CG resources with multiple slots per CG period and multiple PUSCHs per slot. Hence, configuring multiple CG PUSCH occasions per CG period can already be supported by FG 10-28 for shared spectrum, and there is no need to introduce multi-PUSCH CG for shared spectrum with similar or redundant functionalities.

Proposal 3: FG 50-1 is per band reported and is applied to licensed spectrum only.

  - Dynamic indication of unused CG PUSCH occasion(s) based on UCI

| 50. NR_XR_Enh | 50-2 | UCI indication of unused CG-PUSCH transmission occasions | 1. Multiplexing of the Unused transmission occasions UCI (UTO-UCI) on a CG-PUSCH FFS whether to merge this FG into FG 50-1 FFS whether to separate this FG for UTO-UCI multiplexing with HARQ-ACK |  | Yes | N/A | UE is not able to indicate the unused resources in CG |  |  |  |  |  | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - Regarding the 1st FFS, we don’t support to merge this FG 50-2 into FG 50-1. The reasons include at least the following:

- According to the WID objectives for Rel-18 WI [2], there is no dependency between multi-PUSCH CG and UTO-UCI. So they are separate enhancements for XR.

- According to agreements in RAN1, a UTO-UCI that is carried by a transmitted CG PUSCH is applicable to the Nu consecutive and valid CG PUSCH occasions starting from the end of the transmitted CG PUSCH, where Nu is one value from (3, …, 8). The applicable CG PUSCH occasions are not limited to be within one CG period, i.e., they can belong to multiple CG periods. In this regard, UTO-UCI indication can also be applicable to legacy CG configurations with only one CG PUSCH occasion per CG period.

  - Therefore, we think FG 50-2 should be a separate capability from FG 50-1, i.e., there should be no dependency assumed between FG 50-1 and FG 50-2.

Proposal 4: FG 50-2 is supported separately from FG 50-1, i.e., no dependency between FG 50-2 and FG 50-1.

  - PDCCH monitoring resumption after UL NACK

  - According to the discussion in RAN#101 meeting, following proposal was endorsed on PDCCH monitoring resumption after UL NACK [2][4].

Proposal 1: RAN to agree to introduce the feature of "PDCCH monitoring resumption after UL NACK" in Rel-18 XR. - Following TP for TS 38.213 is endorsed. - A new RRC parameter (e.g., PdcchMornitoringResumptionAfterNack) is introduced. - An optional UE capability for the feature is introduced.

  - Based on the RAN conclusion, a new UE capability for the feature will be introduced. Besides, since PDCCH monitoring resumption after UL NACK should be operated when PDCCH skipping is enabled, this new capability needs FG 29-3a or FG 29-3d as the pre-requisite.

Proposal 5: Support the following UE capability for PDCCH monitoring resumption after UL NACK.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50. NR_XR_Enh | 50-3 | PDCCH monitoring resumption after UL NACK | Support of PDCCH monitoring resumption after UL NACK for PDSCH scheduled by DCI | One of {29-3a, 29-3d} | Yes | No | UE does not support PDCCH monitoring resumption after UL NACK | Per band | No | No | N/A |  | Optional with capability signalling |

- Conclusion

In this contribution, UE features on XR enhancements, including CG enhancement and PDCCH monitoring resumption after UL NACK, are discussed in details, with the following proposals and observations:

Proposal 1: For FG 50-1, support one of the following

- Alt-1: FG 50-1 can be applied to both Type 1 CG and Type 2 CG.

- Alt-2: FG 50-1 can be separated for Type 1 CG or Type 2 CG, where maximum number of CG PUSCH occasions per CG period for Type 1 CG or Type 2 CG is separately reported.

Proposal 2: A separate UE capability is introduced to indicate maximum number of multi-PUSCH CG configurations (at least 2) per BWP of a serving cell and across all serving cells, with FG 50-1 and FG 11-9 as pre-requisite.

Proposal 3: FG 50-1 is per band reported and is applied to licensed spectrum only.

Proposal 4: FG 50-2 is supported separately from FG 50-1, i.e., no dependency between FG 50-2 and FG 50-1.

Proposal 5: Support the following UE capability for PDCCH monitoring resumption after UL NACK.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 50. NR_XR_Enh | 50-3 | PDCCH monitoring resumption after UL NACK | Support of PDCCH monitoring resumption after NACK for PDSCH scheduled by DCI | One of {29-3a, 29-3d} | Yes | No | UE does not support PDCCH monitoring resumption after UL NACK | Per band | No | No | N/A |  | Optional with capability signalling |

# References

- R1-2308513	Session Notes for 9.16.10	Ad-Hoc Chair (NTT DOCOMO)

- RP-223502, New WID on XR Enhancements for NR, RAN #98-e, Dec. 12 - 16, 2022.

- Chairman’s Notes, RAN#101.

- RP-231820	PDCCH monitoring resumption after UL NACK	vivo, MediaTek, Qualcomm, ZTE, Sanechips, Ericsson, Huawei, HiSilicon, RAN#101