- 3GPP TSG RAN WG1 #114 bis                                                                                    R1-2309155

Xiamen, China, October 9th - October 13th, 2023

Source:	ZTE

Title:	Discussion on the Self-evaluation results for IoT NTN

Agenda Item:	8.15.3

Document for:    Discussion

- Introduction

In RAN1#114 meeting, following agreement has been achieved for IoT NTN self-evaluation [1]. In this contribution, the simulation results for IoT-NTN self-evaluation are provided.

Agreement For IoT NTN Connection Density LLS: - Agree 10% BLER as target for throughput for NB-IoT Full Buffer and eMTC Full Buffer - Companies to use QPSK-π/4 as well as BPSK-π/2 in evaluations for NB-IoT Full Buffer. - Each company to provide SINR - user spectral efficiency (SE) mapping graph for NB-IoT Full Buffer and eMTC Full Buffer, where: - SE = nominal SE × (1-BLER)

- Discussion on evaluation results

  - Connection density

The SLS and LLS simulation assumptions are provided in [2]. Only the parameters marked with “to be reported” are quoted below for quick reference.

Table 1 Simulation parameters to be reported

| Parameter | Value |
| --- | --- |
| Simulation bandwidth | 15kHz (Single tone) |
| TBS | 256 |
| Modulation order | BPSK-π/2, QPSK-π/4 |
| Number of Resource units | 2,3,4,5,6,8,10 |
| Number of repetition | 1,2,4,8,16 |

The connection density of NB-IoT are evaluated via the methodology (i.e., using full buffer system level simulation with link level simulation) as defined in Report ITU-R M.2412. The UL SINR distribution obtained by SLS for FRF=1 and FRF=3 is shown in Figure 1.

Figure 1 the Uplink SINR distribution of NB-IoT

The 99th percentile of the delay per user D and connection density are shown in Table 2 to Table 3, respectively. The traffic model of packet arrival rate of 1 message / 2 hours / device is used.

Table 2 99th percentile delay recorded in Step 3 under SINRi (s)

| Physical channel | Configuration |  |  | 99th percentile delay | Requirement |
| --- | --- | --- | --- | --- | --- |
|  | FRF | repetitions | Modulation order |  |  |
| NB-IoT | 1 | 1,2,4,8,16 | MCS sweeping with target BLER @10% | 1.5059s | <10s |
|  | 1 | 1 | BPSK-π/2 with fixed MCS | 0.1076s | <10s |
|  | 3 | 1 | QPSK-π/4, MCS sweeping with target BLER @10% | 0.0441s | <10s |

Table 3 Connection density recorded in Step 7 under SINRi (devices per km2)

| Physical channel | Configuration |  |  | Connection density | Requirement |
| --- | --- | --- | --- | --- | --- |
|  | FRF | repetitions | Modulation order |  |  |
| NB-IoT | 1 | 1,2,4,8,16 | MCS sweeping with target BLER @10% | 124 | >500 |
|  | 1 | 1 | BPSK-π/2 with fixed MCS | 783 | >500 |
|  | 3 | 1 | QPSK-π/4, MCS sweeping with target BLER @10% | 2631 | >500 |

For FRF=1, 10RU with BPSK-π/2 is used to achieve satisfactory connection density. However, the resultant BLER is larger than 10%. In addition, MCS sweeping with different repetition number is also used to achieve the target BLER of 10%. However, due to the larger time domain resource used, the density requirement cannot be fulfilled.

For FRF=3, different number of RU is used with MCS sweeping to achieve the target BLER of 10%. And the density requirement can be fulfilled.

Based on the CDF of pre-processing SINR, the spectral efficiency is calculated by the following formula with the corresponding BLER by LLS. In the formula, 256 is the TBS in bit. N is the number of RU, and 1RU is 8ms*15kHz.

The SINR - SE mapping graphs for FRF=1 and FRF=3 are shown in Figure 2.

Figure 2 Spectral efficiency for NB-IoT

Observation 1: For FRF=3, the connection density requirement can be fulfilled with target BLER of 10%.

Observation 2: For FRF=1, the connection density requirement can be fulfilled without target BLER of 10% restriction.

Proposal 1: Capture the evaluation results in Table 2 ~ Table 3 into the TR 37.911.

- Conclusions

In this contribution, the evaluation including analytical study and simulations are presented with following proposals:

Observation 1: For FRF=3, the connection density requirement can be fulfilled with target BLER of 10%.

Observation 2: For FRF=1, the connection density requirement can be fulfilled without target BLER of 10% restriction.

Proposal 1: Capture the evaluation results in Table 2 ~ Table 3 into the TR 37.911.

Reference

- RAN1 Chair's notes, RAN1#114.

- R1-2309153, Discussion   on the evaluation methodology of the self-evaluation, ZTE.

- R1-2308748, [Post-114-ITU-Sat-01] Email discussion on link budget template, RAN1#114, Moderator.

Appendix

- A-1: Link budget

In RAN1#114 meeting, link budget template is provided in the attached .xls to R1-2308233. The link budget for different scenarios are provided below. The related simulation parameters are listed below as suggested in [3].It is observed that the coverage margin at minimum elevation angle is larger than 0, which means the link budget is satisfactory.

Table A.1 Link budget for NB-IoT NTN

| NPDSCH |  |  |  | NPUSCH fmt1 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TBS | BW | #reps | Target BLER | TBS | BW | #reps | Target BLER |
| 256 | 180kHz | 8 | 10% | 256 | 15kHz | 32 | 10% |
| Required SNR = -0.6dB Coverage margin at elevation angle 30° = 0.2dB |  |  |  | Required SNR = -1.65dB Coverage margin at elevation angle 30° = 13.2dB |  |  |  |

- A-2: Calibration results

The calibration curves aligned with TR 38.821 calibration case 9 or case 10 for SLS are provided below. Figure 3 and Figure 4 show the DL and UL calibration curves (i.e., coupling loss, SINR and SIR) for FRF=1 and FR = 3, respectively.

Figure 3 Calibration results for FRF=1

Figure 4 Calibration results for FRF=3