3GPP TSG RAN WG1 #114bis   	                           R1-2309165

Xiamen, China, October 9th – 13th, 2023

Agenda item:		8.1.3.2

Source:		LG Electronics

Title:			SRS enhancement targeting TDD CJT and 8 TX operation

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss on the remaining issues of SRS for 8 Tx operation.

Discussion on SRS for 8 Tx operation

In the last meeting, the following proposal[1] was discussed for collision handling for TDMed 8-port SRS.

Proposal 3.3A: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ and with TDM factor s = 2, the 8 ports being fully[/partially] coherent, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), and when the SRS transmission on a subset of the s OFDM symbols within a group of {1, 2, …, s} is dropped, the UE drops the SRS transmission on the rest of OFDM symbols within the group of {1, 2, …, s}.

When the 8-port SRS resource is mapped onto m≥2 OFDM symbols in a slot, a subset of s OFDM symbols within a group of {1, 2, …, s} may collide with other high priority UL channel/RS. There is a remaining issue regarding whether to drop the rest of OFDM symbols within the group of {1, 2, …, s}.

In legacy, only the collided symbol is partially dropped. However, if we consider this from the perspective of port rather than symbol, it means dropping of entire ports. In legacy, since multiple ports are not mapped onto multiple symbols in case of codebook based SRS, dropping a single symbol does not result in partial port sounding. However, in the case of TDMed 8 Tx SRS resource, partial dropping would mean to support partial SRS port sounding which is not technically necessary and wasting UE Tx power/battery. In addition, in case of repetition factor R=1, there should be a problem of determining TPMI from gNB side only with channel information regarding the partial ports(i.e., 4 ports). More importantly, UE does not need to waste its Tx power by dropping unnecessary SRS transmission, i.e. dropping the rest of OFDM symbols within the group of {1, 2, …, s}.

For these reasons, we support that the UE drops the SRS transmission on the rest of OFDM symbols within the group of {1, 2, …, s} when the SRS transmission on a subset of the s OFDM symbols within a group of {1, 2, …, s} is dropped.

On the other hands, in the last meeting, there was a discussion regarding the timeline issue. The issue occurs when SRS has been scheduled to be sounded on a certain time but gNB attempts to schedule dynamic resource using DCI which has higher priority than SRS during the preparation time for the SRS transmission. In this case, if the dynamic resource with higher priority is partially collided with the SRS and the UE has no sufficient time to cancel the transmission of remaining symbol of the SRS, the SRS may be partially dropped. But this case can be easily avoided by NW scheduler implementation, i.e. UE does not expect collisions between SRS and higher priority resources during the preparation time for the SRS transmission.

Proposal #1: Do not support to introduce partial port drop for CB SRS. When the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), and when the SRS transmission on a subset of the s OFDM symbols within a group of {1, 2, …, s} is dropped, the UE drops the SRS transmission on the rest of OFDM symbols within the group of {1, 2, …, s}.

Conclusion

In this contribution, we discuss about SRS enhancement targeting 8 TX operation. We provide the following proposal.

Proposal #1: Do not support to introduce partial port drop for CB SRS. When the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), and when the SRS transmission on a subset of the s OFDM symbols within a group of {1, 2, …, s} is dropped, the UE drops the SRS transmission on the rest of OFDM symbols within the group of {1, 2, …, s}.

Reference

[1] Chair’s note RAN1 #114