3GPP TSG RAN WG1 #114bis		R1-2309267

Xiamen, China, October 9th – October 13th, 2023

Agenda Item:	7.2

Source:	Google

Title:	Discussion on PUCCH transmission occasion

Document for:	Discussion/Decision

# Introduction

In this contribution, we provide some discussion on the PUCCH transmission occasion.

# Discussion

Currently, a PUCCH transmission occasion is defined as the consecutive symbols for PUCCH within a slot, according to the following text in 38.213.

A PUSCH/PUCCH/SRS/PRACH transmission occasion i is defined by a slot index ns,fμ within a frame with system frame number SFN, a first symbol S within the slot, and a number of consecutive symbols L. For a PUSCH transmission with repetition Type B, a PUSCH transmission occasion is a nominal repetition [6, TS 38.214].

The PUCCH power control is performed per PUCCH transmission occasion. However, in Rel-17, the power control parameters for different PUCCH repetitions can be different, since different PUCCH-spatialRelationInfo can be applied to different PUCCH repetitions based on cyclic mapping or sequential mapping as shown in Figure 1. Then for intra-slot PUCCH repetition, it is possible that different power control parameters can be applied to different PUCCH repetitions. Therefore, similar to PUSCH repetition Type B, at least for the PUCCH transmission occasion for intra-slot PUCCH repetition with more than one PUCCH-spatialRelationInfo applied should be defined in a per repetition level.

Figure 1: Mismatch between current definition of PUCCH transmission occasion and granularity for the Rel-17 mTRP PUCCH power control parameter configuration

Proposal 1: Endorse CR R1-2309268 to clarify that for intra-slot PUCCH repetition with more than one PUCCH-spatialRelationInfo applied, a PUCCH transmission occasion should be defined as one PUCCH repetition.

# Conclusion

In this contribution, we provided discussion on the definition of a PUCCH transmission occasion. Based on the discussion, the following is provided.

Proposal 1: Endorse CR R1-2309268 to clarify that for intra-slot PUCCH repetition with more than one PUCCH-spatialRelationInfo applied, a PUCCH transmission occasion should be defined as one PUCCH repetition.