3GPP TSG RAN WG1 Meeting #114bis	R1-2309279

Xiamen, China, October 9th - 13th, 2023

Agenda Item:	8.8.3

Source:	NEC

Title:	Maintenance on dynamic switching between DFT-S-OFDM and CP-OFDM

Document for:	Discussion and Decision

1	Introduction

In RAN1#114 discussion [1] of dynamic switching between DFT-S-OFDM and CP-OFDM, following agreements are achieved.

Agreement

For PUSCH scheduled by DCI format 0_1/0_2 with dynamic waveform switching indication field configured, and useInterlacePUCCH-PUSCH is not configured, downselect between following options:

- Option 1 (configuration restriction with error case handling):

  - UE does not expect resourceAllocation set to resourceAllocationType0.

  - If DFT-S-OFDM is indicated and resourceAllocation set to dynamicSwitch, UE does not expect MSB of FDRA field set to 0.

- Option 2 (UE only uses resourceAllocation if CP-OFDM is indicated):

  - If DFT-S-OFDM is indicated, UE applies type 1 resource allocation.

  - If CP-OFDM is indicated, UE applies resource allocation according to resourceAllocation IE.

  - Size of FDRA field is aligned between size for type 1 resource allocation and size according to resourceAllocation IE.

Agreement

For PUSCH scheduled by DCI format 0_1/0_2 with dynamic waveform switching indication field configured, downselect between following options:

- Option 1 (configuration restriction with error case handling):

  - UE does not expect dmrs-Type to be set to type2.

- Option 2 (UE only uses dmrs-Type if CP-OFDM is indicated):

  - If DFT-S-OFDM is indicated, UE applies DMRS type 1.

  - If CP-OFDM is indicated, UE applies DMRS type according to dmrs-Type.

Agreement

Introduce two new RRC parameters for configuration of DWS field in DCI formats 0_1/0_2:

- Value range is {enabled, disabled} for each of DCI format 0_1 and DCI format 0_2:

  - “enabled” means that DWS field is present in the DCI format and UE follows DWS field.

  - “disabled means that DWS field is not present and UE follows legacy parameter (transformPrecoder) when scheduled using the DCI format.

In this paper, we discuss and give our views on the remaining issues of dynamic switching between DFT-S-OFDM and CP-OFDM.

2	Discussion

2.1	Field interpretation

In RAN1#112 discussion, two options are provided for interpreting the bit field when dynamic waveform is indicated in DCI. For configuration restriction with error case handling, it may restrict the configuration of frequency domain resource allocation type and DMRS type when OFDM is used for dynamic waveform change, e.g. RA type 0 and DMR type 2 would not be used for OFDM waveform. Since 1-bit field has indicated the waveform used for the DCI, UE could interpret the meaning of the bit field based on the combined information of configuration and waveform. So option 2 can provide the flexibility on usage of dynamic waveform without increasing much complexity on DCI decoding, we support option 2.

Proposal 1: For FDRA field in DCI format 0_1/0_2 with dynamic waveform switching indication field configured, support option 2, i.e. UE only uses resourceAllocation if CP-OFDM is indicated.

Proposal 2: For DMRS field in DCI format 0_1/0_2 with dynamic waveform switching indication field configured, support option 2, i.e. UE only uses dmrs-Type if CP-OFDM is indicated.

2.2	Transform precoder indicator

For the first version of TS38.212, transform precoder indicator field in DCI is specified as following.

-	Transform precoder indicator – 0 or 1 bit

-	1 bit if the higher layer parameter dynamicTransformPrecoderIndicationDCI-0-1 is configured to 'enabled ' and if the UE is configured to monitor DCI format 0_1 with CRC scrambled by C-RNTI or MCS-C-RNTI, where the bit value of 0 indicates that transform precoder is enabled and the bit value of 1 indicates that transform precoder is disabled.

-	0 bit otherwise.

Based on the above specification, transform precoder indicator is absent if the higher layer parameter dynamicTransformPrecoderIndicationDCI-0-1 is configured to 'enabled ' and if the UE is configured to monitor DCI format 0_1 with CRC scrambled by CS-RNTI or SP-CSI-RNTI. However, DCI size for a format should be aligned by different scrambled RNTI values. In this sense, the presence or absence of the transform precoder indicator field is better to be regardless of scrambled RNTI values. On the other hand, for DCI scrambled by CS-RNTI with NDI=0 and SP-CSI-RNTI, the present field should be reserved since dynamic waveform switching is not support per agreements.

Proposal 3: Presence or absence of the transform precoder indicator field is regardless of scrambled RNTI values.

Proposal 4: Adopt the following TP:

-	Transform precoder indicator – 0 or 1 bit

-	1 bit if the higher layer parameter dynamicTransformPrecoderIndicationDCI-0-1 is configured to 'enabled '.

- and if the UE is configured to monitor DCI format 0_1 with CRC scrambled by C-RNTI or MCS-C-RNTI or CS-RNTI with NDI=1, where the bit value of 0 indicates that transform precoder is enabled and the bit value of 1 indicates that transform precoder is disabled.

- otherwise, the bit is reserved.

-	0 bit otherwise.

3	Conclusion

In this contribution, we give our views on dynamic switching between DFT-S-OFDM and CP-OFDM, and propose that:

Proposal 1: For FDRA field in DCI format 0_1/0_2 with dynamic waveform switching indication field configured, support option 2, i.e. UE only uses resourceAllocation if CP-OFDM is indicated.

Proposal 2: For DMRS field in DCI format 0_1/0_2 with dynamic waveform switching indication field configured, support option 2, i.e. UE only uses dmrs-Type if CP-OFDM is indicated.

Proposal 3: Presence or absence of the transform precoder indicator field is regardless of scrambled RNTI values.

Proposal 4: Adopt the following TP:

-	Transform precoder indicator – 0 or 1 bit

-	1 bit if the higher layer parameter dynamicTransformPrecoderIndicationDCI-0-1 is configured to 'enabled '.

- and if the UE is configured to monitor DCI format 0_1 with CRC scrambled by C-RNTI or MCS-C-RNTI or CS-RNTI with NDI=1, where the bit value of 0 indicates that transform precoder is enabled and the bit value of 1 indicates that transform precoder is disabled.

- otherwise, the bit is reserved.

-	0 bit otherwise.

References

[1]	Draft_Minutes_report_RAN1#114_v020