3GPP TSG RAN WG1 #114bis                                                                                            R1-2309418

Xiamen, China, October 9th - October 13th, 2023

Source:	Xiaomi

Title:	Discussions on RAN2 LS on SL positioning MAC agreements

Agenda Item:	5

Document for:	Decision

- Introduction

In the LS from RAN2 [1], RAN2 has provided the RAN2 agreements w.r.t SL-PRS priority, SL-PRS and CBR measurements, and would like to RAN1 take them into account in the future work and provide feedbacks on relevant agreements. In addition, RAN2 also would like to ask whether the two cases where higher layer and/or lower layer signalling can provide the SL-PRS priority when SL-PRS is triggered by peer UE’s lower layer signalling are possible.

In this contribution, we will provide our views on these issues.

- Discussions

2.1	RAN1 progress on congestion control

In the LS from RAN2, following RAN2 agreement is provided to RAN1.

Agreement Support CBR measurement on both shared and dedicated resource pool for SL-PRS transmission.

In the previous RAN1 meeting, RAN1 has already worked on congestion control for both dedicated and shared resource pool. The following agreements have been achieved on CBR measurement for dedicated resource pool.

Agreement For Scheme 2 SL-PRS resource allocation, with regards to the congestion control for a dedicated RP, the following modifications are supported: - Modification 1: For the definition of SL PRS CR and CBR: - Alt. 2: redefine CBR/CR by considering the SL-PRS resource allocation/configuration. Agreement For Scheme 2 SL-PRS resource allocation, with regards to the congestion control for a dedicated RP, the following modifications are supported: - Modification 2: For the evaluation of RSSI used in the CBR definition: - SL-RSSI is measured on a slot configured for transmission of PSCCH and SL-PRS - A single SL-RSSI is measured on symbols with both SL-PRS and PSCCH Agreement For Scheme 2 SL-PRS resource allocation, with regards to the congestion control for a dedicated RP, the following modifications are supported: - For the CR and CBR measurement time window size, - it can be separately configured for a dedicated resource pool and could take the legacy values

Furthermore, the following conclusion has been achieved on CBR measurement for shared resource pool.

Conclusion For Scheme 2 SL-PRS resource allocation, with regards to the congestion control for a shared RP, CBR and CR mechanisms from Rel.16 NR SL are reused. - Add this agreement in the LS related to the priority handling

Proposal 1:  RAN1 confirms that CBR measurement on both shared and dedicated resource pool for SL-PRS transmission has already been discussed and supported during the previous RAN1 meeting.

Further, the following two agreements may have RAN2 impacts, in which congestion control can restrict at least the range of some parameters of SL PRS transmission, which is similar to NR sidelink communication.

Agreement In Scheme 2, - For a dedicated resource pool for positioning, - congestion control can restrict at least the following range of parameters for SL PRS configuration per resource pool by CBR and priority: - Maximum SL PRS transmission power - Maximum Number of SL PRS (re-)transmissions - Discuss further the following four SL PRS transmission parameters: - Minimum Periodicity of SL PRS - Maximum Number of SL PRS resources in a slot - Maximum comb-size of a SL PRS resource in a slot - Maximum Number of OFDM symbols of a SL PRS resource in a slot - For congestion control similar to legacy, the CR limits are (pre)-configured per priority in a resource pool - Note: Similar to SL communication how to achieve the CR limit is left to UE implementation. - For a shared resource pool for positioning, the SL PRS can share the same restriction of PSSCH without specific enhancement in addition to what is already specified. Agreement In Scheme 2, with regards to the congestion control for SL PRS: - SL-PRS congestion processing time: based on both SCS and UE capability, similar to legacy - The maximum number of CBR ranges for SL positioning is 8 - Number of CBR levels is 16 - CBR measurement for SL PRS can be reported to gNB FFS: Whether it is needed to be reported to LMF or another UE

Therefore, RAN1 may need to ask RAN2 to take the agreements in congestion control in the future work.

Proposal 2: RAN1 respectfully ask RAN2 to take the following agreements in congestion control in the future work.

Agreement In Scheme 2, - For a dedicated resource pool for positioning, - congestion control can restrict at least the following range of parameters for SL PRS configuration per resource pool by CBR and priority: - Maximum SL PRS transmission power - Maximum Number of SL PRS (re-)transmissions - Discuss further the following four SL PRS transmission parameters: - Minimum Periodicity of SL PRS - Maximum Number of SL PRS resources in a slot - Maximum comb-size of a SL PRS resource in a slot - Maximum Number of OFDM symbols of a SL PRS resource in a slot - For congestion control similar to legacy, the CR limits are (pre)-configured per priority in a resource pool - Note: Similar to SL communication how to achieve the CR limit is left to UE implementation. - For a shared resource pool for positioning, the SL PRS can share the same restriction of PSSCH without specific enhancement in addition to what is already specified. Agreement In Scheme 2, with regards to the congestion control for SL PRS: - SL-PRS congestion processing time: based on both SCS and UE capability, similar to legacy - The maximum number of CBR ranges for SL positioning is 8 - Number of CBR levels is 16 - CBR measurement for SL PRS can be reported to gNB FFS: Whether it is needed to be reported to LMF or another UE

2.2	Reply on the question from RAN2

According to [1], a question below is included in the LS and RAN2 has asked RAN1 to provide the corresponding feedback.

To RAN1: ACTION: RAN2 would like to ask RAN1 whether the following cases are possible - Higher layer provides the SL-PRS priority when SL-PRS is triggered by peer UE’s lower layer’s signalling - Lower layer signalling provides the SL-PRS priority when SL-PRS is triggered by peer UE’s lower layer signalling

For the first case, in our point of view, according to the following RAN1 agreements, we think it is possible but it should be clarified the SL-PRS priority comes from the UE’s own higher layer, not the peer UE’s. It also should be clarified that in this case the SL-PRS priority can be different from the peer UE.

Agreement For the scheme 2 sensing-based resource allocation, - Rel-16/17 resource (re)-selection procedure is reused for SL-PRS in the shared resource pool. - Study if/what changes are needed - Rel-16[/17] resource (re)-selection procedure with periodic and without periodic reservations is the starting point for the design of SL-PRS in the dedicated resource pool. - Study what changes, if any, are needed at least with regards to the following: sensing window, resource selection window, reservation interval, Resource exclusion mechanism (e.g. definition of resource set for SL-PRS, how RSRP is measured, etc) - From RAN1 perspective, priority value for SL PRS should be provided by higher layers from Tx UE perspective

Proposal 3: RAN1 confirms that it is possible for higher layer provides the SL-PRS priority when SL-PRS is triggered by peer UE’s lower layer’s signalling.

- In this case, the SL-PRS priority can be different from the peer UE.

For the second case, RAN1 has no conclusion until now on whether the priority can come from the peer UE’s low layer signalling, additional discussion and agreement may be needed. It also should be clarified that in this case the SL-PRS priority shall be same as the peer UE because in SCI format 1_B and 2_D there is only 1bit for SL-PRS request, i.e., an additional SL-PRS priority cannot be notified in SCI.

Proposal 4: RAN1 has no conclusion on whether it is possible for lower layer signalling providing the SL-PRS priority when SL-PRS is triggered by peer UE’s lower layer’s signalling.

- This issue can be further discussed in RAN1#114bis meeting

- In this case, the SL-PRS priority shall be same as the peer UE.

- Conclusion

In this contribution, RAN2 LS on SL positioning MAC agreements is discussed. According to the discussions, following proposals are provided.

Proposal 1:  RAN1 confirms that CBR measurement on both shared and dedicated resource pool for SL-PRS transmission has already been discussed and supported during the previous RAN1 meeting.

Proposal 2: RAN1 respectfully ask RAN2 to take the following agreements in congestion control in the future work.

Agreement In Scheme 2, - For a dedicated resource pool for positioning, - congestion control can restrict at least the following range of parameters for SL PRS configuration per resource pool by CBR and priority: - Maximum SL PRS transmission power - Maximum Number of SL PRS (re-)transmissions - Discuss further the following four SL PRS transmission parameters: - Minimum Periodicity of SL PRS - Maximum Number of SL PRS resources in a slot - Maximum comb-size of a SL PRS resource in a slot - Maximum Number of OFDM symbols of a SL PRS resource in a slot - For congestion control similar to legacy, the CR limits are (pre)-configured per priority in a resource pool - Note: Similar to SL communication how to achieve the CR limit is left to UE implementation. - For a shared resource pool for positioning, the SL PRS can share the same restriction of PSSCH without specific enhancement in addition to what is already specified. Agreement In Scheme 2, with regards to the congestion control for SL PRS: - SL-PRS congestion processing time: based on both SCS and UE capability, similar to legacy - The maximum number of CBR ranges for SL positioning is 8 - Number of CBR levels is 16 - CBR measurement for SL PRS can be reported to gNB FFS: Whether it is needed to be reported to LMF or another UE

Proposal 3: RAN1 confirms that it is possible for higher layer provides the SL-PRS priority when SL-PRS is triggered by peer UE’s lower layer’s signalling.

- In this case, the SL-PRS priority can be different from the peer UE.

Proposal 4: RAN1 has no conclusion on whether it is possible for lower layer signalling providing the SL-PRS priority when SL-PRS is triggered by peer UE’s lower layer’s signalling.

- This issue can be further discussed in RAN1#114bis meeting

- In this case, the SL-PRS priority shall be same as the peer UE.

References

- R1-2308834(R2-239343), LS on SL positioning MAC agreements, 3GPP TSG-RAN WG1 Meeting #114bis, October 9th - 13th, 2023.