3GPP TSG RAN WG1 #114bis			R1-2309419

Xiamen, China, October 9th – October 13th, 2023

Title:	[Draft] Reply LS on SL RB set index and LBT failure indication for PSFCH

Response to:	R1-2308832（R2-2309157）

Release:	Rel-18

Work Item:	NR_SL_enh2

Source:	xiaomi (RAN1)

To:	RAN2

Cc:

Attachment:	none

Contact Person:

Name:

E-mail Address:

RAN1 would like to thank RAN2 for the incoming LS in R1-2308832（R2-2309157）. RAN1 has discussed the questions raised in the LS and would like to provide the answers as the following:

Question 1: How is the SL RB set index derived?

Question 2: Is the derived SL RB set index (based on the answer to Question 1) unique within the SL-BWP?

RAN1[reply]:

For question 1, in NR-U, the following method as specified in the TS 38.214 clause 7 to configure the RB set on DL/UL carrier can be reused as baseline.  In SL-U, IntraCellGuardBandsSL-List can also be preconfigured, with the similar method specified in NR-U, the SL RB set index can be derived.

For question 2, the answer is Yes.

TS 38.214 7 UE procedures for transmitting and receiving on a carrier with intra-cell guard bands For operation with shared spectrum channel access for FR1, when the UE is configured with any of IntraCellGuardBandsPerSCS for UL carrier and for DL carrier with SCS configuration μ, the UE is provided with NRB-set,x-1 intra-cell guard bands on a carrier with μ, each defined by start CRB and size in number of CRBs, GB s,xstart,μ and GB s,xsize,μ , provided by higher layer parameters startCRB and nrofCRBs, respectively, where s∈0,1,…,NRB-set,x-2. The subscript x is set to DL and UL for the downlink and uplink, respectively. Where there is no risk of confusion, the subscript x can be dropped. The intra-cell guard bands separate NRB-set,x RB sets, each defined by start and end CRB, RB s,xstart,μ and RB s,xend,μ, respectively. The UE does not expect that nrofCRBs is configured with non-zero value smaller than the applicable intra-cell guard bands as specified in [8, TS 38.101-1] corresponding to μ and carrier size Ngrid,xsize,μ. The UE determines the start and end CRB indices for s∈0,1,…,NRB-set,x-1 as RB s,xstart,μ=Ngrid,xstart,μ+0s=0GB s-1,xstart,μ+GB s-1,xsize,μotherwise and RB s,xend,μ=Ngrid,xstart,μ+Ngrid,xsize,μ-1s=NRB-set,x-1GB s,xstart,μ-1otherwise

2. Actions:

To RAN2

ACTIONS: RAN1 respectfully asks RAN2 to take above responses into account.

3. Dates of Next TSG-RAN WG1 Meetings:

TSG RAN WG1 Meeting #115	November 13th – November 17th, 2023	Chicago, US

TSG RAN WG1 Meeting #116                February 26th – March 1st, 2024                                      Athens，GR