3GPP TSG RAN WG1 #114 bis		      R1-2309494

Xiamen, China, October 9th – October 13th, 2023

Source:	CATT

Title:	Remaining issues on two TAs for UL multi-DCI multi-TRP operation

Agenda Item:	8.1.1.2

Document for:	Discussion and Decision

- Introduction

In RAN1 #114 meeting, following agreements were achieved on two TAs for UL multi-DCI for multi-TRP operation [1].

Agreement For inter-cell multi-DCI based Multi-TRP operation with two TA enhancement, support indication of additionalPCI in the PDCCH order - as baseline capability: support PRACH triggering towards servingCell PCI or active additionalPCI. Conclusion For inter-cell/intra-cell multi-DCI based Multi-TRP operation with two TA enhancement, there is no consensus to introduce RAR-less solution in Rel-18 from RAN1 perspective. Conclusion For inter-cell multi-DCI based Multi-TRP operation with two TA enhancement, for the optional feature where the overlapping duration of the later of the two UL transmissions is reduced the following is concluded: - There is no consensus to specify the overlapping duration in RAN1 specifications. Agreement For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement and PDCCH order CFRA, indicate a representation of the TAG ID with 1 bit (either the first TAG ID or the second TAG ID for the serving cell) as part of TA command in RAR Note: For intra-cell multi-DCI based Multi-TRP operation, only a single NTA,offset is configured. Conclusion For inter-cell multi-DCI based Multi-TRP operation with two TA enhancement, no consensus on introducing the following optional UE capability: optional UE capability: support PRACH triggering towards servingCell PCI, active additionalPCI, or up to 1 inactive additionalPCI Agreement For inter-cell multi-DCI based multi-TRP operation with two TAGs configured in Spcell, when the PDCCH order is transmitted from a TRP associated with additionalPCI, PDCCH RAR and PDSCH RAR of a CFRA are both QCLed with the CORESET associated with the Type I CSS set

In this contribution, remaining issues on two TAs are discussed with the following outline: initial TA acquisition is discussed in section 2. In section 3, TAT issue is handled. Two TAs for UL transmission after BFD/BFR is discussed in section 4.  TPs in TS38.213 are provided in Section 5. A brief conclusion is given in section 6.

- Initial TA acquisition

In RAN1 #112 bis e-meeting, there was the following agreement:

Agreement

For multi-DCI based inter-cell multi-TRP and intra-cell multi-TRP operation with two TAGs configured in a CC, for a CFRA based PDCCH order from one TRP triggering PRACH towards another TRP, study whether and, if needed, how to determine the transmit power of the triggered PRACH preamble

From the agreement, an issue to be solved is the determination of the transmit power for the PDCCH order triggered PRACH. In RAN1 #114 meeting, following proposal is proposed in FL summary according to this issue:

Proposal 5.0

For multi-DCI based inter-cell multi-TRP and intra-cell multi-TRP operation with two TAGs configured in a CC, SSB indicated in the CFRA based PDCCH order is used as the PL-RS for determining the transmit power of the triggered PRACH transmission.

- Note:  the UE expects the SSB index to satisfy the "Known conditions for pathloss reference signal" of Section 8.14.2 of 38.133.

In the legacy mechanism, the SSB/CSI-RS that is QCLed with the DM-RS of the PDCCH order is used as the pathloss RS to estimate the pathloss of PRACH transmission, as agreed in RAN1 #92 meeting.

RAN1 #92: Agreements: - UE assumes that the DMRS of both the received PDCCH order and the PDCCH of the corresponding Msg2 are QCLed with the same SSB/CSI-RS. - gNB configures 9 bits to indicate RACH occasion index. - 6 bits are used to indicate an SSB index - Note: This SSB index is just intended to find the RACH occasion to transmit Msg1 - 3 bits are used to indicate the relative RACH occasion index that corresponds to the indicated SSB index - Note: UE follows the SSB CBRA mapping rule to find the specific RACH occasion. - The SSB/CSI-RS that is QCLed with both the DM-RS of PDCCH order and the DM-RS of the PDCCH of the corresponding Msg2 is used for pathloss estimation associated with Msg1.

The legacy mechanism to determine PL RS and beam of PRACH transmission doesn’t adapt to the multi-DCI based  multi-TRP transmission considering the cross triggered PRACH, e.g., PDCCH order is transmitted from TRP1 and the triggered PRACH transmission is toward TRP2, in that case UE-to-TRP1  has a different pathloss than UE-to-TRP2. New mechanism should be designed to indicate UE the PL RS to determine the transmission power of the PRACH. A straightforward way is to use the SSB indicated in the PDCCH order as the PL RS and estimate the transmission power of the PRACH  on active UL BWP  of carrier  of serving cell  in transmission occasion  by [4]:

[dBm],

Where PCMAX,f,c(i) is provided by the higher layer configuration as follows:

RACH-ConfigGeneric ::=              SEQUENCE {

prach-ConfigurationIndex            INTEGER (0..255),

msg1-FDM                            ENUMERATED {one, two, four, eight},

msg1-FrequencyStart                 INTEGER (0..maxNrofPhysicalResourceBlocks-1),

zeroCorrelationZoneConfig           INTEGER(0..15),

preambleReceivedTargetPower         INTEGER (-202..-60),

preambleTransMax                    ENUMERATED {n3, n4, n5, n6, n7, n8, n10, n20, n50, n100, n200},

powerRampingStep                    ENUMERATED {dB0, dB2, dB4, dB6},

ra-ResponseWindow                   ENUMERATED {sl1, sl2, sl4, sl8, sl10, sl20, sl40, sl80},

}

In addition, this method can also apply to the intra-cell as well as inter-cell multi-TRP.

Proposal 1: To determine the transmit power of the PDCCH ordered PRACH for multi-DCI based multi-TRP operation, support the following proposal:

Proposal 5.0

For multi-DCI based inter-cell multi-TRP and intra-cell multi-TRP operation with two TAGs configured in a CC, SSB indicated in the CFRA based PDCCH order is used as the PL-RS for determining the transmit power of the triggered PRACH transmission.

- Note:  the UE expects the SSB index to satisfy the "Known conditions for pathloss reference signal" of Section 8.14.2 of 38.133.

- On TAT issue

In RAN1 #113 meeting, the following agreements on TAT issue has been achieved based on the RAN2 LS:

Agreement - Proposed answer to Question Q2 in RAN2 LS (R1-2304326): - RAN1 confirms that when the TA timer associated to one TRP expires for a TAG associated with a TCI state, UL or DL operation associated to the another TRP is not impacted. This further depends on PTAG/STAG definition, which is up to RAN2 to decide. - Which UL or DL operation is impacted have not been discussed in RAN1.

From the agreement, only UL or DL operation associated with the TRP with the expired TAT timer will be impacted. Which UL or DL operation is impacted needs to be further discussed. In NR Rel-16/17, one TimeAlignmentTimer(TAT) is used in TA adjustment and is configured to UE in TAG.

TAG ::=                             SEQUENCE {

tag-Id                              TAG-Id,

timeAlignmentTimer                 TimeAlignmentTimer

}

TimeAlignmentTimer ::= ENUMERATED {ms500, ms750, ms1280, ms1920, ms2560, ms5120, ms10240, infinity}

UE will perform TA adjustment in the period indicated by TimeAlignmentTimer. Both gNB and UE consider that uplink synchronization is maintained during this period. If TAT expires, UE will flush all HARQ buffers for all serving cells and release PUSCH/PUCCH/SRS transmission for all serving cells. Meanwhile, UE will maintain NTA of all TAGs [2]. In case that UL non-synchronization happens due to TAT expiration and there is UL data arrival, UE will initiate CBRA.  In case that UL non-synchronization happens due to TAT expiration and there is DL data arrival, PDCCH ordered RACH is initiated [3].

In Rel-18 TA enhancements for multi-TRP operation, 2 TAs are used with each corresponds to one UE-to-TRP uplink transmission. Correspondingly, two TATs will be considered for multi-DCI based multi-TRP operation with each TAT configured independently in each TAG, each TAT corresponds to one UE-to-TRP uplink transmission. In the procedure of TA maintenance, suppose one of the two TATs expires, e.g. TAT1 corresponding to UE-to-TRP1 uplink transmission expires, the behavior of UE should be re-defined considering that the legacy UE behavior is defined per serving cell, if HARQ buffer or PUSCH/PUCCH/SRS is cleared per cell according to the legacy, the UE-to-TRP2 uplink transmission will be terminated, leading to a serious performance degradation. A possible solution is to clear the corresponding HARQ buffer/HARQ process of the expired TAT/TAG and clear the PUSCH/PUCCH/SRS transmission of the expired TAT/TAG. In the implementation, if two HARQ entities are considered for 2 TRPs, when one TAT expires, the HARQ buffer of the expired TRP/TAG shall be cleared. In case one HARQ entity is considered for 2 TRPs in multi-TRP operation, the relationship between HARQ process and TRP/TAG can be recorded by physical layer for each scheduling, when one TAT expires, the statistics was requested by MAC layer and upon the response from physical layer the HARQ process of the expired TRP/TAG will be cleared.

Proposal 2: When one or two TATs expire if two TATs are considered, clear the HARQ buffer/process of the expired TAT/TAG and the corresponding PUSCH/PUCCH/SRS uplink transmission.

- Two TAs for UL transmission after BFD/BFR

In the discussion of RAN1#114 meeting, one issue proposed is the TA/TAG determination for uplink transmission after BFD/successful BFR. Since TAG is only associated with joint or UL TCI state. Considering UE does not apply joint/UL TCI state after BFR is completed, how to determine the TA value of the UL transmission is an issue to be determined.

For multi-DCI based multi-TRP operation in NR Rel-17, after BFR/BFD , the UE assumes antenna port quasi-collocation parameters corresponding to qnew from  q1,0 for the first CORESETs or from  q1,1 for the second CORESETs.  q1,0 and q1,1 are two candidateBeamRS lists, each candidate beam RS list associates with one BFD RS set q0,0 and q0,1.

On the basis of BFD/BFR mechanism defined in Rel-17, an intuitive way is to use the TA/TAG associated to the CORESETPool Index of the qnew for UL transmission after BFD/BFR considering that the baseline assumption in Rel-18 multi-DCI based 2TA enhancement is that the UL/joint TCI states of UL signals/channels associated to one CORESETPool Index correspond to one TAG. However, this mechanism doesn’t adapt to the case of TCI states of one CORESETPool Index associated to both TAGs, which might be adopted as an optional feature in the 2TA enhancement. For the latter case, association between candidate beams with TAG ID might be needed. E.g. UE might expect that candidate beams in a candidateBeamRS-list associated to one BFD-RS set correspond to one TAG. Association of TAG ID with candidate beam can be provided by RRC configuration.

Proposal 3: To determine UL transmission TA/TAG after BFD/BFR, consider the association of TAG ID with candidate beams in a candidateBeamRs-list corresponding to one BFD-RS set, the association of TAG ID with candidate beam can be provided by RRC configuration.

- TPs in TS38.213

In RAN1 #114 meeting, there was the following achievement:

Agreement For intra-cell multi-DCI based Multi-TRP operation with two TA enhancement and PDCCH order CFRA, indicate a representation of the TAG ID with 1 bit (either the first TAG ID or the second TAG ID for the serving cell) as part of TA command in RAR.

From the agreement,  1 bit TAG ID indicating the first TAG ID or the second TAG ID should be included as part of TA command in RAR. This agreement should be captured in section 8.2 of TS38.213. If it is not captured, it will be ambiguous for UE to apply the indicated TA as the first or second initial value for the close loop TA adjustment.

Proposal 4  Capture RAN1 #114 agreement that for intra-cell multi-DCI based Multi-TRP operation with two TA enhancement and PDCCH order CFRA, indicate a representation of the TAG ID with 1 bit(either the first TAG ID or the second TAG ID for the serving cell) as part of TA command in RAR.

## *** Unchanged parts are omitted *** A RAR UL grant schedules a PUSCH transmission from the UE. The contents of the RAR UL grant, starting with the MSB and ending with the LSB, are given in Table 8.2-1. If the value of the frequency hopping flag is 0, the UE transmits the PUSCH without frequency hopping; otherwise, the UE transmits the PUSCH with frequency hopping. The UE determines the MCS of the PUSCH transmission from the first sixteen indexes of the applicable MCS index table for PUSCH as described in [6, TS 38.214]. The TPC command value δmsg2,b,f,c is used for setting the power of the PUSCH transmission, as described in clause 7.1.1, and is interpreted according to Table 8.2-2. The CSI request field is reserved. The ChannelAccess-CPext field indicates a channel access type and CP extension for operation with shared spectrum channel access [15, TS 37.213] in FR1 as defined in Table 7.3.1.1.1-4 in [5, TS 38.212] or Table 7.3.1.1.1-4A in [5, TS 38.212] if channelAccessMode = "semiStatic" is provided. The ChannelAccess-CPext field indicates a channel access type for operation with shared spectrum channel access [15, TS 37.213] in FR2-2 as defined in Table 7.3.1.1.1-4B in [5, TS 38.212] if ChannelAccessMode2-r17 is provided. The TAG ID field indicates the first TAG ID or the second TAG ID for PDCCH order CFRA Table 8.2-1: Random Access Response Grant Content field size RAR grant field; Number of bits. Frequency hopping flag; 1. PUSCH frequency resource allocation; 12, for operation with shared spectrum channel access in FR1 or for FR2-2 when ChannelAccessMode2-r17 is provided 14, otherwise. PUSCH time resource allocation; 4. MCS; 4. TPC command for PUSCH; 3. CSI request; 1. ChannelAccess-CPext; 2, for operation with shared spectrum channel access in FR1 or for FR2-2 when ChannelAccessMode2-r17 is provided 0, otherwise. TAG ID; 1, for indication of the first TAG ID or the second TAG ID for PDCCH order CFRA. Table 8.2-2: TPC Command δmsg2,b,f,c for PUSCH TPC Command; Value (in dB). 0; -6. 1; -4. 2; -2. 3; 0. 4; 2. 5; 4. 6; 6. 7; 8. Unless the UE is configured a SCS, the UE receives subsequent PDSCH using same SCS as for the PDSCH reception providing the RAR message. If the UE does not detect the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI within the window, or if the UE detects the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI within the window and the LSBs of a SFN field in the DCI format 1_0, if included and applicable, are not same as corresponding LSBs of the SFN where the UE transmitted the PRACH, or the UE does not correctly receive a corresponding transport block within the window, the UE procedure is as described in [11, TS 38.321]. ## *** Unchanged parts are omitted ***

- Conclusions

In this contribution, we focus on the remaining issues on two TAs for uplink multi-DCI for multi-TRP operation, initial TA acquisition and TAT issues are mainly discussed. We have the following proposals:

Proposal 1: To determine the transmit power of the PDCCH ordered PRACH for multi-DCI based multi-TRP operation, support the following proposal:

Proposal 5.0

For multi-DCI based inter-cell multi-TRP and intra-cell multi-TRP operation with two TAGs configured in a CC, SSB indicated in the CFRA based PDCCH order is used as the PL-RS for determining the transmit power of the triggered PRACH transmission.

- Note:  the UE expects the SSB index to satisfy the "Known conditions for pathloss reference signal" of Section 8.14.2 of 38.133.

Proposal 2: When one or two TATs expire if two TATs are considered, clear the HARQ buffer/process of the expired TAT/TAG and the corresponding PUSCH/PUCCH/SRS uplink transmission.

Proposal 3: To determine UL transmission TA/TAG after BFD/BFR, consider the association of TAG ID with candidate beams in a candidateBeamRs-list corresponding to one BFD-RS set, the association of TAG ID with candidate beam can be provided by RRC configuration.

Proposal 4:  Capture RAN1 #114 agreement that for intra-cell multi-DCI based Multi-TRP operation with two TA enhancement and PDCCH order CFRA, indicate a representation of the TAG ID with 1 bit(either the first TAG ID or the second TAG ID for the serving cell) as part of TA command in RAR.

## *** Unchanged parts are omitted *** A RAR UL grant schedules a PUSCH transmission from the UE. The contents of the RAR UL grant, starting with the MSB and ending with the LSB, are given in Table 8.2-1. If the value of the frequency hopping flag is 0, the UE transmits the PUSCH without frequency hopping; otherwise, the UE transmits the PUSCH with frequency hopping. The UE determines the MCS of the PUSCH transmission from the first sixteen indexes of the applicable MCS index table for PUSCH as described in [6, TS 38.214]. The TPC command value δmsg2,b,f,c is used for setting the power of the PUSCH transmission, as described in clause 7.1.1, and is interpreted according to Table 8.2-2. The CSI request field is reserved. The ChannelAccess-CPext field indicates a channel access type and CP extension for operation with shared spectrum channel access [15, TS 37.213] in FR1 as defined in Table 7.3.1.1.1-4 in [5, TS 38.212] or Table 7.3.1.1.1-4A in [5, TS 38.212] if channelAccessMode = "semiStatic" is provided. The ChannelAccess-CPext field indicates a channel access type for operation with shared spectrum channel access [15, TS 37.213] in FR2-2 as defined in Table 7.3.1.1.1-4B in [5, TS 38.212] if ChannelAccessMode2-r17 is provided. The TAG ID field indicates the first TAG ID or the second TAG ID for PDCCH order CFRA Table 8.2-1: Random Access Response Grant Content field size RAR grant field; Number of bits. Frequency hopping flag; 1. PUSCH frequency resource allocation; 12, for operation with shared spectrum channel access in FR1 or for FR2-2 when ChannelAccessMode2-r17 is provided 14, otherwise. PUSCH time resource allocation; 4. MCS; 4. TPC command for PUSCH; 3. CSI request; 1. ChannelAccess-CPext; 2, for operation with shared spectrum channel access in FR1 or for FR2-2 when ChannelAccessMode2-r17 is provided 0, otherwise. TAG ID; 1, for indication of the first TAG ID or the second TAG ID for PDCCH order CFRA. Table 8.2-2: TPC Command δmsg2,b,f,c for PUSCH TPC Command; Value (in dB). 0; -6. 1; -4. 2; -2. 3; 0. 4; 2. 5; 4. 6; 6. 7; 8. Unless the UE is configured a SCS, the UE receives subsequent PDSCH using same SCS as for the PDSCH reception providing the RAR message. If the UE does not detect the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI within the window, or if the UE detects the DCI format 1_0 with CRC scrambled by the corresponding RA-RNTI within the window and the LSBs of a SFN field in the DCI format 1_0, if included and applicable, are not same as corresponding LSBs of the SFN where the UE transmitted the PRACH, or the UE does not correctly receive a corresponding transport block within the window, the UE procedure is as described in [11, TS 38.321]. ## *** Unchanged parts are omitted ***

- References

- RAN1#114 meeting, Chairman notes.

- 3GPP TS38.321 v17.5.0

- 3GPP TS38.300 v17.5.0

- 3GPP TS38.213 v18.0.0 section 7.4