3GPP TSG RAN WG1 #114bis		R1-2309576

Xiamen, China, October 9th – 13th, 2023

Source:	OPPO

Title:	Remaining Issues of bandwidth aggregation for positioning measurement

Agenda Item:	8.3.4

Document for:	Discussion and Decision

- Introduction

The Rel-18 WID on expanded and improved NR positioning includes the following RAN1-centric objective of specifying bandwidth aggregation for positioning measurement. In this contribution, we will discuss a few remaining issues related with bandwidth aggregation for DL and UL positioning measurements.

- Discussions

In the measurement of aggregated DL PRS resources, it is still an pending issue how to measure the RSRP or RSRPP. In our view, for the RSRP or RSRPP report, if the corresponding RSTD or UE Rx-Tx time difference is measured from aggregated PRS resources, the UE can also report the RSRP or RSRPP obtained from the same aggregated PRS resources.

Proposal 1: For the measurement of aggregated DL PRS resources, the RSRP or RSRPP measurement shall be based on the aggregated PRS resources.

The description in 38.214 does not clearly describe the behavior of linked DL PRS resources. For the linked DL PRS resource, the UE shall only assume phase continuity between the DL PRS resources on the same symbols in the linked DL PRS resource sets. It is proposed to change the specification to clarify that.

Proposal 2: Endorse the following TP for 38.214:

- Reason for Change: The description on DL PRS linked for bandwidth aggregation is not clear.

- Summary of Changes: Add text to clarify that only the DL PRS resources on the same symbol which are linked for bandwidth aggregation have phase continuity.

- Consequence if not approved:  the behavior on linked DL PRS resources has ambiguity.

#### 5.1.6.5 PRS reception procedure *** Unchanged parts are omitted *** When the UE is expected to perform joint measurements for bandwidth aggregation across DL PRS positioning frequency layers, the UE expects to be configured with linkage information, via higher layer parameter [linkage], between DL PRS resource sets across DL PRS positioning frequency layers. For the linked DL PRS resource sets, the UE is expected to be configured with the same values of QCL, dl-PRS-Periodicity-and-ResourceSetSlotOffset, dl-PRS-NumSymbols, dl-PRS-ResourceTimeGap, dl-PRS-ResourceSymbolOffset, dl-prs-MutingBitRepetitionFactor, dl-PRS-CyclicPrefix, comb size, power per subcarrier, NR-MutingPattern, and NR-DL-PRS-SFN0-Offset, and the UE is expected to be configured with DL PRS resources that maintain uniformly spaced DL PRS RE pattern within a symbol across aggregated DL PRS positioning frequency layers. The UE may assume that DL PRS resources across the linked DL PRS resource sets which satisfy the above conditions are linked for bandwidth aggregation, and the UE may assume phase continuity on the DL PRS resources on same symbol(s); otherwise, the UE does not assume that PRS resources from the linked DL PRS resource sets are linked for bandwidth aggregation. *** Unchanged parts are omitted ***

The following agreement on aperiodic SRS resources linked for bandwidth aggregation was made but not captured in the specification yet:

Agreement With regard to aperiodic positioning SRS for bandwidth aggregation for UEs in RRC_CONNECTED state, support both Option 2 and Option1. - Option 2: Support to use a DCI format 0_3 or 1_3 for multi-cell PDSCH/PUSCH scheduling to trigger SRS resources for bandwidth aggregation in multiple CCs. - Option 1: Support a Rel-17 single DCI scheduling positioning SRS resource sets across the linked carriers, as a separate UE capability. - Reuse Rel-17 DCI framework without modification. - If a single DCI indicates transmission of an aperiodic positioning SRS resource set, UE transmits aperiodic positioning SRS resource sets across all linked carriers for bandwidth aggregation.

Proposal 3: endorse the following TP for TS 38.214:

- Reason for Change: The agreement on how to trigger the aperiodic SRS linked for bandwidth aggregation is not captured in the specification.

- Summary of Changes: Add text to describe the triggering of aperiodic SRS linked for bandwidth aggregation in 38.214 according to the agreement.

- Consequence if not approved:  Specification does not support the transmission of aperiodic SRS linked for bandwidth aggregation.

#### 6.2.1.4 UE sounding procedure for positioning purposes ## *** Unchanged parts are omitted *** The reduced capability UE may be configured, via [higher layer parameter], subject to UE capability, with an UL time window where the UE is not expected to transmit other signals/channels and is only expected to transmit the SRS for positioning using frequency hopping. The UE is expected to be configured with linkage information [linkage] on SRS resource sets for positioning across two or three CCs which are linked for bandwidth aggregation. For the linked SRS resource sets, the UE is expected to be configured with the same values of startPosition, nrofSymbols, periodicityAndOffset, slotOffset, alpha, p0, subcarrier spacing, CP, and comb size, and the UE is expected to maintain phase continuity for the SRS transmission. The UE may assume that SRS resources across the linked SRS resource sets which satisfy the above conditions are linked for bandwidth aggregation, otherwise, the UE does not assume that SRS resources of the linked SRS resource sets are linked for bandwidth aggregation. For the linked SRS resource sets for bandwidth aggregation across CCs, if an SRS configured by the higher layer parameter SRS-PosResource, along with the [switching period] when applicable, collides with other signals or channels on a symbol and is the SRS in that symbol that is dropped, SRS transmission of the linked SRS resource sets across all CCs is dropped on that symbol. For the linked SRS resource sets for bandwidth aggregation across CCs, if an SRS configured by the higher layer parameter SRS-PosResource and the higher layer parameter resoureType in SRS-PosResource is set to ‘aperiodic’ and when the UE receives a downlink DCI, a group commcon DCI, or an uplink DCI based command where a codepoint of the DCI trigger one of the linked SRS resoruce set, the UE transmit aperodic SRS in all the linked SRS resouce sets for bandwidth aggregation across CCs as specified in clause 6.2.1. ## *** Unchanged parts are omitted ***

Regarding the transmission of SP SRS linked for bandwidth aggregation, we reached the following WA:

Working assumption For semi-persistent positioning SRS for bandwidth aggregation, a single MAC CE can activate or deactivate: - SRS resource set(s) in one or two or three of three aggregated carriers - SRS resource set(s) in one or two of two aggregated carriers. Note: the single spatial relation is indicated by the MAC CE for each of two or three aggregated SRS resources.

In our view, this WA shall be confirmed to complete the design of SP SRS for bandwidth aggregation.

Proposal 4: Confirm the following the WA:

Working assumption For semi-persistent positioning SRS for bandwidth aggregation, a single MAC CE can activate or deactivate: - SRS resource set(s) in one or two or three of three aggregated carriers - SRS resource set(s) in one or two of two aggregated carriers. Note: the single spatial relation is indicated by the MAC CE for each of two or three aggregated SRS resources.

- Conclusions

In this contribution, we present our views on aggregation of PRS/SRS for NR positioning and the following proposals are made:

Proposal 1: For the measurement of aggregated DL PRS resources, the RSRP or RSRPP measurement shall be based on the aggregated PRS resources.

Proposal 2: Endorse the following TP for 38.214:

- Reason for Change: The description on DL PRS linked for bandwidth aggregation is not clear.

- Summary of Changes: Add text to clarify that only the DL PRS resources on the same symbol which are linked for bandwidth aggregation have phase continuity.

- Consequence if not approved:  the behavior on linked DL PRS resources has ambiguity.

#### 5.1.6.5 PRS reception procedure *** Unchanged parts are omitted *** When the UE is expected to perform joint measurements for bandwidth aggregation across DL PRS positioning frequency layers, the UE expects to be configured with linkage information, via higher layer parameter [linkage], between DL PRS resource sets across DL PRS positioning frequency layers. For the linked DL PRS resource sets, the UE is expected to be configured with the same values of QCL, dl-PRS-Periodicity-and-ResourceSetSlotOffset, dl-PRS-NumSymbols, dl-PRS-ResourceTimeGap, dl-PRS-ResourceSymbolOffset, dl-prs-MutingBitRepetitionFactor, dl-PRS-CyclicPrefix, comb size, power per subcarrier, NR-MutingPattern, and NR-DL-PRS-SFN0-Offset, and the UE is expected to be configured with DL PRS resources that maintain uniformly spaced DL PRS RE pattern within a symbol across aggregated DL PRS positioning frequency layers. The UE may assume that DL PRS resources across the linked DL PRS resource sets which satisfy the above conditions are linked for bandwidth aggregation, and the UE may assume phase continuity on the DL PRS resources on same symbol(s); otherwise, the UE does not assume that PRS resources from the linked DL PRS resource sets are linked for bandwidth aggregation. *** Unchanged parts are omitted ***

Proposal 3: endorse the following TP for TS 38.214:

- Reason for Change: The agreement on how to trigger the aperiodic SRS linked for bandwidth aggregation is not captured in the specification.

- Summary of Changes: Add text to describe the triggering of aperiodic SRS linked for bandwidth aggregation in 38.214 according to the agreement.

- Consequence if not approved:  Specification does not support the transmission of aperiodic SRS linked for bandwidth aggregation.

#### 6.2.1.4 UE sounding procedure for positioning purposes ## *** Unchanged parts are omitted *** The reduced capability UE may be configured, via [higher layer parameter], subject to UE capability, with an UL time window where the UE is not expected to transmit other signals/channels and is only expected to transmit the SRS for positioning using frequency hopping. The UE is expected to be configured with linkage information [linkage] on SRS resource sets for positioning across two or three CCs which are linked for bandwidth aggregation. For the linked SRS resource sets, the UE is expected to be configured with the same values of startPosition, nrofSymbols, periodicityAndOffset, slotOffset, alpha, p0, subcarrier spacing, CP, and comb size, and the UE is expected to maintain phase continuity for the SRS transmission. The UE may assume that SRS resources across the linked SRS resource sets which satisfy the above conditions are linked for bandwidth aggregation, otherwise, the UE does not assume that SRS resources of the linked SRS resource sets are linked for bandwidth aggregation. For the linked SRS resource sets for bandwidth aggregation across CCs, if an SRS configured by the higher layer parameter SRS-PosResource, along with the [switching period] when applicable, collides with other signals or channels on a symbol and is the SRS in that symbol that is dropped, SRS transmission of the linked SRS resource sets across all CCs is dropped on that symbol. For the linked SRS resource sets for bandwidth aggregation across CCs, if an SRS configured by the higher layer parameter SRS-PosResource and the higher layer parameter resoureType in SRS-PosResource is set to ‘aperiodic’ and when the UE receives a downlink DCI, a group commcon DCI, or an uplink DCI based command where a codepoint of the DCI trigger one of the linked SRS resoruce set, the UE transmit aperodic SRS in all the linked SRS resouce sets for bandwidth aggregation across CCs as specified in clause 6.2.1. ## *** Unchanged parts are omitted ***

Proposal 4: Confirm the following the WA:

Working assumption For semi-persistent positioning SRS for bandwidth aggregation, a single MAC CE can activate or deactivate: - SRS resource set(s) in one or two or three of three aggregated carriers - SRS resource set(s) in one or two of two aggregated carriers. Note: the single spatial relation is indicated by the MAC CE for each of two or three aggregated SRS resources.

- Reference

TS 38.214 v18.0.0