3GPP TSG RAN WG1 #114bis           	                                   R1-2309971

Xiamen, China, October 9th – October 13th, 2023

Agenda Item: 7.1

Source: MediaTek Inc.

Title: Clarify ambiguity on RRC reconfiguration with type 2 CG-PUSCH

Document for: Discussion and decision

- Introduction

During RAN1 #114 in agenda 7.1, there was a discussion related to ambiguity on RRC reconfiguration with type 2 CG-PUSCH, with moderator summary in [1].

As the above-mentioned issue was not concluded in RAN1 #114, this contribution is submitted for further clarification/discussion.

- Ambiguity on RRC reconfiguration with type 2 CG-PUSCH

An illustration figure for this issue from [1] is shown in Figure 1 below. The issue is that NW activates the type 2 CG-PUSCH with an activation DCI but later changs the RRC parameters related to an ongoing Type 2 CG-PUSCH transmission with a RRC reconfiguration. This causes confusion for UE to follow the RRC reconfiguration or ignore the RRC reconfiguration. As the design principle of type 2 CG PUSCH is using activation DCI to actually trigger the CG-PUSCH, changing RRC parameters related to an ongoing Type 2 CG-PUSCH transmission does not fit the principle. An example observed in the field is:

- NW activates the type 2 CG-PUSCH but later changed the RRC configuration of maxRank which will lead to change of interpretation of “Precoding information and number of layers”

  - The IE maxRank is under PUSCH-Config but not under configuredGrantConfig

Figure 1: RRC reconfiguration after activation DCI for type 2 CG-PUSCH [1]

Observation 1: As shown in Figure 1, it is problematic when NW activates the type 2 CG-PUSCH with an activation DCI but later changes the RRC parameters related to an ongoing Type 2 CG-PUSCH transmission with a RRC reconfiguration. This causes confusion for UE to follow the RRC reconfiguration or ignore the RRC reconfiguration.

Observation 2: As the design principle of type 2 CG PUSCH is using activation DCI to actually trigger the CG-PUSCH, changing RRC parameters related to an ongoing Type 2 CG-PUSCH transmission does not fit the principle.

Observation 3: An example observed in the field is:

- NW activates the type 2 CG-PUSCH but later changed the RRC configuration of maxRank which will lead to change of interpretation of “Precoding information and number of layers”

  - The IE maxRank is under PUSCH-Config but not under configuredGrantConfig

With the observations above, we hence have the following proposal:

Proposal 1: RAN1 to endorse the following as RAN1 conclusion:

- UE is not expected to receive RRC reconfiguration changing RRC parameters related to an ongoing Type 2 CG-PUSCH transmission.

- Conclusion

In this contribution, we provide our views on clarifying ambiguity on RRC reconfiguration with type 2 CG-PUSCH with the following observations and proposals:

Observation 1: As shown in Figure 1, it is problematic when NW activates the type 2 CG-PUSCH with an activation DCI but later changes the RRC parameters related to an ongoing Type 2 CG-PUSCH transmission with a RRC reconfiguration. This causes confusion for UE to follow the RRC reconfiguration or ignore the RRC reconfiguration.

Observation 2: As the design principle of type 2 CG PUSCH is using activation DCI to actually trigger the CG-PUSCH, changing RRC parameters related to an ongoing Type 2 CG-PUSCH transmission does not fit the principle.

Observation 3: An example observed in the field is:

- NW activates the type 2 CG-PUSCH but later changed the RRC configuration of maxRank which will lead to change of interpretation of “Precoding information and number of layers”

  - The IE maxRank is under PUSCH-Config but not under configuredGrantConfig

Proposal 1: RAN1 to endorse the following as RAN1 conclusion:

- UE is not expected to receive RRC reconfiguration changing RRC parameters related to an ongoing Type 2 CG-PUSCH transmission.

- Reference

R1-2308569, “FL summary #2 of clarifying several ambiguities with type 2 CG-PUSCH”, Moderator (Qualcomm), RAN1 #114