3GPP TSG RAN WG1 #114bis			R1-2310002

Xiamen, China, October 9th – October 13th, 2023

Agenda item:	8.6.1

Title:	Remaining issues on XR enhancements

Source:	MediaTek Inc.

Document for:	Discussion and Decision

# Introduction

In RAN#99, the following revised WI objectives were approved for XR enhancements related to capacity [1]:

Specify the enhancements related to capacity: - Multiple Configured Grant (CG) PUSCH transmission occasions in a period of a single CG PUSCH configuration (RAN1, RAN2); - Dynamic indication of unused CG PUSCH occasion(s) based on Uplink Control Information (UCI) by the UE (RAN1, RAN2); - Buffer Status Report (BSR) enhancements including at least new Buffer Status Table(s) (RAN2); - Delay reporting of buffered data in uplink (RAN2); - Discard operation of PDU Sets for DL and UL (RAN2, RAN3);

In this contribution, we discuss the remaining details on XR specific capacity improvement enhancements for configured grants, RRC parameters list, and draft UE features.

# Discussion

The Rel-18 XR work item scope includes the RAN1 work to introduce multiple PUSCH transmission occasions and dynamic indication of unused occasions based on the UCI information reported by UE.

# Remaining issues

## PDCCH monitoring resumption after NACK

In the last RAN#101 [2], PDCCH monitoring resumption after sending UL NACK was endorsed. As per RAN#101 guidance, the following needs to be defined in RAN1 to support this feature in Rel-18:

- RRC parameter to enable/disable PDCCH monitoring resumption.

- A UE feature with optional UE capability

Proposal 1: RAN1 shall define a UE feature and RRC parameter for PDCCH monitoring resumption after NACK.

We discuss the details for this RRC parameter and UE feature in section-4 and section-5 below.

## Reply LS on HARQ ID for multi-PUSCHs CG

In the last meeting, RAN2 sent an LS to RAN1 informing about their agreement and requesting further information on the definition of “invalid CG PUSCH” [3].

Note: A configured CG PUSCH is invalid if the CG PUSCH is dropped due to collision with DL symbol(s) indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated or SSB.

There were discussions on the last RAN2 meeting on whether measurement gap configurations or DCI-based SFI indication should also be considered for the definition of “invalid CG PUSCH”. In our view this is further optimization which does not provide clear benefits. Hence there is no need to have any further discussion on “invalid CG PUSCH” definition.

We propose to provide the relevant L1 specification to RAN2 without unnecessary technical discussions.

Proposal 2: RAN1 shall inform RAN2 about the relevant L1 specification clause without having further discussions on the definition of “valid CG PUSCH”.

## Multiple configurations for multi-PUSCHs CG

In the last RAN1-114 meeting [4], the following agreement was made for further down-selection:

Agreement Select one of the following options: · Option 1: Introduce a new capability to indicated maximum number of multi-PUSCH CG configurations (at least 2) per BWP of a serving cell and across all serving cells o FG 50-1 as pre-requisite. o FG 11-9 NOT as pre-requisite · Option 2: Introduce a new capability to indicated maximum number of multi-PUSCH CG configurations (at least 2) per BWP of a serving cell and across all serving cells. The maximum number should not exceed the corresponding maximum number of CG configurations indicated by FG 11-9. o FG 50-1 as pre-requisite. o FG 11-9 as pre-requisite Option 3: Maximum number of multi-PUSCH CG configuration per BWP of a serving cell is one.

Since supporting multiple CG configurations is an optional feature, a UE that supports R18 multi-PUSCHs CG feature may or may not support R16 multiple CG configurations feature. Both cases should be considered for this issue.

Option-1 implies that UE may support multiple CG configurations where each CG configuration configure at least 2 PUSCH TOs while the same UE does not support multiple CG configurations with single PUSCH TO per CG. In our view this creates a contradictory capability reporting which should be avoided.

Option-2 is a more reasonable case where UE that supports R18 multiple CG configurations must also support R16 multiple CG configurations. Otherwise, UE can only support one CG configuration with multi-PUSCHs.

Option-1 simply limits the number of CG configurations with multi-PUSCHs to 1. Since there is no support for UTO-UCI indications across different CG configurations in R18, this is also a reasonable approach to support only one CG configuration with multi-PUSCHs in R18.

In summary, in our view, both Option-2 and Option-3 are reasonable options. Due to lack of cross-CG UTO-UCI indication support in R18, we propose to support only one CG configuration with multi-PUSCHs in R18 (Option-1).

Proposal 3: Support Option-1. The maximum number of multi-PUSCH CG configuration per BWP of a serving cell is one.

# RRC parameters for XR

| Sub-feature group | Parameter name in the spec | New or existing? | Description | Value range | Per (UE, cell, TRP, …) |
| --- | --- | --- | --- | --- | --- |
| Multiple CG PUSCH transmission occasions in a CG period | nrofSlots_InCGperiod | new | Number of consecutive slots for CG PUSCH transmission occasions in a period of a single CG PUSCH configuration | INTEGER(2..32) | Per BWP, in ConfiguredGrantConfig |
| Indication of unused CG PUSCH occasion(s) by UCI | nrof_UTO_UCI | new | Indication of UTO-UCI is enabled if nrof_UTO_UCI is configured. nrof_UTO_UCI is the number of bits in the UTO-UCI bitmap. | INTEGER(3..8) | Per BWP, in ConfiguredGrantConfig |
| Indication of unused CG PUSCH occasion(s) by UCI | Beta-offset_UTO_UCI | new | Beta offset value for UTO-UCI multiplexing on CG PUSCH. Beta-offset_UTO_UCI is required to be present when nrof_UTO_UCI field is configured. | INTEGER (0..31) | Per BWP, in ConfiguredGrantConfig |

The draft list of RRC parameters were agreed in the last RAN1meeting [5]. The following three parameters are listed.

In our view the current list of RRC parameters is stable for XR enhancements for RAN1-led tasks, and therefore no revision is needed.

In addition to the above list of RRC parameters, a new RRC parameter needs to be defined for PDCCH monitoring resumption after NACK feature, as listed below.

Based on the RAN-101’s endorsement of [6], we propose the following:

| Sub-feature group | Parameter name in the spec | New or existing? | Description | Value range | Per (UE, cell, TRP, …) |
| --- | --- | --- | --- | --- | --- |
| PDCCH monitoring resumption after NACK | PdcchMonitoringResumptionAfterNack | new | Number of consecutive slots for CG PUSCH transmission occasions in a period of a single CG PUSCH configuration | {enabled} | Per UE |

Proposal 4: Introduce a new optional RRC parameter (as defined in the table above) to enable/disable PDCCH monitoring resumption after NACK.

# UE feature discussion for XR

Draft UE feature list was agreed in the last RAN1-114 meeting [7]. The following two features are listed.

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE |
| --- | --- | --- | --- | --- | --- | --- |
| 50-1 | Multi-PUSCHs for Configured Grant | 1. Determination of time-domain resource allocation for CG-PUSCHs associated to a multi-PUSCHs CG FFS whether to separate this FG for type-1 and type-2 CG FFS whether to separate this FG for multiple CG configurations FFS whether to separate this FG for shared spectrum FFS whether to report maximum supported number of configured CG-PUSCH TOs in one CG period |  | Yes | N/A | UE is not able to support Multi-PUSCHs per one period in Configured grant in licensed band |
| 50-2 | UCI indication of unused CG-PUSCH transmission occasions | 1. Multiplexing of the Unused transmission occasions UCI (UTO-UCI) on a CG-PUSCH FFS whether to merge this FG into FG 50-1 FFS whether to separate this FG for UTO-UCI multiplexing with HARQ-ACK |  | Yes | N/A | UE is not able to indicate the unused resources in CG |

We propose our views on the current draft list of UE features for XR enhancements in our other Tdoc [8]. In addition to the current list of draft UE features, we propose to include a new UE feature for PDCCH monitoring resumption after NACK.

As per RAN-101 endorsement, a new optional UE feature shall be defined for PDCCH monitoring resumption. Since this feature is a further enhancement on the existing R17 PDCCH skipping, it should be a prerequisite for this new UE feature.

Proposal 5: Introduce a new optional UE feature (as defined in the table below) for PDCCH monitoring resumption after NACK.

| Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE |
| --- | --- | --- | --- | --- | --- | --- |
| 50-3 | PDCCH monitoring resumption after NACK | If the UE transmits a PUCCH or a PUSCH providing a NACK value due to incorrectly decoding a PDSCH scheduled[/activated] by a DCI format received from the serving cell, the UE terminates PDCCH skipping | 29-3a | Yes | N/A | UE is not able to support PDCCH monitoring resumption after sending NACK in uplink. |

# Conclusions

We have the following proposals:

Proposal 1: RAN1 shall define a UE feature and RRC parameter for PDCCH monitoring resumption after NACK.

Proposal 2: RAN1 shall inform RAN2 about the relevant L1 specification clause without having further discussions on the definition of “valid CG PUSCH”.

Proposal 3: Support Option-1. The maximum number of multi-PUSCH CG configuration per BWP of a serving cell is one.

Proposal 4: Introduce a new optional RRC parameter (as defined in the table above) to enable/disable PDCCH monitoring resumption after NACK.

Proposal 5: Introduce a new optional UE feature (as defined in the table below) for PDCCH monitoring resumption after NACK.

# References

- RP-230786, “Updated WID on XR Enhancements for NR”, Nokia, Qualcomm, March 2023.

- Chairman's notes, RAN-101, September 2023.

- R1-2308825, Reply LS on XR capacity enhancements, RAN2, MediaTek, August 2023.

- Chairman's notes, RAN1-114, Toulouse, August 2023.

- R1-2308668, Summary of discussions on RRC parameters for Rel-18 XR WI, Moderator (Qualcomm), August 2023.

- RP-231820, PDCCH monitoring resumption after UL NACK, vivo, MediaTek, Qualcomm, ZTE, Sanechips, Ericsson, Huawei, HiSilicon, September 2023.

- R1-2308521, Updated RAN1 UE features list for Rel-18 NR after RAN1#114, Moderators (AT&T, NTT DOCOMO, INC.), August 2023.