3GPP TSG RAN WG1 #114bis  	            R1-2310065

Xiamen, China, October 9th – October 13th, 2023

Source:	NTT DOCOMO, INC.

Title:	Discussion on UE features for NR NCR

Agenda Item:	8.16.11

Document for: 	Discussion and Decision

Introduction

Updated RAN1 UE features list for Rel-18 NR after RAN1#114 is captured in R1-2308521 [1]. In this contribution, we provide our views on UE features for NR NCR.

Discussions

43-3

| 43. NR_netcon_repeater | 43-3 | Aperiodic beam indication for access link | 1.Support aperiodic beam indication for access link 2. Supported slot-offset k values for reference slot | 43-1 | Yes | N/A | NCR-MT cannot decode the aperiodic beam indication | Per NCR-MT | No | No | Yes | Component 2 candidate values: - 15 kHz: {[0,] 1} - 30 kHz: {[0,]1} - 60 kHz: {[0,]1, 2} - 120 kHz: {[0,]1, 2} | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

One remaining issue in NCR UE feature is whether to support 0 as slot offset k value in aperiodic beam indication. The slot-offset represents the minimum gap between DCI and applicable time resource of beam indication. The values supported in UE capability of beam switching should also be supported by NCR. In UE capability, 7 symbols can be reported for 60kHz for time offset between DCI and PDSCH beam switching. Moreover, NCR may support more advanced capability than UE, thus, 0 slot should be supported.

Proposal 1: for component 2 in FG43-3, support 0 as candidate value of slot offset k, i.e., support component 2 candidate values as follows.

- 15kHz: 0,1

- 30kHz: 0,1

- 60kHz: 0,1,2

- 120kHz: 0, 1,2

Summary

Based on the discussions in Session 2, we propose to adopt the following update for NR NCR UE feature.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 43. NR_netcon_repeater | 43-3 | Aperiodic beam indication for access link | 1.Support aperiodic beam indication for access link 2. Supported slot-offset k values for reference slot | 43-1 | Yes | N/A | NCR-MT cannot decode the aperiodic beam indication | Per NCR-MT | No | No | Yes | Component 2 candidate values: - 15 kHz: {0, 1} - 30 kHz: {0,1} - 60 kHz: {0,1, 2} - 120 kHz: {0,1, 2} | Optional with capability signaling |

Reference

[1]	R1-2308521, Updated RAN1 UE features list for Rel-18 NR after RAN1#114, Moderators (AT&T, NTT DOCOMO, INC.), Aug. 2023