3GPP TSG RAN WG1 #114bis                                R1-2310103

Xiamen, China, October 9th – October 13th, 2023

Source:	Sharp

Title:	Remaining issues on physical channel design framework for NR sidelink evolution on unlicensed spectrum

Agenda Item:	8.2.1.2

Document for:	Discussion and Decision

Introduction

In RAN#97e meeting, Rel-18 work item on support of [1] has the following objectives to study and specify support of sidelink on unlicensed spectrum.

2. Study and specify support of sidelink on unlicensed spectrum for both mode 1 and mode 2 where Uu operation for mode 1 is limited to licensed spectrum only [RAN1, RAN2, RAN4] - Channel access mechanisms from NR-U shall be reused for sidelink unlicensed operation - Assess the applicability of sidelink resource reservation from Rel-16/Rel-17 to sidelink unlicensed operation within the boundaries of unlicensed channel access mechanism and operation - No specific enhancements for Rel-17 resource allocation mechanisms - If the existing NR-U channel access framework does not support the required SL-U functionality, WGs will make appropriate recommendations for RAN approval. - Physical channel design framework: Required changes to NR sidelink physical channel structures and procedures to operate on unlicensed spectrum - The existing NR sidelink and NR-U channel structure shall be reused as the baseline. - No specific enhancements for existing NR SL feature - The study should focus on FR1 unlicensed bands (n46 and n96/n102) and is to be completed by RAN#98. - Note: In sidelink unlicensed operation, the gNB does not perform Type 1 channel access to initiate and share a channel occupancy, neither Type 2 channel access to share an initiated channel occupancy, nor semi-static channel access procedures to access an unlicensed channel.

In this contribution, we discuss the remaining issues of physical channel design for sidelink communication on unlicensed spectrum.

Discussions

PSCCH/PSSCH

RA mode 1

In legacy SL, for RA mode 1, the network needs to provide the SL UE with a “Lowest index of the subchannel allocation to the initial transmission” field. This allows the SL UE to know which sub-channel in the resource pool is used as the starting sub-channel for the initial transmission. In SL-U, either contiguous RB based transmission or interlace RB based transmission can be configured for PSCCH/PSSCH transmission in a resource pool. For contiguous RB based transmission, the “Lowest index of the subchannel allocation to the initial transmission” field can be reused. However, for interlace RB based transmission, where the sub-channel indexes are common to each RB set in the resource pool, providing only the lowest sub-channel index is insufficient for the SL UE to identify the exact frequency resource allocation. In other words, the SL UE cannot determine which sub-channel in which RB set is allocated to the initial transmission. Therefore, it is necessary for the network to further provide the SL UE with a field in DCI format 3_0 to indicate lowest index of the RB set allocation to the initial transmission.

Proposal 1: Adopt the following TP#1 in TS 38.212 to specify frequency resource allocation to initial transmission in RA mode 1 in SL-U.

| Reason for change: | - For RA mode 1, current specification does not specify RB set allocation in DCI format 3_0 for initial transmission when interlace RB-based transmisison is configured for PSCCH/PSSCH transmission. - In subclause 8.3.1.1 of TS38.212, N subChannel SL is a total number of sub-channels in the resource pool to be used for determining frequency reosurce assigenment for Rel-16/17 SL and SL-U where contiguous RB-based transmisison is configured for PSCCH/PSSCH transmisison, while N subChannel RBset is the number of sub-channels for each RB set to be used for determining sub-channel allocation. Therefore, for determinging “Lowest index of the subchannel allocation to the initial transmission” field in DCI format 3_0, same symbols should be used. |
| --- | --- |
| Summary of change: | - Add a new field of “Lowest index of the RB set allocation to the initial transmission” in DCI format 3_0 to specify RB set allocation to initial transmission in RA mode 1. - Clarify N subChannel SL is used to determine the size of “Lowest index of the subchannel allocation to the initial transmission” for Rel-16/17 SL and SL-U where contiguous RB-based transmisison is configured for PSCCH/PSSCH transmisison and N subChannelRBset is used to determine the size of “Lowest index of the subchannel allocation to the initial transmission” for SL-U where interlace RB-based transmisison is configured for PSCCH/PSSCH transmisison |
| Consequences if not approved: | - Interlace RB-based transmisison can not be well supported for RA mode 1. - It is not clear how to determine the size of “Lowest index of the subchannel allocation to the initial transmission” for interlace RB-based transmisision. |

==== TP#1 (TS38.212 V18.0.0 (2023-09)) === ##### 7.3.1.4.1 Format 3_0 DCI format 3_0 is used for scheduling of NR PSCCH and NR PSSCH in one cell. The following information is transmitted by means of the DCI format 3_0 with CRC scrambled by SL-RNTI or SL-CS-RNTI: - Resource pool index –log2I bits, where I is the total number of resource pools for transmission configured by the higher layer parameter sl-TxPoolScheduling, if configured, and sl-DiscTxPoolScheduling, if configured. - Time gap – 3 bits determined by higher layer parameter sl-DCI-ToSL-Trans, as defined in clause 8.1.2.1 of [6, TS 38.214] - HARQ process number – 4 bits. - New data indicator – 1 bit. - Lowest index of the subchannel allocation to the initial transmission –log2(N subChannel SL) bits if the higher layer parameter transmissionStructureForPSCCHandPSSCH in SL-BWP-Config is not configured or configured to ‘contigousRB’; otherwise log2(N subChannel RBset) bits if higher layer parameter transmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured to ‘interlaceRB’, as defined in clause 8.1.2.2 of [6, TS 38.214] - Lowest index of the RB set allocation to the initial transmission –log2(N RBset ) bits as defined in clause 8.1.2.2 of [6, TS 38.214] if the higher layer parameter transmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured to ‘interlaceRB’, 0 bit otherwise - SCI format 1-A fields according to clause 8.3.1.1: - Frequency resource assignment. - Time resource assignment. < Unchanged parts are omitted >

RRC parameter

Likewise, for sidelink configured grant type 1, a new RRC parameter to indicate the starting RB set of sidelink configured grant Type 1 is required as well for interlace RB-based transmission.

Proposal 2: Regarding frequency domain resource indication to initial transmission in sidelink resource allocation mode 1 in SL-U, support the following:

- Sidelink configured grant Type 1 provides RRC parameter sl-StartRBsetCG-Type1 to indicate starting RB set index of the sidelink configured Type 1.

Parameter sl-StartRBsetCG-Type1

| - RAN1 spec | - Description | - Value range | - Default value | - Per (UE, cell, TRP, …) | - UE or cell specific |
| --- | --- | --- | --- | --- | --- |
| - [38.212] | - This field indicates the starting RB set of sidelink configured grant Type 1. An index giving valid RB set index | - 0..3 |  | - Per configured grant | - UE specific |

RB set allocation

Section 8.1.2.2 in TS 38.213 generally specifies resource allocation in frequency domain. Currently, how to determine sub-channel assignment for sidelink transmission is specified, while sub-channel assignment for sidelink transmission has not been specified. Therefore, same section 8.1.2.2 can be used to specify the RB set allocation for interlace RB-based PSCCH/PSSCH transmission.

Proposal 3: Adopt the following to TP#2 in TS38.214 to specify RB set allocation in frequency domain for interlace RB-based PSCCH/PSSCH transmission.

| Reason for change: | In subclasue 8.1.2.2 of TS 38.213, there is a general description for specifying sub-channel allocation in frequency domain. However, for interlace RB-based transmission, the decription for specifying RB set allocation is missed. |
| --- | --- |
| Summary of change: | Add a description to clarify RB set assignment for interlace RB-based transmission. |
| Consequences if not approved: | The description to clarify RB set assignment for interlace RB-based transmisison is missed in 8.1.2.2 regarding resoruce allocation in frequency domain. |

==== TP#2 (TS38.214 V18.0.0 (2023-09)) === #### 8.1.2.2 Resource allocation in frequency domain The resource allocation unit in the frequency domain is the sub-channel. The sub-channel assignment for sidelink transmission is determined using the "Frequency resource assignment" field in the associated SCI. The RB set assignment for sidelink transmission is determined using the "Frequency resource assignment" field in the associated SCI if the higher layer parameter transmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured to ‘interlaceRB’. The lowest sub-channel for sidelink transmission is the sub-channel on which the lowest PRB of the associated PSCCH is transmitted. If a PSSCH scheduled by a PSCCH would overlap with resources containing the PSCCH, the resources corresponding to a union of the PSCCH that scheduled the PSSCH and associated PSCCH DM-RS are not available for the PSSCH. When PSSCH is transmitted on multiple RB sets, the corresponding PSCCH is located on the sub-channel with smallest index of the lowest RB set.

Candidate resource exclusion due to C-LBT failure

According to the RAN2 LS [2], RAN2 asked RAN1 to take two agreements (as highlighted below) into account in RAN1 work. The first agreement has been implemented in RAN1 specification while the second one has not been implemented in RAN1 specification yet.

1. Overall Description: RAN2 discussed mode 2 resource (re)selection due to the detection of consistent LBT (C-LBT) failure on an RB set, and made the following agreements: - MAC informs PHY of the RB set information where SL C-LBT failure was detected. - During resource (re)selection, PHY excludes the resources for the RB set where C-LBT failure was detected. 2. Actions: To RAN WG1 ACTION: RAN2 would like to ask RAN1 to take the above agreements into account in their work.

In the last RAN1 meeting, for contiguous RB-based transmission, it was agreed to exclude candidate resources with lowest sub-channel, including guardband resource blocks, in step 1 of resource selection procedure in section 8.1.4 of TS 38.214. Such exclusion, performed in step 1, ensures that the Mtotal represents the number of remaining candidate resources after exclusion, and the initial candidate resource set SA will not include the unavailable candidate resources. The same principle can also be applied to the exclusion of candidate resources due to consistent LBT failure. That is, in step 1, PHY excludes candidate resources that include at least one RB set in which SL C-LBT failure was detected.

Proposal 4: Adopt the following TP#3 to implement the candidate resources exclusion due to C-LBT failure in Step 1 in section 8.1.4 of TS 38.214.

- Note: the candidate resources exclusion by PHY due to C-BLT failure was agreed in RAN2 and send in the RAN2 LS [2].

| Reason for change: | LS R1-2306371 from RAN2 ask RAN1 to take two agreements into account in RAN1 work. The first one of the two agreements has been implemented in RAN1 specification while the second one has not been implemented in RAN1 specification yet. RAN2 agreements in LS R1-2306371 - MAC informs PHY of the RB set information where SL C-LBT failure was detected. - During resource (re)selection, PHY excludes the resources for the RB set where C-LBT failure was detected. |
| --- | --- |
| Summary of change: | Add descriptions in step 1 of resource selection procedure to specify that PHY excludes candidate resources that include at least one RB set in which SL C-LBT failure was detected. |
| Consequences if not approved: | Candidate resource exclusion due to SL C-LBT failure in PHY is not supported. |

==== TP#3 (TS38.214 V18.0.0 (2023-09)) === - 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 < Unchanged parts are omitted > If the higher layer parameter transmissionStructureForPSCCHandPSSCH is set to ‘contiguousRB', the UE shall exclude candidate single-slot or candidate multi-slot resources with the lowest sub-channel including resource blocks of the intra-cell guardband PRBs, configured by higher layer parameter, intraCellGuardBandsSL-List. If the higher layer parameter transmissionStructureForPSCCHandPSSCH is provided, the UE shall exclude candidate single-slot or candidate multi-slot resources including at least one RB set indicated by the higher layer parameter rbSetsWithConsecutiveLBTFailure. The total number of remaining candidate single-slot resources or candidate multi-slot resources is denoted by Mtotal. < Unchanged parts are omitted >

PSFCH

Resource configuration for N candidate PSFCH occasions

The following agreements regarding N candidate PSFCH occasions were reached in last meetings.

Agreement Regarding one PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s) via (pre-)configuration: - Value range of N at least includes {1, 2, 3, 4} - When N>1, N associated candidate PSFCH occasion(s) for one PSCCH/PSSCH transmission have different time and/or frequency resource with the candidate PSFCH occasion(s) for another PSCCH/PSSCH transmission, at least if these two PSCCH/PSSCH transmissions are on non-overlapped resources

Agreement Regarding “one PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s)” and “For one PSCCH/PSSCH transmission, at least support that its associated candidate PSFCH occasion(s) are in different slots of the same RB set(s)”, support: - Slot index of 1st PSFCH occasion (denoted as slot k) of a PSCCH/PSSCH transmission is determined in the same way as legacy NR SL - The nth PSFCH occasion is in slot k+n-1*P - Alt 1: P is equal to the (pre-)configured PSFCH periodicity, i.e., P is provided by sl-PSFCH-Period - 1≤n≤N - Within a slot including PSFCH, for each RB set, the (pre-)configured PRBs for PSFCH transmission on this RB set are divided into N different PRB sets (denoted as set#1, set#2, …, set#N), which are associated with N candidate PSFCH occasion(s) - Within this RB set, for one sub-channel on one slot of PSCCH/PSSCH transmission, its nth PSFCH occasion includes PRBs belonging to above set#n in slot k+n-1*P - FFS: whether to use 1 or N bitmaps to indicate resource for N candidate PSFCH occasion(s), respectively

For N candidate PSFCH occasions associated with one PSCCH/PSSCH transmission, there is one remaining FFS regarding whether to use 1 or N bitmaps to indicate resource for N candidate PSFCH occasions. Regarding the bitmap format, it was agreed to use a legacy PRB-level bitmap to indicate the set of PRBs for PSFCH transmission in SL-U. The size of the PRB-level bitmap depends on the size of the resource pool and can be up to 275bits. While N bitmaps provide more configuration flexibility than using 1 bitmap, it is unclear if this level of the configuration flexibility is necessary. Additionally, N bitmaps consume more signaling. For instance, to support N = 4 candidate PSFCH occasions, using N bitmaps would require an additional up to 3*275=825 bits, which would be a significant burden for SIB 12 transmission. Therefore, it is preferred to use only 1 bitmap to indicate the resource for N candidate PSFCH occasions in order to reduce signaling consumption and alleviate the burden on SIB 12 transmission.

Proposal 5: Regarding “one PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s)”, use 1 bitmap to indicate resource for N candidate PSFCH occasion(s).

If the resource pool includes multiple RB sets, the legacy resource pool level bitmap will provide a set of PRBs for PSFCH transmission for multiple RB sets. That is, the UE can be provided a set of MPRB, setPSFCH PRBs in the resource pool by the bitmap sl-PSFCH-RB-Set where the set of MPRB, setPSFCH PRBs includes (pre-)configured PRBs for multiple RB sets. However, the current specification has not specified how to divide the set of MPRB, setPSFCH PRB to each RB set in details. It is necessary to clarify that PRBs from the set of MPRB, setPSFCH PRBs, that are within the RB set, are used for PSFCH transmission in the RB set.

In addition, it was also agreed to divide the configured PRBs for PSFCH transmission in an RB set into N PSFCH occasions. However, how to divide the configured PRBs for PSFCH transmission in an RB set into N PSFCH occasions has not been specified in the specification. One straightforward way is to equally divide the set of PRBs indicated as “1” by the bitmap sl-PSFCH-RB-Set, in an ascending order of PRB index, into N candidate PSFCH occasions. For illustration, a set of MPRB, set, kPSFCH PRBs represent PRBs within an RB set k that are indicated as “1” by the bitmap sl-PSFCH-RB-Set. For each PSFCH occasion in a PSFCH slot, to ensure each sub-channel in each slot during PSFCH period to be allocated at least one subset of K3 PRBs, the number of MPRB, set, kPSFCH PRBs is expected to be a multiple of (Nsubchk∙NPSSCHPSFCH∙K3∙N). Figure 1 below is an example of how to divide MPRB, set, kPSFCH PRBs within an RB set k into N PSFCH occasions. As illustrated in the Figure 1, the UE can equally allocate [n∙MPRB.set,kPSFCH/N, n+1∙MPRB.set,kPSFCH/N] PRBs to the n PSFCH occasions.

Figure 1: division of (pre-)configured PRBs for PSFCH in an RB set k into N PSFCH occasions when sl-PSFCH-Type = ‘type2’

Proposal 6: Adopt the following TP#4 in TS38.213 to specify dedicated PRB allocation to a candidate PSFCH transmission occasion in an RB set when sl-PSFCH-Type = ‘type2’.

| Reason for change: | - The sl-PSFCH-RB-Set provides a set of MPRB, setPSFCH PRBs for PSFCH transmission in multiple RB sets in the recource pool. However, current specification does not specify in details how to divide the set of MPRB, setPSFCH PRBs into each RB set when sl-PSFCH-Type = ‘type2’. - It was also agreed to divide the configured PRBs for PSFCH transmission in an RB set into N PSFCH occasions. However, how to divide the configured PRBs for PSFCH transmission in an RB set into N PSFCH occasions has not been specified in the specification when sl-PSFCH-Type = ‘type2’. |
| --- | --- |
| Summary of change: | - Add descriptions to clarify that PRBs for PSFCH transmisison for an RB set are those PRBs from the set of MPRB, setPSFCH PRBs which locate within the RB set. - Add descriptions to clarify how to divide the configured PRBs for PSFCH transmission in an RB set into each of N PSFCH occasions. |
| Consequences if not approved: | - It is not clear which PRBs from the set of MPRB, setPSFCH PRBs are used for an RB set for PSFCH transmission when sl-PSFCH-Type = ‘type2’. - It is not clear how to divide the configured PRBs for PSFCH transmission in an RB set into each of N PSFCH occasions when sl-PSFCH-Type = ‘type2’. |

==== TP#4 (TS38.213 V18.0.0 (2023-09)) === - 16.3.0 UE procedure for transmitting PSFCH with control information < Unchanged parts are omitted > For operation with shared spectrum channel access, when sl-PSFCH-Type = ‘type2’ and within RB-set k, a UE determines a subset of PRBs in a first interlace and, based on sl-PSFCH-RB-Set, a subset of NPRBPSFCH PRBs in a second interlace for a PSFCH transmission with HARQ-ACK information in a resource pool. An index of the first interlace is provided by sl-PSFCH-Type2-CommonInterlace. The UE is provided by sl-PSFCH-RB-Set, a set of MPRB,setPSFCH PRBs in a resource pool for PSFCH transmission with HARQ-ACK information. The UE determines a set of MPRB,set,kPSFCH PRBs from the MPRB,setPSFCH PRBs where the MPRB,set,kPSFCH PRBs are within RB-set k for PSFCH transmission with HARQ-ACK information. For a n-th candidate PSFCH transmission occasion within RB-set k, where 1≤n≤NoccasionPSFCH, the UE allocates n-1⋅MPRB, set,kPSFCH/NoccasionPSFCH,n⋅MPRB, set,kPSFCH/NoccasionPSFCH-1 PRBs from MPRB,set,kPSFCH PRBs to the n-th candidate PSFCH transmission occasion. The NPRBPSFCH PRBs in the second interlace are provided by sl-PSFCH-Type2-DedicatedPRB where, for the n-th candidate PSFCH transmission occasion, 1≤n≤NoccasionPSFCH, and for each interlace l, the UE determines MPRB,k, lPSFCH,n PRBs from the allocated n-1⋅MPRB, set,kPSFCH/NoccasionPSFCH,n⋅MPRB, set,kPSFCH/NoccasionPSFCH-1 PRBs based on sl-PSFCH-RB-Set. The UE expects that MPRB,k,lPSFCH,n is a multiple of NPRBPSFCH. For interlace l, the UE determines a PRB subset with index s to include PRBs NPRBPSFCH⋅s, NPRBPSFCH⋅s+1, …, NPRBPSFCH⋅s+1-1, 0≤s≤MPRB,k,lPSFCH,n/NPRBPSFCH-1. The UE determines the Msubset,kPSFCH,n PRB subsets by ordering the PRB subsets first in an ascending order of PRB subset index within an interlace and second in ascending order of interlace index. For a number of Nsubchk sub-channels in RB-set k and a number of slots for PSSCH transmissions that is not larger than NPSSCHPSFCH and is associated with a slot for PSFCH transmission, the UE allocates the i+j⋅NPSSCHPSFCH⋅Msubch, slot,kPSFCH,n, i+j⋅NPSSCHPSFCH⋅Msubch, slot,kPSFCH,n+1, …, i+1+j⋅NPSSCHPSFCH⋅Msubch, slot,kPSFCH,n-1 PRB subsets from the Msubset,kPSFCH,n PRB subsets to slot i among the slots for PSSCH transmissions that are associated with the slot and sub-channel j for PSFCH transmissions, where Msubch, slot,kPSFCH,n=Msubset,kPSFCH,nNsubchk⋅NPSSCHPSFCH and 0≤i<NPSSCHPSFCH, 0≤j<Nsubchk. The allocation starts in an ascending order of i and continues in an ascending order of j. The UE expects that Msubset,kPSFCH,n is a multiple of Nsubchk∙NPSSCHPSFCH. < Unchanged parts are omitted >

For Sl-PSFCH-Type = type 1, similar to the Sl-PSFCH-Type = type 2, it is also necessary to specify how to determine PRBs from the set of MPRB, setPSFCH PRB for an RB set for PSFCH transmission in the RB set. Additionally, how to determine a set of interlaces for n-th candidate PSFCH occasions has not been specified as well in an RB set. Considering that the type 1 require all PRBs of an interlace are used for one PSFCH transmission, the UE needs to first determine a set of interlaces based on the bitmap sl-PSFCH-RB-Set, and then specifically allocate the set of interlace to a candidate PSFCH occasion in an ascending order of interlace index. By doing these, a UE can clearly understand which interlaces are allocated to the n-th candidate PSFCH occasions associated with its PSSCH transmission.

Proposal 7: Adopt the following TP#5 in TS38.213 to specify dedicated interlace allocation to a candidate PSFCH transmission occasion in an RB set when sl-PSFCH-Type = ‘type1’.

| Reason for change: | - The sl-PSFCH-RB-Set provides a set of MPRB, setPSFCH PRBs for PSFCH transmission in multiple RB sets in the recource pool. However, current specification does not specify in details how to divide the set of MPRB, setPSFCH PRBs into each RB set when sl-PSFCH-Type = ‘type1’. - When sl-PSFCH-Type = ‘type1’, a PSFCH transmisisn occupies all PRBs in the interlace in an RB set. How the UE determines interlaces for PSFCH transmission from (pre-)configured PRBs for an RB set and how the UE divides the interlaces in the RB set into each of N PSFCH occasions is not specified as well. |
| --- | --- |
| Summary of change: | - Add descriptions to clarify that PRBs for PSFCH transmisison for an RB set are those PRBs from the set of MPRB, setPSFCH PRBs which locate within the RB set. - Add descriptions to clarify how to derive interlaces for PSFCH transmission for an RB set and how to divide the derived interlaces in the RB set into each of N PSFCH occasions. |
| Consequences if not approved: | - It is not clear which PRBs from the set of MPRB, setPSFCH PRBs are used for an RB set for PSFCH transmission when sl-PSFCH-Type = ‘type1’. - It is not clear how to derive interlaces for PSFCH transmisison from (pre-)configured PRBs for an RB set and how to divide the derived interlaces in the RB set into each of N PSFCH occasions when sl-PSFCH-Type = ‘type1’. |

==== TP#5 (TS38.213 V18.0.0 (2023-09)) === - 16.3.0 UE procedure for transmitting PSFCH with control information < Unchanged parts are omitted > For operation with shared spectrum channel access, when sl-PSFCH-Type = ‘type1’ and within RB-set k, a UE determines, based on sl-PSFCH-RB-Set, all PRBs of an interlace for one PSFCH transmission with HARQ-ACK information in the resource pool. The UE is provided by sl-PSFCH-RB-Set, a set of MPRB,setPSFCH PRBs in a resource pool for PSFCH transmission with HARQ-ACK information. The UE determines from the MPRB,setPSFCH PRBs a set of MPRB,set,kPSFCH PRBs within RB-set k for PSFCH transmission with HARQ-ACK information. The UE determines a set of interlaces Minterlace,kPSFCH within RB-set k from the MPRB,set,kPSFCH PRBs. For a n-th candidate PSFCH transmission occasion within RB-set k, 1≤n≤NoccasionPSFCH, the UE allocates n-1⋅Minterlace,kPSFCH,n,n⋅Minterlace,kPSFCH,n-1 interlaces from the Minterlace,kPSFCH interlaces to the n-th candidate PSFCH transmission occasion, where Minterlace,kPSFCH,n=Minterlace,kPSFCH/NoccasionPSFCH. For the n-th candidate PSFCH transmission occasion, 1≤n≤NoccasionPSFCH, the UE determines a set of interlaces that includes a number Minterlace,kPSFCH,n of interlaces based on sl-PSFCH-RB-Set. The set of Minterlace,kPSFCH,n interlaces are indexed in an ascending order of interlace indexes. For each interlace of the set of interlaces, all PRBs in the interlace are available for PSFCH transmission. For a number of Nsubchk sub-channels in RB-set k and a number of PSSCH slots that is not larger than NPSSCHPSFCH and is associated with a slot for PSFCH transmission, the UE allocates the i+j⋅NPSSCHPSFCH⋅Msubch, slot,kPSFCH,n, i+1+j⋅NPSSCHPSFCH⋅Msubch, slot,kPSFCH,n-1 interlaces from the Minterlace,kPSFCH,n interlaces to slot i and sub-channel j, where Msubch, slot,kPSFCH,n=Minterlace,kPSFCH,nNsubchk⋅NPSSCHPSFCH, 0≤i<NPSSCHPSFCH, 0≤j<Nsubchk. The allocation starts in an ascending order of i and continues in an ascending order of j. The UE expects that Minterlace,kPSFCH,n is a multiple of Nsubchk∙NPSSCHPSFCH. < Unchanged parts are omitted >

PSFCH transmission for contiguous RB transmission

Current specification specifies that interlace RB based PSFCH transmission including Alt 1-1b and Alt 2-3a is applied to operation with shared spectrum channel access. The legacy PRB-level PSFCH transmission is applied to operation without shared spectrum channel access. However, for operation with shard spectrum channel access where OCB requirement is not required, legacy SL contiguous RB-based transmission can be applied to PSCCH/PSSCH transmission in SL-U. In this case, it is not necessary to require PSFCH transmission to use interlace RB based transmission. Therefore, legacy PRB-level PSFCH transmission can be supported in the operation with share spectrum channel access where OCB requirement is not required.

Proposal 8: Adopt the following TP#6 in TS 38.213 to specify that legacy PRB-level PSFCH transmission is applied to operation without shared spectrum channel access where OCB requirement is not required.

| Reason for change: | Current specification specifies that interlace RB based PSFCH transmission (i.e., sl-PSFCH-Type = ‘type1’ or sl-PSFCH-Type = ‘type2’) is applied to operation with shared spectrum channel access while legacy PRB-level PSFCH transmission is applied to operation without shared spectrum channel access. However, for operation with shared spectrum channel access where OCB requirement is not required, it is not necessary to mandate UE to perform interlace based transmission for PSFCH. Legacy PRB-level PSFCH transmission can be applied to this case. |
| --- | --- |
| Summary of change: | Clarify that interlace RB based PSFCH transmission is applied for operation with shared spectrum channel where interalce RB-based transmission is configured and legacy PRB-level PSFCH transmisison is applied for Rel-16/17 SL and operation with shared spectrum channel where contigous RB-based transmisison is configured. |
| Consequences if not approved: | UE is mandated to perform interlace RB-based PSFCH transmission even if OCB requiremetn is not required. |

==== TP#6 (TS38.213 V18.0.0 (2023-09)) === - 16.3.0 UE procedure for transmitting PSFCH with control information < Unchanged parts are omitted > For operation without shared spectrum channel access or operation with shared spectrum channel access where the higher layer parameter transmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured to ‘contigousRB’, a UE is provided by sl-PSFCH-RB-Set a set of MPRB, setPSFCH PRBs in a resource pool for PSFCH transmission with HARQ-ACK information in a PRB of the resource pool. A UE can be provided by sl-RB-SetPSFCH a set of MPRB, setPSFCH PRBs in a resource pool for PSFCH transmission with conflict information in a PRB of the resource pool. A UE expects that different PRBs are (pre)configured for conflict information and HARQ-ACK information. For a number of Nsubch sub-channels for the resource pool, provided by sl-NumSubchannel, and a number of PSSCH slots associated with a PSFCH slot that is less than or equal to NPSSCHPSFCH, the UE allocates the i+j⋅NPSSCHPSFCH⋅Msubch, slotPSFCH, i+1+j⋅NPSSCHPSFCH⋅Msubch, slotPSFCH-1 PRBs from the MPRB, setPSFCH PRBs to slot i among the PSSCH slots associated with the PSFCH slot and sub-channel j, where Msubch, slotPSFCH=MPRB, setPSFCHNsubch⋅NPSSCHPSFCH, 0≤i<NPSSCHPSFCH, 0≤j<Nsubch, and the allocation starts in an ascending order of i and continues in an ascending order of j. The UE expects that MPRB, setPSFCH is a multiple of Nsubch∙NPSSCHPSFCH. For operation with shared spectrum channel access where the higher layer parameter transmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured to ‘interlaceRB’, when sl-PSFCH-Type = ‘type1’ and within RB-set k, a UE determines, based on sl-PSFCH-RB-Set, all PRBs of an interlace for one PSFCH transmission with HARQ-ACK information in the resource pool. For the n-th candidate PSFCH transmission occasion, 1≤n≤NoccasionPSFCH, the UE determines a set of interlaces that includes a number Minterlace,kPSFCH,n of interlaces based on sl-PSFCH-RB-Set. The set of interlaces are indexed in an ascending order of interlace indexes. For each interlace of the set of interlaces, all PRBs in the interlace are available for PSFCH transmission. For a number of Nsubchk sub-channels in RB-set k and a number of PSSCH slots that is not larger than NPSSCHPSFCH and is associated with a slot for PSFCH transmission, the UE allocates the i+j⋅NPSSCHPSFCH⋅Msubch, slot,kPSFCH,n, i+1+j⋅NPSSCHPSFCH⋅Msubch, slot,kPSFCH,n-1 interlaces from the Minterlace,kPSFCH,n interlaces to slot i and sub-channel j, where Msubch, slot,kPSFCH,n=Minterlace,kPSFCH,nNsubchk⋅NPSSCHPSFCH, 0≤i<NPSSCHPSFCH, 0≤j<Nsubchk. The allocation starts in an ascending order of i and continues in an ascending order of j. The UE expects that Minterlace,kPSFCH,n is a multiple of Nsubchk∙NPSSCHPSFCH. For operation with shared spectrum channel access where the higher layer parameter transmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured to ‘interlaceRB’, when sl-PSFCH-Type = ‘type2’ and within RB-set k, a UE determines a subset of PRBs in a first interlace and, based on sl-PSFCH-RB-Set, a subset of NPRBPSFCH PRBs in a second interlace for a PSFCH transmission with HARQ-ACK information in a resource pool. An index of the first interlace is provided by sl-PSFCH-Type2-CommonInterlace. The NPRBPSFCH PRBs in the second interlace are provided by sl-PSFCH-Type2-DedicatedPRB where, for the n-th candidate PSFCH transmission occasion, 1≤n≤NoccasionPSFCH, and for each interlace l, the UE determines MPRB,k, lPSFCH,n PRBs based on sl-PSFCH-RB-Set. The UE expects that MPRB,k,lPSFCH,n is a multiple of NPRBPSFCH. For interlace l, the UE determines a PRB subset with index s to include PRBs NPRBPSFCH⋅s, NPRBPSFCH⋅s+1, …, NPRBPSFCH⋅s+1-1, 0≤s≤MPRB,k,lPSFCH,n/NPRBPSFCH-1. The UE determines the Msubset,kPSFCH,n PRB subsets by ordering the PRB subsets first in an ascending order of PRB subset index within an interlace and second in ascending order of interlace index. For a number of Nsubchk sub-channels in RB-set k and a number of slots for PSSCH transmissions that is not larger than NPSSCHPSFCH and is associated with a slot for PSFCH transmission, the UE allocates the i+j⋅NPSSCHPSFCH⋅Msubch, slot,kPSFCH,n, i+j⋅NPSSCHPSFCH⋅Msubch, slot,kPSFCH,n+1, …, i+1+j⋅NPSSCHPSFCH⋅Msubch, slot,kPSFCH,n-1 PRB subsets from the Msubset,kPSFCH,n PRB subsets to slot i among the slots for PSSCH transmissions that are associated with the slot and sub-channel j for PSFCH transmissions, where Msubch, slot,kPSFCH,n=Msubset,kPSFCH,nNsubchk⋅NPSSCHPSFCH and 0≤i<NPSSCHPSFCH, 0≤j<Nsubchk. The allocation starts in an ascending order of i and continues in an ascending order of j. The UE expects that Msubset,kPSFCH,n is a multiple of Nsubchk∙NPSSCHPSFCH. < Unchanged parts are omitted >

COT interruption in PSFCH occasions

Regarding COT interruption due to PSFCH occasions where there were no intended PSFCH transmissions, in last RAN1 meeting, three options are listed as below for further discussion where option 1 and 2 aim for seeking some solutions to avoid COT interruption, while option 3 is with no optimization.

Agreement When neither COT initiating UE nor responding UE intends to transmit PSFCH on some PSFCH occasion(s) within a COT, to avoid COT interruption, select one or more of the followings: - Option 1: COT initiating UE or responding UE transmits PSSCH on such PSFCH occasion(s) - FFS details, e.g., how PSSCH Rx UE knows such transmission, etc. - Option 2: COT initiating UE or responding UE transmits a PSFCH-like signal on such PSFCH occasion(s) - FFS details, e.g., signaling design, etc. - Option 3: no optimization for this case

Considering that PSFCH occasions can be (pre-)configured periodically with a period of 1, 2, or 4 slots, it is therefore inevitable that periodically (pre-)configured PSFCH occasions would exist in a COT initiated by a UE. As illustrated in the Figure 2, especially for those periodically (pre-)configured PSFCH occasions in the beginning of the COT, it is more likely that no PSFCH transmission would happen there, which would result in the COT interruption.

Figure 2: PSFCH occasions (pre-)configured periodically with a period of 1 slot.

Without taking extra solutions to PSFCH occasions without intended PSFCH transmissions, it is very difficult for a UE to occupy a channel and a new initiating COT would be usually interrupted within 1 slot if the period of PSFCH is 1 slot. The risk of COT interruption would affect the functionalities of MCSt and UE-to-UE COT sharing, which are important features to SL-U. It has to be also noted that a lot of effort has been made to avoid losing a COT due to gap symbol and S-SSB slot by CP extensions and S-SSB repetitions. Similarly, solutions to avoid COT lost due to PSFCH occasions should be taken to complement effort what has been made to avoid COT lost.

Observation 1: Option 3 (i.e., no optimization on PSFCH occasions within a COT to avoid COT interruption) makes a COT-initiating UE easily lose its COT especially in the case of short PSFCH periodicity, which severely affects functionalities of MCSt and UE-to-UE COT sharing.

For option 1 to transmit PSSCH on PSFCH occasions, the benefit is that a lower code rate for the PSSCH transmission is achieved given more resources can be used to the PSSCH transmission. However, the option 1 only applies in those PSFCH occasions where the COT initiating UE shall make sure there are no PSFCH transmission from other UEs. Otherwise, the option 1 would lead to collision with potential PSFCH transmission from other UEs. In addition, a notification mechanism on how to notify RX UE of whether the PSSCH is additionally transmitted on the PSFCH occasions is required.

Observation 2: On one hand, Option 1 (i.e., transmits PSSCH on such PSFCH occasion) helps to lower code rate of PSSCH transmission. On the other hand, there is the usability problem that option 1 can only apply in those PSFCH occasions where UE shall ensure no PSFCH transmission from other UEs, as well as a new notification mechanism required to notify RX UE of whether PSFCH occasion is used for PSSCH transmission.

For option 2 to transmit PSFCH-like signal, the option 2 can apply in PSFCH occasions where the UE can be aware or unaware of whether other UEs would transmit PSFCH. Sequence generation for PSFCH transmission can be reused for the PSFCH-like signal. PSFCH-like signal would not impact potential PSFCH transmissions from other UEs. For the case that Alt 1-1b is (pre-)configured for PSFCH transmission in a resource pool, the PSFCH-like signal can be transmitted only on the common interlace.

Observation 3: Option 2 (i.e., transmits PSFCH-like signal on such PSFCH occasion) does not impact other UEs’ PSFCH transmissions.

Proposal 9: When neither COT initiating UE nor responding UE intends to transmit PSFCH on some PSFCH occasions(s) within a COT, to avoid COT interruption, option 2 is supported:

- Option 2: COT initiating UE or responding UE transmits a PSFCH-like signal on such PSFCH occasion(s)

Proposal 10: For the case that Alt 1-1b is (pre-)configured for PSFCH transmission in a resource pool, PSFCH-like signal can be transmitted on 1 common interlace.

IUC feature

During the draft CR discussion, it was argued that IUC feature has not been discussed such that the IUC feature can be not supported in the specification such as PSFCH transmission with conflict information in SL-U. However, the IUC feature should be supported in SL-U.

Proposal 11: Support Rel-17 IUC feature for Rel-18 SL UE in SL-U and support the PSFCH transmission with conflict information in SL-U.

Congestion control

Congestion control

In legacy SL, SL-RSSI is measured on a sub-channel in the frequency domain and the OFDM symbols configured for PSCCH/PSSCH in the time domain. However, in a resource pool configured with two candidate starting symbols, the actual OFDM symbols used for a PSCCH/PSSCH transmission may be autonomously determined by a TX UE, i.e. the information is not available at another UE performing SL-RSSI measurement in the slot. If the same SL-RSSI definition is used, the total received power may be underestimated (e.g. some UEs use the 1st candidate starting symbols while others use 2nd candidate starting symbols).

Therefore, we propose that in such a resource pool, SL-RSSI measurement is only performed on OFDM symbols corresponding to PSCCH/PSSCH transmissions using the 2nd candidate starting symbols.

Proposal 12: Adopt the following TP#7 in TS38.215 to specify that SL-RSSI measurement is performed on OFDM symbols corresponding to PSCCH/PSSCH transmissions using the 2nd candidate starting symbol in a resource pool (pre-)configured with two candidate starting symbols.

| Reason for change: | In legacy SL, SL-RSSI is measured on OFSM symbols configured for PSCCH/PSSCH in time domain. In Rel-18, two candidate starting symbols for PSCCH/PSSCH were introduced. Therefore, actual SL transmission may start from 2nd candidate starting symbol so that there may be no corresponding SL transmission prior to the 2nd candidate starting symbol. If following the same SL-RSSI definition for SL-RSSI measurement, the total received power may be underestimated. |
| --- | --- |
| Summary of change: | Clarify that SL-RSSI is measured starting from the next symbol after the second candidate starting symbol when the SL BWP is configured with two candidate starting symbols. |
| Consequences if not approved: | The measured SL-RSSI may be underestimated. |

==== TP#7 (TS38.215 V18.0.0 (2023-09)) === 5.1.25 Sidelink received signal strength indicator (SL RSSI) Definition; Sidelink Received Signal Strength Indicator (SL RSSI) is defined as the linear average of the total received power (in [W]) observed in the configured sub-channel in OFDM symbols of a slot configured for PSCCH and PSSCH, starting from the 2nd OFDM symbol or starting from the next symbol after the second candidate symbol when startingSymbolFirst and startingSymbolSecond are provided for a SL-BWP. For frequency range 1, the reference point for the SL RSSI shall be the antenna connector of the UE. For frequency range 2, SL RSSI shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported SL RSSI value shall not be lower than the corresponding SL RSSI of any of the individual receiver branches.. Applicable for; RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency.

SL CSI-RS

SL CSI-RS transmission

In a resource pool (pre-) configured with two candidate starting symbols, the set of symbols for a PSSCH transmission (plus AGC and GAP symbols) can be one of the following:

- Case 1: all SL symbols except those for PSFCH (in a slot with PSFCH).

- Case 2: all SL symbols starting from the 1st candidate starting symbol (in a slot without PSFCH, and with the use of the 1st candidate starting symbol).

- Case 3: all SL symbols starting from the 2nd candidate starting symbol (in a slot without PSFCH, and with the use of the 2nd candidate starting symbol).

On the other hand, the SL CSI-RS symbol number is signaled as a symbol within a slot. Therefore, it is likely that a symbol number is valid for SL CSI-RS for one of the above cases, but not other(s), e.g. collision with symbols for DM-RS or 2nd-SCI or PTRS. Hence, we propose that SL CSI-RS symbol is separately indicated for the 2 candidate starting symbol, in order not to restrict the flexibility of CSI acquisition due to the introduction of two candidate starting symbols.

Proposal 13: In a resource pool (pre-) configured with two candidate starting symbols, the CSI-RS symbol is separately (pre-) configured for the 1st candidate starting symbol and the 2nd candidate starting symbol.

Conclusion

In this contribution, we have discussed our views on physical channel design for adapting sidelink communication on unlicensed spectrum and have the following proposals and observations.

Proposal 1: Adopt the following TP#1 in TS 38.212 to specify frequency resource allocation to initial transmission in RA mode 1 in SL-U.

| Reason for change: | - For RA mode 1, current specification does not specify RB set allocation in DCI format 3_0 for initial transmission when interlace RB-based transmisison is configured for PSCCH/PSSCH transmission. - In subclause 8.3.1.1 of TS38.212, N subChannel SL is a total number of sub-channels in the resource pool to be used for determining frequency reosurce assigenment for Rel-16/17 SL and SL-U where contiguous RB-based transmisison is configured for PSCCH/PSSCH transmisison, while N subChannel RBset is the number of sub-channels for each RB set to be used for determining sub-channel allocation. Therefore, for determinging “Lowest index of the subchannel allocation to the initial transmission” field in DCI format 3_0, same symbols should be used. |
| --- | --- |
| Summary of change: | - Add a new field of “Lowest index of the RB set allocation to the initial transmission” in DCI format 3_0 to specify RB set allocation to initial transmission in RA mode 1. - Clarify N subChannel SL is used to determine the size of “Lowest index of the subchannel allocation to the initial transmission” for Rel-16/17 SL and SL-U where contiguous RB-based transmisison is configured for PSCCH/PSSCH transmisison and N subChannelRBset is used to determine the size of “Lowest index of the subchannel allocation to the initial transmission” for SL-U where interlace RB-based transmisison is configured for PSCCH/PSSCH transmisison |
| Consequences if not approved: | - Interlace RB-based transmisison can not be well supported for RA mode 1. - It is not clear how to determine the size of “Lowest index of the subchannel allocation to the initial transmission” for interlace RB-based transmisision. |

==== TP#1 (TS38.212 V18.0.0 (2023-09)) === ##### 7.3.1.4.1 Format 3_0 DCI format 3_0 is used for scheduling of NR PSCCH and NR PSSCH in one cell. The following information is transmitted by means of the DCI format 3_0 with CRC scrambled by SL-RNTI or SL-CS-RNTI: - Resource pool index –log2I bits, where I is the total number of resource pools for transmission configured by the higher layer parameter sl-TxPoolScheduling, if configured, and sl-DiscTxPoolScheduling, if configured. - Time gap – 3 bits determined by higher layer parameter sl-DCI-ToSL-Trans, as defined in clause 8.1.2.1 of [6, TS 38.214] - HARQ process number – 4 bits. - New data indicator – 1 bit. - Lowest index of the subchannel allocation to the initial transmission –log2(N subChannel SL) bits if the higher layer parameter transmissionStructureForPSCCHandPSSCH in SL-BWP-Config is not configured or configured to ‘contigousRB’; otherwise log2(N subChannel RBset) bits if higher layer parameter transmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured to ‘interlaceRB’, as defined in clause 8.1.2.2 of [6, TS 38.214] - Lowest index of the RB set allocation to the initial transmission –log2(N RBset ) bits as defined in clause 8.1.2.2 of [6, TS 38.214] if the higher layer parameter transmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured to ‘interlaceRB’, 0 bit otherwise - SCI format 1-A fields according to clause 8.3.1.1: - Frequency resource assignment. - Time resource assignment. < Unchanged parts are omitted >

Proposal 2: Regarding frequency domain resource indication to initial transmission in sidelink resource allocation mode 1 in SL-U, support the following:

- Sidelink configured grant Type 1 provides RRC parameter sl-StartRBsetCG-Type1 to indicate starting RB set index of the sidelink configured Type 1.

Parameter sl-StartRBsetCG-Type1

| - RAN1 spec | - Description | - Value range | - Default value | - Per (UE, cell, TRP, …) | - UE or cell specific |
| --- | --- | --- | --- | --- | --- |
| - [38.212] | - This field indicates the starting RB set of sidelink configured grant Type 1. An index giving valid RB set index | - 0..3 |  | - Per configured grant | - UE specific |

Proposal 3: Adopt the following to TP#2 in TS38.214 to specify RB set allocation in frequency domain for interlace RB-based PSCCH/PSSCH transmission.

| Reason for change: | In subclasue 8.1.2.2 of TS 38.213, there is a general description for specifying sub-channel allocation in frequency domain. However, for interlace RB-based transmission, the decription for specifying RB set allocation is missed. |
| --- | --- |
| Summary of change: | Add a description to clarify RB set assignment for interlace RB-based transmission. |
| Consequences if not approved: | The description to clarify RB set assignment for interlace RB-based transmisison is missed in 8.1.2.2 regarding resoruce allocation in frequency domain. |

==== TP#2 (TS38.214 V18.0.0 (2023-09)) === #### 8.1.2.2 Resource allocation in frequency domain The resource allocation unit in the frequency domain is the sub-channel. The sub-channel assignment for sidelink transmission is determined using the "Frequency resource assignment" field in the associated SCI. The RB set assignment for sidelink transmission is determined using the "Frequency resource assignment" field in the associated SCI if the higher layer parameter transmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured to ‘interlaceRB’. The lowest sub-channel for sidelink transmission is the sub-channel on which the lowest PRB of the associated PSCCH is transmitted. If a PSSCH scheduled by a PSCCH would overlap with resources containing the PSCCH, the resources corresponding to a union of the PSCCH that scheduled the PSSCH and associated PSCCH DM-RS are not available for the PSSCH. When PSSCH is transmitted on multiple RB sets, the corresponding PSCCH is located on the sub-channel with smallest index of the lowest RB set.

Proposal 4: Adopt the following TP#3 to implement the candidate resources exclusion due to C-LBT failure in Step 1 in section 8.1.4 of TS 38.214.

- Note: the candidate resources exclusion by PHY due to C-BLT failure was agreed in RAN2 and send in the RAN2 LS [2].

| Reason for change: | LS R1-2306371 from RAN2 ask RAN1 to take two agreements into account in RAN1 work. The first one of the two agreements has been implemented in RAN1 specification while the second one has not been implemented in RAN1 specification yet. RAN2 agreements in LS R1-2306371 - MAC informs PHY of the RB set information where SL C-LBT failure was detected. - During resource (re)selection, PHY excludes the resources for the RB set where C-LBT failure was detected. |
| --- | --- |
| Summary of change: | Add descriptions in step 1 of resource selection procedure to specify that PHY excludes candidate resources that include at least one RB set in which SL C-LBT failure was detected. |
| Consequences if not approved: | Candidate resource exclusion due to SL C-LBT failure in PHY is not supported. |

==== TP#3 (TS38.214 V18.0.0 (2023-09)) === - 8.1.4 UE procedure for determining the subset of resources to be reported to higher layers in PSSCH resource selection in sidelink resource allocation mode 2 < Unchanged parts are omitted > If the higher layer parameter transmissionStructureForPSCCHandPSSCH is set to ‘contiguousRB', the UE shall exclude candidate single-slot or candidate multi-slot resources with the lowest sub-channel including resource blocks of the intra-cell guardband PRBs, configured by higher layer parameter, intraCellGuardBandsSL-List. If the higher layer parameter transmissionStructureForPSCCHandPSSCH is provided, the UE shall exclude candidate single-slot or candidate multi-slot resources including at least one RB set indicated by the higher layer parameter rbSetsWithConsecutiveLBTFailure. The total number of remaining candidate single-slot resources or candidate multi-slot resources is denoted by Mtotal. < Unchanged parts are omitted >

Proposal 5: Regarding “one PSCCH/PSSCH transmission has N associated candidate PSFCH occasion(s)”, use 1 bitmap to indicate resource for N candidate PSFCH occasion(s).

Proposal 6: Adopt the following TP#4 in TS38.213 to specify dedicated PRB allocation to a candidate PSFCH transmission occasion in an RB set when sl-PSFCH-Type = ‘type2’.

| Reason for change: | - The sl-PSFCH-RB-Set provides a set of MPRB, setPSFCH PRBs for PSFCH transmission in multiple RB sets in the recource pool. However, current specification does not specify in details how to divide the set of MPRB, setPSFCH PRBs into each RB set when sl-PSFCH-Type = ‘type2’. - It was also agreed to divide the configured PRBs for PSFCH transmission in an RB set into N PSFCH occasions. However, how to divide the configured PRBs for PSFCH transmission in an RB set into N PSFCH occasions has not been specified in the specification when sl-PSFCH-Type = ‘type2’. |
| --- | --- |
| Summary of change: | - Add descriptions to clarify that PRBs for PSFCH transmisison for an RB set are those PRBs from the set of MPRB, setPSFCH PRBs which locate within the RB set. - Add descriptions to clarify how to divide the configured PRBs for PSFCH transmission in an RB set into each of N PSFCH occasions. |
| Consequences if not approved: | - It is not clear which PRBs from the set of MPRB, setPSFCH PRBs are used for an RB set for PSFCH transmission when sl-PSFCH-Type = ‘type2’. - It is not clear how to divide the configured PRBs for PSFCH transmission in an RB set into each of N PSFCH occasions when sl-PSFCH-Type = ‘type2’. |

==== TP#4 (TS38.213 V18.0.0 (2023-09)) === - 16.3.0 UE procedure for transmitting PSFCH with control information < Unchanged parts are omitted > For operation with shared spectrum channel access, when sl-PSFCH-Type = ‘type2’ and within RB-set k, a UE determines a subset of PRBs in a first interlace and, based on sl-PSFCH-RB-Set, a subset of NPRBPSFCH PRBs in a second interlace for a PSFCH transmission with HARQ-ACK information in a resource pool. An index of the first interlace is provided by sl-PSFCH-Type2-CommonInterlace. The UE is provided by sl-PSFCH-RB-Set, a set of MPRB,setPSFCH PRBs in a resource pool for PSFCH transmission with HARQ-ACK information. The UE determines a set of MPRB,set,kPSFCH PRBs from the MPRB,setPSFCH PRBs where the MPRB,set,kPSFCH PRBs are within RB-set k for PSFCH transmission with HARQ-ACK information. For a n-th candidate PSFCH transmission occasion within RB-set k, where 1≤n≤NoccasionPSFCH, the UE allocates n-1⋅MPRB, set,kPSFCH/NoccasionPSFCH,n⋅MPRB, set,kPSFCH/NoccasionPSFCH-1 PRBs from MPRB,set,kPSFCH PRBs to the n-th candidate PSFCH transmission occasion. The NPRBPSFCH PRBs in the second interlace are provided by sl-PSFCH-Type2-DedicatedPRB where, for the n-th candidate PSFCH transmission occasion, 1≤n≤NoccasionPSFCH, and for each interlace l, the UE determines MPRB,k, lPSFCH,n PRBs from the allocated n-1⋅MPRB, set,kPSFCH/NoccasionPSFCH,n⋅MPRB, set,kPSFCH/NoccasionPSFCH-1 PRBs based on sl-PSFCH-RB-Set. The UE expects that MPRB,k,lPSFCH,n is a multiple of NPRBPSFCH. For interlace l, the UE determines a PRB subset with index s to include PRBs NPRBPSFCH⋅s, NPRBPSFCH⋅s+1, …, NPRBPSFCH⋅s+1-1, 0≤s≤MPRB,k,lPSFCH,n/NPRBPSFCH-1. The UE determines the Msubset,kPSFCH,n PRB subsets by ordering the PRB subsets first in an ascending order of PRB subset index within an interlace and second in ascending order of interlace index. For a number of Nsubchk sub-channels in RB-set k and a number of slots for PSSCH transmissions that is not larger than NPSSCHPSFCH and is associated with a slot for PSFCH transmission, the UE allocates the i+j⋅NPSSCHPSFCH⋅Msubch, slot,kPSFCH,n, i+j⋅NPSSCHPSFCH⋅Msubch, slot,kPSFCH,n+1, …, i+1+j⋅NPSSCHPSFCH⋅Msubch, slot,kPSFCH,n-1 PRB subsets from the Msubset,kPSFCH,n PRB subsets to slot i among the slots for PSSCH transmissions that are associated with the slot and sub-channel j for PSFCH transmissions, where Msubch, slot,kPSFCH,n=Msubset,kPSFCH,nNsubchk⋅NPSSCHPSFCH and 0≤i<NPSSCHPSFCH, 0≤j<Nsubchk. The allocation starts in an ascending order of i and continues in an ascending order of j. The UE expects that Msubset,kPSFCH,n is a multiple of Nsubchk∙NPSSCHPSFCH. < Unchanged parts are omitted >

Proposal 7: Adopt the following TP#5 in TS38.213 to specify dedicated interlace allocation to a candidate PSFCH transmission occasion in an RB set when sl-PSFCH-Type = ‘type1’.

| Reason for change: | - The sl-PSFCH-RB-Set provides a set of MPRB, setPSFCH PRBs for PSFCH transmission in multiple RB sets in the recource pool. However, current specification does not specify in details how to divide the set of MPRB, setPSFCH PRBs into each RB set when sl-PSFCH-Type = ‘type1’. - When sl-PSFCH-Type = ‘type1’, a PSFCH transmisisn occupies all PRBs in the interlace in an RB set. How the UE determines interlaces for PSFCH transmission from (pre-)configured PRBs for an RB set and how the UE divides the interlaces in the RB set into each of N PSFCH occasions is not specified as well. |
| --- | --- |
| Summary of change: | - Add descriptions to clarify that PRBs for PSFCH transmisison for an RB set are those PRBs from the set of MPRB, setPSFCH PRBs which locate within the RB set. - Add descriptions to clarify how to derive interlaces for PSFCH transmission for an RB set and how to divide the derived interlaces in the RB set into each of N PSFCH occasions. |
| Consequences if not approved: | - It is not clear which PRBs from the set of MPRB, setPSFCH PRBs are used for an RB set for PSFCH transmission when sl-PSFCH-Type = ‘type1’. - It is not clear how to derive interlaces for PSFCH transmisison from (pre-)configured PRBs for an RB set and how to divide the derived interlaces in the RB set into each of N PSFCH occasions when sl-PSFCH-Type = ‘type1’. |

==== TP#5 (TS38.213 V18.0.0 (2023-09)) === - 16.3.0 UE procedure for transmitting PSFCH with control information < Unchanged parts are omitted > For operation with shared spectrum channel access, when sl-PSFCH-Type = ‘type1’ and within RB-set k, a UE determines, based on sl-PSFCH-RB-Set, all PRBs of an interlace for one PSFCH transmission with HARQ-ACK information in the resource pool. The UE is provided by sl-PSFCH-RB-Set, a set of MPRB,setPSFCH PRBs in a resource pool for PSFCH transmission with HARQ-ACK information. The UE determines from the MPRB,setPSFCH PRBs a set of MPRB,set,kPSFCH PRBs within RB-set k for PSFCH transmission with HARQ-ACK information. The UE determines a set of interlaces Minterlace,kPSFCH within RB-set k from the MPRB,set,kPSFCH PRBs. For a n-th candidate PSFCH transmission occasion within RB-set k, 1≤n≤NoccasionPSFCH, the UE allocates n-1⋅Minterlace,kPSFCH,n,n⋅Minterlace,kPSFCH,n-1 interlaces from the Minterlace,kPSFCH interlaces to the n-th candidate PSFCH transmission occasion, where Minterlace,kPSFCH,n=Minterlace,kPSFCH/NoccasionPSFCH. For the n-th candidate PSFCH transmission occasion, 1≤n≤NoccasionPSFCH, the UE determines a set of interlaces that includes a number Minterlace,kPSFCH,n of interlaces based on sl-PSFCH-RB-Set. The set of Minterlace,kPSFCH,n interlaces are indexed in an ascending order of interlace indexes. For each interlace of the set of interlaces, all PRBs in the interlace are available for PSFCH transmission. For a number of Nsubchk sub-channels in RB-set k and a number of PSSCH slots that is not larger than NPSSCHPSFCH and is associated with a slot for PSFCH transmission, the UE allocates the i+j⋅NPSSCHPSFCH⋅Msubch, slot,kPSFCH,n, i+1+j⋅NPSSCHPSFCH⋅Msubch, slot,kPSFCH,n-1 interlaces from the Minterlace,kPSFCH,n interlaces to slot i and sub-channel j, where Msubch, slot,kPSFCH,n=Minterlace,kPSFCH,nNsubchk⋅NPSSCHPSFCH, 0≤i<NPSSCHPSFCH, 0≤j<Nsubchk. The allocation starts in an ascending order of i and continues in an ascending order of j. The UE expects that Minterlace,kPSFCH,n is a multiple of Nsubchk∙NPSSCHPSFCH. < Unchanged parts are omitted >

Proposal 8: Adopt the following TP#6 in TS 38.213 to specify that legacy PRB-level PSFCH transmission is applied to operation without shared spectrum channel access where OCB requirement is not required.

| Reason for change: | Current specification specifies that interlace RB based PSFCH transmission (i.e., sl-PSFCH-Type = ‘type1’ or sl-PSFCH-Type = ‘type2’) is applied to operation with shared spectrum channel access while legacy PRB-level PSFCH transmission is applied to operation without shared spectrum channel access. However, for operation with shared spectrum channel access where OCB requirement is not required, it is not necessary to mandate UE to perform interlace based transmission for PSFCH. Legacy PRB-level PSFCH transmission can be applied to this case. |
| --- | --- |
| Summary of change: | Clarify that interlace RB based PSFCH transmission is applied for operation with shared spectrum channel where interalce RB-based transmission is configured and legacy PRB-level PSFCH transmisison is applied for Rel-16/17 SL and operation with shared spectrum channel where contigous RB-based transmisison is configured. |
| Consequences if not approved: | UE is mandated to perform interlace RB-based PSFCH transmission even if OCB requiremetn is not required. |

==== TP#6 (TS38.213 V18.0.0 (2023-09)) === - 16.3.0 UE procedure for transmitting PSFCH with control information < Unchanged parts are omitted > For operation without shared spectrum channel access or operation with shared spectrum channel access where the higher layer parameter transmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured to ‘contigousRB’, a UE is provided by sl-PSFCH-RB-Set a set of MPRB, setPSFCH PRBs in a resource pool for PSFCH transmission with HARQ-ACK information in a PRB of the resource pool. A UE can be provided by sl-RB-SetPSFCH a set of MPRB, setPSFCH PRBs in a resource pool for PSFCH transmission with conflict information in a PRB of the resource pool. A UE expects that different PRBs are (pre)configured for conflict information and HARQ-ACK information. For a number of Nsubch sub-channels for the resource pool, provided by sl-NumSubchannel, and a number of PSSCH slots associated with a PSFCH slot that is less than or equal to NPSSCHPSFCH, the UE allocates the i+j⋅NPSSCHPSFCH⋅Msubch, slotPSFCH, i+1+j⋅NPSSCHPSFCH⋅Msubch, slotPSFCH-1 PRBs from the MPRB, setPSFCH PRBs to slot i among the PSSCH slots associated with the PSFCH slot and sub-channel j, where Msubch, slotPSFCH=MPRB, setPSFCHNsubch⋅NPSSCHPSFCH, 0≤i<NPSSCHPSFCH, 0≤j<Nsubch, and the allocation starts in an ascending order of i and continues in an ascending order of j. The UE expects that MPRB, setPSFCH is a multiple of Nsubch∙NPSSCHPSFCH. For operation with shared spectrum channel access where the higher layer parameter transmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured to ‘interlaceRB’, when sl-PSFCH-Type = ‘type1’ and within RB-set k, a UE determines, based on sl-PSFCH-RB-Set, all PRBs of an interlace for one PSFCH transmission with HARQ-ACK information in the resource pool. For the n-th candidate PSFCH transmission occasion, 1≤n≤NoccasionPSFCH, the UE determines a set of interlaces that includes a number Minterlace,kPSFCH,n of interlaces based on sl-PSFCH-RB-Set. The set of interlaces are indexed in an ascending order of interlace indexes. For each interlace of the set of interlaces, all PRBs in the interlace are available for PSFCH transmission. For a number of Nsubchk sub-channels in RB-set k and a number of PSSCH slots that is not larger than NPSSCHPSFCH and is associated with a slot for PSFCH transmission, the UE allocates the i+j⋅NPSSCHPSFCH⋅Msubch, slot,kPSFCH,n, i+1+j⋅NPSSCHPSFCH⋅Msubch, slot,kPSFCH,n-1 interlaces from the Minterlace,kPSFCH,n interlaces to slot i and sub-channel j, where Msubch, slot,kPSFCH,n=Minterlace,kPSFCH,nNsubchk⋅NPSSCHPSFCH, 0≤i<NPSSCHPSFCH, 0≤j<Nsubchk. The allocation starts in an ascending order of i and continues in an ascending order of j. The UE expects that Minterlace,kPSFCH,n is a multiple of Nsubchk∙NPSSCHPSFCH. For operation with shared spectrum channel access where the higher layer parameter transmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured to ‘interlaceRB’, when sl-PSFCH-Type = ‘type2’ and within RB-set k, a UE determines a subset of PRBs in a first interlace and, based on sl-PSFCH-RB-Set, a subset of NPRBPSFCH PRBs in a second interlace for a PSFCH transmission with HARQ-ACK information in a resource pool. An index of the first interlace is provided by sl-PSFCH-Type2-CommonInterlace. The NPRBPSFCH PRBs in the second interlace are provided by sl-PSFCH-Type2-DedicatedPRB where, for the n-th candidate PSFCH transmission occasion, 1≤n≤NoccasionPSFCH, and for each interlace l, the UE determines MPRB,k, lPSFCH,n PRBs based on sl-PSFCH-RB-Set. The UE expects that MPRB,k,lPSFCH,n is a multiple of NPRBPSFCH. For interlace l, the UE determines a PRB subset with index s to include PRBs NPRBPSFCH⋅s, NPRBPSFCH⋅s+1, …, NPRBPSFCH⋅s+1-1, 0≤s≤MPRB,k,lPSFCH,n/NPRBPSFCH-1. The UE determines the Msubset,kPSFCH,n PRB subsets by ordering the PRB subsets first in an ascending order of PRB subset index within an interlace and second in ascending order of interlace index. For a number of Nsubchk sub-channels in RB-set k and a number of slots for PSSCH transmissions that is not larger than NPSSCHPSFCH and is associated with a slot for PSFCH transmission, the UE allocates the i+j⋅NPSSCHPSFCH⋅Msubch, slot,kPSFCH,n, i+j⋅NPSSCHPSFCH⋅Msubch, slot,kPSFCH,n+1, …, i+1+j⋅NPSSCHPSFCH⋅Msubch, slot,kPSFCH,n-1 PRB subsets from the Msubset,kPSFCH,n PRB subsets to slot i among the slots for PSSCH transmissions that are associated with the slot and sub-channel j for PSFCH transmissions, where Msubch, slot,kPSFCH,n=Msubset,kPSFCH,nNsubchk⋅NPSSCHPSFCH and 0≤i<NPSSCHPSFCH, 0≤j<Nsubchk. The allocation starts in an ascending order of i and continues in an ascending order of j. The UE expects that Msubset,kPSFCH,n is a multiple of Nsubchk∙NPSSCHPSFCH. < Unchanged parts are omitted >

Observation 1: Option 3 (i.e., no optimization on PSFCH occasions within a COT to avoid COT interruption) makes a COT-initiating UE easily lose its COT especially in the case of short PSFCH periodicity, which severely affects functionalities of MCSt and UE-to-UE COT sharing.

Observation 2: On one hand, Option 1 (i.e., transmits PSSCH on such PSFCH occasion) helps to lower code rate of PSSCH transmission. On the other hand, there is the usability problem that option 1 can only apply in those PSFCH occasions where UE shall ensure no PSFCH transmission from other UEs, as well as a new notification mechanism required to notify RX UE of whether PSFCH occasion is used for PSSCH transmission.

Observation 3: Option 2 (i.e., transmits PSFCH-like signal on such PSFCH occasion) does not impact other UEs’ PSFCH transmissions.

Proposal 9: When neither COT initiating UE nor responding UE intends to transmit PSFCH on some PSFCH occasions(s) within a COT, to avoid COT interruption, option 2 is supported:

- Option 2: COT initiating UE or responding UE transmits a PSFCH-like signal on such PSFCH occasion(s)

Proposal 10: For the case that Alt 1-1b is (pre-)configured for PSFCH transmission in a resource pool, PSFCH-like signal can be transmitted on 1 common interlace.

Proposal 11: Support Rel-17 IUC feature for Rel-18 SL UE in SL-U and support the PSFCH transmission with conflict information in SL-U.

Proposal 12: Adopt the following TP#7 in TS38.215 to specify that SL-RSSI measurement is performed on OFDM symbols corresponding to PSCCH/PSSCH transmissions using the 2nd candidate starting symbol in a resource pool (pre-)configured with two candidate starting symbols.

| Reason for change: | In legacy SL, SL-RSSI is measured on OFSM symbols configured for PSCCH/PSSCH in time domain. In Rel-18, two candidate starting symbols for PSCCH/PSSCH were introduced. Therefore, actual SL transmission may start from 2nd candidate starting symbol so that there may be no corresponding SL transmission prior to the 2nd candidate starting symbol. If following the same SL-RSSI definition for SL-RSSI measurement, the total received power may be underestimated. |
| --- | --- |
| Summary of change: | Clarify that SL-RSSI is measured starting from the next symbol after the second candidate starting symbol when the SL BWP is configured with two candidate starting symbols. |
| Consequences if not approved: | The measured SL-RSSI may be underestimated. |

==== TP#7 (TS38.215 V18.0.0 (2023-09)) === 5.1.25 Sidelink received signal strength indicator (SL RSSI) Definition; Sidelink Received Signal Strength Indicator (SL RSSI) is defined as the linear average of the total received power (in [W]) observed in the configured sub-channel in OFDM symbols of a slot configured for PSCCH and PSSCH, starting from the 2nd OFDM symbol or starting from the next symbol after the second candidate symbol when startingSymbolFirst and startingSymbolSecond are provided for a SL-BWP. For frequency range 1, the reference point for the SL RSSI shall be the antenna connector of the UE. For frequency range 2, SL RSSI shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported SL RSSI value shall not be lower than the corresponding SL RSSI of any of the individual receiver branches.. Applicable for; RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency.

Proposal 13: In a resource pool (pre-) configured with two candidate starting symbols, the CSI-RS symbol is separately (pre-) configured for the 1st candidate starting symbol and the 2nd candidate starting symbol.

References

RP-221938 “WID revision: NR sidelink evolution”, RAN#97e, OPPO.

R1-2306371 “LS on C-LBT failure Recovery”, RAN1#114, TSG RAN WG2.