3GPP TSG-RAN WG1 Meeting #114-bis	R1-2310263

Xiamen, China, October 9 – 13, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| [DRAFT] CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | - | rev | 0 | Current version: | 16.11.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on the rate matching for HARQ-ACK and CG-UCI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | HiSilicon, Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_unlic-Core |  |  |  |  |  | Date: |  |  | 2023-09-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change |  | In RAN1#99, RAN1 agreed that the CG-UCI and HARQ-ACK are jointly encoded (CG-UCI is treated as the same type as a HARQ-ACK) in the case of PUCCH overlapping with CG-PUSCH(s) within a PUCCH group when multiplex of CG-UCI and HARQ-ACK is configured. However, in TS 38.212, the condition to determine the LACK in rate matching parameter QACK' for HARQ-ACK and CG-UCI (i.e.OACK+OCG-UCI>360 in 6.3.2.4.1.5) is different from that for HARQ-ACK only (i.e. of OACK≥360 in 6.3.2.4.1.1). The different conditions do not align with RAN1 agreement and increase UE complexity. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify OACK+OCG-UCI>360 to OACK+OCG-UCI≥360. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current specification do not align with RAN1 agreement and increase unnecessary UE complexity. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.2.4.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

###### 6.3.2.4.1.5	HARQ-ACK and CG-UCI

For HARQ-ACK and CG-UCI transmission on PUSCH with UL-SCH and if numberOfSlotsTBoMS is not present in the resource allocation table, or if numberOfSlotsTBoMS is present in the resource allocation table and the value of numberOfSlotsTBoMS in the row indicated by the Time domain resource assignment field in DCI is equal to 1, the number of coded modulation symbols per layer for HARQ-ACK and CG-UCI transmission, denoted as QACK', is determined as follows:

QACK'=minOACK+OCG-UCI+LACK∙βoffsetPUSCH∙l=0Nsymb,allPUSCH-1MscUCIlr=0CUL-SCH-1Kr,α∙l=l0Nsymb,allPUSCH-1MscUCIl

where

-	OACK is the number of HARQ-ACK bits;

-	OCG-UCI is the number of CG-UCI bits;

-	if  OACK+OCG-UCI≥>360, LACK=11; otherwise LACK is the number of CRC bits for HARQ-ACK and CG-UCI determined according to Clause 6.3.1.2.1;

-	βoffsetPUSCH=βoffsetHARQ-ACK;

-	CUL-SCH is the number of code blocks for UL-SCH of the PUSCH transmission;

-	Kr is the r-th code block size for UL-SCH of the PUSCH transmission;

-	MscPUSCH is the scheduled bandwidth of the PUSCH transmission, expressed as a number of subcarriers;

-	MscPT-RSl is the number of subcarriers in OFDM symbol l that carries PTRS, in the PUSCH transmission;

/********** Unchanged parts are omitted *********/