3GPP TSG RAN WG1#114bis 			R1-2310285

Xiamen, China, October 9 - 13, 2023

Agenda Item:	8.3.2

Source:	Moderator (CATT)

Title:		FL Summary #2 for NR DL and UL carrier phase positioning

Document for:	Discussion and Decision

# Introduction

RAN1 has been working on Rel-18 new work item on expanded and improved NR positioning [1] since RAN1#112 ([2][3][4][5]). The WI was completed from RAN1’s perspective. The purpose of this document is to provide a summary of the discussion of the maintenance issues related to the following objective, based on the submitted contributions ([6-25]).

- Specify physical layer measurements and signalling to support NR DL and UL carrier phase positioning for UE-based, UE-assisted, and NG-RAN node assisted positioning [RAN1, RAN2, RAN3, RAN4].

  - Existing DL PRS and UL SRS for positioning are used for NR carrier phase measurements.

  - Specify measurements that are limited to a single carrier/PFL.

  - Specify corresponding new core requirements, as well as identifying and specifying the impact on the existing RAN4 specification, including RRM measurements without measurement gaps in connected and inactive mode (including PRS measurement period/reporting) and procedures [RAN4].

The discussion will focus on the following aspects:

- Definition of NR carrier phase measurements

- NR carrier phase measurements and reporting

- Differential CPP and PRU

- Integer ambiguity

- Phase error group

- Carrier phase measurement quality

- Multipath mitigation

- Others

Please note that in this FL summary, a FL proposal may be designated as (H)(M)(L) to indicate its high, medium or low priority for online or offline discussions in this meeting. Nevertheless, we encourage all interested companies to provide feedback on all FL proposals. The FL may revise the priority of the proposals based on inputs from interested companies during the meeting, if it is deemed necessary.

# Definitions of NR carrier phase measurements

Agreement (RAN1#112bis-e) Introduce DL reference carrier phase (DL RSCP) and NR DL reference carrier phase difference (DL RSCPD) as DL carrier phase measurements. - Note: It is up to RAN4 to decide whether and how to define the requirements for DL RSCP and/or DL RSCPD. No LS needed to RAN4 for this note. - DL RSCP can be reported together with UE Rx – Tx time difference measurement - DL RSCPD can be reported together with RSTD measurement - FFS: details on how to eliminate unknown initial Rx phase with RSCP/RSCPD reporting can be further discussed Note: Whether to support standalone DL RSCP and/or DL RSCPD reporting, or DL RSCP/DL RSCPD reporting with other new types of measurements (if agreed), can be further discussed. Agreement The specific RF frequency associated with a DL carrier phase measurement is defined as the center frequency of the DL PFL by default. - Note: It is open to further discussion whether a frequency other than the center frequency of the DL PFL can also be the specific RF frequency for non-default case(s), if RAN1 agrees to introduce them. Agreement The specific RF frequency associated with a UL carrier phase measurement is defined, by default, as the center frequency of the transmission bandwidth of the SRS for positioning purpose. - Note: It is open to further discussion whether a frequency other than the center frequency of the UL carrier can also be the specific RF frequency for a non-default case(s), if RAN1 agrees to introduce them.

### 5.1.42 DL reference signal carrier phase (DL RSCP) Definition; DL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements carrying DL PRS configured for the measurement. DL RSCP is associated with the center frequency of the DL positioning frequency layer (PFL) configured for the measurement. For frequency range 1, the reference point for the DL RSCP shall be the antenna connector of the UE. For frequency range 2, the reference point for the DL RSCP shall be the antenna of the UE.. Applicable for; RRC_CONNECTED, RRC_INACTIVE, RRC_IDLE. ### 5.1.43 DL reference signal carrier phase difference (DL RSCPD) Definition; DL reference signal carrier phase difference (RSCPD) is defined as the difference of DL RSCPs measured from DL PRS transmitted in a DL PFL from the transmission point (TP) j and the reference TP i. If UE reports RSCPD measurements together with RSTD measurements in a measurement report element, the reference TP for RSCPD is the same as the reference TP reported for RSTD. For frequency range 1, the reference point for the DL RSCPD shall be the antenna connector of the UE. For frequency range 2, the reference point for the DL RSCPD shall be the antenna of the UE.. Applicable for; RRC_CONNECTED, RRC_INACTIVE, RRC_IDLE. ### 5.2.8 UL reference signal carrier phase (UL RSCP) Definition; UL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements carrying sounding reference signals (SRS) configured for the measurement. UL RSCP is associated with the center frequency of the transmission bandwidth of the SRS for positioning purposes configured for the measurement. The reference point for UL RSCP shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Rx antenna (i.e., the centre location of the radiating region of the Rx antenna), - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector..

## Frequency information of NR carrier phase measurements

Submitted Proposals:

| Huawei, HiSilicon[6] | Proposal 1: For the center frequency associated with CPP measurement, it is determined as follows: - Use NRB to denote the number of RBs for PRS or SRS. - If NRB is even, the center frequency is RE 0 of RB NRB2 - If NRB is odd, the center frequency is RE 6 of RB NRB-12 - Proposal 2: Endorse the following TP to clauses 5.1.42 and 5.2.8 of TS 38.215 - <Unchanged parts> ### 5.1.42 DL reference signal carrier phase (DL RSCP) Definition; DL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements carrying DL PRS configured for the measurement. DL RSCP is associated with the center frequency of the DL positioning frequency layer (PFL) configured for the measurement. The center frequency is defined as - RE 0 of RB NRB/2 if NRB is even - RE 6 of RB (NRB-1)/2 if NRB is odd - where NRB is the number of RBs for DL PRS. For frequency range 1, the reference point for the DL RSCP shall be the antenna connector of the UE. For frequency range 2, the reference point for the DL RSCP shall be the antenna of the UE.. Applicable for; RRC_CONNECTED, RRC_INACTIVE, RRC_IDLE. ### 5.2.8 UL reference signal carrier phase (UL RSCP) Definition; UL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements carrying sounding reference signals (SRS) configured for the measurement. UL RSCP is associated with the center frequency of the transmission bandwidth of the SRS for positioning purposes configured for the measurement. The center frequency is defined as - RE 0 of RB NRB/2 if NRB is even - RE 6 of RB (NRB-1)/2 if NRB is odd - where NRB is the number of RBs for SRS. The reference point for UL RSCP shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Rx antenna (i.e., the centre location of the radiating region of the Rx antenna), - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector.. - <Unchanged parts> Proposal 3: For RedCap positioning with CPP, the PRS/SRS BW is associated with either a hop BW or the combined BW for all hops for the RSCP/RSCPD measurement report, following the RedCap ToA measurement bandwidth. |
| --- | --- |
| LGE[12] | Proposal 1: Support use of RF frequency of carrier phase measurement other than default accounting for UEs in RRC_INACTIVE/RRC_IDLE state. - LMF informs the candidate RF frequency list for DL RSCP/RSPD measurement per PFL, and UE selects a frequency among them. Proposal 2: Adopt TP for 38.215 for the definition of DL RSCP in the Annex section. =============================Unchanged part omitted================================== 5.1.42 DL reference signal carrier phase (DL RSCP) Definition; DL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements carrying DL PRS configured for the measurement. DL RSCP is associated with one of the candidate frequencies the center frequency of the DL positioning frequency layer (PFL) configured for the measurement. For PRU, DL RSCP is associated with multiple frequencies of the DL positioning frequency layers (PFL) configured for the measurement For frequency range 1, the reference point for the DL RSCP shall be the antenna connector of the UE. For frequency range 2, the reference point for the DL RSCP shall be the antenna of the UE.. Applicable for; RRC_CONNECTED, RRC_INACTIVE, RRC_IDLE. =========================Unchanged part omitted================================== |

FL Comments:

In previous meeting, it was agreed that by default the center frequency of the DL PFL is the frequency associated with the DL carrier phase measurements, and the center frequency of the UL SRS transmission bandwidth is the frequency associated with the UL carrier phase measurements.

In [6], it was proposed to further define how the center frequency is derived, which can be used to avoid potential confusion in determining the center frequency.

In [12]. the situation was considered that in RRC_INACTIVE/RRC_IDLE state a UE is only able to measure the DL PRS within the initial DL BWP. In this case, the center frequency of the measured DL PRS may not be the center frequency of the DL PFL. In  this scenario, the RF frequency of carrier phase measurement may be associated with other RF frequencies other than the default frequency, i.e., the center frequency of DL PFL. In FL’s view, we may want to avoid the cases that different UEs provide the carrier phase measurements associated with different carrier frequencies due to the measurement limitation within the initial DL BWP, which may make the implementation of the CPP much more difficult. One potential solution may be, as proposed in [12], is  to let LMF to inform the candidate RF frequency list for DL RSCP/RSPD measurement per PFL for the UE to select. Another simpler solution may be requiring the UE, which supports CPP in RRC_INACTIVE/RRC_IDLE state, must be able to measure the whole DL PFL, but not limited within the initial DL BWP.

### (Closed)(Round 1) Proposal 2.1-1

Adopt the following TP regarding the center frequency of the RSCP measurements in Clauses 5.1.42 and 5.2.8 of TS 38.215.

| Reason for change: | It was agreed that the specific RF frequency associated with a DL carrier phase measurement is defined as the center frequency of the DL PFL. However, how to determine the center frequency associated with carrier phase measurements is not defined. |
| --- | --- |
| Summary of change: | Define how to determine the center frequency associated with carrier phase measurements |
| Consequences if not approved: | It is unclear on how the center frequency associated with carrier phase measurements is determined in the specification. |

=========================Unchanged part omitted================================== 5.1.42 DL reference signal carrier phase (DL RSCP) Definition; DL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements carrying DL PRS configured for the measurement. DL RSCP is associated with the center frequency of the DL positioning frequency layer (PFL) configured for the measurement. The center frequency is defined as the absolute RF frequency corresponding to: - RE 0 of RB NRB/2 if NRB is even - RE 6 of RB (NRB-1)/2 if NRB is odd where NRB is the total number of RBs for DL PRS in the PFL. For frequency range 1, the reference point for the DL RSCP shall be the antenna connector of the UE. For frequency range 2, the reference point for the DL RSCP shall be the antenna of the UE.. Applicable for; RRC_CONNECTED, RRC_INACTIVE, RRC_IDLE. =========================Unchanged part omitted================================== 5.2.8 UL reference signal carrier phase (UL RSCP) Definition; UL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements carrying sounding reference signals (SRS) configured for the measurement. UL RSCP is associated with the center frequency of the transmission bandwidth of the SRS for positioning purposes configured for the measurement. The center frequency is defined as the absolute RF frequency corresponding to: - RE 0 of RB NRB/2 if NRB is even - RE 6 of RB (NRB-1)/2 if NRB is odd where NRB is the total number of RBs for the transmission bandwidth of the SRS. The reference point for UL RSCP shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Rx antenna (i.e., the centre location of the radiating region of the Rx antenna), - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector.. =========================Unchanged part omitted==================================

| Company | comments |
| --- | --- |
| vivo | In our view, the total number of RBs for the transmission bandwidth of SRS is an integer multiple of 4PRB, ‘the center frequency of the transmission bandwidth of the SRS’ is clear and the additional description seems to be not needed. |
| Xiaomi | Not needed. There is also ‘the center frequency’ in the definition of RSSI in TS 38.215, but the definition of it is not needed. |
| ZTE | Share similar view with vivo and Xiaomi, and we think that ‘center frequency’ is enough in current spec. |
| Qualcomm | Not needed |
| LGE | Similar view with other companies that current definition may be enough for determining the “center frequency” |
| CATT | Support |
| FL | The majority of the feedbacks received so far do not think it is necessary for the further clarification on how to determine the “center frequency”. Based on the feedback, the FL will lower the priority of the issue for this meeting from (H) to {L}. |
| Ericsson | Agree with other companies that the current definition in spec is enough. No need for these TPs. |
| Huawei, HiSilicon | Support. Otherwise, the center frequency may be interpreted as the somewhere between the two REs. We are fine to drop the odd number of the RBs, but in general, this definition is future-proof. |
| FL | From the feedback, most companies do not think the TP is needed. Thus, suggest closing the discussion for this meeting. |

### (Closed)(Round 1) Question 2.1-2

What is your view on the following options related to the RF frequency associated with the DL RSCP/RSCPD when UE is in RRC_INACTIVE/RRC_IDLE state?

- Option 1: A UE, which has the capability to support CPP in RRC_INACTIVE/RRC_IDLE state, may measure the DL PRS within its initial DL BWP only. The RF frequency associated with the DL RSCP/RSCPD when UE is in RRC_INACTIVE/RRC_IDLE state can be defined as the center frequency of the measured DL PRS bandwidth;

- Option 2: A UE, which has the capability to support CPP in RRC_INACTIVE/RRC_IDLE state, should measure the DL PRS from the whole DL PFL, i.e., not limited to its initial DL BWP. The RF frequency associated with the DL RSCP/RSCPD when UE is in RRC_INACTIVE/RRC_IDLE state can be defined in the same way as a UE in RRC_CONNECTED state.

| Company | comments |
| --- | --- |
| vivo | Option 2. |
| Xiaomi | In current spec: The UE in RRC_INACTIVE mode, subject to UE capability, is expected to process DL PRS outside or inside of the initial DL BWP. Thus, for CPP, it is straightforward to support measurement of DL PRS outside of initial DL BWP. So we prefer Option 2. |
| ZTE | Option 2. In current discussion, CPP related measurement is reported together with other legacy positioning measurement, i.e., RSTD and RTT. So from our perspective, it’s better to keep align with current measurements for DL PRS. |
| Qualcomm | Option 2 |
| LGE | Support option 1 with modification. It is our understanding that to enable UE who is in RRC_INACTIVE/RRC_IDLE state to measure outside of the initial DL BWP, measurement gap-like configuration is required, which could lead following discussion of configuration method of it, UE behavior within that duration, and so on. As described in our contribution, introducing RF frequency other than default for RSCP/RSCPD measurement will be the simplest solution for this. Either UE reporting measured RF frequency or LMF configuring the candidate frequencies for measurement are possible approach. Following modification can be considered; - Option 1a: A UE, which has the capability to support CPP in RRC_INACTIVE/RRC_IDLE state, may measure the DL PRS within its initial DL BWP only. The RF frequency associated with the DL RSCP/RSCPD when UE is in RRC_INACTIVE/RRC_IDLE state can be configured by LMF and/or reported by UE without configuration. defined as the center frequency of the measured DL PRS bandwidth; FL: For LMF to configure the RF frequency properly, the LMF needs to the information the configuration of UE initial DL BWP. The information is not available to LMF. We have concern in option 2. The option 2 causes undesired restriction on UE capability. Some UE who support RSCP/RSCPD measurement in RRC_INACTIVE/RRC_IDLE state may not desire to have not only complexity due to the frequent RF switching but also power consumption due to the larger BW size. It should be noted that one of the important property in RRC_INCATIVE/RRC_IDLE state is to reduce the UE power consumption by using a smaller BW size than RRC_CONNECTED state. FL: The concern from LGE is valid in FL’s view. However, Option 2 may be the best option for Rl8. The associated drawbacks may be resolved in a future release |
| CATT | Option 2 |
| FL | Based on the feedback, it has the clear majority of the support for Option 2. |
| Ericsson | Option 2 |
| Huawei, HiSilicon | Option 2 |
| Nokia/NSB | Option 2 |
| FL | Agreement A UE, which has the capability to support CPP in RRC_INACTIVE/RRC_IDLE state, should measure the DL PRS from the whole DL PFL, i.e., not limited to its initial DL BWP. The RF frequency associated with the DL RSCP/RSCPD when UE is in RRC_INACTIVE/RRC_IDLE state can be defined in the same way as a UE in RRC_CONNECTED state. |

## Timestamp of UL carrier phase measurements

Background

Agreement For the timestamp associated with a reported RSCP/RSCPD measurement, NR-TimeStamp, with the granularity of a slot, currently defined in TS 37.355, can be reused as the timestamp. - Subject to UE capability, a UE may optionally provide an OFDM symbol index in the timestamp. - Note: It is up to RAN2/RAN3 how to signal the timestamp

Submitted Proposals:

| CATT[15] | Proposal 3: For the timestamp associated with a reported UL RTOA measurement, NR-TimeStamp, with the granularity of a slot, currently defined in TS 38.455, can be reused as the timestamp for UL RSCP. - TRP may optionally provide an OFDM symbol index in the timestamp. - Note: It is up to RAN2/RAN3 how to signal the timestamp. |
| --- | --- |

FL Comments:

It was agreed in RAN1#114 the timestamp associated with a reported RSCP/RSCPD measurement, NR-TimeStamp currently defined in TS 37.355, can be reused as the timestamp. The agreement was for DL RSCP/RSCPD measurement. For UL RSCP, it was proposed in [15] that the NR-TimeStamp currently defined in TS 38.455, should be reused as the timestamp. In FL’s view, it might be helpful, for RAN1 to have an mirror agreement for UL CPP.

### (Closed)(Round 1) Proposal 2.2-1

For the timestamp associated with a reported UL RSCP measurement, NR-TimeStamp, with the granularity of a slot, currently defined in TS 38.455, can be reused as the timestamp.

- The TRP may optionally provide an OFDM symbol index in the timestamp.

- Note: It is up to RAN2/RAN3 how to signal the timestamp

| Company | comments |
| --- | --- |
| Xiaomi | Ok |
| Spreadtrum | Support |
| Samsung1 | OK |
| Qualcomm | OK |
| LGE | OK |
| CATT | Support |
| Ericsson | Ok |
| Huawei, HiSilicon | Support |
| Nokia/NSB | OK |
| FL | Agreement For the timestamp associated with a reported UL RSCP measurement, NR-TimeStamp, with the granularity of a slot, currently defined in TS 38.455, can be reused as the timestamp. - The TRP may optionally provide an OFDM symbol index in the timestamp. - Note: It is up to RAN2/RAN3 how to signal the timestamp |

## Definition of RSCPD

Submitted Proposals:

| Intel[10] | Proposal 1 - For DL RSCPD, RSCP j is measured using DL PRS received in a subframe SF_j from target TP j and RSCP i is measured using DL PRS received in a subframe SF_i from reference TP i that is closest in time to subframe SF_j. - LMF may optionally request a target UE to perform measurements that are restricted to within a time window. |
| --- | --- |

FL Comments:

Regarding NR DL RSCPD measurement, it was agreed that “RSCPD is defined as the difference of RSCPs measured from the DL PRS signals from target TRP and reference TRP.” The agreement does not define when the RSCPs are measured from the DL PRS signals from target TRP and reference TRP. There has been discussion on whether to add the condition on when the RSCPs are measured, including based on RSCPs obtained from the closed subframes (e.g., [10]), or within the indicated time domain window [12]. These proposals have been discussed in the previous meeting without conclusion. Based on the feedback of the previous discussion [26], there were different views on whether there is need for the modification of the definition. We may continue the discussion to see if we can reach a consensus in this meeting.

### (Closed) Proposal 2.3-1

The NR DL reference signal carrier phase difference (RSCPD) is the phase difference between the Transmission Point (TP) j and the reference TP i, defined as RSCP j – RSCP i, where (down-select one of the following options)

- Option 1: RSCP j is measured from the DL PRS signals received in a subframe SF_j from reference TP j and RSCP i is measured from the DL PRS signals received in a subframe SF i, which should be the same as the subframe SF_j ,or if not feasible, the closest to subframe SF_j, from target TP i.

- Option 2: RSCP j and RSCP i are measured from the DL PRS signals from reference TP j and target TP i within an indicated time window.

    - FFS: the details of the time window configuration

| Company | comments |
| --- | --- |
| vivo | We prefer not to modify the definition, but if the LMF wants UE to measure the RSCPD within an indicated time window, the LMF can inform UE the window. |
| Xiaomi | No further clarification at this time. |
| InterDigital | Option 1 but we are not sure if we need “which should be the same as the subframe SF_j ,or if not feasible, the closest to subframe SF_j, from target TP i.” .If RSTD is accompanying RSCPD, subframes the UE is measuring should be the same subframes as for RSTD? |
| ZTE | Option 1. Prefer to reuse the definition for RSTD. |
| Samsung1 | We prefer to have carrier phase continuity between the two measurements, as otherwise the carrier phase difference measurement is not useful. |
| Qualcomm | There is no need to change the definition due to the existience of the “time window” because then we ll have to change any of the other measurements (The window apply to all measurements RSTd, UE Rx-Tx, RSRP, RSPP, etc). RAN4 can change their measurement period specifications to ensure that the indicated time windows are taken into account in the requirements. |
| LGE | Option 2 should be supported. The unknown Rx initial phase error is something cannot be estimated or eliminated unless the differential between measurements is performed where same amount of error is ensured between them. Without guaranteeing error-free measurement also impact on integer ambiguity resolution which turns out to be comparably larger positioning error. If the phase continuity between RSCPs is not ensured to derive some of the RSCPDs and ensured for the others, it will just add the confusion to the LMF. Therefore only RSCPD measurements those are based on RSCPs with phase continuity should be reported. For option 1, it should be noted that the closest subframe from reference TP_i may not close enough to guarantee phase continuity condition. |
| CATT | Option 1. |
| FL | From the feedbacks, it seems it is still very challenging to get any of the options to be agreed. We may wait for more inputs to see whether we should continue the discussion of the proposal in this meeting. |
| Ericsson | no need to change the definition. |
| Huawei, HiSilicon | We do not support the change of the definition of RSCPD. |
| Nokia/NSB | We do not prefer to change the definition. |
| FL | Based on the feedback, most companies do not support either of the Options. Thus, suggest closing the discussion in this meeting. |

## Per Path RSCP/RSCPD definition

Background

Agreement (RAN1#112) NR UL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the UL SRS signal for positioning purpose configured for the measurement. A UL RSCP is associated with a specific RF frequency. - FFS: the reference point of the UL RSCP - FFS: whether/how the measurement timing is defined - Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion Note: The support of MIMO SRS for positioning is transparent to UE Agreement (RAN1#112) For NR DL reference signal carrier phase difference (RSCPD) measurement for NR CPP, the RSCPD is defined as the difference of RSCPs measured from the DL PRS signals from target TRP and reference TRP. - FFS: whether/how to define per path RSCPD - Note: Whether/how to capture the above definition into TS 38.215 depends on whether RAN1 decides to introduce DL carrier phase difference measurement for NR CPP

Submitted Proposals:

| OPPO[16] | Proposal 1: Define per path RSCPD and the UE reports the per path RSCPD of each additional path with reference to the first path in the additional path reporting of DL TDOA and multi-RTT. |
| --- | --- |
| CMCC[17] | Proposal 2: For NR carrier phase positioning, support only the carrier phase of the first path. Proposal 3: Support per path RSCP/RSCPD definition for the first path. |
| Qualcomm[24] | Proposal 1: Do not adopt RSCP/RSCPD definitions for the i-th path for i>1 unless clear benefits from this are demonstrated. |

FL Comments:

The issue on whether to support per path RSCPD definition has been discussed in previous meetings (Note: The per path definition was agreed for RSCP. However, it does not mean reporting RSCP of additional paths is supported in Rel-18, which is under a separate discussion in Section 8). For this meeting, many companies still provided different views as shown in [16][17][24].

Considering that a) RAN1 has spent significant effort on this issue; b) it is unlikely that we can reach a consensus to support it; and c) the WI was completed from RAN1’s perspective, the FL suggest making a conclusion that we will not further discuss the support of per path carrier phase measurements in Rel-18.

### (Closed) Proposal 2.4-1 (as a conclusion)

No further discussion in RAN1 regarding the definition of per path RSCPD in Rel-18.

- Note: This conclusion does not impact the existing definition of the RSCP.

| Company | comments |
| --- | --- |
| vivo | Support |
| Xiaomi | Ok with the conclusion |
| ZTE | Support. |
| Qualcomm | OK |
| LGE | Fine |
| CATT | Support |
| Ericsson | ok |
| Huawei, HiSilicon | Support |
| Nokia/NSB | OK |
| FL | Conclusion No further discussion in RAN1 regarding the definition of per path RSCPD in Rel-18. - Note: This conclusion does not impact the existing definition of the RSCP.. |

## Phase continuity

Submitted Proposals:

| Nokia, NSB[7] | Proposal 9: Support following to maintain phase continuity for RTT-type CP measurement: - The LMF requests the UE to provide RSCP measurement condition (e.g., coherence time of initial phase offset) for each gNB. - The LMF assists UE by providing with the RSCP measurement conditions for each TRP, in case the LMF is provided with the RF features (e.g., coherence time of the initial-phase offset supported by the specific RF-chain/antenna of the gNBs). |
| --- | --- |
| LGE[12] | Proposal 3: DL RSCPD should be derived based on two RSCPs within indicated time window where UE maintains phase continuity. Proposal 4: Adopt TP for 38.215 for the definition of DL RSCPD in the Annex section ========================Unchanged part omitted================================== 5.1.43 DL reference signal carrier phase difference (DL RSCPD) Definition; DL reference signal carrier phase difference (RSCPD) is defined as the difference of DL RSCPs measured from DL PRS transmitted in a DL PFL from the transmission point (TP) j and the reference TP i within the indicated time window. UE is expected to maintain phase continuity during the indicated time window. If UE reports RSCPD measurements together with RSTD measurements in a measurement report element, the reference TP for RSCPD is the same as the reference TP reported for RSTD. For frequency range 1, the reference point for the DL RSCPD shall be the antenna connector of the UE. For frequency range 2, the reference point for the DL RSCPD shall be the antenna of the UE.. Applicable for; RRC_CONNECTED, RRC_INACTIVE, RRC_IDLE. ========================Unchanged part omitted================================== |
| Samsung[13] | Proposal 6: For carrier phase positioning measurement, for RSCPD, a UE should maintain phase continuity between carrier-phase measurement of TRP1 and TRP2. Text proposal 7: (clause 5.1.43 of TS 38.215) DL reference signal carrier phase difference (RSCPD) is defined as the difference of DL RSCPs measured from DL PRS transmitted in a DL PFL from the transmission point (TP) j and the reference TP i. If UE reports RSCPD measurements together with RSTD measurements in a measurement report element, the reference TP for RSCPD is the same as the reference TP reported for RSTD. When the UE reports DL RSCPD measurement(s), the UE is expected to maintain phase continuity between the first carrier phase measurement and the second carrier phase measurement. For frequency range 1, the reference point for the DL RSCPD shall be the antenna connector of the UE. For frequency range 2, the reference point for the DL RSCPD shall be the antenna of the UE. Text proposal 8: The UE may be configured with [higher layer parameter] which contains DL carrier phase measurements performed by a positioning reference unit (PRU) [20, TS 38.305] along with the location information of the PRU. A positioning measurement report with DL RSCPD measurement or DL RSCP measurement includes a phase continuity indicator. The phase continuity indicator indicates whether or not phase continuity has been maintained since the last positioning measurement report with DL RSCPD measurement or DL RSCP measurement. |
| Apple[19] | Proposal 1: No action by RAN1 to eliminate unknown initial Rx phase with RSCP/RSCPD reporting |

FL Comments:

The issue of maintaining phase continuity during the measurement, as proposed in [7][12[13], has been discussed in previous meeting without conclusion. Considering only a few companies provided the feedback I previous discussion [26], the FL likes to further check with the interested companies to see if we should continue the discussion of the issue during the maintenance phase.

(Round 1) Question 2.5-1

What is your view on the following proposals from related to the phase continuity?

- P1 [7]: Support following to maintain phase continuity for RTT-type CP measurement:

  - The LMF requests the UE to provide RSCP measurement condition (e.g., coherence time of initial phase offset) for each gNB.

  - The LMF assists UE by providing with the RSCP measurement conditions for each TRP, in case the LMF is provided with the RF features (e.g., coherence time of the initial-phase offset supported by the specific RF-chain/antenna of the gNBs).

- P2[12]: DL RSCPD should be derived based on two RSCPs within indicated time window where UE maintains phase continuity.

- P2 [13]: For carrier phase positioning measurement, for RSCPD, a UE should maintain phase continuity between carrier-phase measurement of TRP1 and TRP2.

| Company | comments |
| --- | --- |
| vivo | We prefer not to further discuss the issue. |
| ZTE | Share similar view with vivo. For P1, how does the UE get the initial phase offset for each gNB. |
| Spreadtrum | Agree with vivo. |
| Samsung1 | We thing the following aspects are important: - For carrier phase difference, the carrier phase continuity should be maintained between the two carrier phase measurements involved in the measurement. - An indication of carrier phase continuity for consecutive measurements. |
| LGE | As commented in proposal 2.3-1, at least derived DL RSCPD should be based on two RSCPs within indicated time window where UE maintains phase continuity. And we believe what Samsung proposing is effectively same with our proposal. |
| CATT | In our view, it should be up to UE implementation on provide the RSCPD measurements properly to pass RAN4’s requirements.. |
| FL | From the feedback, it seems very unlikely for RAN1 to reach on how a UE implement the phase continuity in order to” eliminate unknown initial Rx phase with RSCP/RSCPD reporting”. Agreement (RAN1#112bis-e) Introduce DL reference carrier phase (DL RSCP) and NR DL reference carrier phase difference (DL RSCPD) as DL carrier phase measurements. - Note: It is up to RAN4 to decide whether and how to define the requirements for DL RSCP and/or DL RSCPD. No LS needed to RAN4 for this note. - DL RSCP can be reported together with UE Rx – Tx time difference measurement - DL RSCPD can be reported together with RSTD measurement - FFS: details on how to eliminate unknown initial Rx phase with RSCP/RSCPD reporting can be further discussed Note: Whether to support standalone DL RSCP and/or DL RSCPD reporting, or DL RSCP/DL RSCPD reporting with other new types of measurements (if agreed), can be further discussed.. Thus, to close the “FFS” in the above agreement, the FL suggest making the following conclusion: - No further discussion in RAN1 on the details on how to “eliminate unknown initial Rx phase with RSCP/RSCPD reporting” in Rel-18. |
| Huawei, HiSilicon | This belongs to RAN4 discussion. |

### (Round 2) Proposal 2.5-1

No further discussion in RAN1 how to maintain phase continuity in order to eliminate unknown initial Rx phase with RSCP/RSCPD reporting in Rel-18.

| Company | comments |
| --- | --- |
| ZTE | Support. |
| Samsung2 | We still think that the carrier phase continuity between carrier phase measurements for RSCPD is important to consider. |

# Carrier phase measurement reporting

## Number of CP measurements associated with one legacy timing measurements

Background

Agreement Introduce DL reference carrier phase (DL RSCP) and NR DL reference carrier phase difference (DL RSCPD) as DL carrier phase measurements. - Note: It is up to RAN4 to decide whether and how to define the requirements for DL RSCP and/or DL RSCPD. No LS needed to RAN4 for this note. - DL RSCP can be reported together with UE Rx – Tx time difference measurement - DL RSCPD can be reported together with RSTD measurement - FFS: details on how to eliminate unknown initial Rx phase with RSCP/RSCPD reporting can be further discussed - Note: Whether to support standalone DL RSCP and/or DL RSCPD reporting, or DL RSCP/DL RSCPD reporting with other new types of measurements (if agreed), can be further discussed. Agreement (RAN1#112bis-e) - Support enabling a TRP to report UL RSCP together with RTOA and/or gNB Rx-Tx time difference measurements to LMF - Note 1: The report of UL carrier phase measurement with gNB Rx – Tx time difference does not necessarily require the report of DL carrier phase measurement with UE Rx – Tx time difference. - Note 2: This doesn’t preclude standalone UL carrier phase measurements reporting. Agreement (RAN1#114) Each DL RSCP/RSCPD measurement instance is obtained with Nsample=1 sample only.

Submitted Proposals:

| Huawei, HiSilicon[6] | Proposal 1: If a UE Rx-Tx time difference/DL RSTD measurement is obtained with Nsample=2 or 4 samples as defined in TS 38.133, the UE Rx-Tx time difference/DL RSTD measurement instance can be associated with (i.e., reported together with) up to Nsample RSCP/RSCPD measurements - Each of RSCP/RSCPD measurements may have its own timestamp. It is up to RAN2/RAN3 on how to define signaling support for the timestamps of the RSCP/RSCPD measurements. |
| --- | --- |
| vivo[9] | Proposal 1: - Each DL RSTD/Rx-Tx difference measurement can be associated with up to N DL RSCPD/RSCP measurements - N can be 1, 2, or 4 up to UE capability. |
| LGE[12] | Proposal 5: A UE Rx-Tx time difference/DL RSTD obtained with N_sample(=2,4) samples can be associated with (i.e., reported together with) up to N_sample RSCP/RSCPD measurements. Proposal 6: Adopt TP for 38.214 for the RSCP/RSCPD measurement reporting. =======================Unchanged part omitted========================== The UE is expected to obtain 1 DL RSCP or DL RSCPD measurement with Nsample,CPP=1 as defined in [11, TS 38.133. When the UE reports DL RSCPD measurement(s) along with DL RSTD measurement(s) or DL RSCP measurement(s) along with UE Rx-Tx time difference measurement(s), whereas DL RSTD measurement(s) and UE-Rx-Tx time difference measurement(s) obtained with Nsample=2 or 4, every DL RSTD measurement(s) and UE-Rx-Tx time difference measurement(s) is associated with Nsample of DL RSCP or DL RSCPD measurement(s). When the UE reports a timestamp associated with a DL RSCP measurement or a DL RSCPD measurement, subject to UE capability, it may include a symbol index in the timestamp. =======================Unchanged part omitted========================== |
| MediaTek[23] | Proposal 2-1: Consider the following TP to TS 38.214 ===================== Unchanged parts omitted ====================== The UE, subject to UE capability, may be requested via [higher layer parameter] to perform positioning measurements on indicated DL PRS resource sets occurring within one or more time window(s) indicated by [higher layer parameter]. The UE is expected to obtain 1 DL RSCP or DL RSCPD measurement within a DL-PRS resource with Nsample=1 as defined in [11, TS 38.133]. The UE, subject to UE capability, may obtain multiple DL RSCP or DL RSCPD measurements within a DL-PRS resource, each measurement associated to a symbol level timestamp, with Nsample=1. ===================== Unchanged parts omitted ====================== |

FL Comments:

In Rel-17, a UE may report one or more RSTD/ Rx-Tx time difference measurement instances, each has its own timestamp, in a single measurement report. Each measurement instance may be obtained based on the DL PRS processing of multiple (Nsample=2 or 4) samples as defined in TS 38.113 for improving RSTD/ Rx-Tx time difference measurement accuracy. For RSCP/RSCPD, the measurements of multiple samples may not be meaningful due to the values of the carrier phases at different samples can be significantly different. Thus, it was agreed in RAN1#114 that each RSCP/RSCPD is obtained from one sample only.

Since each carrier phase measurement has Nsample=1 and each RSTD and/or UE Rx-Tx time difference measurement may use Nsample=2 or 4 samples, then for the time duration of obtaining one RSTD and/or UE Rx-Tx time difference measurement, the UE may obtain Nsample carrier phase measurements. That is, each RSTD and/or UE Rx-Tx time difference measurement can be associated with up to Nsample RSCP/RSCPD measurements as proposed by multiple companies (e.g., [6][9][12][23]). If one RSTD/UE Rx-Tx time difference measurement is associated with multiple RSCP/RSCPD measurements, each RSCP/RSCPD measurements may have their own timestamps, since they are measured at different time instances. The issue was discussed in the last meeting. During the discussion most companies supported it, but some companies proposed one RSTD/UE Rx-Tx time difference measurement is associated with only one RSCP/RSCPD measurement.

A related issue is whether the same RS resource should be used from both CP measurements and legacy measurements if they are reported together. This issue will be discussed in Section 3.4.

(H)(Round 1) Proposal 3.1-1

Subject to UE’s capability, if a UE Rx-Tx time difference/DL RSTD measurement is obtained with Nsample(=2, 4) samples, as defined in TS 38.133, the UE Rx-Tx time difference/DL RSTD measurement can be associated with (i.e., reported together with) up to Nsample RSCP/RSCPD measurements.

- Each RSCP/RSCPD measurements may have its own timestamp, or share the timestamp with UE Rx-Tx time difference/DL RSTD measurement. It is up to RAN2 on how to define signalling support for the timestamps of the RSCP/RSCPD measurements.

| Company | comments |
| --- | --- |
| Xiaomi | As for the sub-bullet, we are wondering if multiple RSCP/RSCPD measurements share the same timestamp, what is the benefit to report more RSCP/RSCPD measurements? FL: From positioning performance point of view, getting more measurements is normally helpful for improving the performance, e.g., it may help to remove the measurement outliers. |
| Samsung1 | The measurement report can include only one RSCP/RSCPD measurement Nsample=1 as agreed in last meeting. Time stamp for this measurement can be included in the report. FL: Yes. |
| Qualcomm | If the RSTD is based on N_sample = 2, and the UE wants to report multiple RSCP measurements for each sample (since we have OFDM-symbol-based timestamp), then wouldn’t the total number of RSCP measurements be more than 2? E..g, the UE reports for a 2-symbol PRS, a UE reports 2 RSCP measurements (a measurement per symbol). Or is the assumption that, within a given sample, the UE can only report a single measurement associated with one OFDM-symbol timestamp? FL: Yes. It can have more than one RSCP measurements in each Nsample=1. For R18, maybe we can limit to 1 measurement for simplicity.. ### (H)(Round 1) Proposal 3.1-1 Subject to UE’s capability, if a UE Rx-Tx time difference/DL RSTD measurement is obtained with Nsample(=2, 4) samples, as defined in TS 38.133, the UE Rx-Tx time difference/DL RSTD measurement are associated with (i.e., reported together with) Nsample RSCP/RSCPD measurements. - A single RSCP/RSCPD measurement is obtained within one sample - Each RSCP/RSCPD measurements has its own timestamp, which may be as, or different from, the timestamp of the associated UE Rx-Tx time difference/DL RSTD measurement. - It is up to RAN2 on how to define signalling support for the reporting of the timestamps of the RSCP/RSCPD measurements. |
| LGE | Support the proposal |
| CATT | Support. Maybe further clarification is needed on whether to support more than one carrier phase measurements within 1 DL PRS processing sample. |
| FL |  |
| Huawei, HiSilicon | Support. Reply to QC: Our understanding is that it is a per-sample RSCP(D) measurement, instead of sub-sample measurement. |
| Nokia/NSB | We are generally okay, but one clarification would be necessary. When the UE shares the timestamp of RSCP/RSCPD with timing measurement, we don’t think the UE cannot include the symbol index. Based on the current agreement, the symbol index in timestamp is only for CP measurement. |

### (Closed)(Round 2) Proposal 3.1-1

Subject to UE’s capability, if a UE Rx-Tx time difference/DL RSTD measurement is obtained with Nsample(=2, 4) samples, as defined in TS 38.133, the UE Rx-Tx time difference/DL RSTD measurement can be associated with (i.e., reported together with) up to Nsample RSCP/RSCPD measurements.

- A single RSCP/RSCPD measurement is obtained within one sample

- Each RSCP/RSCPD measurement has its own timestamp.

  - The timestamp of the RSCP/RSCPD measurement may be the same as the associated UE Rx-Tx time difference/DL RSTD measurement.

- Note: It is up to RAN2 on how to define signalling support for the reporting of the timestamps of the RSCP/RSCPD measurements.

| Company | comments |
| --- | --- |
| FL | Agreement Subject to UE’s capability, if a UE Rx-Tx time difference/DL RSTD measurement is obtained with (=2, 4) samples, as defined in TS 38.133, the UE Rx-Tx time difference/DL RSTD measurement can be associated with (i.e., reported together with) up to RSCP/RSCPD measurements. - A single RSCP/RSCPD measurement is obtained within one sample - Each RSCP/RSCPD measurement has its own timestamp. - Note: It is up to RAN2 on how to define signalling support for the reporting of the timestamps of the RSCP/RSCPD measurements. |

## DL PRS/UL SRS Resources and Resource IDs associated with CP measurements

Background

Agreement (RAN1 #112-bis) Introduce DL reference carrier phase (DL RSCP) and NR DL reference carrier phase difference (DL RSCPD) as DL carrier phase measurements. - Note: It is up to RAN4 to decide whether and how to define the requirements for DL RSCP and/or DL RSCPD. No LS needed to RAN4 for this note. - DL RSCP can be reported together with UE Rx – Tx time difference measurement - DL RSCPD can be reported together with RSTD measurement - FFS: details on how to eliminate unknown initial Rx phase with RSCP/RSCPD reporting can be further discussed - Note: Whether to support standalone DL RSCP and/or DL RSCPD reporting, or DL RSCP/DL RSCPD reporting with other new types of measurements (if agreed), can be further discussed. Agreement (RAN1#112bis-e) - Support enabling a TRP to report UL RSCP together with RTOA and/or gNB Rx-Tx time difference measurements to LMF - Note 1: The report of UL carrier phase measurement with gNB Rx – Tx time difference does not necessarily require the report of DL carrier phase measurement with UE Rx – Tx time difference. - Note 2: This doesn’t preclude standalone UL carrier phase measurements reporting. Agreement (RAN1#112bis-e) For NR UL carrier phase positioning for UE in RRC_CONNECTED and RRC_INACTIVE states, support reuse of existing physical layer procedures for UL positioning (e.g., UL-TDOA), with necessary enhancements in the measurement configuration, measurement request and measurement report (e.g., the configuration related to the NR UL CPP). - FFS: the details of the enhancements. Agreement (RAN1#113) Support the reuse of existing physical layer procedures for DL positioning (e.g., DL-TDOA) with the necessary enhancements in measurement configuration, request and report (e.g., adding the configuration related to the NR DL CPP) for both UE-based and UE-assisted NR DL carrier phase positioning, including - UE in RRC_CONNECTED state with measurement gap. - FFS: UE in RRC_CONNECTED state without measurement gap - UE in RRC_ INACTIVE state Agreement (RAN1#114) When DL RSCPD/RSCP measurements are reported together with the DL RSTD/ UE Rx – Tx time difference measurements, the DL RSCPD/RSCP measurements are obtained from a single DL PFL only.

Submitted Proposals:

| Nokia, NSB[7] | Proposal 7: RAN1 supports the following text proposal on Clause 5.1.6.5 of TS 38.214. <omitted text> For DL UE positioning measurement reporting in higher layer parameter NR-DL-TDOA-SignalMeasurementInformation, the UE may be configured to report the DL Reference Signal Carrier Phase Difference (RSCPD) [7, TS 38.215] measurement along with the DL RSTD. When the UE reports RSCPD measurements the reference nr-DL-PRS-ReferenceInfo is the same as the one reported, for the RSTD measurements. For DL UE positioning measurement reporting in higher layer parameter NR-Multi-RTT-SignalMeasurementInformation the UE may be configured to report the DL Reference Signal Carrier Phase (RSCP) measurement [7, TS 38,215] along with the UE Rx-Tx time difference measurement. When the UE reports DL RSCPD measurement(s) along with DL RSTD measurement(s) or DL RSCP measurement(s) along with UE Rx-Tx time difference measurement(s), the DL RSCPD and/or DL RSCP measurement(s) should be measured from a single DL PRS positioning frequency layer. When the UE reports a DL RSCP measurement along with a UE Rx-Tx time difference measurement, it may include a DL PRS resource set ID, a DL PRS resource ID, an SRS resource ID for positioning, and an indicator [higher layer signaling] indicating whether the DL RSCP measurement is combinable with a UL RSCP measurement associated with the SRS resource ID for positioning. <omitted text>. Proposal 8: When the gNB reports a UL RSCP measurement along with a gNB Rx-Tx time difference measurement, it may also provide a DL PRS resource set ID, a DL PRS resource ID, an SRS resource ID for positioning, and an indicator indicating whether the UL RSCP measurement is combinable with a DL RSCP measurement associated with the DL PRS resource set ID and the DL PRS resource ID. |
| --- | --- |
| Intel[10] | Proposal 3 - When a RSCPD/RSCP and a legacy measurement (e.g., RSTD/RTOA or UE/gNB Rx-Tx time difference) are reported together, it is not restricted that the CP and legacy measurements are performed on the same DL PRS or the same SRS for positioning resources. Proposal 4 - When a UE reports a DL RSCP/RSCPD, support the UE to report the DL PRS resource ID(s) and the DL PRS resource set ID(s), which were used to determine the DL RSCP/RSCPD. - When a TRP reports a UL RSCP, support the TRP to report the UL SRS resource ID(s) and the UL SRS resource set ID(s), which were used to determine the UL RSCP. |
| CATT[15] | Proposal 7: When a UE reports a DL RSCP/RSCPD, support the UE to report the DL PRS resource ID(s) and the DL PRS resource set ID(s), which were used to determine the DL RSCP/RSCPD. Proposal 8: When a TRP reports a UL RSCP, support the TRP to report the UL SRS resource ID(s) and the UL SRS resource set ID(s), which were used to determine the UL RSCP. Proposal 9: RSCPD/RSCP and RSTD/Rx-Tx time difference can be measured from different PRS resources when they are reported together. |
| Apple[19] | Proposal 3: In the measurement report - When a UE reports a DL RSCP/RSCPD, support the UE to report the DL PRS resource ID(s) and the DL PRS resource set ID(s), which were used to determine the DL RSCP/RSCPD. - When a TRP reports a UL RSCP, support the TRP to report the UL SRS resource ID(s) and the UL SRS resource set ID(s), which were used to determine the UL RSCP. |
| NTT DOCOMO[22] | Proposal 2: - Whether RSCP/RSCPD and legacy positioning metric are measured from same PRS resource or different PRS resources should be clarified in the specification. - RSCP/RSCPD and legacy positioning metric are measured from the same PRS resource. |

FL Comments:

Whether to support a UE to report the DL PRS resource ID(s) and the DL PRS resource set ID(s), which were used to determine the DL RSCP/RSCPD, was discussed in the last meeting without the conclusion [26]. During the last discussion, most of the companies were supportive for the UE to report the DL PRS resource ID(s) and the DL PRS resource set ID(s) with the consideration that DL PRS resources used to obtain the DL RSCP/RSCPD and UE/gNB Rx-Tx time difference may or may not be the same (e.g., [7][10[15][19]]. However, some companies proposed that the same resources should be used for the CP and legacy measurements (e.g., [22]). In this case, it does not need to report the DL PRS resource ID(s) and the DL PRS resource set ID(s) specifically for the DL RSCP/RSCPD.

In FL’s view, the same PRS resource(s) are most likely to be used to obtain the DL RSCP/RSCPD and UE/gNB Rx-Tx time difference when both of the measurements are obtained by the same DL PRS processing time instance. However, considering that it was agreed that a DL RSCP/RSCPD is obtained from a single sample (N_sample=1), but a UE/gNB Rx-Tx time difference measurement may be obtained from multiple DL PRS resources at multiple samples (N_sample>1), the PRS resource(s) used to obtain the DL RSCP/RSCPD and UE/gNB Rx-Tx time difference may not always be the same.

In case the DL PRS resource(s)/DL PRS resource set(s) for DL RSCP/RSCPD and UE/gNB Rx-Tx time difference are the same, UE may need only to report DL PRS resource(s)/DL PRS resource set(s) for DL RSCP/RSCPD for singling optimization, which can be decided by RAN2/RAN3.

(H)(Round 1) Proposal 3.2-1

- When a UE reports a DL RSCP/RSCPD, support the UE to report the DL PRS resource ID(s) and the DL PRS resource set ID(s), which were used to determine the DL RSCP/RSCPD.

- When a TRP reports a UL RSCP, support the TRP to report the UL SRS resource ID(s) and the UL SRS resource set ID(s), which were used to determine the UL RSCP.

- Note: It is up to RAN2/RAN3 on the singling design of the reporting of the PRS/SRS resource ID(s) and the PRS/SRS resource set ID(s), when the same or different PRS/SRS resources and the PRS/SRS resource sets are used to determine the UE/gNB Rx-Tx time difference/RSTD and the RSCP/RSCPD, which are reported together in a measurement report.

| Company | comments |
| --- | --- |
| Vivo | Reporting the resource ID for carrier phase measurement only is more like a standalone positioning, we wonder how to achieve IAR in this case. So, we prefer not to pursue the issue. |
| Xiaomi | Support |
| InterDigital | We agree with the FL’s view “the same PRS resource(s) are most likely to be used to obtain the DL RSCP/RSCPD and UE/gNB Rx-Tx time difference when both of the measurements are obtained by the same DL PRS processing time instance”. We don’t see benefits using different PRS resources between RSTD and RSCPD, for example. |
| ZTE | Support. With resource ID and resource set ID reported, the LMF can get more information about the PRS transmission details. |
| Spreadtrum | Share the same view with FL. The same PRS resource can be used for RSCP/RSCPD and RSTD/ UE/gNB Rx-Tx time difference. Thus, this proposal is not needed. |
| Samsung1 | When carrier phase measurements and legacy positioning are in the same report, the same PRS resource should be used. We don’t see a benefit report separate resources for CP. |
| Qualcomm | Since the CPP report is part of the legacy report, the already existing signaling can be reused. No need of a separate PRS resource ID report. |
| LG | We are open to discuss this issue as an optional feature. |
| CATT | Support. |
| FL | It seems most of the feedback so far only consider the same PRS resources are used for both DL RSCP/RSCPD and UE/gNB Rx-Tx time difference when the DL RSCP/RSCPD and UE/gNB Rx-Tx time difference are measured at the same resources and the same time instances. However, DL RSCP/RSCPD and UE/gNB Rx-Tx time difference may not always be measured at the same resources same at the same time instance. |
| Ericsson | As CPP measurements will be part of legacy report, we don’t see the need to report separate resource IDs. |
| Huawei, HiSilicon | We share similar view as QC. No additional resource ID beyond the existing reporting format for timing/power is needed. |
| Nokia/NSB | For RSCP measurement reporting, LMF needs more information for RTT type CP. From the UE side, the same oscillator should be used to transmit UL SRS and perform DL PRS measurement. The UE needs to report if the reported RSCP is combinable or not with a specific UL RSCP measurement from a transmitted SRS. |
| FL | For the comments that RSCP/RSCPD and RSTD/ UE/gNB Rx-Tx time difference have the same PRS resources, one question from me is that: we have the agreement that for one legacy measurement can be associated with up to 4 RSCP/RSCPD measurements, which may or may not be measured at the same time instance. Do we want to force the same PRS resources are used for RSCP/RSCPD and RSTD/ UE/gNB Rx-Tx time difference? ### (H)(Round 1) Proposal 3.2-1 - When a UE reports a DL RSCP/RSCPD, support the UE to report the DL PRS resource ID(s) and the DL PRS resource set ID(s), which were used to determine the DL RSCP/RSCPD, if the DL RSCP/RSCPD measurements are not obtained from the same DL PRS resource(s) that are used to obtain the associated UE Rx-Tx time difference/RSTD measurements. - When a TRP reports a UL RSCP, support the TRP to report the UL SRS resource ID(s) and the UL SRS resource set ID(s), which were used to determine the UL RSCP if the UL RSCP measurements are not obtained from the same UL positioning SRS resource(s) that are used to obtain the associated gNB Rx-Tx time difference/RTOA. |

(H)(Round 2) Proposal 3.2-1

- When a UE reports a DL RSCP/RSCPD, support the UE to indicate the DL PRS resource ID and the DL PRS resource set ID, which were used to determine the DL RSCP/RSCPD, if the DL RSCP/RSCPD measurement is not obtained from the same DL PRS resource(s) that are used to obtain the associated UE Rx-Tx time difference/RSTD measurements.

- When a TRP reports a UL RSCP, support the TRP to report the UL SRS resource ID(s) and the UL SRS resource set ID(s), which were used to determine the UL RSCP if the UL RSCP measurements are not obtained from the same UL positioning SRS resource(s) that are used to obtain the associated gNB Rx-Tx time difference/RTOA.

| Company | comments |
| --- | --- |
| ZTE | Support. This version is better, adding the report condition seems more reasonable to allow UE report DL PRS resource ID and set ID for CP measurement. |
| Samsung2 | Don’t support. When carrier phase measurements and legacy positioning are in the same report, the same PRS resource should be used. We don’t see a benefit report separate resources for carrier phase. |
| InterDigital | Same view as Samsung. |
| FL | To Samsung/ InterDigital: FL: A legacy measurement may be obtained from more than one PRS resources. However, using multiple PRS resources to obtain one carrier phase measurement may not be meaningful, because of the difference of the carrier phase from different PRS resources. In addition, we have the agreement a UE Rx-Tx time difference/DL RSTD measurement is obtained with (=2, 4) samples, while a single RSCP/RSCPD measurement is obtained within one sample only. Thus, for some time instance, there is no legacy measurement. It might be an issue to force the UE to use the same PRS resource for CP and legacy measurements. ### (H)(Round 2) Proposal 3.2-1 (Revised) - The DL PRS resource that is used to obtain a DL RSCP/RSCPD should be: - the same as the DL PRS resource that is used to determine the associated UE Rx-Tx time difference/RSTD; or - one of DL PRS resources that are used to determine the associated UE Rx-Tx time difference/RSTD. In this case, the UE needs to indicate the DL PRS resource that is used to obtain the DL RSCP/RSCPD in the measurement report. |

### (H)(Round 3) Proposal 3.2-1a

- A DL RSCP measurement is obtained by measuring a single DL PRS resource from a TRP.

- The DL PRS resource is either the same DL PRS resource used to obtain the associated UE Rx-Tx time difference measurement, or one of the DL PRS resources used to obtain the associated UE Rx-Tx time difference measurement.

- In the signalling design, the LMF should be able to identify which DL PRS resource and DL PRS resource set are used to obtain the DL RSCP measurement. The signalling design is up to RAN2/RAN3.

| Company | comments |
| --- | --- |

### (H)(Round 3) Proposal 3.2-1b

- A DL RSCPD measurement is obtained by measuring two DL PRS resources: one from the target TRP and one from the reference TRP;

- The two DL PRS resource are either the same DL PRS resources used to obtain the associated UE RSTD measurement, or included in the DL PRS resources used to obtain the associated UE RSTD measurement.

- In the signalling design, the LMF should be able to identify which DL PRS resource and DL PRS resource set are used to obtain the DL RSCP measurement. The signalling design is up to RAN2/RAN3.

| Company | comments |
| --- | --- |

## Reporting of the frequency of the carrier phase measurement

Submitted Proposals:

| Intel[10] | Proposal 2 - For UE-based carrier phase positioning, when LMF forwards the DL carrier phase measurement reported by a PRU to a target UE, the reference frequency associated with the PRU carrier phase measurements should also be forwarded in positioning assistance data. |
| --- | --- |
| CATT[15] | Proposal 10: LMF should provide the same RF frequency information for the carrier phase measurements to all TRPs, which can be used by the TRPs to properly configure the DL PRS resources and/or the UL SRS resources for UEs (including PRU). Proposal 11: Frequency information can be reported together with the associated carrier phase measurements. |

FL Comments:

In [10], it was proposed that when LMF forwards the DL carrier phase measurement reported by a PRU to a target UE for UE-based positioning, the reference frequency associated with the PRU carrier phase measurements should also be forwarded in positioning assistance data. In FL’s view, including the frequency information may help to avoid any confusion especially for UE based positioning in RRC_INACTIVE/RRC_IDLE state.

In [15], it was proposed that the LMF should provide the same RF frequency information for the carrier phase measurements to all TRPs. For DL CPP, the information is used by the TRPs to properly configure the DL PRS resources for RSCP/RSCPD measurements, so that the RSCP/RSCPD measurements from all UEs are associated with the same RF frequency. For UL CPP, the information can also be used for the TRPs to configure the UL SRS resources with the same center frequency, so that the all TRPs can configure the SRS resources properly for all USs with the same center RF frequency.

(H)(Round 1) Proposal 3.3-1

When LMF forwards the carrier phase measurement reported by a PRU to a target UE for UE-based carrier phase positioning, the RF frequency information associated with the PRU carrier phase measurements should also be included in positioning assistance data.

| Company | comments |
| --- | --- |
| vivo | Firstly, we would like to confirm whether LMF needs to forward legacy measurements. FL: Whether LMF needs to forward legacy measurements can be discussed separately in my view, since here we focus on the RF frequency information, and legacy measurements are not associated with RF frequency. Secondly, we wonder whether the “indicated PRS resource set” can be used to indicate the RF frequency information. FL: If we assume the DL PRS configurations for all UEs, including PRUs, are the same, then the answer is yes. Otherwise, the answer will be no. |
| ZTE | We also wonder what does RF frequency information refers to. Can resource ID and resource set ID reflect the RF frequency information? FL: Similar to the response. To vivo’s comments, the answer depends on whether we will force the same DL PRS configurations to be used for all TRPs and all UEs. |
| Qualcomm | It is a more general solution to juts forward the configured PRS resource assistance data. Then, together with the measurement report, the UE will know all the information related to what was measured. I.e., FL: Yes. The drawback is that more data needs to be provided to the UE. When LMF forwards the carrier phase measurement reported by a PRU to a target UE for UE-based carrier phase positioning, the PRS resource(s) configuration associated with the measurements are also included . To vivo: Yes, from our side, legacy measurements should also be forwarded. It should not be just CPP measurements. FL: Yes. This can be further discussed. In my view, forwarding legacy measurements is one way to support the double differential of the legacy measurements to minimize or eliminate the impact of some errors, such as the gNB time-synchronization errors. On the other hand, sending RTD information to UE also have the similar functions. |
| LGE | First, we think that this proposal is related to the issue in Question 2.1-2. If RF frequency other than center frequency is allowed, some mechanism for indicating RF frequency of PRU measurements should be considered. If not, we need to clarify what is the “RF frequency information” is. If the intention is to forward the information of which PFL is used for the measurements, we are fine. However, additional information that represent RF frequency directly would not be useful. FL: If we assume all UEs, including the PRUs, are measuring the same DL PFLs, which have the same configuration, then RF frequency is not needed. If we assume all PFLs, have the same configuration, but not all UE are configurated to measure the same PFL, then some way to indicate PFL or the center frequency of the PFL is needed. |
| CATT | Support |
| Nokia/NSB | The target UE and the PRU may perform the CP measurement for indicated PRS resource set(s). The target UE also has the PRS resource and resource set configuration, which is used by the PRU. We are not sure if it is necessary. We prefer to first discuss if the existing reporting contents are enough or not. |

### (H)(Round 2) Proposal 3.3-1

When LMF forwards the carrier phase measurements reported by a PRU to a target UE for UE-based carrier phase positioning, the following information can be included in the positioning assistance data to the target UE (down-selection):

- Option 1: the RF frequency information associated with the PRU carrier phase measurements.

- Option 2: the DL PRS configuration information that were provided to the PRU for the carrier phase measurements.

| Company | comments |
| --- | --- |
| ZTE | Support option 2. The RF frequency information can be included in the PRS configuration information. |
| Samsung2 | Prefer option 1. |
| InterDigital | Option 2. Regarding the RF frequency, we have the same view as ZTE. |

### (H)(Round 1) Proposal 3.3-2

Support LMF to provide the requested RF frequency information associated with the carrier phase measurements to TRPs for DL CPP and UL CPP.

| Company | comments |
| --- | --- |
| vivo | Same view as proposal 3.3-1 |
| LGE | Similar comment to Proposal 3.3-1. If RF frequency other than the default is introduced, this can be helpful. |
| CATT | Support |

## Standalone reporting of carrier phase measurements

Background

Agreement Introduce DL reference carrier phase (DL RSCP) and NR DL reference carrier phase difference (DL RSCPD) as DL carrier phase measurements. - Note: It is up to RAN4 to decide whether and how to define the requirements for DL RSCP and/or DL RSCPD. No LS needed to RAN4 for this note. - DL RSCP can be reported together with UE Rx – Tx time difference measurement - DL RSCPD can be reported together with RSTD measurement - FFS: details on ho Note: Whether to support standalone DL RSCP and/or DL RSCPD reporting, or DL RSCP/DL RSCPD reporting with other new types of measurements (if agreed), can be further discussed.

Submitted Proposals:

| Spreadtrum[8] | Proposal 1: Do not support standalone DL RSCP and/or DL RSCPD reporting. - Adopt the following TP for section 5.1.6.5 in TS 38.214. Reason for change; Existing positioning technologies such as RSTD and Rx – Tx time difference measurement have been agreed to be reported with RSCPD/RSCP. They can be used for NR carrier phase positioning to obtain integer parts. Therefore, we can stick with the existing framework. Therefore, standalone DL RSCP and/or DL RSCPD reporting is not required.. Summary of change; Section 5.1.6.5 in 38.214: clarify the standalone DL RSCP and/or DL RSCPD reporting is not supported.. Consequences if not approved; It is not clear in the spec whether standalone DL RSCP and/or DL RSCPD can be supported.. Text proposal; TS 38.214 5.1.6.5 PRS reception procedure ************** Unchanged parts omitted************** For DL UE positioning measurement reporting in higher layer parameter NR-DL-TDOA-SignalMeasurementInformation, the UE may be configured to report the DL Reference Signal Carrier Phase Difference (RSCPD) [7, TS 38.215] measurement along with the DL RSTD. When the UE reports RSCPD measurements the reference nr-DL-PRS-ReferenceInfo is the same as the one reported, for the RSTD measurements. For DL UE positioning measurement reporting in higher layer parameter NR-Multi-RTT-SignalMeasurementInformation the UE may be configured to report the DL Reference Signal Carrier Phase (RSCP) measurement [7, TS 38,215] along with the UE Rx-Tx time difference measurement. When the UE reports DL RSCPD measurement(s) along with DL RSTD measurement(s) or DL RSCP measurement(s) along with UE Rx-Tx time difference measurement(s), the DL RSCPD and/or DL RSCP measurement(s) should be measured from a single DL PRS positioning frequency layer. The UE is not expected to be configured to report the DL Reference Signal Carrier Phase Difference (RSCPD) or DL Reference Signal Carrier Phase (RSCP) measurement individually. ************** Unchanged parts omitted**************. |
| --- | --- |
| Samsung[13] | Proposal 7: Support standalone carrier phase measurement report for: - DL reference signal carrier phase (DL RSCP). - DL reference signal carrier phase difference (DL RSCPD). - UL reference signal carrier phase (UL RSCP). For DL UE positioning measurement reporting in higher layer parameter NR-DL-TDOA-SignalMeasurementInformation, the UE may be configured to report the DL Reference Signal Carrier Phase Difference (RSCPD) [7, TS 38.215] measurement along with the DL RSTD. When the UE reports RSCPD measurements the reference nr-DL-PRS-ReferenceInfo is the same as the one reported, for the RSTD measurements. For DL UE positioning measurement reporting in higher layer parameter NR-Multi-RTT-SignalMeasurementInformation the UE may be configured to report the DL Reference Signal Carrier Phase (RSCP) measurement [7, TS 38,215] along with the UE Rx-Tx time difference measurement. The UE may be configured to report DL RSCP or DL RSCPD without other DL positioning measurements. When the UE reports DL RSCPD measurement(s) along with DL RSTD measurement(s) or DL RSCP measurement(s) along with UE Rx-Tx time difference measurement(s), the DL RSCPD and/or DL RSCP measurement(s) should be measured from a single DL PRS positioning frequency layer. Text proposal 9: (clause 5.1.6.5 of TS 38.214) |
| InterDigital[18] | Proposal 1: Adopt the following TP to introduce standalone carrier phase reporting/positioning in Rel-18. The reason for the change is to enable reporting of standalone phase measurements. The summary of change is introduction of standalone phase measurements. The consequence if not approved is that positioning accuracy may degrade. --------------------------<Start of text proposal for TS 38.214>-------------------------- #### 5.1.6.5 PRS reception procedure < Unchanged parts are omitted > For DL UE positioning measurement reporting in higher layer parameter NR-DL-TDOA-SignalMeasurementInformation, the UE may be configured to report the DL Reference Signal Carrier Phase Difference (RSCPD) [7, TS 38.215] measurement along with the DL RSTD. When the UE reports RSCPD measurements the reference nr-DL-PRS-ReferenceInfo is the same as the one reported, for the RSTD measurements. The UE may be configured to report RSCPD measurements without the DL RSTD. For DL UE positioning measurement reporting in higher layer parameter NR-Multi-RTT-SignalMeasurementInformation the UE may be configured to report the DL Reference Signal Carrier Phase (RSCP) measurement [7, TS 38,215] along with the UE Rx-Tx time difference measurement. When the UE reports DL RSCPD measurement(s) along with DL RSTD measurement(s) or DL RSCP measurement(s) along with UE Rx-Tx time difference measurement(s), the DL RSCPD and/or DL RSCP measurement(s) should be measured from a single DL PRS positioning frequency layer. < Unchanged parts are omitted > --------------------------<End of text proposal for TS 38.214>--------------------------. |
| Apple[19] | Proposal 2: Do not support standalone carrier phase positioning at the UE. |
| Lenovo[21] | Proposal 1: RAN1 to deprioritize specification of CP measurements in terms of a standalone method for DL and UL CP measurements. |
| Qualcomm[24] | Proposal 2: Do not support standalone reporting of carrier phase measurements by UE (DL RSCP/RSCPD) or gNB (UL RSCP). |

FL Comments:

Whether to support the standalone carrier phase-based positioning (or standalone reporting of carrier phase measurements by UE (DL RSCP/RSCPD) or gNB (UL RSCP)) has been discussed in the previous meetings. During the discussion of the last meetings, although many companies supported the standalone CPP, but the majority companies did not support it [26].

Considering that a) RAN1 has spent significant effort on this issue; b) it is unlikely that RAN1 can reach a consensus to support it due to no majority support; and c) the WI was already completed from RAN1’s perspective, maybe it is better for us to close the discussion in Rel-18, so we can spend the limited meeting time on other more urgent issues.

### (Closed) Proposal 3.4-1 (as conclusion)

No further discussion in RAN1 regarding the support of standalone reporting of carrier phase measurements in Rel-18.

| Company | comments |
| --- | --- |
| Spreadtrum | Support this proposal. |
| Qualcomm | Support |
| LGE | Although we prefer to have standalone CPP, but it seems difficult to finalize standalone CPP at this stage. |
| CATT | Support |
| Ericsson | ok |
| Huawei, HiSilicon | Support |
| Nokia/NSB | Okay, but we think this information should be sent to RAN2 to help their further work. |
| FL | Conclusion No further discussion in RAN1 regarding the support of standalone reporting of NR carrier phase measurements in Rel-18. |

## Reporting of carrier phase measurements with legacy measurements

Background

Agreement (RAN1#112) For NR carrier phase positioning, at least support the following approach: enable a UE/TRP to report carrier phase measurements together with the legacy positioning measurements to LMF FFS: which legacy positioning measurements among RSTD, RTOA, UE Rx-Tx time difference measurements, gNB Rx-Tx time difference measurements. Agreement (RAN1#112bis-e) - Support enabling a TRP to report UL RSCP together with RTOA and/or gNB Rx-Tx time difference measurements to LMF - Note 1: The report of UL carrier phase measurement with gNB Rx – Tx time difference does not necessarily require the report of DL carrier phase measurement with UE Rx – Tx time difference. - Note 2: This doesn’t preclude standalone UL carrier phase measurements reporting. Agreement (RAN1#112bis-e) For NR UL carrier phase positioning for UE in RRC_CONNECTED and RRC_INACTIVE states, support reuse of existing physical layer procedures for UL positioning (e.g., UL-TDOA), with necessary enhancements in the measurement configuration, measurement request and measurement report (e.g., the configuration related to the NR UL CPP). - FFS: the details of the enhancements. Agreement Support the reuse of existing physical layer procedures for DL positioning (e.g., DL-TDOA) with the necessary enhancements in measurement configuration, request and report (e.g., adding the configuration related to the NR DL CPP) for both UE-based and UE-assisted NR DL carrier phase positioning, including - UE in RRC_CONNECTED state with measurement gap. - FFS: UE in RRC_CONNECTED state without measurement gap - UE in RRC_ INACTIVE state

Submitted Proposals:

| Qualcomm[24] | Proposal 3: UE does not report RSCP together with RSTD. UE does not report RSCPD together with UE-RxTx. |
| --- | --- |

FL Comments:

In [24], it was proposed to define in the specification that “UE does not report RSCP together with RSTD. UE does not report RSCPD together with UE-RxTx”. The issue was discussed in last meeting [26]. From the feedback of the last meeting, no company supported to capture it in RAN1 specification. Some company proposed it to be captured by RAN2. Some companies proposed it can be capture in Chairman note as a conclusion. The FL likes to check with interested companies to see if it is acceptable.

### (Closed) Proposal 3.5-1 (as a conclusion)

From RAN1’s perspective, reporting RSCP together with RSTD, or reporting RSCPD together with UE-RxTx, which are measured from the same DL PFL, are not supported.

| Company | comments |
| --- | --- |
| vivo | OK |
| Xiaomi | No need to capture in RAN1 specification |
| ZTE | We think it’s a common understanding. And no need to clarify this. |
| Samsung1 | This proposal has no spec impact. Spec already describes what is supported. |
| Qualcomm | This conclusion is not needed FL: I am confused. It is Qualcomm’s proposal “UE does not report RSCP together with RSTD. UE does not report RSCPD together with UE-RxTx”. |
| LGE | We think it is already common understanding. |
| CATT | OK |
| FL | Based on the feedback, the FL suggest closing the discussion of the proposal. |
| Ericsson | No need to capture what is not supported in the specs. |
| FL | The discussion is closed due to the lack of the support. |

## Carrier phase compensation

Submitted Proposals:

| Nokia, NSB[7] | Proposal 10: NR CPP should support at least one of the following phase compensation options: - Option 1: The LMF requests the UE (gNB) to perform CP measurement on a specific DL PRS (UL SRS) and to compensate for the obtained measurement considering the UE mobility between the measurement timing (t=t1) and the report timing (t=t2). The UE (gNB) reports a CP measurement and informs the LMF that the CP measurement has been compensated considering the UE mobility. - Option 2: The LMF requests the UE (gNB) to perform CP measurement on a specific DL PRS (UL SRS) and to report additionally the measured Doppler shift for LMF to account the UE mobility in the position location determination. - Option 3: The LMF requests the UE (gNB) to perform a pre-compensation before the transmission of UL SRS (DL PRS). The LMF instructs the gNB (UE) how to measure the pre-compensated signal. The LMF requests the gNB (UE) to report the used pre-compensation, and the LMF accounts the information in the position location determination. |
| --- | --- |

FL Comments:

In [7], it was proposed to consider the carrier phase compensation for UE mobility. In FL’s view, although the issue could be important for CPP performance for moving UEs, it might be too late to discuss the issue given that the WI has entered the maintenance stage.

### (Closed) Question 3.6-1

What is your view on above Proposal 10 from [7] for carrier phase compensation for UE mobility in Rel-18?

| Company | comments |
| --- | --- |
| vivo | Share the same view as FL, we prefer not pursue the issue in the maintenance stage. |
| Samsung1 | As we are now in maintenance, prefer not pursue this. |
| LGE | It seems hard to begin such discussion in maintenance phase. |
| CATT | Share the similar view as other companies. |
| FL | Based on the feedback, the FL suggest closing the discussion of the proposal. |
| Ericsson | Not support as we are in maintenance. |
| FL | The discussion is closed due to the lack of the support. |

# Differential CPP and PRU

## Simultaneous PRS measurements and SRS transmission

Background

Agreement RAN1 will continue discussions on what enhancements to LPP, NRPPa, and/or RAN signaling are necessary to support simultaneous measurements of the same DL-PRS for multiple Ues, including a target UE and a PRU; and to support simultaneous transmission of SRS for multiple Ues, including a target UE and a PRU. RAN1 will include the outcome in an LS to SA2 and RAN2, and cc RAN3. Note: Enhancements might or might not have RAN1 specification impact. Working assumption (RAN1#113) - To enable LMF to optionally request the serving gNB of a UE to configure the transmission of the UL positioning SRS resources from the UE within indicated time window(s), support: - Option 1D: Each of the time windows is defined with the following parameters: - The start of the time window, which is indicated by a combination of subframe number, slot offset and symbol index with respect to the SFN initialization time - The duration of the time window, which is given by a number of consecutive slots/symbols - FFS: the number of the consecutive slots/symbols - (Optional) The periodicity of the time window, which is defined similar to IE PeriodicitySRS in “Requested SRS Transmission Characteristics” in TS 38.455. - FFS: the maximum number of the windows Agreement (RAN1#113) To enable LMF to request the serving gNB and neighboring gNBs of a UE to measure the UL SRS resources from the UE within indicated time window(s), each time window is defined with the following parameters: - The start of the time window, which is indicated by a combination of subframe number, slot offset and symbol index with respect to the SFN initialization time - The duration of the time window, which is given by a number of consecutive slots/symbols - FFS: the number of consecutive slots/symbols - (Optional) The periodicity of the time window, which is defined similar to IE Measurement Periodicity in MEASUREMENT REQUEST in TS 38.455. - FFS: the maximum number of the windows Agreement (RAN1#113) To enable LMF to request the UEs, including target UE and PRU(s), to perform measurements on indicated DL PRS resource set(s) occurring within indicated time window(s), each time window is defined with the following parameters: - The start of the time window, which is indicated by a combination of subframe number, slot offset and symbol index - The duration of the time window, which is given by a number of consecutive slots/symbols - FFS: the number of consecutive slots/symbols - (Optional) The periodicity of the time window, which is defined similar to IE NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset in TS 37.355.FFS: the maximum number of the windows Agreement (RAN1#114) - When a LMF requests the serving gNB of a UE to configure the transmission of the UL positioning SRS resources from the UE within indicated time window(s), - the duration of a time window can be configured by one of the following values: - {1, 2, 4, 8, 12} OFDM symbols - {1, 2, 4, 6, 8, 12, 16} slots - FFS: additional values - the number of the time windows can be configured as: - {1, 2, …, 16} Agreement (RAN1#114) When a LMF requests the serving gNB and neighboring gNBs of a UE to measure the UL SRS resources from the UE within indicated time window(s): - The duration of a time window can be configured as follows: - {1, 2, 4, 6, 8, 12, 16} slots. - the number of the time windows can be: - {1, 2, …, 16} Agreement (RAN1#114) When an LMF requests the UEs, including target UE and PRU(s), to perform measurements on indicated DL PRS resource set(s) occurring within indicated time window(s) - The duration of a time window can be configured as follows: - {1, 2, 4, 6, 8, 12, 16} slots. - the number of the time windows can be: - {1, 2} - FFS: {4, 8} Agreement (RAN1#114) For UE-based carrier phase positioning, support enabling LMF to forward the DL carrier phase measurement reported by a PRU, with additional information of the same PRU to a target UE for UE-based carrier phase positioning in the positioning assistance data. - Note: Whether the forwarded DL carrier phase measurement is DL RSCP and/or DL RSCPD depends at least on which of them is (are) supported by UE capability. - additional information of the same PRU includes at least PRU location. - FFS: additional PRU information, e.g. the AoD of PRU to each TRP, etc. Agreement (RAN1#114) For UE-based carrier phase positioning, when LMF forwards the DL carrier phase measurement reported by a PRU to a target UE, the timestamp associated with the PRU carrier phase measurements should also be forwarded in positioning assistance data.

Submitted Proposals:

| Huawei, HiSilicon[6] | Proposal 2: One bit indicator should be added to indicate whether the measurement within the indicated time window(s) is only for RSCP/RSCPD or whether the target UE and PRU(s) can measure the DL PRS resource set(s) outside the indicated time window(s). Signaling detail is up to RAN2. Proposal 3: Within the indicated measurement time window, all DL PRS resource sets should be from a single positioning frequency layer. |
| --- | --- |
| Nokia, NSB[7] | Proposal 11: RAN1 supports the following text proposal on Clause 5.1.6.5 of TS 38.214. <omitted text> The UE, subject to UE capability, may be requested via [higher layer parameter] to perform positioning measurements on indicated DL PRS resource sets and DL PRS resource(s) occurring within one or more time window(s) indicated by [higher layer parameter]. The UE is expected to obtain 1 DL RSCP or DL RSCPD measurement with Nsample=1 as defined in [11, TS 38.133]. <omitted text> The UE may be configured with [higher layer parameter] which contains DL carrier phase measurements performed by a positioning reference unit (PRU) [20, TS 38.305] along with the location information of the PRU. The UE may request to be provided with DL RSCP and/or DL RSCP measurements of the PRU for specific DL PRS resource(s) within a time window indicated by [higher layer parameter]. <omitted text>. FL: In my view, we may not need to have the requirement “The UE may request to be provided with DL RSCP and/or DL RSCP measurements of the PRU for specific DL PRS resource(s) within a time window indicated by [higher layer parameter]”, since PRU can be seen as an regular UE, and thus it is covered by the existing requirement “The UE, subject to UE capability, may be requested via [higher layer parameter] to perform positioning measurements on indicated DL PRS resource sets and DL PRS resource(s) occurring within one or more time window(s) indicated by [higher layer parameter]”. Proposal 12: RAN1 supports the following text proposal on Clause 5.1.6.5 of TS 38.214. <omitted text> The UE may be configured with [higher layer parameter] which contains DL RSCP and/or RSCPD carrier phase measurements including a timestamp associated with the measurements performed by a positioning reference unit (PRU) [20, TS 38.305] along with the location information of the PRU. <omitted text>. |
| ZTE[11] | Proposal 5: The number of time windows for simultaneous measurement can be {1,2,4,8}. |
| CATT[15] | Proposal 2: Adopt the following TP regarding the timestamp of the phase quality measurements in Clause 5.1.6.5 of TS 38.214 Reason for change:; The specification does not capture the following agreement made in RAN1#114 regarding to the timestamp associated with the PRU carrier phase measurements. Agreement For UE-based carrier phase positioning, when LMF forwards the DL carrier phase measurement reported by a PRU to a target UE, the timestamp associated with the PRU carrier phase measurements should also be forwarded in positioning assistance data.. Summary of change:; capture the following agreement made in RAN1#114 regarding to the timestamp associated with the PRU carrier phase measurements.. Consequences if not approved:; Specification is not align with RAN1’s agreement and incomplete incomplete. #### 5.1.6.5 PRS reception procedure ------------------ Start of text proposal to TS 38.214 v18.0.0------------------------------------------- … The UE may be configured with [higher layer parameter] which contains DL carrier phase measurements performed by a positioning reference unit (PRU) [20, TS 38.305] along with the location information of the PRU and timestamp associated with the PRU carrier phase measurements. … ---------------------------- End of Text proposal to TS 38.214 v18.0.0------------------------------------------ |
| OPPO[16] | Proposal 4: Endorse the following TP for TS 38.214: - Reason for change: we do not define the terminology “DL carrier phase measurements” but it is used in the specification text - Summary of change: Change it to “DL RSCP” or “DL RSCPD” which are officially defined in NR specification - Consequences if not approved: The specification on carrier phase measurement function has ambiguity #### 5.1.6.5 PRS reception procedure #### *** Unchanged parts are omitted *** The UE, subject to UE capability, may be requested via [higher layer parameter] to perform positioning measurements on indicated DL PRS resource sets occurring within one or more time window(s) indicated by [higher layer parameter]. The UE is expected to obtain 1 DL RSCP or DL RSCPD measurement with Nsample=1 as defined in [11, TS 38.133]. When the UE reports a timestamp associated with a DL RSCP measurement or a DL RSCPD measurement, subject to UE capability, it may include a symbol index in the timestamp. If the UE reports LoS/NLoS indicator(s) via higher layer parameter nr-los-nlos-Indicator along with a measurement report containing DL RSCP or DL RSCPD the LoS/NLoS indicator(s) are assumed to also apply to the DL RSCP or DL RSCPD measurements. The UE may be configured with [higher layer parameter] which contains DL RSCP measurements or DL RSCPD measurements performed by a positioning reference unit (PRU) [20, TS 38.305] along with the location information of the PRU. #### *** Unchanged parts are omitted ***. |
| InterDigital[18] | Proposal 4: Adopt the following TP to include PRS resource ID(s) and timestamp associated with PRU measurement in the measurement report by the PRU. The reason for the change is to include additional details to the forwarded measurements. The summary of change is to append PRS resource IDs and timestamp associated with the measurements. The consequence if not approved is that the UE cannot implement UE based positioning using forwarded PRU measurements. --------------------------<Start of text proposal for TS 38.214>-------------------------- #### 5.1.6.5 PRS reception procedure < Unchanged parts are omitted > The UE may be configured with [higher layer parameter] which contains DL carrier phase measurements performed by a positioning reference unit (PRU) [20, TS 38.305] along with the location information of the PRU , and PRS resource ID(s) and timestamp to which the DL carrier phase measurements are associated with. < Unchanged parts are omitted > --------------------------<End of text proposal for TS 38.214>--------------------------. Proposal 7 : Adopt only 1 or 2 as the number of configurable time windows for simultaneous measurements between target PRU and UE |
| Ruijie [20] | Proposal 1: The number of the time windows for UE/PRU to perform measurements on DL PRS resource set(s) can be: {1, 2, 4, 8}. |
| NTT DOCOMO[22] | Proposal 4: - Regarding row 43/44/45 in higher layer parameters list for NR positioning, the following value range can be considered at least. - Row 43: Requesting SRS for positioning configuration from LMF to the serving gNB - the duration of a time window: introduce {1, 2, 4, 8, 12} OFDM symbols and {1, 2, 4, 6, 8, 12, 16} slots - the number of the time windows: introduce {1, 2, …, 16} and does not introduce additional value - Row 44: Requesting SRS for positioning configuration from LMF to the serving gNB and neighbor gNB - the duration of a time window: introduce {1, 2, 4, 6, 8, 12, 16} slots - the number of the time windows: introduce {1, 2, …, 16} - Row 45: Requesting DL-PRS reception from LMF to a UE - the duration of a time window: introduce {1, 2, 4, 6, 8, 12, 16} slots - the number of the time windows: introduce {1, 2} and does not introduce {4, 8} |
| Qualcomm[24] | Proposal 5: The timing windows for UE-PRU alignment are valid in all applicable UE RRC-states, and have all the allowed periodicity values that the IEs they are defined similar to are allowed to have. - The window for UE/PRU DL measurements is applicable in RRC connected, inactive, and idle, and has the same periodicities as NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset in TS 37.355. - The window for serving gNB SRS configuration is applicable in RRC connected and inactive, and has the same periodicities as the IE PeriodicitySRS in “Requested SRS Transmission Characteristics” in TS 38.455. - The window for serving and neighbor gNB SRS measurements is applicable in RRC connected and inactive, and has the same periodicities as the IE Measurement Periodicity in MEASUREMENT REQUEST in TS 38.455 |
| Ericsson[25] | Proposal 2 Clarification: For UE-based carrier phase positioning, when LMF forwards the DL carrier phase measurement reported by a PRU to a target UE, the PRU position and the timestamp associated with the PRU carrier phase measurements should be forwarded in positioning assistance data. |

FL Comments:

In [6], it was proposed to add an indicator indicate whether the measurement within the indicated time window(s) is only for RSCP/RSCPD or also for legacy timing measurements. In FL’s understanding, the indicator might be useful if the LMF has the sufficient information to set the indicator. Another approach may be to let UE to decide it, and provide an indication on whether the UE obtains the legacy timing measurements within the time window.

In [6], it also proposed “Within the indicated measurement time window, all DL PRS resource sets should be from a single positioning frequency layer”. We do not have the agreement on whether one time window applies to all DL PFLs or only a single PFL yet. Thus, at least some clarification is needed.

For the “FFS:{4, 8}” in the agreement, [11][15] proposed to support {4, 8}, but [18][22] did not support them. The difference may come from the different understanding on whether a time window applies only to one PFL or for all PFLs. If a time window applies to all PFLs, there is no need to support more than 2 windows. However, if a window applies to one PFL only, there is a need to support more than 2 windows, some of them are used for legacy measurements.

In [24], it was proposed the concept of the timing windows applies to all RRC states. In addition, [24] also proposed the periodicities of the time windows. In FL’s view, there issues should be considered in RAN1.

In addition, a number of TPs were proposed based on the existing agreements:

- In [11], TPs were proposed to address the issue that the time windows apply not only to CP measurements, but the legacy measurements.

- In [7][16], TPs were proposed to replace “carrier phase measurements” with existing “RSCP” and “RSCPD” measurements.

- In [7][11][15][18][25], it was proposed to capture the agreement related to including the timestamp associated with the PRU carrier phase measurements in positioning assistance data.

### (H)(Round 1)Proposal 4.1-1

Adopt the following TP in Clause 5.1.6.5 of TS 38.214

| Reason for change: | The measurement time windows introduced in Rel-18 are applicable not only to RSCP/RSCPD measurements, but also other measurements. |
| --- | --- |
| Summary of change: | Moving the text that describes the measurements on indicated DL PRS resource sets occurring within one or more window(s) to the location before the text that describes the DL PRS measurements. |
| Consequences if not approved: | Readers may erroneously assume the UE only performs carrier phase measurements, but not other measurements, on indicated DL PRS resource sets occurring within one or more configured time window(s). |

#### 5.1.6.5 PRS reception procedure ===================== Unchanged parts omitted ====================== The UE may be configured with [higher layer parameter] which contains DL carrier phase measurements performed by a positioning reference unit (PRU) [20, TS 38.305] along with the location information of the PRU. The reduced capability UE may be configured to measure and report, subject to UE capability, via [higher layer parameter] the DL RSTD, DL PRS-RSRP, DL PRS-RSRPP, or UE Rx-Tx time difference using receiver frequency hopping for a DL PRS resource, with bandwidth that may be greater than the maximum reduced capability UE bandwidth, within a configured measurement gap. The reduced capability UE performing receiver frequency hopping may be configured to report via [higher layer parameter] one measurement associated with one received frequency hop or one measurement based on multiple hops of the DL PRS. [In RRC_CONNECTED mode], the reduced capability UE is expected to use a single instance of a configured measurement gap to receive all hops of the DL PRS using receiver frequency hopping. For DL UE positioning measurement reporting in higher layer parameter NR-DL-TDOA-SignalMeasurementInformation, the UE may be configured to report the DL Reference Signal Carrier Phase Difference (RSCPD) [7, TS 38.215] measurement along with the DL RSTD. When the UE reports RSCPD measurements the reference nr-DL-PRS-ReferenceInfo is the same as the one reported, for the RSTD measurements. For DL UE positioning measurement reporting in higher layer parameter NR-Multi-RTT-SignalMeasurementInformation the UE may be configured to report the DL Reference Signal Carrier Phase (RSCP) measurement [7, TS 38,215] along with the UE Rx-Tx time difference measurement. When the UE reports DL RSCPD measurement(s) along with DL RSTD measurement(s) or DL RSCP measurement(s) along with UE Rx-Tx time difference measurement(s), the DL RSCPD and/or DL RSCP measurement(s) should be measured from a single DL PRS positioning frequency layer. The UE, subject to UE capability, may be requested via [higher layer parameter] to perform positioning measurements on indicated DL PRS resource sets occurring within one or more time window(s) indicated by [higher layer parameter]. The UE is expected to obtain 1 DL RSCP or DL RSCPD measurement with Nsample=1 as defined in [11, TS 38.133]. When the UE reports a timestamp associated with a DL RSCP measurement or a DL RSCPD measurement, subject to UE capability, it may include a symbol index in the timestamp. If the UE reports LoS/NLoS indicator(s) via higher layer parameter nr-los-nlos-Indicator along with a measurement report containing DL RSCP or DL RSCPD the LoS/NLoS indicator(s) are assumed to also apply to the DL RSCP or DL RSCPD measurements. The UE may be configured with [higher layer parameter] which contains DL carrier phase measurements performed by a positioning reference unit (PRU) [20, TS 38.305] along with the location information of the PRU. ===================== Unchanged parts omitted ======================

| Company | comments |
| --- | --- |
| Xiaomi | Prefer proposal 4.1-2 FL: 4.1-2 addresses a different issue. |
| InterDigital | Support |
| ZTE | Support. The measurement window is no longer limited to RSCP/RSCPD only, which can be applicable to other measurements as well. The text is updated accordingly. And the last 5 paragraphs are edited and dedicated for carrier phase positioning, while the deleted part is not dedicated for carrier phase positioning. Our suggestion is move this part to the beginning, to confirm this measurement window is not designed only for carrier phase measurement. |
| Samsung1 | Change not essential. |
| Qualcomm | The measurement windows are not only for RSCP/RSCPD so some change is needed to make sure 38.214. We are Ok with this change, but if there are others similar suggestions, we can consider these also. However, we consider it essential to make sure that the time windows are applicable to all measurements. |
| LGE | We think it is not essential, but ok with majority view. |
| CATT | OK |
| Ericsson | Change is not essential. |
| Huawei, HiSilicon | In general, we would prefer to have a separate subclause for CPP/BW aggregation/PRS Rx hopping. The current 5.1.6.5 is hard to read. |
| Nokia/NSB | OK |

(H)(Round 1)Proposal 4.1-2

Adopt the following TP in Clause 5.1.6.5 of TS 38.214.

| Reason for change: | - The following agreement made in RAN1#114 meeting is not captured in the current TS 38.214. Agreement For UE-based carrier phase positioning, when LMF forwards the DL carrier phase measurement reported by a PRU to a target UE, the timestamp associated with the PRU carrier phase measurements should also be forwarded in positioning assistance data. - The terminology "DL carrier phase measurements", which is used in the specification, is undefined. |
| --- | --- |
| Summary of change: | - Capture the above agreement that the timestamp associated with the PRU carrier phase measurements should also be forwarded in positioning assistance data. - Change “DL carrier phase measurements” to “DL RSCP/DL RSCPD measurements” which are already defined in TS 38.215. |
| Consequences if not approved: | Specification TS 38.214 is incomplete and includes undefined term of “carrier phase measurements”. |

#### 5.1.6.5 PRS reception procedure ===================== Unchanged parts omitted ====================== The UE may be configured with [higher layer parameter], which contains DL RSCP/RSCPD measurements performed by a positioning reference unit (PRU) [20, TS 38.305], the timestamp associated with the DL RSCP/RSCPD measurements, and the location information of the PRU. ===================== Unchanged parts omitted ======================

| Company | comments |
| --- | --- |
| Xiaomi | Support |
| InterDigital | Support |
| ZTE | Support |
| Samsung1 | OK |
| Qualcomm | Wouldn’t the legacy measurements associated with he RSCP/RSCP be needed? FL We may consider a separate proposal for the legacy measurements. |
| LGE | Ok |
| CATT | OK |
| Ericsson | ok |
| Huawei, HiSilicon | Since Multi-RTT does not support UE-based positioning, why would LMF provide RSCP measurement to the target UE? For the time stamp, it will appear in RAN2 spec anyway, why bother mentioning that in RAN1, and we do not even specify how this time stamp is used in physical layer. |
| FL | During online discussion, there were comments that why the legacy measurements from PRU are not included. The reason is that TP was written based on the existing RAN1 agreements. But, RAN1 does not have the agreement to forward the legacy measurements from PRU to target UE yet. We can try to include them to see if there is any concern. |

- Proposal:

- Adopt the following changes to the previous agreement:

Agreement

For UE-based carrier phase positioning, when LMF forwards the DL RSCPD measurement and RSTD measurements reported by a PRU to a target UE, the timestamp associated with the PRU carrier phase measurements should also be forwarded in positioning assistance data.

(H)(Round 2)Proposal 4.1-2

Adopt the following TP in Clause 5.1.6.5 of TS 38.214.

| Reason for change: | - The following agreement made in RAN1#114 meeting is not captured in the current TS 38.214. Agreement For UE-based carrier phase positioning, when LMF forwards the DL carrier phase measurement reported by a PRU to a target UE, the timestamp associated with the PRU carrier phase measurements should also be forwarded in positioning assistance data. - The terminology "DL carrier phase measurements", which is used in the specification, is undefined. |
| --- | --- |
| Summary of change: | - Capture the above agreement that the timestamp associated with the PRU carrier phase measurements should also be forwarded in positioning assistance data. - Change “DL carrier phase measurements” to “DL RSCP/DL RSCPD measurements” which are already defined in TS 38.215. - Include the UE Rx-Tx time difference/RSTD measurements from PRU to target UE |
| Consequences if not approved: | Specification TS 38.214 is incomplete and includes undefined term of “carrier phase measurements”. |

#### 5.1.6.5 PRS reception procedure ===================== Unchanged parts omitted ====================== The UE may be configured/provided with [higher layer parameter], which contains DL RSCPD measurements and/or RSTD measurements performed by a positioning reference unit (PRU) [20, TS 38.305], the timestamps associated with the measurements, and the location information of the PRU. ===================== Unchanged parts omitted ======================

| Company | comments |
| --- | --- |
| ZTE | Prefer the previous version. As FL mentioned, this version is not aligned with current agreement. |
| Samsung2 | Don’t support. This goes beyond the agreement. |
| InterDigital | In our view, inclusion of timing measurements is aligned with the agreements. The UE (or PRU) reports RSTD and RSCPD together. Therefore, the forwarded measurements should also include the PRU measurements, i.e., timing and phase measurements. |

### (H)(Round 3)Proposal 4.1-2b

Offline Consensus

Adopt the following TP in Clause 5.1.6.5 of TS 38.214.

| Reason for change: | - The following agreement made in RAN1#114 meeting is not captured in the current TS 38.214. Agreement For UE-based carrier phase positioning, when LMF forwards the DL carrier phase measurement reported by a PRU to a target UE, the timestamp associated with the PRU carrier phase measurements should also be forwarded in positioning assistance data. - The terminology "DL carrier phase measurements", which is used in the specification, is undefined. |
| --- | --- |
| Summary of change: | - Capture the above agreement that the timestamp associated with the PRU carrier phase measurements should also be forwarded in positioning assistance data. - Change “DL carrier phase measurements” to “DL RSCP/DL RSCPD measurements” which are already defined in TS 38.215. - Include the UE Rx-Tx time difference/RSTD measurements from PRU to target UE |
| Consequences if not approved: | Specification TS 38.214 is incomplete and includes undefined term of “carrier phase measurements”. |

#### 5.1.6.5 PRS reception procedure ===================== Unchanged parts omitted ====================== The UE may be configured provided with [higher layer parameter], which contains DL carrier phase measurements performed by a positioning reference unit (PRU) [20, TS 38.305], the timestamps associated with the measurements, and along with the location information of the PRU. ===================== Unchanged parts omitted ======================

| Company | comments |
| --- | --- |

### (Closed)(Round 1)Proposal 4.1-3

- All of the timing windows, which were agreed to be introduced for simultaneous PRS measurements and SRS transmission in Rel-18, are applicable for UE in RRC_CONNECTED, RRC_INNACTIVE, and RRC_IDLE states;

- The allowed periodicity values of the time windows for DL measurements are the same as periodicity values of NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset in TS 37.355.

- The allowed periodicity values of the time windows for serving gNB SRS configuration are the same as periodicity values of IE PeriodicitySRS in “Requested SRS Transmission Characteristics” in TS 38.455.

- The allowed periodicity values of the time windows for serving and neighbor gNB SRS measurements are the same as periodicity values of IE Measurement Periodicity in MEASUREMENT REQUEST in TS 38.455.

| Company | comments |
| --- | --- |
| vivo | We think we have reached a related agreement about periodicity about windows, so we wonder about the intention of the proposal FL: When the time window were introduced, we have not make it clear that it applies also for RRC states. |
| ZTE | Share similar view with vivo. The periodicity IE is not confirmed in previous agreement. FL: My understanding for vivo’s proposal is that the periodicities are covered in the previous agreement. |
| Samsung1 | Intention of proposal not clear. |
| LGE | Similar view with vivo. |
| CATT | Periodicities are covered already by exiting agreements. |
| Huawei, HiSIlicon | We would suggest to clarify this discussion only facilitates the discussion of higher layer parameters, rather than seeks RAN1 spec change. |
| FL | Agreement The timing windows, which were agreed to be introduced for simultaneous PRS measurements in Rel-18, are applicable for UE in RRC_CONNECTED, RRC_INACTIVE, and RRC_IDLE states. Note: no RAN1 specification impact. Agreement The timing windows, which were agreed to be introduced for simultaneous SRS for positioning transmission in Rel-18, are applicable for UE in RRC_CONNECTED and RRC_INACTIVE states. Note: no RAN1 specification impact. |

(H)(Round 1) Proposal 4.1-4

The number of the time windows for an LMF to request a UE, which can be a target UE or a PRU, to perform measurements on indicated DL PRS resource set(s) within the indicated time windows, can also be {4, 8}.

| Company | comments |
| --- | --- |
| Xiaomi | Does it mean a time window can only be configured per PFL? FL: Different PFL may have different DL PRS configurations, e.g., different periodicities. Thus, the time window should be per PFL instead of all PFLs. |
| InterDigital | We don’t support this proposal. Justification for having 4 or 8 is not clear. FL: If time window is per PFL, then 2 is not enough for 4 PFLs. |
| ZTE | Support. |
| CATT | Support |
| FL | From the feedback, we may need to clarify the time windows |
| Huawei, HiSilicon | {1, 2} could be enough |
| FL | ### (H)(Round 1) Proposal 4.1-4 (Revised) When an LMF requests the UEs, including target UE and PRU(s), to perform measurements on indicated DL PRS resource set(s) occurring within an indicated time window, the number of the indicated DL PRS resource sets can be up to 2 per DL PFL. |

### (H)(Round 2) Proposal 4.1-4

Offline Consensus

Adopt the following changes to the previous agreement made in RAN1#114:

Agreement When an LMF requests the UEs, including target UE and PRU(s), to perform measurements on indicated DL PRS resource set(s) occurring within indicated time window(s) - The duration of a time window can be configured as follows: - {1, 2, 4, 6, 8, 12, 16} slots. - the number of the time windows can be: - {1, 2} - FFS: {4, 8} - the number of the indicated DL PRS resource set(s) per TRP within a time windows can be {1, 2}: - DL PRS resource sets across all TRPs are in one DL PFL - The number of the indicated DL PRS resource set(s) for all TRPs should be the same - Note: Different PRS resource sets and/or PFLs can be associated with different time windows - Note: It is up to RAN2/RAN3 in the signaling design for the indication of the DL PRS resource sets in the time windows.

| Company | comments |
| --- | --- |

## Assistance information for UE-based CPP

Agreement For UE-based carrier phase positioning, support enabling LMF to forward the DL carrier phase measurement reported by a PRU, with additional information of the same PRU to a target UE for UE-based carrier phase positioning in the positioning assistance data. - Note: Whether the forwarded DL carrier phase measurement is DL RSCP and/or DL RSCPD depends at least on which of them is (are) supported by UE capability. - additional information of the same PRU includes at least PRU location. - FFS: additional PRU information, e.g. the AoD of PRU to each TRP, etc.

Submitted Proposals:

| xiaomi[14] |  | - Proposal 2: Support LMF to provide the differential corrections to a target UE for UE-based carrier phase positioning. |
| --- | --- | --- |
| InterDigital[18] | Proposal 5: Adopt the following TP to enable PRU measurement forwarding to the UE by the LMF based on the request from the UE. The reason for the change is to allow the UE to request PRU measurement forwarding. The summary of change is to introduce a request for the LMF by the UE to forward PRU measurements. The consequence if not approved is that the UE cannot complete UE-based positioning using forwarded PRU measurements. --------------------------<Start of text proposal for TS 38.214>-------------------------- #### 5.1.6.5 PRS reception procedure < Unchanged parts are omitted > The UE may be configured with [higher layer parameter] which contains DL carrier phase measurements performed by a positioning reference unit (PRU) [20, TS 38.305] along with the location information of the PRU. The UE may request the network to receive measurements performed by a PRU. < Unchanged parts are omitted > --------------------------<End of text proposal for TS 38.214>--------------------------. Proposal 6: Adopt the following TP to clarify that the forwarded measurements contain both carrier phase measurements and associated timing measurements. The reason for the change is to introduce clarity in the types of PRU measurements forwarded by the LMF. The summary of change is to clarify that both timing and phase measurements made by the PRU are forwarded by the LMF. The consequence if not approved is that the UE may not be able to determine its location based on forwarded PRU measurements. --------------------------<Start of text proposal for TS 38.214>-------------------------- #### 5.1.6.5 PRS reception procedure < Unchanged parts are omitted > The UE may be configured with [higher layer parameter] which contains DL carrier phase measurements and associated DL RSTD measurements performed by a positioning reference unit (PRU) [20, TS 38.305] along with the location information of the PRU. < Unchanged parts are omitted > --------------------------<End of text proposal for TS 38.214>--------------------------. |
| Lenovo[21] | Proposal 5: In the case of UE-assisted CPP, in order to mitigate the CFO effect of RSCP and RSCPD measurements at UE and PRU, consider the following approaches: - Approach 1: The CFOs should be measured by both the UE and the PRU for each TRP for which measurements are taken, and these CFOs should be reported to the LMF. In addition, the number of symbols separating the RSCP and RSCPD measurements made at UE and PRU should be reported. - Approach 2: The UE and PRU UE are provided with a common reference time in order to perform RSCP and RSCPD measurement with respect to the common reference time applied at both UE and PRU. In this case no reporting of CFO is required by UE and/or PRU UE as the effect of CFO is removed. |  |
| Ericsson[25] | Proposal 1 LMF should not forward DL RSCP measurements of a PRU to a Target UE since differentiation will not be applied to DL RSCP measurements. |  |

FL Comments:

In [14], it was proposed to support LMF to provide the differential corrections to a target UE for UE-based carrier phase positioning. Similar proposal was discussed in previous meetings w/o conclusion. Considering that the WI has entered the maintenance phase, in FL view, we may not have the time to further discuss the issue.

In [18], it was proposed “The UE may request the network to receive measurements performed by a PRU.” For a UE to make the request, the FL assume there is a need of a procedure for UE to first get the information of the surrounding PRU. However, the procedure was not discussed in RAN1 before.

In [18], it was proposed the “associated DL RSTD measurements” are included in assistance data. Considering that the benefits of providing DL RSTD measurements have not evaluated in RAN1, further discussion of the proposal is obviously needed.

In [21], two approaches were proposed for mitigate the CFO effect.

In [26], it was proposed not forwarding DL RSCP measurements of a PRU to a Target. In FL’s understanding, if a PRU provides RSCP to LMF, the LMF may either forward the RSCP measurements to UE based on existing agreement, or convert RSCP to RSCPD, which means the LMF needs to convert DL RSCP measurements of a PRU into DL RSCPD measurements before forwarding it to the UE.

The FL would like to first get the inputs from interested companies to see if we should consider any of these proposals as high-priority for this meeting..

### (Closed) Questions

Interested companies are invited to provide their views on the following proposals related to the assistance information for CPP:

- P1: Proposal 2 from xiaomi[14]

  - Support further discussion: ZTE

- P2: Proposal 5 from InterDigital [18]

  - Support further discussion: IDC

- P3: Proposal 6 from InterDigital [18]

  - Support further discussion: vivo, IDC, Qualcomm

- P4: Proposal 5 from Lenovo [21]

  - Support further discussion: ZTE

- P5: Proposal 1 from Ericsson [25]

  - Support further discussion: vivo, ZTE

| Company | comments |
| --- | --- |
| vivo | We are okay to further discuss P3 and P5 |
| InterDigital | Regarding P2, we disagree with the FL’s view. The UE does not need to know whether there are PRUs around the UE to make the request. The UE can request for PRU measurements (without knowing if there are PRUs around the UE) and if the PRU measurements are not available, the UE will not obtain them from the network. Regarding P3, we have the following agreement from RAN1#113. Does it not apply to PRU measurements? The forwarded PRU measurements should contain both RSTD and RSCPD. Agreement If a UE reports RSCPD measurements together with RSTD measurements in a measurement report element, the reference TRP for RSCPD is the same as the reference TRP reported for RSTD. - The target and the reference TRP are in the same PFL |
| ZTE | We are open to discuss P1, P4 and P5. For P5, we believe it is beneficial for LMF to forward the measurement result of PRU. |
| Samsung1 | These issues seem to be non-essential, so we prefer not to discuss during maintenance. |
| c | We support P3 (proposal 6 from Interdigital) |
| FL | Only a few companies provided the feedback. Let us wait for more input to decide whether we should further discuss the proposal in this meeting. |
| FL | Given the situation that only a few companies provided the feedback, and there are many high-priority issues that we need to resolve, the FL suggest closing the discussion of the proposal in this meeting. |

# Integer Ambiguity

Background

Agreement (RAN1#112bis-e) To support NR carrier phase positioning, further consider the following options: - Option 1: Support a UE/TRP to report the carrier phase measurements of more than one frequency within a PFL/carrier to LMF - NOTE: the frequency can be the carrier frequency or the frequency of a subcarrier - FFS: the details of reporting, e.g., the maximum number of reported frequencies within a PFL/ carrier - Option 2: Introduce and report a new type of UE/TRP measurement based on carrier phase differentials across multiple subcarriers within a PFL/carrier - NOTE: carrier phase differentials across multiple subcarriers within a carrier can be related to time of arrival - Option 3: Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF - Option 4: Support LMF to provide the expected integer ambiguity range at least for UE-based NR CPP in the positioning assistance data. Agreement (RAN1#112bis-e) The specific RF frequency associated with a UL carrier phase measurement is defined, by default, as the center frequency of the transmission bandwidth of the SRS for positioning purpose. - Note: It is open to further discussion whether a frequency other than the center frequency of the UL carrier can also be the specific RF frequency for a non-default case(s), if RAN1 agrees to introduce them.

Submitted Proposals:

| Nokia, NSB[7] | Proposal 13: Support the LMF to provide expected integer ambiguity ranges for the UE-based NR CPP in the positioning assistance data. |
| --- | --- |
| Spreadtrum[8] | Proposal 2: The range or some alternative values of integer N should be provided/reported for carrier phase positioning. - Adopt the following TP for section 5.1.6.5 in TS 38.214. Reason for change; The value of the fractional part can only reach one wavelength at most, so the accuracy of the final positioning result greatly depends on the determination of the integer part. In order to get the final integer part, UE or LMF needs to search integer value N to get the optimal solution. In order to obtain accurate results, large search range of integer values N will be considered. For UE-assisted positioning, the UE can report a recommended search range or some alternative values to TRP or LMF. Hence, we have the following proposal.. Summary of change; Section 5.1.6.5 in 38.214: clarify the UE can report a recommended search range or some alternative values to TRP or LMF.. Consequences if not approved; Large search range of integer values N will be considered. In this case, huge complexity is brought to UE or LMF.. Text proposal; TS 38.214 5.1.6.5 PRS reception procedure ************** Unchanged parts omitted************** If the UE reports LoS/NLoS indicator(s) via higher layer parameter nr-los-nlos-Indicator along with a measurement report containing DL RSCP or DL RSCPD the LoS/NLoS indicator(s) are assumed to also apply to the DL RSCP or DL RSCPD measurements. The UE may be configured to report the candidate values of integer N along with DL RSCP or DL RSCPD measurements. The UE may be configured with [higher layer parameter] which contains DL carrier phase measurements performed by a positioning reference unit (PRU) [20, TS 38.305] along with the location information of the PRU. ************** Unchanged parts omitted**************. |
| Intel[10] | Proposal 5 - Subject to UE and gNB capabilities respectively, use of finer granularity (e.g., 0.5Tc and 0.25Tc) for reporting of RSTD, RTOA and UE/gNB (respectively) Rx-Tx time difference should be available regardless of the capability and configuration of bandwidth aggregation in DL and UL respectively. Proposal 6 For integer ambiguity resolution (IAR), the following options are supported for reporting RSCP/RSCPD with legacy positioning metrics, - (Option 3 variant) Subject to UE’s capability, when CP measurements are reported along with legacy timing-based positioning measurements, the legacy measurements may be reported with finer granularity (e.g., 0.50 or 0.25 Tc) and to serve as estimate of the IA. - (Option 4 variant) As part of assistance data, support LMF to indicate estimate of pseudo-range via provision of: - expected RSTD and associated uncertainty information to a UE using finer granularity (e.g., << 4*Ts) for a given TRP relative to a reference TRP. - expected propagation delay and associated delay uncertainty to a gNB using finer granularity (e.g., << 4*Ts) for a given SRS relative to the UL RTOA reference time |
| ZTE[11] | Proposal 1: Support option 1: Subject to UE’s capability, support LMF to request a UE, to report the carrier phase measurements corresponding to M RF frequencies within a DL PFL. Proposal 2: Support option 3 or option 5: - Option 3: - Subject to UE’s capability, support enabling a UE to optionally report an estimated search range, [-N1, +N1], of the integer ambiguity to LMF. - Support enabling a gNB to optionally report an estimated absolute search range, N2, of the integer ambiguity for each carrier phase measurement to the LMF. - FFS: N1, N2 - Option 5: - Subject to UE’s capability, support enabling a UE to optionally report estimated search range(s) of the integer ambiguity(ies) for DL carrier phase measurements to LMF. - Support enabling a gNB to optionally report an estimated search range(s) of the integer ambiguity(ies) for UL carrier phase measurements to the LMF. - FFS: how the estimated search range(s) are defined Proposal 3: Support option 4: - Support LMF to optionally provide the estimated search range of the integer ambiguity, M, to a target UE in the positioning assistance data for UE-based carrier phase positioning. - M= [0, 1, 2, …, 7] |
| LGE[12] | Proposal 7: To guarantee simultaneous RSCP/RSCPD measurement of PRU and a UE using same RF frequency, supports - PRU can be configured to measure and report RSCP/RSCPD for multiple PFLs. - PRU can be configured to measure and report RSCP/RSCPD for multiple RF frequency within single PFL. |
| Samsung[13] | Proposal 1: For carrier phase positioning measurement, a UE or gNB can be configured to provide a measurement based on the carrier phase of multiple frequencies of DL PRS or UL positioning SRS. Text Proposal 1: (clause 5.1.42 of TS 38.215) DL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements carrying DL PRS configured for the measurement. If multiple frequencies are configured for reporting DL RSCP, the DL RSCP is associated with the configured frequencies, otherwise DL RSCP is associated with the center frequency of the DL positioning frequency layer (PFL) configured for the measurement. For frequency range 1, the reference point for the DL RSCP shall be the antenna connector of the UE. For frequency range 2, the reference point for the DL RSCP shall be the antenna of the UE. Text proposal 2: (clause 5.2.8 of TS 38.215): UL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements carrying sounding reference signals (SRS) configured for the measurement. If multiple frequencies are configured for reporting UL RSCP, the UL RSCP is associated with the configured frequencies, otherwise UL RSCP is associated with the center frequency of the transmission bandwidth of the SRS for positioning purposes configured for the measurement. The reference point for UL RSCP shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Rx antenna (i.e., the centre location of the radiating region of the Rx antenna), - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. |
| xiaomi[14] | - Proposal 1: Support positioning report to contain the carrier phase of multiple sub-carriers. |
| CATT[15] | Proposal 6: Support finer granularity(ies) for Rel-16 timing measurements and corresponding quality indicator (ReportingGranularityfactor k = {-1, -2}) when they are reported together with RSCP/RSCPD measurements. |
| CMCC[17] | Proposal 1: Do not further consider integer ambiguity resolution in the maintenance phase. |
| Apple[19] | Proposal 4: Support to optionally provide an estimated integer ambiguity and/or the search range of the integer ambiguity to a target UE in the positioning assistance data for UE-based carrier phase positioning. For UEs this should be a capability. |
| Lenovo[21] | Proposal 2: Support the measurement of subcarrier phase measurements at UE/gNB. FFS the details of exploiting the subcarrier phases, e.g., use of subcarrier phase differentials, number of considered subcarriers, etc. Proposal 3: RAN1 to further discuss which of the PRS PRBs should be used for performing subcarrier/carrier phase measurements in relation to other signals/channels. Proposal 4: RAN1 to support the following Options from the RAN1#112 meeting: - Option 3: Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF - Option 4: Support LMF to provide the expected integer ambiguity range at least for UE-based NR CPP in the positioning assistance data. |
| Qualcomm[24] | Proposal 4: Do not support reporting of carrier phase measurements for more than one frequency within a PFL/carrier. |

FL Comments:

In RAN1#112, RAN1 reached the agreement to further consider the four options of integer ambiguity resolution for NR CPP. Since RAN1#112, these options have been discussed intensively in each of the meetings, but unfortunately no conclusion was reached. Companies views for this meeting may be summarised as follows:

- Option 1: Support a UE/TRP to report the carrier phase measurements of more than one frequency within a PFL/carrier to LMF

  - Support: ZTE, LGE, Samsung, xiaomi, Lenovo

  - Not Support: Qualcomm

- Option 2: Introduce and report a new type of UE/TRP measurement based on carrier phase differentials across multiple subcarriers within a PFL/carrier

  - Supported by:

  - Not Support:

- Option 3: Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF

  - Support: Spreadtrum, Intel (variant), ZTE, Lenovo

  - Not Support:

- Option 4: Support LMF to provide the expected integer ambiguity range at least for UE-based NR CPP in the positioning assistance data.

  - Supported by: Nokia, NSB, Intel (variant), ZTE, Apple, Lenovo

  - Not Support:

Most of the companies have keep their views as the last meeting. Many companies, which provided their positions on above options in the previous meeting, did not provide their opinions in the contributions to this meeting, maybe it is because the WI has entered the maintenance phase. In addition, some companies suggested no further consider integer ambiguity resolution in the maintenance phase.

During the offline discussion in RAN1#114, it seems most of the companies were fine with Option 3 and Option 4. In FL’s view, for Option 3, we may need to clarify what the search range really means, e.g., it applies to which measurements, i.e., DL RSCP or RSCPD, and whether one search range applies to all measurements, or one search range applies only to some of the measurements, etc. Similarly, for Option 4, it may also need to clarify the search range is valid for which of the measurements, i.e., some of the carrier phase measurements from PRU, or all of the carrier phase measurements from PRU.

In addition, it was proposed in [10][15] to support finer granularity (e.g., 0.5Tc and 0.25Tc) for reporting of RSTD, RTOA and UE/gNB Rx-Tx time difference. The granularity of existing timing measurements is Tc or larger, which is normally a few times of the wavelength of the carrier, which has direct impact on the effectiveness of Option 3 and Option 4, since the expected integer ambiguity ranges of the Option 3 and Option 4 are expected to be around the reported timing measurements.

In FL’s view, it is highly desirable to have at least some of the options to be agreed in Rel-18. Instead of putting all of the options into one proposal, maybe we can have separate discussion to see if any of them can be agreed in this meeting.

### (Closed)(Round 1)Proposal 5-1

Subject to UE’s capability, support LMF to request a UE, to report the RSCP/RSCPD measurements corresponding to M RF frequencies within a DL PFL.

- M = [2, 3]

| Company | comments |
| --- | --- |
| vivo | We prefer not to pursue this issue in the maintenance stage |
| Xiaomi | Support |
| ZTE | Support. |
| Spreadtrum | Support. |
| Samsung1 | Support |
| Qualcomm | Do not pursue further this issue during maintenance |
| LGE | As we commented in our document, M frequency reporting by PRU may be required at least, if RF frequency other than center frequency is supported for UE in RRC_INACTIVE/RRC_IDLE state. |
| CATT | Support. |
| Ericsson | Do not support discussing this issue during maintenance. |
| FL | Conclusion From RAN1’s perspective, there will be no further discussion on the four options that were agreed to consider in the following agreement made in RAN1#112bis-e. |

### (Closed)(Round 1)Proposal 5-2

Subject to UE’s capability, support LMF to request a UE to report a mean value of carrier phase differentials across N subcarriers within a PFL.

- N = [2, 4, 6, 8, 16, 32, 64]

| Company | comments |
| --- | --- |
| vivo | We prefer not to pursue this issue in the maintenance stage |
| Spreadtrum | We don’t the benefits using the mean value. So we think this proposal is not needed. |
| Samsung1 | OK |
| Qualcomm | We don’t see the benefit |
| LGE | We prefer to have this reporting, but it seems hard to discuss further at the maintenance stage. |
| CATT | Support. |
| Ericsson | Do not support discussing this during maintenance. |
| Nokia/NSB | Do not support |
| FL | Conclusion From RAN1’s perspective, there will be no further discussion on the four options that were agreed to consider in the following agreement made in RAN1#112bis-e. |

### (Closed)(Round 1)Proposal 5-3

Subject to UE's capability, support enabling a UE to optionally provides an estimated search range [-N1, +N1] of the integer ambiguity for DL RSCP/RSCPD measurement(s) reported to the LMF

- N1 = {0, 1, 2, 4}

- FFS: N1 is provided for all DL RSCP/RSCPD measurement(s) or there are different search ranges for RSCP/RSCPD measurement(s) measured from different TRPs

Support enabling a gNB to optionally report an estimated search range [-N2, +N2] of the integer ambiguity for UL RSCP to LMF, where

- N2 = {0, 1, 2, 4}

- FFS: N2 is provided for all UL RSCP measurement(s) or there are different search ranges for UL RSCPs obtained from different UL SRS resources

| Company | comments |
| --- | --- |
| Xiaomi | As for the value of N1 and N2, in our opinion, N1 can be larger than N2. As for the FFS, prefer different search ranges for RSCP/RSCPD measurement(s) measured from different TRPs |
| InterDigital | Support |
| ZTE | Support. |
| Spreadtrum | Support. |
| Samsung1 | Don’t support |
| LGE | It seems not necessary. In our understanding, the estimated search range would be useful for relaxing LMF complexity but not so useful for improving positioning accuracy. |
| CATT | Support. |
| Ericsson | Do not support discussing this during maintenance. |
| Huawei, HiSilicon | We do not see the usefulness of this. For FR1 with 3.5GHz, the RF periodicity is 300ps, the search range provided will be up to +/-1.2ns, but the ToA reporting currently is 2ns (4Tc for FR1). |
| Nokia/NSB | Do not support |
| FL | Conclusion From RAN1’s perspective, there will be no further discussion on the four options that were agreed to consider in the following agreement made in RAN1#112bis-e. |

### (Closed)(Round 1)Proposal 5-4

Support LMF to optionally report an estimated search range [-N3, +N3] of the integer ambiguity for DL RSCP/RSCPD to UE for UE-based carrier phase positioning

- N3 = {0, 1, 2, 4}

- FFS: N3 is provided for all DL RSCP/RSCPD measurement(s), or there are different search ranges for DL RSCP/RSCPD measurement(s) measured from different TRPs

| Company | comments |
| --- | --- |
| Xiaomi | Same comments as 5-3 on FFS |
| InterDigital | Support |
| ZTE | Support. |
| Spreadtrum | Support. |
| Samsung1 | Don’t support |
| LGE | In case of UE based positioning, estimated integer value by LMF may be outdated and UE may have better understanding about that. So, we do not think it is beneficial. |
| CATT | Support. |
| Ericsson | Do not support discussing this during maintenance. |
| Huawei, HiSilicon | We do not support it. How can LMF know the range? |
| Nokia/NSB | OK |
| FL | Conclusion From RAN1’s perspective, there will be no further discussion on the four options that were agreed to consider in the following agreement made in RAN1#112bis-e. |

(H)(Round 1)Proposal 5-5

Subject to UE capabilities, support a UE to use the granularity {0.5Tc, 0.25Tc} for the reporting of RSTD/UE Rx-Tx time difference for NR CPP.

Support a gNB to use the granularity {0.5Tc, 0.25Tc} for the reporting of RTOA/gNB Rx-Tx time difference for NR CPP.

| Company | comments |
| --- | --- |
| InterDigital | Support |
| ZTE | The granularity can be determined by RAN4. FL: It might reducing RAN4’s effort if RAN1 can made the decision and let RAN4 to check or confirm it. |
| Samsung1 | Prefer to let RAN4 decide on this. |
| Qualcomm | We are supportive. Note that we already specified smaller K, even down to -6, for PRS BW aggregation. I don’t see why It will be a problem when CPP measurements are included. |
| LGE | Fine in principle, but tend to agree with ZTE’s view. |
| CATT | Support. |
| Ericsson | This can be left to RAN4 to decide. |
| Huawei, HiSilicon | Reply QC: For BW aggregation, it cannot be useful for CPP, because it is also clear how UE should report the RSCPD measurement for PRS configured with BW aggregation, .e.g. which center frequency should be used. In addition, if the smaller values introduced in PRS BW aggregation can also be used for non-aggregated case, we should prefer to ask for confirmation in RAN4. |
| FL | To reduce RAN4’s load, it might be better for RAN1 to made t he decision and ask RAN4 for confirmation.. |

(H)(Round 2)Proposal 5-5

- Subject to UE capabilities, support a UE to use the granularity {0.5Tc, 0.25Tc} for the reporting of RSTD/UE Rx-Tx time difference for NR CPP.

- Support a gNB to use the granularity {0.5Tc, 0.25Tc} for the reporting of RTOA/gNB Rx-Tx time difference for NR CPP.

- Send an LS to RAN4, asking RAN4 to confirm RAN1’s decision and provide the feedback if RAN4 has any concern.

| Company | comments |
| --- | --- |
| ZTE | Thanks for the reply. Still prefer let RAN4 to decide the granularity. If needed, this can be discussed after other problems are solved in CPP. |
| Samsung2 | Don’t support. This can be handled by RAN4. |
| FL | To ZTE/Samsung: RAN4 may not be aware of the purpose of supporting finer granularity for CPP. However about RAN1 sends to request to RAN4, asking RAN4 to consider supporting finer granularity for CPP? ### (H)(Round 2)Proposal 5-5 (Revised) Adopt one of the following options (for down-selection): Option 1: - Subject to UE capabilities, support a UE to use the granularity {0.5Tc, 0.25Tc} for the reporting of RSTD/UE Rx-Tx time difference for NR CPP. - Support a gNB to use the granularity {0.5Tc, 0.25Tc} for the reporting of RTOA/gNB Rx-Tx time difference for NR CPP. - Send an LS to RAN4, asking RAN4 to confirm RAN1’s decision and provide the feedback if RAN4 has any concern. Option 2: - Send an LS to RAN4, asking RAN4 to consider the use of finer granularity(ties) (e.g., 0.5Tc, 0.25Tc) for the reporting of RSTD/UE Rx-Tx time difference for NR CPP. |

### (H)(Round 3)Proposal 5-5

Adopt one of the following options (down-selection):

- Option 1:

  - Subject to UE capabilities, support a UE to use the granularity {0.5Tc, 0.25Tc} for the reporting of RSTD/UE Rx-Tx time difference for NR CPP.

  - Support a gNB to use the granularity {0.5Tc, 0.25Tc} for the reporting of RTOA/gNB Rx-Tx time difference for NR CPP.

  - Send an LS to RAN4, asking RAN4 to confirm RAN1’s decision and provide the feedback if RAN4 has any concern.

- Option 2:

  - Send an LS to RAN4, asking RAN4 whether it is feasible to a UE to support finer granularity(ties) (e.g., 0.5Tc, 0.25Tc) in the reporting of RSTD/UE Rx-Tx time difference for NR CPP, and if yes, provide the finer granularity(ties) to RAN1.

| Company | comments |
| --- | --- |

# Phase error group

Background

Agreement (RAN1#112bis-e) To address the impact of the phase delays on Tx/Rx RF chains, support one or more of the following options (down-selection in RAN1#113): - Option 1a: introduce the definition of UE/TRP Tx/Rx phase error groups (PEGs) for the Tx/Rx of DL PRS/UL SRS signals - Rel-17 definitions of UE/TRP Tx/Rx TEGs can be used as the starting point for defining UE/TRP Tx/Rx PEGs. - FFS: the details of \the UE/TRP Tx/Rx PEGs - Option 1b: Introduce Tx/Rx RF antenna IDs or Tx/Rx RF chain IDs to identify the individual Tx/Rx RF chains for transmitting/receiving the DL PRS/UL SRS signals. - FFS: the details of the Tx/Rx RF antenna IDs or Tx/Rx RF chain IDs - Note: Device transmitting PRS or positioning SRS provides Tx antenna ID or Tx Chain ID. Device receiving PRS or positioning SRS provides Rx antenna ID or Rx Chain ID. - Option 1c: introduce the report of ARP ID for the Rx/Tx of DL PRS/UL SRS signals. - The transmission/reception associated with the same ARP ID is assumed from the same ARP. - FFS: the maximum number of ARP IDs. - Option 2: reuse or enhance the existing Rel-17 definitions of UE/TRP Tx/Rx TEGs with smaller margin value. - Option 3: RAN1 sends an LS to RAN4, requesting RAN4 to consider whether there is a need to define the new UE/TRP Tx/Rx phase error groups (PEGs), introduce new IDs (e.g., Tx/Rx RF antenna IDs ) to present the phase delays for the Tx/Rx of DL PRS/UL SRS signals, or reuse or enhance the existing Rel-17 definitions of UE/TRP Tx/Rx TEGs with smaller margin value, and provide the definitions if RAN4 decides it is needed.

Submitted Proposals:

| Nokia, NSB[7] | Proposal 1: RAN1 supports the following text proposal on Clause 6.2.1.4 of TS 38.214. <omitted text> The UE may be configured with SRS, via [SRS-PosRRC-InactiveConfig-ValidityArea], subject to UE capability, valid in multiple cells within a validity area for RRC_INACTIVE mode. For the configured SRS via [SRS-PosRRC-InactiveConfig-ValidityArea], if the UE in RRC_INACTIVE mode determines that the UE is not able to accurately measure the configured DL RS in [SRS-SpatialRelationInfoPos] for a SRS resource for positioning where the DL RS is semi-persistent or periodic, the UE would not perform SRS transmission of the SRS resource for positioning. If the UE determines that the configured DL RS in [SRS-SpatialRelationInfoPos] for a SRS resource for positioning is being accurately measured, the UE is expected to perform the SRS transmission. The UE and PRU may request to be provided with the necessary SRS configuration(s) that keep(s) the phase error less than a specific threshold value of the phase error. The UE and PRU, subject to its capability, may be requested to report the phase error threshold value(s) associated with specific SRS configuration(s) used for specific SRS transmission(s). <omitted text>. Proposal 2: RAN1 supports the following text proposal on Clause 5.1.6.5 of TS 38.214 <omitted text> For DL UE positioning measurement reporting in higher layer parameter NR-DL-TDOA-SignalMeasurementInformation, the UE may be configured to report the DL Reference Signal Carrier Phase Difference (RSCPD) [7, TS 38.215] measurement along with the DL RSTD. When the UE reports RSCPD measurements the reference nr-DL-PRS-ReferenceInfo is the same as the one reported, for the RSTD measurements. For DL UE positioning measurement reporting in higher layer parameter NR-Multi-RTT-SignalMeasurementInformation the UE may be configured to report the DL Reference Signal Carrier Phase (RSCP) measurement [7, TS 38,215] along with the UE Rx-Tx time difference measurement. When the UE reports DL RSCPD measurement(s) along with DL RSTD measurement(s) or DL RSCP measurement(s) along with UE Rx-Tx time difference measurement(s), the DL RSCPD and/or DL RSCP measurement(s) should be measured from a single DL PRS positioning frequency layer. When the UE or the PRU reports DL RSCPD measurement(s) along with DL RSTD measurement(s) or DL RSCP measurement(s) along with UE Rx-Tx time difference measurement(s), the UE or the PRU may be requested to perform the DL RSCPD and/or DL RSCP measurement(s) from the DL PRS resource(s) that have the phase errors within the same threshold value of the phase errors. <omitted text> The UE may be configured with [higher layer parameter] which contains DL carrier phase measurements performed by a positioning reference unit (PRU) [20, TS 38.305] along with the corresponding threshold value of the phase error and the location information of the PRU. <omitted text>. Proposal 3: UE Tx PEG ID defines the maximum time duration of SRS transmission that the UE can keep the phase error(s) within a threshold value, where each UE Tx PEG ID may be associated with one or multiple UL SRS resources. Proposal 4: TRP Tx PEG ID defines the maximum time duration of PRS transmission that the TRP can keep the phase error(s) within a threshold value, where each TRP Tx PEG ID may be associated with one or multiple DL PRS resources. Proposal 5: RAN1 enables the LMF to request gBN(s) to perform CP measurements from transmitted SRSs associated with a specific UE Tx PEG ID(s) and PRU Tx PEG ID(s). Proposal 6: RAN1 enables gNBs to report to the LMF the TRP Tx PEG ID(s) that are used for the transmission of the PRS resources to the target UE and the PRU. |
| --- | --- |
| Spreadtrum[8] | Proposal 3: Support option 2 for phase delays on Tx/Rx RF chains. |
| vivo[9] | Proposal 2: - Up to RAN4 to judge the availability and the necessity of PEG. |
| ZTE[11] | Proposal 4: To address the impact of the phase delays on Tx/Rx RF chains, support the following option: - Option 1a: introduce the definition of UE/TRP Tx/Rx phase error groups (PEGs) for the Tx/Rx of DL PRS/UL SRS signals |
| Samsung[13] | Proposal 2: The carrier phase measurement is associated with an ID corresponding to reference point. This can be the Rx antenna ID or the Rx RF chain ID. Proposal 3: A UE transmitting SRS for positioning indicates the Tx antenna ID or the Tx RF chain ID used to transmit SRS. Proposal 4: A gNB/TRP transmitting DL PRS indicates the Tx antenna ID or the Tx RF chain ID used to transmit the DL PRS. Text proposal 3: (clause 5.1.6.5 of TS 38.214) When the UE reports DL RSCPD measurement(s) along with DL RSTD measurement(s) or DL RSCP measurement(s) along with UE Rx-Tx time difference measurement(s), the DL RSCPD and/or DL RSCP measurement(s) should be measured from a single DL PRS positioning frequency layer. The UE may be configured to report, via high layer parameter [CPP-RxID-Request], the association information of DL RSCP or DL RSCPD measurement(s) with UE RF receive chain ID via higher layer parameter [CPP-RxID] when the UE reports the DL RSCP or DL RSCPD measurement(s). Text proposal 4: (clause 6.2.1.4 of TS 38.214) If the UE reports a UE Tx TEG ID with a UE Rx-Tx time difference measurement, the UE may report a UE Tx TEG timing error margin value, via high layer parameter nr-UE-TxTEG-TimingErrorMargin, for all the UE Tx TEGs within one NR-Multi-RTT-SignalMeasurementInformation. The UE may be configured to report, via high layer parameter [CPP-TxID-Request], the association information of the already transmitted SRS resource(s) configured by the higher layer parameter SRS-PosResource with UE RF transmit chain ID via higher layer parameter [CPP-TxID]. |
| xiaomi[14] | - Proposal 3: Introduce the definition of the phase error group or reuse TEG with small margin value for initial phase error mitigation. |
| OPPO[16] | Proposal 2: Support Option 3 to address the impact of the phase delays on Tx/Rx RF chains. |
| InterDigital[18] | Proposal 2: Support Option 1a for PEG (introduce the definition of UE/TRP Tx/Rx phase error groups (PEGs) for the Tx/Rx of DL PRS/UL SRS signals) |
| Apple[19] | Proposal 5: To address the impact of the phase delays on Tx/Rx RF chains RAN1 select option 2: - Option 1a: introduce the definition of UE/TRP Tx/Rx phase error groups (PEGs) for the Tx/Rx of DL PRS/UL SRS signals - Option 2: enhance the existing Rel-17 definitions of UE/TRP Tx/Rx TEGs with smaller margin value. |
| NTT DOCOMO[22] | Proposal 1: - Support the following option: - Option 2: reuse or enhance the existing Rel-17 definitions of UE/TRP Tx/Rx TEGs with smaller margin value to be determined by RAN4. |
| Ericsson[25] | Proposal 3 To address the impact of the phase delays on Tx/Rx RF chains, either i) enhance the the definition of a “timing error group” so that it captures both timing and phase errors. (Option 2), or ii) introduce the definition of UE/TRP Tx/Rx phase error groups (PEGs) for the Tx/Rx of DL PRS/UL SRS signals (Option 1a). |

FL Comments:

For address the impact of the phase delays on Tx/Rx RF chains, it was agreed in RAN1#112bis-e to support one or more of the 5 options, which were discussed intensive since RAN1#113 w/o a conclusion [26]. Many companies think this is an issue that needs to be resolved (e.g.., [7][8][9][11][13][14][16][18][19][22][25]). Based on the feedback in the submitted contributions to this meeting, the companies’ views can be summarized as follows.

- Option 1a: introduce the definition of UE/TRP Tx/Rx phase error groups (PEGs) for the Tx/Rx of DL PRS/UL SRS signals

  - Rel-17 definitions of UE/TRP Tx/Rx TEGs can be used as the starting point for defining UE/TRP Tx/Rx PEGs.

  - Supported by: Nokia, NSB, ZTE, InterDigital, Apple, Ericsson

  - Not supported by:

- Option 1b: Introduce Tx/Rx RF antenna IDs or Tx/Rx RF chain IDs to identify the individual Tx/Rx RF chains for transmitting/receiving the DL PRS/UL SRS signals.

  - Supported by: Samsung

  - Not supported by:

- Option 1c: introduce the report of ARP ID for the Rx/Tx of DL PRS/UL SRS signals.

  - The transmission/reception associated with the same ARP ID is assumed from the same ARP.

  - Supported by:

  - Not supported by:

- Option 2: reuse or enhance the existing Rel-17 definitions of UE/TRP Tx/Rx TEGs with smaller margin value.

  - Supported by: Spreadtrum, Xiaomi, Apple, NTT DOCOMO, Ericsson

  - Not supported by:

- Option 3: RAN1 sends an LS to RAN4, requesting RAN4 to consider whether there is a need to define the new UE/TRP Tx/Rx phase error groups (PEGs), introduce new IDs (e.g., Tx/Rx RF antenna IDs) to present the phase delays for the Tx/Rx of DL PRS/UL SRS signals, or reuse or enhance the existing Rel-17 definitions of UE/TRP Tx/Rx TEGs with smaller margin value, and provide the definitions if RAN4 decides it is needed.

  - Supported by: vivo, OPPO

  - Not supported by:

Considering the feedback received, there is still significant divergence in companies’ views. Basically, all of the companies that provided their opinions in this meeting have kept their positions unchanged.

To resolve the critical issue, the FL is wondering if we can have a compromised solution, which is based on Option 1a with the consideration of Option 1b and Option 3. Option 1b proposes to introduce Tx/Rx RF antenna IDs or Tx/Rx RF chain IDs. If we limit the DL PRS resources and UL SRS resources of a PEG to a single Tx/Rx RF chain for CPP, then PEG ID represents basically the RF chain ID. Actually, RAN4 has taken the similar approach for BW aggregation for positioning. Option 3 requests RAN4 to work on the issue. An alternative is RAN1 provides the initial inputs for RAN4 to decide/confirm. This may reduce RAN4’s effort.

(H)(Round 1) Proposal 6-1

Introduce the definition of UE/TRP Tx/Rx phase error groups (PEGs) for the Tx/Rx of DL PRS/UL SRS signals:

- Rel-17 definitions of UE/TRP Tx/Rx TEGs are used as the starting point for defining UE/TRP Tx/Rx PEGs.

- The PRS resources or SRS resources associated to one Tx/Rx PEG should be transmitted/received from a single Tx/Rx RF chain only.

- The values of the Tx/Rx PEG error margins are defined by RAN4

- Send an LS to RAN4, requesting RAN4 to check if RAN1’s approach is feasible and acceptable.

| Company | comments |
| --- | --- |
| vivo | We wonder the second sub-bullet is needed, it different with the definition of TEG. |
| Xiaomi | We are generally fine with this proposal. But for the 2nd sub-bullet, we think it is more restricted. |
| InterDigital | Support |
| ZTE | Can support the main bullet first. |
| Samsung1 | As CPP has more strict requirements on the tolerance of the phase measurement, the necessity of the first bullet is not clear and we prefer to remove. |
| Qualcomm | We believe this is late to be added in maintenance phase. It is not a correction to add a new feature. |
| LGE | Considering it is maintenance phase, we think reusing TEG is only possible option for now. |
| CATT | Support. |
| Ericsson | Agree with LGE. Reusing TEG framework as much as possible seems the most likeliest possibility. |
| Huawei, HiSilicon | Do not support. |
| Nokia/NSB | Support. It is a little late but we see a benefit to provide a better performance of CP positioning. |
| FL | It seems most of the companies are not fine with the current form of (H)(Round 2) Proposal 6-1. It seems we are still in a deadlock situation on whether and how to handle the phase delays. We are already in maintenance phase. Thus, we need to conclude the discussion if RAN1 cannot reach a consensus to take any of the options. ### (H)(Round 2) Proposal 6-1 To address the impact of the phase delays on Tx/Rx RF chains, down-select one or more of the following options (down-selection in RAN1#114bis): - Option 1a: introduce the definition of UE/TRP Tx/Rx phase error groups (PEGs) for the Tx/Rx of DL PRS/UL SRS signals - Rel-17 definitions of UE/TRP Tx/Rx TEGs can be used as the starting point for defining UE/TRP Tx/Rx PEGs. - FFS: the details of \the UE/TRP Tx/Rx PEGs - Option 1b: Introduce Tx/Rx RF antenna IDs or Tx/Rx RF chain IDs to identify the individual Tx/Rx RF chains for transmitting/receiving the DL PRS/UL SRS signals. - FFS: the details of the Tx/Rx RF antenna IDs or Tx/Rx RF chain IDs - Note: Device transmitting PRS or positioning SRS provides Tx antenna ID or Tx Chain ID. Device receiving PRS or positioning SRS provides Rx antenna ID or Rx Chain ID. - Option 1c: introduce the report of ARP ID for the Rx/Tx of DL PRS/UL SRS signals. - The transmission/reception associated with the same ARP ID is assumed from the same ARP. - FFS: the maximum number of ARP IDs. - Option 2: reuse or enhance the existing Rel-17 definitions of UE/TRP Tx/Rx TEGs with smaller margin value. - Option 3: RAN1 sends an LS to RAN4, requesting RAN4 to consider whether there is a need to define the new UE/TRP Tx/Rx phase error groups (PEGs), introduce new IDs (e.g., Tx/Rx RF antenna IDs ) to present the phase delays for the Tx/Rx of DL PRS/UL SRS signals, or reuse or enhance the existing Rel-17 definitions of UE/TRP Tx/Rx TEGs with smaller margin value, and provide the definitions if RAN4 decides it is needed. - Option 4: Make a conclusion that no further discussion on UE/TRP Tx/Rx phase error groups (PEGs) in Rel-18. |

### (H)(Round 2) Proposal 6-1

To address the impact of the phase delays on Tx/Rx RF chains, down-select one or more of the following options (down-selection in RAN1#114bis):

- Option 1a: introduce the definition of UE/TRP Tx/Rx phase error groups (PEGs) for the Tx/Rx of DL PRS/UL SRS signals

  - Rel-17 definitions of UE/TRP Tx/Rx TEGs can be used as the starting point for defining UE/TRP Tx/Rx PEGs.

  - FFS: the details of \the UE/TRP Tx/Rx PEGs

- Option 1b: Introduce Tx/Rx RF antenna IDs or Tx/Rx RF chain IDs to identify the individual Tx/Rx RF chains for transmitting/receiving the DL PRS/UL SRS signals.

  - FFS: the details of the Tx/Rx RF antenna IDs or Tx/Rx RF chain IDs

  - Note: Device transmitting PRS or positioning SRS provides Tx antenna ID or Tx Chain ID. Device receiving PRS or positioning SRS provides Rx antenna ID or Rx Chain ID.

- Option 1c: introduce the report of ARP ID for the Rx/Tx of DL PRS/UL SRS signals.

  - The transmission/reception associated with the same ARP ID is assumed from the same ARP.

  - FFS: the maximum number of ARP IDs.

- Option 2: reuse or enhance the existing Rel-17 definitions of UE/TRP Tx/Rx TEGs with smaller margin value.

- Option 3: RAN1 sends an LS to RAN4, requesting RAN4 to consider whether there is a need to define the new UE/TRP Tx/Rx phase error groups (PEGs), introduce new IDs (e.g., Tx/Rx RF antenna IDs ) to present the phase delays for the Tx/Rx of DL PRS/UL SRS signals, or reuse or enhance the existing Rel-17 definitions of UE/TRP Tx/Rx TEGs with smaller margin value, and provide the definitions if RAN4 decides it is needed.

- Option 4: Make a conclusion that no further discussion on UE/TRP Tx/Rx phase error groups (PEGs) in Rel-18.

| Company | comments |
| --- | --- |
| ZTE | Option 1a. |
| Samsung2 | Option 1b. |
| InterDigital | Option 1a |
| NTT DOCOMO | Considering maintenance phase, we prefer TEG framework (i.e., Option 2) to reduce specification impacts. |

# (Closed) CP measurement quality

Agreement Support UE/TRP to report the phase quality indication for the RSCP/RSCPD measurements. The phase quality indication includes the following fields: - phase quality index - phase quality resolution The values of the phase quality index and phase quality resolution are left for RAN4.

Submitted Proposals:

| Samsung[13] | When the UE reports DL RSCPD measurement(s) along with DL RSTD measurement(s) or DL RSCP measurement(s) along with UE Rx-Tx time difference measurement(s), the DL RSCPD and/or DL RSCP measurement(s) should be measured from a single DL PRS positioning frequency layer. A positioning measurement report with DL RSCPD measurement or DL RSCP measurement includes carrier phase measurement quality indication. The quality indication includes a phase quality index and a phase quality resolution as defined in [11, TS 38.133]. Text proposal 6: |
| --- | --- |
| CATT[15] | Proposal 1: Adopt the following TP regarding the reporting of the phase quality indication for the RSCP/RSCPD measurements in Clause 5.1.6.5 of TS 38.214 Error! Reference source not found.. Reason for change:; The specification does not capture the following agreement made in RAN1#114 regarding to the report of the quality indication associated with DL RSCP/RSCPD measurement. Agreement Support UE/TRP to report the phase quality indication for the RSCP/RSCPD measurements. The phase quality indication includes the following fields: - phase quality index - phase quality resolution. Summary of change:; Add the report of the quality indication associated with DL RSCP/RSCPD measurement according to the agreement made in RAN1#114.. Consequences if not approved:; Specification is not align with RAN1’s agreement and incomplete incomplete. #### 5.1.6.5 PRS reception procedure --------------------------- Start of text proposal to TS 38.214 v18.0.0------------------------------------------- … The UE may be configured with [higher layer parameter] which contains DL carrier phase measurements performed by a positioning reference unit (PRU) [20, TS 38.305] along with the location information of the PRU. The UE may be configured to report quality metrics [higher layer parameter] corresponding to the DL RSCP and RSCPD measurements which include the following fields [17, TS 37.355]: - [phase quality index] which provides the best estimate of the uncertainty of the measurement - [phase quality resolution] which specifies the resolution levels used in the [phase quality index] field. … ----------------------------- End of Text proposal to TS 38.214 v18.0.0------------------------------------------ |
| OPPO[16] | Proposal 3: Endorse the following TP for TS 38.214 - Reason for change: one agreement was not captured - Summary of change: Add text description in Section 5.1.6.5 in TS 38.214 to capture the agreement - Consequences if not approved: The design of carrier phase measurement is not completed #### 5.1.6.5 PRS reception procedure #### *** Unchanged parts are omitted *** For DL UE positioning measurement reporting in higher layer parameter NR-DL-TDOA-SignalMeasurementInformation, the UE may be configured to report the DL Reference Signal Carrier Phase Difference (RSCPD) [7, TS 38.215] measurement along with the DL RSTD. When the UE reports RSCPD measurements the reference nr-DL-PRS-ReferenceInfo is the same as the one reported, for the RSTD measurements. For DL UE positioning measurement reporting in higher layer parameter NR-Multi-RTT-SignalMeasurementInformation the UE may be configured to report the DL Reference Signal Carrier Phase (RSCP) measurement [7, TS 38,215] along with the UE Rx-Tx time difference measurement. When the UE reports DL RSCPD measurement(s) along with DL RSTD measurement(s) or DL RSCP measurement(s) along with UE Rx-Tx time difference measurement(s), the DL RSCPD and/or DL RSCP measurement(s) should be measured from a single DL PRS positioning frequency layer. The UE may be configured to report quality metrics corresponding to the DL RSCP and DL RSCPD measurements which include the following fields: - phase quality index - phase quality resolution #### *** Unchanged parts are omitted ***. |

FL Comments:

It was agreed in RAN1#114 to support UE/TRP to report the phase quality indication for the RSCP/RSCPD measurements. The phase quality indication includes the following fields: phase quality index and phase quality resolution. Multiple companies (e.g., [13][15][16]) have proposed to capture the agreement into the TS 38.214.

### (Closed)(Round 1)Proposal 7-1

Adopt the following TP regarding the reporting of the phase quality indication for the RSCP/RSCPD measurements in Clause 5.1.6.5 of TS 38.214.

| Reason for change: | The specification does not capture the following agreement made in RAN1#114 regarding to the report of the quality indication associated with DL RSCP/RSCPD measurement. Agreement Support UE/TRP to report the phase quality indication for the RSCP/RSCPD measurements. The phase quality indication includes the following fields: - phase quality index - phase quality resolution |
| --- | --- |
| Summary of change: | Add the report of the quality indication associated with DL RSCP/RSCPD measurement according to the agreement made in RAN1#114. |
| Consequences if not approved: | Specification is not align with RAN1’s agreement and incomplete incomplete |

#### 5.1.6.5 PRS reception procedure ===================== Unchanged parts omitted ====================== The UE may be configured with [higher layer parameter] which contains DL carrier phase measurements performed by a positioning reference unit (PRU) [20, TS 38.305] along with the location information of the PRU. The UE may be configured to report quality metrics [higher layer parameter] corresponding to the DL RSCP and RSCPD measurements which include the following fields [17, TS 37.355]: - [phase quality index] which provides the best estimate of the uncertainty of the measurement - [phase quality resolution] which specifies the resolution levels used in the [phase quality index] field. ===================== Unchanged parts omitted ======================

| Company | comments |
| --- | --- |
| vivo | ok |
| Xiaomi | OK |
| Samsung1 | OK |
| Qualcomm | We prefer not to include the expression “provides the best estimate of the” : The UE may be configured to report quality metrics [higher layer parameter] corresponding to the DL RSCP and RSCPD measurements which include the following fields [17, TS 37.355]: - [phase quality index] which provides the best estimate of the uncertainty of the measurement - [phase quality resolution] which specifies the resolution levels used in the [phase quality index] field. FL: No strong view on QC’s suggestion. The wording “provides the best estimate of the” comes from the wording in current specification for the quality information of the legacy measurements. |
| LGE | Fine |
| CATT | Support. |
| Huawei, HiSilicon | OK |
| Nokia/NSB | Ok with either the proposal or revised version from Qualcomm. |
| FL | See Chairman’s notes for the agreement. |

# Multipath mitigation

## Carrier phase measurements for additional paths

Background

Agreement (RAN1#112) NR DL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the DL PRS signals configured for the measurement. A RSCP is associated with a specific RF frequency. - FFS: the reference point of the RSCP - FFS: whether/how the measurement timing is defined - Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion - Note: Whether to capture the above definition into TS 38.215 depends on whether RAN1 decides to introduce DL carrier phase measurement for NR CPP Agreement (RAN1#112) Rel-17 LOS/NLOS indication (when indicated) applies for the carrier phase measurement(s) in the same report.

Submitted Proposals:

| Nokia, NSB[7] | Proposal 11: NR CPP should optionally support reporting the carrier phase of additional paths for both DL and UL. |
| --- | --- |
| Spreadtrum[8]] | Proposal 4: For carrier phase positioning, both carrier phases of the first path and the additional paths can be reported. Proposal 5: For multipath mitigation, support the reporting of other existing channel information together with carrier phase positioning. |
| Samsung[13] | Proposal 5: For carrier phase positioning measurement, a UE or gNB provides a measurement based on the first (LOS) received multi-path. Text proposal 5: (clause 5.1.6.5 of TS 38.214) If the UE reports LoS/NLoS indicator(s) via higher layer parameter nr-los-nlos-Indicator along with a measurement report containing DL RSCP or DL RSCPD the LoS/NLoS indicator(s) are assumed to also apply to the DL RSCP or DL RSCPD measurements. If the UE is configured to report DL RSCP or DL RSCPD, the UE reports DL RSCP or DL RSCPD for LoS component. |
| CMCC[17] | Proposal 2: For NR carrier phase positioning, support only the carrier phase of the first path. |

FL Comments:

The issue on whether to support the reporting of the carrier phase of additional paths has been discussed in previous meetings. For this meeting, many companies still provided different views as shown in (e.g., [7][8][13][17]).

Considering that a) RAN1 has spent significant effort on this issue; b) it is unlikely that we can reach a consensus to support reporting of the carrier phase of additional paths; and c) the WI was completed from RAN1’s perspective, the FL suggest making a conclusion that we will not further discuss the reporting of the carrier phase of additional paths in Rel-18.

(Round 1) Proposal 8.1-1 (as a conclusion)

From RAN1’s perspective, there will be no further discussion regarding the support of reporting carrier phase measurements of additional paths in Rel-18.

| Company | comments |
| --- | --- |
| InterDigital | We have a question for clarification. If we make this conclusion, does this mean that the network does not know which path the reported phase measurement corresponds to? FL: It means UE does not report the measurements from additional path(s). That mean the UE reports the carrier phase for the first path only. We can write the conclusion in that way also. ### (Round 1) Proposal 8.1-1 (as a conclusion) From RAN1’s perspective, only the carrier phase measurements of first path is supported in Rel-18. |
| ZTE | Support. |
| Samsung1 | We prefer to report carrier phase for first (LOS) path. The benefit of reporting additional multi-path is not clear. |
| Qualcomm | Support |
| LGE | Fine |
| CATT | Support. |
| Huawei, HiSilicon | OK |
| Nokia/NSB | OK |
| FL | For clarity, it may be better to say in Rel-18, only the carrier phase measurements (i.e., RSCP/RSCPD) of the first path are supported in Rel-18. |

### (Round 2) Proposal 8.1-1

Only the carrier phase measurements (i.e., DL/UL RSCP, DL RSCPD) of the first path are supported in Rel-18.

| Company | comments |
| --- | --- |
| ZTE | Support. |
| Samsung2 | Support |
| NTT DOCOMO | Support |

## (Closed) LOS/NLOS indication for standalone carrier phase positioning

Background

Agreement (RAN1#112) Rel-17 LOS/NLOS indication (when indicated) applies for the carrier phase measurement(s) in the same report.

Submitted Proposals:

| InterDigital[18] | Proposal 3: The UE reports RSCPD if the LOS/NLOS indicator associated PRS resource or TRP is above the configured threshold |
| --- | --- |
| Apple[19] | Proposal 6: the Rel-17 LOS/NLOS agreements for additional paths (when indicated) can be applied to carrier phase measurement(s) in the same report if measurements are reported for additional paths. |

FL Comments:

In [18], the suggestion was made to report RSCPD if the LOS/NLOS indicator associated with PRS resources or TRP exceeds the configured threshold. However, the motivation behind this proposal remains unclear to the FL. In [19] it was proposed applying the LOS/NLOS indication to carrier phase measurements in the same report if measurements are reported for additional paths. However, given the current discussion, the likelihood of supporting carrier phase measurements for additional paths in Rel-18 appears very slim.

Interested companies may provide their comments on the above Proposal 3 [18] and Proposal 6 [19] in the following table.

| Company | comments |
| --- | --- |
| FL | The discussion is closed. No company provides comments. |

## (Closed) RRC Parameters

Submitted Proposals:

| NTT DOCOMO[22] | Proposal 3: - Regarding row 36/37 in higher layer parameters list for NR positioning, the following modification can be considered. - Row 36: LMF to request UE to report DL RSCP measurement together with UE Rx-Tx time difference measurement - LMF to request UE to report DL RSCP measurement from a single DL PFL together with UE Rx-Tx time difference measurement - Row 37: LMF to request UE to report DL RSCPD measurement together with DL-RSTD measurement - LMF to request UE to report DL RSCPD measurement from a single DL PFL together with DL-RSTD measurement Proposal 4: - Regarding row 43/44/45 in higher layer parameters list for NR positioning, the following value range can be considered at least. - Row 43: Requesting SRS for positioning configuration from LMF to the serving gNB - the duration of a time window: introduce {1, 2, 4, 8, 12} OFDM symbols and {1, 2, 4, 6, 8, 12, 16} slots - the number of the time windows: introduce {1, 2, …, 16} and does not introduce additional value - Row 44: Requesting SRS for positioning configuration from LMF to the serving gNB and neighbor gNB - the duration of a time window: introduce {1, 2, 4, 6, 8, 12, 16} slots - the number of the time windows: introduce {1, 2, …, 16} - Row 45: Requesting DL-PRS reception from LMF to a UE - the duration of a time window: introduce {1, 2, 4, 6, 8, 12, 16} slots - the number of the time windows: introduce {1, 2} and does not introduce {4, 8} |
| --- | --- |

FL Comments:

In [22], suggestions were provided for RRC parameters of CPP. Most of the suggestions were made based on the agreement reached in RAN1#114. Considering that the RRC parameters will be further discussed in this meeting. These proposals can be further discussed in the session for RRC parameters.

| Company | comments |
| --- | --- |

# 2nd Online Session

## Offline Consensus(1)

Adopt the TP in Section 9.1 of R1-2310285 for Clause 5.1.6.5 of TS 38.214.

| Reason for change: | The following agreement made in RAN1#114 meeting is not captured in the current TS 38.214. Agreement For UE-based carrier phase positioning, when LMF forwards the DL carrier phase measurement reported by a PRU to a target UE, the timestamp associated with the PRU carrier phase measurements should also be forwarded in positioning assistance data. |
| --- | --- |
| Summary of change: | Capture the above agreement in TS 38.214 |
| Consequences if not approved: | Specification TS 38.214 is incomplete |

#### 5.1.6.5 PRS reception procedure ===================== Unchanged parts omitted ====================== The UE may be configuredprovided with [higher layer parameter] which contains DL carrier phase measurements performed by a positioning reference unit (PRU) [20, TS 38.305], the timestamps associated with the measurements, and along with the location information of the PRU. ===================== Unchanged parts omitted ======================

## Offline Consensus(2)

Adopt the following changes to the previous agreement made in RAN1#114:

Agreement When an LMF requests the UEs, including target UE and PRU(s), to perform measurements on indicated DL PRS resource set(s) occurring within indicated time window(s) - The duration of a time window can be configured as follows: - {1, 2, 4, 6, 8, 12, 16} slots. - the number of the time windows can be: - {1, 2} - FFS: {4, 8} - the number of the indicated DL PRS resource set(s) per TRP within a time windows can be {1, 2}: - DL PRS resource sets across all TRPs are in one DL PFL - The number of the indicated DL PRS resource set(s) for all TRPs should be the same - Note: Different PRS resource sets and/or PFLs can be associated with different time windows - Note: It is up to RAN2/RAN3 in the signaling design for the indication of the DL PRS resource sets in the time windows.

## FL Proposals

### (Round 2) Proposal 8.1-1

Only the carrier phase measurements (i.e., DL/UL RSCP, DL RSCPD) of the first path are supported in Rel-18.

### (H)(Round 3) Proposal 3.2-1a

- A DL RSCP measurement is obtained by measuring a single DL PRS resource from a TRP.

- The DL PRS resource is either the same DL PRS resource used to obtain the associated UE Rx-Tx time difference measurement, or one of the DL PRS resources used to obtain the associated UE Rx-Tx time difference measurement.

- In the signalling design, the LMF should be able to identify which DL PRS resource and DL PRS resource set are used to obtain the DL RSCP measurement. The signalling design is up to RAN2/RAN3.

### (H)(Round 3) Proposal 3.2-1b

- A DL RSCPD measurement is obtained by measuring two DL PRS resources: one from the target TRP and one from the reference TRP;

- The two DL PRS resource are either the same DL PRS resources used to obtain the associated UE RSTD measurement, or included in the DL PRS resources used to obtain the associated UE RSTD measurement.

- In the signalling design, the LMF should be able to identify which DL PRS resource and DL PRS resource set are used to obtain the DL RSCP measurement. The signalling design is up to RAN2/RAN3.

### (H)(Round 2) Proposal 3.3-1

When LMF forwards the carrier phase measurements reported by a PRU to a target UE for UE-based carrier phase positioning, the following information can be included in the positioning assistance data to the target UE (down-selection):

- Option 1: the RF frequency information associated with the PRU carrier phase measurements.

- Option 2: the DL PRS configuration information that were provided to the PRU for the carrier phase measurements.

### (H)(Round 3)Proposal 5-5

Adopt one of the following options (down-selection):

- Option 1:

  - Subject to UE capabilities, support a UE to use the granularity {0.5Tc, 0.25Tc} for the reporting of RSTD/UE Rx-Tx time difference for NR CPP.

  - Support a gNB to use the granularity {0.5Tc, 0.25Tc} for the reporting of RTOA/gNB Rx-Tx time difference for NR CPP.

  - Send an LS to RAN4, asking RAN4 to confirm RAN1’s decision and provide the feedback if RAN4 has any concern.

- Option 2:

  - Send an LS to RAN4, asking RAN4 whether it is feasible to a UE to support finer granularity(ties) (e.g., 0.5Tc, 0.25Tc) in the reporting of RSTD/UE Rx-Tx time difference for NR CPP, and if yes, provide the finer granularity(ties) to RAN1.

### (Round 2) Proposal 2.5-1

No further discussion in RAN1 how to maintain phase continuity in order to eliminate unknown initial Rx phase with RSCP/RSCPD reporting in Rel-18.

### (H)(Round 2) Proposal 6-1

To address the impact of the phase delays on Tx/Rx RF chains, down-select one or more of the following options (down-selection in RAN1#114bis):

- Option 1a: introduce the definition of UE/TRP Tx/Rx phase error groups (PEGs) for the Tx/Rx of DL PRS/UL SRS signals

  - Rel-17 definitions of UE/TRP Tx/Rx TEGs can be used as the starting point for defining UE/TRP Tx/Rx PEGs.

  - FFS: the details of \the UE/TRP Tx/Rx PEGs

- Option 1b: Introduce Tx/Rx RF antenna IDs or Tx/Rx RF chain IDs to identify the individual Tx/Rx RF chains for transmitting/receiving the DL PRS/UL SRS signals.

  - FFS: the details of the Tx/Rx RF antenna IDs or Tx/Rx RF chain IDs

  - Note: Device transmitting PRS or positioning SRS provides Tx antenna ID or Tx Chain ID. Device receiving PRS or positioning SRS provides Rx antenna ID or Rx Chain ID.

- Option 1c: introduce the report of ARP ID for the Rx/Tx of DL PRS/UL SRS signals.

  - The transmission/reception associated with the same ARP ID is assumed from the same ARP.

  - FFS: the maximum number of ARP IDs.

- Option 2: reuse or enhance the existing Rel-17 definitions of UE/TRP Tx/Rx TEGs with smaller margin value.

- Option 3: RAN1 sends an LS to RAN4, requesting RAN4 to consider whether there is a need to define the new UE/TRP Tx/Rx phase error groups (PEGs), introduce new IDs (e.g., Tx/Rx RF antenna IDs ) to present the phase delays for the Tx/Rx of DL PRS/UL SRS signals, or reuse or enhance the existing Rel-17 definitions of UE/TRP Tx/Rx TEGs with smaller margin value, and provide the definitions if RAN4 decides it is needed.

- Option 4: Make a conclusion that no further discussion on UE/TRP Tx/Rx phase error groups (PEGs) in Rel-18.

## TP2

(H)(Round 1)Proposal 4.1-1

Adopt the TP in Section 9.4 of R1-2310285 for Clause 5.1.6.5 of TS 38.214

| Reason for change: | The measurement time windows introduced in Rel-18 are applicable not only to RSCP/RSCPD measurements, but also other measurements. |
| --- | --- |
| Summary of change: | Moving the text that describes the measurements on indicated DL PRS resource sets occurring within one or more window(s) to the location before the text that describes the DL PRS measurements. |
| Consequences if not approved: | Readers may erroneously assume the UE only performs carrier phase measurements, but not other measurements, on indicated DL PRS resource sets occurring within one or more configured time window(s). |

#### 5.1.6.5 PRS reception procedure ===================== Unchanged parts omitted ====================== The UE may be configured with [higher layer parameter] which contains DL carrier phase measurements performed by a positioning reference unit (PRU) [20, TS 38.305] along with the location information of the PRU. The reduced capability UE may be configured to measure and report, subject to UE capability, via [higher layer parameter] the DL RSTD, DL PRS-RSRP, DL PRS-RSRPP, or UE Rx-Tx time difference using receiver frequency hopping for a DL PRS resource, with bandwidth that may be greater than the maximum reduced capability UE bandwidth, within a configured measurement gap. The reduced capability UE performing receiver frequency hopping may be configured to report via [higher layer parameter] one measurement associated with one received frequency hop or one measurement based on multiple hops of the DL PRS. [In RRC_CONNECTED mode], the reduced capability UE is expected to use a single instance of a configured measurement gap to receive all hops of the DL PRS using receiver frequency hopping. For DL UE positioning measurement reporting in higher layer parameter NR-DL-TDOA-SignalMeasurementInformation, the UE may be configured to report the DL Reference Signal Carrier Phase Difference (RSCPD) [7, TS 38.215] measurement along with the DL RSTD. When the UE reports RSCPD measurements the reference nr-DL-PRS-ReferenceInfo is the same as the one reported, for the RSTD measurements. For DL UE positioning measurement reporting in higher layer parameter NR-Multi-RTT-SignalMeasurementInformation the UE may be configured to report the DL Reference Signal Carrier Phase (RSCP) measurement [7, TS 38,215] along with the UE Rx-Tx time difference measurement. When the UE reports DL RSCPD measurement(s) along with DL RSTD measurement(s) or DL RSCP measurement(s) along with UE Rx-Tx time difference measurement(s), the DL RSCPD and/or DL RSCP measurement(s) should be measured from a single DL PRS positioning frequency layer. The UE, subject to UE capability, may be requested via [higher layer parameter] to perform positioning measurements on indicated DL PRS resource sets occurring within one or more time window(s) indicated by [higher layer parameter]. The UE is expected to obtain 1 DL RSCP or DL RSCPD measurement with Nsample=1 as defined in [11, TS 38.133]. When the UE reports a timestamp associated with a DL RSCP measurement or a DL RSCPD measurement, subject to UE capability, it may include a symbol index in the timestamp. If the UE reports LoS/NLoS indicator(s) via higher layer parameter nr-los-nlos-Indicator along with a measurement report containing DL RSCP or DL RSCPD the LoS/NLoS indicator(s) are assumed to also apply to the DL RSCP or DL RSCPD measurements. The UE may be configured with [higher layer parameter] which contains DL carrier phase measurements performed by a positioning reference unit (PRU) [20, TS 38.305] along with the location information of the PRU. ===================== Unchanged parts omitted ======================

# References

- RP-223549 New WID on Expanded and Improved NR Positioning Intel Corporation, CATT, Ericsson

- 3GPP RAN1#112 Chairman's Notes.

- 3GPP RAN1#112bis-e Chairman's Notes.

- 3GPP RAN1#113 Chairman's Notes.

- 3GPP RAN1#114 Chairman's Notes.

- R1-2308877	Maintenance of CPP	Huawei, HiSilicon

- R1-2308955	Remaining issues on NR DL and UL carrier phase positioning	Nokia, Nokia Shanghai Bell

- R1-2308986	Remaining issues on NR DL and UL carrier phase positioning	Spreadtrum Communications

- R1-2309074	Remaining issues on carrier phase positioning	vivo

- R1-2309198	Remaining details on NR DL and UL Carrier Phase Positioning	Intel Coporation

- R1-2309222	Maintenance on carrier phase positioning	ZTE

- R1-2309326	Remaining issues on carrier phase positioning in NR	LG Electronics

- R1-2309375	Maintenance on NR DL and UL Carrier Phase Positioning	Samsung

- R1-2309458	Remaining issues on NR DL and UL carrier phase positioning	xiaomi

- R1-2309526	Maintenance on NR DL and UL carrier phase positioning	CATT

- R1-2309575	Remaining Issues of NR carrier phase positioning	OPPO

- R1-2309671	Maintenance on carrier phase positioning	CMCC

- R1-2309798	Remaining issues for positioning based on NR carrier phase measurement	InterDigital, Inc.

- R1-2309833	On NR DL and UL carrier phase positioning	Apple

- R1-2309938	Discussions on NR DL and UL carrier phase positioning	Ruijie Network Co. Ltd

- R1-2309948	DL/UL CPP Maintenance Discussion	Lenovo

- R1-2310033	Remaining issues on NR DL and UL carrier phase positioning	NTT DOCOMO, INC.

- R1-2310090	Maintenance for carrier phase positioning	MediaTek Korea Inc.

- R1-2310141	Maintenance for NR Carrier Phase Positioning	Qualcomm Incorporated

- R1-2310198	Remaining issues on NR DL and UL carrier phase positioning	Ericsson

- R1-2308219	FL Summary #3 for NR DL and UL carrier phase positioning	Moderator (CATT)

# Appendix

## Agreements made in RAN1#112

Agreement To enable UE-based and UE-assisted NR carrier phase positioning (CPP), one or both of the following new measurements should be introduced: - DL carrier phase (CP), which is obtained by a UE measuring the DL PRS signal(s) from a TRP. - FFS: The detailed definition of the DL CP - DL carrier phase difference (CPD), which is the difference of two DL CPs from two TRPs - FFS: The detailed definition of the DL CPD To enable NG-RAN node-assisted NR carrier phase positioning (CPP), the following new measurement should be introduced: - UL carrier phase (CP), which is obtained by a TRP measuring the UL SRS for positioning or MIMO SRS from a UE. - FFS: The detailed definition of the UL CP Agreement NR DL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the DL PRS signals configured for the measurement. A RSCP is associated with a specific RF frequency. - FFS: the reference point of the RSCP - FFS: whether/how the measurement timing is defined - Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion - Note: Whether to capture the above definition into TS 38.215 depends on whether RAN1 decides to introduce DL carrier phase measurement for NR CPP Agreement For NR DL reference signal carrier phase difference (RSCPD) measurement for NR CPP, the RSCPD is defined as the difference of RSCPs measured from the DL PRS signals from target TRP and reference TRP. - FFS: whether/how to define per path RSCPD - Note: Whether/how to capture the above definition into TS 38.215 depends on whether RAN1 decides to introduce DL carrier phase difference measurement for NR CPP Agreement For NR carrier phase positioning, at least support the following approach: enable a UE/TRP to report carrier phase measurements together with the legacy positioning measurements to LMF - FFS: which legacy positioning measurements among RSTD, RTOA, UE Rx-Tx time difference measurements, gNB Rx-Tx time difference measurements. Agreement NR UL reference signal carrier phase (RSCP) (of i-th path) is defined as the phase of the channel response at the i-th path delay derived from the resource elements (REs) that carry the UL SRS signal for positioning purpose configured for the measurement. A UL RSCP is associated with a specific RF frequency. - FFS: the reference point of the UL RSCP - FFS: whether/how the measurement timing is defined - Note: the i-th path is used for the sake of definition, whether only the first path or additional paths will be supported is subject to further discussion - Note: The support of MIMO SRS for positioning is transparent to UE Agreement To support NR carrier phase positioning, further consider the following options: - Option 1: Support a UE/TRP to report the carrier phase measurements of more than one frequency within a PFL/carrier to LMF - NOTE: the frequency can be the carrier frequency or the frequency of a subcarrier - FFS: the details of reporting, e.g., the maximum number of reported frequencies within a PFL/ carrier - Option 2: Introduce and report a new type of UE/TRP measurement based on carrier phase differentials across multiple subcarriers within a PFL/carrier - NOTE: carrier phase differentials across multiple subcarriers within a carrier can be related to time of arrival - Option 3: Support a UE/TRP to optionally report an estimated integer ambiguity and/or search range of the integer ambiguity to LMF - Option 4: Support LMF to provide the expected integer ambiguity range at least for UE-based NR CPP in the positioning assistance data. Agreement Rel-17 LOS/NLOS indication (when indicated) applies for the carrier phase measurement(s) in the same report.

## Agreements made in RAN1#112bis-e

Agreement Introduce DL reference carrier phase (DL RSCP) and NR DL reference carrier phase difference (DL RSCPD) as DL carrier phase measurements. - Note: It is up to RAN4 to decide whether and how to define the requirements for DL RSCP and/or DL RSCPD. No LS needed to RAN4 for this note. - DL RSCP can be reported together with UE Rx – Tx time difference measurement - DL RSCPD can be reported together with RSTD measurement - FFS: details on how to eliminate unknown initial Rx phase with RSCP/RSCPD reporting can be further discussed - Note: Whether to support standalone DL RSCP and/or DL RSCPD reporting, or DL RSCP/DL RSCPD reporting with other new types of measurements (if agreed), can be further discussed. Agreement Support one of the following options for the definition of the reference point of the UE/TRP carrier phase measurements (down-selection in RAN1#113). - Option 1: - The reference point of the UE carrier phase measurements is defined the same as the reference point of RSTD for frequency range 1 and frequency range 2. - The reference point of the TRP carrier phase measurements is defined the same as the reference point of RTOA for frequency range 1 and frequency range 2. - Note: It is up to UE/TRP’s implementation on how to map the carrier phase to the reference point for reporting. - Option 2: - The reference point of the UE/TRP carrier phase measurements is defined as the antenna phase center of the UE/TRP Rx antenna for frequency range 1 and frequency range 2. - UE/TRP should provide the antenna phase center offset (PCO), i.e., the relative position between the antenna phase center and the antenna connector to LMF - FFS: the more details of the PCO reporting, e.g., in LCS or GCS frame Agreement To enable simultaneous transmission of UL SRS for positioning by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the serving gNB of a UE to configure the transmission of the [indicated] UL SRS resources from the UE within indicated time window(s). - FFS: the details of the time window, e.g., the start time, duration, periodicity for the time window(s), within the vicinity of a reference SRS configuration or use the existing message of Scheduled Location time - Enabling LMF to request the serving gNB and neighboring gNBs of the UE to measure the [indicated] UL SRS resources from the UE within indicated time window(s). - Note: this may be a different indicated time window Agreement To enable simultaneous measurements on same DL PRS by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the UEs, including target UE and PRU(s), to perform measurements on [indicated] DL PRS resources occurring within indicated time window(s). - FFS: the details of the configuration of the indicated time window(s), e.g., the start time, duration, periodicity for the time window(s), as well as the relationship with the Scheduled Location time. Agreement Support the reuse of existing physical layer procedures for DL positioning (e.g., DL-TDOA) with the necessary enhancements in measurement configuration, request and report (e.g., adding the configuration related to the NR DL CPP) for both UE-based and UE-assisted NR DL carrier phase positioning, including - UE in RRC_CONNECTED state with measurement gap. - FFS: UE in RRC_CONNECTED state without measurement gap - UE in RRC_ INACTIVE state Agreement The specific RF frequency associated with a DL carrier phase measurement is defined as the center frequency of the DL PFL by default. - Note: It is open to further discussion whether a frequency other than the center frequency of the DL PFL can also be the specific RF frequency for non-default case(s), if RAN1 agrees to introduce them. Agreement The specific RF frequency associated with a UL carrier phase measurement is defined, by default, as the center frequency of the transmission bandwidth of the SRS for positioning purpose. - Note: It is open to further discussion whether a frequency other than the center frequency of the UL carrier can also be the specific RF frequency for a non-default case(s), if RAN1 agrees to introduce them. Agreement - Support enabling a TRP to report UL RSCP together with RTOA and/or gNB Rx-Tx time difference measurements to LMF - Note 1: The report of UL carrier phase measurement with gNB Rx – Tx time difference does not necessarily require the report of DL carrier phase measurement with UE Rx – Tx time difference. - Note 2: This doesn’t preclude standalone UL carrier phase measurements reporting. Agreement Further study whether and how to support a UE/TRP to report the carrier phase measurement quality indication for corresponding the phase measurements. Agreement For NR UL carrier phase positioning for UE in RRC_CONNECTED and RRC_INACTIVE states, support reuse of existing physical layer procedures for UL positioning (e.g., UL-TDOA), with necessary enhancements in the measurement configuration, measurement request and measurement report (e.g., the configuration related to the NR UL CPP). - FFS: the details of the enhancements. Agreement Adopt one of the following options for a timestamp associated with a reported RSCP/RSCPD measurement (make the decision in RAN1#113): - Option 1: - NR-TimeStamp, currently defined in TS 37.355, is reused as the timestamp with the granularity of a slot. - FFS: Whether to clarify in the specification the reported RSCP/RSCPD value presents the RSCP/RSCPD of a specific OFDM symbol within the slot identified by the NR-TimeStamp. - Option 2: - NR-TimeStamp, currently defined in TS 37.355, should be enhanced to include the OFDM symbol index in a slot, as the timestamp for RSCP/RSCPD measurements. Agreement To address the impact of the phase delays on Tx/Rx RF chains, support one or more of the following options (down-selection in RAN1#113): - Option 1a: introduce the definition of UE/TRP Tx/Rx phase error groups (PEGs) for the Tx/Rx of DL PRS/UL SRS signals - Rel-17 definitions of UE/TRP Tx/Rx TEGs can be used as the starting point for defining UE/TRP Tx/Rx PEGs. - FFS: the details of \the UE/TRP Tx/Rx PEGs - Option 1b: Introduce Tx/Rx RF antenna IDs or Tx/Rx RF chain IDs to identify the individual Tx/Rx RF chains for transmitting/receiving the DL PRS/UL SRS signals. - FFS: the details of the Tx/Rx RF antenna IDs or Tx/Rx RF chain IDs - Note: Device transmitting PRS or positioning SRS provides Tx antenna ID or Tx Chain ID. Device receiving PRS or positioning SRS provides Rx antenna ID or Rx Chain ID. - Option 1c: introduce the report of ARP ID for the Rx/Tx of DL PRS/UL SRS signals. - The transmission/reception associated with the same ARP ID is assumed from the same ARP. - FFS: the maximum number of ARP IDs. - Option 2: reuse or enhance the existing Rel-17 definitions of UE/TRP Tx/Rx TEGs with smaller margin value. - Option 3: RAN1 sends an LS to RAN4, requesting RAN4 to consider whether there is a need to define the new UE/TRP Tx/Rx phase error groups (PEGs), introduce new IDs (e.g., Tx/Rx RF antenna IDs ) to present the phase delays for the Tx/Rx of DL PRS/UL SRS signals, or reuse or enhance the existing Rel-17 definitions of UE/TRP Tx/Rx TEGs with smaller margin value, and provide the definitions if RAN4 decides it is needed.

## Agreements made in RAN1#113

Agreement Support the following definition of the reference point of the UE/TRP carrier phase measurements: - The reference point of the UE carrier phase measurements is defined the same as the reference point of RSTD for both frequency range 1 and frequency range 2. - The reference point of the TRP carrier phase measurements is defined the same as the reference point of RTOA for both frequency range 1 and frequency range 2. - Note: It is up to UE/TRP’s implementation on how to map the carrier phase to the reference point for measurement reporting. Agreement Adopt the following modifications on the agreements made in RAN1#112bis-e: To enable simultaneous transmission of UL SRS for positioning by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the serving gNB of a UE to configure the transmission of the UL SRS resources from the UE within indicated time window(s). - FFS: the details of the time window, e.g., the start time, duration, periodicity for the time window(s), within the vicinity of a reference SRS configuration or use the existing message of Scheduled Location time - Enabling LMF to request the serving gNB and neighboring gNBs of the UE to measure the UL SRS resources from the UE within indicated time window(s). - Note: this may be a different indicated time window To enable simultaneous measurements on same DL PRS by a target UE and a PRU, support the following enhancements: - Enabling LMF to request the UEs, including target UE and PRU(s), to perform measurements on indicated DL PRS resource sets occurring within indicated time window(s). - FFS: the details of the configuration of the indicated time window(s), e.g., the start time, duration, periodicity for the time window(s), as well as the relationship with the Scheduled Location time. Agreement For UE-based carrier phase positioning, support enabling LMF to forward the DL carrier phase measurement reported by a PRU, with additional information of the same PRU to a target UE for UE-based carrier phase positioning in the positioning assistance data. - Note: Whether the forwarded DL carrier phase measurement is DL RSCP and/or DL RSCPD depends at least on which of them is (are) supported by UE capability. - additional information of the same PRU includes at least PRU location. - FFS: additional PRU information, e.g. the AoD of PRU to each TRP, etc. Agreement If a UE reports RSCPD measurements together with RSTD measurements in a measurement report element, the reference TRP for RSCPD is the same as the reference TRP reported for RSTD. - The target and the reference TRP are in the same PFL Agreement - From RAN1’s perspective, carrier phase positioning for UE in RRC_IDLE state is supported for UE-based and UE-assisted positioning in Rel-18. - Note: No additional specification work is expected specifically related to carrier phase positioning for UE in RRC_IDLE state in RAN1. Conclusion From RAN1’s perspective, carrier phase positioning for UE in RRC_CONNECTED state without measurement gap is not supported in Rel-18. Working assumption - To enable LMF to optionally request the serving gNB of a UE to configure the transmission of the UL positioning SRS resources from the UE within indicated time window(s), support: - Option 1D: Each of the time windows is defined with the following parameters: - The start of the time window, which is indicated by a combination of subframe number, slot offset and symbol index with respect to the SFN initialization time - The duration of the time window, which is given by a number of consecutive slots/symbols - FFS: the number of the consecutive slots/symbols - (Optional) The periodicity of the time window, which is defined similar to IE PeriodicitySRS in “Requested SRS Transmission Characteristics” in TS 38.455. - FFS: the maximum number of the windows Agreement To enable LMF to request the serving gNB and neighboring gNBs of a UE to measure the UL SRS resources from the UE within indicated time window(s), each time window is defined with the following parameters: - The start of the time window, which is indicated by a combination of subframe number, slot offset and symbol index with respect to the SFN initialization time - The duration of the time window, which is given by a number of consecutive slots/symbols - FFS: the number of consecutive slots/symbols - (Optional) The periodicity of the time window, which is defined similar to IE Measurement Periodicity in MEASUREMENT REQUEST in TS 38.455. - FFS: the maximum number of the windows Agreement To enable LMF to request the UEs, including target UE and PRU(s), to perform measurements on indicated DL PRS resource set(s) occurring within indicated time window(s), each time window is defined with the following parameters: - The start of the time window, which is indicated by a combination of subframe number, slot offset and symbol index - The duration of the time window, which is given by a number of consecutive slots/symbols - FFS: the number of consecutive slots/symbols - (Optional) The periodicity of the time window, which is defined similar to IE NR-DL-PRS-Periodicity-and-ResourceSetSlotOffset in TS 37.355.FFS: the maximum number of the windows R1-2305972 FL Summary #3 for NR DL and UL carrier phase positioning Moderator (CATT)

## Agreements made in RAN1#114

Agreement - Confirm the following working assumption with modification made in RAN1#113: Working assumption - To enable LMF to optionally request the serving gNB of a UE to configure the transmission of the UL positioning SRS resources from the UE within indicated time window(s), support: - Option 1D: Each of the time windows is defined with the following parameters: - The start of the time window, which is indicated by a combination of system frame number, slot offset and symbol index with respect to the SFN initialization time - The duration of the time window, which is given by a number of consecutive slots/symbols - FFS: the number of the consecutive slots/symbols - (Optional) The periodicity of the time window, which is defined similar to IE PeriodicitySRS in “Requested SRS Transmission Characteristics” in TS 38.455. - FFS: the maximum number of the windows Agreement - When a LMF requests the serving gNB of a UE to configure the transmission of the UL positioning SRS resources from the UE within indicated time window(s), - the duration of a time window can be configured by one of the following values: - {1, 2, 4, 8, 12} OFDM symbols - {1, 2, 4, 6, 8, 12, 16} slots - FFS: additional values - the number of the time windows can be configured as: - {1, 2, …, 16} Agreement When a LMF requests the serving gNB and neighboring gNBs of a UE to measure the UL SRS resources from the UE within indicated time window(s): - The duration of a time window can be configured as follows: - {1, 2, 4, 6, 8, 12, 16} slots. - the number of the time windows can be: - {1, 2, …, 16} Agreement When an LMF requests the UEs, including target UE and PRU(s), to perform measurements on indicated DL PRS resource set(s) occurring within indicated time window(s) - The duration of a time window can be configured as follows: - {1, 2, 4, 6, 8, 12, 16} slots. - the number of the time windows can be: - {1, 2} - FFS: {4, 8} Agreement Each DL RSCP/RSCPD measurement instance is obtained with Nsample=1 sample only. Conclusion From RAN1’s perspective, the granularity and the range of the RSCP/RSCPD measurements can be defined by RAN4. Agreement Endorse the following RAN1 reply on PRU procedures RAN1 reply: Current RAN1 specifications do not support a mechanism to ensure simultaneous measurements/transmissions (e.g. in the same slot(s)) for multiple UEs, including a target UE and a PRU. RAN1 will continue discussions on what enhancements to LPP, NRPPa, and/or RAN signaling are necessary to support simultaneous measurements of the same DL-PRS for multiple UEs, including a target UE and a PRU; and to support simultaneous transmission of SRS for multiple UEs, including a target UE and a PRU. Note: The enhancements might or might not have RAN1 specification impact.. In above reply, RAN1 indicated that RAN1 would continue the discussions on the enhancements that are necessary to support simultaneous measurements of the same DL-PRS for multiple UEs, and the simultaneous transmission of SRS for multiple UEs, including a target UE and a PRU. In this LS, RAN1 informs SA2 that RAN1 has continued the discussion, and reached the following agreements related to the support of simultaneous measurements of the same DL-PRS for multiple UEs and the simultaneous transmission of SRS for multiple UEs, including a target UE and a PRU. Furthermore, it is RAN1’s understanding that the simultaneous measurements/transmissions for multiple UEs, including a target UE and a PRU, is applicable to RAN1’s on-going work related to NR carrier phase positioning, and is also applicable to the remaining uplink and downlink positioning measurements and methods. Therefore, RAN1’s agreements related to the NR carrier phase positioning are provided in Section 4 in this LS for information. To SA2: Action: RAN1 respectfully asks SA2 to take the above reply into account for future work. To RAN2/RAN3: Action: RAN1 respectfully asks RAN2/RAN3 to work on the necessary higher-layer signalling support, and provide feedback if there is any concern. Agreement Endorse the draft LS in R1-2308643. Final LS in R1-2308644. Agreement For the timestamp associated with a reported RSCP/RSCPD measurement, NR-TimeStamp, with the granularity of a slot, currently defined in TS 37.355, can be reused as the timestamp. - Subject to UE capability, a UE may optionally provide an OFDM symbol index in the timestamp. - Note: It is up to RAN2/RAN3 how to signal the timestamp Agreement When DL RSCPD/RSCP measurements are reported together with the DL RSTD/ UE Rx – Tx time difference measurements, the DL RSCPD/RSCP measurements are obtained from a single DL PFL only. Note: From RAN1’s perspective, the reporting of the carrier phase measurements from one DL PFL has no impact on the reporting of the DL RSTD and/or UE Rx – Tx time difference measurements from the same DL PFL or other DL PFLs. Agreement For UE-based carrier phase positioning, when LMF forwards the DL carrier phase measurement reported by a PRU to a target UE, the timestamp associated with the PRU carrier phase measurements should also be forwarded in positioning assistance data. Agreement Support UE/TRP to report the phase quality indication for the RSCP/RSCPD measurements. The phase quality indication includes the following fields: - phase quality index - phase quality resolution The values of the phase quality index and phase quality resolution are left for RAN4.