3GPP TSG RAN WG1 Meeting #114bis	R1-2310331

Xiamen, China, October 9th – October 13th, 2023

Agenda Item:	8.1.3.2

Source:	Moderator (Futurewei)

Title:	FL Summary #2 on SRS enhancements

Document for:	Discussion and decision

# Introduction

In RAN#94-e, a Work Item for Rel-18 on “MIMO Evolution for Downlink and Uplink” was approved, and the motivations, scopes, and objectives were agreed and then updated in [1]. Among the objectives, the underlined in the following are related to SRS enhancements, mainly in the aspects of SRS for TDD Coherent Joint Transmission (CJT or C-JT) and 8 Tx operation:

- Study, and if justified, specify enhancements of CSI acquisition for Coherent-JT targeting FR1 and up to 4 TRPs, assuming ideal backhaul and synchronization as well as the same number of antenna ports across TRPs, as follows:

  - Rel-16/17 Type-II codebook refinement for CJT mTRP targeting FDD and its associated CSI reporting, taking into account throughput-overhead trade-off

  - SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization, with the constraints that 1) without consuming additional resources for SRS; 2) reuse existing SRS comb structure; 3) without new SRS root sequences

  - Note: the maximum number of CSI-RS ports per resource remains the same as in Rel-17, i.e. 32

- Study, and if justified, specify UL DMRS, SRS, SRI, and TPMI (including codebook) enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices

  - Note: Potential restrictions on the scope of this objective (including coherence assumption, full/non-full power modes) will be identified as part of the study.

The Work Item has achieved 90% completion at RAN level by RAN#101 [2], and a set of Rel-18 specifications have been approved.

23 contributions [3-25] have been submitted to Agenda Item 8.1.3.2 of RAN1#114bis on SRS Enhancements targeting TDD CJT and 8 Tx operations. Main views and further discussion points based on these contributions are collected in this document. Any additional inputs from any company can also be provided in this document.

# SRS enhancements to manage inter-TRP cross-SRS interference targeting TDD CJT

## TS38.211 TPs

### Description of the bitmap for hopping subset

Lenovo pointed out that the description of the bitmap for hopping subsets is not clear. For example, the current specification has:

where scshSRSn and ncshSRS is the nth entry and the cardinality of the set

Scsh={scshSRS0, scshSRS1, …,scshSRSncshSRS-1}

respectively, where Scsh is given by the higher-layer parameter cyclicShiftHoppingSubset if configured, otherwise Scsh={0, 1,…,KnSRScs,max-1}.

…

where scohSRSn and ncohSRS is the nth entry and the cardinality of the set

Scoh={scohSRS0, scohSRS1, …,scohSRSncohSRS-1}
respectively, where Scoh is given by the higher-layer parameter combOffsetHoppingSubset if configured, otherwise Scoh={0, 1,…,KTC-1}.

The description of the bitmap could be alternatively captured in TS38.311, but so far it has not been captured. In any case, it makes sense to make this clear in RAN1 specs.

Lenovo proposed to change to:

where scshSRSn and ncshSRS is the nth entry and the cardinality of the set

Scsh={scshSRS0, scshSRS1, …,scshSRSncshSRS-1}

respectively, where Scsh is determined based on the bitmap indicated by the higher-layer parameter cyclicShiftHoppingSubset if configured, otherwise Scsh={0, 1,…,KnSRScs,max-1}. For the bitmap, the length is nSRScs,max and the bit with index scshSRSi-1 (1≤i≤ncshSRS) is the i-th bit set as 1.

The relevant agreement is:

Agreement

When a subset of comb offsets for comb offset hopping is configured, and when a subset of cyclic shifts for cyclic shift hopping is configured, support the following option for configuring the subset S={S(0), S(1), …, S(z-1)} with 1<z<Z, where Z=KTC for comb offset hopping and Z=nSRScs,max for cyclic shift hopping, and:

  - Option 1b: S(0), S(1), …, S(z-1) are configured via a Z-length bitmap with S(i-1) being the i-th bit set as 1.

A proposal is suggested with some rewording of Lenovo’s. Please also see the next Section 2.1.3 for the subset entry indexing which is related to these texts.

Proposal 2.1.1: Adopt the text proposal for TS38.211 on cyclic shift / comb offset hopping subset bitmap:

6.4.1.4.2	Sequence generation

<Unchanged text is omitted>

where scshSRSn and ncshSRS is the nth entry and the cardinality of the set

Scsh={scshSRS0, scshSRS1, …,scshSRSncshSRS-1}

respectively, where Scsh is given by the higher-layer parameter cyclicShiftHoppingSubset if configured, otherwise Scsh={0, 1,…,KnSRScs,max-1}. The higher-layer parameter [cyclicShiftHoppingSubset] includes a bitmap of nSRScs,max bits with 1<ncshSRS<nSRScs,max bits being set, where the nth bit being set corresponds to scshSRSn.

<Unchanged text is omitted>

6.4.1.4.3	Mapping to physical resources

<Unchanged text is omitted>

where scohSRSn and ncohSRS is the nth entry and the cardinality of the set

Scoh={scohSRS0, scohSRS1, …,scohSRSncohSRS-1}
respectively, where Scoh is given by the higher-layer parameter combOffsetHoppingSubset if configured, otherwise Scoh={0, 1,…,KTC-1}. The higher-layer parameter [combOffsetHoppingSubset] includes a bitmap of KTC bits with 1<ncohSRS<KTC bits being set, where the nth bit being set corresponds to scohSRSn.

<Unchanged text is omitted>

| Company | View |
| --- | --- |
| Apple | - We are open clarify |
| QC | - It seems from RAN1 perspective, the spec is already clear, and the red part can be instead captured in RRC spec. Having said that, we are ok either way, but should it be “being set to 1”? |
| Samsung | - Our view is that current specification is already clear. |
| NTT DOCOMO | - Open. If applied we echo QC’s addition “to 1” before “corresponds to …”. |
| Sharp | - We think this proposal is not essential because the description of higher layer parameter can be specified in RRC spec. |
| LGE | - Same view as QC. |
| ZTE | - We agree with QC and SS that current RAN1 spec is already clear. This point should be better captured in RRC spec. |
| vivo | - For the higher-layer parameter [combOffsetHoppingSubset] and [cyclicShiftHoppingSubset], generally it seems better to capture the details of the bitmap in TS 38.331. |
| OPPO | - Intend to agree with QC. |
| InterDigital | - Agree with vivo. |
| Google | - We failed to see the necessity. Current spec is clear. |
| Xiaomi | - We think it is sufficient that the added part is captured in RRC spec. |
| Lenovo | - We are fine with Proposal 2.1.1 with QC’s modification “being set to 1”. If the bitmap related description is put in TS 38.211, it is friendly for reading. Also, the similar description for bitmap also happens for description of CSI-RS pattern (section 7.4.1.5.3 in TS 38.211). Thus, we think both are OK with putting this description in TS 38.211 or TS 38.331 but prefer to include it in TS 38.211 for friendly reading. |
| Fujitsu | - Agree with Samsung that RAN1 spec is clear. |
| Nokia, NSB | - We don’t see much need for this. |
| New H3C | - Current spec is clear and proposal isn’t required |
| Huawei, HiSilicon | - Support. Agree with Lenovo’s assessment. |
| CATT | - Current specification is already clear. |
| Ruijie | - Not essential. Current spec is clear enough. |
| FL | - Most of the companies are ok with this proposal with Qualcomm’s correction. There are a few companies who believe this is not essential, which is reasonable if RAN2 spec captures the bitmap description, but so far it seems such a RAN2 description is not available. So I suggest we try this proposal during Monday online. |

Closed with the following agreement:

Proposal 2.1.1A:

Adopt the text proposal for TS38.211 on cyclic shift / comb offset hopping subset bitmap:

6.4.1.4.2	Sequence generation

<Unchanged text is omitted>

where  and is the (+1)th entry and the cardinality of the set

respectively, where  is given by the higher-layer parameter cyclicShiftHoppingSubset if configured, otherwise . The higher-layer parameter [cyclicShiftHoppingSubset] includes a bitmap of  bits with  bits being set, where the (n+1)th bit being set to 1 corresponds to

<Unchanged text is omitted>

6.4.1.4.3	Mapping to physical resources

<Unchanged text is omitted>

where  and is the (+1)th entry and the cardinality of the set

respectively, where  is given by the higher-layer parameter combOffsetHoppingSubset if configured, otherwise . The higher-layer parameter [combOffsetHoppingSubset] includes a bitmap of  bits with  bits being set, where the (n+1)th bit being set to 1 corresponds to

<Unchanged text is omitted>

### Hopping ID

Lenovo, Samsung, and Futurewei proposed to align the hopping ID notation / parameter name. The current specification uses two different notations for the cyclic shift hopping ID and comb offset hopping ID, but based on the following agreement, there should be only one new hopping ID.

Agreement

For SRS comb offset hopping and/or cyclic shift hopping, for a SRS resource, the hopping pattern initialization ID determined by cinit=nIDhop, where nIDhop is a new ID for cyclic shift hopping and/or comb offset hopping.

The following proposal is suggested. Note that Lenovo also proposed to modify the RRC parameter names and how they are configured; my personal view is that we can wait until the CR review phase after this meeting.

Proposal 2.1.2: Adopt the text proposal for TS38.211 on cyclic shift / comb offset hopping ID notation:

6.4.1.4.2	Sequence generation

<Unchanged text is omitted>

The pseudo-random sequence ci is defined by clause 5.2.1 and shall be initialized with cinit=nIDcshhop at the beginning of each radio frame for which nf mod 128=0, where the cyclic-shift hopping identity nIDcshhop is contained in the higher-layer parameter cyclicShiftHopping.

<Unchanged text is omitted>

6.4.1.4.3	Mapping to physical resources

<Unchanged text is omitted>

The pseudo-random sequence ci is defined by clause 5.2.1 and shall be initialized with cinit=nIDcohhop at the beginning of each radio frame for which nf mod 128=0, where the comb hopping identity nIDcohhop is contained in the higher-layer parameter combOffsetHopping.

<Unchanged text is omitted>

| Company | View |
| --- | --- |
| Apple | - We also think alignment of the notation is good |
| QC | - Ok. |
| Samsung | - We are okay. |
| NTT DOCOMO | - Support |
| Sharp | - Support |
| LGE | - OK |
| ZTE | - Okay to change the notation. |
| Vivo | - Support |
| OPPO | - Fine. |
| InterDigital | - Support. |
| Google | - We do not think this change is necessary. |
| Spreadtrum | - Support |
| Lenovo | - Support |
| Fujitsu | - Fine with the proposal. |
| Nokia, NSB | - Support. |
| NEC | - Support |
| New H3C | - OK |
| Huawei, HiSilicon | - Support. |
| CATT | - Support. |
| Ruijie | - Support. |
| FL | - Most companies support the “(n+1)th” part of this proposal. Will discuss during Monday online. |

Closed with the following agreement:

Proposal 2.1.2:

Adopt the text proposal for TS38.211 on cyclic shift / comb offset hopping ID notation:

6.4.1.4.2	Sequence generation

<Unchanged text is omitted>

The pseudo-random sequence  is defined by clause 5.2.1 and shall be initialized with  at the beginning of each radio frame for which , where the cyclic-shift hopping identity  is contained in the higher-layer parameter cyclicShiftHopping.

<Unchanged text is omitted>

6.4.1.4.3	Mapping to physical resources

<Unchanged text is omitted>

The pseudo-random sequence  is defined by clause 5.2.1 and shall be initialized with  at the beginning of each radio frame for which , where the comb hopping identity  is contained in the higher-layer parameter combOffsetHopping.

<Unchanged text is omitted>

### Subset entry indexing

Samsung proposed to change the subset entry indexing of s(n) from “the nth entry” to “the (n+1)th entry”. Though either can be well understood from the context, I checked and noticed that the current specification always follows the convention of the (n+1)th entry. Therefore, I suggest adopting Samsung’s proposal to be consistent with the rest of TS38.211. If the proposal in Section 2.1.1 is also considered, the corresponding indexes will also be updated.

Proposal 2.1.3: Adopt the text proposal for TS38.211 on cyclic shift / comb offset hopping subset entry indexing:

6.4.1.4.2	Sequence generation

<Unchanged text is omitted>

where scshSRSn and ncshSRS is the (n+1)th entry and the cardinality of the set

Scsh={scshSRS0, scshSRS1, …,scshSRSncshSRS-1}

respectively, where Scsh is given by the higher-layer parameter cyclicShiftHoppingSubset if configured, otherwise Scsh={0, 1,…,KnSRScs,max-1}. [The higher-layer parameter [cyclicShiftHoppingSubset] includes a bitmap of nSRScs,max bits with 1<ncshSRS<nSRScs,max bits being set, where the (n+1)th bit being set corresponds to scshSRSn. ]

<Unchanged text is omitted>

6.4.1.4.3	Mapping to physical resources

<Unchanged text is omitted>

where scohSRSn and ncohSRS is the (n+1)th entry and the cardinality of the set

Scoh={scohSRS0, scohSRS1, …,scohSRSncohSRS-1}
respectively, where Scoh is given by the higher-layer parameter combOffsetHoppingSubset if configured, otherwise Scoh={0, 1,…,KTC-1}. [The higher-layer parameter [combOffsetHoppingSubset] includes a bitmap of KTC bits with 1<ncohSRS<KTC bits being set, where the (n+1)th bit being set corresponds to scohSRSn.]

<Unchanged text is omitted>

| Company | View |
| --- | --- |
| Apple | - Discuss together with Proposal 2.1.1 |
| QC | - Support. |
| Samsung | - Support text “(n+1)th” only. |
| NTT DOCOMO | - Support |
| Sharp | - Support |
| LGE | - Support |
| ZTE | - Agree with SS, and support “(n+1)th” only. |
| Vivo | - Support the modification of “(n+1)th” |
| OPPO | - Agree with SS. |
| InterDigital | - Support the “(n+1)th” change. |
| Google | - Support the change of (n+1) only. |
| Lenovo | - Support the “(n+1)th” change. |
| Fujitsu | - Support the change of “(n+1)”. |
| Nokia, NSB | - Agree with Samsung. |
| NEC | - OK |
| New H3C | - OK |
| Huawei, HiSilicon | - Support. |
| CATT | - Support the change of (n+1) only. |
| Ruijie | - Support the “(n+1)th” change. |
| FL | - Most companies support the “(n+1)th” part of this proposal. Will discuss during Monday online. |

Closed with the following agreement:

Proposal 2.1.3A:

Adopt the text proposal for TS38.211 on cyclic shift / comb offset hopping subset entry indexing:

6.4.1.4.2	Sequence generation

<Unchanged text is omitted>

where  and is the th entry and the cardinality of the set

respectively, where  is given by the higher-layer parameter cyclicShiftHoppingSubset if configured, otherwise .

<Unchanged text is omitted>

6.4.1.4.3	Mapping to physical resources

<Unchanged text is omitted>

where  and is the th entry and the cardinality of the set

respectively, where  is given by the higher-layer parameter combOffsetHoppingSubset if configured, otherwise .

<Unchanged text is omitted>

### Bitmap size

Huawei proposed to explicitly capture that the subset size should be larger than 1, according to the following agreement.

Agreement

When a subset of comb offsets for comb offset hopping is configured, and when a subset of cyclic shifts for cyclic shift hopping is configured, support the following option for configuring the subset S={S(0), S(1), …, S(z-1)} with 1<z<Z, where Z=KTC for comb offset hopping and Z=nSRScs,max for cyclic shift hopping, and:

  - Option 1b: S(0), S(1), …, S(z-1) are configured via a Z-length bitmap with S(i-1) being the i-th bit set as 1.

Huawei’s suggestion is to add the following descriptions:

The higher-layer parameter cyclicShiftHoppingSubset includes more than one bit set as 1.

The higher-layer parameter combOffsetHoppingSubset includes more than one bit set as 1.

I think it should be further specified that the subset size is smaller than nSRScs,max or KTC as the agreement states, since otherwise the subset does not need to be configured. So the following proposal is suggested, but if companies think Proposal 2.1.1 is sufficient to address this issue, this proposal may be skipped.

Proposal 2.1.4: Adopt the text proposal for TS38.211 on cyclic shift / comb offset hopping subset size:

6.4.1.4.2	Sequence generation

<Unchanged text is omitted>

where scshSRSn and ncshSRS is the nth entry and the cardinality of the set

Scsh={scshSRS0, scshSRS1, …,scshSRSncshSRS-1}

respectively, where Scsh is given by the higher-layer parameter cyclicShiftHoppingSubset if configured, otherwise Scsh={0, 1,…,KnSRScs,max-1}. The higher-layer parameter [cyclicShiftHoppingSubset] includes more than 1 and less than nSRScs,max bits being set.

<Unchanged text is omitted>

6.4.1.4.3	Mapping to physical resources

<Unchanged text is omitted>

where scohSRSn and ncohSRS is the nth entry and the cardinality of the set

Scoh={scohSRS0, scohSRS1, …,scohSRSncohSRS-1}
respectively, where Scoh is given by the higher-layer parameter combOffsetHoppingSubset if configured, otherwise Scoh={0, 1,…,KTC-1}. The higher-layer parameter [combOffsetHoppingSubset] includes more than 1 and less than KTC bits being set.

<Unchanged text is omitted>

| Company | View |
| --- | --- |
| Apple | - Discuss together with Proposal 2.1.1 |
| QC | - Proposal 2.1.1 is sufficient. Even w/o Proposal 2.1.1, the corresponding description can be captured in RRC spec. |
| Samsung | - Our view is that current specification is already clear. |
| NTT DOCOMO | - Discuss together with 2.1.1. |
| LGE | - Same view as Samsung. |
| ZTE | - NOT needed. This point should be better captured in RRC spec. |
| vivo | - It can be captured in TS 38.331 together with the wording in section 2.1.1. |
| OPPO | - The same view as in Proposal 2.1.1. |
| InterDigital | - Agree with vivo. |
| Google | - Do not support |
| Xiaomi | - Discuss together with Proposal 2.1.1, and we think proposal 2.1.1 is sufficient. |
| Lenovo | - We think it can be discussed together with proposal 2.1.1. |
| Fujitsu | - Similar view as other companies, the TP is not needed. |
| Nokia, NSB | - Agree with Samsung. |
| New H3C | - Agree with samsung |
| Huawei, HiSilicon | - Support. |
| CATT | - Agree with Samsung. And the corresponding description could be captured in RRC spec. |
| Ruijie | - Same view as Samsung that current spec is clear enough. |

Closed with the agreement in Sec. 2.1.1.

### Other proposals for TS38.211

The following proposals are also for comb offset hopping / cyclic shift hopping. My initial assessment is that these changes are not absolutely necessary to TS38.211 (though maybe necessary to TS38.214). Companies’ views are appreciated.

- TP 1: InterDigital proposed to add restrictions of no hopping under TDM as follows:

6.4.1.4.2	Sequence generation

The quantity fcshnf ,ns,fμ,l' is given by

-	if the higher-layer parameter cyclicShiftHopping is not configured, or if cyclicShiftHopping is configured and nrofSRS-Ports-n8 equals ports8tdm:

fcshnf,ns,fμ,l'=0

- if the higher-layer parameter cyclicShiftHopping is configured and nrofSRS-Ports-n8 does not equal ports8tdm:

fcshnf ,ns,fμ,l'= 
scshSRSm=07c8nf mod 128Nslotframe,μNsymbslot+ns,fμNsymbslot+l0+l'+m2mmod ncshSRS

…

6.4.1.4.3	Mapping to physical resources

< Unchanged parts are omitted >

The quantity fcohnf,ns,fμ,l' is given by

-	if the higher-layer parameter combOffsetHopping is not configured, or if combOffsetHopping is configured and nrofSRS-Ports-n8 equals ports8tdm:

fcohnf, ns,fμ,l'=0

-	if the higher-layer parameter combOffsetHopping is configured and nrofSRS-Ports-n8 does not equal ports8tdm:

fcohnf ,ns,fμ,l'= 
scohSRSm=07c8nf mod 128Nslotframe,μNsymbslot+ns,fμNsymbslot+l0+l''+m2mmod ncohSRS

  - However, TS38.214 clearly specifies that the hopping and TDM cannot be configured to the same SRS resource, so it seems that the original TS38.211 is sufficient. Specifically, TS38.214 has:

The UE is not expecting that the cyclic shift hopping and the higher layer parameter [tdm] are configured simultaneously.

The UE is not expecting that the comb offset hopping and the higher layer parameter [tdm] are configured simultaneously.

- TP 2: Lenovo proposed to add descriptions as follows:

-	If the higher-layer parameter cyclicShiftHopping is configured when the higher-layer parameter usage in the SRS-ResourceSet is configured as ‘antennaSwitching’:

…

-	if the higher-layer parameter combOffsetHopping is configured when the higher-layer parameter usage in the SRS-ResourceSet is configured as ‘antennaSwitching’:

  - The proposal is technically correct. However, TS38.214 clearly specifies that the hopping can only be configured to SRS with usage antennaSwitching, so it seems that the original TS38.211 is sufficient.

- TP 3: Vivo proposed the following:

If the higher-layer parameter hoppingFinerGranularity is configured and the higher-layer parameter cyclicShiftHoppingSubset is not configured, K=2, otherwise K=1.

  - The proposal is technically correct. Generally speaking, the agreement that finer granularity and cyclic shift hopping subset cannot be configured at the same time should be captured in TS38.214 (which is discussed in Proposal 2.2.1 below), and TS38.211 just needs to be consistent with TS38.214’s description. So, it seems that the current TS38.211 can still work fine and may not be absolutely necessary to change, as long as TS38.214 captures this.

- TP 4: Xiaomi proposed to add ‘aperiodic/periodic/semi-persistent’ when describing hopping.

  - The proposal is technically correct. However, it seems not necessary as most other descriptions that are applicable to aperiodic/periodic/semi-persistent do not have such wording.

| Company | View |
| --- | --- |
| QC | - TP1: Agree with FL. Not needed as already captured in 214. - TP2: Agree with FL. Not needed as already captured in 214. - TP3: Not needed. The existing text captures the following two if conditions from the agreement in RAN1 #114: - If cyclicShiftHoppingSubset is not configured, S{n}=n. - If cyclicShiftHoppingFinerGranularity is not configured, Y=nSRScs,max and K=1. - If cyclicShiftHoppingFinerGranularity is configured, Y=K.nSRScs,max, and K=2 - TP4: Not necessary. |
| Samsung | - TP1 and TP2 are already captured in current specification. - TP3 and TP4 are not essential. |
| Sharp | - TP1/TP2/TP3/TP4: Not essential |
| LGE | - For TP1,2,4, agree with FL’s assessment. - For TP3, same view as QC. |
| ZTE | - Current spec is clear enough, and the above TPs are NOT needed. |
| vivo | - TP1/TP2/TP4: Not essential. - TP3: our original motivation is to clarify the value of K is directly determined by the higher layer parameter cyclicShiftHoppingFinerGranularity, i.e., as long as the higher layer parameter cyclicShiftHoppingFinerGranularity is configured, then K=2, no matter whether the subset is configured or not in theory. As for that finer granularity can’t be configured with the subset simultaneously, it is additional restriction which should be captured in TS 28.214. - Therefore, the key point is that if we remove “and the higher-layer parameter cyclicShiftHoppingSubset is not configured”, the spec seems still clear enough. So we think it can be removed considering the redundancy. |
| OPPO | - We think all the TPs are unnecessary. |
| Google | - Do not support |
| Xiaomi | - TP4 is proposed for TS 38.214, not for TS 38.211. In TS 38.214, some descriptions apply aperiodic/periodic/semi-persistent to illustrate what time domain type of SRS resources are adopted for some UE feature as follows. ---------------- TS38.214--------------------- ### 6.2.1 UE sounding procedure ---------------Omitted text --------------------- - Defining start RB index hopping for partial frequency sounding in different SRS frequency hopping periods for aperiodic/periodic/semi-persistent SRS based on the hopping pattern khop as described in clause 6.4.1.4.3 in [4, TS 38.211. If not configured, then start RB hopping is not enabled and khop is fixed to be 0 for all SRS symbols. ---------------Omitted text --------------------- - For cyclic shift hoppling and/or comb offset hopping, we have agreed that aperiodic/periodic/semi-persistent SRS resource are supported. This should be clarified in TS 38.214. |
| Lenovo | - TP2 is proposed only for accuracy and avoiding misunderstanding. We agreed it is also captured in TS 38.214. If the majority view is not necessary, we are fine with the majority view. |
| Fujitsu | - Similar view as other companies, the TPs are unnecessary. |
| Nokia, NSB | - We don’t think it is necessary. |
| New H3C | - Not essential |
| Huawei, HiSilicon | - TP1: Already captured in 214. - TP2: Already captured in 214. - TP3: Fine. - TP4: Fine given the further clarification by Xiaomi. |
| CATT | - All TPs are not essential. |
| Ruijie | - All the TPs seem not necessary. |

#### ROUND2

Based on the inputs, TP1 and TP2 are not needed.

@vivo: I think your view on TP3 makes sense, especially since we are also discussing Proposal 2.2.1 for TS38.214. We can give TP3 a bit more time to discuss. In any case, this seems to be a very minor issue.

@Xiaomi: Sorry I misunderstood. We can continue to discuss this in Sec. 2.2.3.

Proposal 2.1.5: Adopt the text proposal for TS38.211 on cyclic shift hopping finer granularity:

6.4.1.4.2	Sequence generation

<Unchanged text is omitted>

If the higher-layer parameter hoppingFinerGranularity is configured and the higher-layer parameter cyclicShiftHoppingSubset is not configured, K=2, otherwise K=1.

<Unchanged text is omitted>

| Company | View |
| --- | --- |
| New H3C | - OK |
| Xiaomi | - The current spec is clear. We prefer to not removing it. |
| Nokia, NSB | - Fine with removing it. Then, we can |
| Samsung | - We can live with it if we consider Proposal 2.2.1B together. |
| Lenovo | - We are fine with it on account of the related discussion of Proposal 2.2.1B. |
| OPPO | - We are fine to remove it. |
| CMCC | - OK to remove it if we have Proposal 2.2.1B. |
| Huawei, HiSilicon | - Fine. |
| LGE | - Same view as Samsung. |
| Sharp | - OK |

## TS38.214 TPs

### Cyclic shift hopping subset and finer granularity configuration

Lenovo, Samsung, vivo, and Futurewei pointed out that TS38.214 should capture the agreement that cyclic shift hopping subset and finer granularity cannot be configured simultaneously to a SRS resource. Several different ways to capture this were provided, with no essential differences. Below proposal is provided, which should also be sufficient to address vivo’s comment in Section 2.1.5.

Proposal 2.2.1: Adopt the text proposal for TS38.214 on cyclic shift hopping subset and finer granularity configuration:

6.2.1	UE sounding procedure

<Unchanged text is omitted>

-	Cyclic shift, as defined by the higher layer parameter cyclicShift-n2, cyclicShift-n4, or cyclicShift-n8 for transmission comb value 2, 4 or 8, and described in clause 6.4.1.4 of [4, TS 38.211]. When cyclic shift hopping is configured by the higher layer parameter [cyclicShiftHopping] for an SRS resource in an SRS resource set with the usage configured as ‘antennaSwitching’, subject to UE capabilities, cyclic shift is updated at every symbol as described in [clause 6,4,1,4 of [4, TS 38.211]]. For the cyclic shift hopping, a UE can be configured with a subset of cyclic shifts by the higher layer parameter [cyclicShiftHoppingSubset], where the cyclic shift hopping is performed only across the cyclic shifts configured in the subset. For the cyclic shift hopping, a UE can be configured with finer hopping granularity of K=2 by the higher layer parameter [hoppingFinerGranularity]. The UE is not expecting that [hoppingFinerGranularity] is configured when [cyclicShiftHoppingSubset] is configured for an SRS resource. The UE is not expecting that the cyclic shift hopping and the higher layer parameter [tdm] are configured simultaneously.

<Unchanged text is omitted>

| Company | View |
| --- | --- |
| Apple | - We also think clarification is better |
| QC | - It seems the restrictions of RRC configuration does not belong to RAN1 spec. Anyway, the existing text in 211 (w/o TP3 in the previous section) seems to be enough. |
| Samsung | - We are fine with having such clarification. |
| NTT DOCOMO | - Open. |
| ZTE | - Support such clarification. |
| vivo | - Support the proposal. - And an additional TP can be considered which is inspired by FL, i.e., “The UE is not expecting that the cyclic shift hopping and the higher layer parameter [tdm] are configured simultaneously for an SRS resource” to clarify that these two higher layer parameters are for the same resource, if needed. |
| OPPO | - Fine with the proposal. |
| InterDigital | - Agree with vivo. |
| Google | - OK to capture it either in RAN1 spec or 38.331. |
| Spreadtrum | - Fine with the proposal. |
| Xiaomi | - We are fine to clarify. |
| Lenovo | - Support the proposal. |
| Fujitsu | - Fine to have the clarification. |
| Nokia, NSB | - We share view with Qualcomm. TS38.211 text is enough. - TS38.211 “If the higher-layer parameter hoppingFinerGranularity is configured and the higher-layer parameter cyclicShiftHoppingSubset is not configured, K=2, otherwise K=1.” |
| New H3C | - Support |
| Huawei, HiSilicon | - Support. |
| CATT | - Support the proposal. The clarification is necessary. |
| Ruijie | - Fine with the clarification. |
| FL | - Most companies are fine with this proposal, and we can further consider vivo’s new suggestion to further clarify “for an SRS resource”. - @QC, Nokia, NSB: My impression is that it is common to describe if some configuration combinations are not allowed in TS38.214. I suggest we give this a try in Monday online. |

#### ROUND2

It seems that we need to consider some rewording of the previous proposal. The following is suggested:

Proposal 2.2.1B:

Adopt the text proposal for TS38.214 on cyclic shift hopping subset and finer granularity configuration:

6.2.1	UE sounding procedure

<Unchanged text is omitted>

-	Cyclic shift, as defined by the higher layer parameter cyclicShift-n2, cyclicShift-n4, or cyclicShift-n8 for transmission comb value 2, 4 or 8, and described in clause 6.4.1.4 of [4, TS 38.211]. When cyclic shift hopping is configured by the higher layer parameter [cyclicShiftHopping] for an SRS resource in an SRS resource set with the usage configured as ‘antennaSwitching’, subject to UE capabilities, cyclic shift is updated at every symbol as described in [clause 6,4,1,4 of [4, TS 38.211]]. For the cyclic shift hopping, a UE can be configured with a subset of cyclic shifts by the higher layer parameter [cyclicShiftHoppingSubset], where the cyclic shift hopping is performed only across the cyclic shifts configured in the subset. For the cyclic shift hopping, a UE can be configured with finer hopping granularity of K=2 by the higher layer parameter [hoppingFinerGranularity]. The UE is not expecting that [hoppingFinerGranularity] is configured when [cyclicShiftHoppingSubset] is configured for an SRS resource. The UE is not expecting that the cyclic shift hopping and the higher layer parameter [tdm] are configured simultaneously for an SRS resource.

<Unchanged text is omitted>

| Company | View |
| --- | --- |
| New H3C | - OK |
| Xiaomi | - We are OK. |
| Nokia, NSB | - OK with agreeing both Proposal 2.1.5 and Proposal 2.2.1B |
| Samsung | - We are fine with this. |
| Lenovo | - We are fine with this. |
| OPPO | - Fine with the proposal. |
| CMCC | - OK with this. |
| Huawei, HiSilicon | - Fine. |
| LGE | - OK with this proposal. |
| Ruijie | - Fine with the proposal. |
| Sharp | - OK |

### Cyclic shift / comb offset hopping combined with one of group / sequence hopping

We had the following agreement:

Agreement

Whether SRS cyclic shift hopping can be combined with one of group / sequence hopping on a SRS resource depends on UE feature/capability design.

Lenovo pointed out that TS38.214 should capture this agreement, i.e., explicitly specifying that cyclic shift / comb offset hopping can be combined with one of group / sequence hopping for a SRS resource depending on UE feature/capability design. Such a combination is sometimes not explicitly captured in TS38.214 (partially because there could be many such combinations, and these combinations are possible in implementations unless the specs exclude them). As long as TS38.214 does not exclude the combination and UE feature/capability design support it, it can be considered as supported in the standards. Anyway, we can see if companies think the following proposal is agreeable or not. Needless to say, this should also be discussed in UE feature/capability design.

Proposal 2.2.2: Adopt the text proposal for TS38.214 on cyclic shift / comb offset hopping combined with one of group / sequence hopping:

6.2.1	UE sounding procedure

<Unchanged text is omitted>

-	Cyclic shift, as defined by the higher layer parameter cyclicShift-n2, cyclicShift-n4, or cyclicShift-n8 for transmission comb value 2, 4 or 8, and described in clause 6.4.1.4 of [4, TS 38.211]. When cyclic shift hopping is configured by the higher layer parameter [cyclicShiftHopping] for an SRS resource in an SRS resource set with the usage configured as ‘antennaSwitching’, subject to UE capabilities, cyclic shift is updated at every symbol as described in [clause 6,4,1,4 of [4, TS 38.211]]. For the cyclic shift hopping, a UE can be configured with a subset of cyclic shifts by the higher layer parameter [cyclicShiftHoppingSubset], where the cyclic shift hopping is performed only across the cyclic shifts configured in the subset. The UE is not expecting that the cyclic shift hopping and the higher layer parameter [tdm] are configured simultaneously. The UE can be configured with SRS cyclic shift hopping and one of group/sequence hopping on an SRS resource subject to UE capability.

…

-	Transmission comb offset, as defined by the higher layer parameter combOffset-n2, combOffset-n4, and combOffset-n8 for transmission comb value 2, 4, or 8, and described in clause 6.4.1.4 of [4, TS 38.211]. When comb offset hopping is configured by the higher layer parameter [combOffsetHopping] for an SRS resource in an SRS resource set with the usage configured as ‘antennaSwitching’, subject to UE capabilities, transmission comb offset(s) are updated as described in [clause 6,4,1,4 of [4, TS 38.211]]. For the comb offset hopping, a UE can be configured with a subset of comb offsets by the higher layer parameter [combOffsetHoppingSubset], where the comb offset hopping is performed only across the comb offsets configured in the subset. The UE is not expecting that the comb offset hopping and the higher layer parameter [tdm] are configured simultaneously. The UE can be configured with SRS comb offset hopping and one of group/sequence hopping on an SRS resource subject to UE capability.

<Unchanged text is omitted>

| Company | View |
| --- | --- |
| Apple | - We also think clarification is better |
| QC | - Not needed. The UE feature is enough. |
| Samsung | - We are fine with this clarification. |
| NTT DOCOMO | - We think UE feature description seems to be enough. |
| Sharp | - Not essential |
| LGE | - Similar view as QC, DOCOMO. |
| ZTE | - Support such clarification. |
| Vivo | - Not needed |
| OPPO | - We also think it is not essential. |
| Google | - Do not support |
| Spreadtrum | - Support |
| Xiaomi | - Discussed in UE feature. |
| Lenovo | - We think it is better for this clarification. If we check TS 38.214 specification, the similar description on UE capability appears many times. Thus, we support proposal 2.2.2. |
| Fujitsu | - Agree with QC and NTT DOCOMO. |
| NEC | - Support |
| New H3C | - Open to discuss |
| Huawei, HiSilicon | - Agree with FL’s assessment. |
| CATT | - It is not essential |
| Ruijie | - Not needed since current description is enough. |

Based on the inputs, we can close the discussion for now. We may revisit after UE feature discussions, if needed.

### Other proposals for TS38.214

The following proposals are also for comb offset hopping / cyclic shift hopping. My initial assessment is that they are not absolutely necessary to TS38.214. Companies’ views are appreciated.

- TP 1: Samsung proposed to wrap the (separate) description of comb offset hopping pattern with repetition into the description of transmission comb offset.

  - This makes logical sense. However, this seems to be an editorial TP and does not require any technical discussion. It can be proposed during CR review phase and the editor can handle it.

- TP 2: Samsung proposed to add the description of hopping ID being contained in cyclicShiftHopping and/or combOffsetHopping.

  - This can be one logical way to design the RRC IE/fields. My feeling is that exactly how this will be implemented depends on RAN2’s decision. So I suggest we wait for RAN2’s decision.

| Company | View |
| --- | --- |
| QC | - Not sure which Samsung TP the TP 1 above is referring to. But we noticed that the following change suggested by Samsung is an essential correction. In fact, the endorsed CR (R1-2308707) captured it correctly, but somehow in the final spec, this is not same as the endorsed CR. k0(pi)=nshiftNscRB+kTC(pi)+koffsetl'+fcoh(nf,ns,fμ,l') kTC(pi)+koffsetl'+fcoh(nf,ns,fμ,l') mod KTC |
| Samsung | - Our view is that TP1 refers what QC mentioned as above equation. We are fine either way having agreement during the meeting or editor can handle on CR review phase. - Re TP2, we are fine with FL’s suggestion. |
| NTT DOCOMO | - Okay to go with TP1 without technical discussion. |
| ZTE | - Re TP1, support to correct the equation as QC mentioned. - Re TP2, we agree with FL that it depends on RAN2’s decision. |
| Google | - Agree with the equation correction mentioned by QC |
| Lenovo | - For TP1, support to update the equation as QC mentioned. - For TP2, agree with FL’s assessment. |
| Fujitsu | - Agree with QC’s suggestion on the equation correction. |
| Huawei, HiSilicon | - TP1: Agree with QC. - TP2: Agree with FL’s assessment. |
| Ruijie | - TP1: share similar view with QC. - TP2: wait for RAN2’s decision. |
| FL | - @QC, Samsung: That’s a good catch. Indeed the issue exists. However, this seems to be only an editorial issue, and can be corrected by the editor without any technical discussion/agreement. |

#### ROUND2

As commented above, the equation correction to be aligned with the approved CR can be viewed as an editorial issue.

@Xiaomi: We can discuss your suggested TP here.

Proposal 2.2.3: Adopt the text proposal for TS38.214 on comb offset hopping and/or cyclic shift hopping:

6.2.1	UE sounding procedure

<Unchanged text is omitted>

-	Cyclic shift, as defined by the higher layer parameter cyclicShift-n2, cyclicShift-n4, or cyclicShift-n8 for transmission comb value 2, 4 or 8, and described in clause 6.4.1.4 of [4, TS 38.211]. When cyclic shift hopping is configured by the higher layer parameter [cyclicShiftHopping] for an aperiodic/periodic/semi-persistent SRS resource in an SRS resource set with the usage configured as ‘antennaSwitching’, subject to UE capabilities, cyclic shift is updated at every symbol as described in [clause 6,4,1,4 of [4, TS 38.211]]. For the cyclic shift hopping, a UE can be configured with a subset of cyclic shifts by the higher layer parameter [cyclicShiftHoppingSubset], where the cyclic shift hopping is performed only across the cyclic shifts configured in the subset. The UE is not expecting that the cyclic shift hopping and the higher layer parameter [tdm] are configured simultaneously.

-	Transmission comb value, as defined by the higher layer parameter transmissionComb described in clause 6.4.1.4 of [4, TS 38.211].

-	Transmission comb offset, as defined by the higher layer parameter combOffset-n2, combOffset-n4, and combOffset-n8 for transmission comb value 2, 4, or 8, and described in clause 6.4.1.4 of [4, TS 38.211]. When comb offset hopping is configured by the higher layer parameter [combOffsetHopping] for an aperiodic/periodic/semi-persistent SRS resource in an SRS resource set with the usage configured as ‘antennaSwitching’, subject to UE capabilities, transmission comb offset(s) are updated as described in [clause 6,4,1,4 of [4, TS 38.211]]. For the comb offset hopping, a UE can be configured with a subset of comb offsets by the higher layer parameter [combOffsetHoppingSubset], where the comb offset hopping is performed only across the comb offsets configured in the subset. The UE is not expecting that the comb offset hopping and the higher layer parameter [tdm] are configured simultaneously.

<Unchanged text is omitted>

| Company | View |
| --- | --- |
| New H3C | - OK |
| Xiaomi | - For partial frequency sounding, it illustrates that the definition of start RB index hopping for aperiodic/periodic/semi-persistent SRS is based on the hopping pattern in current specification. - Cyclic shift hopping and comb offset hopping are new UE feature, and we have discussed and agreed that both two new features can be configured for aperiodic/periodic/semi-persistent SRS resource. Hence, such agreement should be captured in spec for clarification. |
| Samsung | - We are not sure what the difference between putting “aperiodic/periodic/semi-persistent” and not putting is. |
| CMCC | - Aperiodic/periodic/semi-persistent is all the current supported SRS time behavior. Not sure the difference between emphasizing it and not having it. |
| Huawei, HiSilicon | - Fine. |
| Xiaomi | @Samsung @CMCC, in our view, if the TP is not supported, people does not know whether all time domain types of SRS resource are supported. Such TP can provides the information to help clarifying this and better understanding the new feature, i.e., cyclic shift hopping and comb offset hopping. |
| Ruijie | Fine with the update. |

## Other proposals comb offset hopping / cyclic shift hopping:

There were proposals that are editorial / alignment related proposals and do not need to be discussed / agreed in this meeting. They can be submitted by individual companies during post-RAN1#114bis draft CR review process, or if companies agree then we can compile a list for the editors to consider. Whether/how they will be adopted will be determined by the editors.

The following proposals are also for comb offset hopping / cyclic shift hopping:

- Issue 1: RRC parameters

  - Several companies proposed TPs regarding RRC parameters. My feeling is that they may be updated after RAN2 CR is agreed, or be discussed in MIMO RRC parameter email discussion.

- Issue 2: Applicable usages for hopping

  - Huawei and Futurewei proposed to support cyclic shift / comb offset hopping for all SRS usages. Discussions and agreement would be needed if this is to be supported.

Views can be provided for the above proposals. Any other views, issues, and clarifications can also be provided.

| Company | View |
| --- | --- |
| QC | - For Issue 2, we support the proposal as there is no reason to limit this feature only to antenna switching, especially given that fact that relaxing this restriction does not have any spec impact. |
| Samsung | - Issue 1 can be handled after RAN2’s decision is stable. - Issue 2 is not essential and this agenda item aims on SRS for TDD CJT. |
| NTT DOCOMO | - We support not to limit this feature for other usages in Issue 2. |
| ZTE | - Re issue 2, we support the proposal, because SRS with other usages could also cause interference. |
| Google | - Issue 2 is not essential |
| Xiaomi | - Issue 1: We prefer that RRC parameters related can be discussed in MIMO RRC parameter email discussion. - Issue 2: We are fine to further discussion in this meeting. |
| Lenovo | - For issue 1, we are fine with Moderator’s assessment that they can be deal with after RAN1’s further discussion and RAN’2 final decision. - For issue 2, it is discussed in previous meetings but there is no agreement here since it may be related with the scope of this work item. So we are open for the discussion. |
| Nokia, NSB | - We support the proposal in Issue 2. |
| Huawei, HiSilicon | - Issue 1: Agree with FL’s assessment. - Issue 2: Can be discussed in this meeting. |
| Ruijie | - Issue 1: agree with FL. - Issue 2: open to discussion. |

#### ROUND2

Issue 1 can be closed.

For Issue 2:

- Support to include CB/NCB/BM SRS: QC, NTT DOCOMO, Nokia, NSB, Huawei, HiSilicon, ZTE, Futurewei

- Concerns: Samsung, Google

We can give the following proposal a try. However, if a quick consensus is not achieved, we should close this discussion with a conclusion.

Proposal 2.3: SRS comb offset hopping / cyclic shift hopping can be configured for a SRS resource in a SRS resource set with usage ‘codebook’. NCB and BM are not supported for comb offset hopping / cyclic shift hopping

| Company | View |
| --- | --- |
| New H3C | - OK |
| Nokia, NSB | - Support |
| Samsung | - We still think that this is not essential. |

#### Tuesday offline consensus

Proposal 2.3A: SRS comb offset hopping / cyclic shift hopping can be configured for a SRS resource in a SRS resource set with usage ‘codebook’.

- SRS comb offset hopping / cyclic shift hopping are NOT supported for a SRS resource in a SRS resource set with usage ‘nonCodebook’ or ‘beamManagement’.

# SRS enhancements targeting 8 Tx operation

## Collision handling for TDMed ports

We had the following agreement for further study:

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), and when the SRS transmission on a subset of the s OFDM symbols within a group of {1, 2, …, s} is dropped, study at least the following solutions:

- Whether or not a UE drops the SRS transmission on the rest of OFDM symbols within the group of {1, 2, …, s}, based on, for example, the usage, coherency, and/or repetition configuration.

- Whether or not a UE changes the transmission order of the subsets of ports.

The general positions are:

Option 1: When the SRS transmission on a subset of the s OFDM symbols within a group of {1, 2, …, s} is dropped, UE drops the SRS transmission on some or all of the rest of OFDM symbols within the one or more groups of {1, 2, …, s} in a slot, based on, for example, the usage, coherency, and/or repetition factor.

  - Supporting: Futurewei, Google, Huawei, HiSilicon, KDDI, LG, NEC, Nokia, Nokia Shanghai Bell, NTT DOCOMO, Qualcomm, Ruijie, Sharp, Spreadtrum, Xiaomi, ZTE (14 15 proponents)

  - Option 1A: Considering usage / coherency, e.g., applied to usage ehaviou with fully/partially coherent antenna ports

    - Futurewei, KDDI, LG, Nokia, Nokia Shanghai Bell, NTT DOCOMO, Ruijie, ZTE

  - Option 1B: Considering repetition factor

    - Huawei, HiSilicon, Sharp, ZTE

  - Option 1C: Considering UE preparation time

    - Futurewei, KDDI, Qualcomm

Option 2: Legacy per-OFDM symbol based dropping rules are kept for TDMed 8-port SRS.

  - Supporting: Apple, CATT, CMCC (enhance SRI), Ericsson, Fijitsu, KDDI, Lenovo, NTT DOCOMO (open), OPPO, Qualcomm (enhance coherent assumption), Ruijie, Samsung, vivo, Xiaomi (10 14 proponents)

The opinions are quite split. We should first decide between Option 1 and Option 2; to this aim, we can try the following proposal which is to support Option 1. If Option 1 is agreed, further decision is required regarding the how the dropping depends on SRS repetition factor and/or UE preparation time for SRS transmission.

Proposal 3.1: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ and with TDM factor s = 2, the 8 ports being fully/partially coherent, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), and when the SRS transmission on a subset of the s OFDM symbols within a group of {1, 2, …, s} is dropped, the UE can drop the SRS transmission on the rest of OFDM symbols within the group of {1, 2, …, s}, depending on SRS repetition factor and/or UE preparation time for SRS transmission.

It should be understood that if no consensus is achieved for supporting Option 1, the legacy rules of per-symbol dropping will be applied, which is the following proposed conclusion. However, the legacy per-symbol SRS dropping may lead to issues of SRI determination, which can be discussed in 8.1.4.2, or may be left for implementation as some companies suggested.

Proposed Conclusion: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ and with TDM factor s = 2, the 8 ports being fully/partially coherent, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), and when the SRS transmission on a subset of the s OFDM symbols within a group of {1, 2, …, s} is dropped, the UE can still transmit the SRS on the rest of OFDM symbols within the group of {1, 2, …, s}.

| Company | View |
| --- | --- |
| Samsung | - Since this is maintenance phase, we can live with legacy dropping rule. |
| NTT DOCOMO | - Open. Prefer having a new rule as in proposal 3.1, while we understand we are in the phase of maintenance. |
| Sharp | - For Proposal 3.1, there is the issue on SRI determination when the SRS is transmitted within Tproc,2. Otherwise, the scheduling restriction of high priority channel is needed. - For Proposed Conclusion, there is still the issue on SRI determination. For example, the UE needs to determine a spatial filter for coherent PUSCH transmission based on a partial/non-coherent SRS resource indicated by SRI, if 8 ports of the SRS resource are mapped on different slots/symbols. - For this situation, the issue on the SRI determination should be solved. Even if the issue of the SRI determination is discussed in 8.1.4.2, we think the same discussion would be repeated. - In our view, a straightforward way is that the non-overlapped SRS transmissions on non-consecutive OFDM symbols are not used for the SRI determination. The UE can assume that these SRS transmissions were not transmitted. In other words, the indicated SRI should be associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is combined with 8 SRS ports mapped on two or more consecutive OFDM symbols. - We prefer the following proposal: Proposed Conclusion: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ and with TDM factor s = 2, the 8 ports being fully/partially coherent, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), and when the SRS transmission on a subset of the s OFDM symbols within a group of {1, 2, …, s} is dropped, the UE can still transmit the SRS on the rest of OFDM symbols within the group of {1, 2, …, s}. For SRI determination, the UE does not use the SRS resource with 8 ports that are mapped on non-consecutive OFDM symbols. |
| LGE | - Support proposal 3.1. - We disagree that Option 2 is same as legacy since it introduces new behavior, i.e. partial port dropping for codebook-based UL operation. This will create more subsequent issues related to port/resource determination that would have a larger spec impact than Option 1. - One example is the SRI determination issue. The below figure is from R1-2309665. - In legacy, SRS resource for PUSCH transmission in SRI indication determined based on the most recent SRS transmission. So, in the above figure, the partially dropped SRS1 in slot2 is used for PUSCH transmission which has ambiguity for UE to determine PUSCH ports and for gNB to indicate TPMI. Then, in R1-2309665, it was proposed to use fully sounded SRS1 in Slot0 instead of SRS1 in Slot2 for the PUSCH transmission which obviously introduces different behaviour compared to legacy. - Moreover, if repetition within each slot is configured for SRS1 and if partial ports are dropped in-between repetitions, it also creates another issue on port/resource determination. - Option1 does not cause the above issues related to port/resource determination and has less spec impact. |
| ZTE | - Support option 1A and 1B. Besides, we’d like to further elaborate our points as follows: For an 8-port SRS resource configured with TDM and R≥2, and whose 8 ports are fully coherent, the 8 ports are mapped onto Ns = 2R OFDM symbols in a slot according to the pattern {{1, 2}, …, {1, 2}} (totally R groups of TDMed SRS symbols of {1, 2}), within each group of TDMed SRS symbols, when one SRS symbol is dropped due to collision with another higher-priority uplink signal, - If the other non-collided SRS symbol cannot be combined with another SRS symbol of the SRS resource within the SRS coherent time (e.g., 4 symbols) before or after the other non-collided SRS symbol to construct a full 8-port SRS transmission, UE should drop the other non-collided SRS symbol, - Otherwise, UE should transmit the other non-collided SRS symbol. As illustrated in the following figure, in this situation, UE should transmit the non-collided SRS symbols. We are open to hear other companies’ views on the SRS coherent time. |
| vivo | - Support option 2. - For full coherent case, it is up to gNB implementation to use the rest ports. - For partial/non coherent case, the rest ports can be used for new precoding. |
| OPPO | - We prefer Option 2. We can be fine with Option 1A. |
| QC | - We are fine to take either option in general. But we have a few comments for the formulation of the proposals. - For Proposal 3.1: “Considering UE preparation time” is not an option. It must be part of the proposal. Without taking care of UE preparing timeline, the whole proposal does not work. Furthermore, there is ambiguity in “the UE can drop the SRS transmission on the rest of OFDM symbols”. We think it is better to make the proposal more affirmative by “the UE can drops the SRS transmission on the rest of OFDM symbols”. - For the conclusion, we also think the coherency after dropping has to be clarified, as part of the conclusion. We request to add the following sub-bullet. - For full coherent or partial coherent PUSCH, the coherence between SRS and PUSCH is not resumed until the next SRS transmission without SRS dropping. - Without this sub-bullet, in the future, someone could point to UE vendors and claim full coherent UE failed coherency test with SRS dropping. |
| Google | - OK with proposal 3.1. |
| Spreadtrum | - Fine with proposal 3.1. SRI issue cannot be ignored for option 2. Left for implementation seems not to be a good solution which restricts the scheduling flexibility. Comparing the spec-effort, option 1 is more preferred. |
| Xiaomi | - We are open to both options. |
| Lenovo | - Support option 2. |
| Fujitsu | - This is not an essential issue. The legacy scheme can work. - We support Option 2. |
| Nokia, NSB | - Support FL proposal 3.1 |
| NEC | - Support FL proposal 3.1 |
| CMCC | - Support option 2. From network’s perspective, gNB may configure the same SRS resources for codebook and antenna switching to reduce the SRS overhead, so it is beneficial to have a unified design for all usage, coherency, and repetition configuration. So, we prefer legacy per-OFDM symbol-based dropping rules for TDMed 8-port SRS. - Some companies complain that SRI issue for option 2, it is not a big issue, which could be simply solved with further clarification, such as the indicated SRI in slot n is associated with the most recent transmission of SRS resource, where the SRS resource is combined with 8 ports SRS with same TCI state and same transmission power. |
| New H3C | - support |
| Huawei, HiSilicon | - Support FL proposal 3.1. |
| CATT | - Support option 2. Issues of SRI determination can be solved by implementation. |
| Ruijie | - Fine with FL proposal 3.1. |
| KDDI | - We support the FL proposal 3.1 and we prefer option 1A. - Especially for partial/non coherent SRS, it can be combined with the other SRS transmitted in a short time, as ZTE has shown. |
| FL | - The supporting lists are updated. Companies’ views are still quite split. - @QC: I think whether to change the wording may depend on the details of which of Options 1A, 1B, or 1C is adopted. The intention of the current formulation of Proposal 3.1 is to allow for further discussion of the dropping details, rather than ending with the proposal conclusion. - Regardless of whether which option is adopted, companies will need more time to discuss the subsequent details (down-selection among Options 1A, 1B, and 1C, or SRI handling). So I suggest we discuss during Monday online so that we can have more time to work on the next step this week. |

#### ROUND2

Based on today’s discussion, both options still stand, but we may start to consider the subsequent details for both options so that companies can have a more complete picture.

- For Proposal 3.1, we can list Options 1A, 1B, or 1C for potential down-selection.

- For the Proposed Conclusion, we can list a few possible ways to clarify the existing specification ehaviour. Companies seem not interested in complicated changes to the existing specifications.

Proposal 3.1A: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ and with TDM factor s = 2, the 8 ports being fully/partially coherent, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), and when the SRS transmission on a subset of the s OFDM symbols within a group of {1, 2, …, s} is dropped, the UE can drop the SRS transmission on the rest of OFDM symbols within the group of {1, 2, …, s}, depending on SRS repetition factor and/or UE preparation time for SRS transmission. Down select from:

- Option 1B: Whether the UE drops the SRS transmission on the rest of OFDM symbols within the group of {1, 2, …, s} depends on SRS repetition factor.

- Option 1C: Whether the UE drops the SRS transmission on the rest of OFDM symbols within the group of {1, 2, …, s} depends on UE preparation time for SRS transmission.

Proposed Conclusion: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ and with TDM factor s = 2, the 8 ports being fully/partially coherent, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), and when the SRS transmission on a subset of the s OFDM symbols within a group of {1, 2, …, s} is dropped, the UE can still transmit the SRS on the rest of OFDM symbols within the group of {1, 2, …, s}. Down select one from the following clarifications of the SRI association:

- Option 2A: The indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI, provided that all the ports in the SRS resource are transmitted. [For fully coherent]

- Option 2B: The indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI, provided that all the ports in the SRS resource are transmitted, or some ports in the SRS resource are transmitted while other ports are dropped according to clause 6.2.1 of TS38.214.

- Option 2B-1: The indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI, provided that some ports in the SRS resource are transmitted while other ports are dropped according to clause 6.2.1 of TS38.214.

- Option 2B-2: The indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI, provided that at least some ports in the SRS resource are transmitted while other ports are dropped according to clause 6.2.1 of TS38.214.

- Option 2C: The indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI, provided that all the SRS ports corresponding to the antenna ports for scheduled PUSCH transmission in the SRS resource are transmitted. [For fully coherent]

| Company | View |
| --- | --- |
| New H3C | - Ok in general |
| LGE | - Support Option 1C. - I think option 2B has ambiguity. It is unclear whether UE report it or gNB configure it between two different UE behaviors. Combining the two behaviors does not make sense. We propose to revise Option 2B as below. - Option 2B: The indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI, provided that all the ports in the SRS resource are transmitted, or some ports in the SRS resource are transmitted while other ports are dropped according to clause 6.2.1 of TS38.214. |
| Nokia, NSB | - We prefer simple operation. - Proposal 3.1A1: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ and with TDM factor s = 2, the 8 ports being fully/partially coherent, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), and when the SRS transmission on a subset of the s OFDM symbols within a group of {1, 2, …, s} is dropped, the UE can drop the SRS transmission on the rest of OFDM symbols within the group of {1, 2, …, s}. depending on SRS repetition factor and/or UE preparation time for SRS transmission. |
| Samsung | - Our view is that this is no consensus. Hence we think the original option 2 or not having anything (i.e., legacy symbol-level dropping) is enough. |
| Sharp | - Support Option 2A. |
| OPPO | - We prefer Option 2, e.g. legacy symbol level dropping. - Regarding SRI issue, we think Option 2B (including LGE’s version) is equivalent to current description in 38.214. In such case, maybe no conclusion is also fine for Option 2B. |
| Huawei, HiSilicon | - Our first preference is original FL proposal 3.1. Actually, the current Option 1B and 1C are towards different issues (phase discontinuity & transmission timeline) and should be jointly considered. - If finally we go with proposed conclusion due to the divergence, as explained during online, to avoid ambiguity for UE PUSCH ports determination and gNB TPMI calculation, at least further clarification is needed in 214. We prefer the following option 2C: - Option 2C: The indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI, provided that all the SRS ports corresponding to the antenna ports for scheduled PUSCH transmission in the SRS resource are transmitted. |
| Xiaomi | - We think if all the ports can be derived within one SRS resource it would work, we prefer Option 2C proposed by HW. |

#### Post-Tuesday offline

We had some good technical discussions during yesterday’s online, today’s offline, and several unofficial offline. A few high-level observations from me based on the discussions:

Overall, the issues we are trying to address are not ‘typical cases’. They do not seem to worth significant spec effort / changes.

TDMed 8-port SRS may not be typical cases, given the UE capability limitations. Furthermore, collisions leading to 4 out of the 8 ports are dropped could occur, but likely to be rare. Note that AP SRS has high priority, so dropping of AP SRS is even less unlikely, if not impossible.

In addition, fully-coherent 8-port SRS configured with TDM may be even less common. If more power is needed, configuring the fully-coherent 8-port SRS with repetition may be preferred over TDM. The gNB knows there could be risks with fully-coherent TDMed 8-port SRS; such as partial dropping or power imbalance between the 2 symbols. And if the very rare event of a partial dropping occurs, the gNB can trigger / wait for another SRS / PUSCH transmission, that is, implementation-based solutions should be sufficient to deal with this corner case.

If we focus on partial/non-coherent TDMed 8 Tx, partial dropping based on the current specifications can still work well. gNB can still use the other 4 ports which are transmitted.

Therefore, it seems that Option 1 is not necessary. We can focus on Option 2, and see if additional clarification is needed to avoid potential ambiguity in the current spec text “The indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI”.

If companies think the current spec text already covers the case of partial transmission of the TDMed 8-port SRS, and there is no ambiguity, no spec change is needed.

If companies want to clarify the current spec text, we could try the following alternatives:

    - Conclude this issue with a clarification, and leave it to the editor to decide if/how to capture it.

    - Conclude this issue with a TP with minimum changes to the current spec text.

There were some discussions on Huawei’s Option 2C. I personally think it may be reformulated into a UE assumption (e.g., the UE expects the antenna ports of scheduled PUSCH transmission correspond to only the SRS ports transmitted in the SRS resource), which may ease UE vendor’s concern, but this may be an overkill for such a small issue.

The following is suggested:

Proposal 3.1-1: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ / ‘antennaSwitching’ and with TDM factor s = 2, the 8 ports being fully/partially coherent, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), and when the SRS transmission on a subset of the s OFDM symbols within a group of {1, 2, …, s} is dropped, the UE can still transmit the SRS on the rest of OFDM symbols within the group of {1, 2, …, s}.

- The indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI, and some or all of the ports in the SRS resource are transmitted.

  - [Whether or how to capture this is left to the editor.]

| Company | View |
| --- | --- |
| Huawei, HiSilicon | - Although our preference is Option 2C or the updated version by FL, we can live with the current proposal, only if the wording is modified as following: - The indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI, and some or all of the ports in the SRS resource are transmitted. - [Whether or how to capture this is left to the editor.] |
| OPPO | - We are fine with the intention of the proposal. However, the main bullet, e.g. per symbol SRS dropping, is the same as legacy. For the sub-bullet, “some or all of the ports in the SRS resource are transmitted” covers all cases for “transmission of SRS resource” (since if all ports are dropped, there is no transmission of SRS resource, and cannot be associated with SRI). In this case, is there any specification impact for this proposal? |
| ZTE | - We share the same view with OPPO. Since the red part actually does not make any difference, we prefer to take the proposal as a conclusion with no spec impact. |
| QC | - I think we have had enough discussion about SRS to PUSCH association and it seems we don’t go anywhere. By the way, we don’t agree with this phrase “and some or all of the ports in the SRS resource are transmitted”, because we don’t see this phrase adds any information or clarify anything (i.e., the red phrase has no spec impact). - However, taking it as a conclusion without spec impact is not the right way to go. We think our concern is not addressed with just a conclusion. The concern from our side is about phase coherence. Can we address coherence directly by adding the following to the end of the proposal? - For full coherent or partial coherent PUSCH, the coherence between SRS and PUSCH is not required until after the next SRS transmission without SRS dropping. |
| Sharp | - We support Huawei’s proposal. The below figure from R1-2309665 is SRI determination example. For this figure, the indicated SRI is associated with the SRS resource in slot 2 where ‘some of the ports in the SRS resource are transmitted’. However, the SRS resource in slot 2 should not be applied to full-coherent PUSCH transmission. Therefore, “some or” should be removed. On the other hand, the indicated SRI is associated with the SRS resource in slot 0 where ‘all of the ports in the SRS resource are transmitted’. The SRS resource in slot 0 is proper to be applied to full-coherent PUSCH transmission. |

#### Update for Wednesday Online

Thank you all for the discussions. Based on the inputs, I further updated the proposal as follows. Please note that the note provided by Qualcomm is primarily from the perspective of RAN4 UE requirement, and may not have a direct RAN1 impact.

Proposal 3.1-1A: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ / ‘antennaSwitching’ and with TDM factor s = 2, the 8 ports being fully/partially coherent, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), and when the SRS transmission on a subset of the s OFDM symbols within a group of {1, 2, …, s} is dropped, the UE can still transmit the SRS on the rest of OFDM symbols within the group of {1, 2, …, s}.

- The indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI, and where some or all of the ports in the SRS resource are transmitted.

  - [Whether or how to capture this is left to the editor.]

  - Note: If the 8 ports are also fully coherent, the UE is not required to maintain the coherence between the partial SRS transmission and the PUSCH scheduled by the PDCCH.

  - Note (Alt): The UE expects the antenna ports of scheduled PUSCH transmission correspond to only the SRS ports transmitted in the SRS resource.

## TS38.214 TPs

### 8T8R issues in TS38.214 Clause “6.2.1.2	UE sounding procedure for DL CSI acquisition”

Several companies suggested modifications to the current TS38.214 regarding 8T8R.

- TP 1: CATT, Lenovo, and NTT DOCOMO proposed to remove the limitation of no more than one SRS resource set to be triggered/configured:

For 1T2R, 1T4R, 2T4R, 1T6R, 1T8R, 2T6R, 2T8R, or 4T8R, or 8T8R, the UE shall not expect to be configured or triggered with more than one SRS resource set with higher layer parameter usage set as 'antennaSwitching' in the same slot. For 1T=1R, 2T=2R 4T=4R, or 8T=8R, the UE shall not expect to be configured or triggered with more than one SRS resource set with higher layer parameter usage set as 'antennaSwitching' in the same symbol.

  - This seems reasonable, since the limitation was meant for the case of xTyR with x<y. We can see if companies are fine with this TP, which is formulated into Proposal 3.2.1-1 below.

  - NTT DOCOMO also suggested an alternative, which differentiates the cases of TDM and non-TDM. We can further consider this, but the above TP could be a simpler solution.

- TP 2: Google proposed to remove the brackets so that 2 SP SRS resource sets can be allowed for 8T8R:

-	For 1T=1R, or 2T=2R, 4T=4R or 8T=8R, up to two SRS resource sets each with one SRS resource can be configured, where the number of SRS ports for each resource is equal to 1, 2,  4 or 8 if the UE is not indicating srs-AntennaSwitching2SP-1Periodic. Two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' can be configured and the two SRS resource sets configured with 'semi-persistent' are not activated at the same time, or up to two SRS resource sets can be configured, if the UE is indicating srs-AntennaSwitching2SP-1Periodic, where each SRS resource set has one SRS resource, the number of SRS ports for each resource is equal to 1, 2,  4[, or 8] or

  - This seems reasonable. We can see if companies are fine with removing the brackets. This is formulated into Proposal 3.2.1-2 below.

- TP 3: Samsung proposed to describe 8T8R based on a new, separate UE capability:

When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set as 'antennaSwitching', the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitch ('t1r2' for 1T2R, 't1r1-t1r2' for 1T=1R/1T2R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't8r8' for 8T8R, 't1r1-t1r2-t1r4' for 1T=1R/1T2R/1T4R, 't1r4-t2r4' for 1T4R/2T4R, 't1r1-t1r2-t2r2-t2r4' for 1T=1R/1T2R/2T=2R/2T4R, 't1r1-t1r2-t2r2-t1r4-t2r4' for 1T=1R/1T2R/2T=2R/1T4R/2T4R, 't1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r1-t2r2' for 1T=1R/2T=2R, 't4r4' for 4T=4R, or 't1r1-t2r2-t4r4' for 1T=1R/2T=2R/4T=4R) or the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitchBeyond4Rx (‘t1r1’ for 1T=1R, ‘t2r2’ for 2T=2R, ‘t1r2’ for 1T2R, ‘t4r4’ for 4T=4R, ‘t2r4’ for 2T4R, ‘t1r4’ for 1T4R, ‘t2r6’ for 2T6R, ‘t1r6’ for 1T6R, ‘t4r8’ for 4T8R, ‘t2r8’ for 2T8R, ‘t1r8’ for 1T8R) or the UE may be configured with only one of the following configurations depending on the indicated UE capability [newUECapabilitySupporting8T8R] [(‘t8r8’ for 8T=8R)]:

  - It seems this depends on the UE capability discussion, and we may revisit based on the outcomes of UE capability discussions.

Proposal 3.2.1-1: Adopt the text proposal for TS38.214 on removing the limitation of no more than one SRS resource set to be triggered/configured:

6.2.1.2	UE sounding procedure for DL CSI acquisition

<Unchanged text is omitted>

For 1T2R, 1T4R, 2T4R, 1T6R, 1T8R, 2T6R, 2T8R, or 4T8R, or 8T8R, the UE shall not expect to be configured or triggered with more than one SRS resource set with higher layer parameter usage set as 'antennaSwitching' in the same slot. For 1T=1R, 2T=2R 4T=4R, or 8T=8R, the UE shall not expect to be configured or triggered with more than one SRS resource set with higher layer parameter usage set as 'antennaSwitching' in the same symbol.

<Unchanged text is omitted>

Proposal 3.2.1-2: Adopt the text proposal for TS38.214 on allowing 2 SP SRS resource sets for 8T8R per UE capability:

6.2.1.2	UE sounding procedure for DL CSI acquisition

<Unchanged text is omitted>

-	For 1T=1R, or 2T=2R, 4T=4R or 8T=8R, up to two SRS resource sets each with one SRS resource can be configured, where the number of SRS ports for each resource is equal to 1, 2,  4 or 8 if the UE is not indicating srs-AntennaSwitching2SP-1Periodic. Two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' can be configured and the two SRS resource sets configured with 'semi-persistent' are not activated at the same time, or up to two SRS resource sets can be configured, if the UE is indicating srs-AntennaSwitching2SP-1Periodic, where each SRS resource set has one SRS resource, the number of SRS ports for each resource is equal to 1, 2,  4[, or 8] or

<Unchanged text is omitted>

| Company | View |
| --- | --- |
| Samsung | - We are fine with Proposal 3.2.1-1 and Proposal 3.2.1-2. - Regarding TP3, we understand that the UE capability discussion should be handled first, but it is obvious that the legacy UE capability “supportedSRS-TxPortSwitch” cannot report “t8r8”. Hence, at least the 1st revision part of TP3 shall be needed. |
| NTT DOCOMO | - On 3.2.1-1, the current formulation is indeed our best preference. We just wonder whether TDMed case needs to be handled in a similar manner to downgrading configuration. - 3.2.1-2 is fine. - On 3.2.1-3, we understand the intention. And agree the first revision is necessary. The second revision may need to be discussed further considering UE feature design as well. Key point in our view is that FG for 8Tx SRS for AS can/need to indicate the support of downgrading configuration additionally. We thought it isn’t necessary since all the downgrading configuration for 8Rx is covered in Rel-17 capability. In the meanwhile we have identified in 8.16.1 that it may not be a common understanding. Would like to hear the group’s opinion. |
| Sharp | - Proposal 3.2.1-1: Support. - Proposal 3.2.1-2: Support. |
| LGE | - Regarding TP2, do we have agreement for this? I’m not sure it is essential. |
| ZTE | - We are fine with Proposal 3.2.1-1 and Proposal 3.2.1-2. - Re TP3, we agree with FL that it depends on the UE capability discussion. |
| vivo | - Proposal 3.2.1-1: Support. - Proposal 3.2.1-2: Support. |
| OPPO | - Fine with TP1 and TP2. |
| QC | - Proposal 3.2.1-1: We don’t support the proposal. We understand that the intention of the TP. We agree that with 8T8R, there is only 1 SRS resource set needed so there is no motivation to configure more than 1 in practice. However, removing 8T8R can be interpret at NW can configure more than 1 SRS resource set for 8T8R, which is incorrect, and we believe it is not the intention of the TP neither. Anyway, keep the restriction for 8T8R does not hurt anything. So we think current specification is fine. - Proposal 3.2.1-2: We don’t support the proposal in current form. We are ok to support 8 ports SRS for “srs-AntennaSwitching2SP-1Periodic”. But it should be under a dedicated/new Rel-18 capability. So, if we add a Rel-18 UE capability, srs-AntennaSwitching2SP-1Periodic-8ports-SRS, we are fine. - Regarding TP3 from Samsung, we support it. We think a new Rel-18 capability should be introduced for 8T8R. |
| Google | - OK with both proposals. |
| Spreadtrum | - Fine with proposal 3.2.1-1 and 3.2.1-2. |
| Xiaomi | - Fine with Proposal 3.2.1-1 and Proposal 3.2.1-2 |
| Lenovo | - Proposal 3.2.1-1: Support. - Proposal 3.2.1-2: Support. |
| Fujitsu | - Proposal 3.2.1-1: The change seems not necessary. - Proposal 3.2.1-2: This requires more discussion. We could be fine if majority companies agree to have it. |
| Nokia, NSB | - Proposal 3.2.1-1: Support. - Proposal 3.2.1-2: Support. |
| CMCC | - Proposal 3.2.1-1: Support. - Proposal 3.2.1-2: Support. |
| New H3C | - Fine with proposal 3.2.1-1 and 3.2.1-2. |
| Huawei, HiSilicon | - Proposal 3.2.1-1: Support. - Proposal 3.2.1-2: Support. |
| CATT | - Proposal 3.2.1-1: Support. - In RAN#110 meeting, it was agreed that keep the existing value of the maximum number of SRS resource sets (as provided in Rel-17 antenna switching nTnR) for 8T8R. Therefore configured or triggered with more than one SRS resource set with higher layer parameter usage set as 'antennaSwitching' in the same slot shall be supported for 8T8R. - Proposal 3.2.1-2: Support. - Regarding TP3, we support it. |
| Ruijie | - Support proposal 3.2.1-1 and proposal 3.2.1-2. |

#### ROUND2

There is a large number of supporters for Proposal 3.2.1-1 and Proposal 3.2.1-2. We can continue to discuss.

@NTT DOCOMO: Could you please clarify “the current formulation” is the current proposal formulation or spec formulation?

@QC: For Proposal 3.2.1-1, it seems that companies wish to reuse existing xTxR mechanism as much as possible, and not to introduce any new limitations.

Both Proposal 3.2.1-2 and TP3 actually make sense but the descriptions depend on UE feature decisions. Maybe we can try a placeholder approach.

Proposal 3.2.1-1: Adopt the text proposal for TS38.214 on removing the limitation of no more than one SRS resource set to be triggered/configured:

6.2.1.2	UE sounding procedure for DL CSI acquisition

<Unchanged text is omitted>

For 1T2R, 1T4R, 2T4R, 1T6R, 1T8R, 2T6R, 2T8R, or 4T8R, or 8T8R, the UE shall not expect to be configured or triggered with more than one SRS resource set with higher layer parameter usage set as 'antennaSwitching' in the same slot. For 1T=1R, 2T=2R 4T=4R, or 8T=8R, the UE shall not expect to be configured or triggered with more than one SRS resource set with higher layer parameter usage set as 'antennaSwitching' in the same symbol.

<Unchanged text is omitted>

Proposal 3.2.1-2A: Adopt the text proposal for TS38.214 on allowing 2 SP SRS resource sets for 8T8R per UE capability:

6.2.1.2	UE sounding procedure for DL CSI acquisition

<Unchanged text is omitted>

-	For 1T=1R, or 2T=2R, 4T=4R or 8T=8R, up to two SRS resource sets each with one SRS resource can be configured, where the number of SRS ports for each resource is equal to 1, 2,  4 or 8 if the UE is not indicating srs-AntennaSwitching2SP-1Periodic. Two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' can be configured and the two SRS resource sets configured with 'semi-persistent' are not activated at the same time, or up to two SRS resource sets can be configured, if the UE is indicating srs-AntennaSwitching2SP-1Periodic or [srs-AntennaSwitching2SP-1Periodic8T8R], where each SRS resource set has one SRS resource, the number of SRS ports for each resource is equal to 1, 2,  4[, or 8] or

<Unchanged text is omitted>

Proposal 3.2.1-3: Adopt the text proposal for TS38.214 for new 8T8R UE capability:

6.2.1.2	UE sounding procedure for DL CSI acquisition

<Unchanged text is omitted>

When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set as 'antennaSwitching', the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitch ('t1r2' for 1T2R, 't1r1-t1r2' for 1T=1R/1T2R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't8r8' for 8T8R, 't1r1-t1r2-t1r4' for 1T=1R/1T2R/1T4R, 't1r4-t2r4' for 1T4R/2T4R, 't1r1-t1r2-t2r2-t2r4' for 1T=1R/1T2R/2T=2R/2T4R, 't1r1-t1r2-t2r2-t1r4-t2r4' for 1T=1R/1T2R/2T=2R/1T4R/2T4R, 't1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r1-t2r2' for 1T=1R/2T=2R, 't4r4' for 4T=4R, or 't1r1-t2r2-t4r4' for 1T=1R/2T=2R/4T=4R) or the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitchBeyond4Rx (‘t1r1’ for 1T=1R, ‘t2r2’ for 2T=2R, ‘t1r2’ for 1T2R, ‘t4r4’ for 4T=4R, ‘t2r4’ for 2T4R, ‘t1r4’ for 1T4R, ‘t2r6’ for 2T6R, ‘t1r6’ for 1T6R, ‘t4r8’ for 4T8R, ‘t2r8’ for 2T8R, ‘t1r8’ for 1T8R) or the UE may be configured with only one of the following configurations depending on the indicated UE capability [newUECapabilitySupporting8T8R] [(‘t8r8’ for 8T=8R)]:

<Unchanged text is omitted>

| Company | View |
| --- | --- |
| New H3C | - OK |
| Nokia, NSB | - OK |
| Samsung | - Support. |
| Huawei, HiSilicon | - Proposal 3.2.1-1: Support - Proposal 3.2.1-2A: Support - Proposal 3.2.1-3: Fine. |

### TDM description issues in TS38.214

Several companies suggested modifications to the current TS38.214 regarding TDMed 8 ports.

- TP 1: CATT proposed to clarify the R value when TDM is configured but R is not configured:

6.2.1	UE sounding procedure

-	Number of OFDM symbols in the SRS resource, starting OFDM symbol of the SRS resource within a slot including repetition factor R as defined by the higher layer parameter resourceMapping and described in clause 6.4.1.4 of [4, TS 38.211]. If R is not configured, R is equal to a half of the number of OFDM symbols in the SRS resource if the higher layer parameter [tdm] is configured, otherwisethen R is equal to the number of OFDM symbols in the SRS resource.

  - The TP seems reasonable. This is formulated into Proposal 3.2.2-1 below with some rewording.

- TP 2: Sharp and Futurewei proposed to clarify the frequency hopping behavior description when TDM is configured.

  - It seems that a TP is needed for the frequency hopping sub-clause, otherwise the spec appears to be incomplete. To avoid lengthy description, it is suggested to adopt a simple TP that refers to the equations in TS38.211 for the details, which is formulated into Proposal 3.2.2-2 below.

- TP 3: Samsung proposed to clarify that the TDMed S symbols are consecutive:

6.2.1	UE sounding procedure

-  Support of time division mapping subsets of ports of the SRS resource into S consecutive symbols (S=2), as defined by the higher layer parameter [tdm]nrofSRS-Ports-n8, where the SRS ports are evenly distributed in two symbols. This applies when the SRS resource set is configured with higher layer parameter usage in SRS-ResourceSet set to ‘codebook’, or ‘antennaSwitching’, and nrofSRS-Ports is set to ‘n8’ nrofSRS-Ports-n8 is set to ‘ports8tdm’.

  - The addition of “consecutive” is aligned with existing agreement. However, this is already specified in TS38.211, and TS38.214 does not have to describe. Companies can further discuss. Also the TP includes changes related to RRC parameters, which can be discussed later.

Proposal 3.2.2-1: Adopt the text proposal for TS38.214 on the R value when TDM is configured but R is not configured:

6.2.1	UE sounding procedure

<Unchanged text is omitted>

-	Number of OFDM symbols in the SRS resource, starting OFDM symbol of the SRS resource within a slot including repetition factor R as defined by the higher layer parameter resourceMapping and described in clause 6.4.1.4 of [4, TS 38.211]. If R is not configured, R is equal to a half of the number of OFDM symbols in the SRS resource if the higher layer parameter [tdm] is configured, and then R is equal to the number of OFDM symbols in the SRS resource if the higher layer parameter [tdm] is not configured.

<Unchanged text is omitted>

Proposal 3.2.2-2: Adopt the text proposal for TS38.214 on the frequency hopping behavior when TDM is configured:

6.2.1	UE SRS frequency hopping procedure

If for a SRS resource, the higher-layer parameter [tdm] is configured, the corresponding UE SRS frequency hopping procedure is specified in clause 6.4.1.4.3 of [4, TS 38.211]. Otherwise, the UE SRS frequency hopping procedure is specified in this clause.

<Unchanged text is omitted>

| Company | View |
| --- | --- |
| Samsung | - Support in principle both Proposal 3.2.2-1 and Proposal 3.2.2-2. |
| NTT DOCOMO | - Ok with both. |
| Sharp | - Proposal 3.2.2-1: Support - Proposal 3.2.2-2: Support |
| ZTE | - Re Proposal 3.2.2-1, we prefer the following wording: “if R is not configured, R = Ns/s”, where Ns is the number of SRS symbols, and s is the TDM factor. - Re Proposal 3.2.2-2, we are not sure whether this wording is proper, because 38.211 specifies the SRS frequency hopping behavior for both TDM being configured and not configured. |
| vivo | - Proposal 3.2.2-1: Support - Proposal 3.2.2-2: Support |
| OPPO | - Proposal 3.2.2-1: Fine - Proposal 3.2.2-2: We have similar view as ZTE. We think some modification on 6.2.1 is necessary since the description in 6.4.1.4.3 of 38.211 is not clear enough (otherwise the whole 6.2.1 in 38.214 would be redundant). We provided simple TP in our contribution (R1-2309567). |
| QC | - Proposal 3.2.2-1: We are fine with the proposal. - Proposal 3.2.2-2: We don’t support the proposal. Even when tdm is not configure, the legacy SRS frequency hopping procedure in Rel-15 is specified in 38.211, no? |
| Google | - OK with both proposals |
| Spreadtrum | - Fine with proposal 3.2.2-1 and 3.2.2-2. |
| Xiaomi | - Proposal 3.2.2-1: Fine - Proposal 3.2.2-2: fine, oppo’s TP seems also fine with us. |
| Lenovo | - Proposal 3.2.2-1: Support - Proposal 3.2.2-2: Support in principle. |
| Fujitsu | - Proposal 3.2.2-1: Ok - Proposal 3.2.2-2: Not support. Why the hopping procedure is split between 38.211 and 38.214? |
| NEC | - Support. |
| CMCC | - Proposal 3.2.2-1: Support. - Proposal 3.2.2-2: Support. |
| New H3C | - OK with both proposals |
| Huawei, HiSilicon | - Proposal 3.2.2-1: Not support. As clarified during post-114 draft CR stage for 214, “If R is not configured” seems only applies to positioning SRS, the simultaneous support of which and 8-port SRS is not even discussed yet. - Proposal 3.2.2-2: Generally fine given the potential complex modification, but indeed as companies mentioned the formulation will become a little bit split. |
| CATT | - Proposal 3.2.2-1: Support. - Proposal 3.2.2-2: Not support. We prefer to clarify SRS frequency hopping behavior for TDM in TS 38.214. - Regarding TP3, we also have a draft TP on it as follows(R1-2309497). Since s in other sections or specifications are used to indicate TDM factor for TDM based SRS resource, we prefer not to introduce S and refer to TS38.211 to make the spec clear. ### 6.2.1 UE sounding procedure <Unrelated part omitted> - Support of time division mapping s (s = 2) subsets of ports of the SRS resource into the OFDM symbols occupied by the SRS resource, as defined by the higher layer parameter [tdm], where the SRS ports are distributed as defined in [4, TS 38.211] clause 6.4.1.4.2. This applies when the SRS resource set is configured with higher layer parameter usage in SRS-ResourceSet set to ‘codebook’, or ‘antennaSwitching’, and nrofSRS-Ports is set to ‘n8’. |
| Ruijie | - Support proposal 3.2.2-1 and 3.2.2-2. |
| FL | - For Proposal 3.2.2-1, could Huawei provide more information from the specs about R being not configured is for positioning SRS? If that is indeed the case, this proposal is not needed since positioning SRS has only 1 port. I could find in TS38.331 that repetitionFactor is mandatory for a SRS resource for Rel-15 baseline, but is optional for Rel-16/17. Further discussion may be needed to understand this part. - TS38.331: resourceMapping SEQUENCE { startPosition INTEGER (0..5), nrofSymbols ENUMERATED {n1, n2, n4}, repetitionFactor ENUMERATED {n1, n2, n4} }, |

#### ROUND2

For Proposal 3.2.2-1, after further checking of TS38.331, repetitionFactor should always be configured for SRS resources (except for positioning SRS). Therefore, this proposal is no longer needed.

For Proposal 3.2.2-2, OPPO’s TP in their contribution can also be a good starting point, but it is not complete, e.g., frequency hopping may be combined with comb offset hopping, in which case, more changes are needed. Could proponents provide a complete TP (and hopefully, not overly complicated)? Alternatively, we can try the following updated Proposal 3.2.2-2A, which may address some companies’ concerns.

@CATT: It seems to me that your TP is mainly a rephrasing of existing spec and there is no essential correction. Please correct me if I am mistaken.

Proposal 3.2.2-2A: Adopt the text proposal for TS38.214 on the frequency hopping behavior when TDM is configured:

6.2.1	UE SRS frequency hopping procedure

If for a SRS resource, the higher-layer parameter [tdm] is configured, the corresponding UE SRS frequency hopping procedure is specified in clause 6.4.1.4.3 of [4, TS 38.211]. If for a SRS resource, the higher-layer parameter [tdm] is not configured, the UE SRS frequency hopping procedure is specified in clause 6.4.1.4.3 of [4, TS 38.211] and this clause.

<Unchanged text is omitted>

| Company | View |
| --- | --- |
| New H3C | - OK |
| Huawei, HiSilicon | - Fine. |
| Sharp | - OK |

After further checking, I updated the proposal as follow to incorporate TDM and/or comb offset hopping. Please comment on this updated version.

Proposal 3.2.2-2B: Adopt the text proposal for TS38.214 on the frequency hopping behavior when TDM or comb offset hopping is configured:

6.2.1	UE SRS frequency hopping procedure

If for a SRS resource, the higher-layer parameter [tdm] is configured or higher-layer parameter [combOffsetHopping] is configured, the corresponding UE SRS frequency hopping procedure is specified in clause 6.4.1.4.3 of [4, TS 38.211]. If for a SRS resource, the higher-layer parameter [tdm] is not configured and higher-layer parameter [combOffsetHopping] is not configured, the UE SRS frequency hopping procedure is specified in clause 6.4.1.4.3 of [4, TS 38.211] and this clause.

<Unchanged text is omitted>

| Company | View |
| --- | --- |
| OPPO | - We still think it is a bit strange that the frequency hopping in 38.211 covers all the cases, but the description in 38.214 only covers the frequency hopping without TDMed SRS and comb offset hopping. We propose to discuss this issue next meeting, with a completed CR for 38.214, considering both TDMed SRS and comb offset hopping. |

## Other proposals for 8Tx SRS

Several companies provided other discussion points. Again, some editorial / RRC parameter name related TPs can be discussed later during the CR review phase and are not included here.

DP 1: Same TDM pattern for SRS resources for a SRS resource set

  - Supported by: Apple

  - Seems reasonable. Requires discussion and a new agreement.

DP 2: Whether to downgrade configuration of SRS for antenna switching

  - Supported by: CMCC

  - Seems reasonable. Requires discussion and a new agreement. May also be discussed in UE feature/capability session.

DP 3: Not support TDM if the UE cannot transmit at PCmax on a symbol

  - Supported by: Huawei, HiSilicon, OPPO

  - This was already ‘unofficially’ excluded per chairman’s request. May be discussed in UE feature/capability session.

DP 4: Perform power scaling over s OFDM symbols for TDM when one of the OFDM symbols overlaps with another UL transmission

  - Supported by: Google

  - Seems reasonable. Requires discussion. A bit similar to TDMed dropping issue but less important. May also have timeline issue.

DP 5: Maintain the same power over s OFDM symbols for TDM

  - Supported by: Qualcomm

  - Seems reasonable, and also related to DP4.

DP 6: New PCMAX,f,cSRS(i) for TDMed 8-port SRS when the UE is not capable of transmitting at PCMAX per OFDM symbol with 8/s ports

  - Supported by: Qualcomm

  - This was already ‘unofficially’ excluded per chairman’s request.

DP 7: 2 or 4 SRS resource sets for NCB

  - Supported by: ZTE Xiaomi

  - Requires discussion and a new agreement.

Views can be provided for the above discussion points, and details / proposals can also be included.

| Company | View |
| --- | --- |
| ZTE | - DP1: Support. - DP2: Need further discussion. - DP3: Support. - DP4: Open to discuss. - DP5: It should be discussed together with DP4. - DP6: NOT support. If a UE is not capable of transmitting at Pcmax at a symbol, it should not be configured with TDM. - DP7: It seems not our proposal. |
| OPPO | - We are fine with DP 1,3-5. |
| QC | - Even if DP6 is considered as excluded or it is just optimization, DP 5 is needed, otherwse if power varies, the whole SRS sounding with tdm seems broken… DP 5: Maintain the same power over s OFDM symbols for TDM |
| Google | - We think DP4 and DP5 should be discussed together. |
| Huawei, HiSilicon | - DP1: Open to discuss. - DP2: Open to discuss. - DP3: Support. - DP4: Open to discuss. - DP5: Open to discuss. - DP6: Not support. - DP7: Not support. |
| CATT | - DP1: Ok - DP2: Ok to discuss. - DP3: Not support. - DP4: Ok to discuss. |

# Conclusions

## For Monday Online

Proposal 2.1.2: Adopt the text proposal for TS38.211 on cyclic shift / comb offset hopping ID notation:

6.4.1.4.2	Sequence generation

<Unchanged text is omitted>

The pseudo-random sequence ci is defined by clause 5.2.1 and shall be initialized with cinit=nIDcshhop at the beginning of each radio frame for which nf mod 128=0, where the cyclic-shift hopping identity nIDcshhop is contained in the higher-layer parameter cyclicShiftHopping.

<Unchanged text is omitted>

6.4.1.4.3	Mapping to physical resources

<Unchanged text is omitted>

The pseudo-random sequence ci is defined by clause 5.2.1 and shall be initialized with cinit=nIDcohhop at the beginning of each radio frame for which nf mod 128=0, where the comb hopping identity nIDcohhop is contained in the higher-layer parameter combOffsetHopping.

<Unchanged text is omitted>

Proposal 2.1.3A: Adopt the text proposal for TS38.211 on cyclic shift / comb offset hopping subset entry indexing:

6.4.1.4.2	Sequence generation

<Unchanged text is omitted>

where scshSRSn and ncshSRS is the (n+1)th entry and the cardinality of the set

Scsh={scshSRS0, scshSRS1, …,scshSRSncshSRS-1}

respectively, where Scsh is given by the higher-layer parameter cyclicShiftHoppingSubset if configured, otherwise Scsh={0, 1,…,KnSRScs,max-1}.

<Unchanged text is omitted>

6.4.1.4.3	Mapping to physical resources

<Unchanged text is omitted>

where scohSRSn and ncohSRS is the (n+1)th entry and the cardinality of the set

Scoh={scohSRS0, scohSRS1, …,scohSRSncohSRS-1}
respectively, where Scoh is given by the higher-layer parameter combOffsetHoppingSubset if configured, otherwise Scoh={0, 1,…,KTC-1}.

<Unchanged text is omitted>

Proposal 2.1.1A: Adopt the text proposal for TS38.211 on cyclic shift / comb offset hopping subset bitmap:

6.4.1.4.2	Sequence generation

<Unchanged text is omitted>

where scshSRSn and ncshSRS is the nth entry and the cardinality of the set

Scsh={scshSRS0, scshSRS1, …,scshSRSncshSRS-1}

respectively, where Scsh is given by the higher-layer parameter cyclicShiftHoppingSubset if configured, otherwise Scsh={0, 1,…,KnSRScs,max-1}. The higher-layer parameter [cyclicShiftHoppingSubset] includes a bitmap of nSRScs,max bits with 1<ncshSRS<nSRScs,max bits being set, where the nth bit being set to 1 corresponds to scshSRSn.

<Unchanged text is omitted>

6.4.1.4.3	Mapping to physical resources

<Unchanged text is omitted>

where scohSRSn and ncohSRS is the nth entry and the cardinality of the set

Scoh={scohSRS0, scohSRS1, …,scohSRSncohSRS-1}
respectively, where Scoh is given by the higher-layer parameter combOffsetHoppingSubset if configured, otherwise Scoh={0, 1,…,KTC-1}. The higher-layer parameter [combOffsetHoppingSubset] includes a bitmap of KTC bits with 1<ncohSRS<KTC bits being set, where the nth bit being set to 1 corresponds to scohSRSn.

<Unchanged text is omitted>

Proposal 2.2.1A: Adopt the text proposal for TS38.214 on cyclic shift hopping subset and finer granularity configuration:

6.2.1	UE sounding procedure

<Unchanged text is omitted>

-	Cyclic shift, as defined by the higher layer parameter cyclicShift-n2, cyclicShift-n4, or cyclicShift-n8 for transmission comb value 2, 4 or 8, and described in clause 6.4.1.4 of [4, TS 38.211]. When cyclic shift hopping is configured by the higher layer parameter [cyclicShiftHopping] for an SRS resource in an SRS resource set with the usage configured as ‘antennaSwitching’, subject to UE capabilities, cyclic shift is updated at every symbol as described in [clause 6,4,1,4 of [4, TS 38.211]]. For the cyclic shift hopping, a UE can be configured with a subset of cyclic shifts by the higher layer parameter [cyclicShiftHoppingSubset], where the cyclic shift hopping is performed only across the cyclic shifts configured in the subset. For the cyclic shift hopping, a UE can be configured with finer hopping granularity of K=2 by the higher layer parameter [hoppingFinerGranularity]. The UE is not expecting that [hoppingFinerGranularity] is configured when [cyclicShiftHoppingSubset] is configured for an SRS resource. The UE is not expecting that the cyclic shift hopping and the higher layer parameter [tdm] are configured simultaneously for an SRS resource.

<Unchanged text is omitted>

Proposal 3.1: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ and with TDM factor s = 2, the 8 ports being fully/partially coherent, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), and when the SRS transmission on a subset of the s OFDM symbols within a group of {1, 2, …, s} is dropped, the UE can drop the SRS transmission on the rest of OFDM symbols within the group of {1, 2, …, s}, depending on SRS repetition factor and/or UE preparation time for SRS transmission.

Supporting: Futurewei, Google, Huawei, HiSilicon, KDDI, LG, NEC, Nokia, Nokia Shanghai Bell, NTT DOCOMO, Qualcomm, Ruijie, Spreadtrum, Xiaomi, ZTE (15 proponents)

Proposed Conclusion: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ and with TDM factor s = 2, the 8 ports being fully/partially coherent, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), and when the SRS transmission on a subset of the s OFDM symbols within a group of {1, 2, …, s} is dropped, the UE can still transmit the SRS on the rest of OFDM symbols within the group of {1, 2, …, s}.

Supporting: Apple, CATT, CMCC (enhance SRI), Ericsson, Fijitsu, KDDI, Lenovo, NTT DOCOMO (open), OPPO, Qualcomm (enhance coherent assumption), Ruijie, Samsung, vivo, Xiaomi (14 proponents)

## Monday Agreements

Proposal 2.1.2:

Adopt the text proposal for TS38.211 on cyclic shift / comb offset hopping ID notation:

6.4.1.4.2	Sequence generation

<Unchanged text is omitted>

The pseudo-random sequence  is defined by clause 5.2.1 and shall be initialized with  at the beginning of each radio frame for which , where the cyclic-shift hopping identity  is contained in the higher-layer parameter cyclicShiftHopping.

<Unchanged text is omitted>

6.4.1.4.3	Mapping to physical resources

<Unchanged text is omitted>

The pseudo-random sequence  is defined by clause 5.2.1 and shall be initialized with  at the beginning of each radio frame for which , where the comb hopping identity  is contained in the higher-layer parameter combOffsetHopping.

<Unchanged text is omitted>

Proposal 2.1.3A:

Adopt the text proposal for TS38.211 on cyclic shift / comb offset hopping subset entry indexing:

6.4.1.4.2	Sequence generation

<Unchanged text is omitted>

where  and is the th entry and the cardinality of the set

respectively, where  is given by the higher-layer parameter cyclicShiftHoppingSubset if configured, otherwise .

<Unchanged text is omitted>

6.4.1.4.3	Mapping to physical resources

<Unchanged text is omitted>

where  and is the th entry and the cardinality of the set

respectively, where  is given by the higher-layer parameter combOffsetHoppingSubset if configured, otherwise .

<Unchanged text is omitted>

Proposal 2.1.1A:

Adopt the text proposal for TS38.211 on cyclic shift / comb offset hopping subset bitmap:

6.4.1.4.2	Sequence generation

<Unchanged text is omitted>

where  and is the (+1)th entry and the cardinality of the set

respectively, where  is given by the higher-layer parameter cyclicShiftHoppingSubset if configured, otherwise . The higher-layer parameter [cyclicShiftHoppingSubset] includes a bitmap of  bits with  bits being set, where the (n+1)th bit being set to 1 corresponds to

<Unchanged text is omitted>

6.4.1.4.3	Mapping to physical resources

<Unchanged text is omitted>

where  and is the (+1)th entry and the cardinality of the set

respectively, where  is given by the higher-layer parameter combOffsetHoppingSubset if configured, otherwise . The higher-layer parameter [combOffsetHoppingSubset] includes a bitmap of  bits with  bits being set, where the (n+1)th bit being set to 1 corresponds to

<Unchanged text is omitted>

## Tuesday offline consensus

Proposal 2.3A: SRS comb offset hopping / cyclic shift hopping can be configured for a SRS resource in a SRS resource set with usage ‘codebook’.

- SRS comb offset hopping / cyclic shift hopping are not supported for a SRS resource in a SRS resource set with usage ‘nonCodebook’ or ‘beamManagement’.

## For Wednesday Online

Tuesday offline consensus

Proposal 2.3A: SRS comb offset hopping / cyclic shift hopping can be configured for a SRS resource in a SRS resource set with usage ‘codebook’.

- SRS comb offset hopping / cyclic shift hopping are not supported for a SRS resource in a SRS resource set with usage ‘nonCodebook’ or ‘beamManagement’.

Proposal 2.2.1B:

Adopt the text proposal for TS38.214 on cyclic shift hopping subset and finer granularity configuration:

6.2.1	UE sounding procedure

<Unchanged text is omitted>

-	Cyclic shift, as defined by the higher layer parameter cyclicShift-n2, cyclicShift-n4, or cyclicShift-n8 for transmission comb value 2, 4 or 8, and described in clause 6.4.1.4 of [4, TS 38.211]. When cyclic shift hopping is configured by the higher layer parameter [cyclicShiftHopping] for an SRS resource in an SRS resource set with the usage configured as ‘antennaSwitching’, subject to UE capabilities, cyclic shift is updated at every symbol as described in [clause 6,4,1,4 of [4, TS 38.211]]. For the cyclic shift hopping, a UE can be configured with a subset of cyclic shifts by the higher layer parameter [cyclicShiftHoppingSubset], where the cyclic shift hopping is performed only across the cyclic shifts configured in the subset. For the cyclic shift hopping, a UE can be configured with finer hopping granularity of K=2 by the higher layer parameter [hoppingFinerGranularity]. The UE is not expecting that [hoppingFinerGranularity] is configured when [cyclicShiftHoppingSubset] is configured for an SRS resource. The UE is not expecting that the cyclic shift hopping and the higher layer parameter [tdm] are configured simultaneously for an SRS resource.

<Unchanged text is omitted>

Proposal 2.1.5: Adopt the text proposal for TS38.211 on cyclic shift hopping finer granularity:

6.4.1.4.2	Sequence generation

<Unchanged text is omitted>

If the higher-layer parameter hoppingFinerGranularity is configured and the higher-layer parameter cyclicShiftHoppingSubset is not configured, K=2, otherwise K=1.

<Unchanged text is omitted>

Proposal 3.2.2-2B: Adopt the text proposal for TS38.214 on the frequency hopping behavior when TDM or comb offset hopping is configured:

6.2.1	UE SRS frequency hopping procedure

If for a SRS resource, the higher-layer parameter [tdm] is configured or higher-layer parameter [combOffsetHopping] is configured, the corresponding UE SRS frequency hopping procedure is specified in clause 6.4.1.4.3 of [4, TS 38.211]. If for a SRS resource, the higher-layer parameter [tdm] is not configured and higher-layer parameter [combOffsetHopping] is not configured, the UE SRS frequency hopping procedure is specified in clause 6.4.1.4.3 of [4, TS 38.211] and this clause.

<Unchanged text is omitted>

Proposal 3.1-1A: For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ / ‘antennaSwitching’ and with TDM factor s = 2, the 8 ports being fully/partially coherent, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), and when the SRS transmission on a subset of the s OFDM symbols within a group of {1, 2, …, s} is dropped, the UE can still transmit the SRS on the rest of OFDM symbols within the group of {1, 2, …, s}.

- The indicated SRI in slot n is associated with the most recent transmission of SRS resource identified by the SRI, where the SRS resource is prior to the PDCCH carrying the SRI, and where some or all of the ports in the SRS resource are transmitted.

  - [Whether or how to capture this is left to the editor.]

  - Note: If the 8 ports are also fully coherent, the UE is not required to maintain the coherence between the partial SRS transmission and the PUSCH scheduled by the PDCCH.

  - Note (Alt): The UE expects the antenna ports of scheduled PUSCH transmission correspond to only the SRS ports transmitted in the SRS resource.

Proposal 3.2.1-1: Adopt the text proposal for TS38.214 on removing the limitation of no more than one SRS resource set to be triggered/configured:

6.2.1.2	UE sounding procedure for DL CSI acquisition

<Unchanged text is omitted>

For 1T2R, 1T4R, 2T4R, 1T6R, 1T8R, 2T6R, 2T8R, or 4T8R, or 8T8R, the UE shall not expect to be configured or triggered with more than one SRS resource set with higher layer parameter usage set as 'antennaSwitching' in the same slot. For 1T=1R, 2T=2R 4T=4R, or 8T=8R, the UE shall not expect to be configured or triggered with more than one SRS resource set with higher layer parameter usage set as 'antennaSwitching' in the same symbol.

<Unchanged text is omitted>

Proposal 3.2.1-2A: Adopt the text proposal for TS38.214 on allowing 2 SP SRS resource sets for 8T8R per UE capability:

6.2.1.2	UE sounding procedure for DL CSI acquisition

<Unchanged text is omitted>

-	For 1T=1R, or 2T=2R, 4T=4R or 8T=8R, up to two SRS resource sets each with one SRS resource can be configured, where the number of SRS ports for each resource is equal to 1, 2,  4 or 8 if the UE is not indicating srs-AntennaSwitching2SP-1Periodic. Two SRS resource sets configured with resourceType in SRS-ResourceSet set to 'semi-persistent' and one SRS resource set configured with resourceType in SRS-ResourceSet set to 'periodic' can be configured and the two SRS resource sets configured with 'semi-persistent' are not activated at the same time, or up to two SRS resource sets can be configured, if the UE is indicating srs-AntennaSwitching2SP-1Periodic or [srs-AntennaSwitching2SP-1Periodic8T8R], where each SRS resource set has one SRS resource, the number of SRS ports for each resource is equal to 1, 2,  4[, or 8] or

<Unchanged text is omitted>

Proposal 3.2.1-3: Adopt the text proposal for TS38.214 for new 8T8R UE capability:

6.2.1.2	UE sounding procedure for DL CSI acquisition

<Unchanged text is omitted>

When the UE is configured with the higher layer parameter usage in SRS-ResourceSet set as 'antennaSwitching', the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitch ('t1r2' for 1T2R, 't1r1-t1r2' for 1T=1R/1T2R, 't2r4' for 2T4R, 't1r4' for 1T4R, 't8r8' for 8T8R, 't1r1-t1r2-t1r4' for 1T=1R/1T2R/1T4R, 't1r4-t2r4' for 1T4R/2T4R, 't1r1-t1r2-t2r2-t2r4' for 1T=1R/1T2R/2T=2R/2T4R, 't1r1-t1r2-t2r2-t1r4-t2r4' for 1T=1R/1T2R/2T=2R/1T4R/2T4R, 't1r1' for 1T=1R, 't2r2' for 2T=2R, 't1r1-t2r2' for 1T=1R/2T=2R, 't4r4' for 4T=4R, or 't1r1-t2r2-t4r4' for 1T=1R/2T=2R/4T=4R) or the UE may be configured with only one of the following configurations depending on the indicated UE capability supportedSRS-TxPortSwitchBeyond4Rx (‘t1r1’ for 1T=1R, ‘t2r2’ for 2T=2R, ‘t1r2’ for 1T2R, ‘t4r4’ for 4T=4R, ‘t2r4’ for 2T4R, ‘t1r4’ for 1T4R, ‘t2r6’ for 2T6R, ‘t1r6’ for 1T6R, ‘t4r8’ for 4T8R, ‘t2r8’ for 2T8R, ‘t1r8’ for 1T8R) or the UE may be configured with only one of the following configurations depending on the indicated UE capability [newUECapabilitySupporting8T8R] [(‘t8r8’ for 8T=8R)]:

<Unchanged text is omitted>

# References

RP-223276, WID Update: MIMO Evolution for Downlink and Uplink, Samsung (Moderator), RAN#98-e.

RP-231717, Status report for NR MIMO Evolution for Downlink and Uplink, Samsung (Moderator), RAN#101.

R1-2308927, Maintenance of SRS enhancement for TDD CJT and UL 8Tx operation in Rel-18, Huawei, HiSilicon.

R1-2308933, SRS enhancements for TDD CJT and 8TX operation, FUTUREWEI.

R1-2308963, Remaining Details on Rel-18 SRS Enhancements, InterDigital, Inc.

R1-2308977, Remaining issues on SRS enhancement targeting TDD CJT and 8 TX operation, Spreadtrum Communications.

R1-2309017, Maintenance on SRS enhancement targeting TDD CJT and 8 TX operation, ZTE.

R1-2309064, Maintenance on SRS enhancements, vivo.

R1-2309165, SRS enhancement targeting TDD CJT and 8 TX operation, LG Electronics.

R1-2309252, Remaining issues for SRS enhancements targeting TDD CJT and 8 TX operation, Ericsson.

R1-2309256, On SRS Enhancement, Google.

R1-2309276, Remaining issues on SRS enhancement, NEC.

R1-2309319, Maintenance on SRS enhancement, Lenovo.

R1-2309365, Views on SRS enhancements, Samsung.

R1-2309428, Maintenance on SRS enhancements, xiaomi.

R1-2309497, Maintenance on SRS enhancement, CATT.

R1-2309567, Remaining issues on SRS enhancement targeting TDD CJT and 8 TX operation, OPPO.

R1-2309663, Remaining issues on SRS enhancement targeting TDD CJT and 8 TX operation, CMCC.

R1-2309767, Discussions on SRS enhancement targeting TDD CJT and 8 TX operation, Ruijie Network Co. Ltd.

R1-2309824, Views on Rel-18 MIMO SRS enhancement, Apple.

R1-2309919, Maintenance of SRS enhancement for TDD CJT and 8Tx operation, Nokia, Nokia Shanghai Bell.

R1-2309965, Maintenance on SRS enhancement targeting TDD CJT and 8 TX operation, Sharp.

R1-2310026, Remaining issues on SRS enhancement, NTT DOCOMO, INC.

R1-2310131, SRS enhancement for TDD CJT and 8 Tx operation, Qualcomm Incorporated.

R1-2310223, Remaining issues on SRS enhancement targeting TDD CJT and 8 TX operation, KDDI Corporation.

# Appendix

## Appendix 1: Agreements from RAN1#109-e

Agreement

For SRS EVM, adopt combined relevant parts from Rel-17 SRS EVM and Rel-18 FDD CJT EVM as starting point

- Details are provided in Appendix 3 of R1-2205330 for system-level simulations

- Details are provided in Appendix 4 of R1-2205330 for link-level simulations.

Agreement

For 8 Tx SRS, a starting point of UE antenna configurations can be:

- (M, N, P; Mg,Ng; Mp, Np) = (2,2,2; 1,1; 2,2), (dH, dV) = (0.5, 0.5)λ, or

- (M, N, P; Mg,Ng; Mp, Np) = (1,4,2; 1,1; 1,4), (dH, dV) = (0.5, 0.5)λ.

- FFS other 8 Tx UE antenna configuration and alignment with outcomes from other agenda items.

Agreement

For SRS EVM, consider additional EVM as follows

- Realistic channel estimation based on sequence generation for SRS modelling, at least for TDD CJT SRS LLS and 8 Tx SRS LLS as baseline

- Evaluation metrics for 8 Tx SRS LLS can be MSE , BLER or throughput

- TDL-C for TDD CJT SRS LLS can be included as optional.

Agreement

Consider the scenario where there exists SRSs sent by a UE and utilized by multiple TRPs for channel estimation, and the pathlosses between the UE and the TRPs differ by at least x dB in Rel-18 SRS study

- x can be {3,6,10}, and other values can be used.

Agreement

Study the following for SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS interference randomization and/or capacity enhancement

- Randomized frequency-domain resource mapping for SRS transmission

  - E.g., further enhancements to frequency hopping, comb hopping

- Randomized code-domain resource mapping for SRS transmission

  - E.g., cyclic shift hopping/randomization, sequence hopping/randomization, per-hop sequence from a long SRS sequence

- Randomized transmission of SRS

    - E.g., pseudo-random muting of SRS transmission for periodic and semi-persistent SRS

- Per-TRP power control and/or power control of one SRS towards to multiple TRPs

- SRS TD OCC

- Increasing the maximum number of cyclic shifts

  - E.g., multiplying mask sequence to the legacy SRS sequence to effectively increase the maximum cyclic shifts

- Precoded SRS for DL CSI acquisition

- Enhanced signaling for flexible SRS transmission

  - E.g., dynamic update of SRS parameters

- Partial frequency sounding extensions

  - E.g., larger partial frequency sounding factor, starting RB location hopping enhancements, partial frequency hopping on other bandwidths corresponding to b, bhop<b<BSRS   besides the last bandwidth BSRS

- Enhanced configuration of SRS transmission to enable more efficient SRS parameter assignment

  - E.g., configuration of v (sequence index within a group) per SRS resource

  - E.g., configuration of cyclic shift per SRS port per SRS resource.

- Resource mapping for SRS transmission based on network-provided parameters or system parameters

  - E.g., SRS resource mapping based on network-provided parameters (e.g., configurable indexes) or system parameters (e.g., slot index)

Note: PAPR performance and maintaining DFT waveform property should be considered when deciding the enhancement for Rel-18.

Agreement

Study the potential enhancements for SRS of 8T8R with usage antennaSwitching.

Agreement

Study the potential enhancements for SRS for 8 Tx operation

- SRS resource(s) with 8 ports are configured for codebook-based PUSCH

- Up to 8 single-port SRS resources are configured for non-codebook-based PUSCH

Agreement

For SRS enhancements to enable 8 Tx UL operation to support 4 and more layers per UE in UL targeting CPE/FWA/vehicle/Industrial devices, study aspects include, for SRS for CB/NCB/AS,

- Design parameters, including the maximum number of SRS resource sets, number of SRS resource sets, number of SRS resources, number of ports per resource, number of OFDM symbols, the allowed configurations for comb / comb shifts / cyclic shifts, number of simultaneous ports / resources / resource sets per OFDM symbol

- For the next decision point, study

  - Whether to support 8 ports in one or multiple resources

  - Whether to support 8 ports in one or multiple OFDM symbols

  - The maximum number of SRS resource sets.

- Note: For SRS for NCB, number of ports per SRS resource is still 1 (same as R15)

| Rel-18 SLS Assumptions for TDD CJT SRS |  |  |
| --- | --- | --- |
| Parameter |  | Value |
| Duplex, Waveform |  | TDD, OFDM |
| Multiple access |  | OFDMA |
| Scenario |  | Outdoor OptA Companies can simulate from the following 2 layouts. 1) Outdoor (typical 57-sector, or 21-sector, SLS): OptionA: 1 TRP per sector, 3 sectors per site. N_TRP (#TRPs): 2, 3, 4 (N_TRP is semi-statically chosen based on, e.g. RSRP). The N_TRP TRPs can be selected either only from the same site (intra-site - limited to 3 TRPs), or also from other sites (inter-site) - company should describe what is assumed OptionB: N_TRP co-located (at BS) panels per sector - companies describe how the panels are (azimuthally) oriented - Dense Urban (macro only) 200m ISD or Urban Macro 500m ISD 2) Indoor Hotspot: model in TS 38.802 - N_TRP (#TRPs): 2, 3, 4 (N_TRP is semi-statically chosen based on, e.g. RSRP). |
| Frequency Range |  | FR1 only, 3.5GHz |
| Inter-BS (site) distance |  | Outdoor: 200m or 500m Indoor Hotspot: per TS 38.802 |
| Channel generation model |  | According to the TR 38.901 Difference in propagation delays between UE and N_TRP TRPs is taken into account in the composite Channel Impulse Response (CIR) for CJT. Otherwise, company should state if per-TRP delay offset (to "zero") is performed in the simulation. Per WID, ideal synchronization and backhaul should be assumed. Optionally, companies may present results with phase/frequency error and should state the assumed frequency error models and values. |
| Antenna setup and port layouts at gNB |  | - 8 ports: (4,4,2,1,1,1,4), (dH,dV) = (0.5, 0.8)λ - 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ - 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ - 64 ports: (8,8,2,1,1,4,8), (dH,dV) = (0.5, 0.8)λ Total #ports = N_TRP x {8,16,32,64} |
| Antenna setup and port layouts at UE |  | 4RX: (1,2,2,1,1,1,2), (dH,dV) = (0.5, 0.5)λ for rank > 2 |
| BS Tx power |  | Dense Urban or Urban Macro: - Per TRP: 44 dBm for 20MHz, 47dBm for 40MHz, 51dBm for 100MHz Indoor: per TRP 24dBm |
| BS antenna height |  | Depending on scenarios (cf. table A.2.1-1 of TS 38.802): DU (25m), UMa (25m), Indoor Hotspot (3m) |
| UE antenna height & gain |  | Follow TR36.873 |
| UE receiver noise figure |  | 9dB |
| Modulation |  | Up to 256QAM |
| Coding on PDSCH |  | LDPC Max code-block size=8448bit |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 30kHz |
| Number of RBs |  | 52RB for 20MHz, 104RB for 40MHz, 272RB for 100MHz |
| Frame structure |  | DSUDD, or companies to state the used frame structure |
| MIMO scheme |  | SU/MU-MIMO with rank adaptation is a baseline For low RU, SU-MIMO or SU/MU-MIMO with rank adaptation are assumed For medium/high RU, SU/MU-MIMO with rank adaptation is assumed |
| MIMO layers |  | For all evaluation, companies to provide the assumption on the maximum MU layers |
| Overhead |  | Companies shall provide the downlink overhead assumption |
| Traffic model |  | FTP 1 or FTP 3 with 20%, 50% or 70% traffic load |
| UE distribution |  | According to TS 38.802 - DU and UMa: 80% indoor (3km/h), 20% outdoor (30km/h) - Indoor Hotspot: 100% indoor (3km/h) |
| UE receiver |  | MMSE-IRC as the baseline receiver |
| DL Channel estimation |  | Realistic |
| Evaluation Metric |  | DL throughput |
| Baseline for performance evaluation |  | R17 SRS design |
| SRS modeling for UL channel estimation |  | Companies to state the used SRS periodicity. Companies to state the SRS channel estimation modeling Number of ports = 2 or 4 Tx power = 23 dBm |

| Rel-18 LLS Assumptions for TDD CJT SRS |  |
| --- | --- |
| Parameter | Value |
| Scenario | N_TRP (#TRPs): 2, 3, 4 |
| Carrier frequency and subcarrier spacing | 3.5 GHz with 30 kHz SCS |
| System bandwidth | 20MHz, 40MHz, 100MHz |
| Channel model | CDL-B or CDL-C in TR 38.901 with 30ns or 300ns delay spread as baseline for MU-MIMO and SU-MIMO Note: Other delay spread is not precluded. Difference in propagation delays between UE and N_TRP TRPs is taken into account in the composite Channel Impulse Response (CIR) for CJT. Otherwise, company should state if per-TRP delay offset (to "zero") is performed in the simulation. Per WID, ideal synchronization and backhaul should be assumed. Optionally, companies may present results with phase/frequency error and should state the assumed frequency error models and values. |
| UE velocity | 3km/h |
| Antennas at UE | 1T4R, 2T4R, 4T4R |
| Antennas at gNB | 64 ports: (8,8,2,1,1,4,8), (dH,dV) = (0.5, 0.8)λ 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ 16 ports: (8,4,2,1,1,2,4), (dH,dV) = (0.5, 0.8)λ |
| Rank and MCS | Rank/MCS can be adaptive or fixed. |
| Evaluation metrics | MSE, BLER or throughput |
| Baseline | R17 SRS design |
| Precoding granularity | Fixed: 2, 4 or wideband for DL, wideband for UL. |
| SRS configurations | Companies to state the used SRS periodicity. Frequency hopping：Companies to state whether SRS frequency hopping is enabled and the hopping pattern if so. |
| DL SNR | Companies to state the used difference between DL SNR and UL SNR |

## Appendix 2: Agreements from RAN1#110

Agreement

For Rel-18 reference signal enhancements, support and specify the following features (the agreed WID scopes apply):

- SRS enhancement to manage inter-TRP cross-SRS interference targeting TDD CJT via SRS capacity enhancement and/or interference randomization;

RAN1 should strive to minimize the number of schemes supported in Rel-18

- SRS enhancements to enable 8 Tx UL operation and 8T8R SRS for DL operation.

Target usage includes antenna switching, codebook/non-codebook based SRS

Agreement

For 8 Tx SRS, at least support

- 8 ports in 1 SRS resource for ‘antennaSwitching’;

  - FFS 8 ports in one or multiple SRS resources for ‘codebook’

Above does not imply support for 8 ports in one or multiple OFDM symbols

Agreement

For the maximum number of SRS resource sets for SRS with 8T8R with ‘antennaSwitching’, keep the existing value of the maximum number of SRS resource sets (as provided in Rel-17 antenna switching nTnR)

Agreement

For an 8-port SRS resource in an SRS resource set with usage antennaSwitching (i.e., for 8T8R antenna switching), the 8-port SRS resource is transmitted in at least one OFDM symbol.

FFS: the resource transmitted in multiple OFDM symbols where different ports are mapped to different symbols.

Agreement

For SRS resource set(s) with usage ‘nonCodebook’ support 8 1-port SRS resources in one or multiple OFDM symbols.

- Note: The maximum number of simultaneous SRS resources is determined via UE-capability signalling.

## Appendix 3: Agreements from RAN1#110bis-e

Agreement

Support at least one of the following for SRS interference randomization

- Randomized code-domain resource mapping for SRS transmission by introducing cyclic shift hopping / randomization to SRS resource

- Comb offset hopping for SRS

  - The comb offset is determined pseudo-randomly as a function of time (e.g., slot index, symbol index) and/or NW configured ID with a certain UE-specific initialization.

  - FFS: Other details, e.g., how the comb offset value is determined by the parameters for each SRS port of a SRS resource for a SRS transmission occasion.

Agreement

For comb offset hopping for SRS and for randomized code-domain resource mapping for SRS transmission via cyclic shift hopping / randomization, further study the following:

- The hopping pattern (e.g., the pseudo-random sequence, time-domain granularity for hopping)

- The time-domain parameter and/or behavior (e.g., slot index, symbol index, re-initialization behavior)

- Network-configured ID for UE-specific initialization

- How the comb offset / cyclic shift value is determined by the parameters for each SRS port of a SRS resource for a SRS transmission occasion

- Potential issue on multiplexing with legacy UEs if CS hopping and/or comb offset hopping are enabled

- Applicability to periodic/semi-persistent/aperiodic SRS

Other details are not excluded

Agreement

For SRS TD OCC for SRS enhancements for TDD CJT, study:

- Comparison against SRS on 1 OFDM symbol

- Comparison against SRS repeated on multiple OFDM symbols

- Study the following aspects: evaluation performance, SRS overhead, per-symbol per-port transmission power, impact of channel delay, dropping rules of collision with other uplink resource, etc.

Agreement

For per-TRP power control and/or power control of one or multiple SRS transmission occasions towards to multiple TRPs, study the options for an SRS resource set:

- Option 1:

  - Same power control process for all SRS resources of an SRS resource set where the power control process is based on one Po value and one closed loop state and jointly on more than one DL pathloss RS and/or more than one alpha

  - Each transmission occasion of the SRS resource is towards multiple TRPs

- Option 2:

  - More than 1 power control processes each for a subset of SRS resource of an SRS resource set where each of the power control process is based on a different UL power control parameter set (Po, alpha, and closed loop state) associated with a different DL pathloss RS

  - Different transmission occasions of the SRS resource can be towards different TRPs

Conclusion

The discussion of resource mapping for SRS transmission based on network-provided parameters or system parameters is merged into the discussions of other SRS enhancements for TDD CJT.

Conclusion

- No further discussion of increasing the maximum number of cyclic shifts for CJT SRS.

- No further discussion of partial frequency sounding extensions for CJT SRS.

Agreement

For an 8-port SRS resource in a SRS resource set ‘antennaSwitching’ (i.e., for 8T8R antenna switching), when the SRS resource is configured with m OFDM symbols (m >= 1), at least support the 8 ports mapped onto each of the m OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof).

- m takes the legacy values, i.e., 1,2,4,8,10,12,14.

Agreement

For one single SRS resource in a SRS resource set with usage ‘codebook’ for 8Tx PUSCH, when the SRS resource is configured with n ports (n <= 8) and m OFDM symbols (m >= 1), at least support the n ports mapped onto each of the m OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof).

- n can be 8

- m takes the legacy values, i.e., 1,2,4,8,10,12,14.

## Appendix 4: Agreements from RAN1#111

Agreement

For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port,

FFS: Hopping pattern

Support at least hopping based on slot index, OFDM symbol index

  - FFS: Use of symbol group based on repetition factor

  - FFS: Additional details on intra-slot hopping based on OFDM symbol index, inter-slot hopping based on slot index, per occasion of SRS resource

  - FFS: Re-initialization periodicity

Applicable to at least periodic/semi-persistent SRS with usage antennaSwitching

FFS: Other types of SRS

FFS: Configuring a subset of comb offsets / cyclic shifts for comb offset hopping / cyclic shift hopping, respectively

FFS: Combined comb offset hopping and cyclic shift hopping, supporting both, or down selecting one

Agreement

For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port, the hopping pattern is determined based on:

Option 1: The hopping pattern is based on the pseudo-random sequence c(i), initialized with a network-configured ID.

FFS: The ID could be cell ID nIDcell, SRS sequence identity nIDSRS, C-RNTI, or a new ID

FFS: The relation between the legacy group / sequence hopping and the new hopping

Agreement

For SRS interference randomization, support one from the following options  (to be decided in RAN1#112):

- Opt. 1: Cyclic shift hopping

- Opt. 2: Comb offset hopping

- Opt. 3: Both cyclic shift hopping and comb offset hopping

  - FFS: details including whether to support separate and/or combined hopping

  - FFS: details on UE capability and signaling

Conclusion

No consensus on enhanced signaling for flexible SRS transmission in Rel-18

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), at least support:

- For comb 2, support 1 and 2 comb offsets

- For comb 4, support 2 and [4] comb offset

- For comb 8, support 4 comb offsets

Agreement

For single SRS resource in a SRS resource set with usage ‘codebook’ for 8Tx PUSCH or ‘antennaSwitching’ (i.e., for 8T8R antenna switching), when the SRS resource is configured with 8 ports and m OFDM symbols (m > 1), support the case of 8 ports mapped onto the m OFDM symbols

- Option 1: Different SRS ports are mapped onto different OFDM symbols (i.e., TDM)

- FFS: m can be legacy values, i.e., 2,4,[8,10,12,14].

## Appendix 5: Agreements from RAN1#112

Agreement

For SRS interference randomization, support:

Opt. 3: Both cyclic shift hopping and comb offset hopping.

  - At least the two features can be separately configured

  - FFS: Combined cyclic shift hopping and comb offset hopping for a UE

  - FFS: Separate or combined with SRS sequence group hopping / sequence hopping

  - FFS: Associated UE capability

Agreement

For SRS comb offset hopping and/or cyclic shift hopping, for each SRS port, the hopping pattern is determined based on the pseudo-random sequence c(i), initialized with one of the following IDs.

Option 1: Reuse the SRS sequence identity nIDSRS.

Option 2: Introduce new ID(s).

  - FFS: the value range, one new ID or two separate new IDs, default ID(s)

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s, support the 8 ports equally partitioned into s subsets with each subset having 8/s different ports.

At least s = 2.

  - FFS: s = 4, s = 8.

m = 2,4,8, 10,12,14, and m is a multiple of s.

Each of the m OFDM symbols has only one subset. Reuse the existing resource mapping designed for 8/s ports on each OFDM symbol.

  - Including frequency-domain resource allocation and mapping to cyclic shifts. FFS port indexing within the subset of 8/s ports.

  - FFS: down selection from existing resource mapping designs

FFS: which subset of 8/s ports are mapped onto each OFDM symbol.

FFS: the TDM factor s is configured as an explicit RRC parameter or determined implicitly from other parameters.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy non-TDMed schemes (repetition, frequency hopping, partial sounding, or a combination thereof),

- Option 2: For comb 4, do not support 4 comb offsets.

Agreement

For SRS comb offset hopping and/or cyclic shift hopping, the time-domain hopping behavior depends on at least the slot index ns,fμ within a radio frame and OFDM symbol index l', and select at least one of the following options:

Option 1: Within a slot, hopping based on the repetition factor R and symbol index that is the same across the R repetitions.

Option 2: Within a slot, hopping based on only the symbol index l'.

Option 3: No intra-slot hopping.

FFS: Time domain hopping behaviour further depends on system frame number (SFN) nf.

  - FFS:  reinitialization periodicity of N radio frames or reinitialization based on system frame number.

FFS: Whether to adopt the same option(s) for comb offset hopping and cyclic shift hopping (if supported separately)

FFS: At least support reinitialization at the beginning of each radio frame.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s ≥ 2, the m OFDM symbols are adjacent, and select one of the following options regarding the TDM pattern:

Option 2-1: the s subsets of ports are mapped cyclically as {1, 2, …, s,1, 2, …, s} on the m OFDM symbols.

Option 2-2: the s subsets of ports are mapped sequentially as {1, …, 1, 2, …, 2, s, …, s} on the m OFDM symbols.

Conclusion

No consensus to support the following for TDD CJT SRS enhancement in Rel-18:

Further enhancements to frequency hopping

Sequence hopping/randomization, per-hop sequence from a long SRS sequence

Enhanced configuration of SRS transmission to enable more efficient SRS parameter assignment

Precoded SRS for DL CSI acquisition

Pseudo-random muting of SRS transmission for periodic and semi-persistent SRS

Configuration of  (sequence index within a group) per SRS resource

Multiplying mask sequence to the legacy SRS sequence

Conclusion

No consensus to support SRS TD OCC for TDD CJT SRS enhancement in Rel-18.

## Appendix 6: Agreements from RAN1#112bis-e

Agreement

For SRS comb offset hopping and/or cyclic shift hopping, for a SRS resource, the hopping pattern initialization ID determined by cinit=nIDhop, where nIDhop is a new ID for cyclic shift hopping and/or comb offset hopping.

- The range of the new ID is from 0 to 1023

Agreement

For a SRS resource configured with comb offset hopping and/or cyclic shift hopping,

- If the repetition factor R = 1, within a slot, the time-domain hopping behavior depends on the OFDM symbol index l' of each symbol.

- If the repetition factor R > 1,

  - For cyclic shift hopping, within a slot, the time-domain hopping behavior depends on the OFDM symbol index l' of each symbol.

  - For comb offset hopping, within a slot, the time-domain hopping behavior depends on one of the following alternatives:

    - Alt1: The OFDM symbol index l' of the first symbol across the R repetitions.

    - Alt2: The OFDM symbol index l' of each symbol.

    - Alt3: The OFDM symbol index l' of each symbol or the first symbol across the R repetitions based on configuration, and FFS configuration details.

Agreement

For a SRS resource configured with comb offset hopping, if the repetition factor R > 1, within a slot, the time-domain hopping behavior depends on the OFDM symbol index l' of each symbol or the first symbol across the R repetitions based on RRC configuration, and FFS configuration details.

- UE can indicate whether it supports one or both the options. Details to be discussed in UE feature.

Agreement

For SRS comb offset hopping / cyclic shift hopping, support reinitialization at the beginning of every N radio frame(s), where N ≥ 1.

- FFS: N is fixed or configurable.

Agreement

Whether SRS comb offset hopping can be combined with one of group / sequence hopping on a SRS resource depends on UE feature/capability design.

- FFS: Whether SRS cyclic shift hopping can be combined with one of group / sequence hopping on a SRS resource depends on UE feature/capability design.

FFS: UE feature/capability design details.

Conclusion

No consensus on enhanced per-TRP power control and/or power control of one SRS towards to multiple TRPs in Rel-18.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), and when the resource is assigned with kTC>1 comb offsets, determine the mapping from the ports to comb offsets as follows:

- If kTC=2, ports {1000, 1002, 1004, 1006} are mapped on the first comb offset, and {1001, 1003, 1005, 1007} on the second comb offset

- If kTC=4, ports {1000, 1004} are mapped on the first comb offset, {1001, 1005} on the second comb offset, {1002, 1006} on the third comb offset, and {1003, 1007} on the fourth comb offset.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), and when the resource is configured with comb KTC=2 and with maximum nSRScs,max=8 cyclic shifts per comb offset, the number of comb offset(s) and the cyclic shift locations are determined based on the one RRC configured cyclic shift location nSRScs as follows:

- If nSRScs<nSRScs,max/2, then 1 comb offset is used, otherwise 2 comb offsets are used.

- The 8 cyclic shift locations for the 8 ports are {nSRScs, (nSRScs+1) mod nSRScs,max, …,(nSRScs+7) mod nSRScs,max}, reusing the existing equation nSRScs,i=nSRScs+nSRScs,maxpi-1000NapSRS mod nSRScs,max in 38.211 6.4.1.4.2.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), and when the resource is assigned with comb 4 or comb 8, decide one of the following options:

- Option 1: the cyclic shift positions are completely aligned across the comb offsets on the same OFDM symbol.

  - For comb KTC=4, k=2. For comb KTC=8, k=4. For port pi, nSRScs,i=nSRScs+nSRScs,maxpi-1000kNapSRS/k mod nSRScs,max.

- Option 2: the cyclic shift positions are unaligned across the comb offsets on the same OFDM symbol for comb 4, and the cyclic shift positions are aligned on only 2 of the 4 comb offsets on the same OFDM symbol for comb 8.

  - For comb KTC=4, k=2. For comb KTC=8, k=4.  Example: For port pi, nSRScs,i=nSRScs+(pi-1000mod k)+nSRScs,maxpi-1000kNapSRS/k mod nSRScs,max. FFS equation details.

- FFS: potential impact on PAPR, if any.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM onto m ≥ 2 OFDM symbols in a slot and with TDM factor s, the s subsets of ports are mapped cyclically as {{1, 2, …, s}, …, {1, 2, …, s}} on the m OFDM symbols.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM with TDM factor s, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), the SRS transmissions within each of the m/s groups of {1, 2, …, s} use the same set of subcarriers. If consecutive groups of {1, 2, …, s} are configured as repetition, then the SRS transmissions of the consecutive groups use the same set of subcarriers.

- Note: applicable to the SRS resource with or without FH/RPFS.

- FFS the scenario where comb offset hopping is configured for the SRS resource.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), and when the SRS transmission on a subset of the s OFDM symbols within a group of {1, 2, …, s} is dropped, study at least the following solutions:

- Whether or not a UE drops the SRS transmission on the rest of OFDM symbols within the group of {1, 2, …, s}, based on, for example, the usage, coherency, and/or repetition configuration.

- Whether or not a UE changes the transmission order of the subsets of ports.

## Appendix 7: Agreements from RAN1#113

Agreement

For SRS comb offset hopping / cyclic shift hopping reinitialization periodicity of N radio frame(s):

- N = 128

Agreement

Support configuring a subset of comb offsets when comb offset hopping is configured, and configuring a subset of cyclic shifts when cyclic shift hopping is configured.

- The subset configuration applies to all the port(s) in the SRS resource, and all the port(s) in the SRS resource has (have) the same hopping offset value khopping(pi) on an OFDM symbol.

- This is a UE-optional feature.

Agreement

For SRS cyclic shift hopping, support finer time-delay-domain granularity, e.g., αi=2πnSRScs,inSRScs,max+2πnSRScs,offsetK×nSRScs,max, where nSRScs,offset can be randomly chosen from 0,1,…K×nSRScs,max-1 at each SRS transmission.

- Note: The finer granularity above only applies to the cyclic shift offsets when cyclic shift hopping is enabled.

If a subset for cyclic shifts is configured, this feature cannot be configured.

Above is a UE optional feature.

Agreement

SRS comb offset hopping / cyclic shift hopping can be configured for aperiodic SRS.

Agreement

Whether SRS cyclic shift hopping can be combined with one of group / sequence hopping on a SRS resource depends on UE feature/capability design.

Agreement

SRS comb offset hopping and cyclic shift hopping can be configured for a SRS resource at the same time as a separate UE capability. No joint hopping scheme is supported.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’, when the 8 ports are mapped onto one or more OFDM symbols using legacy schemes (repetition, frequency hopping, partial sounding, or a combination thereof), and when the resource is assigned with comb 4 on 2 comb offsets (KTC=4, kTC=2) or comb 8 on 4 comb offsets (KTC=8, kTC=4), the cyclic shift positions are completely aligned across the comb offsets on the same OFDM symbol.

- For port pi, nSRScs,i=nSRScs+nSRScs,maxpi-1000kTCNapSRS/kTC mod nSRScs,max.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and with TDM factor s > 1, the UE splits a linear value PSRS of SRS transmission power equally across the SRS ports configured on each OFDM symbol, if the UE is capable of transmitting at PCMAX per OFDM symbol with 8/s ports, where PCMAX is specified in the current specifications.

- Note: This may be captured in the specification in a few different but equivalent ways, and it is up to the editor to decide.

Conclusion

There is no consensus on the support of the following feature in RAN1:

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM, support TDM factor s = 4.

## Appendix 8: Agreements from RAN1#114

Agreement

When finer time-delay-domain granularity for SRS cyclic shift hopping is configured, K is 2

  - FFS (to be decided this week) Support of K=4

Agreement

For the SRS hopping formula in cyclic shift hopping or comb offset hopping except for SRS configured with TDM, let N=128 and M=7:

- For cyclic shift hopping: αi=2πnSRScs,inSRScs,max+2πfCS, hopKnSRScs,max, where fCS, hop=S{(m=0Mc8t+m∙2m) mod Y}, and

  - t=modSFN,NNslotframe,μNsymbslot+ns,fμNsymbslot+l0+l',

  - If cyclicShiftHoppingSubset is not configured, S{n}=n.

    - If cyclicShiftHoppingFinerGranularity is not configured, Y=nSRScs,max and K=1.

    - If cyclicShiftHoppingFinerGranularity is configured, Y=K.nSRScs,max, and K=2

  - If cyclicShiftHoppingSubset is configured, S{n} denotes the (n+1)th element of the configured subset, Y is the number of elements in the subset, and K=1.

- For comb offset hopping: k0(pi)=nshiftNscRB+kTC(pi)+fTC, hop+koffsetl' mod KTC, where fTC, hop=S{(m=0Mc8t+m∙2m) mod Y}, and

  - t=mod(SFN,N)Nslotframe,μNsymbslot+ns,fμNsymbslot+l'',

    - l''=l0+l' if R=1 or UE is provided with combOffsetHoppingWithRepetition=Per-symbol; otherwise, l'' is the OFDM symbol index of the first symbol across the R repetitions within the slot.

  - If combOffsetHoppingSubset is not configured, S{n}=n, and Y=KTC.

  - If combOffsetHoppingSubset is configured, S{n} denotes the (n+1)th element of the configured subset, and Y is the number of elements in the subset.

Conclusion

When finer time-delay-domain granularity for SRS cyclic shift hopping is configured, K = 4 is not supported.

Agreement

When a subset of comb offsets for comb offset hopping is configured, and when a subset of cyclic shifts for cyclic shift hopping is configured, support the following option for configuring the subset S={S(0), S(1), …, S(z-1)} with 1<z<Z, where Z=KTC for comb offset hopping and Z=nSRScs,max for cyclic shift hopping, and:

  - Option 1b: S(0), S(1), …, S(z-1) are configured via a Z-length bitmap with S(i-1) being the i-th bit set as 1.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM with TDM factor s = 2, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot the port, down select from the following options:

  - Option 1: The first subset includes ports {1000, 1001, 1004, 1005}, and the second subset includes {1002, 1003, 1006, 1007}.

Conclusion

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM with TDM factor s and repetition factor R, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), and when cyclic shift hopping is configured for the SRS resource,

- Option A4: Do not support cyclic shift hopping for 8-port SRS with TDM.

Conclusion

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM with TDM factor s and repetition factor R, when the s subsets of ports are mapped onto m ≥ 2 OFDM symbols in a slot according to the pattern {{1, 2, …, s}, …, {1, 2, …, s}} (totally m/s groups of {1, 2, …, s}), and when comb offset hopping is configured for the SRS resource,

  - Option B5: Do not support comb offset hopping for 8-port SRS with TDM.

Agreement

For an 8-port SRS resource in a SRS resource set with usage ‘codebook’ or ‘antennaSwitching’ and resource mapping based on TDM with TDM factor s, when sequence/group hopping is configured for the SRS resource, the time-domain behavior of hopping depends only on the OFDM symbol index l’ of each symbol.