3GPP TSG-RAN WG1 Meeting # 114bis	R1-2310340

# Xiamen, China, October 9th – 13th, 2023

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.901 | CR | Draft | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on angle scaling procedure for CDL channel in TR38.901 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2023-10-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The angle scaling (and shifting) procedure of CDL channel is described in Section 7.7.5.1 of TR38.901 where the following formula is provided: ϕn,scaled=ASdesiredASmodel∙ϕn,model-μϕ,model+μϕ,desired. The following issues are identified with this scaling formula: - Issue #1: If ASdesiredASmodel≠1, the current angle scaling formula of TR38.901 may cause issue with azimuth angle discontinuity at 180° and -180°. It can cause unwanted consequences when the azimuth angles are around 180° or -180°. To fix this issue, WrapTo180ϕn,model-μϕ,model can be used instead of ϕn,model-μϕ,model for azimuth angles. - Issue #2: If ASdesiredASmodel≠1, the mean angle is also impacted by the AS scaling factor ASdesiredASmodel, and as a result, the current angle scaling formula of TR38.901 does not result in desired mean angle μϕ,desired. To resolve this issue, a two-step approach can be used as follows: ϕn,intermediate=ASdesiredASmodel∙WrapTo180ϕn,model-μϕ,model ϕn,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired. - Issue # 3: If ASdesiredASmodel≠1, the current angle scaling formula of TR38.901 does not result in the desired ASdesired. Specifically, linear scaling of the angles by ASdesiredASmodel does not necessarily result in ASdesired using the AS definition in Annex A of TR38.901, i.e. AS=-2lnn=1Nm=1Mexpjϕn,mPn,mn=1Nm=1MPn,m. Mathematically speaking, ASdesired≠-2lnn=1Nm=1MexpjASdesiredASmodelϕn,mPn,mn=1Nm=1MPn,m with ASmodel≜-2lnn=1Nm=1Mexpjϕn,mPn,mn=1Nm=1MPn,m. To fix this issue, we define the function ASx≜-2lnn=1Nm=1Mexpjxϕn,mPn,mn=1Nm=1MPn,m where x≥0 is the angle scaling factor. For a given ASdesired, we need to find x*=ScalingFactor(ASdesired) such that ASx*=ASdesired. It can be shown that ScalingFactor(ASdesired)≠ASdesiredASmodel for almost all values of ASdesired, except for the obvious value of ASdesired=ASmodel and possibly a few other values of ASdesired corresponding to intersections of ASx and gx≜ASmodel×x. Also, it can be seen that ASx for each of AOA, AOD, ZOA, and ZOD of CDL-B is bounded above by some constant. In other words, for each of AOA, AOD, ZOA, and ZOD, there is a certain threshold ASTH>0 such that a value of ASdesired>ASTH cannot be achieved by linear scaling of the angles. Moreover, it can be shown that AS(x) is not an invertible function, and thus, there might be more than one answer for ScalingFactor(ASdesired) for a given ASdesired, in which case we can take the minimum value of those answers as the value of ScalingFactor(ASdesired): ScalingFactor(ASdesired)≜minx≥0{x: ASx=ASdesired}. In summary, the following angle scaling formula would resolve all three issues #1, #2, and #3: ϕn,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired, where ϕn,intermediate=ScalingFactor(ASdesired)∙Angleϕn,model-μϕ,model, and Anglex=x,if x is a zenith angleWrapTo180x,if x is an azimuth angle. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modify equation (7.7-5) of TR38.901 as follows: - Wrap the azimuth ray angles to interval [-180,180]. - Introduce a two-step approach using intermediate ray angles to achieve desired mean angles after AS scaling. - Modify the AS scaling factor to achieve the desired rms angular spread. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Using the current formula in TR 38.901 for scaling of CDL ray angles will lead to invalid channel models for certain simuation setups which will then results in invalid evaluation results for radio communication. It will also potentially lead to misalignment beteen different companies’ results if some companies fix the issues on their ownand some companies do not fix them and/or different companies use different approaches to fix the issues. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.7.5.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated Impact analysis: This CR has isolated impact as it only affects the channel characteristics of certain simulation setups for CDL models. The CR has no direct impact on UE implementation or network implementation. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 7.7.5.1	CDL extension: Scaling of angles

The angle values of CDL models are fixed, which is not very suitable for MIMO simulations for several reasons; The PMI statistics can become biased, and a fixed precoder may perform better than open-loop and on par with closed-loop or reciprocity beamforming. Furthermore, a CDL only represents a single channel realization. The predefined angle values in the CDL models can be generalized by introducing angular translation and scaling. By translation, mean angle can be changed to  and angular spread can be changed by scaling. The translated and scaled ray angles can be obtained according to the following equation:

ϕn,scaled=ϕn,intermediate-μϕ,intermediate+μϕ,desired,                  (7.7-5)

where

ϕn,intermediate=ScalingFactor(ASdesired)∙Angleϕn,model-μϕ,model,

and

Anglex=x,if x is a zenith angleWrapTo180x,if x is an azimuth angle.

ScalingFactorASdesired=minx≥0{x: ASx=ASdesired}.

ASx=-2lnn=1Nm=1Mexpjxϕn,mPn,mn=1Nm=1MPn,m

ϕn,scaled=ASdesiredASmodel∙ϕn,model-μϕ,model+μϕ,desired

in which:

is the tabulated CDL ray angle	is the rms angular spread of the tabulated CDL including the offset ray angles, calculated using the angular spread definition in Annex A

is the mean angle of the tabulated CDL, calculated using the definition in Annex A

μϕ,intermediate is the mean angle of the intermediate ray angles, calculated using the definition in Annex A

is the desired mean angle

is the desired rms angular spread

is the resulting scaled ray angle.

WrapTo180(⋅) is a function which wraps an azimuth angle to the interval [-180, 180].

The angular scaling is applied on the ray angles including offsets from the tabulated cluster angles. Typical angular spreads for different scenarios can be obtained from the system-level model.

Example scaling values are:

-	AOD spread (ASD) for each CDL model: {5, 10, 15, 25} degrees.

-	AOA spread (ASA) for each CDL model: {30, 45, 60} degrees.

-	ZOA spread (ZSA) for each CDL model: {5, 10, 15} degrees.

-	ZOD spread (ZSD) for each CDL model: {1, 3, 5} degrees.

The angular scaling and translation can be applied to some or all of the azimuth and zenith angles of departure and arrival.

Note 1: The azimuth angles may need to be wrapped around to be within [0, 360] degrees, while the zenith angles may need to be clipped to be within [0, 180] degrees.

Note 2: For each of AOA, AOD, ZOA, and ZOD, not all desired AS values, ASdesired, larger than the model AS value, ASmodel, may be achievable by equation (7.7-5).