3GPP TSG RAN WG1 #114bis                                               R1-2310522

Xiamen, China, October 9th – October 13th, 2023

Source:		Moderator (vivo)

Title:		Summary of discussion on PUSCH selection rule for UCI multiplexing

Agenda Item:		7.1

Document for: 	Discussion and Decision

- Introduction

The document is to collect companies’ inputs and provide a summary for the discussion

R1-2309048	Draft Rel-16 CR on PUSCH selection rule for UCI multiplexing	vivo

R1-2309747	Discussion on PUSCH selection rule for UCI multiplexing	Huawei, HiSilicon

R1-2310254	Discussion on PUSCH selection rule for UCI multiplexing	Ericsson

R1-2309049	Draft Rel-17 CR on PUSCH selection rule for UCI multiplexing	vivo

Ths issue was discussed in the last meeting and the summary can be found in R1-2308532.

Companies are highly appreciated providing your inputs.

- Discussions

- Background

In RAN1#97 meeting, the following conclusion was made for UCI multiplexing to clarify the common understanding on intended UE behavior per specification:

Conclusion: For the issue raised in the draft CR R1-1906302, the intended UE behavior per specification is commonly understood as follows: - For UCI multiplexing, within a PUCCH group, on PUSCH, the following two steps are performed with step 1 first, then followed by step 2: - Step 1: UCI in overlapped PUCCH transmissions is multiplexed into one PUCCH resource (resource Z). This step is done per PUCCH slot. - Step 2: UCI, that doesn’t include SR, in Z is multiplexed into one PUSCH, if Z overlaps with at least one PUSCH, following the priorities (sequentially from high to low) as listed below. - First priority: PUSCH with A-CSI as long as it overlaps with Z - Second priority: earliest PUSCH slot(s) based on the start of the slot(s) - If there are still multiple PUSCHs overlap with Z in the earliest PUSCH slot(s), follow the following priorities (sequentially from high to low) - Third priority: Dynamic grant PUSCHs > PUSCHs configured by respective ConfiguredGrantConfig or semiPersistentOnPUSCH - Fourth priority: PUSCHs on serving cell with smaller serving cell index > PUSCHs on serving cell with larger serving cell index - Fifth priority: Earlier PUSCH transmission > later PUSCH transmission Note: The clarification applies to both cases with the same (except the second priority part) and different numerologies among PUCCH and PUSCHs.

In the previous meetings, an issue of HARQ-ACK multiplexing on PUSCH in the absence of PUCCH was discussed, the following agreements were made in RAN1#107-e and RAN1#109-e meetings to address the issue. Note that, according to the agreement, prioritization rules to select PUSCH for multiplexing are identical to 38.213 and there is no intention to change the prioritization rules.

Agreement For Rel-16 with one PUSCH and no overlapping PUCCH with HARQ-ACK within a span of one PUCCH slot (both single carrier and UL CA), if the UL-TDAI is not equal to 4 (for Type 2 codebook) or equal to 1 (for Type 1 codebook), the UE multiplexes HARQ-ACK following the UL-TDAI into the PUSCH. Agreement For Rel-16 UEs, in the scenario with more than one PUSCH (overlapping and non-overlapping) and no overlapping PUCCH with HARQ-ACK within a span on one PUCCH slot (both single carrier and UL CA), for a unified design, the following should be specified: - Selection of the candidate PUSCH for multiplexing: PUSCHs without UL-TDAI=4 in case Type 2 CB, and without UL-TDAI n.e. 1 in case of Type 1 CB within the PUCCH slot are candidates - Prioritization rules to select PUSCH for multiplexing. Prioritization rules are identical to 38.213 - Limitations for multiplexing - UE expects to multiplex HARQ-ACK on only 1 PUSCH selected based on step 2 in the PUCCH slot. - All the PUSCHs in the determined candidate set after step 1 have to satisfy Rel-15 UCI multiplexing timeline, defined with respect the starting symbol of the earliest PUSCH transmission in the candidate set. The above specified behavior is supported subject to a new Rel-16 UE capability [Multiplexing-HARQ-ACK-without-PUCCH-on-PUSCH] - FFS: the details of the capability signaling

To capture the above agreements, the following CRs were endorsed for Rel-16 and Rel-17, respectively.

Final CRs in R1-2205628 Correction for HARQ-ACK multiplexing on PUSCH in the absence of PUCCH Moderator (Apple), Ericsson, CATT Decision: (TS38.213, Rel-16, CR#0316, Cat. F) is agreed. R1-2205629 Correction for HARQ-ACK multiplexing on PUSCH in the absence of PUCCH Moderator (Apple), Ericsson, CATT Decision: (TS38.213, Rel-17, CR#0317, Cat. A) is agreed.

In this meeting, Tdocs [1], [2], [4] intend to align the following spec in TS 38.213 with the above conclusion.

The following issue was identified in [1]-[4]:

Issue: The PUSCH selection rule for UCI multiplexing rule in current spec is not aligned with the conclusion

Per the conclusion, for the PUSCH selection rule for UCI multiplexing rule, the intended UE behavior per specification is as follows:

  - if a PUCCH overlaps with at least one PUSCH, following the priorities (sequentially from high to low) as listed below.

    - First priority: PUSCH with A-CSI as long as it overlaps with Z

    - Second priority: earliest PUSCH slot(s) based on the start of the slot(s)

    - If there are still multiple PUSCHs overlap with Z in the earliest PUSCH slot(s), follow the following priorities (sequentially from high to low)

      - Third priority: Dynamic grant PUSCHs > PUSCHs configured by respective ConfiguredGrantConfig or semiPersistentOnPUSCH

      - Fourth priority: PUSCHs on serving cell with smaller serving cell index > PUSCHs on serving cell with larger serving cell index

      - Fifth priority: Earlier PUSCH transmission > later PUSCH transmission

Note: The clarification applies to both cases with the same (except the second priority part) and different numerologies among PUCCH and PUSCHs.

According to the current specification, when there are multiple PUSCHs overlapping with one PUCCH, the following priority order is used to determine a PUSCH for UCI multiplexing, which is different with that in Rel-15 for the case with different numerologies among PUCCH and PUSCHs.

- First priority: PUSCH with A-CSI

- Second priority: Dynamic grant PUSCHs > PUSCHs configured by respective ConfiguredGrantConfig or semiPersistentOnPUSCH

- Third priority: PUSCHs on serving cell with smaller serving cell index > PUSCHs on serving cell with larger serving cell index

- Fourth priority: Earlier PUSCH transmission > later PUSCH transmission

It is observed that the Second priority in the obove conclusion is missed from the current specification which results in different PUSCH selection rule for the case a PUCCH overlapps with multiple PUSCHs in different slots and may cause a misunderstanding on the selected PUSCH for UCI multiplexing between gNB and UE in the concerned case since gNB may not know a UE is a Rel-15 UE or a Rel-16/17 UE.

Thus, the following TP is proposed by Huawei for Rel-15.

## TP#1, changes on TS38.213, v15.15.0

------------------------------- Start of Text Proposal for TS 38.213 ------------------------------- 9 UE procedure for reporting control information < Unchanged parts are omitted > If a UE multiplexes aperiodic CSI in a PUSCH and the UE would multiplex UCI that includes HARQ-ACK information in a PUCCH that overlaps with the PUSCH and the timing conditions for overlapping PUCCHs and PUSCHs in Clause 9.2.5 are fulfilled, the UE multiplexes only the HARQ-ACK information in the PUSCH and does not transmit the PUCCH. If a UE transmits multiple PUSCHs on respective serving cells with reference to a single PUCCH slot and the UE would multiplex UCI in one of the multiple PUSCHs, and the multiple PUSCHs fulfil the conditions in Clause 9.2.5 for UCI multiplexing, the UE multiplexes the UCI in a PUSCH from the earliest PUSCH slot(s) based on the start of the slot(s). If a UE transmits multiple PUSCHs in a slot on respective serving cells that include first PUSCHs that are scheduled by DCI format(s) 0_0 or DCI format(s) 0_1 and second PUSCHs configured by respective ConfiguredGrantConfig or semiPersistentOnPUSCH, and the UE would multiplex UCI in one of the multiple PUSCHs, and the multiple PUSCHs fulfil the conditions in Clause 9.2.5 for UCI multiplexing, the UE multiplexes the UCI in a PUSCH from the first PUSCHs. If a UE transmits multiple PUSCHs in a slot on respective serving cells and the UE would multiplex UCI in one of the multiple PUSCHs and the UE does not multiplex aperiodic CSI in any of the multiple PUSCHs, the UE multiplexes the UCI in a PUSCH of the serving cell with the smallest ServCellIndex subject to the conditions in Clause 9.2.5 for UCI multiplexing being fulfilled. If the UE transmits more than one PUSCHs in the slot on the serving cell with the smallest ServCellIndex that fulfil the conditions in Clause 9.2.5 for UCI multiplexing, the UE multiplexes the UCI in the earliest PUSCH that the UE transmits in the slot. If a UE transmits a PUSCH over multiple slots and the UE would transmit a PUCCH with HARQ-ACK and/or CSI information over a single slot and in a slot that overlaps with the PUSCH transmission in one or more slots of the multiple slots, and the PUSCH transmission in the one or more slots fulfills the conditions in Clause 9.2.5 for multiplexing the HARQ-ACK and/or CSI information, the UE multiplexes the HARQ-ACK and/or CSI information in the PUSCH transmission in the one or more slots. The UE does not multiplex HARQ-ACK and/or CSI information in the PUSCH transmission in a slot from the multiple slots if the UE would not transmit a single-slot PUCCH with HARQ-ACK and/or CSI information in the slot in case the PUSCH transmission was absent. If the PUSCH transmission over the multiple slots is scheduled by a DCI format 0_1, the same value of a DAI field is applicable for multiplexing HARQ-ACK information in the PUSCH transmission in any slot from the multiple slots where the UE multiplexes HARQ-ACK information. A HARQ-ACK information bit value of 0 represents a negative acknowledgement (NACK) while a HARQ-ACK information bit value of 1 represents a positive acknowledgement (ACK). < Unchanged parts are omitted >

Similar TPs are proposed by vivo and Huawei for Rel-16.

## TP#2, changes on TS38.213, v16.15.0

Version 1 from [1]:

- UE procedure for reporting control information < Unchanged parts are omitted > When a UE transmits multiple PUSCHs on respective serving cells in a slot with reference to slots for PUCCH transmissions and the multiple PUSCHs overlap with a PUCCH carrying UCI in the slot, the UE selects all the PUSCHs overlapping with the PUCCH as the candidate PUSCHs for UCI multiplexing within the slot. If a UE would transmit a single PUSCH scheduled by a DCI format that includes a DAI field on a serving cell in a slot with reference to slots for PUCCH transmissions without any other PUSCH that would be transmitted on any serving cell in the slot and the UE does not determine any PUCCH carrying HARQ-ACK information in the slot, or if the UE indicates the corresponding capability mux-HARQ-ACK-withoutPUCCH-onPUSCH and the UE transmits multiple PUSCHs on respective serving cells in a slot with reference to slots for PUCCH transmissions and the UE does not determine any PUCCH carrying HARQ-ACK information in the slot and at least one of the multiple PUSCHs is scheduled by a DCI format that includes a DAI field, the UE selects the single PUSCH or all the multiple PUSCHs in the slot as the candidate PUSCHs for HARQ-ACK multiplexing within the slot except for any PUSCH among the multiple PUSCHs that is scheduled by a DCI format that includes a DAI field that is equal to 4 in case the UE is configured with pdsch-HARQ-ACK-Codebook = dynamic or with pdsch-HARQ-ACK-Codebook-r16, or is equal to 0 in case the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static. The UE determines the PUSCH for UCI multiplexing by applying the following procedure on the candidate PUSCHs as described in this clause where the timing conditions for overlapping PUCCHs and PUSCHs in clause 9.2.5 are fulfilled: - If the candidate PUSCHs include a PUSCH with aperiodic CSI, and the UE would multiplex UCI that includes HARQ-ACK information in a PUCCH that overlaps with the PUSCH, the UE multiplexes only the HARQ-ACK information in the PUSCH and does not transmit the PUCCH. - Else - If the candidate PUSCHs are in more than one slots starting at different time, the UE multiplexes the UCI in a PUSCH from the PUSCHs in a slot starting earliest and does not transmit the PUCCH. - If the candidate PUSCHs include first PUSCHs that are scheduled by DCI formats and second PUSCHs configured by respective ConfiguredGrantConfig or semiPersistentOnPUSCH in the slot starting earliest, , the UE multiplexes the UCI in a PUSCH from the first PUSCHs and does not transmit the PUCCH. - If the candidate PUSCHs include PUSCHs on more than one serving cells in the slot starting earliest, the UE multiplexes the UCI in a PUSCH on the serving cell with the smallest ServCellIndex and does not transmit the PUCCH . If the UE transmits more than one PUSCHs in the slot starting earliest on the serving cell with the smallest ServCellIndex , the UE multiplexes the UCI in the earliest PUSCH that the UE transmits in the slot and does not transmit the PUCCH. If a UE transmits a PUSCH over multiple slots or multiple PUSCHs over multiple slots that are scheduled by a DCI format 0_1, and the UE would transmit a PUCCH with HARQ-ACK and/or CSI information over a single slot that overlaps with the PUSCH transmission in one or more slots of the multiple slots, and the PUSCH transmission in the one or more slots fulfills the conditions in clause 9.2.5 for multiplexing the HARQ-ACK and/or CSI information, the UE multiplexes the HARQ-ACK and/or CSI information in the PUSCH transmission in the one or more slots. The UE does not multiplex HARQ-ACK and/or CSI information in the PUSCH transmission in a slot from the multiple slots if the UE would not transmit a single-slot PUCCH with HARQ-ACK and/or CSI information in the slot in case the PUSCH transmission was absent. < Unchanged parts are omitted >

Version 2 from [2]:

------------------------------- Start of Text Proposal for TS 38.213 ------------------------------- 9 UE procedure for reporting control information < Unchanged parts are omitted > If a UE multiplexes aperiodic CSI in a PUSCH and the UE would multiplex UCI that includes HARQ-ACK information in a PUCCH that overlaps with the PUSCH and the timing conditions for overlapping PUCCHs and PUSCHs in clause 9.2.5 are fulfilled, the UE multiplexes only the HARQ-ACK information in the PUSCH and does not transmit the PUCCH. When a UE transmits multiple PUSCHs on respective serving cells in a slot with reference to slots for PUCCH transmissions and the multiple PUSCHs overlap with a PUCCH carrying UCI in the slot, the UE selects all the PUSCHs overlapping with the PUCCH as the candidate PUSCHs for UCI multiplexing within the slot. If a UE would transmit a single PUSCH scheduled by a DCI format that includes a DAI field on a serving cell in a slot with reference to slots for PUCCH transmissions without any other PUSCH that would be transmitted on any serving cell in the slot and the UE does not determine any PUCCH carrying HARQ-ACK information in the slot, or if the UE indicates the corresponding capability mux-HARQ-ACK-withoutPUCCH-onPUSCH and the UE transmits multiple PUSCHs on respective serving cells in a slot with reference to slots for PUCCH transmissions and the UE does not determine any PUCCH carrying HARQ-ACK information in the slot and at least one of the multiple PUSCHs is scheduled by a DCI format that includes a DAI field, the UE selects the single PUSCH or all the multiple PUSCHs in the slot as the candidate PUSCHs for HARQ-ACK multiplexing within the slot except for any PUSCH among the multiple PUSCHs that is scheduled by a DCI format that includes a DAI field that is equal to 4 in case the UE is configured with pdsch-HARQ-ACK-Codebook = dynamic or with pdsch-HARQ-ACK-Codebook-r16, or is equal to 0 in case the UE is configured with pdsch-HARQ-ACK-Codebook = semi-static. The UE determines the PUSCH for UCI multiplexing by applying the following procedure on the candidate PUSCHs as described in this clause: - If a UE multiplexes aperiodic CSI in a candidate PUSCH and the UE would multiplex UCI that includes HARQ-ACK information in a PUCCH that overlaps with the candidate PUSCH and the timing conditions for overlapping PUCCHs and candidate PUSCHs in clause 9.2.5 are fulfilled, the UE multiplexes only the HARQ-ACK information in the candidate PUSCH and does not transmit the PUCCH. - If the candidate PUSCHs are in different slots, and candidate PUSCHs fulfil the conditions in clause 9.2.5 for UCI multiplexing, the UE multiplexes UCI in one of candidate PUSCH in the earliest PUSCH slot(s) based on the start of the slot(s). - If the candidate PUSCHs include first PUSCHs that are scheduled by DCI formats and second PUSCHs configured by respective ConfiguredGrantConfig or semiPersistentOnPUSCH, and the UE would multiplex UCI in one of the candidate PUSCHs, and the candidate PUSCHs fulfil the conditions in clause 9.2.5 for UCI multiplexing, the UE multiplexes the UCI in a PUSCH from the first PUSCHs. - If the UE would multiplex UCI in one of the candidate PUSCHs and the UE does not multiplex aperiodic CSI in any of the candidate PUSCHs, the UE multiplexes the UCI in a PUSCH of the serving cell with the smallest ServCellIndex subject to the conditions in clause 9.2.5 for UCI multiplexing being fulfilled. If the UE transmits more than one PUSCHs in the slot on the serving cell with the smallest ServCellIndex that fulfil the conditions in clause 9.2.5 for UCI multiplexing, the UE multiplexes the UCI in the earliest PUSCH that the UE transmits in the slot. If a UE transmits a PUSCH over multiple slots or multiple PUSCHs over multiple slots that are scheduled by a DCI format 0_1, and the UE would transmit a PUCCH with HARQ-ACK and/or CSI information over a single slot that overlaps with the PUSCH transmission in one or more slots of the multiple slots, and the PUSCH transmission in the one or more slots fulfills the conditions in clause 9.2.5 for multiplexing the HARQ-ACK and/or CSI information, the UE multiplexes the HARQ-ACK and/or CSI information in the PUSCH transmission in the one or more slots. The UE does not multiplex HARQ-ACK and/or CSI information in the PUSCH transmission in a slot from the multiple slots if the UE would not transmit a single-slot PUCCH with HARQ-ACK and/or CSI information in the slot in case the PUSCH transmission was absent. < Unchanged parts are omitted >

Ericsson thinks it is important to examine whether the CR results in different UE behaviour as compared to the existing specification. They have identified few cases as shown in Figure 1 that the CR changes the UE behaviour. Based on this observation, they observe the following and propose:

Observation 1	The selected PUSCH for UCI multiplexing can be different in some cases following the existing specification as compared to the proposed draft CR.

Proposal 1	The CR accompanied with any optional capability is not acceptable.  Rejecting the CR implies that the UE follows the procedures in existing specification.

Figure 1: Comparing PUSCH selection for UCI multiplexing based on existing specification and CR

- Company views (Round 1)

Please kindly provide your views in the tables below.

- Question 1: Do you agree that the UE behaviour in Rel-15 should follow the conclusion in RAN1#97 meeting?

| Company | YES/NO | Comment |
| --- | --- | --- |
| MTK |  | Normally, this answer should be yes. However, for a part of spec which has been stable and implemented in field for a long time, maybe following spec instead of RAN1 conclusion is better. After all, vendor product implementation team colleagues read the spec, not RAN1 conclusion. |
| DCM |  | The most important thing would be that UE implementation is aligned. We are not sure all UE implementation follow the conclusion.. |
| CATT | YES | We raised this issue in RAN1#97 and companies did not agree to change the spec but only agreed a conclusion to clarify the UE behavior which is considered as common understanding among companies. The intended UE behavior is clear from the conclusion, which is used to explain the spec. The conclusion should not be ignored. Conclusion: For the issue raised in the draft CR R1-1906302, the intended UE behavior per specification is commonly understood as follows: ……. |
| Apple |  | Similar view as MTK |
| QC |  | Same view as MTK. |
| Nokia, NSB |  | Same view as DCM and MTK. |
| ZTE |  | Same view as MTK. |
| vivo | YES | Rel-15 spec may not be clear enough and the conclusion was made to clarify the correct understanding for the spec. According to the 3GPP rule, when UE implements the PUSCH selection rules, both spec and conclusion shall be checked together. |
| Huawei, HiSilicon | Yes | The conclusion reached in RAN1#97 shall be respected. It is clearly stated the UE behavior and UE shall perform PUSCH selection based on the conclusion. We disagree the opinion that “spec is spec”, and ignore the conclusion RAN has made. If so, any conclusion made in RAN1 could be discard, since they are not captured in the spec. |
| Ericsson |  | The question here is that what it is the current status of the implementation. Based on the feedback, it seems to us that majority UE vendors have implemented according to the spec. The situation would have been different if the spec with conclusion had been considered for implementation (which for some cases are, but it seems for this case, is not). Therefore, we understand the frustration that in RAN1#97 the proposed changes was not considered to be captured in spec, and we symphatize with that. But our point is that since UL CA is implemented, we need to take implementation into account. |
| LGE |  | Same view as MTK. |

- Question 2: Do you think the PUSCH selection rule in Rel-16 is not intended to be changed?

| Company | YES/NO | Comment |
| --- | --- | --- |
| MTK | Yes | It is not intended to be changed, in principle. |
| DCM | Yes |  |
| CATT | Yes |  |
| Apple | Yes | No change to spec |
| QC | Yes |  |
| Nokia, NSB |  | The question is irrelevant to the question at hand. |
| ZTE | Yes |  |
| vivo | Yes | The agreement is very clear that prioritization rules are identical to 38.213. There was no intension to make any change. |
| Huawei, HiSilicon | Yes | The PUSCH selection rule in Rel-16 and later release should be same as Rel-15 |
| Ericsson |  | We except same behaviour from UE irrespective of release. |
| LGE | Yes |  |

- Question 3: Since different implementations are observed in the field from companies. Moderator think the following alternatives can be considered. Which alternative can you accept/support?

- Alt 1: No any conclusion/CR for the issue.

- Alt 2: It is up to implementation and it is understood that a UE may implement following the order in the conclusion or the spec.

- Alt 3: Update the conclusion in RAN1#97 meeting to remove the second priority, i.e. Second priority: earliest PUSCH slot(s) based on the start of the slot(s), and renumber the other priorities.

- Alt 4: Adopt a CR from Rel-16 or later release to capture the second priority in the conclusion, i.e. Second priority: earliest PUSCH slot(s) based on the start of the slot(s), in the spec clearly.

| Company | Alt | Comment |
| --- | --- | --- |
| MTK | 1 or 3 | If most of UE chip vendors are doing this part the same way, we tend to think maybe it’s better to leave spec as it currently is. |
| DCM | 3 or 4 | Dependent on the current UE implementation. |
| CATT | Alt 4 (1st preference) Alt 3 (2nd preference) | We would prefer to have a unified UE behavior so we prefer Alt 3/4. |
| Apple | 1 |  |
| QC | 1 | We understand the history/context of this isse. Unfortunately, there are two different UE implementations already. We cannot change the UEs already deployed in the field. We recommend to conclude all future UEs’ implementation to follow the current specification. |
| Nokia, NSB |  | Depending on what the prevailing current implementation is. A new conclusion making it clear that the specification as it now stands should be followed could be useful if that is what the clear majority of the implementations support. This could be Alt 3. |
| ZTE | 1 or 3 | It’s an unfortunate situation that the concussion is not aligned with the current spec. But spec is spec, in such situation, we need to keep the current spec. We are ok to further update the conclusion. |
| vivo | 3 or 4 | For Alt 1 or alt 2, the issue is still open. Considering the UL CA with different SCS case is deployed in the field, we should not sweep the problem under the carpet. As a proposer of the CR, we of course prefer alt 4. Either alt 3 or alt 4 is acceptable to us. |
| Huawei, HiSilicon | Alt.4 | Our first preference is updating the spec from R15, considering it is in a quite late stage, we are also fine to correct it since R16 or later release. |
| Ericsson |  | QC suggestion would be a good way forward. Please also see our previous comment. This is a special (but important case) that we have found out there is inconsistency. And since the feature is implemented (please note that is not the case for many other features), and implementations had followed the spec, we should respect that since that is much more important. |
| LGE | 1 or 3 | No spec change is preferred, and OK to update the conclusion if necessary. |

- Additional comment(s) if any: Please address your additional comments and input here.

| Company | Comment |
| --- | --- |

- Summary of Round1

Based on the feedback in round 1, it can be summried that company agree that normally, both spec and conclusion should be respected, but implementation in the field is also important to take into account for the issue and majority UE vendors have implemented according to the spec.

Company views can be summaried in following table.

| Alternative | Support/accept |
| --- | --- |
| Alt 1 | MTK, Aplle, QC(to conclude all future UEs’ implementation to follow the current specification), ZTE, [Ericsson](QC suggestion would be a good way forward) |
| Alt 3 | MTK, DCM, CATT, ZTE, vivo, Nokia |
| Alt 4 | DCM, CATT, vivo, Huawei, HiSilicon |

The feature is implemented, everybody should know the concenquence if we take nothing for the issue, i.e., different impementations exsit in the filed. Maybe nobody want to see that. Let’s try QC suggestion forward:

Proposed Conclusion:

For the issue raised in R1-2309048, for PUSCH selection rule for UCI multiplexing,

it is concluded that all UEs’ implementation from [Rel-17?] follow the current specification.

- Company views (Round 2)

Proposed Conclusion:

For the issue raised in R1-2309048, for PUSCH selection rule for UCI multiplexing,

it is concluded that all UEs’ implementation from [Rel-17?] follow the current specification.

Please kindly provide your views in the table below if you have strong concern.

| Company | Support/object | Comment |
| --- | --- | --- |
| LGE |  | OK if majority is fine to have it. |
| Huawei, HiSilicon | Object | We have strong concern on this conclusion. It is just simply reverting previous conclusion based on part of companies favor, which makes the procedure of RAN1 discussion less reliable. For moving forward, we provide following two options additionaly. Option 1: Proposed conclusion: For the issue raised in R1-2309048, for PUSCH selection rule for UCI multiplexing,it is concluded that all UEs’ implementation from [Rel-17?] follow the conclusion agreed in RAN1 #97. Option 2: Propose conclusion: For the issue raised in R1-2309048, for PUSCH selection rule for UCI multiplexing, it is concluded that UE might have different implementations. We respect companies views, if no consensus on the UE haviour in this meeting, we think option 2 could be taken. |
| DCM |  | We are fine with the proposal, and also fine with Option 1 in HW’s comment, based on majority companies’ preference. Personal feeling is that we should respect RAN1#97 conclusion, but this feeling would not be important. |
| Huawei, HiSilicon 2 | Obeject | After further offline discussion with some companies. It seems supporting the second priprity as a UE capability is another possible solution. So we provide following proposal to check with the whole group. Proposal: For the issue raised in R1-2309048, the second priority for PUSCH selection: earliest PUSCH slot(s) based on the start of the slot(s) from the conclusion in RAN #97 is capured in TS 38.213 as a UE capability, the following draft CR is endorsed since Rel-16. - UE procedure for reporting control information < Unchanged parts are omitted > < Unchanged parts are omitted > The UE determines the PUSCH for UCI multiplexing by applying the following procedure on the candidate PUSCHs as described in this clause: - If a UE multiplexes aperiodic CSI in a candidate PUSCH and the UE would multiplex UCI that includes HARQ-ACK information in a PUCCH that overlaps with the candidate PUSCH and the timing conditions for overlapping PUCCHs and candidate PUSCHs in clause 9.2.5 are fulfilled, the UE multiplexes only the HARQ-ACK information in the candidate PUSCH and does not transmit the PUCCH. - If the candidate PUSCHs are in different slots, and the UE indicates SUPPORT for a capability to determine PUSCH from multiple based on the timing of slots, and candidate PUSCHs fulfil the conditions in clause 9.2.5 for UCI multiplexing, the UE multiplexes UCI in one of candidate PUSCH in the earliest PUSCH slot(s) based on the start of the slot(s). - If the candidate PUSCHs include first PUSCHs that are scheduled by DCI formats and second PUSCHs configured by respective ConfiguredGrantConfig or semiPersistentOnPUSCH, and the UE would multiplex UCI in one of the candidate PUSCHs, and the candidate PUSCHs fulfil the conditions in clause 9.2.5 for UCI multiplexing, the UE multiplexes the UCI in a PUSCH from the first PUSCHs. < Unchanged parts are omitted >. |
| vivo |  | We are fine with the proposed conclusion. But it seems some company have strong concern on it. We think Huawei proposes a good way, may be the only way forward. We are fine to have a CR with UE capability as proposed above by Huawei. Company are welcome to share their views on Huawei proposal. |
| MTK |  | Fine with the proposed conclusion. Open to discuss the UE capability approach Huawei proposed. |

# Summary of the discussion

Based on the feedback from companies, it is observed that

For the issue raised in R1-2309048, for PUSCH selection rule for UCI multiplexing, this is in a special but important case that we have found out there are different implemtations in the field. Some companies have implemented according to the conclusion in RAN1#97 (with the Second priority in the coclusion in RAN1#97), while majority companies have implemented according to the spec (without the Second priority in the coclusion in RAN1#97).

It is also observed that

Company agree that normally, both spec and conclusion should be respected, but implementation in the field is also important to take into account for the issue.

Possible proposal:

For the issue raised in R1-2309048, the second priority for PUSCH selection: earliest PUSCH slot(s) based on the start of the slot(s) from the conclusion in RAN #97 is capured in TS 38.213 as a UE capability, a corresponding CR can be endorsed since [Rel-16].

# Reference

- R1-2309048	Draft Rel-16 CR on PUSCH selection rule for UCI multiplexing	vivo

- R1-2309747	Discussion on PUSCH selection rule for UCI multiplexing	Huawei, HiSilicon

- R1-2310254	Discussion on PUSCH selection rule for UCI multiplexing	Ericsson

- R1-2309049	Draft Rel-17 CR on PUSCH selection rule for UCI multiplexing	vivo