3GPP TSG-RAN WG1 Meeting #118	R1-2405845

Maastricht, Netherlands, August 19 - 23, 2024

Agenda Item:	8.1

Source:	Huawei, HiSilicon

Title:	Discussions on remaining issues of R18 NR sidelink from RAN1#117

Document for:	Discussion and Decision

- Introduction

In this contribution, we provide our views on remaining issues of R18 Sidelink from RAN1#117.

For the issues in sidelink unlicensed channel access, we provide our analysis on:

- COT sharing flag

- Inter-UE blocking issue

- CAPC value for both S-SSB and PSFCH transmission

For the issues in sidelink unlicensed PHY channel design, we provide our analysis on:

- S-SSB transmission in Non-Anchor RB set

- Discussions

Channel access mechanism

    - COT sharing flag

In RAN1#117 [1], COT sharing flag had been discussed but no agreement had been reached. Some companies propose that the 'COT sharing flag' is not explicitly defined in TS 37.213, it will be confusing to say “1 bit as defined in [14, TS 37.213]”, and give the proposal as follow,

< Start of text proposal for TS 38.212 > #### 8.3.1.1 SCI format 1-A SCI format 1-A is used for the scheduling of PSSCH and 2nd-stage-SCI on PSSCH The following information is transmitted by means of the SCI format 1-A: < Unchanged parts are omitted > - COT sharing flag – 0 or 1 bit - 1 bit if the higher layer parameter transmissionStructureForPSCCHandPSSCH in SL-BWP-Config is configured; - 0 bit otherwise. < Unchanged parts are omitted >

In TS 37.213, there is the definition of COT sharing information, and the parameter of  “COT sharing flag” is designed to indicate the function of COT sharing, and which is corresponding to COT sharing information, thus we prefer to keep “ 1 bit as defined in [14, TS 37.213]”, which can help to understanding the purpose for this parameter field.

Proposal 1: Keep the explanation of the COT sharing flag “1 bit as defined in [14, TS 37.213]”  in TS 38.212.

    - Inter-UE blocking issue

In RAN1#117 FLS [1], some companies think the definition of N consecutive resource(s) and M consecutive resource(s) is unclear for MCSt case. For different understandings, physical layer will perform different ways of resource exclusion, for example, as explained in FLS, if N =2 and slot MCSt = 2, whether N = 2 means “resources in 4 slots” or “resources in 2 slots” is unclear.

Thus, in order to avoid confusion, an LS is needed to inform RAN2 of this clarification.

Proposal 2: Send an LS to inform RAN2 that N consecutive resource(s) and M consecutive resource(s) in Option 1 for inter-UE blocking refer to those in case of single-slot resource.

    - CAPC value for both S-SSB and PSFCH transmission

In RAN1#117 FLS [1], some companies think CAPC value defined in current specification only consider the case ‘PSFCH-only’ and ‘S-SSB-only’, not for ‘PSFCH’ and ‘S-SSB’, and this means that e.g., when SL burst only include PSFCH and SSB, there is no associated CAPC value.

However, as our understanding, for the case PSFCH and S-SSB are transmitted adjacently, current spec is clear for how to address the channel combination case, i.e., “UE can perform Type1 channel access procedure separately corresponding to each channel”, and UE can perform MCSt or SL burst transmission if the gap condition is satisfied.

Proposal 3: No need to define additional CAPC value for “PSFCH+SSB” transmission.

Physical channel design framework

    - S-SSB Transmission in Non-Anchor RB Set

An issue [2] regarding S-SSB transmission in non-anchor RB set was raised in RAN1#117. Some companies propose that UE may attempt to transmit S-SSB on non-anchor RB set when anchor RB set is outside the resource pool.

We noted in existing specification, it’s stated that a UE attempts to transmit at least S-SS/PSBCH blocks in the slots including S-SS/PSBCH blocks in the anchor RB set regardless the anchor RB set is within its resource pool or not, and UE behavior for transmitting S-SSB on non-anchor RB set is not precluded. Therefore, the issue is already addressed from existing specification and no specification impact is needed.

Copied from TS 38.213 16.1 Synchronization procedures < Unchanged parts are omitted > For operation with shared spectrum channel access, a UE attempts to transmit at least S-SS/PSBCH blocks in the slots including S-SS/PSBCH blocks in the anchor RB set. The UE applies CP extension to the first symbol of an S-SS/PSBCH block and within the first one or two symbols before the first symbol of the S-SS/PSBCH block according to an index [4, TS 38.211] provided by sl-CPE-StartingPositionS-SSB. The UE assumes PRB(s) in an intra-cell guard band [6, TS 38.214] are not used for transmission of S-SS/PSBCH blocks. < Unchanged parts are omitted > For operation with shared spectrum channel access and for each slot that includes S-SS/PSBCH blocks, a UE is provided, by sl-NumOfAdditionalSSSBOccasion, a number NadditionalS-SSB of additional candidate S-SS/PSBCH block transmission occasions. When the UE determines to transmit S-SS/PSBCH blocks on additional candidate S-SS/PSBCH block transmission occasions, the UE attempts to transmit S-SS/PSBCH blocks at least in the anchor RB set. When NadditionalS-SSB>0, for S-SS/PSBCH block with index iS-SSB, the UE determines indexes of slots that include the additional candidate S-SS/PSBCH block transmission occasions as NoffsetS-SSB+NintervalS-SSB+1⋅iS-SSB +( NgapS-SSB+1)⋅(iS-SSB+1), where - NgapS-SSB is a slot gap, provided by sl-GapOfAdditionalSSSB-Occasion, for determining the additional candidate S-SS/PSBCH block transmission occasions, and - iS-SSB is an index of the additional candidate S-SS/PSBCH block transmission occasions, with 0≤iS-SSB≤NadditionalS-SSB-1. < Unchanged parts are omitted >

Proposal 4: Regarding the issue on S-SSB transmission in non-anchor RB Set, no specification change is needed.

- Conclusions

This contribution gives views on maintenance for sidelink operation in Rel-18 specifications. The following proposals are made:

Proposal 1: Keep the explanation of the COT sharing flag “1 bit as defined in [14, TS 37.213]”  in TS 38.212.

Proposal 2: Send an LS to inform RAN2 that N consecutive resource(s) and M consecutive resource(s) in Option 1 for inter-UE blocking refer to those in case of single-slot resource.

Proposal 3: No need to define additional CAPC value for “PSFCH+SSB” transmission.

Proposal 4: Regarding the issue on S-SSB transmission in non-anchor RB Set, no specification change is needed.

- References

- R1-2405356, “FL summary for AI 8.1: SL-U channel access and RA (EOM)”, Moderator (OPPO), RAN#111, May 20 – 24, 2024.

- R1-2404086, “Draft CR for Correcting S-SSB Transmission in Non-Anchor RB Set”, Samsung, Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024