3GPP TSG RAN WG1 #118		R1-2406367

Maastricht, NL, August 19th – 23rd, 2024

Source:	CATT

Title:	Discussion on 	SBFD TX/RX/measurement procedures

Agenda Item:	9.3.1

Document for:	Discussion and Decision

Introduction

For Rel-19 NR duplex evolution, the followings were agreed in last meeting for SBFD TX/RX/measurement procedures [1].

Agreement For CSI report associated with periodic/semi-persistent CSI-RS, discuss and decide whether to support the following options. - Option A: For separate CSI reports on SBFD and non-SBFD, one CSI-ReportConfig is associated with CSI-RS(s) restricted to SBFD symbols only and the second CSI-ReportConfig is associated with CSI-RS(s) restricted to non-SBFD symbols only. - gNB configuration may not ensure that the CSI-RS associated with each CSI-ReportConfig is confined to either SBFD symbols or non-SBFD symbols only. - For the CSI-ReportConfig associated with CSI-RS(s) restricted to SBFD symbols only, only CSI-RS transmission occasions within SBFD symbols are used for CSI derivation. For the CSI-ReportConfig associated with CSI-RS(s) restricted to non-SBFD symbols only, only CSI-RS transmission occasions within non-SBFD symbols are used for CSI derivation. - Option B: Enhance Rel-18 NES CSI reporting framework to support one CSI-ReportConfig with one sub-configuration associated with SBFD symbols and the other sub-configuration associated with non-SBFD symbols Working Assumption For frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, support one contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL usable PRBs. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured) - RAN1 to further study the impact on CSI processing timeline in SBFD symbols to process the CSI-RS across the two DL subbands Agreement For a single TRP scenario, support separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols - Decide in RAN1#118 whether separate UL power control for FR2 is supported based on UL spatial relation Info framework and/or unified TCI state framework. Agreement For a contiguous CSI-RS resource which overlaps with SBFD subband boundaries, only CSI-RS frequency resources within DL usable PRBs are valid for SBFD-aware. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured) Agreement For a CSI reporting subband which overlaps with at least one PRB within DL usable PRBs and one or more PRBs outside DL usable PRBs, the CSI reporting subband includes PRB(s) within DL usable PRBs only. Agreement If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 4 (dynamically scheduled DL reception vs. dynamically scheduled UL transmission) in a SBFD symbol for SBFD-aware UEs, it is considered as an error case if a dynamically scheduled DL reception in DL subband(s) without repetitions overlaps with a dynamically scheduled UL transmission in UL subband without repetitions. - FFS DL reception with repetitions and/or UL transmission with repetitions Agreement If link direction indication is not supported nor provided for a SBFD symbol, collision Case 3 (semi-statically configured DL reception vs. semi-statically configured UL transmission) in the SBFD symbol for SBFD-aware UEs is an error case. - An SBFD-aware UE does not expect to receive both dedicated higher layer parameters configuring transmission in UL usable PRBs from the UE and dedicated higher layer parameters configuring reception in DL usable PRBs in the SBFD symbol - An SBFD-aware UE does not expect to receive both dedicated higher layer parameters configuring transmission in UL usable PRBs from the UE and cell specific higher layer parameters configuring reception in DL usable PRBs in the SBFD symbol - Cell-specifically configured DL reception refers to PDCCH in Type-0/0A/1/2 CSS set - FFS: Additional cases Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with one of the configurations. - Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only - Configuration 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols - FFS granularity of the configuration, e.g. per UE, per channel/signal etc. - FFS whether support of configuration 2 is subject to UE capability Agreement For RRC connected mode UEs, SBFD subband time period: - When only one TDD-UL-DL pattern is configured, the period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon. - When two TDD-UL-DL patterns are configured, the period is the same as the sum of the two TDD-UL-DL pattern periods configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon. Agreement For cell-specific indication of SBFD subband frequency location, adopt the following option. - Option 1: Frequency locations of UL subband and DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s). Agreement For frequency domain resource allocation Type 1 for PDSCH in a single slot scheduled at least by DCI format in USS when PRG is determined as one of the values among {2, 4}, - Option 1-1: Only the assigned PRBs within DL usable PRBs are considered to be valid for PDSCH. Assigned PRBs that fall outside DL usable PRBs are considered to be invalid and should not be used for PDSCH resource mapping. - Existing RB indexing and VRB-to-PRB mapping are reused - The number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only - FFS: DMRS sequence mapping - FFS: when PRG is determined as wideband - FFS: partial PRG Agreement Support separate FH offsets for PUSCH transmissions in SBFD symbols and non-SBFD symbols respectively. - FFS: How to indicate/determine the FH offsets for PUSCH transmissions in SBFD and non-SBFD symbols, respectively. - FFS: Whether/how to update FH calculation to only consider the UL usable PRBs Agreement For frequency resource allocation Type 0 for PDSCH or PUSCH in a single slot by DCI based scheduling (without repetition or TBoMS), when an assigned RBG overlaps with the subband boundary, the number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only and assigned PRBs within UL usable PRBs only for PDSCH and PUSCH respectively. Agreement Re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission - FFS whether a slot consisting of SSB symbols is considered as a full DL slot or SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs.

In this contribution, further discussions on SBFD TX/RX/measurement procedures in Rel-19 NR duplex evolution are given.

Discussion

Semi-static indication of SBFD subbands locations

Indication of SBFD subband time location

It was agreed that SBFD symbols are configured in consecutive manner within a TDD-UL-DL pattern period. For detailed indication of SBFD symbols within a TDD-UL-DL pattern period, the following options can be considered.

- Option 1: Separate indication of starting SBFD symbol and ending SBFD symbol

- Option 2: Joint indication of starting SBFD symbol and number of SBFD symbols

- Option 3: Separate indications of number of slots/symbols of non-SBFD symbols at the beginning and at the end of a TDD-UL-DL pattern period

For option 1, the starting/ending SBFD symbol can be indicated by starting/ending symbol index within TDD-UL-DL pattern period. Considering the maximum 320 slots within a TDD-UL-DL pattern period, the required number of bits for symbol index indication within TDD-UL-DL pattern period is log2320×14=13 leading to total 26 bits. Alternatively, the starting/ending SBFD symbol can be indicated by starting/ending slot index and starting/ending symbol index within the starting/ending slot. In this case, log2320=9 bits are required to indicate slot index and log214=4 bits are required to indicate symbol index within a slot. The total required bits are also 26 bits.

For option 2, starting SBFD symbol and number of SBFD symbols are jointly indicated by RIV within TDD-UL-DL pattern period. Given that there are maximum 320×14=4480 symbols, the number of required bits is log24480×(4480+1)2=24 bits.

Option 3 follows the design of cell specific UL/DL TDD configuration. From signaling overhead point of view, log2320=9 bits are required to indicate the number of slots and log214=4 bits are required to indicate the number of symbols. Thus the total required bits are 26 bits.

Based on the above analysis, all the three options are viable options and the signaling overhead is similar.

Proposal 1: For detailed indication of SBFD symbols within a TDD-UL-DL pattern period, down-select from the following options:

- Option 1: Separate indication of starting SBFD symbol and ending SBFD symbol;

- Option 2: Joint indication of starting SBFD symbol and number of SBFD symbols;

- Option 3: Separate indications of number of slots/symbols of non-SBFD symbols at the beginning and at the end of a TDD-UL-DL pattern period.

Based on RAN4’s agreement in [2] [3], a transition period is needed between the non-SBFD slot and SBFD slot and vice versa, and the transition period requirement is FFS in RAN4. From RAN1 point of view, two options can be considered on whether transition period is indicated to SBFD aware UEs.

- Option 1: Transition period is transparent to UE

- Option 2: Transition period is predefined or explicitly indicated to UE

For option 1, it is up to gNB configuration/scheduling to make sure that there is no DL/UL transmissions during transition period. However, for SPS PDSCH, CG PUSCH and repetitions, it would impose additional gNB configuration/scheduling restrictions.

For option 2, transition period is assumed between SBFD and non-SBFD symbols at UE side based on predefined rules or explicit indication from gNB. For example, if duration of transition period is defined in the spec, SBFD aware UEs can always assume transition period between SBFD and non-SBFD symbols according to predefined duration. Or gNB can explicitly indicate the presence and/or the duration of transition period to SBFD aware UEs. SBFD aware UEs would assume no transmissions and receptions during transition period which helps to increase gNB’s configuration/scheduling flexibility. Thus, Option 2 is preferred.

Proposal 2: Support to predefine or explicitly indicate transition period between SBFD and non-SBFD symbols.

Indication of SBFD subband frequency location

Based on the agreement achieved in RAN1#116bis meeting, reference starting PRB for cell-specific configuration of frequency locations of SBFD subbands is the starting PRB of the carrier with the same SCS as subband. Similar to BWP indication, RIV can be used for cell-specific UL/DL subband frequency indication. The RIV is interpreted as definition in existing TS38.214 with the assumption of maximum UL/DL subband bandwidth of 275 PRBs.

Proposal 3: Support UL/DL subband frequency indication via RIV with the assumption of maximum UL/DL subband bandwidth of 275 PRBs.

Enhancements to frequency resource allocation in SBFD symbols

For frequency domain resource allocation Type 1 for PDSCH in a single slot scheduled at least by DCI format in USS when PRG is determined as one of the values among {2, 4}, Option 1-1 was agreed in last meeting while the DMRS sequence mapping was FFS.

- Option 1-1: Only the assigned PRBs within DL usable PRBs are considered to be valid for PDSCH. Assigned PRBs that fall outside DL usable PRBs are considered to be invalid and should not be used for PDSCH resource mapping.

  - Existing RB indexing and VRB-to-PRB mapping are reused

  - The number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only

  - FFS: DMRS sequence mapping

In existing specification, DMRS sequence is mapped to REs within CRBs allocated for PDSCH transmission. For Option 1-1, DMRS sequence should be mapped to REs within allocated CRBs within DL usable PRBs.

Proposal 4: DMRS sequence is mapped to REs within allocated CRBs within DL usable PRBs for Option 1-1.

The issue of wideband precoder in case of non-contiguous DL subbands was identified and studied in SI. The following two options were studied.

- Option 1: non-contiguous frequency resources across two DL subbands but contiguous frequency resource within each DL subband can be allocated

- Option 2: non-contiguous frequency resources across two DL subbands cannot be allocated

Based on the study, RAN1 agreed that Option 1 can achieve better scheduling flexibility and higher DL data rate. Compared with Option 2, Option 1 requires UE to handle two non-contiguous segments of contiguous RBs that may increase UE complexity for channel estimation. From perspectives of scheduling flexibility and DL data rate, Option 1 is preferred.

Proposal 5: For wideband precoder, non-contiguous frequency resources across two DL subbands but contiguous frequency resource within each DL subband is preferred.

The issue of PRG(s) overlapping with subband boundary was studied in SI. Based on the study, RAN1 agreed that, if the part of DL PRG inside the DL subband can be used, better scheduling flexibility and resource utilization can be achieved, however degraded channel estimation quality in the partial PRG is expected compared to a PRG due to limited RBs in the partial PRG.  Besides, UE complexity could increase if this feature is supported. Considering that partial RBG in case of unaligned boundaries between RBG and SBFD subbands is supported as agreed in SI, when RBG size is same as PRG size, not supporting transmission in partial PRG means partial RBG is not supported, which contradicts the existing agreement. From perspectives of scheduling flexibility and resource utilization, we prefer to utilize the part of DL PRG inside the DL usable PRBs. Furthermore, at most two partial PRG within a BWP is supported in current specification. In order to avoid introducing additional UE complexity, partial RBG can be supported in case the number of partial RBGs in SBFD symbols does not exceed two.

Proposal 6: For PRG overlapping with subband boundary, the part of DL PRG inside the DL usable PRBs can be used in case the number of partial RBGs in SBFD symbols does not exceed two.

Transmissions and receptions across SBFD symbols and non-SBFD symbols

For transmissions and receptions across SBFD symbols and non-SBFD symbols in different slots, it was agreed in last meeting that one of the following two configurations can be provided to UE:

- Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only

- Configuration 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols

Remaining issues for Configuration 1

For configuration 1, UE is only allowed to transmit or receive in one symbol type. It should be decided in which symbol type the transmissions/receptions are restricted to and what the UE behaviour is in the other symbol type.

The following options can be considered to determine the symbol type:

- Option 1: Based on symbol type of the 1st transmission/reception slot/symbol

- Option 2: Based on RRC configuration

For transmissions/receptions with scheduling or activation DCI, the symbol type can be determined based on symbol type of the 1st transmission/reception slot/symbol. gNB can dynamically determine the symbol type for each transmission/reception with scheduling or activation DCI.

For transmissions/receptions without scheduling or activation DCI, the symbol type can be determined based on RRC configuration. For example, for Type 1 CG PUSCH and P/SP PUCCH, new RRC parameters can be introduced to indicate the supported symbol type. For SRS, separate SRS resource sets can be configured for SBFD and non-SBFD symbols respectively. For P/SP CSI-RS, gNB configures CSI-RS to be confined to one symbol type only or gNB configures the symbol type for the P/SP CSI-RS by RRC.

After the symbol type used for transmissions/receptions has been determined, UE behaviour in the other invalid symbol type should be defined. For PUSCH repetition with available slot counting, TBoMS and PUCCH repetition, UE could postpone transmissions in the other symbol type. For the other transmissions/receptions except for SPS HARQ-ACK with defer, transmissions/receptions in the other symbol type are invalid. For SPS HARQ-ACK with defer, UE behavior in the other symbol type can be further studied.

An example is shown in Figure 1, for a PUSCH repetition type A with available slot counting indicated by DCI, the first transmission slot is an SBFD slot, hence the PUSCH repetition should be restricted to SBFD slots. In non-SBFD slot, PUSCH Rep#3 is postponed to the next available SBFD slot.

Figure 1: PUSCH repetition restricted in SBFD slots

Another example is shown in Figure 2. For a CG PUSCH, the transmissions are restricted to SBFD slots according to RRC configuration. Consequently, the CG PUSCH transmission occasions in non-SBFD slots are invalid.

Figure 2: CG PUSCH restricted in SBFD slots

Proposal 7: For configuration 1, the valid symbol type is defined as below:

- For transmissions/receptions with scheduling or activation DCI, the symbol type is determined based on symbol type of the 1st transmission/reception slot/symbol;

- For transmissions/receptions w/o scheduling or activation DCI, the symbol type is determined based on RRC configuration.

Proposal 8: For configuration 1, UE behavior for transmissions/receptions in the invalid symbol type can be defined as below:

- For PUSCH repetition with available slot counting, TBoMS and PUCCH repetition, UE could postpone transmissions in the invalid symbol type;

- For the other transmissions/receptions except for SPS HARQ-ACK with defer, transmissions/receptions are invalid in the invalid symbol type;

- FFS SPS HARQ-ACK with defer in the invalid symbol type.

Remaining issues for Configuration 2

For configuration 2, both SBFD symbols and non-SBFD symbols can be used for transmissions/receptions. If the frequency resources are all within usable PRBs, then the transmissions and receptions can be performed across SBFD symbols and non-SBFD symbols in different slots. An example is shown in Figure 3(a). The UE behaviour when the frequency resources of transmissions/receptions overlap with RBs outside UL/DL usable PRBs in SBFD symbols should be decided.

For PUSCH repetition Type A with available slot counting and TBoMS, a PUSCH in a slot overlapping with RBs outside UL usable PRBs in SBFD symbols can be postponed. An example is shown in Figure 3(b), where PUSCH Rep#2 with frequency resource overlapping with RBs outside UL usable PRBs is postponed to the next available non-SBFD slot.

Figure 3: PUSCH repetition with available slot counting in configuration 2

For PDSCH receptions and other PUSCH transmissions (CG PUSCH, PUSCH repetition type A w/o available slot counting and multi-PUSCH scheduling), UE behavior can be down-selected from the following options:

- Option 1: A PDSCH reception overlapping with RBs outside DL usable PRBs/A PUSCH transmission overlapping with RBs outside UL usable PRBs in SBFD symbols is invalid

- Option 2: Only the assigned PRBs within DL/UL usable PRBs are valid for PDSCH/PUSCH in SBFD symbols

For option 1, it is simple and straightforward. An SPS PDSCH reception occasion/a CG PUSCH transmission occasion overlapping with RBs outside DL/UL usable PRBs is invalid. For PDSCH repetitions, PUSCH repetitions w/o available slot counting, and multi-PDSCH/PUSCH scheduling, a PDSCH reception occasion/a PUSCH transmission occasion overlapping with RBs outside DL/UL usable PRBs is dropped.

For option 2, enhancements are considered to utilize the frequency resource in usable PRBs in SBFD symbols when partial frequency resources overlapping with RBs outside UL usable PRBs. But different from single slot transmission/reception, TBS for SPS PDSCH, PDSCH repetitions, CG PUSCH and PUSCH repetition should be determined based on assigned PRBs. For multi-PDSCH/PUSCH scheduled by a single DCI, TBS in each slot can be independently determined based on available PRBs.

As shown in Figure 4, for a CG PUSCH overlapping with RBs outside UL usable PRBs in SBFD symbols, CG transmission occasions in SBFD slots are invalid with option 1. But with option 2, CG transmission occasions in SBFD slots are considered as valid, and only the assigned PRBs within UL usable PRBs are used for CG PUSCH transmissions.

Figure 4: CG PUSCH in configuration 2

In addition, whether configuration 2 is subject to UE capability can be further discussed. Considering that supporting configuration 2 would introduce more complexity, we support that configuration 2 is defined as an optional UE capability for SBFD aware UEs.

Proposal 9: For configuration 2, if the frequency resources of transmissions/receptions overlap with RBs outside UL/DL usable PRBs in SBFD symbols,

- For PUSCH repetition Type A with available slot counting and TBoMS, a PUSCH in a slot overlapping with RBs outside UL usable PRBs in SBFD symbols can be postponed;

- For PDSCH receptions and other PUSCH transmissions, down select from the following two options:

  - Option 1: A PDSCH reception overlapping with RBs outside DL usable PRBs/A PUSCH transmission overlapping with RBs outside UL usable PRBs in SBFD symbols is invalid;

  - Option 2: Only the assigned PRBs within DL/UL usable PRBs are valid for PDSCH/PUSCH in SBFD symbols.

Proposal 10: For transmissions and receptions across SBFD symbols and non-SBFD symbols in different slots, define configuration 2 as an optional UE capability for SBFD aware UEs.

CSI report with CSI-RS instances in SBFD and non-SBFD symbols in different slots

For CSI report associated with periodic/semi-persistent CSI-RS across SBFD symbols and non-SBFD symbols in different slots, the following options for SBFD-aware UEs were agreed for discussion in last meeting:

- Option A: For separate CSI reports on SBFD and non-SBFD, one CSI-ReportConfig is associated with CSI-RS(s) restricted to SBFD symbols only and the second CSI-ReportConfig is associated with CSI-RS(s) restricted to non-SBFD symbols only.

  - gNB configuration may not ensure that the CSI-RS associated with each CSI-ReportConfig is confined to either SBFD symbols or non-SBFD symbols only.

    - For the CSI-ReportConfig associated with CSI-RS(s) restricted to SBFD symbols only, only CSI-RS transmission occasions within SBFD symbols are used for CSI derivation. For the CSI-ReportConfig associated with CSI-RS(s) restricted to non-SBFD symbols only, only CSI-RS transmission occasions within non-SBFD symbols are used for CSI derivation.

- Option B: Enhance Rel-18 NES CSI reporting framework to support one CSI-ReportConfig with one sub-configuration associated with SBFD symbols and the other sub-configuration associated with non-SBFD symbols

For option A, new RRC signalling can be added in CSI-ReportConfig to indicate whether it is applicable to SBFD symbols or for non-SBFD symbols. Accordingly, only CSI-RS transmission occasions within the applicable symbol type are used for CSI derivation.

For option B, following Rel-18 NES CSI reporting framework, two sub-configurations can be configured in one CSI-ReportConfig, where each sub-configuration can be configured with a CSI-RS resource for channel measurement. The symbol type corresponding to each sub-configuration can be predefined, e.g. the first sub-configuration is for non-SBFD symbols and the second sub-configuration is for SBFD symbols. For interference measurement, considering that interference level can be different in SBFD symbols and non-SBFD symbols, separate CSI-IM and NZP CSI-RS for different symbol types should be supported. Separate CSI-IM resources for interference measurements can be supported based on existing resource-wise association between CSI-RS resources for channel measurement and CSI-IM resources for interference measurements. However, only one NZP CSI-RS resource can be configured in the resource set for interference measurement as in current specification, which limits the configurability of CSI reports when the gNB needs interference measurements based on NZP CSI-RS for different symbol types. Enhancements can be considered to support two NZP CSI-RS resources contained in the resource set for interference measurement for SBFD UE.

Observation 1: If Rel-18 NES CSI reporting framework is reused for CSI reporting for different symbols types, only one NZP CSI-RS resource can be configured in the resource set for interference measurement.

Proposal 11: For CSI report associated with CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots, support Option A and Option B.

- For option A, new RRC signalling can be added in CSI-ReportConfig to indicate whether only CSI-RS transmission occasions in SBFD symbols or in non-SBFD symbols are used for CSI derivation;

- For Option B, the following enhancements can be supported:

  - The first sub-configuration is for non-SBFD symbols and the second sub-configuration is for SBFD symbols;

  - Support two NZP CSI-RS resources in the resource set for interference measurement in one CSI-ReportConfig.

Separate configurations for SBFD and non-SBFD symbols

In this section, separate resources, FH parameters, UL power control parameters and/or beam/spatial relation for SRS, PUCCH and PUSCH are discussed.

Separate UL power control parameters and/or spatial relation

It was agreed that separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols are supported for single TRP scenario. A remaining issue is that whether separate UL power control for FR2 is supported based on UL spatial relation info framework and/or unified TCI state framework.

In Rel-17, unified TCI state framework was introduced, which uses a unified mechanism to determine the beam and uplink power for all channels and simplifies the original UL spatial relation info framework. It is the most likely way to be adopted in actual deployment, and is also the design direction of 6G in the future. Hence, we prefer to support separate UL power control for FR2 based on unified TCI state framework.

Proposal 12: For a single TRP scenario, support separate UL power control for FR2 based on unified TCI state framework.

Based on unified TCI state framework, there are two possible ways to determine separate UL power in SBFD symbols and non-SBFD symbols:

- Option 1: One TCI codepoint is mapped to two TCI states for SBFD symbols and non-SBFD symbols respectively

- Option 2: Same unified TCI state associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols

For option 1, two TCI states for one TCI codepoint are activated by MAC CE. As shown in Figure 5, one TCI codepoint is mapped to two TCI states for SBFD symbols and non-SBFD symbols respectively. DCI indicates one of the TCI codepoints to determine two unified TCI states for SBFD symbols and non-SBFD symbols. UE determines unified TCI state including UL power control parameters based on symbol type of the UL transmission.

Figure 5: Separate TCI states for non-SBFD symbols and non-SBFD symbols

Option 2 is illustrated in Figure 6, where two sets of UL power control parameters for SBFD symbols and non-SBFD symbols are configured for one unified TCI state. UL power control parameters can be determined based on symbol type of the UL transmission.

Figure 6: Separate UL power control parameters for SBFD symbols and non-SBFD symbols

Option 1 supports separate unified TCI states including beam related parameters and UL power control parameters for SBFD and non-SBFD symbols. Option 2 means same beam but separate UL power control parameters for SBFD and non-SBFD symbols. Both options can be further studied to be down selected.

Proposal 13: For separate PUCCH power control parameters for SBFD and non-SBFD symbols, the following options can be further studied:

- Option 1: One TCI codepoint is mapped to two TCI states for SBFD symbols and non-SBFD symbols respectively. Unified TCI states including UL power control parameters are determined based on symbol type of the UL transmission.

- Option 2: Same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols. UL power control parameters are determined based on symbol type of the UL transmission.

For mTRP scenario, separate SRS resource sets are configured corresponding to different TRPs, and power control parameters and beam/spatial relations are configured separately for each SRS resource set. The same mechanism can be applied to SRS transmissions in SBFD symbols and non-SBFD symbols in different slots, where two SRS resource sets can be configured for SBFD symbols and non-SBFD symbols respectively.

Proposal 14: Support separate SRS resource sets for SBFD symbols and non-SBFD symbols.

Separate PUCCH resources

According to current specification, a single PUCCH configuration is provided per PHY priority which is applicable to all slots.

Following the existing design, PUCCH configuration needs to consider different available resources for PUCCH transmissions in SBFD slot and full UL slot, which would degrade the PUCCH resource allocation flexibility.

Figure 7: RRC parameter structure in PUCCH-Config

The current RRC parameter structure for PUCCH-Config per PHY priority is defined as in Figure 7. At most four PUCCH resources sets can be included in PUCCH-Config, and multiple PUCCH resources can be configured for each PUCCH resource set. For each PUCCH resource, parameters including startingPRB and secondHopPRB are configured. Hence, the following options can be considered for PUCCH resource configuration enhancement:

    - Option 1: Support separate configurations of PUCCH-Config for SBFD symbols and non-SBFD symbols.

    - Option 2: Under PUCCH-Config, support separate PUCCH resource sets for SBFD symbols and non-SBFD symbols.

    - Option 3: For a certain PUCCH resource set, support separate PUCCH resources for SBFD symbols and non-SBFD symbols.

    - Option 4: For a certain PUCCH resource, support separate configurations of parameters including at least startingPRB and secondHopPRB.

    - Option 5: No separate configurations of PUCCH resource sets for SBFD symbols and non-SBFD symbols. PUCCH resource in SBFD symbols are derived based on RB offset(s) configuration/indication/determination.

Option 1 supports to separately configure all PUCCH parameters for SBFD symbols and non-SBFD symbols with the largest signaling overhead. However, there is no need to configure all PUCCH parameters separately for SBFD symbols and non-SBFD symbols except for frequency resources. In addition, PUCCH repetition across SBFD symbols and non-SBFD symbols cannot be supported with option 1 when different PUCCH formats or different UCI coding rates are configured for different symbol types, since UE may not be able to combine PUCCH repetitions in different symbol types with different PUCCH formats or different UCI coding rates.

Option 2 and option 3 require less signaling overhead compared with option 1, but they still cannot support PUCCH repetition across SBFD symbols and non-SBFD symbols when different PUCCH formats or different UCI coding rates are configured for different symbol types.

Option 4 supports separate PUCCH frequency resources for SBFD symbol and non-SBFD symbol and requires least signaling overhead among options 1-4. PUCCH repetition across SBFD symbols and non-SBFD symbols can be supported with option 4 by only applying different frequency resources in different symbol types.

For option 5, indication or additional rules should be defined to determine the RB offset(s). It does not bring more benefit compared with option 4 if RRC configure the RB offset(s). Otherwise if DCI is used for indicating the RB offset(s), it is more flexible but DCI overhead is increased. If RB offset(s) are determined based on additional rules, discussion are needed on in which case and how the RB offset(s) should be determined, and how gNB could ensure that all UEs determine the proper PUCCH resource in UL subband.

Comparing the above options, option 4 is straightforward and has less signaling overhead, hence it is preferred to adopt option 4.

Proposal 15: For PUCCH resource configuration for SBFD aware UEs, support separate configurations of parameters including at least startingPRB and secondHopPRB for a certain PUCCH resource.

Separate FH parameters

The available UL frequency resources are different in SBFD symbols and non-SBFD symbols. For PUSCH, separate FH offsets in SBFD symbols and non-SBFD symbols were agreed in last meeting. In current specification, one FH offset is configured for frequency hopping of Type-1 CG PUSCH. Two or four FH offsets are configured for frequency hopping of Type-2 CG PUSCH or dynamic PUSCH and DCI indicates one of the FH offsets used for the PUSCH. For SBFD aware UEs, additional one FH offset for Type-1 CG PUSCH and additional two or four FH offsets for Type-2 CG PUSCH or dynamic PUSCH can be configured to determine the hopping offset in SBFD symbols. If the additional FH offsets for PUSCH transmissions in SBFD symbols are not configured, then it can be considered that frequency hopping in SBFD symbols is not enabled. For an SBFD aware UE, the number of bits used to indicate FH offset in DCI can be determined according to existing rule, e.g. based on the size of the active UL BWP, and the number of FH offsets for SBFD symbols should not exceed the number of FH offsets for non-SBFD symbols.

For PUSCH frequency hopping, the following formula is used to determine the starting RB in each hop. It can be seen that for the second hop, modulo based on UL BWP is applied to avoid the case that the second hop is out of UL BWP bandwidth. For PUSCH in SBFD symbols, the formula should be updated by replacing UL BWP by UL usable PRBs.

Proposal 16: Support separate configurations of FH offsets for PUSCH transmissions in SBFD symbols and non-SBFD symbols,

- In case FH offsets for PUSCH transmissions in SBFD symbols are not configured, FH for PUSCH transmissions in SBFD symbols is disabled;

- The number of FH offsets for SBFD symbols does not exceed the number of FH offsets for non-SBFD symbols;

- The number of bits used to indicate FH offset in DCI is determined according to existing rule.

Proposal 17: For PUSCH in SBFD symbols, update the PUSCH frequency hopping formula by replacing UL BWP by UL usable PRBs.

Collision handling in SBFD symbols

For SBFD operation, half-duplex operation is assumed at UE side. How to resolve the collision between transmissions and receptions, and order of different collision types are discussed in this section.

Collision between transmissions and receptions

The following two schemes were agreed to be considered to resolve the issue of time domain conflict of transmission and reception in the same SBFD symbol for SBFD aware UE:

- Option 1: UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any).

- Option 2: link direction is indicated by gNB explicitly.

Option 1 follows the design principles of legacy NR UEs in flexible symbols and it is proposed to be supported.

For option 2, the transmission direction can be indicated to UE by RRC or DCI, reusing TDD-UL-DL-ConfigDedicated or dynamic SFI. The indicated link directions can be different for different SBFD-aware UEs. The potential benefits of Option 2 are UE complexity reduction and/or UE power consumption reduction. From specification point of view, both options can be supported.

Proposal 18: For SBFD aware UEs link direction determination, support Option 1 and Option 2 if the benefits of Option 2 are agreed.

- Option 1: UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any).

- Option 2: link direction is indicated by gNB explicitly.

For option 1, six collision cases were agreed for discussion. Collision handling rules should be defined for each collision case.

- Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission

  - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH

- Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission

  - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH

- Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission

- Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission

- Case 5: SSB vs. dynamically scheduled or configured UL transmission

  - e.g., PUSCH, PUCCH, PRACH, SRS

- Case 6: Dynamic or semi-static DL vs. valid RO

It was agreed to reuse the existing collision handling principles in NR for operation on flexible symbols on a single carrier in unpaired spectrum for case 1 to case 4.

For case 3, it was agreed that an SBFD-aware UE does not expect to receive both dedicated higher layer parameters configuring transmission in UL usable PRBs from the UE and dedicated higher layer parameters configuring reception in DL usable PRBs in the SBFD symbol; and an SBFD-aware UE does not expect to receive both dedicated higher layer parameters configuring transmission in UL usable PRBs from the UE and cell specific higher layer parameters configuring reception in DL usable PRBs in the SBFD symbol. For cell-specifically configured DL reception, PDCCH in Type-0B CSS set should also be included in addition to PDCCH in Type-0/0A/1/2 CSS set.

Proposal 19: If link direction indication is not supported nor provided for a SBFD symbol, cell-specifically configured DL reception defined in collision Case 3 refers to PDCCH in Type-0/0A/0B/1/2 CSS set.

For case 1/2/4, whether additional handling for DL reception with repetitions and/or UL transmission with repetitions is needed is FFS. Considering that legacy NR does not have any special handling for the repetition case, we think there is no need to introduce any special handling for this case either for SBFD.

Proposal 20: If link direction indication is not supported nor provided for a SBFD symbol, no special handling for DL reception with repetitions and/or UL transmission with repetitions is introduced for collision Case 1/2/4.

For Case 5, it was agreed in RAN1#117 to re-use the existing collision handling principle for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission.

The remaining issue is whether/how UL subband in SSB symbols can be used. It is obvious that if only DL receptions within DL usable PRBs are allowed for SBFD aware UEs in SSB symbols configured with SBFD subbands, it would cause reduced resource utilization efficiency. The percentage of wasted resources/DL total resources is shown in Table 1. It can be observed that more than 5% DL resources may be wasted for short SSB periods of 5 and 10ms.

Table 1: Percentage of wasted resources/DL total resources

| SSB configuration/period | 5ms | 10ms | 20ms | 40ms | 80ms | 160ms |
| --- | --- | --- | --- | --- | --- | --- |
| Case A (L=4) | 5.71% | 2.86% | 1.43% | 0.71% | 0.48% | 0.36% |
| Case A (L=8) | 11.43% | 5.71% | 2.86% | 1.43% | 0.95% | 0.71% |
| Case B (L=4) | 2.86% | 1.43% | 0.71% | 0.36% | 0.24% | 0.18% |
| Case B (L=8) | 5.71% | 2.86% | 1.43% | 0.71% | 0.48% | 0.36% |
| Case C (L=4) | 2.86% | 1.43% | 0.71% | 0.36% | 0.24% | 0.18% |
| Case C (L=8) | 5.71% | 2.86% | 1.43% | 0.71% | 0.48% | 0.36% |
| Case D (L=64) | - | 5.71% | 2.86% | 1.43% | 0.95% | 0.71% |
| Case E (L=64) | - | 2.86% | 1.43% | 0.71% | 0.48% | 0.36% |
| Assumptions: 1. All SSB symbols are configured as SBFD symbols; 2. UL subband occupies 20% carrier bandwidth; 3. Frame structure is XXXXU. |  |  |  |  |  |  |

Observation 2: If only DL receptions within DL usable PRBs are allowed for SBFD aware UEs in SSB symbols, more than 5% DL resources may be wasted for short SSB periods of 5 and 10ms.

Alternatively, DL receptions within UL usable PRBs can be allowed, i.e. SSB symbols fallback to full DL symbols. However, the problem is that if only SSB symbols fallback to full DL symbols, there are too many transition points. Therefore, if DL receptions within DL usable PRBs are allowed, a longer duration including SSB symbols and other adjacent symbols should be considered for fallback, e.g. a slot consisting of SSB symbols.

Proposal 21: For SSB symbols configured with SBFD subbands, discuss and decide whether SSB symbols can fall back to full DL symbols. If supported, a longer duration including SSB symbols and other adjacent symbols should be considered for fallback.

As the SBFD symbols configured on SSB symbols cannot be used for UL transmission, the collision of UL transmission and SSB symbols should be considered. For single slot transmission scheduled by DCI, gNB can avoid to schedule UL transmissions to be overlapped with SSB symbols. But for configured transmissions and transmissions with repetitions, UE behavior needs to be defined. From simplification point of view, reusing existing collision handling schemes for UL transmission overlapping with SSB symbol(s) is preferred.

Proposal 22: For UL transmission overlapping with SBFD symbol(s) configured on SSB symbol(s), reuse existing collision handling schemes for UL transmission overlapping with SSB symbol(s).

For case 6, collision handling rule for half-duplex FDD RedCap UE can be reused. For example, for PDCCH order triggered PRACH, it can be considered as dynamically scheduled UL transmission. For higher layer triggered PRACH, whether prioritize DL reception or PRACH transmission can be up to UE implementation.

Proposal 23: For SBFD aware UEs, if UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any), the following collision handling rules for HD-FDD RedCap UE is reused:

- UE does not expect collision between PRACH triggered by PDCCH order and DL channel/signal with scheduling DCI in the same SBFD symbol;

- For collision between PRACH triggered by PDCCH order and PDCCH, PDSCH, CSI-RS or DL PRS configured by higher layers in the same SBFD symbol, UE does not receive the PDCCH, PDSCH, CSI-RS or DL PRS configured by higher layers;

- For collision between PRACH triggered by higher layer and DL channel/signal in the same SBFD symbol, whether to prioritize DL channel/signal or PRACH transmission is up to UE implementation.

Order of different collision types

In general, there are four types of collision for a SBFD aware UE as below, the processing order for these collision types should be considered:

1) Collision between transmissions/receptions and unusable time/frequency resources;

2) Collision between transmissions with same transmission direction;

3) Collision between transmissions and receptions;

4) Collision between transmissions/receptions and the link direction indicated by gNB (if supported);

Figure 8: Different collision types in SBFD symbol

Collision type 4 has not been agreed yet, hence we discuss the first three collision types. As shown in Figure 8, there are three kinds of collision in slot n. The collision between PUSCH and DL usable PRBs is the first kind of collision, the collision between HARQ-ACK and PUSCH is the second kind of collision, and the collision between HARQ-ACK and PDSCH-1 or between PUSCH and PDSCH-2 is the third kind of collision. It is necessary to determine the execution order of the solutions to resolve these collisions. Different execution order may lead to different channels to be transmitted at last as shown below

- Case 1: If the first collision type is resolved first, PUSCH is dropped at the first step, then the collision between HARQ-ACK and PDSCH-1 should be resolved, assuming dynamic HARQ-ACK is prioritized over PDSCH-1, PUCCH with HARQ-ACK and PDSCH-2 can be transmitted at last in this case.

- Case 2: If the second collision type is resolved first, the HARQ-ACK would be multiplexed on PUSCH, then there are still two kinds of collision, collision between PUSCH and DL usable PRBs, and collision between PUSCH and PDSCH-2.

  - Case 2-1: assuming the collision between PUSCH with HARQ-ACK and PDSCH-2 should be resolved first and dynamic HARQ-ACK is prioritized over PDSCH-2, then PDSCH-2 should be dropped, then the PUSCH with HARQ-ACK is dropped for collision with DL usable PRBs, only PDSCH-1 could be transmitted at last.

  - Case 2-2: assuming the collision between PUSCH with HARQ-ACK and DL usable PRBs should be resolved first, then PUSCH with HARQ-ACK should be dropped, PDSCH-1 and PDSCH-2 could be transmitted at last.

- Case 3: If the collision between downlink and uplink is resolved first, then for collision between PDSCH-1 and HARQ-ACK, assuming dynamic HARQ-ACK is prioritized over PDSCH-1, then PDSCH-1 should be dropped, for collision between PDSCH-2 and PUSCH, assuming PUSCH is prioritized over PDSCH-2, then PDSCH-2 should be dropped. Then there are still two kinds of collision, collision between PUSCH and DL usable PRBs, and collision between PUSCH and HARQ-ACK.

  - Case 3-1: assuming the collision between PUSCH with HARQ-ACK should be resolved first and HARQ-ACK is multiplexed on PUSCH, then the PUSCH with HARQ-ACK is dropped for collision with DL usable PRBs, no channel could be transmitted at last.

  - Case 3-2: assuming the collision between PUSCH and DL subband should be resolved first, then PUSCH should be dropped, only PUCCH with HARQ-ACK could be transmitted at last.

It can be seen that gNB and UE may have different understanding on which channel could be transmitted based on different understanding on the order of resolution of different collision types, hence it is proposed to study collision handling rules for the order of different collision types.

Proposal 24: For SBFD aware UEs, study the order of collision handling of the following collision types:

- Type 1: Collision between transmissions/receptions and unusable time/frequency resources;

- Type 2: Collision between transmissions with same transmission direction;

- Type 3: Collision between transmissions and receptions;

- Type 4: Collision between transmissions/receptions and the link direction indicated by gNB (if supported).

Conclusion

In this contribution, we discuss SBFD TX/RX/measurement procedures and give the following observations and proposals.

Observation 1: If Rel-18 NES CSI reporting framework is reused for CSI reporting for different symbols types, only one NZP CSI-RS resource can be configured in the resource set for interference measurement.

Observation 2: If only DL receptions within DL usable PRBs are allowed for SBFD aware UEs in SSB symbols, more than 5% DL resources may be wasted for short SSB periods of 5 and 10ms.

Proposal 1: For detailed indication of SBFD symbols within a TDD-UL-DL pattern period, down-select from the following options:

- Option 1: Separate indication of starting SBFD symbol and ending SBFD symbol;

- Option 2: Joint indication of starting SBFD symbol and number of SBFD symbols;

- Option 3: Separate indications of number of slots/symbols of non-SBFD symbols at the beginning and at the end of a TDD-UL-DL pattern period.

Proposal 2: Support to predefine or explicitly indicate transition period between SBFD and non-SBFD symbols.

Proposal 3: Support UL/DL subband frequency indication via RIV with the assumption of maximum UL/DL subband bandwidth of 275 PRBs.

Proposal 4: DMRS sequence is mapped to REs within allocated CRBs within DL usable PRBs for Option 1-1.

Proposal 5: For wideband precoder, non-contiguous frequency resources across two DL subbands but contiguous frequency resource within each DL subband is preferred.

Proposal 6: For PRG overlapping with subband boundary, the part of DL PRG inside the DL usable PRBs can be used in case the number of partial RBGs in SBFD symbols does not exceed two.

Proposal 7: For configuration 1, the valid symbol type is defined as below:

- For transmissions/receptions with scheduling or activation DCI, the symbol type is determined based on symbol type of the 1st transmission/reception slot/symbol;

- For transmissions/receptions w/o scheduling or activation DCI, the symbol type is determined based on RRC configuration.

Proposal 8: For configuration 1, UE behavior for transmissions/receptions in the invalid symbol type can be defined as below:

- For PUSCH repetition with available slot counting, TBoMS and PUCCH repetition, UE could postpone transmissions in the invalid symbol type;

- For the other transmissions/receptions except for SPS HARQ-ACK with defer, transmissions/receptions are invalid in the invalid symbol type;

- FFS SPS HARQ-ACK with defer in the invalid symbol type.

Proposal 9: For configuration 2, if the frequency resources of transmissions/receptions overlap with RBs outside UL/DL usable PRBs in SBFD symbols,

- For PUSCH repetition Type A with available slot counting and TBoMS, a PUSCH in a slot overlapping with RBs outside UL usable PRBs in SBFD symbols can be postponed;

- For PDSCH receptions and other PUSCH transmissions, down select from the following two options:

  - Option 1: A PDSCH reception overlapping with RBs outside DL usable PRBs/A PUSCH transmission overlapping with RBs outside UL usable PRBs in SBFD symbols is invalid;

  - Option 2: Only the assigned PRBs within DL/UL usable PRBs are valid for PDSCH/PUSCH in SBFD symbols.

Proposal 10: For transmissions and receptions across SBFD symbols and non-SBFD symbols in different slots, define configuration 2 as an optional UE capability for SBFD aware UEs.

Proposal 11: For CSI report associated with CSI-RS instances occur in both SBFD symbols and non-SBFD symbols in different slots, support Option A and Option B.

- For option A, new RRC signalling can be added in CSI-ReportConfig to indicate whether only CSI-RS transmission occasions in SBFD symbols or in non-SBFD symbols are used for CSI derivation;

- For Option B, the following enhancements can be supported:

  - The first sub-configuration is for non-SBFD symbols and the second sub-configuration is for SBFD symbols;

  - Support two NZP CSI-RS resources in the resource set for interference measurement in one CSI-ReportConfig.

Proposal 12: For a single TRP scenario, support separate UL power control for FR2 based on unified TCI state framework.

Proposal 13: For separate PUCCH power control parameters for SBFD and non-SBFD symbols, the following options can be further studied:

- Option 1: One TCI codepoint is mapped to two TCI states for SBFD symbols and non-SBFD symbols respectively. Unified TCI states including UL power control parameters are determined based on symbol type of the UL transmission.

- Option 2: Same unified TCI state is associated with separate UL power control parameters for SBFD symbols and non-SBFD symbols. UL power control parameters are determined based on symbol type of the UL transmission.

Proposal 14: Support separate SRS resource sets for SBFD symbols and non-SBFD symbols.

Proposal 15: For PUCCH resource configuration for SBFD aware UEs, support separate configurations of parameters including at least startingPRB and secondHopPRB for a certain PUCCH resource.

Proposal 16: Support separate configurations of FH offsets for PUSCH transmissions in SBFD symbols and non-SBFD symbols,

- In case FH offsets for PUSCH transmissions in SBFD symbols are not configured, FH for PUSCH transmissions in SBFD symbols is disabled;

- The number of FH offsets for SBFD symbols does not exceed the number of FH offsets for non-SBFD symbols;

- The number of bits used to indicate FH offset in DCI is determined according to existing rule.

Proposal 17: For PUSCH in SBFD symbols, update the PUSCH frequency hopping formula by replacing UL BWP by UL usable PRBs.

Proposal 18: For SBFD aware UEs link direction determination, support Option 1 and Option 2 if the benefits of Option 2 are agreed.

- Option 1: UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any).

- Option 2: link direction is indicated by gNB explicitly.

Proposal 19: If link direction indication is not supported nor provided for a SBFD symbol, cell-specifically configured DL reception defined in collision Case 3 refers to PDCCH in Type-0/0A/0B/1/2 CSS set.

Proposal 20: If link direction indication is not supported nor provided for a SBFD symbol, no special handling for DL reception with repetitions and/or UL transmission with repetitions is introduced for collision Case 1/2/4.

Proposal 21: For SSB symbols configured with SBFD subbands, discuss and decide whether SSB symbols can fall back to full DL symbols. If supported, a longer duration including SSB symbols and other adjacent symbols should be considered for fallback.

Proposal 22: For UL transmission overlapping with SBFD symbol(s) configured on SSB symbol(s), reuse existing collision handling schemes for UL transmission overlapping with SSB symbol(s).

Proposal 23: For SBFD aware UEs, if UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any), the following collision handling rules for HD-FDD RedCap UE is reused:

- UE does not expect collision between PRACH triggered by PDCCH order and DL channel/signal with scheduling DCI in the same SBFD symbol;

- For collision between PRACH triggered by PDCCH order and PDCCH, PDSCH, CSI-RS or DL PRS configured by higher layers in the same SBFD symbol, UE does not receive the PDCCH, PDSCH, CSI-RS or DL PRS configured by higher layers;

- For collision between PRACH triggered by higher layer and DL channel/signal in the same SBFD symbol, whether to prioritize DL channel/signal or PRACH transmission is up to UE implementation.

Proposal 24: For SBFD aware UEs, study the order of collision handling of the following collision types:

- Type 1: Collision between transmissions/receptions and unusable time/frequency resources;

- Type 2: Collision between transmissions with same transmission direction;

- Type 3: Collision between transmissions and receptions;

- Type 4: Collision between transmissions/receptions and the link direction indicated by gNB (if supported).

Reference

- Chairman's Notes RAN1#117, 3GPP TSG RAN WG1 Meeting #117, Fukuoka City, Fukuoka, Japan, May 20th – 24th, 2024.

- R4-2316941, WF on SBFD BS requirements, Ericsson, RAN4 #108bis, Xiamen, China, 9th ‒ 13th October, 2023.

- R4-2409958, Way Forward for [110bis][313] NR_duplex_evo, Samsung, RAN4 #111, Fukuoka City, Fukuoka, Japan, 20th – 24th May, 2024.