3GPP TSG RAN WG1 #118		R1-2406537

Maastricht, NL, August 19th – 23rd, 2024

Agenda item:	9.6.1

Source:	NEC

Title:              	Discussion on LP-WUS and LP-SS design

Document for:	Discussion and Decision

Introduction

In the RAN1#117, the following agreements and working assumptions on LP-WUS and LP-SS design were achieved:

Agreement Support overlaid OFDM sequence based on existing NR sequence type for LP-WUS - Down select among gold sequence, m sequence and ZC sequence - FFS the overlaid OFDM sequence is time or frequency domain sequence. - FFS how to reuse the existing sequences - Note: Strive to minimize the impact on OOK detection performance - If overlaid OFDM sequence is supported for LP-SS, the same sequence type is used for both LP-SS and LP-WUS Agreement For the LP-SS sequence used in a cell, - Option 1: the information necessary for determining the sequence is explicitly configured - FFS: Additional support of determining the sequence by predefined rule without configuration Agreement (further amended as shown in red on Friday) The LP-WUS and LP-SS design assumes the residual frequency error after frequency error correction without considering impact of drift, is up to X ppm for OOK-based LP-WUR. - FFS X which is no larger than 20ppm - FFS: Initial frequency error assumption: 200ppm - Initial frequency error assumption: up to company report For the overlaid OFDM sequence design of LP-WUS, it is assumed that the residual frequency error for OFDM-based LP-WUR after frequency error correction without considering impact of drift is not larger than Y. - FFS Y which is no larger than 20ppm and lower than X - FFS: Initial frequency error assumption: 200ppm - Initial frequency error assumption: up to company report Agreement For overlaid OFDM sequence(s) for LP-WUS in time or frequency domain, down-selection from the following: - Option 1-1: overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing - The length of overlaid sequence(s) depends on the number of REs used for LP-WUS and the value of M - Option 1-2: overlaid sequence(s) are the sequence(s) of an OFDM symbol before DFT/LS processing - The length of overlaid sequence(s) depends on the number of REs used for LP-WUS - Option 2: overlaid sequence(s) are the sequence(s) of an OFDM symbol before IFFT processing - The length of overlaid sequence(s) depends on the number of REs used for LP-WUS - Option 3: overlaid sequence(s) are the sequence(s) of an OOK on symbol in time domain after IFFT processing - The length of overlaid sequence(s) depends on IFFT size and the value of M FFS: same or different options are applied for OOK-1 and OOK-4 M>1. Agreement (further amended as shown in red on Friday) Update agreement in last meeting as below: From RAN1 perspective, support X PRBs for LP-WUS and LP-SS with SCS 30kHz (blanked guard RBs are not included) for a channel bandwidth equal or larger than 5MHz - X to be down-selected between = 11 and 12 PRBs - FFS the number of PRBs for 15kHz is 11 PRBs or 22 PRBs? - FFS if other number of PRBs needed, for LP-SS and LP-WUS with a channel bandwidth equal or less than 5MHz FFS: Whether the above is applicable to FR2 Agreement Further down-select the number of binary LP-SS sequences for the ‘ON-OFF’ pattern: - 3 - 4 - 8 - 16 Agreement For the binary LP-SS sequence type for the ‘ON-OFF’ pattern in a LP-SS, further down-selection from the following: - Gold sequence - M sequence - Computer searched sequence - FFS: the length of LP-SS sequence Agreement The following SNR values for LP-WUR are reported by companies to achieve coverage of PUSCH for message3 for difference noise figures for calibration purposes: - NF of LR = NF of MR+ 8dB: the reported SNR value range is [-9.05,2.94] dB - NF of LR = NF of MR+ 5dB: the reported SNR value range is [-6.5, 5.58] dB - NF of LR = NF of MR+ 2dB: the reported SNR value range is [-4.04,7.95] dB - Note 1: The above is observed based on the following assumptions used for calibration: - Carrier frequency: 2.6 GHz - The number of Tx chains: 1 - MIL of MSG 3: use the average one in R17 coverage, i.e.,153.51 dB for non-redcap UE - Transmit antenna gain correction factors for WUS: up to company report - Noise Figure: All three values +2dB, +5dB, +8dB on top of NF of MR (7dB) are to be reported, SNR for different assumptions on NF are determined separately Agreement For evaluation of LP-WUS and LP-SS design to achieve coverage of PUSCH for message3 from RAN1 perspective, at least the following SNR values should be considered: - SNR=-3dB for NF of LR = NF of MR+ 8dB - SNR= -0.5dB for NF of LR = NF of MR+ 5dB - SNR=2dB for NF of LR = NF of MR+ 2dB - Note 1: The NF of MR is assumed as 7dB Agreement For RRC idle/inactive state, down-select among the following options for at least indicating subgroup information using LP-WUS: - Option 1: A LP-WUS indicates a bitmap with each bit corresponding to one subgroup of N subgroups for part of, one or more PO(s), e.g., N is 8~16, 24 - Number of information bits for a LP-WUS is at least N, single LP-WUS to wake up one or more subgroups - Option 2: A LP-WUS indicates a codepoint value corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs, e.g., N is 8~256 - Number of information bits for a LP-WUS is at least ceil (log2(X1)), where X1 is the number of codepoints indicating one or more subgroups. X1 is reported by companies, X1 could be smaller, equal to or larger than N. - Option 3: A LP-WUS indicates multiple codepoint values with each corresponding to one or more subgroup(s) from N subgroups for part of, one or more POs, e.g., N is 8~256 - Number of information bits for a LP-WUS is at least K*ceil (log2(X2)), where X2 is the number of codepoints indicating one or more subgroups. X2 is reported by companies, X2 could be smaller, equal to or larger than N. - K is the number of multiple codepoint values in a LP-WUS where K is larger than 1 - How to satisfy FAR is reported by companies, e.g., FEC/CRC - Note: multiple TDMed LP-WUSs can be used to support more subgroups for each option. - Note: Y% effective paging rate per PO is reported by companies - The followings are considered when down-select among options: - The number of supported UE subgroups per PO: M - Average network overhead to indicate the number of UE subgroups M per PO - False wake up rate due to subgroup-based indication, which will impact the power saving gain - Paging latency - Note: Coverage target shall be met under 1%BLER, 1% FAR (for false alarm from noise)

In this contribution, we provide our views on LP-WUS and LP-SS design.

Discussion

  - LP-WUS and LP-SS generation

In the Rel-18 LP-WUS/WUR study item, four variants of OOK waveforms as well as the FSK and OFDM waveforms were studied, evaluation and down-selection of the waveforms were made during the SI. In the Rel-19 WID, OOK-1 and OOK-4 were selected as the candidates for LP-WUS and LP-SS waveform. Fig. 1 shows a general framework for the OOK-1/OOK-4 waveform generation.

Fig 1. A common structure for OOK-1 and OOK-4

In RAN1#116, it is agreed that both OOK-1 and OOK-4 are supported for LP-WUS generation, however the details are FFS. In our view, a common design for OOK-1 and OOK-4 can be achieved based on the OOK-4 framework, where OOK-1 can be treated as a special case of OOK-4, i.e., the number of bits per OFDM symbol is equal to 1. The only difference from the original version of OOK-1 may be the overlaid sequence has to be defined in time domain rather than in frequency domain. Considering it is natural that time domain overlaid sequence will be used for OOK-4, reusing the same design of time domain overlaid sequence for OOK-1 may need little standard effort.

Proposal 1: for LP-WUS and LP-SS generation, support a common design for OOK-1 and OOK-4, where OOK-1 can be a special case of OOK-4 with M=1.

For the legacy OFDM symbol generation, CP lengths of OFDM symbols in a subframe are not uniform. For NCP, the first OFDM symbol of each half subframe (e.g., the 0th and 7th symbols of a slot for 15KHz SCS, as shown in Fig.2) has a longer CP (i.e., 160*Ts) than other symbols (i.e., 144*Ts ) in a frame.

Fig 2. Symbol lengths in a subframe for 15KHz SCS

For legacy OFDM signal, CP is removed at the receiver side, therefore the useful parts for all the symbols in a subframe are always same. However, for a LP-WUR, it is not clear whether the CP removal will be supported. For a LP-WUR with baseband envelop detection, it may be easy to support the CP removal. However, for a LP-WUR with RF/IF envelope detection, considering the relatively poor performance of the components (e.g., 1bit ADC with low sampling rate may be used), whether and how the CP can be removed is still an open issue.

Even if the CP can be removed by LP-WUR, there are still issues to be solved. Fig. 3 gives an example of the LP-WUR receiver windows of OOK symbols (OOK-1 is assumed) when time domain sync error occurs. If the receiver timing is later than the symbol timing, the receiver window for an OOK-OFF symbol may be partially overlapped with the next OOK symbol. As shown in Fig. 3, if the second OOK symbol is an OOK-OFF symbol and the third symbol is an OOK-ON symbol, then some energy of the third OOK-ON symbol will be collected by the receiver window for the second OOK-OFF symbol, and this leaked energy cannot be eliminated by CP removal. Therefore the detection performance of the OOK-OFF symbol will be degraded. In order to avoid such an inter-symbol-interference, zero-CP or partial zero-CP (e.g., the front part of the CP are mapped with zero values) can be considered, e.g., if the CP of the last OOK-ON symbol can be mapped with zero values, there may be no energy leakage to the receiver window for the preceding OOK-OFF symbol.

Fig 3. Example of miss-alignment of receiver timing and symbol timing for OOK-1

For OOK-4, there will be multiple bits mapped into one OFDM symbol. Fig.4 gives an example of OOK-4 waveform, where 4 OOK symbols are mapped into one OFDM symbol. If CP is added normally, i.e., copy from the end of OFDM symbol, as Fig.4 shows, the CP part will be same as the end part of OOK3 symbol. However, such a CP may have negative impact on the detection of OOK0. Firstly, the CP is not a copy from the end of OOK0, so it has no ISI cancellation effect as the ordinary CP even if an OFDM based LP-WUR is used. Secondly, considering the potential sync error, some of the energy of the CP may be leaked into the receiver window of OOK0, therefore interference from CP to OOK0 may occur if OOK3 is an OOK-ON symbol and OOK0 is an OOK-OFF symbol. Zero-CP or partial zero-CP can also be considered to alleviate this issue.

Fig 4. Example of adding CP for an OOK-4 waveform

Proposal 2: study the inter-symbol-interference (ISI) issue and the CP-to-OOK interference issue due to the sync error, consider utilizing zero-CP or partial zero-CP to avoid the interference.

In the Rel-18 SI, the frequency location of LP-WUS and LP-SS is studied, and it is concluded that it is beneficial if LP-WUS can be flexibly configured within a carrier. In our view, to further enhance the flexibility, it is beneficial to support multiple frequency bands for LP-WUS in a carrier, i.e., gNB may configure multiple frequency bands for LP-WUS transmission, and UE may select a frequency band from the multiple frequency bands for LP-WUS monitoring based on its UE-ID or the PF/PO it is intended to monitor. Based on this, UEs may be allocated to different frequency bands, and therefore the paging false alarm probability may be reduced due to the grouping by frequency bands selection.

Proposal 3: support flexibly configuring frequency locations of one or more LP-WUS bands within a carrier, UE can select an LP-WUS band based on its UE ID or a PF/PO it is intended to monitor.

  - Design of LP-WUS

In the Rel-18 SI, both sequence based LP-WUS and message based LP-WUS were studied. The sequence based LP-WUS may have better coverage, however the supported payload size may be less than message based LP-WUS. In our view, it is beneficial to support a large number of UE subgroups in order to avoid frequently paging false alarm. This means the payload size of LP-WUS may have to be relatively large. Therefore, in order to support more UE subgroups in one LP-WUS, at least the message based LP-WUS should be supported. As shown in the Fig. 5, a message based LP-WUS may comprise a preamble, a message and a CRC, wherein the preamble is used for finer synchronization and AGC (if have), and the message part conveys the content of LP-WUS.

Fig 5. Example of message based LP-WUS

Proposal 4: support message based LP-WUS structure with a preamble and a CRC.

If message based LP-WUS is used, repetition of LP-WUS can be considered to improve the coverage. Specifically, gNB may transmit same content in multiple LP-WUS monitoring occasions. At the UE side, the received signal of the multiple LP-WUS monitoring occasions may be combined to improve performance, or UE can also attempt to detect LP-WUS in the multiple LP-WUS monitoring occasions independently. Both the two methods can effectively enhance the detection probability, therefore the coverage can be improved.

Proposal 5: support repetition of LP-WUS to improve the coverage.

In RAN1#117, several options for the definition of overlaid sequence for LP-WUS were agreed for down-selection. Where option 1-1 and option 1-2 are defined before DFT/LS processing, option 2 is defined before IFFT, and option 3 is defined after IFFT. In our view, all these methods can work, however, define the sequence before DFT/LS processing is the easiest way, other options can be derived based on option 1-1 with gNB implementation. Regarding the difference between option 1-1 and option 1-2, we think they are technically same but option 1-1 is cleaner than option 1-2, since there will be no difference for OOK-1 and OOK-4.

Proposal 6: for the overlaid sequence for LP-WUS, support option 1-1, i.e., overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing, for both OOK-1 and OOK-4.

For the overlaid OFDM sequence for LP-WUS, several options were agreed for further study in RAN1#116b. In our view, information carried by sequence selection from multiple candidate sequence is widely used in existing NR spec and can also be considered for OFDM based LP-WUR. Regarding option 2 and option 3, since anyway an overlaid sequence will be used for the generation of each OOK ON symbol, it is naturally to map overlaid sequence on all of the OOK ON symbol, therefore we see no reason to use option 3. Regarding option 2-1 and option 2-2, in our view, network may map all the information bits on the overlaid OFDM sequence, however the LP-WUR may only receive part of the OOK symbols to acquire all the information bits jointly based on the OOK on-off pattern and the overlaid OFDM sequence.

Proposal 7: regarding the overlaid OFDM sequence(s) of LP-WUS, support option 2-2, i.e. the overlaid OFDM sequence(s) carry all information bits of LP-WUS.

Proposal 8: UE is allowed to not receive all the OOK symbols of LP-WUS, and UE can acquire all the information bits based on the OOK on-off pattern and the overlaid sequence of partial OOK symbols of the LP-WUS.

  - Design of LP-SS

Different from LP-WUS, LP-SS can be a common signal for all the UEs which are in the LP-WUS monitoring mode in a cell. Generally, the LP-SS is used for maintenance of time/frequency synchronization and RRM measurement, therefore it is not necessary to convey too much information on it. Therefore, the LP-SS can be a sequence based signal. The existing sequence generation method in NR spec, e.g., m sequence, gold sequence, etc., can be the start point of the LP-SS sequence design.

Proposal 9: for the binary sequence of LP-SS, reuse the existing sequence generation method in NR, e.g., m-sequence, gold sequence.

An open issue for LP-SS is whether the overlaid sequence for LP-SS should be specified and whether it is used for sync and RRM measurement. In our view, considering the overlaid sequence will be defined for LP-WUS, reusing same overlaid sequence design for LP-SS will be straightforward and may cost acceptable standard effort, therefore we think the overlaid sequence should be specified for LP-SS. In addition, for a LR with OFDM detection capability, we see no reason to prevent it utilizing the overlaid sequence for synchronization, however, for the RRM measurement, we think it may depend on RAN4 decision.

Proposal 10: for the overlaid OFDM sequence(s) for LP-SS, support option 3, i.e., specify the overlaid OFDM sequence(s) targeting for OOK waveform generation and also targeting for sync [and RRM measurement] for OFDM-based LP-WUR using the overlaid sequence of LP-SS.

In RAN1#116, it is agreed that beam based transmission of LP-SS is supported. Similar as SSB burst set, an LP-SS set may be defined which comprises multiple LP-SS and each of the LP-SS may be QCLed with an SSB. For example, the LP-SSs in an LP-SS set may be indexed as 0, 1, …M-1, and M may be equal to the number of SSBs in an SSB burst, in addition, the indices may be used to determine the QCL relationship between LP-SS and the SSB, e.g., an LP-SS and an SSB with same index can be assumed to be QCLed.

Similar as the SSB, the time domain positon of an LP-SS can be utilized for a UE to determine the frame/subframe/slot/symbol timing. A straightforward method is to let a LP-SS and its associated SSB multiplexed in same OFDM symbols, therefore they can share same timing information, if UE detected a LP-SS, it may determine the timing information based on the SSB configuration. On the other hand, due to the LP-SS and the SSB are QCLed, it is naturally that gNB can transmit them with a same TX beam, and gNB does not to find another time domain resource to transmit the LP-SS with this TX beam.

In addition, in order to improve the coverage, repetition of LP-SS may be supported. As shown in Fig. 6, if repetition is configured for LP-SS, a LP-SS may be transmitted in two consecutive SSB period, UE may combine the received signal of the two LP-SS to enhance the detection performance.

Proposal 11: support QCL relationship between an LP-SS and an SSB.

Proposal 12: support FDM multiplexing of an LP-SS and its QCLed SSB.

Proposal 13: support repetition of an LP-SS in an LP-SS periodicity.

Fig 6. Example of FDMed LP-SS and SSB

Conclusion

In this contribution, we discuss the LP-WUS and LP-SS design, and the following proposals are made:

Proposal 1: for LP-WUS and LP-SS generation, support a common design for OOK-1 and OOK-4, where OOK-1 can be a special case of OOK-4 with M=1.

Proposal 2: study the inter-symbol-interference (ISI) issue and the CP-to-OOK interference issue due to the sync error, consider utilizing zero-CP or partial zero-CP to avoid the interference.

Proposal 3: support flexibly configuring frequency locations of one or more LP-WUS bands within a carrier, UE can select an LP-WUS band based on its UE ID or a PF/PO it is intended to monitor.

Proposal 4: support message based LP-WUS structure with a preamble and a CRC.

Proposal 5: support repetition of LP-WUS to improve the coverage.

Proposal 6: for the overlaid sequence for LP-WUS, support option 1-1, i.e., overlaid sequence(s) are the sequence(s) of an OOK on symbol before DFT/LS processing, for both OOK-1 and OOK-4.

Proposal 7: regarding the overlaid OFDM sequence(s) of LP-WUS, support option 2-2, i.e. the overlaid OFDM sequence(s) carry all information bits of LP-WUS.

Proposal 8: UE is allowed to not receive all the OOK symbols of LP-WUS, and UE can acquire all the information bits based on the OOK on-off pattern and the overlaid sequence of partial OOK symbols of the LP-WUS.

Proposal 9: for the binary sequence of LP-SS, reuse the existing sequence generation method in NR, e.g., m-sequence, gold sequence.

Proposal 10: for the overlaid OFDM sequence(s) for LP-SS, support option 3, i.e., specify the overlaid OFDM sequence(s) targeting for OOK waveform generation and also targeting for sync [and RRM measurement] for OFDM-based LP-WUR using the overlaid sequence of LP-SS.

Proposal 11: support QCL relationship between an LP-SS and an SSB.

Proposal 12: support FDM multiplexing of an LP-SS and its QCLed SSB.

Proposal 13: support repetition of an LP-SS in an LP-SS periodicity.

References

- RP-234056, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR), CMCC, RAN meeting #102, Dec. 11-15, 2023