Agenda item:	9.3.1

Source: 	Lenovo

Title: 	Views on SBFD TX/RX Measurement Procedures

Document for:	Discussion and Decision

# Introduction

In this contribution, we discuss various aspects related to the SBFD TX/RX measurement procedures at a serving cell from the perspective of SBFD aware UEs and provide our views on some of the agreements made in RAN1#117.

# Baseline SBFD Configuration

The baseline SBFD operation, at least for UEs in RRC_CONNECTED mode as captured in TR 38.858 [1], includes semi-static and explicit configurations of time and frequency locations of at least the SBFD UL subband, which can be located at either side of the carrier or at the middle part of the carrier and that the SBFD UL subband configuration across different SBFD symbols is the same. In our view, SIB1-based signaling of the SBFD subband time and frequency configurations should be supported to reduce the configuration overhead and to be able to support the random-access operations in SBFD subbands, including that for RRC_IDLE and INACTIVE RRC states.

- RAN1 to support SIB1-based signaling of time and frequency locations of SBFD subbands.

## Semi-static configurations of time location of SBFD subbands

In the time domain, it has been agreed that the SBFD symbols are configured semi-statically in a consecutive manner within a SBFD period to reduce the number of transition points between SBFD and non-SBFD symbols, and that a slot can consist of SBFD symbols and non-SBFD symbols. The SBFD period is defined by the following agreement from RAN1#117 meeting.

Agreement For RRC connected mode UEs, SBFD subband time period: - When only one TDD-UL-DL pattern is configured, the period is the same as TDD-UL-DL pattern period configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon. - When two TDD-UL-DL patterns are configured, the period is the same as the sum of the two TDD-UL-DL pattern periods configured by dl-UL-TransmissionPeriodicity in TDD-UL-DL-ConfigCommon.

In the current specifications, a UE may optionally be configured with a second TDD UL/DL pattern by TDD-UL-DL-ConfigCommon. Two different scenarios may occur when a UE is configured by two TDD UL/DL patterns:

- Scenario 1: Two TDD-UL-DL patterns are configured and the SBFD symbols are configured for only one of the patterns.

- Scenario 2: Two TDD-UL-DL patterns are configured and the SBFD symbols are configured for both patterns.

With Scenario 2, how to configure the SBFD pattern for each TDD UL/DL pattern is yet to be discussed. One direct option is to provide a specific SBFD pattern for each configured TDD UL/DL pattern. In another option, one SBFD pattern can be provided for the two configured TDD UL/DL patterns to reduce the indication overhead. However, considering that the two TDD UL/DL patterns normally have different configurations (i.e., the two configured patterns are different), how to link a single SBFD pattern to two TDD UL/DL patterns need to be studied.

- In the scenario where a UE is configured with two TDD UL/DL patterns and the SBFD symbols are configured for both patterns, RAN1 to study the option where a single SBFD pattern is provided for the two TDD UL/DL patterns.

Moreover, two options were listed in the following agreement regarding the link direction determination for a SBFD-aware UE transmission and reception in an SBFD symbol. In our opinion, Option 1 is sufficient and there is no need to introduce an explicit link direction indication for SBFD-aware UEs.

- Support Option 1 regarding link direction determination for a SBFD-aware UE transmission and reception in an SBFD symbol.

Option 1: UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any).

Agreement For SBFD-aware UE transmission and reception in an SBFD symbol, consider the following options to determine link direction, i.e. whether to transmit or to receive in the SBFD symbol. - Option 1: UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any). - Option 2: link direction is indicated by gNB explicitly. Other options are not precluded.

## Semi-static configurations of frequency locations of SBFD subbands

In the frequency domain, it has been already agreed that 1) the semi-static configuration of SBFD UL subband can be located at either side of the carrier or at the middle part of the carrier and that the configuration across different SBFD symbols is the same, 2) the indication of cell-specific frequency locations of UL subband and DL subband(s) are explicitly configured, whereas the guardband(s), if any, are implicitly derived as the RBs which are not within UL subband or DL subband(s), and 3) the cell-specific frequency locations of SBFD subbands are separately configured for each SCS configuration in SCS-SpecificCarrierList, where for each SCS configuration, the reference starting PRB is the PRB determined by the SCS configuration and offsetToCarrier corresponding to this subcarrier spacing.

The frequency domain length of a SBFD UL subband is composed of one or more of consecutive RBs or RBGs and can be naturally characterized by a start RB/RBG and a number (a length) of RBs/RBGs, as exemplified in Option A of Figure 1, where both (i.e., start RB/RBG and number of RBs/RBGs) are constrained by the total number of RBs/RBGs within a target carrier or BWP. Eventually, the number of RBs/RBGs of an allocated UL subband will occupy a certain percentage of the RBs/RBGs within a target carrier or BWP, e.g., 20% of the total number of RBs within a target BWP, as exemplified in Option B of Figure 1. Here, we observe that indicating the number of RBs/RBGs of the SBFD UL subband as a percentage value of, e.g., the total number of RBs/RBGs within the target BWP requires, in general, less indication overhead, which is highly important in case the SBFD configurations is agreed to be included in SIB1 messages.

- Indicating the number of RBs/RBGs of an UL subband as a percentage value of, e.g., the total number of RBs/RBGs within the target BWP requires, in general, less indication overhead.

To illustrate the above observation, let us consider a scenario where the total number of RBs of a target BWP is 273 RBs (for 100 MHz BWP with 30 kHz SCS) and the SBFD UL subband frequency location is characterised by a start RB = 105 and a number of RBs = 55. Using the resource indication value (RIV) mapping method (defined in TS 38.214 - 5.1.2.2), the start RB (i.e., RBstart=105) and the number of RBs (i.e., LRBs=55) would be encoded into RIV = 14847. However, when considering that the number of RBs = 20% (i.e., LRBs=f55273=f(0.2014) ≈20, where f. is a rounding function to the nearest integer), then the start RB and the number of RBs would be encoded into RIV = 5259, which obviously requires less indication overhead.

- The semi-static indication of frequency location of SBFD UL subband is indicated explicitly via a start RB/RBG and a number of RBs/RBGs values and the number of RBs/RBGs is indicated as a percentage value of a total number of RBs/RBGs.

Figure 1: Frequency domain location indication options for a SBFD UL subband. Different than Option A, Option B reduces the indication overhead by assuming that the number of RBs/RBGs of the SBFD UL subband is indicated as a percentage value of the BWP total number.

# DL Reception and UL Transmission

In RAN1#117 meeting, for an SBFD aware UE, the following agreement captures two configuration options for UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots, where each transmission and reception within a slot has either all SBFD or all non-SBFD symbols. Configuration 1 may reduce complexity of DL receptions and UL transmissions since it restricts the DL receptions and the UL transmissions to a single symbols type, but it may potentially reduce the SBFD performance, when compared to Configuration 2. From this viewpoint, Configuration 2 seems more advantageous and should be supported.

- For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots, support Configuration 2 captured in a RAN1#117 meeting agreement.

Agreement For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots (each transmission/reception within a slot has either all SBFD or all non-SBFD symbols) for an SBFD aware UE, the SBFD-aware UE is provided with one of the configurations. - Configuration 1: The transmissions/receptions are restricted to SBFD symbols only or non-SBFD symbols only - Configuration 2: The transmissions/receptions can be in SBFD symbols and non-SBFD symbols - FFS granularity of the configuration, e.g. per UE, per channel/signal etc. - FFS whether support of configuration 2 is subject to UE capability

## DL Reception

For legacy UEs, the gNB can schedule their DL signals/channels in symbols/slots with a SBFD UL subband as long as the indicated DL signal/channel resources do not overlap with the UL subband. For SBFD aware UEs, enhanced resource allocation schemes may be used for efficient resource utilization.

### PDSCH

For PDSCH reception with downlink type 0 resource allocation, a SBFD aware UE should perform rate matching around an SBFD UL subband, when an allocated PDSCH resource overlaps with the SBFD UL subband, as captured by the following RAN1#116bis and RAN1#117 meetings agreements.

Agreement: For frequency resource allocation Type 0 for PDSCH or PUSCH in a single slot by DCI based scheduling (without repetition or TBoMS), when an assigned RBG overlaps with the subband boundary, only the PRBs within DL usable PRBs are considered to be valid for PDSCH reception and only the PRBs within UL usable PRBs are considered to be valid for PUSCH transmission. SBFD aware UE does not expect to be assigned with a RBG for PDSCH which is fully outside DL usable PRBs or a RBG for PUSCH which is fully outside UL usable PRBs.

Agreement For frequency resource allocation Type 0 for PDSCH or PUSCH in a single slot by DCI based scheduling (without repetition or TBoMS), when an assigned RBG overlaps with the subband boundary, the number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only and assigned PRBs within UL usable PRBs only for PDSCH and PUSCH respectively.

Moreover, the following agreement related to the frequency domain resource allocation Type 1 for PDSCH was reached in the last RAN1#116bis meeting.

Agreement For frequency domain resource allocation Type 1 for PDSCH in a single slot scheduled at least by DCI format in USS, discuss and decide whether/which of the following options is supported. - Option 1-1: Only the assigned PRBs within DL usable PRBs are considered to be valid for PDSCH. Assigned PRBs that fall outside DL usable PRBs are considered to be invalid and should not be used for PDSCH resource mapping. - Existing RB indexing and VRB-to-PRB mapping are reused - The number of PRBs for TBS determination is based on the assigned PRBs within DL usable PRBs only - FFS: DMRS sequence mapping - Option 1-2: Only the assigned PRBs within DL usable PRBs are considered to be valid for PDSCH. Assigned PRBs that fall outside DL usable PRBs are considered to be invalid and should not be used for PDSCH resource mapping. - Existing RB indexing and VRB-to-PRB mapping are reused - The number of PRBs for TBS determination is based on the assigned PRBs as legacy - FFS: DMRS sequence mapping - Option 2: Introduce new RB indexing/PRB bundle indexing to ensure VRBs are mapped to DL usable PRBs only. - Existing VRB-to-PRB mapping is reused - Legacy TBS determination method is used - FFS: DMRS sequence mapping - Option 3: Modify VRB-to-PRB mapping interleaver to ensure VRBs are mapped to DL usable PRBs only. - Existing RB indexing/PRB bundle indexing is reused - If the interleaver is not enabled, Option 1-1 or Option 1-2 is used - Legacy TBS determination method is used FFS: DMRS sequence mapping

In our view, the above agreement should be separately discussed for interleaved and non-interleaved cases. For PDSCH reception with non-interleaved downlink type 1 resource allocation, a SBFD aware UE should perform rate matching around an SBFD UL subband, when an allocated PDSCH resource overlaps with the SBFD UL subband.

- For PDSCH reception with non-interleaved downlink type 1 resource allocation, support Option 1-1, where a SBFD aware UE performs rate matching around the SBFD UL subband.

On the other hand, for PDSCH reception with interleaved downlink type 1 resource allocation, the number of virtual resource blocks (VRBs) is set to an active BWP size NBWPsize physical resource blocks (PRBs) minus the number of PRBs overlapping with the SBFD UL subband. That is, the VRBs are indexed without considering RBs overlapping with the SBFD UL subband. For interleaved VRB-to-PRB mapping, resource block bundles are defined based on resource blocks that do not overlap with a time and frequency resource of the SBFD UL subband.

- For PDSCH reception with interleaved downlink type 1 resource allocation, support Option 2, where a SBFD aware UE indexes VRBs and defines resource block bundles based on resource blocks not overlapping with the SBFD UL subband.

Moreover, the following two agreements related to an SPS PDSCH configuration without repetitions, PDSCH repetitions across SBFD symbols and non-SBFD symbols in different slots, and multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each PDSCH within a slot has either all SBFD or all non-SBFD symbols was reached in the last RAN1#116bis meeting.

Agreement For an SPS PDSCH configuration without repetitions, if the reception occasions are across SBFD symbols and non-SBFD symbols where each reception occasion has either all SBFD or all non-SBFD symbols, discuss and decide whether/which of the following option(s) are supported. - Option 1: Separate resource allocations for SBFD symbols and non-SBFD symbols - FFS other separate configurations for SBFD symbols and non-SBFD symbols - Option 2: Single resource configuration/indication for one symbol type (SBFD or non-SBFD symbol) and RB offset(s) configuration/indication/determination to determine resource for the other symbol type - Option 3: An SPS PDSCH reception occasion overlapping with RBs outside DL usable PRBs in SBFD symbols is invalid - Option 4: Only SPS PDSCH reception occasion in one symbol type is valid and SPS PDSCH reception occasion in the other symbol type is invalid - Option 5: Only the assigned PRBs within DL usable PRBs in SBFD symbols are considered to be valid for SPS PDSCH - Other options are not precluded

Agreement For PDSCH repetitions across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols, and for multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols in different slots, where each PDSCH within a slot has either all SBFD or all non-SBFD symbols, discuss and decide whether/which of the following option(s) are supported. - Option 1: Separate FDRA configuration/indications/interpretations for SBFD symbols and non-SBFD symbols - Option 2: Single FDRA configuration/indication for one symbol type (SBFD or non-SBFD symbol) and RB offset(s) configuration/indication/determination to determine resource for the other symbol type - Option 3: A PDSCH in a slot overlapping with RBs outside DL usable PRBs in SBFD symbols is invalid, e.g. the PDSCH in the slot is dropped - Option 4: Only PDSCH in one symbol type is valid and PDSCH in the other symbol type is invalid - Option 5: For a PDSCH in a slot overlapping with RBs outside DL usable PRBs in SBFD symbols, only the assigned PRBs within DL usable PRBs are considered to be valid - Option 6: gNB does not schedule any PDSCH in SBFD symbols in a slot to be overlapping with PRBs outside DL usable PRBs - Other options are not precluded FFS: Applicable conditions

For an SPS PDSCH configuration without repetitions, in our view, Option 4 in the above agreement should be supported, as it provides a sufficient flexibility and requires less specs impact.

- For an SPS PDSCH configuration without repetitions, Support Option 4 in the agreement made in RAN1#116bis meeting.

Option 4: Only SPS PDSCH reception occasion in one symbol type is valid and SPS PDSCH reception occasion in the other symbol type is invalid.

On the other hand, for PDSCH repetitions and for multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each repetition/PDSCH within a slot has either all SBFD or all non-SBFD symbols, Option 5 in the above agreement should be supported, as it provides a sufficient flexibility and requires less specs impact.

- For PDSCH repetitions and for multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each repetition/PDSCH within a slot has either all SBFD or all non-SBFD symbols, Option 5 in the above agreement should be supported.

Option 5: For a PDSCH in a slot overlapping with RBs outside DL usable PRBs in SBFD symbols, only the assigned PRBs within DL usable PRBs are considered to be valid.

Some RBs of a PDSCH PRG (precoding granularity) may overlap with the SBFD UL subband resulting in a reduction in the actual number of consecutive PRBs in the PRG. The current TS 38.214 specification supports actual number of consecutive PRBs in each PRG could be one or more and not always equal to the configured precoding granularity.

- For SBFD-aware UEs and PRG(s) with size of 2 and 4 that overlaps with subband boundary, the actual number of consecutive PRBs in each PRG can be one or more and not always equal to the configured precoding granularity as already supported in current TS 38.214 specification.

In legacy, when fdmschemeB is configured for a UE, for wideband case, the PDSCH occasion corresponding to first TCI state is located at the first half PRBs of the allocated resource, and the PDSCH occasion corresponding to second TCI state is located at the remaining allocated PRBs. When the allocated resource is overlapped with SBFD UL subband, the PRBs number for each TCI state should be decided. For example, the invalid RPBs in UL subband could be excluded firstly, then the PDSCH occasion corresponding to first TCI state is located at the first half valid PRBs of the allocated PRBs, and the PDSCH occasion corresponding to second TCI state is located at the remaining valid RRBs of the allocated PRBs. For PRG(s) with size of 2 and 4, whether the invalid PRG should be indexed to determined the PDSCH occasion corresponding to each TCI state should also be determined.

- For SBFD-aware UEs configured with fdmschemeB and two TCI states for a PDSCH transmission, if the allocated resource overlaps with SBFD UL subband boundary, the actual PRBs for each PDSCH occasion corresponding to each TCI state should be determined.

In legacy, if a UE detects a DCI format 2_1 for a serving cell from the configured set of serving cells, the UE may assume that no transmission to the UE is present in PRBs and in symbols that are indicated by the DCI format 2_1, from a set of PRBs and a set of symbols of the last monitoring period. Wherein, the set of PRBs is equal to the active DL BWP and symbols indicated as uplink by TDD-UL-DL-ConfigurationCommon are excluded from the set of symbols of the last monitoring period. Similarly, we think the PRBs in the SBFD UL subband could also be excluded from the set of PRBs for indication the punctured PRBs with finer granularity.

### PDCCH

The following was concluded and captured in TR 38.858 for CORESET configuration with SBFD:

[TR 38.858 [1]] gNB can configure a CORESET and a search space in a way such that the MOs of the search space occur in either SBFD or non-SBFD symbols, or the MOs of the search space occur in both SBFD and non-SBFD symbols but the associated CORESET does not overlap the boundary of a DL subband in SBFD symbols. If it is agreed to be beneficial that a CORESET and a search space are configured that the MOs of the search space occur in both SBFD and non-SBFD symbols and the associated CORESET overlaps the boundary of a DL subband in SBFD symbols, at least the following options can be considered for SBFD-aware UE: - Option 1: Separate valid resources for the CORESET in SBFD symbols and in non-SBFD symbols. - Option 2: Rate matching or puncturing on the REG(s) of a PDCCH outside DL subband(s). - Option 3: UE does not monitor a PDCCH candidate if it is mapped to one or more REs that overlap with REs outside DL subband(s). - Option 4: Drop search space(s) when the associated CORESET overlaps with RBs outside DL subband(s) - Option 5: Separate search spaces associated with a CORESET in SBFD and non-SBFD symbols Note: These options are applicable to at least USS.

It is generally beneficial to enhance CORESET configuration by allowing one configuration to indicate two sets of resources for non-SBFD symbols and SBFD symbols. Not only this is beneficial in the sense of configuration flexibility, but also it allows configuring CORESETs that may occur on non-SBFD and SBFD symbols in different slots without using up the UE’s budget of the number of CORESET configurations. This enhancement can be introduced by allowing an additional resource indication in the CORESET configuration with minimum specification effort. For example, an additional Frequency Domain Resources parameter can be added to the Control Resource Set IE for SBFD symbols.

- Enhance CORESET configuration to include an additional frequency-domain indication for SBFD symbols.

For PDCCH reception, a frequency domain resource for a control resource set (CORESET) is indicated by a bitmap, where each bit of the bitmap corresponds to a group of 6 RBs and the grouping start from the first RB group in a BWP or a multicast broadcast service (MBS) common frequency resource (CFR) where the CORESET is configured. A bit that is set to 1 indicates that this RB group belongs to the frequency domain resource of this CORESET. Bits corresponding to a group of RBs not fully contained in the BWP within which the CORESET is configured are set to zero.

Depending on the number of RBs and the location of allocated RBs for a SBFD UL subband, some groups of 6 RBs may not be usable for CORESET resource allocation, as shown in Figure 2, which may cause frequency resource fragmentation. To alleviate the complexity of scheduling DL signals/channels, an enhancement for CORESET resource allocation can be considered. For example, an RB offset applicable to an SBFD symbol can be considered.

- SBFD UL subband may cause frequency resource fragmentation for CORESET resource on SBFD symbols.

- For CORESET frequency resource allocation in SBFD symbols, support RB offset to CORESET RB groups on SBFD symbols.

Figure 2: Full duplex UL subband allocation and a set of 6RB groups for CORESET resource allocation.

### CSI-RS

In the current specifications, a CSI-RS resource is allocated using a contiguous frequency domain resource allocation (FDRA) scheme in the form of a start RB and a number of RBs. The frequency bandwidth of a CSI-RS (i.e., the number of RBs) can be any value within the range of [24 RBs, 28 RBs, 32 RBs, …, 276 RBs], i.e., an integer multiple of 4 RBs, as small as 24 RBs and as large as 276 RBs. However, in some SBFD configurations, the total number of DL RBs within a SBFD slot/symbol are distributed over two non-contiguous DL subbands. Therefore, the current contiguous FDRA scheme cannot allocate a CSI-RS over the two non-contiguous DL subbands. To solve this issue, several options were identified in TR 38.858 [1], and the pros and cons of every option were captured as well.

[TR 38.858 [1] Section 6.1.2] Frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs are studied considering the following options: - Option 1: Two contiguous CSI-RS resources that are linked - Option 2: One CSI-RS resource - Option 2-1: Non-contiguous CSI-RS resource allocation - Option 2-2: One contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL subband (s) For all the options, there is no impact on CSI-RS sequence generation. Option 1 requires additional signalling to link two CSI-RS resources in two DL subbands. Option 2-1 requires new RRC structure to configure non-contiguous RBs for one CSI-RS resource, which may require additional signalling overhead. Option 2-2 can reuse the existing signalling design for CSI-RS resource configuration. Option 2-2 can be used to resolve the potential unaligned boundaries between CSI-RS resource configuration and SBFD subbands. Further discussion is required on the UE complexity due to UE capability of maximum number of configured CSI-RS resources and/or processing non-contiguous CSI-RS.

In RAN1#117 meeting, the following working assumption and agreement were reached regarding the frequency resource allocation of a CSI-RS across downlink subbands for SBFD-aware UEs.

Working Assumption For frequency resource allocation for CSI-RS across downlink subbands for SBFD-aware UEs, support one contiguous CSI-RS resource allocation with non-contiguous CSI-RS resource derived by excluding frequency resources outside DL usable PRBs. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured) - RAN1 to further study the impact on CSI processing timeline in SBFD symbols to process the CSI-RS across the two DL subbands

Agreement For a contiguous CSI-RS resource which overlaps with SBFD subband boundaries, only CSI-RS frequency resources within DL usable PRBs are valid for SBFD-aware. - No impact on CSI-RS sequence generation - CSI-RS sequence mapping is applied to CSI-RS resources within DL usable PRBs only (effectively, this is same as the case when the CSI-RS sequence mapped to the RBs outside the DL usable PRBs are punctured)

Effectively, the above working assumption is Option 2-2 identified in TR 38.858 [1], where a non-contiguous CSI-RS resource across downlink subbands for SBFD-aware UEs is derived by excluding the frequency resources (RBs) outside DL usable PRBs, which translates to puncturing the contiguous CSI-RS sequence mapped to the RBs outside the DL usable PRBs. However, depending on the allocated CSI-RS number of RBs, the UL subband number of RBs, and the two GBs number of RBs (if configured), it may happen that the number of “usable” RBs of the derived non-contiguous CSI-RS resource be less than the minimum number of 24 RBs required by current specs, as exemplified in Figure 4.

Figure 4: Graphical illustrations of Option 2-2 for frequency resource allocation for a CSI-RS resource across two downlink subbands for SBFD-aware UEs.

- With Option 2-2 for non-contiguous frequency-domain resource allocation of a CSI-RS resource, depending on the allocated CSI-RS number of RBs, UL subband number of RBs, and GBs number of RBs, it may happen that the number of “usable” RBs of the derived non-contiguous CSI-RS resource be less than the minimum number of 24 RBs required by current specs.

Moreover, Option 2-1 can also be supported and specified to provide more CSI-RS allocation flexibility across two downlink subbands. Note that, with Option 2-1, a non-contiguous CSI-RS resource can be formed by two contiguous CSI-RS resource parts each configured per DL subband and categorized by a start RB and a number of RBs.

- In addition to Option 2-2, RAN1 to support Option 2-1 identified in TR 38.858 for CSI-RS allocation across two downlink subbands to provide more CSI-RS allocation flexibility.

However, CSI-RS resource provided by Option 2-1 is mainly intended to SBFD symbols, and therefore it may not be directly used during non-SBFD symbols. Since single CSI-RS resource configuration across SBFD symbols and non-SBFD symbols is within the Rel-19 WI scope, how to enhance Option 2-1 to resolve this latter issue should be discussed.

- If Option 2-1 for non-contiguous frequency-domain resource allocation of a CSI-RS resource is supported, RAN1 to study solutions/enhancements so that the single CSI-RS resource configuration provided by Option 2-1 is applicable for both SBFD symbols and non-SBFD symbols.

### CSI Reporting

In RAN1#117 meeting, the following two agreements were reached regarding the CSI reporting for a CSI report associated with periodic/semi-persistent CSI-RS resource across SBFD and non-SBFD slots/symbols.

Agreement For CSI report associated with periodic/semi-persistent CSI-RS, discuss and decide whether to support the following options. - Option A: For separate CSI reports on SBFD and non-SBFD, one CSI-ReportConfig is associated with CSI-RS(s) restricted to SBFD symbols only and the second CSI-ReportConfig is associated with CSI-RS(s) restricted to non-SBFD symbols only. - gNB configuration may not ensure that the CSI-RS associated with each CSI-ReportConfig is confined to either SBFD symbols or non-SBFD symbols only. - For the CSI-ReportConfig associated with CSI-RS(s) restricted to SBFD symbols only, only CSI-RS transmission occasions within SBFD symbols are used for CSI derivation. - For the CSI-ReportConfig associated with CSI-RS(s) restricted to non-SBFD symbols only, only CSI-RS transmission occasions within non-SBFD symbols are used for CSI derivation. - Option B: Enhance Rel-18 NES CSI reporting framework to support one CSI-ReportConfig with one sub-configuration associated with SBFD symbols and the other sub-configuration associated with non-SBFD symbols

Agreement For a CSI reporting subband which overlaps with at least one PRB within DL usable PRBs and one or more PRBs outside DL usable PRBs, the CSI reporting subband includes PRB(s) within DL usable PRBs only.

In our opinion, due to none or minimum specs impact, Option A from the above agreement for CSI report associated with periodic/semi-persistent CSI-RS is sufficient and should be supported.

- For CSI report associated with periodic/semi-persistent CSI-RS, support Option A from RAN1#117 agreement.

### CLI-RSSI

In TR 38.858 [1], the following is stated concerning UE-to-UE CLI-RSSI measurement/report across downlink subbands.

[TR 38.858 [1] Section 6.2] For UE-to-UE CLI-RSSI measurement/report across downlink subbands, the following methods are studied. Note that Alt #1 and Alt #2 are supported in existing specifications. - Alt #1: separate CLI-RSSI measurement resources/reports in each DL subband - Alt #2: CLI-RSSI measure/report in one DL subband only - Alt #3: CLI-RSSI measurement/report based on non-contiguous CLI-RSSI resource across downlink subbands Alt #1 allows flexible configuration of measurement reporting in one DL subband or two DL subbands but it consumes multiple CLI-RSSI measurement resources from the UE capability budget. Alt #2 restricts gNB configuration flexibility and does not account for whether or not the CLI is asymmetric across two DL subbands. This method does not consume multiple CLI-RSSI measurement resources from UE capability point of view. Alt #3 requires additional specification efforts to support non-contiguous CLI-RSSI resource allocation across downlink subbands. This method is similar to non-contiguous CSI-RS resource allocation. A single CLI-RSSI report based on non-contiguous CLI-RSSI resource may be sufficient. This method does not consume multiple CLI-RSSI measurement resources from UE capability point of view. Note that it does not imply whether L1 or L2 based measurement is supported.

As captured in TR 38.858 [1], both Alt#1 and Alt#2 are supported in existing specifications. However, Alt#1 would double the CLI-RSSI measurement and reporting resources, while Alt #2 is based on an optimistic assumption that the CLI-RSSI measurements on the two DL subbands are symmetric, which is a weak assumption in practice, and we may end up over/under-estimating the actual CLI-RSSI level on the other DL subband. Comparing to the first two methods, Alt#3 has potential of obtaining accurate CLI-RSSI measurements, while at the same time reducing the number of CLI-RSSI measurements resources.

- For inter-UE CLI-RSSI measurement/report across downlink subbands, support Alt#3 of the agreement made in TR 38.858.

Alt#3: CLI-RSSI measurement/report based on non-contiguous CLI-RSSI resource across downlink subbands

## UL Transmissions

In addition to the time and frequency location of SBFD UL subband, information on subcarrier spacing, cyclic prefix (CP) type, and uplink configurations such as PUSCH, PUCCH, RACH, CG-PUSCH, and/or SRS configurations may also be included within the explicit configurations of SBFD UL subband. To limit the configuration signaling overhead, uplink configurations (e.g., PUCCH, PUSCH, SRS, RACH) of the SBFD UL subband can be provided by a bandwidth part (BWP) ID. Thus, the UE determines the uplink configurations from configurations of an UL BWP indicated by the BWP ID. Additionally, the subcarrier spacing and the CP type for the SBFD UL subband can be assumed to be the same as subcarrier spacing and CP type of the indicated UL BWP, if the subcarrier spacing and the CP type are not separately configured for the SBFD UL subband. However, in some scenarios, when a UE determines uplink configurations of an SBFD UL subband from configurations of an UL BWP indicated by a BWP ID, some resource configurations of the UL BWP may not be directly applicable to the SBFD UL subband. For example, if a CG PUSCH resource configured in the UL BWP indicated by the BWP ID is not confined within the SBFD UL subband, the corresponding CG PUSCH configuration is excluded for SBFD operation. In this case, a separate CG PUSCH configuration might be provided for CG transmission in the SBFD UL subband. Figure 8 shows an example SBFD UL subband configured in 2 slots within every 5 slots (i.e., a DL-UL pattern of periodicity of 5 slots). The uplink configurations of the SBFD UL subband are derived from UL BWP 1 configuration parameters.

- PUSCH/PUCCH/CG-PUSCH/RACH/SRS configurations in a UL subband for SBFD operation can be derived from an associated UL BWP (UL BWP ID). Separate CG PUSCH/PUCCH/RACH/SRS configuration(s) can be provided for the SBFD UL subband.

Figure 8: Full duplex UL subband overlapping with a DL BWP.

### PUSCH

In the last RAN1#116bis meeting, the following agreement related to CG PUSCH configuration without repetitions was reached ‎[2]. In our view, Option 1 provides the most flexibility and therefore should be supported.

- Support Option 1 for a CG PUSCH configuration without repetitions, where the transmission occasions are across SBFD symbols and non-SBFD symbols and each transmission occasion has either all SBFD or all non-SBFD symbols.

Option 1: Separate resource configurations for SBFD symbols and non-SBFD symbols.

Agreement For a CG PUSCH configuration without repetitions, if the transmission occasions are across SBFD symbols and non-SBFD symbols where each transmission occasion has either all SBFD or all non-SBFD symbols, discuss and decide whether/which of the following option(s) are supported. - Option 1: Separate resource configurations for SBFD symbols and non-SBFD symbols - FFS type 2 CG PUSCH - FFS other separate configurations for SBFD symbols and non-SBFD symbols - Option 2: Single resource configuration/indication for one symbol type (SBFD or non-SBFD symbol) and RB offset(s) configuration/indication/determination to determine resource for the other symbol type - Option 3: A CG PUSCH transmission occasion overlapping with RBs outside UL usable PRBs in SBFD symbols is invalid - Option 4: Only CG PUSCH transmission occasion in one symbol type is valid and CG PUSCH transmission occasion in the other symbol type is invalid - Option 5: Only the assigned PRBs within UL usable PRBs in SBFD symbols are considered to be valid for CG PUSCH Other options are not precluded

In legacy, for PUSCH repetition type A and TBoMS, the UE determines N∙K slots for a PUSCH transmission. If AvailableSlotCounting is enabled, a slot is not counted in the number of N∙K slots for PUSCH transmission of a PUSCH repetition Type A if at least one of the symbols indicated by the indexed row of the used resource allocation table in the slot overlaps with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst.

For dynamic grant (DG) or CG, some PUSCH transmissions of PUSCH repetition type A or TBoMS would be at symbols/slots with a full duplex UL subband and the other PUSCH transmissions would be in normal symbols/slots without the full duplex UL subband, the PUSCH transmission in SBFD symbols/slots could be outside UL subband. For DG, gNB could handle this case by limiting all the repetitions inside the UL subband, but for CG, the handling might be difficult, and limiting all repetitions in the UL subband is not efficient. Then how to determine the resources to transmit the PUSCH transmission of PUSCH repetition type A or TBoMS in SBFD slots and non-SBFD symbols/slots should be studied. Several options were listed in the following agreement made in the last RAN1#116bis meeting.

Agreement For PUSCH repetition type-A across SBFD symbols and non-SBFD symbols in different slots where each repetition has either all SBFD or all non-SBFD symbols, and for multi-PUSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols in different slots, where each PUSCH within a slot has either all SBFD or all non-SBFD symbols, and for TBoMS across SBFD symbols and non-SBFD symbols in different slots, where each transmission within a slot has either all SBFD or all non-SBFD symbols, discuss and decide whether/which of the following option(s) are supported. - Option 1: Separate FDRA configuration/indications/interpretations for SBFD symbols and non-SBFD symbols - Option 2: Single FDRA configuration/indication for one symbol type (SBFD or non-SBFD symbol) and RB offset(s) configuration/indication/determination to determine resource for the other symbol type - Option 3: A PUSCH in a slot overlapping with RBs outside UL usable PRBs in SBFD symbols is invalid, e.g. the PUSCH in the slot is dropped/postponed - Option 4: Only PUSCH in one symbol type is valid and PUSCH in the other symbol type is invalid - Option 5: For a PUSCH in a slot overlapping with RBs outside UL usable PRBs in SBFD symbols, only the assigned PRBs within UL usable PRBs are considered to be valid - Option 6: gNB does not schedule any PUSCH in SBFD symbols in a slot to be overlapping with PRBs outside UL usable PRBs - Other options are not precluded FFS: Applicable conditions

In our view, the following two potential enhancements can be considered.

- Option 1: For PUSCH repetition Type A or TBoMS transmission, if a PUSCH transmission in a slot is outside the UL subband, and overlaps with SBFD symbol or slot, the slot is not counted in the number of N∙K slots for PUSCH transmission if AvailableSlotCounting is enabled; otherwise, the PUSCH transmission in the slot is cancelled. This option is simple but has scheduling restrictions and resource utilization inefficiencies for PUSCH repetition Type A or TBoMS.

- Option 2: Two different frequency domain resource could be determined separately for the PUSCH transmissions of PUSCH repetition type A or TBoMS at SBFD and non-SBFD symbols/slots with the frequency domain resource for SBFD symbol/slot restricted to within the UL subband. As for how to determine two different frequency domain resources for the PUSCH transmissions of PUSCH repetition type A or TBoMS at SBFD and non-SBFD symbols/slots could be studied further. This option can provide higher resource utilization efficiency compared to Option 1.

Similar enhancements could be used for PUSCH repetition type B taking into consideration there could be two type of definition including nominal repetition and actual repetition. Therefore, we should determine the cancelled repetition is a nominal repetition or an actual repetition in option 1, and the two different frequency domain resource could be determined separately for nominal repetition or actual repetition in option 2.

- For PUSCH repetition type A/B and TBoMS across SBFD and non-SBFD symbols/slots, support Option 1 of the agreement made in RAN1#116bis meeting, where two frequency domain resources could be determined separately for SBFD and non-SBFD symbols/slots.

### PUCCH

Similarly, for PUCCH transmission with repetition, some of PUCCH transmissions might be in SBFD symbols and others may be in non-SBFD symbols, then how to determine the resources to transmit the PUCCH transmission in SBFD symbols and non-SBFD symbols should also be studied. For example, in case the PUCCH resources configured for the active UL BWP is within the frequency range of the UL subband, it can be considered to reuse the PUCCH resources for the PUCCH repetition in the UL subband. Otherwise, two PUCCH resource set could be configured or two PUCCH resource associated with same PUCCH index within one PUCCH resource could be considered for PUCCH transmissions in SBFD and non-SBFD symbols. Besides, resource determination for the cases of PUCCH frequency hopping and non-hopping should be considered.

- For PUCCH repetition across SBFD and non-SBFD symbols/slot, the PUCCH resources configured for the active BWP can be reused in case the resources are within the frequency range of the UL subband. Otherwise, two PUCCH resources could be determined separately for SBFD and non-SBFD symbols/slots.

### SRS

In the last RAN1#116bis meeting, the following agreement related to SRS were reached ‎[2].

Agreement Study the feasibility and enhancements to support separate power control and/or spatial relation for SRS, PUCCH and PUSCH in SBFD and non-SBFD symbols in different slots, including repetition and non-repetition, by considering existing schemes, e.g. multi-TRP PUCCH/PUSCH repetition schemes.

In our view, it is beneficial to enhance SRS configuration by allowing one configuration to indicate two sets of resources, frequency hopping parameters, and beams for non-SBFD symbols and SBFD symbols. The benefits include additional configuration flexibility and saving the UE’s budget of the number of SRS configurations by avoiding duplicate configurations for SRS on SBFD symbols. This enhancement can be introduced by allowing additional indications of resources, frequency hopping parameters, and beams with minimum specification effort.

- Enhance SRS configuration to include additional indications of resources, frequency hopping parameters, and beams.

### UL Power Control (PC)

In the last RAN1#117 meeting, the following agreement related to power control was reached.

Agreement For a single TRP scenario, support separate UL power control for PUSCH/PUCCH/SRS transmissions in SBFD symbols and non-SBFD symbols - Decide in RAN1#118 whether separate UL power control for FR2 is supported based on UL spatial relation Info framework and/or unified TCI state framework.

The motivation for having different/separate UL PC for PUSCH/PUCCH/SRS transmissions for different time slots is based on the assumption/expectation that the UL interference level observed by gNB during the SBFD slots would be higher than the UL interference level observed during the non-SBFD slots. In some scenarios, such an assumption/expectation is true when considering the fact that the number of UL interference sources during the SBFD slots is larger than that during the non-SBFD slots. In contrast to the non-SBFD slots, where the UL interference comes mainly from the UL transmissions of other UEs, SBFD slots have in addition both self-interference and inter-gNB CLI sources that are originated from the DL transmissions from the same gNB and the other (nearby and co-located) gNBs, respectively. With this in mind, it is beneficial to have separate UL PC parameters to enhance the UL transmission in different time slots, where the general assumption is that the UL PC parameters configured for SBFD slots will result in a higher UL PC level than that configured for non-SNFD slots, e.g., to compensate for the higher UL interference level. However, needless to say that the UL interference level observed by a gNB during a time slot is dependent on several factors, including the deployment scenario, the channel and traffic conditions, the coordination level between nearby gNBs, the adopted self-interference and inter-gNB CLI handling methods, etc, where some of these factors are up the gNBs decision and implementations.

- The UL interference level observed by a gNB during a time slot is highly dependent on the deployment scenario, the channel and traffic conditions, the coordination level between nearby gNBs, as well as the adopted self-interference and inter-gNB CLI handling methods.

From the above agreement, it is not clear whether the separate UL power control for PUSCH/PUCCH/SRS transmissions are provided/configured dynamically or semi-statically. With the dynamic configurations of UL PC parameters, the gNB can dynamically adjust the configured UL PC parameters for a time slot based on, e.g., the expected and/or measured self-interference and inter-gNB CLI at the gNB, at the expense of high configuration overhead. However, with the semi-static configurations of UL PC parameters, the UE may end up using a higher UL PC level than required on a SBFD time slot, e.g., due to the change on the expected and/or measured self-interference and inter-gNB CLI at the gNB. To provide the best of two approaches, we propose to configure the separate UL power control for PUSCH/PUCCH/SRS transmissions in a semi-static manner but adjust the association between a configured UL power control parameter/set and a slot/symbol type dynamically.

- Support semi-static configuration of separate UL power control for PUSCH/PUCCH/SRS transmissions, with dynamic association between a configured UL power control parameter set and a symbol type.

# UE Collision Handling

Besides the possible mismatch between configured resource for a CG PUSCH and the SBFD UL subband described in section 6, collision may also happen between UL/DL in time domain, for example, between the UL transmission in SBFD UL subband and DL reception in the DL subband within SBFD symbols, especially when both the UL transmission and the DL reception are configured by RRC message. In this case, potential enhancements should be studied on determining the channels or signals to be transmitted/received. In the last meeting, the following agreement is reached regarding collisions between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol for SBFD-aware UEs.

Agreement: For SBFD-aware UEs, collisions between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol may be addressed or alleviated with proper scheduling. The following cases of potential collisions, [if link direction indication is not supported or provided], can be further studied to see if any change to the current specs is necessary: - Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH - Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH - Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission - Case 5: SSB vs. dynamically scheduled or configured UL transmission - e.g., PUSCH, PUCCH, PRACH, SRS - Case 6: Dynamic or semi-static DL vs. valid RO Note: In addition to collision between UL transmission and DL reception in the same SBFD symbol(s), collision between UL transmission and DL reception in different symbol(s) due to lack of sufficient transition time between Tx/Rx at UE side is also included.

In RAN1#116bis meeting, the following agreements related to collision Case 1 and Case 2 were reached ‎[2].

Agreement If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 1 (dynamically scheduled DL reception vs. semi-statically configured UL transmission) in the SBFD symbol for SBFD-aware UEs, reuse the existing collision handling principles and timeline in NR for operation on flexible symbols on a single carrier in unpaired spectrum, i.e. UL transmission is cancelled if cancellation timeline is met. - The above does not imply link direction indication is supported - FFS on dynamically scheduled DL reception with repetition

Agreement If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 2 (semi-statically configured DL reception vs. dynamically scheduled UL transmission) in the SBFD symbol for SBFD-aware UEs, reuse the existing collision handling principles in NR for operation on flexible symbols on a single carrier in unpaired spectrum, i.e. UE does not receive DL channel/signal. - The above does not imply link direction indication is supported FFS on dynamically scheduled UL transmission with repetition

In RAN1#117 meeting, the following agreements related to collision Case 3 and Case 4 were reached.

Agreement If link direction indication is not supported nor provided for a SBFD symbol, for collision Case 4 (dynamically scheduled DL reception vs. dynamically scheduled UL transmission) in a SBFD symbol for SBFD-aware UEs, it is considered as an error case if a dynamically scheduled DL reception in DL subband(s) without repetitions overlaps with a dynamically scheduled UL transmission in UL subband without repetitions. - FFS DL reception with repetitions and/or UL transmission with repetitions

Agreement If link direction indication is not supported nor provided for a SBFD symbol, collision Case 3 (semi-statically configured DL reception vs. semi-statically configured UL transmission) in the SBFD symbol for SBFD-aware UEs is an error case. - An SBFD-aware UE does not expect to receive both dedicated higher layer parameters configuring transmission in UL usable PRBs from the UE and dedicated higher layer parameters configuring reception in DL usable PRBs in the SBFD symbol - An SBFD-aware UE does not expect to receive both dedicated higher layer parameters configuring transmission in UL usable PRBs from the UE and cell specific higher layer parameters configuring reception in DL usable PRBs in the SBFD symbol - Cell-specifically configured DL reception refers to PDCCH in Type-0/0A/1/2 CSS set - FFS: Additional cases

Agreement Re-use the existing collision handling principles for NR TDD that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission - FFS whether a slot consisting of SSB symbols is considered as a full DL slot or SSB symbols configured with SBFD subbands are SBFD symbols and only DL receptions within DL usable PRBs are allowed for SBFD aware UEs.

In our view, further enhancements can be made in the case of UE DL-UL collision, e.g., to determine the channels or signals to be transmitted/received.

- RAN1 to study potential enhancements on determining the channels or signals to be transmitted/received in the case of UE DL-UL collision.

# Conclusions

In summary, we propose the followings for Rel-19 NR subband non-overlapping full duplex operation:

- RAN1 to support SIB1-based signaling of time and frequency locations of SBFD subbands.

- In the scenario where a UE is configured with two TDD UL/DL patterns and the SBFD symbols are configured for both patterns, RAN1 to study the option where a single SBFD pattern is provided for the two TDD UL/DL patterns.

- Support Option 1 regarding link direction determination for a SBFD-aware UE transmission and reception in an SBFD symbol.

Option 1: UE determines link direction based on configured/scheduled transmissions/receptions and collision handling (if any).

- Indicating the number of RBs/RBGs of an UL subband as a percentage value of, e.g., the total number of RBs/RBGs within the target BWP requires, in general, less indication overhead.

- The semi-static indication of frequency location of SBFD UL subband is indicated explicitly via a start RB/RBG and a number of RBs/RBGs values and the number of RBs/RBGs is indicated as a percentage value of a total number of RBs/RBGs.

- For UL transmissions and DL receptions across SBFD symbols and non-SBFD symbols in different slots, support Configuration 2 captured in a RAN1#117 meeting agreement.

- For PDSCH reception with non-interleaved downlink type 1 resource allocation, support Option 1-1, where a SBFD aware UE performs rate matching around the SBFD UL subband.

- For PDSCH reception with interleaved downlink type 1 resource allocation, support Option 2, where a SBFD aware UE indexes VRBs and defines resource block bundles based on resource blocks not overlapping with the SBFD UL subband.

- For an SPS PDSCH configuration without repetitions, Support Option 4 in the agreement made in RAN1#116bis meeting.

Option 4: Only SPS PDSCH reception occasion in one symbol type is valid and SPS PDSCH reception occasion in the other symbol type is invalid.

- For PDSCH repetitions and for multi-PDSCH scheduled by a single DCI across SBFD symbols and non-SBFD symbols, where each repetition/PDSCH within a slot has either all SBFD or all non-SBFD symbols, Option 5 in the above agreement should be supported.

Option 5: For a PDSCH in a slot overlapping with RBs outside DL usable PRBs in SBFD symbols, only the assigned PRBs within DL usable PRBs are considered to be valid.

- For SBFD-aware UEs and PRG(s) with size of 2 and 4 that overlaps with subband boundary, the actual number of consecutive PRBs in each PRG can be one or more and not always equal to the configured precoding granularity as already supported in current TS 38.214 specification.

- For SBFD-aware UEs configured with fdmschemeB and two TCI states for a PDSCH transmission, if the allocated resource overlaps with SBFD UL subband boundary, the actual PRBs for each PDSCH occasion corresponding to each TCI state should be determined.

- Enhance CORESET configuration to include an additional frequency-domain indication for SBFD symbols.

- SBFD UL subband may cause frequency resource fragmentation for CORESET resource on SBFD symbols.

- For CORESET frequency resource allocation in SBFD symbols, support RB offset to CORESET RB groups on SBFD symbols.

- With Option 2-2 for non-contiguous frequency-domain resource allocation of a CSI-RS resource, depending on the allocated CSI-RS number of RBs, UL subband number of RBs, and GBs number of RBs, it may happen that the number of “usable” RBs of the derived non-contiguous CSI-RS resource be less than the minimum number of 24 RBs required by current specs.

- In addition to Option 2-2, RAN1 to support Option 2-1 identified in TR 38.858 for CSI-RS allocation across two downlink subbands to provide more CSI-RS allocation flexibility.

- If Option 2-1 for non-contiguous frequency-domain resource allocation of a CSI-RS resource is supported, RAN1 to study solutions/enhancements so that the single CSI-RS resource configuration provided by Option 2-1 is applicable for both SBFD symbols and non-SBFD symbols.

- For CSI report associated with periodic/semi-persistent CSI-RS, support Option A from RAN1#117 agreement.

- For inter-UE CLI-RSSI measurement/report across downlink subbands, support Alt#3 of the agreement made in TR 38.858.

Alt#3: CLI-RSSI measurement/report based on non-contiguous CLI-RSSI resource across downlink subbands

- PUSCH/PUCCH/CG-PUSCH/RACH/SRS configurations in a UL subband for SBFD operation can be derived from an associated UL BWP (UL BWP ID). Separate CG PUSCH/PUCCH/RACH/SRS configuration(s) can be provided for the SBFD UL subband.

- Support Option 1 for a CG PUSCH configuration without repetitions, where the transmission occasions are across SBFD symbols and non-SBFD symbols and each transmission occasion has either all SBFD or all non-SBFD symbols.

Option 1: Separate resource configurations for SBFD symbols and non-SBFD symbols.

- For PUSCH repetition type A/B and TBoMS across SBFD and non-SBFD symbols/slots, support Option 1 of the agreement made in RAN1#116bis meeting, where two frequency domain resources could be determined separately for SBFD and non-SBFD symbols/slots.

- For PUCCH repetition across SBFD and non-SBFD symbols/slot, the PUCCH resources configured for the active BWP can be reused in case the resources are within the frequency range of the UL subband. Otherwise, two PUCCH resources could be determined separately for SBFD and non-SBFD symbols/slots.

- Enhance SRS configuration to include additional indications of resources, frequency hopping parameters, and beams.

- The UL interference level observed by a gNB during a time slot is highly dependent on the deployment scenario, the channel and traffic conditions, the coordination level between nearby gNBs, as well as the adopted self-interference and inter-gNB CLI handling methods.

- Support semi-static configuration of separate UL power control for PUSCH/PUCCH/SRS transmissions, with dynamic association between a configured UL power control parameter set and a symbol type.

- RAN1 to study potential enhancements on determining the channels or signals to be transmitted/received in the case of UE DL-UL collision.

# References

3GPP TR 38.858 V18.0.0 (2023-12), “Study on Evolution of NR Duplex Operation”

RAN1#116bis Chair’s Notes.