3GPP TSG RAN WG1 #118                                                                                       R1-2406811

Maastricht, NL, August 19th – 23rd, 2024

Agenda Item:	9.4.1.1

Source:	Lenovo

Title:	Discussion on the evaluation assumptions for Ambient IoT devices

Document for:	Discussion

# Introduction

In RAN#103 meeting, for Rel-19 study item of ambient IoT, the relevant scopes have been revised in RP-240826 [1].

This contribution provides discussion into the Including assumptions on coverage and coexistence evaluations, link budget calculations, and remaining design targets of TR 38.848 for Ambient IoT devices.

# Ambient IoT use cases and KPIs for RAN design targets.

## Ambient IoT use cases.

TSG RAN#101 has completed a Rel-18 RAN-level study item on Ambient IoT with the following four use cases captured in TR 38.848:

- Inventory control

- Sensor data

- Positioning

- Actuator command

However, the inventory and actuator command has been prioritized in Ambient IoT Rel19 SID at TSG RAN#102 and their use case, applicable scenarios and KPIs from SA1 TS 22.369 are provided below for reference.

Inventory: The purpose is to discover what goods (e.g. boxes, containers, packages, tools) are present in a specific area. Upon request sent by the network within the specific area, Ambient IoT devices attached to these goods report an identifier associated with the good, possibly supplemented with other information such as status, measurement results and/or location.

The representative use cases related to inventory from SA1 TR 28.840 is provided below for reference.

| rUC (representative use case) | Applicable SA1 UCs / traffic scenarios |
| --- | --- |
| rUC1: Indoor inventory | 5.1 Automated warehousing 5.2 Medical instruments inventory management and positioning 5.4 Non-Public Network for logistics 5.5 Automobile manufacturing 5.7 Airport terminal / shipping port 5.15 Smart laundry 5.16 Automated supply chain distribution 5.18 Fresh food supply chain 5.27 End-to-end logistics 6.1 Flower auction 6.3 Electronic shelf label |

Table 1: Representative SA1 use case for Inventory.

| Message size | 96/256 bits |
| --- | --- |
| User experienced data rate | < 2 kbit/s |
| Communication rage, Indoor | 30-50m |
| Device speed | 3 km/h – 10 km/h |
| Maximum allowed end to end latency | Several seconds |
| Communication service availability | 99% |
| Device density | <1.5 million devices/km² |
| Service area dimension | 1 km² – 10 km² |

Table 2: KPIs for Inventory

Actuator command. With actuator control, the Ambient IoT device is associated with an actuator. Transfer of actuator commands is generally initiated by the network.

| rUC (Representative use case) | Applicable SA1 UCs / traffic scenarios |
| --- | --- |
| rUC4: Indoor command | 5.11 Online modification of medical instruments status 5.17 Device activation and deactivation 5.26 Elderly Health Care 5.29 Device Permanent Deactivation 6.3 Electronic shelf label |

Table 3: Representative SA1 use case for actuator command.

| Message size | < 100 Bytes |
| --- | --- |
| User experienced data rate | 2 kbit/s |
| Communication rage, Indoor | 50m |
| Device speed | Stationary |
| Maximum allowed end to end latency | Several seconds |
| Communication service availability | 99% |
| Device density | < 1.5 million devices/km² |
| Service area dimension | < 250 m² for home, and 15,800 square meters for supermarket |
| Transfer interval | 20 mins - 120 mins |

Table 4: KPIs for actuator command

## RAN design target.

### RAN Device categorization.

In TSG RAN#102, two types of device category containing different device power consumption was agreed to be studied in Rel19 Ambient IoT SI.

- Type 1: ~1 µW peak power consumption, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, neither DL nor UL amplification in the device. The device’s UL transmission is backscattered on a carrier wave provided externally.

- Type 2: ≤ a few hundred µW peak power consumption1, has energy storage, initial sampling frequency offset (SFO) up to 10X ppm, both DL and/or UL amplification in the device.

  - Type 2A: Device’s UL transmission may be generated internally by the device (active)

  - Type 2B: Backscattered on a carrier wave provided externally (semi-passive).

Proposal 1: Consider the candidate target peak power consumption for the passive Ambient IoT device type 2B containing amplification and storage between 300 to 500 µW.

Target power consumption of LP-WUS [5] is 1mW and hence the active Ambient IoT device power consumption target can less than 1mW.

Proposal 2: Consider the candidate target peak power consumption for the active Ambient IoT device type 2A containing amplification and storage between 500 to 1000 µW.

Initial sampling frequency error varies according to the device type due to the different receiver architecture such as RF ED, IF ED, ZIF, etc. In general, for passive devices (device 1/ device 2a) that are equipped low-cost receiver, the SFO can be between 10^4 - 10^5, while for active device type (device 2b), SFO can be assumed between 10^3 – 10^4.

Proposal 3: For evaluating Ambient IoT, for example for synchronization evaluation, consider different initial sampling frequency offset based on the device type and the supported receiver architecture.

- Device type 1 and Device type 2a : 10^4 - 10^5 ppm

- Device type 2b: 10^3 - 10^4 ppm

### Message size/TB size:

The message sizes larger than about 1000 bits is provided in TR 38.848 and while in the inventory use case, the electronic product code (EPC) minimum memory size is 96 bits which can go up to 128 bits, while the tag identifier ROM memory (TID) which contains the manufacturer and chipset information with length from 32-80 bits. The optional user memory length to store more data can be up to 512 bits. The reserved memory space has 32 bits length containing password to access and kill passive tags. So, the total message size for the inventory use case is EPC + TID + reserved memory + user memory which amounts 600 to 800 bits and it is stored in different memory regions as explained above.

Figure 1: Memory bank of RFID

In the actuator command, the message size is about 100 bytes/800 bits long. For comparison, the maximum TBS size in NB-IoT is 1000 bits and considering that the Ambient IoT device type is an order of magnitude smaller than that of the NB-IoT, the maximum TBS size is order of magnitude smaller compared to the NB-IoT. The maximum TBS for inventory use case is chosen according to the transmission of the EPC bits without segmentation.

Proposal 4: For evaluating Ambient IoT, consider candidate maximum TBS for UL transmission:

- 100-150 bits for Passive device Types 1, 2B

- 200-250 bits for Active device Type 2A

### Latency

The TR 38.848 mentions short latency target of 1 sec and long latency target of 10 sec and the charging time is not included in the latency budget. The inventory and the actuator command use case latency are shown as several seconds in SA1 TS 22.269 and hence the longer latency target is more appropriate for the Ambient IoT devices being investigated in this SID.  The Ambient IoT devices dissipates energy while participating in the inventory process and hence the energy harvesting time and charging capacity for group of devices within the inventory round needs to be evaluated.

Proposal 5: Consider long latency target of 10 seconds considering latency of inventory and actuator command use case requirement is provided as several seconds.

- Evaluate the sustainable operation time, energy harvesting, different scheduling mechanism within the inventory process and its impact on latency

Proposal 6: The following performance metric is considered for evaluation purpose only,

- Inventory completion time for multiple A-IoT devices [s]

- For inventory use case, the  ‘Inventory completion time for multiple A-IoT devices’ is defined as the time a reader successfully read completed the inventory process for [Z]% of A-IoT devices for a given number of reachable A-IoT devices within corresponding coverage by the reader

- FFS: Z = {99%(Mandatory), 90%(Optional)}

Proposal 7: The assumption to be considered for the invention completion time evaluation includes:

- Duration of the random access round (ms)

- Device distribution

- Number of Devices

- Message size

- Capacitor sizes

- Impact of sustainable operation time of the device including RF energy harvesting and the related component such as rectifier resistances, capacitance sizes, initial stored energy etc.,

- Power consumption for Tx, sleep, Rx etc.,  for each device type

- Scheduling methodology e.g., slotted Aloha etc.,

.

### Connection density

The connection density in TR 38.848 is 150 devices per 100m2 for indoor scenarios. Assuming an Ambient IoT devices attached to a wooden pallet in a warehousing. The devices are distributed symmetrically with 0.8m horizontal placement and then assuming three vertical racks for pallet placement with each vertical rack of 1.5m vertical height. Usually, subset of device population is selected for inventory round thereby preventing collision from devices that need not be inventorized. RAN1 should also discuss the number of Ambient IoT devices participating in an inventory round or number of Ambient IoT devices to be inventorized in a given area.

Figure 2: Illustrating 2D distribution of pallets in a warehousing.

Proposal 8: RAN1 should evaluate the number of devices to be inventorized in a given area in an inventory round, considering

- Collision due to the number of devices participating in an inventory round.

- Target latency considering the duty cycle within an inventory round.

## Evaluation of sustainable operation time within an inventory round

The RAN#103 clarified to study energy harvesting on device availability for transmission and reception procedures and the charging time due to energy harvesting can be assumed up to several tens of seconds. The factors affecting the RF harvesting of the circuitry such as circuit resistances, the received power, and rectifier efficiency.

At the beginning of the inventory process, it is not possible to assume that every Ambient IoT device is fully charged and able to withstand entire inventory round with the stored energy.  Hence, the Ambient IoT device might need to harvest energy to sustain the Tx/Rx operation within the inventory round. Also, the Ambient IoT device might need to harvest energy to sustain the Rx operation outside the inventory round to regularly monitor for the inventory request command from the reader. Since the harvesting is integral part of the Ambient IoT device, hence the minimum capacitance size and the resistance for harvesting should be defined as part of the evaluation.

The Ambient IoT device can periodically wake up to monitor for the Query command within the inventory round and once the inventory of the Ambient IoT device is finished, the Ambient IoT may sleep until the end of the inventory round. Hence, the power consumption of the Ambient IoT device should take into consideration on the periodic Rx, synchronization. Also, the power consumption of transmitting RACH and EPC ID by the Ambient IoT device must be take into consideration within the inventory round. The Ambient IoT device might need to maintain minimum power consumption within the inventory round to maintain the RAM memory.

Figure 8: Illustration of Duty cycle based operation of Ambient IoT device in an inventory round

The consumption of power state for receiver and periodic synchronization is around 55µW and sleep state power consumption to maintain RAM memory is 0.5µW per query round, where each query round is 10ms. The Tx power consumption is around 500µW which includes the 2 transmission opportunities for random access and EPC ID transmission and 2 reception opportunities to receive configuration.

Proposal 9: Evaluate the power consumption of the Ambient IoT device within a inventory round considering duty cycle-based operation,

- Periodic Rx and synchronization

- Minimum sleep state to maintain the RAM memory

- Tx operation for transmitting random access and EPC ID

The successful inventory completion time of the Ambient IoT devices is defined as the probability of z% Ambient IoT devices to receive the inventory command, periodic query command and transmit the EPC ID within the inventory latency budget. The non-availability of enough energy from the capacitor to sustainably operates the Ambient IoT device within an inventory round may lead to Ambient IoT device outage. Such outage probability considering the non-availability of energy to transmit the EPC ID within an inventory round of X devices should be defined for evaluation.

The received power for RF harvesting depends on the distance of the device from the emitter/reader. Figure x evaluates the outage probability of Ambient IoT devices considering the received power for various capacitance sizes.

Figure 9: Sustainable operation time of device with duty cycle and w/o duty cycle, distance = 12m

The device availability and device monitoring procedure to monitor wake-up, trigger etc., needs to be evaluated in RAN1 to determine the sustainable operation time of an Ambient IoT device by comparing its operation with and without duty-cycle based operation, power model of transmission, reception, sleep/harvesting for different device types and the impact on the incident received power level on PCE and the charging time leading to Ambient IoT device to start an inventory round with different stored energy. Since the harvesting is an integral part of the Ambient IoT device estimating the sustainable operation time of a device with a capacitance storage size, received incident power, and rectifier efficiency to withstand an inventory round using duty cycle-based operation is important.

Observation 1:  The minimum capacitance size to sustainably operate the device within an inventory round varies with the received power i.e., E2H link budget.

Proposal 10: Consider the outage probability as non-availability of energy from the capacitor to sustainably operates the Ambient IoT device within an inventory round to transmit EPC ID.

Proposal 11: Consider the rectifier efficiency as a function of received power for storing the harvested energy in device capacitor.

In figure 10, a simplified Ambient IoT device dropping model considering a single cell scenario with 100 devices uniformly distributed in 20m x 20m square indoor deployment can be considered to evaluate the sustainable operation of Ambient IoT device and RF energy harvesting within an inventory round.

Figure 10: Simplified dropping model for a single cell scenario for RF energy harvesting evaluation

The received power from the E2H link budget is calculated for each of the Ambient IoT devices based on the distance, transmit power and gain of the reader and losses summarized in the following table.

| Modulation factor | Fade Margin (dB) | On-object penalty (dB) | Polarization Mismatch (dB) | TxP (dBm) | Antenna Gain BS (dBi) | Antenna Gain Device (dBi) | Activation threshold (dBm) | Cable Loss (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 3 | 0.9 | 3 | 33 | 6 | 0 | -35 | 3 |

Table 8: Parameters used for E2H link budget

In this evaluation, the outage probability of 100 Ambient IoT devices using the slotted aloha scheme within an inventory round is calculated for different capacitance sizes. Considering the deployment scenario in figure 10, the received power of all devices is within the activation threshold of the devices. The received power of most of the devices is between -10dBm and -30dBm within the 20x20 grid. The outage probability of 100 devices distributed according to the above-mentioned deployment model as a function of capacitor sizes using slotted aloha scheduling is shown in Figure 11.

Figure 11: Shows the outage  probability as a function of capacitance sizes considering slotted Aloha

Each of these devices selects a random occasion for transmission within an inventory round of 1second. The required minimum capacitance to sustainably operate the Ambient IoT device within 1sec inventory round can be calculated by considering the power consumption due to the scheduling latency is around 15µF.

Observation 2: The required minimum capacitance size to sustainably operate the Ambient IoT device in a slotted Aloha scheme is 15µF.

Proposal 12: Compare the always-on scenario Vs duty cycle-based operation and study to extend the sustainable operation thereby lowering the required capacitance size of Ambient IoT device.

The device with low received power needs to be scheduled earlier to operate sustainably without any outage probability. However, the slotted aloha scheme of randomly scheduling the devices in an inventory round does not take into consideration the received power nor the available energy at the capacitor. The scheduling of such Ambient IoT devices can be sorted according to the received power and the available energy at the capacitor to minimize the outage probability. Such mechanism also relaxes the minimum required capacitance size (for example, from 15µF to 5µF) to sustainably operate the device without outage. Figure 12, shows the comparison on the outage probability between the random slotted aloha and energy aware scheduling for 100 devices.

Figure 12: Shows the comparison of outage for different scheduling mechanism for Ambient IoT devices

Observation 3:  The minimum required capacitance size to achieve certain outage probability can be relaxed using energy aware scheduling.

Proposal 13: Consider studying scheduling of Ambient IoT device by taking into consideration the available energy at the capacitor and the received power.

The sustainable operation is the ability of the devices to operate successfully without going into outage within an inventory round. In Figure 13, the sustainable operation of the device before the start of the inventory assumes that the capacitance charge is full and then evaluation shows the sustainable operation time duration for various capacitance sizes. The distance of the node from the emitter is varied to show the effect on the sustainable operation time. As the distance increases, the received power decreases and thus affecting the operation of these devices. The Figure 13, also compares the sustainable operation time with and without energy harvesting to show the positive impact of the energy harvesting on the sustainable operation time of these device.

Figure 5: Shows sustainable operation time of Ambient IoT devices with different distances and for various capacitance values.

Observation 4: Sustainable operation time of the device is defined as the time duration of the Ambient IoT devices to operate successfully within an inventory round without going into outage and the sustainable operation time of a device varies with the distance from the emitter.

Observation 5: Energy harvesting having positive impact on the sustainable operation time of the Ambient IoT device.

Proposal 14: Evaluate the sustainable operational duration of Ambient IoT devices with and without Energy harvesting within an inventory round.

# Conclusion

Below is the summary of proposals and observations from our contribution.

Proposal 1: Consider the candidate target peak power consumption for the passive Ambient IoT device type 2B containing amplification and storage between 300 to 500 µW.

Proposal 2: Consider the candidate target peak power consumption for the active Ambient IoT device type 2A containing amplification and storage between 500 to 1000 µW.

Proposal 3: For evaluating Ambient IoT, for example for synchronization evaluation, consider different initial sampling frequency offset based on the device type and the supported receiver architecture.

- Device type 1 and Device type 2a : 10^4 - 10^5 ppm

- Device type 2b: 10^3 - 10^4 ppm

Proposal 4: For evaluating Ambient IoT, consider candidate maximum TBS for UL transmission:

- 100-150 bits for Passive device Types 1, 2B

- 200-250 bits for Active device Type 2A

Proposal 5: Consider long latency target of 10 seconds considering latency of inventory and actuator command use case requirement is provided as several seconds.

- Evaluate the sustainable operation time, energy harvesting, different scheduling mechanism within the inventory process and its impact on latency

Proposal 6: The following performance metric is considered for evaluation purpose only,

- Inventory completion time for multiple A-IoT devices [s]

- For inventory use case, the  ‘Inventory completion time for multiple A-IoT devices’ is defined as the time a reader successfully read completed the inventory process for [Z]% of A-IoT devices for a given number of reachable A-IoT devices within corresponding coverage by the reader

- FFS: Z = {99%(Mandatory), 90%(Optional)}

Proposal 7: The assumption to be considered for the invention completion time evaluation includes:

- Duration of the random access round (ms)

- Device distribution

- Number of Devices

- Message size

- Capacitor sizes

- Impact of sustainable operation time of the device including RF energy harvesting and the related component such as rectifier resistances, capacitance sizes, initial stored energy etc.,

- Power consumption for Tx, sleep, Rx etc.,  for each device type

- Scheduling methodology e.g., slotted Aloha etc.,

Proposal 8: RAN1 should evaluate the number of devices to be inventorized in a given area in an inventory round, considering

- Collision due to the number of devices participating in an inventory round.

- Target latency considering the duty cycle within an inventory round.

Proposal 9: Evaluate the power consumption of the Ambient IoT device within a inventory round considering duty cycle-based operation,

- Periodic Rx and synchronization

- Minimum sleep state to maintain the RAM memory

- Tx operation for transmitting random access and EPC ID

Observation 1:  The minimum capacitance size to sustainably operate the device within an inventory round varies with the received power i.e., E2H link budget.

Proposal 10: Consider the outage probability as non-availability of energy from the capacitor to sustainably operates the Ambient IoT device within an inventory round to transmit EPC ID.

Proposal 11: Consider the rectifier efficiency as a function of received power for storing the harvested energy in device capacitor.

Observation 2: The required minimum capacitance size to sustainably operate the Ambient IoT device in a slotted Aloha scheme is 15µF.

Proposal 12: Compare the always-on scenario Vs duty cycle-based operation and study to extend the sustainable operation thereby lowering the required capacitance size of Ambient IoT device.

Observation 3:  The minimum required capacitance size to achieve certain outage probability can be relaxed using energy aware scheduling.

Proposal 13: Consider studying scheduling of Ambient IoT device by taking into consideration the available energy at the capacitor and the received power.

Observation 4: Sustainable operation time of the device is defined as the time duration of the Ambient IoT devices to operate successfully within an inventory round without going into outage and the sustainable operation time of a device varies with the distance from the emitter.

Observation 5: Energy harvesting having positive impact on the sustainable operation time of the Ambient IoT device.

Proposal 14: Evaluate the sustainable operational duration of Ambient IoT devices with and without Energy harvesting within an inventory round.

# References

RP-240826, “revised SID: Study on solutions for Ambient IoT (Internet of Things) in NR”, RAN#103, March. 2024.

TR38.848, Study on Ambient IoT (Internet of Things) in RAN

Joshua D. Griffin, et. al, Complete Link Budgets for Backscatter-Radio and RFID Systems