3GPP TSG RAN WG1 #118			R1-2407138

Maastricht, NL, August 19th – 23rd, 2024

Agenda Item:	9.11.4

Source:	Viasat, Inmarsat

Title:	Views on uplink capacity enhancement for IoT NTN

Document for:	Discussion and decision

# Introduction

During RAN1#117, the following agreements were reached:

Agreement

For 3.75kHz single-tone OCC for NPUSCH format 1, RAN1 supports either symbol-level OCC or slot-level OCC. Other OCC schemes are not pursued.

For 15kHz single-tone OCC for NPUSCH format 1, RAN1 supports either symbol-level OCC or slot-level OCC. Other OCC schemes are not pursued.

Agreement

Inter-repetition OCC for NPRACH is not studied further in RAN1.

Agreement

- For the time-domain DMRS pattern (including blanked DMRS, if any):

  - For 15kHz single-tone, RAN1 strives to reuse the Rel-17 DMRS pattern

  - For 3.75kHz single-tone

    - RAN1 studies

      - Rel-17 DMRS pattern

      - A new DMRS pattern

  - The DMRS overhead (including blanked DMRS, if any) for OCC is the same as for Rel-17

In this paper we present Viasat’s views on UL Capacity Enhancements for IoT NTN, as an NTN operator with live NB-IoT NTN networks deployed in the field.

It should be noted that there are a number of commonalities in observations that can be made about OCC for IoT NTN and NR NTN at least in light of upcoming high-performance NGSO systems.  Thus, this paper shares a number of observations with our corresponding paper on NR NTN (R1-2407139).

# Discussion

## Background

During RAN1#117 and preceding RAN1 meetings, as well as during the scope revision discussions at RAN#104, whether to include support for both 3.75 kHz and 15 kHz SCS was the subject of heavy discussions.

Similarly, whether the scope should include support for multi-tone, as well as single tone, was also the subject of heavy discussions.

In this paper, we would like to provide an NTN operator’s perspective on the importance of including support for both SCS, as well as both single-tone and multi-tone transmission schemes and their applicability to current GEO/GSO systems already deployed and operational as well as applicability to future upcoming NGSO systems.

## Current GEO systems and 3.75 kHz and 15 kHz SCS support

NB-IoT NTN has been the first NTN technology to be deployed to the market, even before Rel-17 work had been completed, over existing GEO/GSO space segment infrastructure in FR1.  These systems are currently already deployed live, and support real traffic.

The systems currently deployed support applications ranging from smartphone emergency SOS, beacon, tracking and messaging, to IoT, dedicated satellite phones, UAV tracking , command and telemetry and many more, in both mobile, semi-mobile and fixed scenarios.

These applications are characterized by UE of different types, different antenna performance and characteristics, different behaviors and dynamic link conditions, experiencing very different patterns of link fading and multipath.  This diversity of applications, including the support of Smartphone devices, is unprecedented for NB-IoT networks, and has required a different approach to the deployment and operation of the NB-IoT system.

Different NB-IoT Uplink transmission schemes and sub-carrier spacings, such as 3.75 kHz single tone, 15 kHz single tone and 15 kHz multi-tone offer different advantages and disadvantages, that have to often be mixed together in order to achieve optimal user performance and efficient system utilization.

For example, 3.75 kHz transmission schemes offer very good link UL link budget and higher channel multiplexing capacity (up to 48 users), but require significantly longer transmission times due to the longer Resource Unit (RU) duration.   This makes them very well suited for challenging link budgets of low-power, bursty and massive IoT applications, especially for fixed IoT devices, but can present issues when adopted in more dynamic environments, such as mobile assets, due to the high probability of channel fading affecting the link during the duration of the UL transmission.

15 kHz SCS, on the other hand, are better suited to higher power devices and mobile environments where the link might experience fast intermittent fading.

Current live NB-IoT NTN systems implement both schemes.  The cell capacity advantages gained by the application of OCC should be applicable to both schemes, to allow operators the freedom to configure the network to maximize the user experience and efficient system utilization.

In both cases, single-tone schemes are very well suited for GEO deployments, due to the high multiplexing capacity in the frequency domain and the narrow bandwidth, maximizing power spectral density of the UE transmission, which can effectively counteract the UE transmit power limitations and the challenging link budget.

Observation 1: Both 3.75kHz SCS and 15 kHz are currently deployed in NB-IoT NTN with actual live networks using these SCS.

Proposal 1: Include support for both 3.75kHz and 15kHz SCS, based on real-world operator deployments.

## Single tone vs multi-tone and upcoming NGSO systems

While single tone techniques are extremely versatile and can provide effective multiplexing and efficient system capacity across both GEO and NGSO systems, they still rely on longer transmission durations, which can be sub-optimal in scenarios with fast dynamic fading and multipath.

For example, for devices such as smartphones or animal trackers, which experience fast deep fading and multipath during their normal usage scenarios, the transmission durations required by single tone techniques can often be too long.  Furthermore, they increase the overall transmission latency, especially in half duplex systems such as NB-IoT.

Multi-tone schemes using 15 kHz sub-carrier spacing, on the other hand, are well suited to maximize fast release of the radio channel and optimized scheduling, which would make them even better suited to highly-mobile environments with lots of dynamic fading, however their use in GEO is often limited by the tight link budget.

For upcoming high-performance FR1 NGSO systems, on the other hand, and for both NGSO and GSO systems with the introduction of higher transmit power classes (e.g. HPUE), it is expected that the use of multi-tone will become more and more viable.

In the case of NGSO systems, many of these systems rely on fast beam hopping in order to maximize the tradeoff between satellite payload power and coverage efficiency.  In these systems, beams may be switched on/off rapidly and on schedule that may or may not be aligned with the traffic scheduler.  These schedules are typically based on cell/beam demand, overall traffic flow, congestion needs, coverage control signal requirements and satellite power constraints.  Any transmission scheme or link enhancement technique that relies on long transmission durations and multiple temporal repetitions will impose significant and unnecessary constraints on the beam hopping scheduler, thus causing system inefficiencies.

It expected that all future NB-IoT NTN systems will employ OCC in order to leverage the significant capacity gains, therefore the limitation to single-tone transmission schemes would severely limit the scheduler choices and efficient multiplexing, potentially rendering multi-tone based scheduling completely unusable. This would be very undesirable.

Observation 2: In the presence of fast beam hopping, techniques that rely on time domain repetitions put significantly harder constraints on the scheduler.

Observation 3:  Discarding of frequency-domain OCC techniques for NB-IoT multi-tone may have been premature.

Observation 4: Lack of multi-tone support would severely limit implementation of future systems, in particular high performance FR1 NGSO.

Proposal 2: Specify both single-tone OCC and multi-tone OCC.

# Conclusion

Observation 1: Both 3.75kHz SCS and 15 kHz are currently deployed in NB-IoT NTN with actual live networks using these SCS.

Proposal 1: Include support for both 3.75kHz and 15kHz SCS, based on real-world operator deployments.

Observation 2: In the presence of fast beam hopping, techniques that rely on time domain repetitions put significantly harder constraints on the scheduler.

Observation 3:  Discarding of frequency-domain OCC techniques for NB-IoT multi-tone may have been premature.

Observation 4: Lack of multi-tone support would severely limit implementation of future systems, in particular high performance FR1 NGSO.

Proposal 2: Specify both single-tone OCC and multi-tone OCC.

# References

- R1-2407139 - Views on uplink capacity enhancement for NR NTN (Viasat, Inmarsat)