3GPP TSG RAN WG1 #118	R1-2407186

Maastricht, Netherlands, August 19th – 23rd, 2024

Source: 	Intel Corporation

Title: 	AI/ML for CSI compression

Agenda item:	9.1.3.2

Document for: 	Discussion and Decision

# Introduction

The TR for the Rel-18 study item on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface [1] contains broad analysis of AI/ML-based CSI compression with a two-sided model and CSI prediction with a UE-sided model. However, there is no consensus on the recommendation for normative work for both sub-use cases.

The new work item on AI/ML for NR air interface has been approved in [2]. The objectives of this work item include additional study on CSI feedback enhancement AI/ML sub-use cases: CSI compression with a two-sided model and CSI prediction with a UE-sided model. The additional study is mainly targeting to investigate AI/ML models complexity/performance improving the corresponding gains with respect to existing non-AI/ML-based solutions and other aspects requiring further study/conclusion captured in the TR [1].

Study objectives with corresponding checkpoints in RAN#105 (Sept ’24): - CSI feedback enhancement [RAN1]: - For CSI compression (two-sided model), further study ways to: - Improve trade-off between performance and complexity/overhead - e.g., considering extending the spatial/frequency compression to spatial/temporal/frequency compression, cell/site specific models, CSI compression plus prediction (compared to Rel-18 non-AI/ML based approach), etc. - Alleviate/resolve issues related to inter-vendor training collaboration. while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843. - For CSI prediction (one-sided model), further study performance gain over Rel-18 non-AI/ML based approach and associated complexity, while addressing other aspects requiring further study/conclusion as captured in the conclusions section of the TR 38.843 (e.g., cell/site specific model could be considered to improve performance gain).

In this contribution, we provide considerations and analysis on aspects of AI/ML-based CSI compression with two-sided model including CSI computational complexity, extension of spatial/frequency CSI compression sub-use case, model performance monitoring and inter-vendor collaboration for two-sided AI/ML model training.

This contribution is a revision of [8] with typos fixed (highlighted in red – Table 1, Observation 2, Table 2, Observation 3) and evaluation results with FTP1 traffic model added in Section 2.2 (Table 2a, Table 2b, Observation 3a).

Discussion

## CSI computational complexity

According to the RAN1 conclusion from the Rel-18 study on AI/ML for NR air interface [1], inconclusive observations on the trade-off between performance and complexity/overhead is a reason for the lack of RAN1 consensus on the recommendation of CSI compression for normative work. Most of the AI/ML models used as CSI reconstruction part or CSI generation part for evaluations in Rel-18 SI had complexity larger than 10 MFLOPs which is significantly larger comparing to computation complexity of non-AI/ML CSI feedback which raises questions for the implementation feasibility of AI/ML CSI compression. According to the objective from the Rel-19 WI [2], study for CSI compression should be focused on trade-off between performance and complexity/overhead.

CSI compression complexity has a high importance for the UE power consumption which is a critical factor for the user experience. Besides that, if not dimensioned properly for appropriate trade-off, high CSI compression may lead to system performance degradation due to longer CSI processing times at the UE, or larger number of CPUs (CSI Processing Units) occupied by the CSI report limiting simultaneous use of multiple DL component carriers in carrier aggregation scenario.

The complexity of PMI reconstruction at the gNB side is much smaller comparing to complexity of PMI search for PMI codebooks supported in NR. However, gNB should be able to process CSI feedback from multiple UEs simultaneously. Also, in carrier aggregation scenario, number of CSI reports for processing at the gNB side may be scaled up with the number of component carriers. Multi-fold increase in complexity for CSI reconstruction may lead to higher gNB energy consumption and larger CSI application latency due to gNB processing time.

Thus, it is essential to study complexity for CSI compression based on two-sided AI/ML model and improve RAN1 understanding on the complexity of non-AI/ML CSI compression.

For the case of 32 CSI-RS ports, 2 Rx antennas at the UE, 52 PRBs bandwidth, 4 PRB subband size, CSI-RS density 1, for a UE configured with eType II PMI codebook parameters L = 4, N3 = 13, M = 4, K0 = 16, the complexity of PMI search at the UE can be roughly estimated as 1 MFLOPs. For the same configuration, the complexity of PMI reconstruction at the gNB is up to 20 kFLOPs (0.02 MFLOPs).

Observation 1:

- For the case of 32 CSI-RS ports, 2 Rx antennas at the UE, 52 PRB bandwidth, 4 PRB subband size, CSI-RS density 1, Enhanced Type II PMI codebook with L = 4, N3 = 13, M = 4, K0 = 16.

  - Complexity of PMI search is ~1 MFLOPs.

  - Complexity of PMI reconstruction is ~0.02 MFLOPs.

As it was shown in our contribution to the previous RAN1 meeting [3], the performance of CSI compression with two-sided AI/ML model degrades if the complexity for the utilized model is reduced. Thus, information about model complexity is as important as the CSI overhead value for the AI/ML-based solution. Without RAN1 observation and/or conclusion on the complexity of AI/ML models it would be difficult to decide in RAN plenary on the potential continuation of work on AI/ML CSI compression.

One way to capture the complexity aspects in the corresponding observations is to specify range for AI/ML model complexity values shared by companies together with the corresponding ranges for performance gains. However, as it is stated in [1], the actual model complexity may differ from the model complexity in the evaluation with respect to platform-dependent optimization on model implementations.

Moreover, models designed by companies for the purpose of evaluation in RAN1 with the agreed evaluation assumptions and scenarios might be significantly different from AI/ML models implemented at the UE or gNB side and/or reference models specified in 3GPP. Thus, related observations in the TR should be qualified to indicate that the complexity values presented in the observations or conclusions should not be treated as a precise complexity estimate for implementation of AI/ML-based CSI compression.

Proposal 1:

- For the observations with AI/ML CSI compression performance gains over PMI codebook, capture range for the AI/ML model complexity together with the corresponding range for performance gains.

  - For all the observations with AI/ML model complexity, the following note should be added: “AI/ML model complexity depends on various platform-dependent choices for model implementation. The values reported here should be considered as representative values and not as a precise complexity estimate for implementation of AI/ML-based CSI compression.”.

Evaluation results for localized models

To achieve higher performance gains over eType II PMI codebook and/or lower inference complexity, AI/ML model can be optimized (trained) for a specific local region (e.g., one specific site). RAN1 made the following agreements on AI/ML-based CSI compression using localized models.

Agreement For the evaluation of AI/ML-based CSI compression using localized models in Release 19, study the following aspects of the performance/complexity trade-off when comparing the localized model with a benchmark model that is not localized: - Performance of the localized model that has similar or lower complexity as the benchmark model. - Model complexity of the localized model that achieves similar or better performance as the benchmark model. Agreement For the evaluation of AI/ML-based CSI compression using localized models in Release 19, consider the following options as a starting point to model the spatial correlation in the dataset for a local region: - Option 1: The dataset is derived from UEs dropped within the local region, with spatial consistency modelling as per TR 38.901. - E.g., Dropped in a specific cell or within a specific boundary. - Option 2: By using a scenario/configuration specific to the local region. - E.g., Indoor-outdoor ratio, LOS-NLOS ratio, TXRU mapping, etc. Note: While modelling the spatial correlation, strive to ensure that the dataset distribution also correctly captures the decorrelation due to temporal variations in the channel. To report methods to generate training and testing dataset. Agreement For the results template used to collect evaluation results for AI/ML-based CSI compression using localized models, adopt Table 1 used in Rel-18 as starting point, capturing the generalized model result and the localized model result as separate columns, with the following additions for the localized model: - Dataset description - Local region modelling: e.g., Option 1 or Option 2, and further details - Temporal modelling: e.g., how temporal variation is modelled in train and test sets - Dataset description for generalized model

In accordance with the above agreements, performance evaluations were carried out to compare performance of eType II PMI codebook with AI/ML model for CSI compression trained and tested on a dataset corresponding to a specific local region. UMa scenario with 32 antenna ports at the gNB side and 2 Rx antenna ports at the UE side was considered. 10 MHz bandwidth with 15 kHz subcarrier spacing were assumed for evaluations. For the dataset generation, Option 1 was used. Channels for outdoor UEs with NLOS signal propagation and with spatial consistency procedure specified in TR 38.901 were collected for each UE in a drop and for each cell (corresponding to a sector of one site). Other simulation assumptions are aligned with the RAN1 agreements.

Training dataset consists of 400k samples corresponding to 40k UEs and 10 samples per UE, where each sample corresponds to channels measured 100 ms apart. Testing dataset corresponds to 25k samples with 10 samples per UE as for training dataset. Different UEs were assumed for testing and training while the time instances are the same for both datasets. Spatial domain CSI compression using AI/ML model with transformer backbone was assumed with complexity of 374 MFLOPs and 7 MFLOPs for CSI generation part as well as CSI reconstruction part (network structure is the same for CSI reconstruction part and CSI generation part, except the last and the first layer dimensions respectively). CSI with 64 bits is assumed for the AI/ML model, eTypeII PMI codebook with parameter combination 1 (paramCombination-r16 = 1).

SGCS evaluation results are presented in Table 1 for localized models trained and tested in different cells and simulation drops with SGCS for eType II calculated on testing dataset for each cell/drop. Column for mean vales correspond to SGCS averaged over all the evaluated cells/drops for localized AI/ML models and eType II. Column for global model corresponds to evaluations without spatial consistency but for outdoor UEs with 100% NLOS probability.

Table 1. SGCS evaluation results for localized AI/ML model (64 bits) in comparison to 
eType II PMI codebook (parameter combination 1).

|  | Global model | Cell 1, Drop 1 | Cell 2, Drop 1 | Cell 3, Drop 1 | Cell 1, Drop 2 | Cell 2, Drop 2 | Cell 3, Drop 2 | Average (Site-spec.) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| eType II (1 MFLOPs) | 0.605 | 0.567 | 0.666 | 0.762 | 0.525 | 0.557 | 0.556 | 0.605 |
| AI/ML (270 MFLOPs) | 0.596 | 0.593 | 0.687 | 0.813 | 0.530 | 0.567 | 0.605 | 0.632 |
|  | (-1.5%) | (+4.6) | (+3.2) | (+6.7) | (+1.0%) | (+1.8%) | (+8.9%) | (+4.5%) |
| AI/ML (7 MFLOPs) | 0.562 | 0.556 | 0.656 | 0.758 | 0.512 | 0.527 | 0.560 | 0.595 |
|  | (-7.1%) | (-2.0%) | (-1.6%) | (-0.5%) | (-2.5%) | (-5.3%) | (+0.8%) | (-1.8%) |

As it can be seen from the above evaluation results, SGCS performance gain for localized AI/ML model varies across different cells and drops. Comparing to global AI/ML model, localized model allows to achieve higher performance gain over eType II codebook for higher complexity model. However, for the evaluated scenarios and methods, PMI codebook still outperforms model with lower complexity for localized model.

Observation 2:

- For AI/ML model with complexity 270 MFLOPs per part of two-sided model, training/testing on dataset for a local region allows to achieve higher SGCS gain over eType II codebook when compared to use of a dataset trained over global region without spatial consistency.

- For AI/ML model with complexity 7 MFLOPs per part of two-sided model, higher SGCS values are observed for eType II PMI codebook comparing to AI/ML models trained/tested on datasets for a local region and global region with larger loss for dataset trained over global region without spatial consistency.

For the same scenario used for SGCS evaluation, SLS performance evaluations were carried out. UE throughput values for full-buffer traffic model are presented in Table 2 for general and local environments. General environment corresponds to scenario without spatial consistency and averaged over multiple UE random drops. Local environment corresponds to scenario with spatial consistency averaged over different sets of active UEs in the same cells and over all the simulated cell which corresponds to cell 1-3 and drop 1-2 assumed for SGCS evaluations. The same AI/ML models were used for both SGCS and throughput results – for general environment, global model was used, while for local environment local model was applied for each cell/drop. Table with detailed evaluation assumptions is provided in the Appendix.

Table 2. SLS evaluation results with full-buffer traffic model for localized AI/ML model (64 bits) 
in comparison to eType II PMI codebook (parameter combination 1).

| Environment |  | General |  |  | Local |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CSI compression |  | eTypeII | AI/ML | AI/ML | eTypeII | AI/ML | AI/ML |
| Complexity (MFLOPs) |  | 1 | 7 | 270 | 1 | 7 | 270 |
| UE throughput (Mb/s) | Average | 10.33 (0%) | 9.8 (-5%) | 10.31 (0%) | 8.89 (0%) | 8.77 (-1%) | 9.22 (4%) |
|  | 5% CDF | 4.12 (0%) | 3.79 (-8%) | 4.02 (-2%) | 3.46 (0%) | 3.21 (-7%) | 3.39 (-2%) |
|  | 50% CDF | 9.43 (0%) | 8.83 (-6%) | 9.28 (-2%) | 8.00 (0%) | 7.83 (-2%) | 8.27 (3%) |
|  | 95% CDF | 19.55 (0%) | 19.26 (-1%) | 20.18 (3%) | 17.34 (0%) | 17.59 (1%) | 18.44 (6%) |
| Resource utilization (%) |  | 100% | 100% | 100% | 100% | 100% | 100% |

As it can be seen from the above evaluation results, for high model complexity, while the performance in global environment is similar for AI/ML-based CSI compression and eTypeII PMI codebook, AI/ML models trained and applied locally per each cell outperforms conventional solution in average throughput with ~4% performance gain. In all the simulated cases eTypeII PMI codebook outperforms AI/ML based CSI compression with lower complexity which is aligned with the results for SGCS presented above.

Observation 3:

- For AI/ML model with complexity 270 MFLOPs per part of two-sided model, training/testing on dataset for a local region allows to achieve ~4% gain for average throughput comparing to eTypeII in case of full-buffer traffic.

- For AI/ML model with complexity 7 MFLOPs per part of two-sided model, performance loss is observed comparing to eTypeII PMI codebook in case of full-buffer traffic.

Evaluation results for non-full-buffer traffic model are presented in Table 2a for general environment with global AI/ML models and Table 2b with local environment and local AI/ML models. Performance gains comparing to the benchmark (eType II PMI codebook) are presented in brackets.

Table 2a. SLS evaluation results with non-full buffer traffic model for global AI/ML models (64 bits) 
in comparison to eType II PMI codebook (parameter combination 1).

| CSI compression |  | AIML | AIML | AIML | AIML | AIML | AIML |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Complexity (MFLOPs) |  | 7 | 270 | 7 | 270 | 7 | 270 |
| UE throughput (Mb/s) | Average | 29 (-3%) | 29.6 (-1%) | 35.6 (-1%) | 35.9 (0%) | 42.7 (0%) | 42.8 (0%) |
|  | 5% CDF | 10.7 (-8%) | 11.2 (-4%) | 17.9 (-5%) | 18.4 (-3%) | 37.9 (1%) | 38 (1%) |
|  | 50% CDF | 30 (-2%) | 30.6 (-1%) | 39.1 (-1%) | 39.3 (0%) | 43.7 (0%) | 43.7 (0%) |
|  | 95% CDF | 43.7 (0%) | 43.8 (0%) | 44.2 (0%) | 44.2 (0%) | 44.4 (0%) | 44.4 (0%) |
| Resource utilization (%) |  | 71% | 71% | 52% | 51% | 14% | 14% |

Table 2b. SLS evaluation results with non-full buffer traffic model for local AI/ML models (64 bits) 
in comparison to eType II PMI codebook (parameter combination 1).

| CSI compression |  | AIML | AIML | AIML | AIML | AIML | AIML |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Complexity (MFLOPs) |  | 7 | 270 | 7 | 270 | 7 | 270 |
| UE throughput (Mb/s) | Average | 27.6 (-2%) | 28.3 (1%) | 35 (-1%) | 35.2 (0%) | 42.7 (0%) | 42.7 (0%) |
|  | 5% CDF | 9.6 (-6%) | 10.2 (1%) | 16.8 (-3%) | 17.3 (1%) | 37.6 (-1%) | 37.7 (0%) |
|  | 50% CDF | 27.9 (-2%) | 28.8 (1%) | 38.6 (-1%) | 38.9 (0%) | 43.7 (0%) | 43.7 (0%) |
|  | 95% CDF | 43.6 (0%) | 43.7 (0%) | 44.1 (0%) | 44.2 (0%) | 44.4 (0%) | 44.4 (0%) |
| Resource utilization (%) |  | 72% | 72% | 52% | 52% | 14% | 14% |

Observation 3a:

- For AI/ML model with complexity 270 MFLOPs per part of two-sided model, AI/ML CSI compression with training/testing on dataset for a local region has similar performance comparing to eTypeII in case of non-full buffer traffic.

- For AI/ML model with complexity 7 MFLOPs per part of two-sided model, performance loss is observed comparing to eTypeII PMI codebook in case of non-full buffer traffic.

## Inter-vendor collaboration for two-sided AI/ML model training

One of the major aspects to enable support of two-sided model for CSI compression is alignment of CSI generation part and CSI reconstruction part applied at the UE side and gNB side respectively. During RAN1 #116 meeting, RAN1 agreed to study the following options to alleviate/resolve issues related to inter-vendor training collaboration and model parts alignment for AI/ML-based CSI compression using two-sided model [5]:

- Option 1: Fully standardized reference model (structure + parameters),

- Option 2: Standardized dataset (CSI feedback bits and associated target CSI),

- Option 3: Standardized reference model structure + Parameter exchange between NW-side and UE-side,

- Option 4: Standardized data / dataset format + Dataset exchange between NW-side and UE-side,

- Option 5: Standardized model format + Reference model exchange between NW-side and UE-side.

During Rel-18 SI on AI/ML for air interface, RAN1 extensively discussed pros and cons for different training collaboration types. For consistency, the outcomes from the discussion on training collaboration types should be reused for the discussion on alignment of CSI generalization part and CSI reconstruction part.

For Option 1, it is assumed that collaboration between different companies happens at the stage of reference model standardization. Once a reference model is specified, the model is shared among all the companies through distribution of the corresponding technical specification (TS). From the perspective of a UE vendor which trains UE-part model, the case of specified CSI generation part model is very similar to Type 1 NW side training with transfer of the trained CSI generation part from the NW side to UE-side, while the case of specified CSI reconstruction part model is very similar to Type 2 sequential training with FP and BP information exchange between pre-trained CSI reconstruction part to train CSI generation part. While this approach may be appealing in terms of being the simplest once one or more reference models are standardized, the challenge remains in the standardization efforts and alignment necessary to define the reference models. To appreciate the challenges, the potential list of aspects that may need to be aligned that is currently being discussed in RAN WG4 is presented in Table 3 [4].

Table 3. Potential details that need to be aligned on for Option 1 (fully standardized reference model) (from [4])

| Category | Parameter | Description/Examples |
| --- | --- | --- |
| Model architecture parameters | Model type | Transformer, CNN, RNN, MLP |
|  | Model depth | Number of layers |
|  | Layer type | Fully connected, convolutional, activation layer, etc. |
|  | Layer size | Neuron count and configuration |
|  | Quantization method for the encoder output | Scalar, vector (with codebook) |
|  | Encoder-decoder interface | Number of bits of latent message |
|  | Fixed point representation | Int8, int16, floating point etc |
|  | Format of input to encoder/output of decoder |  |
| Model Training related parameters | Training procedure | FFS (e.g Initialization method, training duration, training completion criteria, collaboration type, encoder assumption, etc) |
|  | Loss function | SGCS, NMSE, etc. |
|  | Training datasets | Channel model, number of Tx/Rx ports Other parameters FFS (e.g. rank) |
|  | Hyperparameters | Learning rate, batch size, regularization techniques and strength, optimization algorithm, etc. |
|  | Cross-validation details | Dataset splits for training/testing/validation |
| Generalization (may be applicable to all four options) | Performance requirements on test dataset(s) | Mean SGCS, etc. |
| Scalability (may be applicable to all four options) | Supported antenna port configurations | (2,8,2), (2,4,2), etc. |
|  | Supported feedback payloads | Low, medium, high overhead (with specified number of bits) |
| Note: Parameters in green are agreed. The parameters in yellow are tentatively agreed. The other parameters are still under discussion. Other parameters that are not yet listed might also be needed. |  |  |

Further, in the email discussion “[111][133] NR_AIML_air - email discussions after RAN4#111 meeting” on 3GPP_TSG_RAN_WG4 email reflector, detailed definition of the model is provided to further study testability issue for the AI/ML CSI compression sub-use case.

For Option 2, a two-sided model shall be trained to derive the dataset with CSI feedback bits and associated target CSI. The training procedure to specify dataset may be done at a single side (at one vendor) or involve more complex procedure with joint or separate training for multiple vendors. After the dataset is specified, CSI generation/reconstruction part model training for actual operation in the field are trained similar to Type 3 separate training procedure.

During RAN1 #116bis meeting, the following conclusions and observations were noted regarding Options 1 and 2 [6]:

Conclusion: - Conclude, from RAN1 perspective, that Option 1, if feasible for specification, eliminate the inter-vendor collaboration complexity (e.g., whether bilateral collaboration is required between vendors). - It is RAN1’s understanding that Option 1 corresponds to RAN4 options, e.g., RAN4-Option3, or RAN4-Option4. Further study and final conclusion on interoperability and RAN4 testing of the RAN4-Option3 and RAN4-Option4 is up to RAN4. Observation - Option 1 and 2 may have limited performance in the field compared to Options 3, 4, and 5, further study is needed - Option 1 and 2 may require high specification effort from RAN1 perspective. Conclusion - Deprioritize Option 2 for inter-vendor training collaboration. - Note: This deprioritization shall not affect the ongoing discussion in RAN4 on RAN4-Option3 and RAN4-Option4.

We focus our attention on Options 3/4/5 in the following.

During RAN1 #116bis meeting, the following agreements were made to further clarify Options 3/4/5 and identify sub-options for Options 3 and 5 [6]:

Agreement - For Option 3, further define the two sub-options: - 3a: Parameters received at the UE or UE-side goes through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., potential re-training, re-development of a different model, and/or offline testing. - 3b: Parameters received at the UE are directly used for inference at the UE without offline engineering, potentially with on-device operations. - For Option 5, further define the two sub-options: - 5a: Model received at the UE or UE-side goes through offline engineering at the UE-side (e.g., UE-side OTT server), e.g., potential re-training, re-development of a different model, and/or offline testing. - 5b: Model received at the UE are directly used for inference at the UE without offline engineering, potentially with on-device operations. - For Option 4, it is clarified that: - Dataset received at the UE or UE-side goes through offline engineering at the UE- side (e.g., UE-side OTT server), e.g., model training or offline testing. - Note: The descriptions under each option are only for the purpose of simplified discussion and do not mean deprioritizing any other flavors (such as an exchange originating from the UE-side and ending at the NW-side) from potential specification. Agreement - For Option 3/4/5, focus further discussion on the following assumptions: - Option 3a/5a - The model(5a)/parameter(3a) exchange originates from the NW-side and ends at the UE-side. - Model(5a)/parameters(3a) exchanged from the NW-side to UE-side is either CSI generation or reconstruction part or both. - Option 3a-1/5a-1: Model/Parameters exchanged from the NW-side to UE-side is CSI generation part. - Option 3a-2/5a-2: Model/Parameters exchanged from the NW-side to UE-side is CSI reconstruction part. - Option 3a-3/5a-3: Model/Parameters exchanged from the NW-side to UE-side are both CSI generation part and CSI reconstruction part. - Some additional information, if necessary, may be shared from the NW-side to help UE-side offline engineering and provide performance guidance. - Performance target - Dataset or information related to collecting dataset - Study different methods of exchanging, e.g., over the air-interface, offline delivery, etc. - Option 3b - The method of exchanging is over the air-interface via model transfer/delivery Case z4. - The parameter exchange is from NW to UE. - Parameters exchanged from the NW-side to UE-side is CSI generation part. - Option 5b - The method of exchanging is over the air-interface via model transfer/delivery Case z4, assuming that the model structure is aligned based on offline inter-vendor collaboration. - The model exchange is from NW to UE. - Model exchanged from the NW-side to UE-side is CSI generation part. - Option 4: - The dataset exchange originates from the NW-side and ends at the UE-side. - Option 4-1: Dataset exchanged from the NW-side to UE-side consists of (target CSI, CSI feedback). - Option 4-2: Dataset exchanged from the NW-side to UE-side consists of (CSI feedback, reconstructed target CSI). - Option 4-3: Dataset exchanged from the NW-side to UE-side consists of (target CSI, CSI feedback, reconstructed target CSI). - Some additional information, if necessary, may be shared from the NW-side to help UE-side offline engineering and provide performance guidance. - Performance target - Study different methods of exchanging, e.g., over the air-interface, offline delivery, etc. - Note: For each option/sub-option of interest, companies to bring discussion on how inter-vendor collaboration complexity, interoperability, and feasibility may be addressed. Companies to strive to provide solution(s) that can address all the following aspects: inter-vendor collaboration complexity, performance, interoperability, and feasibility. - Note: The descriptions under each option are only for the purpose of simplified discussion and do not mean deprioritizing any other flavors (such as an exchange originating from the UE-side and ending at the NW-side) from potential specification.

Option 3 and Option 5 correspond to different cases of Type 1 training (for Options 3a-1/3a-3/3b and 5a-1/5a-3/5b) or Type 3 training starting with NW side (for Options 3a-2 and 5a-2), where, for Option 3 family, constraints are introduced for the model structure and there is exchange of model weights only, while transfer/delivery of model structure with the corresponding weights is considered in Option 5 family.

From the actual model training perspective, Option 4 is similar to Option 2, with the exception that the dataset is not specified, but exchanged between different entities.

As discussed in our companion contribution [7], we consider it more practical to focus on the cases with model/parameter transfer from NW-side to UE-side instead of from UE-to-NW transfer, and thus, focus on the former case in the rest of the discussion that follows.

While Options 3b and 5b are identified to use parameter/model transfer from NW-side to UE-side over the air interface via model transfer/delivery Case z4 (i.e., “model transfer in an open format of a known model structure at UE, i.e., exact model structure has been previously identified between NW and UE and for which the UE has explicitly indicated its support”), for Options 3a and 5a, both offline and over the air interface transfer/delivery may be supported following, e.g., model transfer/delivery Cases y and z4 respectively. Note that model transfer/delivery Cases z2 (at least for UE-sided model), z3, and z5 have been agreed to be deprioritized during prior RAN1 meetings.

Proposal 2:

- For Options 3a and 5a, consider support of offline and over the air interface transfer/delivery following, e.g., model transfer/delivery Cases y and z4 respectively.

Compared to Options 3a-1/5a-1 and 3a-3/5a-3, where the transferred parameters/model from NW-side to UE-side includes the CSI generation part, Options 3a-2/5a-2 offer a higher level of flexibility to UE implementation to optimize the parameters/model(s) for CSI generation part as long as the developed model(s) for CSI generation part are compatible with the parameters/model shared for CSI reconstruction part from the NW-side.

Although, as explained above, both Options 3a/b and Options 5a/b can be seen as realizations of training collaboration Type 1, Options 3a/b can be expected to provide greater flexibility to UE (and NW) implementations compared to Options 5a/b. Note that here, the comparison is between Options 3 and 5 for variants ‘a-1’, ‘a-2’, ‘a-3’, and ‘-b’ respectively, and not between variants ‘a-1’, ‘a-2’, ‘a-3’, and ‘-b’.

Observation 4:

- In terms of implementation flexibility and potential for optimization in the field:

  - Compared to Options 3a-1/5a-1 and 3a-3/5a-3, Options 3a-2/5a-2 offer a higher level of flexibility to UE implementation to optimize the parameters/model(s) for CSI generation part as long as the developed model(s) for CSI generation part are compatible with the parameters/model shared for CSI reconstruction part from the NW-side.

  - Options 3a/b can be expected to provide greater flexibility to UE (and NW) implementations compared to Options 5a/b.

    - Note: Here, the comparison is between Options 3 and 5 for variants ‘a-1’, ‘a-2’, ‘a-3’, and ‘-b’ respectively, and not between variants ‘a-1’, ‘a-2’, ‘a-3’, and ‘-b’.

Due to the need for parameters/model to go through offline engineering for Options 3a/5a, when compared to Options 3b/5b, there are inherent additional latency and burden/efforts that can impact their practical applicability in context of LCM. While offline engineering and re-training/re-development of model(s) at the UE side can be desirable from the perspective of UE implementation, as argued in our companion contribution [7], it may not be an essential component to support cases involving parameters/model transfer from NW to UE. This is true especially for Option 3 family that involves specified model structure(s) with parameters being transferred from NW-side to UE-side. In most cases, given that the model structure is known with the transfer of parameters from the NW-side, it can be possible to use the model for inference with limited on-device adjustments to the received set of parameters without requiring expensive procedures of offline engineering. Thus, in our view, at least for Option 3, Option 3b should be prioritized over Options 3a.

Observation 5:

- Compared to Options 3b/5b, Options 3a/5a can be expected to incur inherent additional latency and burden/efforts that can impact their practical applicability/responsiveness in context of LCM.

- At least for Option 3 family that involves specified model structure(s) with parameters being transferred from NW-side to UE-side, it may not be essential for received parameters from NW-side to go through offline engineering for re-training/re-development.

Proposal 3:

- At least for Option 3 for inter-vendor collaboration, Option 3b is prioritized over Options 3a.

Further, for the Option 3a/5a family and Option 4, it was identified during the last meeting that potential additional information that may be shared from the NW-side to help UE-side offline engineering and provide performance guidance. Towards this, for Option 3a/5a family, the following were listed [6]:

- Some additional information, if necessary, may be shared from the NW-side to help UE-side offline engineering and provide performance guidance. - Performance target - Dataset or information related to collecting dataset

Similarly, for Option 4, the following was listed [6]:

- Some additional information, if necessary, may be shared from the NW-side to help UE-side offline engineering and provide performance guidance. - Performance target

First, we note that if dataset is shared from NW-side to UE-side, then Options 3a/5a effectively turn into combinations of Options 3a/5a and Option 4 respectively. In general, combinations of these options may be feasible and can be considered at a later stage when the basic options are stabilized/down-selected. On the other hand, provision of “information related to collecting dataset” is similar to/same as providing configuration(s)/indication(s) related to data collection that could be interpreted as part of provisions to ensure consistency between training and inference and/or MI-Option 1 (discussed in the context of Model Identification) and is not fundamentally unique to only Options 3a/5a. For instance, the guidance may not be limited for offline engineering only but also for any on-device adjustments in case of Options 3b/5b or Option 4 family. Hence, it would be prudent not to have parallel discussions on provision of “information related to collecting dataset” in the current context of inter-vendor collaboration efforts.

Proposal 4:

- For Options 3a/5a, provision of dataset or information related to collecting data are not considered further in the context of inter-vendor collaboration for two-sided models.

- Note: This does not imply that provision of dataset or information related to collecting data are precluded.

Next, on “performance target”, further discussions are necessary in RAN1 as to how such “targets” may be interpreted by a UE and associated expectations from a UE (if any). Clearly, a UE should not be expected to achieve such “targets” – for that, RAN4 minimum performance requirements should be utilized. The primary reason being that it can be rather challenging to determine such performance targets with high accuracy at the NW side due to dependency on various factors that may be local to the UE or its immediate environment.

Accordingly, any guidance on performance targets can be effectively translated to guidance on model/functionality LCM – for instance, targets/guidance on “CSI quality” in terms of suitable quality metrics and thresholds/targets on CSI/PMI accuracy performance could be provided by the NW to the UE. For instance, metrics for CSI quality determination could be based on NMSE, GCS, SGCS, etc. Alternatively, or additionally, for certain specified or configured CSI quality metrics, a UE could be configured to report the quality of the compressed CSI that the NW may further utilize for model/functionality LCM.

Proposal 5:

- If performance targets are provided by NW-side to UE-side to help UE-side offline engineering and/or on-device adjustments and provide performance guidance, they are interpreted as assistance information and it is not expected that the UE should achieve the corresponding targets.

Proposal 6:

- Towards providing assistance to UE-side to help UE-side offline engineering and/or on-device adjustments and provide performance guidance, further study options for NW-side to provide targets/guidance on “CSI quality” represented using suitable metrics (e.g., NMSE, SGCS, etc.) and thresholds/targets on CSI accuracy performance.

  - Alternatively, or additionally, for certain specified or configured CSI quality metrics, a UE could be configured to report the quality of the compressed CSI that the NW may further utilize for model/functionality LCM.

Options 4-1, 4-2, 4-3 all involve transfer of datasets from NW-side to UE-side, with differences in terms of which model(s) (CSI generation, CSI reconstruction, or both) the different datasets (as input/output pairs) correspond to. Each of these options allow for flexibility in terms of model development, with alignment between the NW-side and UE-side on dataset(s) for CSI generation part, CSI reconstruction part, or both. All these cases can be mapped to training collaboration Type 3, starting with NW-side training. “NW-side training first” is attributed considering the direction of dataset transfer from NW-side to UE-side.

Beyond the above discussion points, we propose to consider mapping of the different options to the training collaboration types identified during the Rel-18 SI. Given the considerable amount of studies and analyses of the different identified characteristics reported in TR 38.843 for the different training collaboration types, such a mapping from the options identified during RAN1 #116 and RAN1 #116bis can facilitate a comprehensive understanding of these options.

Proposal 7:

- RAN1 to discuss mapping of different options for training collaboration agreed at RAN1#116 and training collaboration types assumed for UE/NW part training used in the actual operation at UE/NW side.

  - Consider the below table as a starting point for the discussion.

  - Note: Transfer/delivery from UE-side to NW-side are not listed below for compactness.

|  | For UE-part training | For NW-part training |
| --- | --- | --- |
| Option 1 | CSI generation part is specified: Type 1 NW side training. CSI reconstruction part is specified: Type 2 sequential training. | CSI generation part is specified: Type 2 sequential training. CSI reconstruction part is specified: Type 1 UE side training. |
| Option 2 | Type 3 NW-first | Type 3 UE-first |
| Option 3a-1 | CSI generation part weights are transferred/delivered from NW side to UE side: Type 1 NW side training. |  |
| Option 3a-2 | CSI reconstruction part weights are transferred/delivered from NW side to UE side: Type 3 starting with NW side training. |  |
| Option 3a-3 | CSI generation part and CSI reconstruction part weights are transferred/delivered from NW side to UE side: Type 1 NW side training. |  |
| Option 3b | CSI generation part weights are transferred/delivered from NW side to UE side: Type 1 NW side training. |  |
| Option 4-1/4-2/4-3 | Dataset is transferred/delivered from NW side to UE side: Type 3 NW-first. |  |
| Option 5a-1 | CSI generation part model is transferred/delivered from NW side to UE side: Type 1 NW side training. |  |
| Option 5a-2 | CSI generation part model is transferred/delivered from NW side to UE side: Type 3 starting with NW side training. |  |
| Option 5a-3 | CSI generation part and CSI reconstruction part model is transferred/delivered from NW side to UE side: Type 1 NW side training. |  |
| Option 5b | CSI generation part model is transferred/delivered from NW side to UE side: Type 1 NW side training. |  |

Some entries of the tables for pros and cons of training collaboration types (Table 5.1-1 and Table 5.1-2 in [1]) are not filled in due to lack of consensus in Rel-18 SI on AI/ML for air interface. Given that study on inter-vendor collaboration is not yet finalized, pros and cons for different training collaboration types shall be further discussed based on progress achieved in Rel-18.

Proposal 8:

- Training collaboration types are further discussed based on progress achieved in the Rel-18 SI.

  - Re-discuss entries in Table 5.1-1 and Table 5.1-2 of TR 38.843 where consensus has not been reached.

Type 1 training: Extendibility. After training procedure is finished and the corresponding models are deployed, it is still possible to update one of the models for Type 1 training. For example, for NW side training, CSI reconstruction part model can be updated such that it is still compatible with the CSI generation part model in use. The above is possible since the side which trained the AI/ML model originally has all the information to train new model which is compatible with the old one. For example, it can be done by using CSI generated by the deployed CSI generation part model for training of the new/updated NW side model. Similar logic is applied to UE side training for update of CSI generation part model.

Proposal 9:

- For CSI compression using two-sided model use case, the following changes (in red) are endorsed for the table with the pros/cons of training collaboration type 1.

| Training type Characteristics | Type1: NW side |  | Type 1: UE side |  |
| --- | --- | --- | --- | --- |
|  | Unknown model structure at UE | Known model structure at UE | Unknown model structure at NW | Known model structure at NW |
| Extendibility: to train new UE-side model compatible with NW-side model in use; | Yes | Yes | Yes | Yes |
| Extendibility: To train new NW-side model compatible with UE-side model in use | Yes | Yes | Yes | Yes |

Type 2 sequential training: Feasibility of allowing UE and NW side to develop/update models separately. For Type 2 sequential training it is assumed that NW side holds trained CSI reconstruction part and UE side can train CSI generation part by obtaining FP/BP (Forward/Backward propagation) information from the NW side. In this case CSI reconstruction part model can be developed/updated at the NW side separately from the development/update of CSI generation part model at the UE side (i.e., models may have an arbitrary model structure except alignment on input/output formats and quantization). After model is developed/updated, UE side or NW side shall initiate/request model training procedure. Thus, in our view, it is feasible to allow UE and NW side to develop/update models separately for Type 2 sequential training.

Extendibility for type 3 training.

For NW first Type 3 training, after training procedure is completed and both NW side and UE side models are deployed, it is still possible to update NW side model while maintaining compatibility with the deployed UE side model. This can be done by using training dataset and CSI generation part used at the first step of separate training procedure. Also, in this case NW can evaluate the performance of the new NW side model by using a testing dataset and decide whether to deploy the new model or not. Thus, extendibility to train new NW-side model compatible with UE-side model in use is supported for NW first Type 3 training. Similar logic is applied for extendibility to train new UE-side model compatible with NW-side model in use in case of UE first Type 3 training.

Proposal 10:

- For CSI compression using two-sided model use case, the following changes (in red) are endorsed for the table with the pros/cons of training collaboration types 2 and 3.

| Training types Characteristics | Type 2 |  | Type 3 |  |
| --- | --- | --- | --- | --- |
|  | Simultaneous | Sequential NW first (note 1) | NW first | UE first |
| Feasibility of allowing UE side and NW side to develop/update models separately | Infeasible | Feasible | Feasible | Feasible |
| Extendibility: to train new UE-side model compatible with NW-side model in use; | Not support | Support | Support | Support |
| Extendibility: To train new NW-side model compatible with UE-side model in use | Not support | Not Support | Support | Support |

## Discussion on RAN1 conclusion for AI/ML CSI compression

AI/ML CSI compression is the only use-case under study which corresponds to two-sided AI/ML model implemented with CSI generation part at the UE and CSI reconstruction part at the gNB. Comparing to other use-cases with one-sided AI/ML model, AI/ML CSI compression requires significant efforts in 3GPP to specify a solution for inter-vendor model training and model testing.

Another significant issue is AI/ML model development for CSI generation and CSI reconstruction. Development of AI/ML model requires detailed optimization of performance and complexity considering such aspects as model generalization and scalability. If a reference AI/ML model is specified in 3GPP, targeting use in the field, discussion on the model design is expected to take large amount of time units. Data collection, AI/ML model design and model training outside of 3GPP may require significant efforts and resources from vendors and/or operators.

Additional features such as temporal-domain compression and site-specific models opens more issues for consideration. Support of localized (site-specific) models might require model transfer via air interface introducing additional signalling overhead which should be considered in model design. Solution for temporal-domain CSI compression shall consider different time-domain CSI configurations including aperiodic/periodic measurements and reporting. AI/ML model for periodic CSI measurements/reporting might be very different from the AI/ML model for aperiodic operation (e.g., recurrent neural network and feed-forward neural network respectively).

Based on the complexity of AI/ML models used for RAN1 evaluations so far, ease of implementation of AI/ML CSI compression is questionable for both UE side and gNB side. As discussed in section 2.1 of this contribution, high complexity leads to larger hardware cost, increased power consumption and/or performance degradation due to potentially larger CSI latency and larger number of occupied CSI processing units.

The above factors, coupled with the rather modest gains (and losses in some cases) observed in our evaluations, make AI/ML-based solution for CSI compression less appealing for implementation and deployment.

Considering the above, specification of AI/ML CSI compression with two-sided model is a challenging task for completion in Rel-19 timeframe.

Observation 6:

- High model complexity, limited performance gains, unresolved inter-vendor training collaboration, and testability issues make it very challenging to support AI/ML CSI compression with two-sided model in Rel-19 timeframe.

Conclusion

In this contribution, we have provided our views on AI/ML-based CSI compression sub-use case. The following observations and proposals were made.

Observation 1:

- For the case of 32 CSI-RS ports, 2 Rx antennas at the UE, 52 PRB bandwidth, 4 PRB subband size, CSI-RS density 1, Enhanced Type II PMI codebook with L = 4, N3 = 13, M = 4, K0 = 16.

  - Complexity of PMI search is ~1 MFLOPs.

  - Complexity of PMI reconstruction is ~0.02 MFLOPs.

Proposal 1:

- For the observations with AI/ML CSI compression performance gains over PMI codebook, capture range for the AI/ML model complexity together with the corresponding range for performance gains.

  - For all the observations with AI/ML model complexity, the following note should be added: “AI/ML model complexity depends on various platform-dependent choices for model implementation. The values reported here should be considered as representative values and not as a precise complexity estimate for implementation of AI/ML-based CSI compression.”.

Observation 2:

- For AI/ML model with complexity 374 MFLOPs per part of two-sided model, training/testing on dataset for a local region allows to achieve higher SGCS gain over eType II codebook when compared to use of a dataset trained over global region without spatial consistency.

- For AI/ML model with complexity 7 MFLOPs per part of two-sided model, higher SGCS values are observed for eType II PMI codebook comparing to AI/ML models trained/tested on datasets for a local region and global region with larger loss for dataset trained over global region without spatial consistency.

Observation 3:

- For AI/ML model with complexity 374 MFLOPs per part of two-sided model, training/testing on dataset for a local region allows to achieve ~4% gain for average throughput comparing to eTypeII in case of full-buffer traffic.

- For AI/ML model with complexity 7 MFLOPs per part of two-sided model, performance loss is observed comparing to eTypeII PMI codebook in case of full-buffer traffic.

Proposal 2:

- For Options 3a and 5a, consider support of offline and over the air interface transfer/delivery following, e.g., model transfer/delivery Cases y and z4 respectively.

Observation 4:

- In terms of implementation flexibility and potential for optimization in the field:

  - Compared to Options 3a-1/5a-1 and 3a-3/5a-3, Options 3a-2/5a-2 offer a higher level of flexibility to UE implementation to optimize the parameters/model(s) for CSI generation part as long as the developed model(s) for CSI generation part are compatible with the parameters/model shared for CSI reconstruction part from the NW-side.

  - Options 3a/b can be expected to provide greater flexibility to UE (and NW) implementations compared to Options 5a/b.

    - Note: Here, the comparison is between Options 3 and 5 for variants ‘a-1’, ‘a-2’, ‘a-3’, and ‘-b’ respectively, and not between variants ‘a-1’, ‘a-2’, ‘a-3’, and ‘-b’.

Observation 5:

- Compared to Options 3b/5b, Options 3a/5a can be expected to incur inherent additional latency and burden/efforts that can impact their practical applicability/responsiveness in context of LCM.

- At least for Option 3 family that involves specified model structure(s) with parameters being transferred from NW-side to UE-side, it may not be essential for received parameters from NW-side to go through offline engineering for re-training/re-development.

Proposal 3:

- At least for Option 3 for inter-vendor collaboration, Option 3b is prioritized over Options 3a.

Proposal 4:

- For Options 3a/5a, provision of dataset or information related to collecting data are not considered further in the context of inter-vendor collaboration for two-sided models.

- Note: This does not imply that provision of dataset or information related to collecting data are precluded.

Proposal 5:

- If performance targets are provided by NW-side to UE-side to help UE-side offline engineering and/or on-device adjustments and provide performance guidance, they are interpreted as assistance information and it is not expected that the UE should achieve the corresponding targets.

Proposal 6:

- Towards providing assistance to UE-side to help UE-side offline engineering and/or on-device adjustments and provide performance guidance, further study options for NW-side to provide targets/guidance on “CSI quality” represented using suitable metrics (e.g., NMSE, SGCS, etc.) and thresholds/targets on CSI accuracy performance.

  - Alternatively, or additionally, for certain specified or configured CSI quality metrics, a UE could be configured to report the quality of the compressed CSI that the NW may further utilize for model/functionality LCM.

Proposal 7:

- RAN1 to discuss mapping of different options for training collaboration agreed at RAN1#116 and training collaboration types assumed for UE/NW part training used in the actual operation at UE/NW side.

  - Consider the below table as a starting point for the discussion.

  - Note: Transfer/delivery from UE-side to NW-side are not listed below for compactness.

|  | For UE-part training | For NW-part training |
| --- | --- | --- |
| Option 1 | CSI generation part is specified: Type 1 NW side training. CSI reconstruction part is specified: Type 2 sequential training. | CSI generation part is specified: Type 2 sequential training. CSI reconstruction part is specified: Type 1 UE side training. |
| Option 2 | Type 3 NW-first | Type 3 UE-first |
| Option 3a-1 | CSI generation part weights are transferred/delivered from NW side to UE side: Type 1 NW side training. |  |
| Option 3a-2 | CSI reconstruction part weights are transferred/delivered from NW side to UE side: Type 3 starting with NW side training. |  |
| Option 3a-3 | CSI generation part and CSI reconstruction part weights are transferred/delivered from NW side to UE side: Type 1 NW side training. |  |
| Option 3b | CSI generation part weights are transferred/delivered from NW side to UE side: Type 1 NW side training. |  |
| Option 4-1/4-2/4-3 | Dataset is transferred/delivered from NW side to UE side: Type 3 NW-first. |  |
| Option 5a-1 | CSI generation part model is transferred/delivered from NW side to UE side: Type 1 NW side training. |  |
| Option 5a-2 | CSI generation part model is transferred/delivered from NW side to UE side: Type 3 starting with NW side training. |  |
| Option 5a-3 | CSI generation part and CSI reconstruction part model is transferred/delivered from NW side to UE side: Type 1 NW side training. |  |
| Option 5b | CSI generation part model is transferred/delivered from NW side to UE side: Type 1 NW side training. |  |

Proposal 8:

- Training collaboration types are further discussed based on progress achieved in the Rel-18 SI.

  - Re-discuss entries in Table 5.1-1 and Table 5.1-2 of TR 38.843 where consensus has not been reached.

Proposal 9:

- For CSI compression using two-sided model use case, the following changes (in red) are endorsed for the table with the pros/cons of training collaboration type 1.

| Training type Characteristics | Type1: NW side |  | Type 1: UE side |  |
| --- | --- | --- | --- | --- |
|  | Unknown model structure at UE | Known model structure at UE | Unknown model structure at NW | Known model structure at NW |
| Extendibility: to train new UE-side model compatible with NW-side model in use; | Yes | Yes | Yes | Yes |
| Extendibility: To train new NW-side model compatible with UE-side model in use | Yes | Yes | Yes | Yes |

Proposal 10:

- For CSI compression using two-sided model use case, the following changes (in red) are endorsed for the table with the pros/cons of training collaboration types 2 and 3.

| Training types Characteristics | Type 2 |  | Type 3 |  |
| --- | --- | --- | --- | --- |
|  | Simultaneous | Sequential NW first (note 1) | NW first | UE first |
| Feasibility of allowing UE side and NW side to develop/update models separately | Infeasible | Feasible | Feasible | Feasible |
| Extendibility: to train new UE-side model compatible with NW-side model in use; | Not support | Support | Support | Support |
| Extendibility: To train new NW-side model compatible with UE-side model in use | Not support | Not Support | Support | Support |

Observation 6:

- High model complexity, limited performance gains, unresolved inter-vendor training collaboration, and testability issues make it very challenging to support AI/ML CSI compression with two-sided model in Rel-19 timeframe.

References

- 3GPP TR 38.843 Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface (Release 18), V2.0.0 (2023-12).

- RP-234039 New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm (Moderator), 3GPP TSG RAN Meeting #102, Edinburgh, Scotland, December 11-15, 2023.

- R1-2403976 AI/ML for CSI compression, Intel Corporation

- R4-2403712, WF on AI/ML, Qualcomm, RAN4 #110, February 2024.

- 3GPP RAN1 Chairman’s notes, RAN1 #116, February 2024.

- 3GPP RAN1 Chairman’s notes, RAN1 #116bis, April 2024.

- R1-2406018 Other study aspects of AI/ML for air interface, Intel Corporation.

- R1-2406017 AI/ML for CSI compression, Intel Corporation

Appendix

Table 4. System level simulation assumptions for Dense Urban Macro

| Parameter |  | Value |
| --- | --- | --- |
| Carrier frequency |  | 2 GHz |
| Cell layout |  | 1 site with 3 sectors (cells). Cell size according to 38.901 with ISD of 200 m. |
| Channel model |  | According to TR 38.901, NLOS only. With/without spatial consistency. |
| Antenna setup and port layouts at gNB |  | 32 ports: (8,8,2,1,1,2,8), (dH,dV) = (0.5, 0.8)λ |
| Antenna setup and port layouts at UE |  | 2 Rx: (1,1,2,1,1,1,1), (dH,dV) = (0.5, 0.5)λ |
| BS Tx power |  | 41 dBm |
| BS antenna height |  | 25 m |
| UE antenna height |  | 1.5 m |
| UE receiver noise figure |  | 9 dB |
| Bandwidth |  | 10 MHz (52 PRB) |
| Numerology | Slot/non-slot | 14 OFDM symbol slot |
|  | SCS | 15 kHz |
| CSI feedback |  | CSI periodicity is 5 ms, CSI delay is 4 ms, CSI subband size is 4 PRBs. Max UE rank 1. |
| MU-MIMO |  | Max BS rank 8. |
| UE distribution |  | 100% outdoor (3 km/h), 10 UEs per cell. |